3GPP TSG RAN WG1 #108-e			                                R1-2201868

e-Meeting, February 21st – March 3rd, 2022

Agenda item:	8.8.1.1

Source: 	CMCC

Title: 	Remaining issues on PUSCH repetition type A enhancement

Document for:	Discussion and Decision

Introduction

In the RAN1 #107bis-e meeting, remaining issues, text proposals and RRC parameters were discussed.

Conclusions are drawn for some of them. Some are still left for further discussion.

In this contribution, we provide our views on the remaining issues and some of the TPs.

Discussion on the remaining issues

- 2.1 Issue #2-6 The slot indicated by K2 offset for the available slot counting

During the discussion, it was suggested having the clarification that whether the slot indicated by K2 offset is by default an available slot or not. But the opinions were not convergent. Feature lead provided 4 statements searching for companies’ opinions.

- For DG-PUSCH, when AvailableSlotCounting is enables, and for K=1, select on from the following - Option 1: A UE does not assume that PUSCH symbols overlap with DL symbol or SSB symbol in the slot indicated by K2 offset is the slot which is not counted in K available slot(s). - Option 2: A UE assumes that PUSCH symbols can overlap with DL symbol or SSB symbol in the slot indicated by K2 offset. - Option 2-1: When overlapping, the UE drops the PUSCH transmission in the slot indicated by K2 offset and does not perform the PUSCH transmission in later slots, either. - Option 2-2: When overlapping, the UE does not perform the PUSCH transmission in the slot indicated by K2 offset and performs the PUSCH transmission in the next available slot subject to PUSCH dropping rules. - For DG-PUSCH, when AvailableSlotCounting is enables, and for K>1, select on from the following - Option 1: A UE does not assume that the slot indicated by K2 offset is the slot which is not counted in K available slot(s). - Option 2: A UE assumes that the slot indicated by K2 offset can be the slot which is not counted in K available slot(s). - For CG-PUSCH, when AvailableSlotCounting is enables, and for K=1, select on from the following - Option 1: A UE does not assume that PUSCH symbols overlap with DL symbol or SSB symbol in the slot determined in 38.321 Section 5.8.2. - Option 2: A UE assumes that PUSCH symbols can overlap with DL symbol or SSB symbol in the slot determined in 38.321 Section 5.8.2. - Option 2-1: When overlapping, the UE drops the PUSCH transmission in the slot determined in 38.321 Section 5.8.2 and does not perform the PUSCH transmission in later slots, either. - Option 2-2: When overlapping, the UE does not perform the PUSCH transmission in the slot determined in 38.321 Section 5.8.2 and performs the PUSCH transmission in the next available slot subject to PUSCH dropping rules. - For CG-PUSCH, when AvailableSlotCounting is enables, and for K>1, select on from the following - Option 1: A UE does not assume that the slot determined in 38.321 Section 5.8.2 is the slot which is not counted in K available slot(s). - Option 2: A UE assumes that the slot determined in 38.321 Section 5.8.2 can be the slot which is not counted in K available slot(s).

From our understanding, the available slot defined in Rel-17 should fit to all the repetitions including both K=1 and K>1. No exceptional case is defined during the work item. Then the determination procedure is same for all K values. That is if the 1st slot indicated by K2 does not fulfil the indicated time domain resource allocation, it should be considered as not available. UE should find and use the next available slot after that. Based on the determination procedure of available slot, gNB and UE should have the same understanding of which slot is available for PUSCH transmission. Both gNB and UE should have the same knowledge about whether the K2 indicated slot is available or not. This is also the same case for the legacy PUSCH transmission with and without repetitions.

Observation 1:

No exceptional case is defined for the available slot determination. All K repetitions should be transmitted in the available slots.

For the legacy UE, single slot PUSCH transmission would be scheduled by gNB but with no guarantee that the K2 indicated slot can fulfil the time domain resource allocation. This is straightforward that gNB have the full information of resource allocation and occupation and gNB could and should perform the scheduling properly.

When the repetition or aggregation factor is configured, there is also no guarantee that all the consecutive physical slots can fulfill the time domain resource allocations. The slot which cannot fulfil the TDRA would be dropped but the repetition is still counted. The aggregation factor should be set larger than 1. The 1st slot of the aggregated slots has the possibility of not fulfilling the TDRA allocations. And there is no explicit guarantee from the specification that the 1st slot should fulfil the TDRA and PUSCH transmission in that slot would happen. For the repetition factor numberOfRepetitions, it could be configured as a single transmission or multiple transmissions. For the 1st slot of repetitions or single slot transmission, there is also no guarantee that the slot indicated by K2 should fulfil the TDRA allocation and the PUSCH in that slot would happen.

Observation 2:

For single slot transmission and multiple slot transmission of PUSCH, there is no guarantee from the specification that the 1st slot indicated by K2 should fulfil the TDRA allocation. Whether the 1st slot fulfils the TDRA allocation depends on gNB scheduling.

Then based on the observations from above, the legacy UEs (Rel-15/16) does not have the assumptions that the slot indicated by K2 should fulfil the TDRA and the PUSCH in that slot would happens. And for the UE with Rel-17 repetition enhancements, the UEs should implement the available slot counting procedure to the all the K values including K=1. Then for DG when AvailableSlotCounting is enabled, and for K=1, option 2-2 is preferred. For DG-PUSCH, when AvailableSlotCounting is enabled, and for K>1, option 2 is preferred. And it is similar for the CG-PUSCH transmission.

Proposal 1:

For both DG and CG-PUSCH, when AvailableSlotCounting is enabled, UE should assume that the PUSCH symbols can overlap with DL symbols or SSB symbols for both K=1 and K>1.

Proposal 2:

More specifically, for each question from 4th round,

- For DG-PUSCH, when AvailableSlotCounting is enables, and for K=1, Option 2-2 is preferred.

  - Option 2-2: When overlapping, the UE does not perform the PUSCH transmission in the slot indicated by K2 offset and performs the PUSCH transmission in the next available slot subject to PUSCH dropping rules.

- For DG-PUSCH, when AvailableSlotCounting is enables, and for K>1, Option 2 is preferred.

  - Option 2: A UE assumes that the slot indicated by K2 offset can be the slot which is not counted in K available slot(s).

- For CG-PUSCH, when AvailableSlotCounting is enables, and for K=1, Option 2-2 is preferred.

  - Option 2-2: When overlapping, the UE does not perform the PUSCH transmission in the slot determined in 38.321 Section 5.8.2 and performs the PUSCH transmission in the next available slot subject to PUSCH dropping rules.

- For CG-PUSCH, when AvailableSlotCounting is enables, and for K>1, Option 2 is preferred.

  - Option 2: A UE assumes that the slot determined in 38.321 Section 5.8.2 can be the slot which is not counted in K available slot(s).

- 2.2 Issue#2-5: Available slot counting for PUSCH repetition Type A with K=1

During the discussion in the last meeting, FL’s proposal to Issue #2-5 is updated as below.

For DG-PUSCH when AvailableSlotCounting is enabled, select one from:

- A slot for DG-PUSCH repetition Type A with K=1 is determined by physical slots.

- A slot for DG-PUSCH repetition Type A with K=1 is determined by available slots.

For CG-PUSCH when AvailableSlotCounting is enabled, select one from:

- A slot for CG-PUSCH repetition Type A with K=1 is determined by physical slots.

- A slot for CG-PUSCH repetition Type A with K=1 is determined by available slots.

And questions are raised for 2nd round.

Q1: Do you agree on the above updated FL proposal to Issue#2-5?

Q2: Please provide your preference for DG-PUSCH and for CG-PUSCH.

As discussed in section 2.1, when the available slot counting is enabled, it should apply to all the repetition factors, including K=1. Currently, we do not have any agreements to support the exceptional case of K=1. Then according to the definition of the available slot counting, the slot for both DG-PUSCH repetition type A with K=1 should be determined by available slots. In addition, DG-PUSCH and CG-PUSCH should have a unified determining rule for this issue, since there is no motivation to treat the DG and CG PUSCH with variant rules.

Proposal 3:

Based on current agreements, for both DG-PUSCH and CG-PUSCH, when AvailableSlotCounting is enabled, the slot for DG/CG PUSCH repetition Type A with K=1 is determined by available slots.

Proposal 4:

DG-PUSCH and CG-PUSCH should adopt a unified determining rule for the available slot counting for PUSCH repetition type A with K=1.

The conflict is between the K2 indication and the definition of the available slots. K2 is used to indicate the physical slot which is allocated for PUSCH or the 1st slot for the PUSCH repetitions. But if the indicated slot cannot fulfil the TDRA allocation, the actual transmission should be either cancelled but still counted or delayed to the next available slot after the one indicated by K2. According to the definition of available slot counting, the PUSCH repetitions should be transmitted in the available slot. Then the behaviour is to delay K=1 repetition to the next available slot. If the K2 indicated slot is an available slot, there won’t be any conflict. The key issue here is still whether the slot indicated by K2 should be an available slot. As we discussed in the section 2.1, it depends on gNB’s scheduling. According to the current conclusion from WI, the behaviour is clear for the case that the slot indicated by K2 is not an available slot meanwhile the repetition factor K=1. No additional modification to the specification is needed.

Proposal 5:

Current conclusions cover the case that the slot indicated by K2 is not an available slot while the repetition factor equals 1. No additional modification to the specification is needed.

- 2.3 Issue#2-4: Available slot counting for DG-PUSCH scheduled by DCI format 0_0 with other RNTI than TC-RNTI

As we discussed in the previous meetings, DCI format 0_0 cannot schedule the PUSCH with repetitions in Rel-15 and Rel-16. And in Rel-17 RAN1 also do not support PUSCH repetition with other RNTI than TC-RNTI. The agreements in #107e are copied below for reference

Agreement - Rel-17 does not support numberOfRepetitions-r17 for DG-PUSCH scheduled by DCI format 0_0 and for Type 2 CG-PUSCH activated by DCI format 0_0. - repK-r17 supporting up-to-32 repetitions is introduced and is applicable to Type 1 CG-PUSCH and Type 2 CG-PUSCH (irrespective of the activating DCI format). - Note: No RAN1 spec impact is expected. - The possible values of repK-r17 includes 16 and 32. FFS: other values. - numberOfRepetitions-r17 is not applicable to Type 1 CG-PUSCH repetition type A. Agreement - All the following combinations support the counting based on available slots. - DG-PUSCH with Rel-15 repetition factor - Type-1 CG-PUSCH with Rel-15 repetition factor - Type-2 CG-PUSCH with Rel-15 repetition factor - DG-PUSCH with Rel-16 repetition factor - Type-2 CG-PUSCH with Rel-16 repetition factor - DG-PUSCH with Rel-17 repetition factor - Type-1 CG-PUSCH with Rel-17 repetition factor, if supported in Issue#1-1 - Type-2 CG-PUSCH with Rel-17 repetition factor

The available slot counting is defined for the PUSCH repetition type A. And since the DCI format 0_0 with other RNTI than TC-RNTI cannot schedule the PUSCH with repetitions, then the available slot counting should not apply to DG-PUSCH scheduled by DCI format 0_0 with RNTI than TC-RNTI even when AvailableSlotCounting is enabled.

Proposal 6:

The available slot counting should not apply to DG-PUSCH scheduled by DCI format 0_0 with RNTI than TC-RNTI even when AvailableSlotCounting is enabled.

- 2.4 Issue#3-2: Corrections on frequency hopping

Though the title of issue #3-2 is for the corrections on frequency hopping, the actual issue is for the description of PUSCH scheduled by DCI format 0_0 and whether the DCI format 0_0 can schedule PUSCH repetition Type A. In the 4th round discussion of this issue, the discussion is converged to two options as below.

Option 1

6.3.1 Frequency hopping for PUSCH repetition Type A and for TB processing over multiple slots For PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2 and for TB processing over multiple slots (as determined according to procedures defined in Clause 6.1.2.1 for scheduled PUSCH, or Clause 6.1.2.3 for configured PUSCH) and for PUSCH scheduled by DCI format 0_0 with CRC scrambled by C-RNTI, CS-RNTI or MCS-C-RNTI, a UE is configured for frequency hopping by the higher layer parameter frequencyHoppingDCI-0-2 in pusch-Config for PUSCH transmission scheduled by DCI format 0_2, and by frequencyHopping provided in pusch-Config for PUSCH transmission scheduled by a DCI format other than 0_2, and by frequencyHopping provided in onfiguredGrantConfig for configured PUSCH transmission. For PUSCH repetition Type A scheduled by RAR UL grant or by DCI format 0_0 with CRC scrambled by TC-RNTI, a UE is configured for frequency hopping by the frequency hopping flag information field of the RAR UL grant, and by the frequency hopping flag information field of DCI format 0_0 with CRC scrambled by TC-RNTI, respectively. One of two frequency hopping modes can be configured: - Intra-slot frequency hopping, applicable to single slot and multi-slot configured PUSCH transmission and multi-slot PUSCH transmission scheduled by DCI format 0_1 or 0_2 and each of multiple PUSCH transmissions scheduled by a DCI if the higher layer parameter pusch-TimeDomainAllocationListForMultiPUSCH is configured. - Inter-slot frequency hopping, applicable to multi-slot PUSCH transmission.

Option 2 (with insertion of “repetition Type A”)

6.3.1 Frequency hopping for PUSCH repetition Type A and for TB processing over multiple slots For PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2 and for TB processing over multiple slots (as determined according to procedures defined in Clause 6.1.2.1 for scheduled PUSCH, or Clause 6.1.2.3 for configured PUSCH) and for PUSCH repetition Type A scheduled by DCI format 0_0 with CRC scrambled by C-RNTI, CS-RNTI or MCS-C-RNTI, a UE is configured for frequency hopping by the higher layer parameter frequencyHoppingDCI-0-2 in pusch-Config for PUSCH transmission scheduled by DCI format 0_2, and by frequencyHopping provided in pusch-Config for PUSCH transmission scheduled by a DCI format other than 0_2, and by frequencyHopping provided in onfiguredGrantConfig for configured PUSCH transmission. For PUSCH repetition Type A scheduled by RAR UL grant or by DCI format 0_0 with CRC scrambled by TC-RNTI, a UE is configured for frequency hopping by the frequency hopping flag information field of the RAR UL grant, and by the frequency hopping flag information field of DCI format 0_0 with CRC scrambled by TC-RNTI, respectively. One of two frequency hopping modes can be configured: - Intra-slot frequency hopping, applicable to single slot and multi-slot configured PUSCH transmission and multi-slot PUSCH transmission scheduled by DCI format 0_1 or 0_2 and each of multiple PUSCH transmissions scheduled by a DCI if the higher layer parameter pusch-TimeDomainAllocationListForMultiPUSCH is configured. - Inter-slot frequency hopping, applicable to multi-slot PUSCH transmission.

As we discussed during the 107bis meeting and in above, the DCI format 0_0 with other RNTI than TC-RNTI cannot scheduled the PUSCH with repetitions in Rel-15/16 and Rel-17. We cannot use the “PUSCH repetition Type A scheduled by DCI format 0_0 with CRC scrambled by …” and giving the misleading impression that the DCI format 0_0 can schedule the repetitions. Then the Option 1 without insertion of “repetition Type A ” is preferred.

Proposal 7:

The TP without insertion of “repetition Type A” provide correct information that DCI format 0_0 cannot schedule PUSCH repetitions is preferred.

In addition, some company mentioned that the description in TS 38.331 is also confusing whether the DCI format 0_0 support PSUCH repetitions.

frequencyHopping The value intraSlot enables 'Intra-slot frequency hopping' and the value interSlot enables 'Inter-slot frequency hopping'. If the field is absent, frequency hopping is not configured for 'pusch-RepTypeA' (see TS 38.214 [19], clause 6.3). The field frequencyHopping applies to DCI format 0_0 and 0_1 for 'pusch-RepTypeA'.

No repetition transmission is supported by DCI format 0_0 scheduling. Then only intra-slot frequency hopping is supported for DCI format 0_0. But current configuration of frequency hopping covers both intra and inter slot frequency hopping. And the last sentence for the description of frequency hopping could be mis-interpreted as both DCI format 0_0 and 0_1 support 'pusch-RepTypeA', which is the same issue we try to avoid in the TP of frequecy hopping.

Another understanding is that the single slot PUSCH schduled by DCI format 0_0 is included in the general concept of ”PUSCH repetition Type A”. From our understanding, repetition Type A with K=1 could be considered as single slot PUSCH transmission. But the conflicted part is that, the DCI format 0_0 cannot schedule the PUSCH with repetitions even K =1. And the PUSCH repetition type A is introduced in Rel-16 and depends on UE capabilities. Single slot PUSCH scheduled by DCI format 0_0 is introduced in Rel-15 and is mandated to be supported without UE capabilities. If the ”PUSCH repetition Type A” cannot be supported by one UE, can we interpret that the UE cannot support DCI format 0_0 scheduling as well ? It is proposed to clarify the above issues which will facilitate the discussion of issue #3-2.

Proposal 8:

It should be clarified the following issues,

- Whether the PUSCH repetition type A covers both PUSCH with repetitions and DCI format 0_0 scheduled single slot PUSCH

- Whether the frequencyHopping is applied to DCI format 0_0.

Conclusions

In this contribution, we provide our views on the remaining issues and some of the TPs. The observations and proposals are as below.

Observation 1:

No exceptional case is defined for the available slot determination. All K repetitions should be transmitted in the available slots.

Observation 2:

For single slot transmission and multiple slot transmission of PUSCH, there is no guarantee from the specification that the 1st slot indicated by K2 should fulfil the TDRA allocation. Whether the 1st slot fulfils the TDRA allocation depends on gNB scheduling.

Proposal 1:

For both DG and CG-PUSCH, when AvailableSlotCounting is enabled, UE should assume that the PUSCH symbols can overlap with DL symbols or SSB symbols for both K=1 and K>1.

Proposal 2:

More specifically, for each question from 4th round,

- For DG-PUSCH, when AvailableSlotCounting is enables, and for K=1, Option 2-2 is preferred.

  - Option 2-2: When overlapping, the UE does not perform the PUSCH transmission in the slot indicated by K2 offset and performs the PUSCH transmission in the next available slot subject to PUSCH dropping rules.

- For DG-PUSCH, when AvailableSlotCounting is enables, and for K>1, Option 2 is preferred.

  - Option 2: A UE assumes that the slot indicated by K2 offset can be the slot which is not counted in K available slot(s).

- For CG-PUSCH, when AvailableSlotCounting is enables, and for K=1, Option 2-2 is preferred.

  - Option 2-2: When overlapping, the UE does not perform the PUSCH transmission in the slot determined in 38.321 Section 5.8.2 and performs the PUSCH transmission in the next available slot subject to PUSCH dropping rules.

- For CG-PUSCH, when AvailableSlotCounting is enables, and for K>1, Option 2 is preferred.

  - Option 2: A UE assumes that the slot determined in 38.321 Section 5.8.2 can be the slot which is not counted in K available slot(s).

Proposal 3:

Based on current agreements, for both DG-PUSCH and CG-PUSCH, when AvailableSlotCounting is enabled, the slot for DG/CG PUSCH repetition Type A with K=1 is determined by available slots.

Proposal 4:

DG-PUSCH and CG-PUSCH should adopt a unified determining rule for the available slot counting for PUSCH repetition type A with K=1.

Proposal 5:

Current conclusions cover the case that the slot indicated by K2 is not an available slot while the repetition factor equals 1. No additional modification to the specification is needed.

Proposal 6:

The available slot counting should not apply to DG-PUSCH scheduled by DCI format 0_0 with RNTI than TC-RNTI even when AvailableSlotCounting is enabled.

Proposal 7:

The TP without insertion of “repetition Type A” provide correct information that DCI format 0_0 cannot schedule PUSCH repetitions is preferred.

Proposal 8:

It should be clarified the following issues,

- Whether the PUSCH repetition type A covers both PUSCH with repetitions and DCI format 0_0 scheduled single slot PUSCH

- Whether the frequencyHopping is applied to DCI format 0_0.

References

- [1] Chairman’s Notes for 3GPP TSG RAN WG1 #107-e, e-Meeting, November 11th – 19th, 2021