3GPP TSG RAN WG1 #108-e			                                  R1-2201871

e-Meeting, February 21th – March 3rd, 2022

Agenda item:	8.8.2

Source: 	CMCC

Title: 	Remaining issues on PUCCH enhancements

Document for:	Discussion and Decision

Introduction

In the RAN1 #107bis-e meeting, the PUCCH enhancements was discussed focusing on the dynamic PUCCH repetition factor indication and DMRS bundling across PUCCH repetition [1]. The agreements will be listed within the corresponding sections.

In this contribution, we provide our views on the PUCCH enhancements.

Discussion

2.1 Applicability of dynamic PUCCH repetition factor indication

In the last meeting, the following agreements have been reached

Agreement Support dynamic PUCCH repetition factor indication for all PUCCH formats including format 0, 1, 2, 3, 4 with a unified mechanism as agreed in RAN1#106e under agenda 8.8.2. Note: it does not impact the discussion of slot level or sub-slot level repetition

Dynamic PUCCH repetition is applied to PUCCH format 0 and 2 based on the agreements made under IIot/URLLC WI. Currently the repetition is slot-based, and sub-slot-based repetition is in FFS. The fact that we worry about is whether or not a CE UE has to support sub-slot-based repetition which is not necessary for PUCCH format 1, 3 and 4 repetition. The motivation to mix the functionality of URLLC and Coverage Enhancement is to reuse the same mechanism and reduce the workload, but the details and the differences between the slot-based and sub-slot-based repetition may still need more discussion.

Proposal 1:

The details and the differences between the slot-based and sub-slot-based repetition may still need more discussion.

2.2 DMRS bundling across PUCCH repetitions

The DMRS bundling of PUCCH DMRS is similar with the joint channel estimation of PUSCH. The coverage could be improved due to the enhancements of the accuracy of channel estimations. The PUCCH repetition under the consecutive slots could be benefit from the DMRS bundling. The PUCCH repetition number could be reduced due to the improvement of channel estimation. Similar as joint channel estimation, the DMRS bundling should consider the power consistency, phase consistency, and the use of time-frequency resources. Since the frequency hopping is usually configured for the PUCCH transmission. The combination of frequency hopping and DMRS bundling should also discussed.

In the last meeting, the following agreements have been reached.

Agreement For the interaction between inter-slot frequency hopping and DMRS bundling for PUCCH/PUSCH repetitions, a UE performs the “hopping intervals determination”, “configured TDW determination”, and “actual TDW determination” in a sequential ordering, based on the following option 1. - Option 1: “hopping intervals determination” -> “configured TDW determination” -> “actual TDW determination” - DMRS bundling shall be restarted at the beginning of each frequency hop - DMRS bunding is per actual TDW - FFS: Frequency hopping pattern is determined by physical slot indices. - FFS: different FH pattern determination for PUCCH and PUSCH - FFS: details of FH pattern design - Support separate RRC configuration(s) for hopping interval and configured TDW length. - if hopping interval is not configured, the default hopping interval is the same as the configured TDW length - FFS: if both hopping interval and TDW length are not configured - Note: hopping interval is only determined by the configuration of hopping interval if hopping interval is configured

The legacy mechanism uses the parity of the physical slot to indicate how the UE perform UL transmission with FH. From our point of view, using physical slot makes it easier to match with the TDD slot duration.

Proposal 2:

Frequency hopping pattern should be determined by physical slot indices.

- 2.3 Value range for frequencyhopping-interval

In the last meeting, we discussed the following table for the RRC parameters, and the value range for frequencyhopping-interval is still FFS.

| WI code | Sub-feature group | Parameter name in the spec | New or existing? | Description | Value range | Default value aspect | Per (UE, cell, TRP, …) | UE-specific or Cell-specific |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR_cov_enh-Core | PUCCH enhancements | PUCCH-nrofSlots-r17 | new | A new repetition parameter corresponding to Rel-17 dynamic PUCCH repetition factor indication. The new repetition parameter is configured per PUCCH resource and should be in PUCCH-Resource. Note: a PUCCH resource not configured with PUCCH-nrofSlots-r17 can attain the value of 1 according to when the Rel-15/16 parameter nrofSlots is not configured. | ENUMERATED {2, 4, 8} |  | In PUCCH-resource | UE-specific |
| NR_cov_enh-Core | DM-RS bundling for PUCCH | PUCCH-DMRS-Bundling | new | Enabling/disabling of DM-RS bundling and time domain window for PUCCH repetitions. | ENUMERATED {enabled, disable } |  | in PUCCH-Config Note: The RRC parameter “PUCCH-DMRS-Bundling” is per UL BWP. PUCCH DMRS Bundling is not supported for PUCCH format 0/2 | UE-specific |
| NR_cov_enh-Core | DM-RS bundling for PUCCH | PUCCH-TimeDomainWindowLength | new | Length of a [nominal] time domain window in slots for DMRS bundling for PUCCH. | INTEGER (2..[8]) Note: the value shall not exceed the maximum duration |  | in PUCCH-Config Note: The RRC parameter “PUCCH-TimeDomainWindowLength” is per UL BWP. PUCCH DMRS Bundling is not supported for PUCCH format 0/2 | UE-specific |
| NR_cov_enh-Core | DM-RS bundling for PUCCH | PUCCH-Window-Restart | new | UE bundles PUCCH DM-RS slots remaining in a bundling nominal time domain window after a slot for which events violate power consistency and phase continuity requirements UE bundles PUCCH DM-RS remaining in a nominal time domain window after dynamic event(s) triggered by DCI or MAC-CE that violate power consistency and phase continuity requirements Note: Events, which are triggered by DCI or MAC CE, but regarded as semi-static events, e.g. frequency hopping, UL beam switching for multi-TRP operation, or other if defined, are excluded. | ENUMERATED {enabled, disable } |  | in PUCCH-Config | UE-specific |
| NR_cov_enh-Core | DM-RS bundling for PUCCH | PUCCH-Frequencyhopping-Interval | New | Number of consecutive slots for UE to perform Rel-17 inter-slot frequency hopping with inter-slot bundling for PUCCH Note: When DMRS bundling for PUCCH is enabled by PUCCH-DMRS-Bundling, PUCCH frequency hopping interval is only determined by the configuration of PUCCH hopping interval if PUCCH hopping interval is configured. | FFS | PUCCH-TimeDomainWindowLength | in PUCCH-Config | UE-specific |
| NR_cov_enh-Core | DM-RS bundling for PUSCH | PUSCH-Frequencyhopping-Interval | New | Number of consecutive slots for UE to perform Rel-17 inter-slot frequency hopping with inter-slot bundling for PUSCH. Note 1: This RRC parameter is shared for both DG-PUSCH and CG-PUSCH Note 2: When DMRS bundling for PUSCH is enabled by PUSCH-DMRS-Bundling, PUSCH frequency hopping interval is only determined by the configuration of PUSCH hopping interval if PUSCH hopping interval is configured. | FFS | PUSCH-TimeDomainWindowLength | in PUSCH-Config | UE-specific |

The numbers chosen for Frequency hopping interval are based on the fraction of the repetition factor. The value range for repetition of PUCCH is {2, 4, 8}. If the value of FH-interval can be {1}, then the UE would do FH every slot though the PUCCH repetitions, which means DMRS bundling is disabled. The same scenario also happens when the value of FH-interval is {8}, which means UE would transmit PUCCH repetition without FH, in other word, the FH is disabled. For us, the implicitly indication to disable FH/DMRS-bundling does not harm. Both options below are acceptable to us,

Option 1 :Value range for PUCCH-Frequencyhopping-Interval is set to “{2, 4,}”
Option 2 :Value range for PUCCH-Frequencyhopping-Interval is set to “{1, 2, 4, 8}”

Considering without redundant function, such as configuring FH/DMRS bundling but without enabling them, more straight forward one (option 2) is preferred.

Proposal 3：

The value range for PUCCH-Frequencyhopping-Interval is set to “{2, 4,}” or “{1, 2, 4, 8}”, the second one is preferred.

2.4 TPC command and TA command

The power control and TA command was fully discussed in the AI 8.8.1.3. The TA command was concluded as an event. And one working assumption was made for TPC command. For the DMRS bundling of PUCCH, the conclusion from AI 8.8.1.3 could be fully reused.

Proposal 4：

The conclusion of PUSCH JCE for the TPC command and TA updates could be reused for DMRS bundling of PUCCH repetitions,

Conclusions

In this contribution, we provide our views on the PUCCH enhancements. The proposals are as below.

Proposal 1:

The details and the differences between the slot-based and sub-slot-based repetition may still need more discussion.

Proposal 2:

Frequency hopping pattern should be determined by physical slot indices.

Proposal 3：

The value range for PUCCH-Frequencyhopping-Interval is set to “{2, 4,}” or  “{1, 2, 4, 8}”, the second one is preferred.

Proposal 4：

The conclusion of PUSCH JCE for the TPC command and TA updates could be reused for DMRS bundling of PUCCH repetitions,

References

- [1] Draft Report of 3GPP TSG RAN WG1 #107bis-e, e-Meeting, 17th January - 25th January 2022