3GPP TSG RAN WG1 #108-e																																																		  R1-2201880

e-Meeting, February 21st – March 3rd, 2022

Source:	CMCC

Title:	Remaining issues on enhancements to time and frequency synchronization for IoT NTN

Agenda Item:	8.14.1

Document for:	Discussion and Decision

Introduction

In RAN1#107-e meeting, agreements were achieved on time and frequency synchronization for IoT NTN [1].

GNSS validity: Agreement The UE autonomously determines its GNSS validity duration X and reports information associated with this valid duration to the network via RRC signalling. - X = {10s, 20s, 30s, 40s, 50s, 60s, 5 min, 10 min, 15 min, 20 min, 25 min, 30 min, 60 min, 90 min, 120 min, infinity} Agreement Send LS to RAN2 to take the following RAN1 agreements into consideration to specify the aspects related to GNSS position validity: - For sporadic short transmission, UE in RRC_CONNECTED should go back to idle mode and re-acquire a GNSS position fix if GNSS becomes outdated - The UE autonomously determines its GNSS validity duration X and reports information associated with this valid duration to the network via RRC signalling. - X = {10s, 20s, 30s, 40s, 50s, 60s, 5 min, 10 min, 15 min, 20 min, 25 min, 30 min, 60 min, 90 min, 120 min, infinity} - Note: The duration of the short transmission is not longer than the “validity timer for UL synchronization” referred to in the WID objective (but which still needs further discussion for specifying further details) R1-2112847 –DRAFT LS to RAN2 on GNSS validity duration for IoT NTN - Final LS in R1-2112848 Validity timer for UL Synchronization: Agreement The serving satellite ephemeris and common TA related parameters are signalled in the same SIB message and have the same epoch time. Agreement A single validity duration for both serving satellite ephemeris and common TA related parameters is broadcast on the SIB. Agreement Validity timer for UL synchronization should be started/restarted with configured timer validity duration at the epoch time of the assistance information. Agreement Validity timer duration is configured per cell and indicated to the UE in X bits with: · Value range {5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 55, 60, 120, 180, 240} · Unit is second · FFS Additional values for GEO For NPUSCH for NB-IoT and PUSCH/PUCCH for eMTC: Agreement For eMTC PUCCH/PUSCH with frequency hopping enabled, the UE can adjust the uplink transmit timing when hopping to a new narrowband if the frequency hopping interval is less than or equal to the configured transmission segment duration. Agreement For eMTC PUCCH, a 3-bit field to indicate K=7 values for the uplink transmission segment duration: - 2 4 8 16 32 64 128 subframes Agreement For eMTC PUCCH/PUSCH with frequency hopping enabled, the UE can adjust the uplink transmit timing and transmit frequency when hopping to a new narrowband if the frequency hopping interval is less than or equal to the configured transmission segment duration. Agreement UE pre-compensation per segment of NPUSCH for NB-IoT and PUSCH/PUCCH for eMTC is applied from one segment to the next segment by using one or more of the following methods if supported by UE implementation 1. UE may drop / Insert samples / Puncture OFDM symbols 2. UE may blank subframes / slots where UE skip a slot or a subframe The total transmission time is not changed UE autonomously Drop / insert samples / Puncture OFDM symbols or Blank subframes / slots where UE drops a subframe / slot The method used for the UE pre-compensation is known to the eNB by a single UE capability - UE Blank subframes / slots where UE skip a slot or a subframe (slot is based on Sub Carrier Spacing) FFS Details of method(s) to drop / insert samples, blanking subframes / slots (slot is based on Sub Carrier Spacing) For NPRACH for NB-IoT and PRACH for eMTC: Agreement For NB-IoT, UE pre-compensation per segment of NPRACH is applied from one segment to the next segment by using one or more of the following methods if supported by UE implementation - UE may drop / Insert samples - UE may blank subframe / repetition unit where UE drops a subframe / repetition unit The total transmission time is not changed FFS Details of method(s) to drop / insert samples / blank subframe / repetition unit FFS Specification impact Agreement For eMTC, UE pre-compensation per segment of PRACH is applied from one segment to the next segment by drop / insert samples in Guard Period of PRACH preamble. - The total transmission time is not changed - FFS Details of method(s) to drop / insert samples UL segmented transmission configuration: Agreement UL transmission segment duration with one value X per NPUSCH for NB-IoT and PUSCH/PUCCH for eMTC may be indicated on SIB. - For NB-IoT/eMTC, X is one of K candidate values for the UL transmission segment duration of NPUSCH/PUSCH/PUCCH - The value X for eMTC PUSCH applies for full-PRB allocation and should be divided by 2, 4 and 8 for sub-PRB allocation of 6, 3 and 2-out-of-3 tones allocation, respectively. Agreement At least UL transmission segment duration with one value X for NPRACH for NB-IoT and PRACH for eMTC may be indicated on SIB - For NB-IoT/eMTC, X is one of K candidate values for the UL transmission segment duration of NPRACH/PRACH - FFS One value X, one or more values Xi Agreement UL Segmented transmission NPRACH/NPUSCH for NB-IoT is not supported in GEO based on UE feature . Agreement For NB-IoT NTN, the network configures one of K values for the UL transmission segment duration of each PRACH preamble format in a k-bit field, where the size of the k-bit field and the number of K candidate values depend on the preamble format. - Format 0 and format 1: 3-bit field, K=6 candidate values 2.4.(TCP+TSEQ), 4.4.(TCP+TSEQ), 8.4.(TCP+TSEQ), 16.4.(TCP+TSEQ), 32.4.(TCP+TSEQ), 64.4.(TCP+TSEQ) - Format 2: 3-bit field, K=5 candidate values 1.6.(TCP+TSEQ), 2.6.(TCP+TSEQ), 4.6.(TCP+TSEQ), 8.6.(TCP+TSEQ), 16.6.(TCP+TSEQ) Agreement Support network re-configuration of UL transmission segment by dedicated RRC Signalling DL Synchronization enhancements: Agreement For DL synchronization enhancements: - Signal Part-of ARFCN indication on MIB for bands where RAN4 cannot introduce a 200 kHz channel raster and the legacy 100 kHz raster is used, otherwise for bands where RAN4 can introduce a 200 kHz channel raster there is no signalling of the part-of ARFCN indication on MIB. Agreement For IoT NTN, indicate two LSBs of the ARFCN in the MIB. R1-2112689 –DRAFT LS to RAN4 on DL synchronization enhancements for IoT NTN - Final LS in R1-2112768 Synchronization aspects common to IoT NTN and NR NTN: Agreement The following agreements from NR NTN are re-used for IoT NTN The granularity of Common TA is set to be 1.Ts Conclusion The following conclusion from NR NTN is re-used for IoT NTN Conclusion: Do not define a TA margin. Working assumption: Higher-layer parameters TACommon, TACommonDrift, TACommonDriftVariation are indicated with the following range, granularity and bits allocation: Parameter name; Value range; Granularity; Bits allocation. TACommon; 0 ...8316827 (i.e: 0… 270.73 ms); 32.55208 ×10-3 μs; 23 bits. TACommonDrift; - 261935… + 261935 (i.e: -53.33 μs/s… +53.33 μs/s); 0.2×10-3μs/s; 19 bits. TACommonDriftVariation; 0…29470 (0…0.60 μs/s2); 0.2×10-4μs/s2; 15 bits. - Value ranges are given in unit of corresponding granularity. Agreement Confirm the working assumption made at RAN1#106-bis-e on serving satellite ephemeris bit allocations for LEO/MEO/GEO based non-terrestrial access network: - Support serving satellite ephemeris format bit allocations for LEO/MEO/GEO based non-terrestrial access network: - Position and velocity state vector ephemeris format is 17 bytes payload. - The field size for position (m) is 78 bits - Position range is driven by GEO : +/- 42 200 km - The quantization step is 1.3m for position - The field size for velocity (m/s) is 54 bits - Velocity range is driven by LEO@600 km: +/- 8000 m/s - The quantization step is 0.06 m/s for Velocity - Orbital parameter ephemeris format 18 byte payload - Semi-major axis α (m) is 33 bits - Range: [6500, 43000]km - Eccentricity e is 19 bits - Range: ≤ 0.015 - Argument of periapsis ω (rad) is 24 bits - Range: [0, 2π] - Longitude of ascending node (Ω rad) is 21 bits - Range: [0, 2π] - Inclination i (rad) is 20 bits - Range: [- π/2 , + π/2] - Mean anomaly M (rad) at epoch time to is 24 bits - Range: [0, 2π] Agreement Using indicated Higher-layer Common TA parameters, if configured, the UE can determine the one-way propagation time ( Delaycommon) used for NTA, common calculation as follows: Delaycommont= DCommon tepocℎ+ DCommonDrift× t−tepocℎ+DCommonDriftVariation× t−tepocℎ2 Where: - DCommon = TACommon 2, DCommonDrift=TACommonDrift2 and DCommonDriftVariation=TACommonDriftVariation2 - TACommon, TACommonDrift and TACommonDriftVariation are Common TA parameter defined in RAN1 Meeting #106-bis-e - Delaycommont is the distance between the satellite and the uplink time synchronization reference point divided by the speed of light. DL and UL are frame aligned at the reference point with an offset given by NTA,offset. - NTA, common is derived by the UE based on Delaycommont to pre-compensate the two-way transmission delay between the uplink time reference point and the satellite.

In this contribution, we will discuss some time and frequency synchronization related aspects for IoT over NTN.

Discussion

GNSS Measurements

In RRC_IDLE

In RAN1#107-e meeting, there was a discussion on GNSS Measurements in RRC_IDLE, but no consensus was achieved. Moderator suggested for more discussion on the following proposed conclusion [2].

Conclusion - Acquisition of GNSS position fix during paging procedure is up to UE implementation and network configuration of paging timers considering GNSS measurement duration (e.g. GNSS Time To First Fix with cold start of typically 10 seconds) impact in NTN scenario. These paging timers are not specified in 3GPP in legacy paging procedure (i.e. T3413 / T3415).

For sporadic DL traffic, UE may perform GNSS measurements after a paging occasion and only if it has been paged to reduce battery consumption. GNSS measurement duration can be up to 10 seconds, which implies that after MME requests the lower layer to start paging, it may receive paging response after a long time (e.g., 10 seconds).

Figure 1: Paging procedure using S-TMSI.

As shown in Figure 1, in current specification (TS 24.301), to initiate high level paging procedure initiated by the MME, the EMM (EPS Mobility Managed) entity in the network requests the lower layer to start paging and shall start the timer:

- T3415 for this paging procedure, if the network accepted to use eDRX for the UE and the UE does not have a PDN connection for emergency bearer services.

- Otherwise, T3413 for this paging procedure.

T3413/T3415 is a supervision timer for the paging procedure. The MME can re-attempt the paging procedure if T3413/T3415 expires before a response is received.

Note that the expiry time of T3413/T3415 is implementation dependent and is not specified in 3GPP, network operator may configure expiry time of T3413/T3415 considering GNSS measurement duration (e.g., 10 seconds) impact in NTN scenario. Thus, for sporadic DL traffic, the existing timers (e.g., T3413/T3415) can be configured large enough to ensure a sufficient gap to accommodate GNSS acquisition after decoding the paging message and before initiating UL transmission.

Based on the above discussion, we agree with Moderator’s conclusion made in the last meeting.

Observation 1: For sporadic DL traffic, UE may perform GNSS measurements after a paging occasion and only if it has been paged to reduce battery consumption. The existing timers (e.g., T3413/T3415) can be configured large enough to ensure a sufficient gap to accommodate GNSS acquisition after decoding the paging message and before initiating UL transmission.

Proposal 1: Support the following conclusion.

- Acquisition of GNSS position fix during paging procedure is up to UE implementation and network configuration of paging timers considering GNSS measurement duration (e.g. GNSS Time To First Fix with cold start of typically 10 seconds) impact in NTN scenario. These paging timers are not specified in 3GPP in legacy paging procedure (i.e. T3413 / T3415).

In RRC_CONNECTED

In RAN1#107-e meeting, there was a discussion on GNSS Measurements in RRC_CONNECTED, and the following agreements were achieved.

Agreement The UE autonomously determines its GNSS validity duration X and reports information associated with this valid duration to the network via RRC signalling. - X = {10s, 20s, 30s, 40s, 50s, 60s, 5 min, 10 min, 15 min, 20 min, 25 min, 30 min, 60 min, 90 min, 120 min, infinity} Agreement Send LS to RAN2 to take the following RAN1 agreements into consideration to specify the aspects related to GNSS position validity: - For sporadic short transmission, UE in RRC_CONNECTED should go back to idle mode and re-acquire a GNSS position fix if GNSS becomes outdated - The UE autonomously determines its GNSS validity duration X and reports information associated with this valid duration to the network via RRC signalling. - X = {10s, 20s, 30s, 40s, 50s, 60s, 5 min, 10 min, 15 min, 20 min, 25 min, 30 min, 60 min, 90 min, 120 min, infinity} - Note: The duration of the short transmission is not longer than the “validity timer for UL synchronization” referred to in the WID objective (but which still needs further discussion for specifying further details)

In typical case, the network should ensure UL transmission to be terminated before GNSS position fix outdated. The gNB may estimate GNSS position fix validity duration based on conservative estimation, e.g., 10s, or based on UE’s reported on the rest validity duration of GNSS position fix.

Report of the rest GNSS position fix validity duration can be considered as an enhancement functionality for UE is medium or high velocity. For sporadic short transmission, the report may be triggered by the network before UL transmission is scheduled.

Proposal 2: UE reports GNSS position fix validity duration to be used by network to move UE to RRC_IDLE can be considered as an enhancement functionality.

- The rest GNSS position fix validity duration after the reporting may be reported.

- The report may be triggered by the network before UL transmission is scheduled.

Validity timer for UL synchronization

Update of assistance information

In RAN1#107-e meeting, agreements were achieved on the epoch time of assistance information [1]:

Agreement The serving satellite ephemeris and common TA related parameters are signalled in the same SIB message and have the same epoch time. Agreement A single validity duration for both serving satellite ephemeris and common TA related parameters is broadcast on the SIB. Agreement Validity timer for UL synchronization should be started/restarted with configured timer validity duration at the epoch time of the assistance information.

As discussed in our company’s contribution in NTN [3], it can be observed that the update period (e.g., 160ms) and the validity duration (e.g., 5~240s) for the assistance information are much smaller than SI modification period (e.g., 1~3 hours). To enable quick initial access procedure, the SIB carrying assistance information needs to be sent frequently, which means the assistance information needs to be sent multiple times in one SFN period.

If the validity duration is greater than or equal to one SFN period, according to the definition of the epoch time of assistance information (it is associated with a SFN or with a SI window), different assistance information is associated with different time, thus the value in each assistance information should be varying.

According to the current specification, SIB changes will trigger system information modification procedure, then as the assistance information is varying at least in each SFN period, all UEs need to read new system information frequently, which may contradict the original intention of configuring a larger validity duration for the assistance information. Therefore, the following proposal is proposed.

Proposal 3: Update of assistance information in SIB will not trigger system information modification procedure.

- It is up to RAN2 to determine detailed solutions for updating the assistance information. (e.g., changes of the assistance information should neither result in system information change notifications nor in a modification of systemInfoValueTag in SIB1, just like “timeInfoUTC” field acts in SIB16.)

Validity timer duration

In RAN1#107-e meeting, the following agreement was achieved on IoT NTN validity timer [1]:

Agreement Validity timer duration is configured per cell and indicated to the UE in X bits with: · Value range {5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 55, 60, 120, 180, 240} · Unit is second · FFS Additional values for GEO

Similar to the discussion in case of NR NTN, larger values in IoT NTN validity timer duration range for the case of GEO are also needed.

According to the specification description for satellite mobility in GEO Scenarios,

From section 5.3.2 of TS 38.811: In principle, the Geostationary satellite is fixed and therefore no Doppler shift is induced except that due to possible UE motion. In reality the satellite is moving around its nominal orbital position, due to perturbations (e.g. sun, moon) and to non-spherical component earth attraction which impact the earth gravitational force. The average tangential velocity of a geostationary satellite with respect to an earth point is around 2.74 m/s.

Considering Geostationary satellites move slowly, larger values than 240 seconds may be needed for GEO.

Proposal 4: For NTN validity duration configuration, larger values than 240 seconds are needed for GEO scenario.

It can be observed that even the transmission equipment is usually considered quasi static with respect to the UE with only small Doppler shift, the GEO satellite is moving relatively slowly. Therefore, “infinity” may not be proper for GEO validity duration.

Proposal 5: “Infinity” is not needed in the NTN validity duration value range for the case of GEO.

Synchronization aspects common to IoT NTN and NR NTN

Common TA

In RAN1#107-e meeting, the following Working assumption was made.[1].

Working assumption: Higher-layer parameters TACommon, TACommonDrift, TACommonDriftVariation are indicated with the following range, granularity and bits allocation: Parameter name; Value range; Granularity; Bits allocation. TACommon; 0 ...8316827 (i.e: 0… 270.73 ms); 32.55208 ×10-3 μs; 23 bits. TACommonDrift; - 261935… + 261935 (i.e: -53.33 μs/s… +53.33 μs/s); 0.2×10-3μs/s; 19 bits. TACommonDriftVariation; 0…29470 (0…0.60 μs/s2); 0.2×10-4μs/s2; 15 bits. - Value ranges are given in unit of corresponding granularity.

The indication of higher-layer parameters TACommon, TACommonDrift, TACommonDriftVariation for IoT NTN can re-use the conclusion of NTN.

Proposal 6: Confirm the above working assumption. Higher-layer parameters TACommon, TACommonDrift, TACommonDriftVariation are indicated with the following range, granularity and bits allocation:

| Parameter name | Value range | Granularity | Bits allocation |
| --- | --- | --- | --- |
| TACommon | 0 ...8316827 (i.e: 0… 270.73 ms) | 32.55208 ×10-3 μs | 23 bits |
| TACommonDrift | - 261935… + 261935 (i.e: -53.33 μs/s… +53.33 μs/s) | 0.2×10-3μs/s | 19 bits |
| TACommonDriftVariation | 0…29470 (0…0.60 μs/s2) | 0.2×10-4μs/s2 | 15 bits |
| - Value ranges are given in unit of corresponding granularity |  |  |  |

Conclusions

In this contribution, we share our views on related issues on UL time and frequency synchronization enhancements for IoT over NTN. The observations and proposals are summarized as follows:

Observation 1: For sporadic DL traffic, UE may perform GNSS measurements after a paging occasion and only if it has been paged to reduce battery consumption. The existing timers (e.g., T3413/T3415) can be configured large enough to ensure a sufficient gap to accommodate GNSS acquisition after decoding the paging message and before initiating UL transmission.

Proposal 1: Support the following conclusion.

- Acquisition of GNSS position fix during paging procedure is up to UE implementation and network configuration of paging timers considering GNSS measurement duration (e.g. GNSS Time To First Fix with cold start of typically 10 seconds) impact in NTN scenario. These paging timers are not specified in 3GPP in legacy paging procedure (i.e. T3413 / T3415).

Proposal 2: UE reports GNSS position fix validity duration to be used by network to move UE to RRC_IDLE can be considered as an enhancement functionality.

- The rest GNSS position fix validity duration after the reporting may be reported.

- The report may be triggered by the network before UL transmission is scheduled.

Proposal 3: Update of assistance information in SIB will not trigger system information modification procedure.

- It is up to RAN2 to determine detailed solutions for updating the assistance information. (e.g., changes of the assistance information should neither result in system information change notifications nor in a modification of systemInfoValueTag in SIB1, just like “timeInfoUTC” field acts in SIB16.)

Proposal 4: For NTN validity duration configuration, larger values than 240 seconds are needed for GEO scenario.

Proposal 5: “Infinity” is not needed in the NTN validity duration value range for the case of GEO.

Proposal 6: Confirm the above working assumption. Higher-layer parameters TACommon, TACommonDrift, TACommonDriftVariation are indicated with the following range, granularity and bits allocation:

| Parameter name | Value range | Granularity | Bits allocation |
| --- | --- | --- | --- |
| TACommon | 0 ...8316827 (i.e: 0… 270.73 ms) | 32.55208 ×10-3 μs | 23 bits |
| TACommonDrift | - 261935… + 261935 (i.e: -53.33 μs/s… +53.33 μs/s) | 0.2×10-3μs/s | 19 bits |
| TACommonDriftVariation | 0…29470 (0…0.60 μs/s2) | 0.2×10-4μs/s2 | 15 bits |
| - Value ranges are given in unit of corresponding granularity |  |  |  |

References

- Chairman's Notes RAN1#107-e, November 11th – 19th, 2021.

- R1-2112803, “Summary #4 of AI 8.15.1 	Enhancements to time and frequency synchronization”, November 11th – 19th, 2021.

- R1-2201853, “Remaining issues on enhancements on UL time and frequency synchronization for NTN”, e-Meeting, February 21st – March 3rd, 2022.