3GPP TSG RAN WG1 #108-e										 		  R1-2201897

e-Meeting, February 21st – March 3rd, 2022

Agenda item:	8.1.2.1

Source:	NEC

Title:	Discussion on remaining issues on multi-TRP for PUCCH and PUSCH

Document for:	Discussion and Decision

- Introduction

Rel-17 enhancements for PDCCH, PUCCH and PUSCH based on multi-TRP have been finished, while there are some remaining issues. In this contribution, we provided our views on some remaining issues for PUCCH and PUSCH.

- Discussion

PUCCH

In Rel-17, both inter-slot and intra-slot PUCCH repetition were agreed. And for intra-slot repetition, it was agreed in RAN1#105-e meeting:

Agreement Confirm the working assumption with removing brackets on [consecutive] and adding UE capability. - For PUCCH reliability enhancement, support multi-TRP intra-slot repetition (Scheme 3) for all PUCCH formats. - The same PUCCH resource carrying UCI is repeated for X = 2 [consecutive] sub-slots within a slot. - Refer the design details related to sub-slot configurations (e.g. other values of X) to Rel-17 eIIoT - Note1: The decision of supporting scheme 3 is only applicable for multi-TRP operation. - This feature is optional.

There is one remaining issue to be clarified, i.e whether other values of X (except 2) are supported for intra-slot PUCCH repetition, we provides our views as follows:

- No support of other values of X (except 2) for intra-slot PUCCH repetition based on multi-TRP

The agreement is a little ambiguous on supporting other values of X, only saying “(e.g. other values of X)”, in our understanding, there is no detailed discussion on this in MIMO session, regarding the maintenance phase with limited TU, we prefer to support only X=2 for intra-slot PUCCH repetition based on multi-TRP.

And there is no agreement to support combination of intra-slot and inter-slot PUCCH repetition in case of multi-TRP, in other words, if the PUCCH resource is configured with two spatial settings or two sets of power control parameters, and if the UE is configured with subslotLengthForPUCCH, then intra-slot PUCCH repetition (scheme 3) is applied, and the PUCCH is repeated in 2 consecutive subslots, in this case, the value of NPUCCHrepeat should be ignored or UE just does not expect to be configured with NPUCCHrepeat or with value NPUCCHrepeat>1 in case of intra-slot PUCCH repetition based on multi-TRP. So we propose:

Proposal 1: In case of intra-slot PUCCH repetition based on multi-TRP, support PUCCH repetition with only X = 2 consecutive sub-slots within a slot. And when a PUCCH is configured with two spatial settings or two sets of power control parameters, if UE is configured with subslotLengthForPUCCH, intra-slot PUCCH repetition is applied, and the UE ignores value of NPUCCHrepeat. We propose a following TP to reflect this in TS:

TS38.213 - 9.2.6 PUCCH repetition procedure < Unchanged parts are omitted > When a PUCCH resource used for repetitions of a PUCCH transmission by a UE includes first and second spatial settings, or first and second sets of power control parameters, as described in [11, TS 38.321] and in clause 7.2.1, and if the UE is provided subslotLengthForPUCCH, the UE transmits first and second repetitions of the PUCCH transmission with the UCI consecutively within a slot as described in [4, TS 38.211], and the UE ignores value of NPUCCHrepeat if configured, and a number of symbols for each repetition of PUCCH transmission is indicated by subslotLengthForPUCCH, and the UE uses the first and second spatial settings, or the first and second sets of power control parameters, for first and second repetitions of the PUCCH transmission, respectively; otherwise, the UE - uses the first and second spatial settings, or the first and second sets of power control parameters, for first and second repetitions of the PUCCH transmission, respectively, when NPUCCHrepeat=2, - alternates between the first and second spatial settings, or between the first and second sets of power control parameters, respectively, per NPUCCHswitch repetitions of the PUCCH transmission, where NPUCCHswitch=1 if mappingPattern = 'cyclicMapping'; else, NPUCCHswitch=2. < Unchanged parts are omitted >

- Support of other values of X (e.g. 4 or 8) for intra-slot PUCCH repetition based on multi-TRP

If companies support other values of X, we think just simply referring to sub-slot configuration in Rel-17 IIoT for MIMO seems not suitable. What we discussed is intra-slot repetition, and we don’t have agreement to support combination of intra-slot and inter-slot repetition. And in Rel-17 IIoT discussion, sub-slot length can be 2 or 7 symbols for normal CP, and repetition number can be 2, 4, or 8. While in MIMO discussion, if we support intra-slot repetition, only some repetition number can be supported, shown as follows:

| Sub-slot length (number of symbols) | Repetition number NPUCCHrepeat | IIoT | Intra-slot PUCCH repetition in MIMO |
| --- | --- | --- | --- |
| 2 | 2 | Supported | Supported |
| 7 | 2 | Supported | Supported |
| 2 | 4 | Supported | Can be supported |
| 2 | 8 | Supported | Not supported. This will exceed one slot, i.e. it’s not intra-slot repetition. |
| 7 | 4 | Supported | Not supported. This will exceed one slot, i.e. it’s not intra-slot repetition. |
| 7 | 8 | Supported | Not supported. This will exceed one slot, i.e. it’s not intra-slot repetition. |

Based on above discussion, combination of sub-slot length and repetition number to be {2*8}, {7*4}, {7*8} will exceed boundary of a slot, which is not intra-slot repetition discussed in MIMO. If companies want to support these combinations, it will need more detailed discussion, which we think can not be finished in the limited remaining time. So if other values of X to be supported, we prefer to support only X=4 when sub-slot length is 2.

Proposal 2: In case of intra-slot PUCCH repetition based on multi-TRP, if other values of X to be supported, we prefer only X=4 when sub-slot length is configured to be 2. And we propose an alternative TP to reflect this in TS:

TS38.213 v17.0.0 - 9.2.6 PUCCH repetition procedure A UE can be indicated to transmit a PUCCH over NPUCCHrepeat slots using a PUCCH resource, where - if the PUCCH resource is indicated by a DCI format and includes PUCCH-nrofSlots, NPUCCHrepeat is provided by PUCCH-nrofSlots - otherwise, NPUCCHrepeat is provided by nrofSlots < Unchanged parts are omitted > When a PUCCH resource used for repetitions of a PUCCH transmission by a UE includes first and second spatial settings, or first and second sets of power control parameters, as described in [11, TS 38.321] and in clause 7.2.1, the UE - uses the first and second spatial settings, or the first and second sets of power control parameters, for first and second repetitions of the PUCCH transmission, respectively, when NPUCCHrepeat=2, - alternates between the first and second spatial settings, or between the first and second sets of power control parameters, respectively, per NPUCCHswitch repetitions of the PUCCH transmission, where NPUCCHswitch=1 if mappingPattern = 'cyclicMapping'; else, NPUCCHswitch=2, - if the UE is provided subslotLengthForPUCCH with value 7 for normal cyclic prefix or value 6 for extended cyclic prefix, NPUCCHrepeat= 2 regardless the value of PUCCH-nrofSlots or nrofSlots if configured, and if the UE is provided subslotLengthForPUCCH with value 2, the UE is expected to be configured with value NPUCCHrepeat to be 2 or 4, and the NPUCCHrepeat repetitions of the PUCCH transmission are consecutive within a slot as described in [4, TS 38.211]. < Unchanged parts are omitted >

PUSCH

In RAN1#104-e meeting, following agreement for PUSCH enhancements was achieve:

Agreement For single DCI based M-TRP PUSCH repetition schemes, support non-codebook based PUSCH transmission with following considerations. - Increase the maximum number of SRS resource sets to two, and associated CSI-RS resource can be configured per SRS resource set. - FFS: Enhancements on SRI field in DCI to indicate the two beams for repetition

While this is not reflected in TS, if we don’t describe this explicitly in spec, it seems ambiguous that whether per SRS resource set can be configured with an associated CSI-RS resource, so we propose a following TP to capture this.

Proposal 3: To capture an associated CSI-RS resource configured to each one of two SRS resource sets for non-codebook based PUSCH transmission explicitly in TS 38.214.

TS 38.214 - 6.1.1.2 Non-Codebook based UL transmission For non-codebook based transmission, PUSCH can be scheduled by DCI format 0_0, DCI format 0_1, DCI format 0_2 or semi-statically configured to operate according to Clause 6.1.2.3. If this PUSCH is scheduled by DCI format 0_1, DCI format 0_2, or semi-statically configured to operate according to Clause 6.1.2.3, the UE can determine its PUSCH precoder(s) and transmission rank based on the SRI(s) when multiple SRS resources are configured, where the SRI(s) is given by one or two SRS resource indicator(s) in DCI according to clause 7.3.1.1.2 and 7.3.1.1.3 of [5, 38.212] for DCI format 0_1 and DCI format 0_2, or the SRI is given by srs-ResourceIndicator according to clause 6.1.2.3, or two SRIs given by srs-ResourceIndicator and srs-ResourceIndicator2 according to clause 6.1.2.3.. The SRS-ResourceSet(s) applicable for PUSCH scheduled by DCI format 0_1 and DCI format 0_2 are defined by the entries of the higher layer parameter srs-ResourceSetToAddModList and srs-ResourceSetToAddModListDCI-0-2 in SRS-config, respectively. The UE shall use one or multiple SRS resources for SRS transmission, where, in a SRS resource set, the maximum number of SRS resources which can be configured to the UE for simultaneous transmission in the same symbol and the maximum number of SRS resources are UE capabilities. The SRS resources transmitted simultaneously occupy the same RBs. Only one SRS port for each SRS resource is configured. Only one or two SRS resource sets can be configured in srs-ResourceSetToAddModList with higher layer parameter usage in SRS-ResourceSet set to 'nonCodebook', and only one or two SRS resource sets can be configured in srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'nonCodebook'. When two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'nonCodebook', one or two SRIs are given by the DCI fields of two SRS resource indicators in clause 7.3.1.1.2 and 7.3.1.1.3 of [5, TS 38.212] for DCI format 0_1 and 0_2. The UE applies the indicated SRI(s) to one or more PUSCH repetitions according to the associated SRS resource set of a PUSCH repetition according to clause 6.1.2.1. The maximum number of SRS resources per SRS resource set that can be configured for non-codebook based uplink transmission is 4. Each of the indicated one or two SRIs in slot n is associated with the most recent transmission of SRS resource(s) of associated SRS resource set identified by the SRI, where the SRS transmission is prior to the PDCCH carrying the SRI. When two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'nonCodebook', the UE is not expected to be configured with different number of SRS resources in the two SRS resource sets. When the PDCCH candidates are associated with a search space set configured with searchSpaceLinking, for the purpose of determining the most recent transmission of SRS resource(s) identified by the SRI, the PDCCH candidate that starts earlier in time among the two configured PDCCH candidates is used. For non-codebook based transmission, the UE can calculate the precoder used for the transmission of SRS based on measurement of an associated NZP CSI-RS resource. When two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'nonCodebook', each of the two SRS resource sets can be configured with one associated CSI-RS resource. A UE can be configured with only one NZP CSI-RS resource for each of the one or two SRS resource sets with higher layer parameter usage in SRS-ResourceSet set to 'nonCodebook' if configured. - If aperiodic SRS resource set is configured, the associated NZP-CSI-RS is indicated via SRS request field in DCI format 0_1 and 1_1, as well as DCI format 0_2 (if SRS request field is present) and DCI format 1_2 (if SRS request field is present), where AperiodicSRS-ResourceTrigger and AperiodicSRS-ResourceTriggerList (indicating the association between aperiodic SRS triggering state(s) and SRS resource sets), triggered SRS resource(s) srs-ResourceSetId, csi-RS (indicating the associated NZP-CSI-RS-ResourceId) are higher layer configured in SRS-ResourceSet. The SRS-ResourceSet(s) associated with the SRS request by DCI format 0_1 and 1_1 are defined by the entries of the higher layer parameter srs-ResourceSetToAddModList and the SRS-ResourceSet(s) associated with the SRS request by DCI format 0_2 and 1_2 are defined by the entries of the higher layer parameter srs-ResourceSetToAddModListDCI-0-2. A UE is not expected to update the SRS precoding information if the gap from the last symbol of the reception of the aperiodic NZP-CSI-RS resource and the first symbol of the aperiodic SRS transmission is less than 42 OFDM symbols. - If the UE configured with aperiodic SRS associated with aperiodic NZP CSI-RS resource, the presence of the associated CSI-RS is indicated by the SRS request field if the value of the SRS request field is not '00' as in Table 7.3.1.1.2-24 of [5, TS 38.212] and if the scheduling DCI is not used for cross carrier or cross bandwidth part scheduling. If UE is configured with minimumSchedulingOffsetK0 in the active DL BWP and the currently applicable minimum scheduling offset restriction K0,min is larger than 0, the UE does not expected to receive the scheduling DCI with the SRS request field value other than '00'. The CSI-RS is located in the same slot as the SRS request field. If the UE configured with aperiodic SRS associated with aperiodic NZP CSI-RS resource, any of the TCI states configured in the scheduled CC shall not be configured with qcl-Type set to 'typeD'. - If periodic or semi-persistent SRS resource set is configured, the NZP-CSI-RS-ResourceId for measurement is indicated via higher layer parameter associatedCSI-RS in SRS-ResourceSet. < Unchanged parts are omitted >

In addition, there seems a typo in TS38.212. The second PTRS-DMRS association field should depend on presence of the second SRS resource set indicator field instead of the SRS resource set indicator field.

Proposal 4: Support following TP to determine second PTRS-DMRS associated field depends on presence of the second SRS resource set indicator field.

TS 38.212 - 7.3.1.1.2 Format 0_1 < Unchanged parts are omitted > - PTRS-DMRS association – number of bits determined as follows - 0 bit if PTRS-UplinkConfig is not configured in either dmrs-UplinkForPUSCH-MappingTypeA or dmrs-UplinkForPUSCH-MappingTypeB and transform precoder is disabled, or if transform precoder is enabled, or if maxRank=1; - 2 bits otherwise, where Table 7.3.1.1.2-25/7.3.1.1.2-25A and 7.3.1.1.2-26 are used to indicate the association between PTRS port(s) and DMRS port(s) when one PT-RS port and two PT-RS ports are configured by maxNrofPorts in PTRS-UplinkConfig respectively, and the DMRS ports are indicated by the Antenna ports field. When the SRS resource set indicator field is present and maxRank>2, this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to SRS resource indicator field and/or Precoding information and number of layers field according to Table 7.3.1.1.2-25 and 7.3.1.1.2-26. When the SRS resource set indicator field is present and maxRank=2, the MSB of this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to SRS resource indicator and/or Precoding information and number of layers field, and the LSB of this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to Second SRS resource indicator field and/or Second Precoding information field, according to Table 7.3.1.1.2-25A. If "Bandwidth part indicator" field indicates a bandwidth part other than the active bandwidth part and the "PTRS-DMRS association" field is present for the indicated bandwidth part but not present for the active bandwidth part, the UE assumes the "PTRS-DMRS association" field is not present for the indicated bandwidth part. - Second PTRS-DMRS association – 2 bits if PTRS-DMRS association field and Second SRS resource set indicator field are present and maxRank>2; 0 bit otherwise. Table 7.3.1.1.2-25 and 7.3.1.1.2-26 are used to indicate the association between PTRS port(s) and DMRS port(s) corresponding to Second SRS resource indicator field and/or Second precoding information field when one PT-RS port and two PT-RS ports are configured by maxNrofPorts in PTRS-UplinkConfig respectively, and the DMRS ports are indicated by the Antenna ports field. < Unchanged parts are omitted > - 7.3.1.1.3 Format 0_2 < Unchanged parts are omitted > - PTRS-DMRS association – number of bits determined as follows - 0 bit if PTRS-UplinkConfig is not configured in either dmrs-UplinkForPUSCH-MappingTypeA or dmrs-UplinkForPUSCH-MappingTypeB and transform precoder is disabled, or if transform precoder is enabled, or if maxRankDCI-0-2=1; - 2 bits otherwise, where Table 7.3.1.1.2-25/7.3.1.1.2-25A and 7.3.1.1.2-26 are used to indicate the association between PTRS port(s) and DMRS port(s) when one PT-RS port and two PT-RS ports are configured by maxNrofPorts in PTRS-UplinkConfig respectively, and the DMRS ports are indicated by the Antenna ports field. When the SRS resource set indicator field is present and maxRankDCI-0-2>2, this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to SRS resource indicator field and/or Precoding information and number of layers field according to Table 7.3.1.1.2-25 and 7.3.1.1.2-26 field according to Table 7.3.1.1.2-25 and 7.3.1.1.2-26. When the SRS resource set indicator field is present and maxRankDCI-0-2=2, the MSB of this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to SRS resource indicator field and/or Precoding information and number of layers field, and the LSB of this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to Second SRS resource indicator field and/or Second Precoding information field, according to Table 7.3.1.1.2-25A. If "Bandwidth part indicator" field indicates a bandwidth part other than the active bandwidth part and the "PTRS-DMRS association" field is present for the indicated bandwidth part but not present for the active bandwidth part, the UE assumes the "PTRS-DMRS association" field is not present for the indicated bandwidth part. - Second PTRS-DMRS association – 2 bits if PTRS-DMRS association field and Second SRS resource set indicator field are present and maxRankDCI-0-2>2; 0 bit otherwise. Table 7.3.1.1.2-25 and 7.3.1.1.2-26 are used to indicate the association between PTRS port(s) and DMRS port(s) corresponding to Second SRS resource indicator field and/or Second precoding information field when one PT-RS port and two PT-RS ports are configured by maxNrofPorts in PTRS-UplinkConfig respectively, and the DMRS ports are indicated by the Antenna ports field. < Unchanged parts are omitted >

Another issue is on RV sequence applied for PUSCH transmission towards the second TRP with the second TCI state, the design principle agreed is to apply RV pattern (0,2,3,1) separately for PUSCH repetition of different TRPs, as captured below in agreements.

Agreement For single DCI based PUSCH multi-TRP enhancements, support the following RV mapping for PUSCH repetition Type A, - DCI indicates the first RV for the first PUSCH repetition, and the RV pattern (0 2 3 1) is applied separately to PUSCH repetitions of different TRPs with a possibility of configuring RV offset for the starting RV for the second TRP (The same method as PDSCH scheme 4) - FFS: Reuse of the same method for PUSCH repetition Type B. Agreement For single DCI based M-TRP PUSCH repetition Type B, support the following RV mapping, - DCI indicates the first RV for the first PUSCH actual repetition, and the RV pattern (0 2 3 1) is applied separately to PUSCH actual repetitions of different TRPs with a possibility of configuring RV offset for the starting RV for the first actual repetition towards second TRP (The same method as PDSCH scheme 4).

However, in current TS, the RV sequence applied to second TRP is not a cyclic shifted version of legacy RV sequence (0 2 3 1) but with an offset per each element in this sequence. One of the possible issue is that, some new RV sequences are introduced with non-self-decodable RVs transmitted consecutively and in the first two transmission, as shown in the analysis below, RV sequence (2 1 3 0) or (1 2 0 3) may be applied to PUSCH transmission to the second TRP.

According to Table 5.1.2.1-3 of TS 38.214, when rvs=1 rvid indicated by the DCI scheduling the PDSCH; rvid to be applied to nth transmission occasion with second TCI state. n mod 4 = 0; n mod 4 = 1; n mod 4 = 2; n mod 4 = 3. 0; 1; 3; 0; 2. 2; 3; 0; 2; 1. 3; 0; 2; 1; 3. 1; 2; 1; 3; 0. when rvs=3 rvid indicated by the DCI scheduling the PDSCH; rvid to be applied to nth transmission occasion with second TCI state. n mod 4 = 0; n mod 4 = 1; n mod 4 = 2; n mod 4 = 3. 0; 3; 1; 2; 0. 2; 1; 2; 0; 3. 3; 2; 0; 3; 1. 1; 0; 3; 1; 2.

To accurately reflect agreement that RV pattern (0 2 3 1) is applied separately to PUSCH repetitions of different TRP, we proposal the following TP.

Proposal 5: To reflect the agreement that RV pattern (0 2 3 1) is applied separately to PUSCH repetitions of different TRP, adopting the following TP.

TS 38.214 The UE may expect that each PDSCH transmission occasion is limited to two transmission layers. For all PDSCH transmission occasions associated with the first TCI state, the redundancy version to be applied is derived according to Table 5.1.2.1-2, where n is counted only considering PDSCH transmission occasions associated with the first TCI state. The redundancy version for PDSCH transmission occasions associated with the second TCI state is derived according to Table 5.1.2.1-3, where additional shifting operation for each redundancy version rvs is configured by higher layer parameter sequenceOffsetforRV and n is counted only considering PDSCH transmission occasions associated with the second TCI state. Table 5.1.2.1-3: Applied redundancy version for the second TCI state when sequenceOffsetforRV is present 0; (0+ rvs) mod 4; (2+ rvs) mod 4; (3+ rvs) mod 4; (1+ rvs) mod 4. 2; (2+ rvs) mod 4; (3+ rvs) mod 4; (1+ rvs) mod 4; (0+ rvs) mod 4. 3; (3+ rvs) mod 4; (1+ rvs) mod 4; (0+ rvs) mod 4; (2+ rvs) mod 4. 1; (1+ rvs) mod 4; (0+ rvs) mod 4; (2+ rvs) mod 4; (3+ rvs) mod 4. rvid indicated by the DCI scheduling the PDSCH; rvs=1, rvid to be applied to nth transmission occasion with second TCI state. n mod 4 = 0; n mod 4 = 1; n mod 4 = 2; n mod 4 = 3. 0; 2; 3; 1; 0. 2; 3; 1; 0; 2. 3; 1; 0; 2; 3. 1; 0; 2; 3; 1. rvid indicated by the DCI scheduling the PDSCH; rvs=2, rvid to be applied to nth transmission occasion with second TCI state. n mod 4 = 0; n mod 4 = 1; n mod 4 = 2; n mod 4 = 3. 0; 3; 1; 0; 2. 2; 1; 0; 2; 3. 3; 0; 2; 3; 1. 1; 2; 3; 1; 0. rvid indicated by the DCI scheduling the PDSCH; rvs=3, rvid to be applied to nth transmission occasion with second TCI state. n mod 4 = 0; n mod 4 = 1; n mod 4 = 2; n mod 4 = 3. 0; 1; 0; 2; 3. 2; 0; 2; 3; 1. 3; 2; 3; 1; 0. 1; 3; 1; 0; 2.

- Conclusion

In this contribution, we provided our views on some remaining issues on PUSCH and PUCCH based on multi-TRP transmission, and we proposed that:

Proposal 1: In case of intra-slot PUCCH repetition based on multi-TRP, support PUCCH repetition with only X = 2 consecutive sub-slots within a slot. And when a PUCCH is configured with two spatial settings or two sets of power control parameters, if UE is configured with subslotLengthForPUCCH, intra-slot PUCCH repetition is applied, and the UE ignores value of NPUCCHrepeat. We propose a following TP to reflect this in TS:

TS38.213 - 9.2.6 PUCCH repetition procedure < Unchanged parts are omitted > When a PUCCH resource used for repetitions of a PUCCH transmission by a UE includes first and second spatial settings, or first and second sets of power control parameters, as described in [11, TS 38.321] and in clause 7.2.1, and if the UE is provided subslotLengthForPUCCH, the UE transmits first and second repetitions of the PUCCH transmission with the UCI consecutively within a slot as described in [4, TS 38.211], and the UE ignores value of NPUCCHrepeat if configured, and a number of symbols for each repetition of PUCCH transmission is indicated by subslotLengthForPUCCH, and the UE uses the first and second spatial settings, or the first and second sets of power control parameters, for first and second repetitions of the PUCCH transmission, respectively; otherwise, the UE - uses the first and second spatial settings, or the first and second sets of power control parameters, for first and second repetitions of the PUCCH transmission, respectively, when NPUCCHrepeat=2, - alternates between the first and second spatial settings, or between the first and second sets of power control parameters, respectively, per NPUCCHswitch repetitions of the PUCCH transmission, where NPUCCHswitch=1 if mappingPattern = 'cyclicMapping'; else, NPUCCHswitch=2. < Unchanged parts are omitted >

Proposal 2: In case of intra-slot PUCCH repetition based on multi-TRP, if other values of X to be supported, we prefer only X=4 when sub-slot length is configured to be 2. And we propose an alternative TP to reflect this in TS:

TS38.213 v17.0.0 - 9.2.6 PUCCH repetition procedure A UE can be indicated to transmit a PUCCH over NPUCCHrepeat slots using a PUCCH resource, where - if the PUCCH resource is indicated by a DCI format and includes PUCCH-nrofSlots, NPUCCHrepeat is provided by PUCCH-nrofSlots - otherwise, NPUCCHrepeat is provided by nrofSlots < Unchanged parts are omitted > When a PUCCH resource used for repetitions of a PUCCH transmission by a UE includes first and second spatial settings, or first and second sets of power control parameters, as described in [11, TS 38.321] and in clause 7.2.1, the UE - uses the first and second spatial settings, or the first and second sets of power control parameters, for first and second repetitions of the PUCCH transmission, respectively, when NPUCCHrepeat=2, - alternates between the first and second spatial settings, or between the first and second sets of power control parameters, respectively, per NPUCCHswitch repetitions of the PUCCH transmission, where NPUCCHswitch=1 if mappingPattern = 'cyclicMapping'; else, NPUCCHswitch=2, - if the UE is provided subslotLengthForPUCCH with value 7 for normal cyclic prefix or value 6 for extended cyclic prefix, NPUCCHrepeat= 2 regardless the value of PUCCH-nrofSlots or nrofSlots if configured, and if the UE is provided subslotLengthForPUCCH with value 2, the UE is expected to be configured with value NPUCCHrepeat to be 2 or 4, and the NPUCCHrepeat repetitions of the PUCCH transmission are consecutive within a slot as described in [4, TS 38.211]. < Unchanged parts are omitted >

Proposal 3: To capture an associated CSI-RS resource configured to each one of two SRS resource sets for non-codebook based PUSCH transmission explicitly in TS 38.214.

TS 38.214 - 6.1.1.2 Non-Codebook based UL transmission For non-codebook based transmission, PUSCH can be scheduled by DCI format 0_0, DCI format 0_1, DCI format 0_2 or semi-statically configured to operate according to Clause 6.1.2.3. If this PUSCH is scheduled by DCI format 0_1, DCI format 0_2, or semi-statically configured to operate according to Clause 6.1.2.3, the UE can determine its PUSCH precoder(s) and transmission rank based on the SRI(s) when multiple SRS resources are configured, where the SRI(s) is given by one or two SRS resource indicator(s) in DCI according to clause 7.3.1.1.2 and 7.3.1.1.3 of [5, 38.212] for DCI format 0_1 and DCI format 0_2, or the SRI is given by srs-ResourceIndicator according to clause 6.1.2.3, or two SRIs given by srs-ResourceIndicator and srs-ResourceIndicator2 according to clause 6.1.2.3.. The SRS-ResourceSet(s) applicable for PUSCH scheduled by DCI format 0_1 and DCI format 0_2 are defined by the entries of the higher layer parameter srs-ResourceSetToAddModList and srs-ResourceSetToAddModListDCI-0-2 in SRS-config, respectively. The UE shall use one or multiple SRS resources for SRS transmission, where, in a SRS resource set, the maximum number of SRS resources which can be configured to the UE for simultaneous transmission in the same symbol and the maximum number of SRS resources are UE capabilities. The SRS resources transmitted simultaneously occupy the same RBs. Only one SRS port for each SRS resource is configured. Only one or two SRS resource sets can be configured in srs-ResourceSetToAddModList with higher layer parameter usage in SRS-ResourceSet set to 'nonCodebook', and only one or two SRS resource sets can be configured in srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'nonCodebook'. When two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'nonCodebook', one or two SRIs are given by the DCI fields of two SRS resource indicators in clause 7.3.1.1.2 and 7.3.1.1.3 of [5, TS 38.212] for DCI format 0_1 and 0_2. The UE applies the indicated SRI(s) to one or more PUSCH repetitions according to the associated SRS resource set of a PUSCH repetition according to clause 6.1.2.1. The maximum number of SRS resources per SRS resource set that can be configured for non-codebook based uplink transmission is 4. Each of the indicated one or two SRIs in slot n is associated with the most recent transmission of SRS resource(s) of associated SRS resource set identified by the SRI, where the SRS transmission is prior to the PDCCH carrying the SRI. When two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'nonCodebook', the UE is not expected to be configured with different number of SRS resources in the two SRS resource sets. When the PDCCH candidates are associated with a search space set configured with searchSpaceLinking, for the purpose of determining the most recent transmission of SRS resource(s) identified by the SRI, the PDCCH candidate that starts earlier in time among the two configured PDCCH candidates is used. For non-codebook based transmission, the UE can calculate the precoder used for the transmission of SRS based on measurement of an associated NZP CSI-RS resource. When two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'nonCodebook', each of the two SRS resource sets can be configured with one associated CSI-RS resource. A UE can be configured with only one NZP CSI-RS resource for each of the one or two SRS resource sets with higher layer parameter usage in SRS-ResourceSet set to 'nonCodebook' if configured. - If aperiodic SRS resource set is configured, the associated NZP-CSI-RS is indicated via SRS request field in DCI format 0_1 and 1_1, as well as DCI format 0_2 (if SRS request field is present) and DCI format 1_2 (if SRS request field is present), where AperiodicSRS-ResourceTrigger and AperiodicSRS-ResourceTriggerList (indicating the association between aperiodic SRS triggering state(s) and SRS resource sets), triggered SRS resource(s) srs-ResourceSetId, csi-RS (indicating the associated NZP-CSI-RS-ResourceId) are higher layer configured in SRS-ResourceSet. The SRS-ResourceSet(s) associated with the SRS request by DCI format 0_1 and 1_1 are defined by the entries of the higher layer parameter srs-ResourceSetToAddModList and the SRS-ResourceSet(s) associated with the SRS request by DCI format 0_2 and 1_2 are defined by the entries of the higher layer parameter srs-ResourceSetToAddModListDCI-0-2. A UE is not expected to update the SRS precoding information if the gap from the last symbol of the reception of the aperiodic NZP-CSI-RS resource and the first symbol of the aperiodic SRS transmission is less than 42 OFDM symbols. - If the UE configured with aperiodic SRS associated with aperiodic NZP CSI-RS resource, the presence of the associated CSI-RS is indicated by the SRS request field if the value of the SRS request field is not '00' as in Table 7.3.1.1.2-24 of [5, TS 38.212] and if the scheduling DCI is not used for cross carrier or cross bandwidth part scheduling. If UE is configured with minimumSchedulingOffsetK0 in the active DL BWP and the currently applicable minimum scheduling offset restriction K0,min is larger than 0, the UE does not expected to receive the scheduling DCI with the SRS request field value other than '00'. The CSI-RS is located in the same slot as the SRS request field. If the UE configured with aperiodic SRS associated with aperiodic NZP CSI-RS resource, any of the TCI states configured in the scheduled CC shall not be configured with qcl-Type set to 'typeD'. - If periodic or semi-persistent SRS resource set is configured, the NZP-CSI-RS-ResourceId for measurement is indicated via higher layer parameter associatedCSI-RS in SRS-ResourceSet. < Unchanged parts are omitted >

Proposal 4: Support following TP to determine second PTRS-DMRS associated field depends on presence of the second SRS resource set indicator field.

TS 38.212 - 7.3.1.1.2 Format 0_1 < Unchanged parts are omitted > - PTRS-DMRS association – number of bits determined as follows - 0 bit if PTRS-UplinkConfig is not configured in either dmrs-UplinkForPUSCH-MappingTypeA or dmrs-UplinkForPUSCH-MappingTypeB and transform precoder is disabled, or if transform precoder is enabled, or if maxRank=1; - 2 bits otherwise, where Table 7.3.1.1.2-25/7.3.1.1.2-25A and 7.3.1.1.2-26 are used to indicate the association between PTRS port(s) and DMRS port(s) when one PT-RS port and two PT-RS ports are configured by maxNrofPorts in PTRS-UplinkConfig respectively, and the DMRS ports are indicated by the Antenna ports field. When the SRS resource set indicator field is present and maxRank>2, this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to SRS resource indicator field and/or Precoding information and number of layers field according to Table 7.3.1.1.2-25 and 7.3.1.1.2-26. When the SRS resource set indicator field is present and maxRank=2, the MSB of this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to SRS resource indicator and/or Precoding information and number of layers field, and the LSB of this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to Second SRS resource indicator field and/or Second Precoding information field, according to Table 7.3.1.1.2-25A. If "Bandwidth part indicator" field indicates a bandwidth part other than the active bandwidth part and the "PTRS-DMRS association" field is present for the indicated bandwidth part but not present for the active bandwidth part, the UE assumes the "PTRS-DMRS association" field is not present for the indicated bandwidth part. - Second PTRS-DMRS association – 2 bits if PTRS-DMRS association field and Second SRS resource set indicator field are present and maxRank>2; 0 bit otherwise. Table 7.3.1.1.2-25 and 7.3.1.1.2-26 are used to indicate the association between PTRS port(s) and DMRS port(s) corresponding to Second SRS resource indicator field and/or Second precoding information field when one PT-RS port and two PT-RS ports are configured by maxNrofPorts in PTRS-UplinkConfig respectively, and the DMRS ports are indicated by the Antenna ports field. < Unchanged parts are omitted > - 7.3.1.1.3 Format 0_2 < Unchanged parts are omitted > - PTRS-DMRS association – number of bits determined as follows - 0 bit if PTRS-UplinkConfig is not configured in either dmrs-UplinkForPUSCH-MappingTypeA or dmrs-UplinkForPUSCH-MappingTypeB and transform precoder is disabled, or if transform precoder is enabled, or if maxRankDCI-0-2=1; - 2 bits otherwise, where Table 7.3.1.1.2-25/7.3.1.1.2-25A and 7.3.1.1.2-26 are used to indicate the association between PTRS port(s) and DMRS port(s) when one PT-RS port and two PT-RS ports are configured by maxNrofPorts in PTRS-UplinkConfig respectively, and the DMRS ports are indicated by the Antenna ports field. When the SRS resource set indicator field is present and maxRankDCI-0-2>2, this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to SRS resource indicator field and/or Precoding information and number of layers field according to Table 7.3.1.1.2-25 and 7.3.1.1.2-26 field according to Table 7.3.1.1.2-25 and 7.3.1.1.2-26. When the SRS resource set indicator field is present and maxRankDCI-0-2=2, the MSB of this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to SRS resource indicator field and/or Precoding information and number of layers field, and the LSB of this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to Second SRS resource indicator field and/or Second Precoding information field, according to Table 7.3.1.1.2-25A. If "Bandwidth part indicator" field indicates a bandwidth part other than the active bandwidth part and the "PTRS-DMRS association" field is present for the indicated bandwidth part but not present for the active bandwidth part, the UE assumes the "PTRS-DMRS association" field is not present for the indicated bandwidth part. - Second PTRS-DMRS association – 2 bits if PTRS-DMRS association field and Second SRS resource set indicator field are present and maxRankDCI-0-2>2; 0 bit otherwise. Table 7.3.1.1.2-25 and 7.3.1.1.2-26 are used to indicate the association between PTRS port(s) and DMRS port(s) corresponding to Second SRS resource indicator field and/or Second precoding information field when one PT-RS port and two PT-RS ports are configured by maxNrofPorts in PTRS-UplinkConfig respectively, and the DMRS ports are indicated by the Antenna ports field. < Unchanged parts are omitted >

Proposal 5: To reflect the agreement that RV pattern (0 2 3 1) is applied separately to PUSCH repetitions of different TRP, adopting the following TP.

TS 38.214 The UE may expect that each PDSCH transmission occasion is limited to two transmission layers. For all PDSCH transmission occasions associated with the first TCI state, the redundancy version to be applied is derived according to Table 5.1.2.1-2, where n is counted only considering PDSCH transmission occasions associated with the first TCI state. The redundancy version for PDSCH transmission occasions associated with the second TCI state is derived according to Table 5.1.2.1-3, where additional shifting operation for each redundancy version rvs is configured by higher layer parameter sequenceOffsetforRV and n is counted only considering PDSCH transmission occasions associated with the second TCI state. Table 5.1.2.1-3: Applied redundancy version for the second TCI state when sequenceOffsetforRV is present 0; (0+ rvs) mod 4; (2+ rvs) mod 4; (3+ rvs) mod 4; (1+ rvs) mod 4. 2; (2+ rvs) mod 4; (3+ rvs) mod 4; (1+ rvs) mod 4; (0+ rvs) mod 4. 3; (3+ rvs) mod 4; (1+ rvs) mod 4; (0+ rvs) mod 4; (2+ rvs) mod 4. 1; (1+ rvs) mod 4; (0+ rvs) mod 4; (2+ rvs) mod 4; (3+ rvs) mod 4. rvid indicated by the DCI scheduling the PDSCH; rvs=1, rvid to be applied to nth transmission occasion with second TCI state. n mod 4 = 0; n mod 4 = 1; n mod 4 = 2; n mod 4 = 3. 0; 2; 3; 1; 0. 2; 3; 1; 0; 2. 3; 1; 0; 2; 3. 1; 0; 2; 3; 1. rvid indicated by the DCI scheduling the PDSCH; rvs=2, rvid to be applied to nth transmission occasion with second TCI state. n mod 4 = 0; n mod 4 = 1; n mod 4 = 2; n mod 4 = 3. 0; 3; 1; 0; 2. 2; 1; 0; 2; 3. 3; 0; 2; 3; 1. 1; 2; 3; 1; 0. rvid indicated by the DCI scheduling the PDSCH; rvs=3, rvid to be applied to nth transmission occasion with second TCI state. n mod 4 = 0; n mod 4 = 1; n mod 4 = 2; n mod 4 = 3. 0; 1; 0; 2; 3. 2; 0; 2; 3; 1. 3; 2; 3; 1; 0. 1; 3; 1; 0; 2.

- References

- TS 38.213 V17.0.0

- TS 38.214 V17.0.0

- TS 38.212 V17.0.0 .