3GPP TSG RAN WG1 #108-e										 	  	  R1-2201898

e-Meeting, February 21st – March 3rd, 2022

Agenda item:	8.1.3

Source:	NEC

Title:               Discussion remaining issues on SRS enhancement

Document for:	Discussion and Decision

- Introduction

Rel-17 SRS enhancements have been finished, while there are some remaining issues. In this contribution, we provided our views on some remaining issues.

- Discussion

CS for 4-port SRS based on comb 8

In RAN1#107-e meeting [1], there was a working assumption for 4-port SRS based on comb 8, i.e. maximum number of CS is 6.

Working assumption To support 4 ports with Max CS = 6, - Port 0 and Port 2 locate in n_CS and (n_CS+3) mod 6 in comb offset k_TC, respectively. - Port 1 and Port 3 locate in n_CS and (n_CS+3) mod 6 in comb offset (k_TC + 4) mod 8, respectively. - Note: n_CS and k_TC are the configured CS and comb offset values. - Note: This working assumption can be revisited if Max CS = 12 is agreed.

4-port SRS with comb 8 is based on FDM for ports 0/2 and ports 1/3, while the cyclic shift values for ports 0/2 and ports 1/3 are same in current structure, i.e. sequences for ports 0/2 and ports 1/3 on different REs are same, which will lead to higher PAPR issue as discussed ever since Rel-16. So we propose different cyclic shift values for ports 0/2 and ports 1/3, similar as legacy structure for 4-port SRS. In Rel-15/16, 4-port SRS may also map on different REs based on different values of comb offset, while the cyclic shift values are different for different SRS ports, so there is no PAPR issue.

We compare the simulation results between same cyclic shift values and different cyclic shift values for Ports 0/2 and Ports 1/3 based on different root sequences for SRS, and phase of beamforming factor of each Tx for SRS is a random value within range of 0-2pi. For the results shown in Figure 1, we can see PAPR can be reduced with different cyclic shift values for Ports 0/2 and Ports 1/3, and this structure is also aligned with legacy structure, and with marginal spec effort, performance can be improved, especially for coverage enhancement.

Figure 1. CCDF of PAPR for 4-ports with same and different CS for Ports 0/2 and Ports 1/3

Proposal 1: For 4 ports SRS with Max CS=6, support different cyclic shift values for Ports 0/2 and Ports 1/3, revise the working assumption to be:

- Port 0 and Port 2 locate in n_CS and (n_CS+3) mod 6 in comb offset k_TC, respectively.

- Port 1 and Port 3 locate in (n_CS+1) mod 6 and (n_CS+3+1) mod 6 in comb offset (k_TC + 4) mod 8, respectively.

Maximum number of CS for RPFS

In RAN1#107-e meeting, there was a conclusion for number of RBs for RPFS achieved.

Conclusion No consensus to have further restriction on the number of RBs for RPFS in Rel-17. - No introduction of new sequence length

In current spec, the maximum number of CS is determined based on value of KTC, the reason is that in Rel-15/16 SRS framework, the number of RBs for SRS is a multiple of 4, so maximum number of CS can be represented as a function of only KTC. But to be accurate, the final facor to determine the maximum number of CS is actually the length of SRS sequence.

Table 6.4.1.4.2-1: Maximum number of cyclic shifts nSRScs,max as a function of KTC.

| KTC | nSRScs,max |
| --- | --- |
| 2 | 8 |
| 4 | 12 |
| 8 | 6 |

In Rel-15/16, based on number of RBs for SRS a multiple of 4, the SRS sequence length and maximum number of cyclic shift values are actually as follows:

| KTC | nSRScs,max | SRS sequence length Msc,bSRS=mSRS,bNscRBKTC |
| --- | --- | --- |
| 2 | 8 | Actually a multiple of 24 |
| 4 | 12 | Actually a multiple of 12 |
| 8 | 6 | Actually a multiple of 6 |

Based on the conclusion, there is no further restriction on the number of RBs (1PFmSRS, BSRS) for RPFS, as long as there is no new sequence length. While this will cause some values of number of RBs for SRS, for example, when the number of RBs configured for SRS is mSRS, BSRS = 20, and if PF=4, then the actual number of RBs for the SRS is actually 20/4=5. If comb value KTC is configured to be 2, the final sequence length for SRS is Msc,bSRS=mSRS,bNscRBKTCPF =30, this sequence length exists in legacy TS, so this configuration satisfies requirement. While there is an issue, that is if based on current spec, the maximum number of CS for this configuration is 8 (based on the function of KTC=2), but the actually maximum number of CS for this SRS configuration can not support 8, as the length of SRS sequence is 30, only a multiple of 6. So actually, the maximum number of CS supported for this SRS configuartion is only 6, i.e. nSRScs,max should be 6. While based on current function, nSRScs,max=8, only if the cyclic shift value αi=2πnSRScs,inSRScs,max is 0 or 4, i.e. only if αi=2π08 (same as αi=2π06) and αi=2π48 (same as αi=2π36), the SRS ports can be orthogonal. In other words, up to 2 orthogonal ports can be supported based on different CS. Even based on network configuration to guarantee the orthogonality is not a good way, as:

- Capacity of SRS is actually reduced

Taking the above configuration as example (mSRS, BSRS = 20, PF=4, KTC = 2), if nSRScs,max can be determined to be 6, then each value of CS with {0, 1, 2, 3, 4, 5} can be used to achieve orthogonality, i.e. more orthogonal ports can be supported compared to nSRScs,max determined to be 8 (only two ports can be orthogonal).

- 4-port SRS can not be supported

Still taking the above configuration as example (mSRS, BSRS = 20, PF=4, KTC = 2), based on current framework, nSRScs,max=8, and nSRScs,i=nSRScs+nSRScs,maxpi-1000NapSRS mod nSRScs,max, taking for the four SRS ports, nSRScs,i are {nSRScs+0, nSRScs+2, nSRScs+4, nSRScs+6} mod 8, and based on the comb offset determination formula

kTCpi=kTC+KTC2 mod KTCif NapSRS=4, pi∈1001, 1003, and nSRScs,max=6 kTC+KTC2 mod KTC if NapSRS=4, pi∈1001, 1003, and nSRScs∈nSRScs,max2, …, nSRScs,max-1kTCotherwise

Ports 0 and 2 are always on same REs (kTC), and for Ports 1/3, only when nSRScs∈nSRScs,max2, …, nSRScs,max-1, the port can be on different REs (kTC+KTC2 mod KTC). While the possible values of CS for the 4 ports are:

|  | Port 0 | Port 1 | Port 2 | Port 3 |
| --- | --- | --- | --- | --- |
| CS | 0 | 2 | 4 | 6 |
|  | 1 | 3 | 5 | 7 |
|  | 2 | 4 | 6 | 0 |
|  | 3 | 5 | 7 | 1 |
|  | 4 | 6 | 0 | 2 |
|  | 5 | 7 | 1 | 3 |
|  | 6 | 0 | 2 | 4 |
|  | 7 | 1 | 3 | 5 |

Based on the formula, for Port 1 and Port 3, only when nSRScs∈{4,5,6,7}, marked with blue and red, the port will be mapped to different REs, but from the above table, only one of Port 1 and 3 will be mapped to different REs, in other words, at least one of Port 1 and Port 3 will be mapped on same REs as Port 0 and Port 2 (in above table, red marked and blue marked are not overlapped in same configuration), while as we discussed above, based on nSRScs,max=8, orthogonality can not be achieved between the ports.

The discussed configuration above is not a corner case, as shown in Table 1 and Table 2. In Table, we listed the sequence length for different SRS bandwidth configuration, in case of KTC = 2 and PF=2. Based on current maximum number of CS determination scheme, nSRScs,max=8, and sequences can be orthogonal when the sequence length is a multiple of 8, while we can see all yellow marked sequence length is not a multiple of 8 (actually a multiple of 6), in other words, based on nSRScs,max=8, number of orthogonal ports will be significantly reduced, and 4-port SRS can not be supported, especially for frequency hopping case, there are quite limited case available in last and second last columns.

Table 1. SRS sequence length (mSRS,bNscRBKTCPF ) in case of KTC = 2 and PF=2

|  | mSRS,0PF | Sequence length | mSRS,1PF | Sequence length | mSRS,2PF | Sequence length | mSRS,3PF | Sequence length |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 4/2=2 | 12 | 4/2=2 | 12 | 4/2=2 | 12 | 4/2=2 | 12 |
| 1 | 8/2=4 | 24 | 4/2=2 | 12 | 4/2=2 | 12 | 4/2=2 | 12 |
| 2 | 12/2=6 | 36 | 4/2=2 | 12 | 4/2=2 | 12 | 4/2=2 | 12 |
| 3 | 16/2=8 | 48 | 4/2=2 | 12 | 4/2=2 | 12 | 4/2=2 | 12 |
| 4 | 16/2=8 | 48 | 8/2=4 | 24 | 4/2=2 | 12 | 4/2=2 | 12 |
| 5 | 20/2=10 | 60 | 4/2=2 | 12 | 4/2=2 | 12 | 4/2=2 | 12 |
| 6 | 24/2=12 | 72 | 4/2=2 | 12 | 4/2=2 | 12 | 4/2=2 | 12 |
| 7 | 24/2=12 | 72 | 12/2=6 | 36 | 4/2=2 | 12 | 4/2=2 | 12 |
| 8 | 28/2=14 | 84 | 4/2=2 | 12 | 4/2=2 | 12 | 4/2=2 | 12 |
| 9 | 32/2=16 | 96 | 16/2=8 | 48 | 8/2=4 | 24 | 4/2=2 | 12 |
| 10 | 36/2=18 | 108 | 12/2=6 | 36 | 4/2=2 | 12 | 4/2=2 | 12 |
| 11 | 40/2=20 | 120 | 20/2=10 | 60 | 4/2=2 | 12 | 4/2=2 | 12 |
| 12 | 48/2=24 | 144 | 16/2=8 | 48 | 8/2=4 | 24 | 4/2=2 | 12 |
| 13 | 48/2=24 | 144 | 24/2=12 | 72 | 12/2=6 | 36 | 4/2=2 | 12 |
| 14 | 52/2=26 | 156 | 4/2=2 | 12 | 4/2=2 | 12 | 4/2=2 | 12 |
| 15 | 56/2=28 | 168 | 28/2=14 | 84 | 4/2=2 | 12 | 4/2=2 | 12 |
| 16 | 60/2=30 | 180 | 20/2=10 | 60 | 4/2=2 | 12 | 4/2=2 | 12 |
| 17 | 64/2=32 | 192 | 32/2=16 | 96 | 16/2=8 | 48 | 4/2=2 | 12 |
| 18 | 72/2=36 | 216 | 24/2=12 | 72 | 12/2=6 | 36 | 4/2=2 | 12 |
| 19 | 72/2=36 | 216 | 36/2=18 | 108 | 12/2=6 | 36 | 4/2=2 | 12 |
| 20 | 76/2=38 | 228 | 4/2=2 | 12 | 4/2=2 | 12 | 4/2=2 | 12 |
| 21 | 80/2=40 | 240 | 40/2=20 | 120 | 20/2=10 | 60 | 4/2=2 | 12 |
| 22 | 88/2=44 | 264 | 44/2=22 | 132 | 4/2=2 | 12 | 4/2=2 | 12 |
| 23 | 96/2=48 | 288 | 32/2=16 | 96 | 16/2=8 | 48 | 4/2=2 | 12 |
| 24 | 96/2=48 | 288 | 48/2=24 | 144 | 24/2=12 | 72 | 4/2=2 | 12 |
| 25 | 104/2=52 | 312 | 52/2=26 | 156 | 4/2=2 | 12 | 4/2=2 | 12 |
| 26 | 112/2=56 | 336 | 56/2=28 | 168 | 28/2=14 | 84 | 4/2=2 | 12 |
| 27 | 120/2=60 | 360 | 60/2=30 | 180 | 20/2=10 | 60 | 4/2=2 | 12 |
| 28 | 120/2=60 | 360 | 40/2=20 | 120 | 8/2=4 | 24 | 4/2=2 | 12 |
| 29 | 120/2=60 | 360 | 24/2=12 | 72 | 12/2=6 | 36 | 4/2=2 | 12 |
| 30 | 128/2=64 | 384 | 64/2=32 | 192 | 32/2=16 | 96 | 4/2=2 | 12 |
| 31 | 128/2=64 | 384 | 64/2=32 | 192 | 16/2=8 | 48 | 4/2=2 | 12 |
| 32 | 128/2=64 | 384 | 16/2=8 | 48 | 8/2=4 | 24 | 4/2=2 | 12 |
| 33 | 132/2=66 | 396 | 44/2=22 | 132 | 4/2=2 | 12 | 4/2=2 | 12 |
| 34 | 136/2=68 | 408 | 68/2=34 | 204 | 4/2=2 | 12 | 4/2=2 | 12 |
| 35 | 144/2=72 | 432 | 72/2=36 | 216 | 36/2=18 | 108 | 4/2=2 | 12 |
| 36 | 144/2=72 | 432 | 48/2=24 | 144 | 24/2=12 | 72 | 12/2=6 | 36 |
| 37 | 144/2=72 | 432 | 48/2=24 | 144 | 16/2=8 | 48 | 4/2=2 | 12 |
| 38 | 144/2=72 | 432 | 16/2=8 | 48 | 8/2=4 | 24 | 4/2=2 | 12 |
| 39 | 152/2=76 | 456 | 76/2=38 | 228 | 4/2=2 | 12 | 4/2=2 | 12 |
| 40 | 160/2=80 | 480 | 80/2=40 | 240 | 40/2=20 | 120 | 4/2=2 | 12 |
| 41 | 160/2=80 | 480 | 80/2=40 | 240 | 20/2=10 | 60 | 4/2=2 | 12 |
| 42 | 160/2=80 | 480 | 32/2=16 | 96 | 16/2=8 | 48 | 4/2=2 | 12 |
| 43 | 168/2=84 | 504 | 84/2=42 | 252 | 28/2=14 | 84 | 4/2=2 | 12 |
| 44 | 176/2=88 | 528 | 88/2=44 | 264 | 44/2=22 | 132 | 4/2=2 | 12 |
| 45 | 184/2=92 | 552 | 92/2=46 | 276 | 4/2=2 | 12 | 4/2=2 | 12 |
| 46 | 192/2=96 | 576 | 96/2=48 | 288 | 48/2=24 | 144 | 4/2=2 | 12 |
| 47 | 192/2=96 | 576 | 96/2=48 | 288 | 24/2=12 | 72 | 4/2=2 | 12 |
| 48 | 192/2=96 | 576 | 64/2=32 | 192 | 16/2=8 | 48 | 4/2=2 | 12 |
| 49 | 192/2=96 | 576 | 24/2=12 | 72 | 8/2=4 | 24 | 4/2=2 | 12 |
| 50 | 208/2=104 | 624 | 104/2=52 | 312 | 52/2=26 | 156 | 4/2=2 | 12 |
| 51 | 216/2=108 | 548 | 108/2=54 | 324 | 36/2=18 | 108 | 4/2=2 | 12 |
| 52 | 224/2=112 | 672 | 112/2=56 | 336 | 56/2=28 | 168 | 4/2=2 | 12 |
| 53 | 240/2=120 | 720 | 120/2=60 | 360 | 60/2=30 | 180 | 4/2=2 | 12 |
| 54 | 240/2=120 | 720 | 80/2=40 | 240 | 20/2=10 | 60 | 4/2=2 | 12 |
| 55 | 240/2=120 | 720 | 48/2=24 | 144 | 16/2=8 | 48 | 8/2=4 | 24 |
| 56 | 240/2=120 | 720 | 24/2=12 | 72 | 12/2=6 | 36 | 4/2=2 | 12 |
| 57 | 256/2=128 | 768 | 128/2=64 | 384 | 64/2=32 | 192 | 4/2=2 | 12 |
| 58 | 256/2=128 | 768 | 128/2=64 | 384 | 32/2=16 | 96 | 4/2=2 | 12 |
| 59 | 256/2=128 | 768 | 16/2=8 | 48 | 8/2=4 | 24 | 4/2=2 | 12 |
| 60 | 264/2=132 | 792 | 132/2=66 | 396 | 44/2=22 | 132 | 4/2=2 | 12 |
| 61 | 272/2=136 | 816 | 136/2=68 | 408 | 68/2=34 | 204 | 4/2=2 | 12 |
| 62 | 272/2=136 | 816 | 68/2=34 | 204 | 4/2=2 | 12 | 4/2=2 | 12 |
| 63 | 272/2=136 | 816 | 16/2=8 | 48 | 8/2=4 | 24 | 4/2=2 | 12 |

Similarly, we listed the sequence length in case of KTC = 2 and PF=4 in Table 2, the issue is much more severe, (the sequence length is still a multiple of 6), if determining the maximum number of CS based on current rule (KTC only), most cases especially for frequency hopping can not be used, e.g. for the last row with , none configuration is available.

Table 2. SRS sequence length (mSRS,bNscRBKTCPF ) in case of KTC = 2 and PF=4

|  | mSRS,0PF | Sequence length | mSRS,1PF | Sequence length | mSRS,2PF | Sequence length | mSRS,3PF | Sequence length |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 4/4=1 | 6 | 4/4=1 | 6 | 4/4=1 | 6 | 4/4=1 | 6 |
| 1 | 8/4=2 | 12 | 4/4=1 | 6 | 4/4=1 | 6 | 4/4=1 | 6 |
| 2 | 12/4=3 | 18 | 4/4=1 | 6 | 4/4=1 | 6 | 4/4=1 | 6 |
| 3 | 16/4=4 | 24 | 4/4=1 | 6 | 4/4=1 | 6 | 4/4=1 | 6 |
| 4 | 16/4=4 | 24 | 8/4=2 | 12 | 4/4=1 | 6 | 4/4=1 | 6 |
| 5 | 20/4=5 | 30 | 4/4=1 | 6 | 4/4=1 | 6 | 4/4=1 | 6 |
| 6 | 24/4=6 | 36 | 4/4=1 | 6 | 4/4=1 | 6 | 4/4=1 | 6 |
| 7 | 24/4=6 | 36 | 12/4=3 | 18 | 4/4=1 | 6 | 4/4=1 | 6 |
| 8 | 28/4=7 | 42 | 4/4=2 | 6 | 4/4=1 | 6 | 4/4=1 | 6 |
| 9 | 32/4=8 | 48 | 16/4=4 | 24 | 8/4=2 | 12 | 4/4=1 | 6 |
| 10 | 36/4=9 | 54 | 12/4=3 | 18 | 4/4=1 | 6 | 4/4=1 | 6 |
| 11 | 40/4=10 | 60 | 20/4=5 | 30 | 4/4=1 | 6 | 4/4=1 | 6 |
| 12 | 48/4=12 | 72 | 16/4=4 | 24 | 8/4=2 | 12 | 4/4=1 | 6 |
| 13 | 48/4=12 | 72 | 24/4=6 | 36 | 12/4=3 | 18 | 4/4=1 | 6 |
| 14 | 52/4=13 | 78 | 4/4=1 | 6 | 4/4=1 | 6 | 4/4=1 | 6 |
| 15 | 56/4=14 | 84 | 28/4=7 | 42 | 4/4=1 | 6 | 4/4=1 | 6 |
| 16 | 60/4=15 | 90 | 20/4=5 | 30 | 4/4=1 | 6 | 4/4=1 | 6 |
| 17 | 64/4=16 | 96 | 32/4=8 | 48 | 16/4=4 | 24 | 4/4=1 | 6 |
| 18 | 72/4=18 | 108 | 24/4=6 | 36 | 12/4=3 | 18 | 4/4=1 | 6 |
| 19 | 72/4=18 | 108 | 36/4=9 | 54 | 12/4=3 | 18 | 4/4=1 | 6 |
| 20 | 76/4=19 | 114 | 4/4=1 | 6 | 4/4=1 | 6 | 4/4=1 | 6 |
| 21 | 80/4=20 | 120 | 40/4=10 | 60 | 20/4=5 | 30 | 4/4=1 | 6 |
| 22 | 88/4=22 | 132 | 44/4=11 | 66 | 4/4=1 | 6 | 4/4=1 | 6 |
| 23 | 96/4=24 | 144 | 32/4=8 | 48 | 16/4=4 | 24 | 4/4=1 | 6 |
| 24 | 96/4=24 | 144 | 48/4=12 | 72 | 24/4=6 | 36 | 4/4=1 | 6 |
| 25 | 104/4=26 | 156 | 52/4=13 | 78 | 4/4=2 | 6 | 4/4=1 | 6 |
| 26 | 112/4=28 | 168 | 56/4=14 | 84 | 28/4=7 | 42 | 4/4=1 | 6 |
| 27 | 120/4=30 | 180 | 60/4=30 | 90 | 20/4=5 | 30 | 4/4=1 | 6 |
| 28 | 120/4=30 | 180 | 40/4=10 | 60 | 8/4=2 | 12 | 4/4=1 | 6 |
| 29 | 120/4=30 | 180 | 24/4=6 | 36 | 12/4=3 | 18 | 4/4=1 | 6 |
| 30 | 128/4=32 | 192 | 64/4=16 | 96 | 32/4=8 | 48 | 4/4=1 | 6 |
| 31 | 128/4=32 | 192 | 64/4=16 | 96 | 16/4=4 | 24 | 4/4=1 | 6 |
| 32 | 128/4=32 | 192 | 16/4=4 | 24 | 8/4=2 | 12 | 4/4=1 | 6 |
| 33 | 132/4=33 | 198 | 44/4=11 | 66 | 4/4=1 | 6 | 4/4=1 | 6 |
| 34 | 136/4=34 | 204 | 68/4=17 | 102 | 4/4=1 | 6 | 4/4=1 | 6 |
| 35 | 144/4=36 | 216 | 72/4=18 | 108 | 36/4=9 | 54 | 4/4=1 | 6 |
| 36 | 144/4=36 | 216 | 48/4=12 | 72 | 24/4=6 | 36 | 12/4=3 | 18 |
| 37 | 144/4=36 | 216 | 48/4=12 | 72 | 16/4=4 | 24 | 4/4=1 | 6 |
| 38 | 144/4=36 | 216 | 16/4=4 | 24 | 8/4=2 | 12 | 4/4=1 | 6 |
| 39 | 152/4=38 | 228 | 76/4=19 | 114 | 4/4=1 | 6 | 4/4=1 | 6 |
| 40 | 160/4=40 | 240 | 80/4=20 | 120 | 40/4=10 | 60 | 4/4=1 | 6 |
| 41 | 160/4=40 | 240 | 80/4=20 | 120 | 20/4=5 | 30 | 4/4=1 | 6 |
| 42 | 160/4=40 | 240 | 32/4=4 | 48 | 16/4=4 | 24 | 4/4=1 | 6 |
| 43 | 168/4=42 | 252 | 84/4=21 | 126 | 28/4=7 | 42 | 4/4=1 | 6 |
| 44 | 176/4=44 | 264 | 88/4=22 | 132 | 44/4=11 | 66 | 4/4=1 | 6 |
| 45 | 184/4=46 | 276 | 92/4=23 | 138 | 4/4=1 | 6 | 4/4=1 | 6 |
| 46 | 192/4=48 | 288 | 96/4=24 | 144 | 48/4=12 | 72 | 4/4=1 | 6 |
| 47 | 192/4=48 | 288 | 96/4=24 | 144 | 24/4=6 | 36 | 4/4=1 | 6 |
| 48 | 192/4=48 | 288 | 64/4=16 | 96 | 16/4=4 | 24 | 4/4=1 | 6 |
| 49 | 192/4=48 | 288 | 24/4=6 | 36 | 8/4=2 | 12 | 4/4=1 | 6 |
| 50 | 208/4=52 | 312 | 104/4=26 | 156 | 52/4=13 | 78 | 4/4=1 | 6 |
| 51 | 216/4=54 | 274 | 108/4=27 | 162 | 36/4=9 | 54 | 4/4=1 | 6 |
| 52 | 224/4=56 | 336 | 112/4=28 | 168 | 56/4=14 | 84 | 4/4=1 | 6 |
| 53 | 240/4=60 | 360 | 120/4=30 | 180 | 60/4=15 | 90 | 4/4=1 | 6 |
| 54 | 240/4=60 | 360 | 80/4=20 | 120 | 20/4=5 | 30 | 4/4=1 | 6 |
| 55 | 240/4=60 | 360 | 48/4=12 | 72 | 16/4=4 | 24 | 8/4=2 | 12 |
| 56 | 240/4=60 | 360 | 24/4=6 | 36 | 12/4=3 | 18 | 4/4=1 | 6 |
| 57 | 256/4=64 | 384 | 128/4=32 | 192 | 64/4=16 | 96 | 4/4=1 | 6 |
| 58 | 256/4=64 | 384 | 128/4=32 | 192 | 32/4=8 | 48 | 4/4=1 | 6 |
| 59 | 256/4=64 | 384 | 16/4=4 | 24 | 8/4=2 | 12 | 4/4=1 | 6 |
| 60 | 264/4=66 | 396 | 132/4=33 | 198 | 44/4=22 | 66 | 4/4=1 | 6 |
| 61 | 272/4=68 | 408 | 136/4=34 | 204 | 68/4=34 | 102 | 4/4=1 | 6 |
| 62 | 272/4=68 | 408 | 68/4=17 | 102 | 4/4=2 | 6 | 4/4=1 | 6 |
| 63 | 272/4=68 | 408 | 16/4=4 | 24 | 8/4=2 | 12 | 4/4=1 | 6 |

So we think the determination of nSRScs,max should be updated, otherwise, the SRS enhancement is meaningless.

Referring to the update for RPFS on SRS sequence,

Msc,bSRS=mSRS,bNscRBKTC

Msc,bSRS=mSRS,bNscRBKTCPF

The sequence length is determined based on two parameters KTC and PF, and the SRS sequence length and maximum number of cyclic shift values are actually as follows:

| KTC | PF | SRS sequence length (and no new SRS sequence introduced) Msc,bSRS=mSRS,bNscRBKTCPF |
| --- | --- | --- |
| 2 | 2 | Actually a multiple of 12, similar with Rel-15/16 configuration of (KTC=4 (no PF)), so nSRScs,max should be 12. |
| 4 | 2 | Actually a multiple of 6, similar with Rel-15/16 configuration of (KTC=8 (no PF)), so nSRScs,max should be 6. |
| 8 | 2 | Actually a multiple of 6, similar with Rel-15/16 configuration of (KTC=8 (no PF)), so nSRScs,max should be 6. |
| 2 | 4 | Actually a multiple of 6, similar with Rel-15/16 configuration of (KTC=8 (no PF)), so nSRScs,max should be 6. |
| 4 | 4 | Actually a multiple of 6, similar with Rel-15/16 configuration of (KTC=8 (no PF)), so nSRScs,max should be 6. |
| 8 | 4 | Actually a multiple of 6, similar with Rel-15/16 configuration of (KTC=8 (no PF)), so nSRScs,max should be 6. |

In this case, we think it’s natural to determine maximum number of CS based on a KTC and PF.

For example,

If PF is configured, maximum number of cyclic shifts nSRScs,max as a function of KTC and PF.

| KTC | PF if configured | nSRScs,max |
| --- | --- | --- |
| 2 | 2 | 12 |
| 4 | 2 or 4 | 6 |
| 8 | 2 or 4 | 6 |
| 2 | 4 | 6 |

Proposal 2: If PF is configured to be 2 or 4, the maximum number of cyclic shifts should be based on the SRS sequence length, for example, a function of KTC and PF, and when KTC=2 and PF=2, nSRScs,max=12, otherwise nSRScs,max=6.

- Conclusion

In this contribution, we provided our views on SRS enhancement, and we proposed that:

Proposal 1: For 4 ports SRS with Max CS=6, support different cyclic shift values for Ports 0/2 and Ports 1/3, revise the working assumption to be:

- Port 0 and Port 2 locate in n_CS and (n_CS+3) mod 6 in comb offset k_TC, respectively.

- Port 1 and Port 3 locate in (n_CS+1) mod 6 and (n_CS+3+1) mod 6 in comb offset (k_TC + 4) mod 8, respectively.

Proposal 2: If PF is configured to be 2 or 4, the maximum number of cyclic shifts should be based on the SRS sequence length, for example, a function of KTC and PF, and when KTC=2 and PF=2, nSRScs,max=12, otherwise nSRScs,max=6.

- References

- Chairman’s notes, RAN1#107-e, November 11th – 19th, 2021.