3GPP TSG RAN WG1 #108-e													R1-2201907

e-Meeting, February 21st – March 3rd, 2022

Agenda Item:	8.11.1.2

Source: 	NEC

Title:	Discussion on mode 2 enhancements

Document for:	Discussion/Decision

Discussion

Finalization of when UE-B generates and/or transmits an explicit request

It’s agreed in last meeting [1] that the inter-UE coordination triggered by UE-B’s explicit request in Scheme 1 could be enabled by resource pool level (pre-) configuration and one of the following alternatives could be configured:

- Alt 1: it is up to UE-B’s implementation whether or not to trigger the request generation

- Alt 2: the request generation can be triggered only when UE-B has data to be transmitted to UE-A

Scheme 1 based inter-UE coordination will be beneficial to optimize the selected resources for a UE. On the other hand, if all UE (even alt.2 with data transmission) are requesting inter-UE coordination, the signalling overhead will be extremely large in the system level. Hence, some more conditions should be defined for a UE-B to request the coordination information. From a straightforward point of view, higher layer configuration and a high traffic priority could be considered, i.e., when enabled by configuration or UE-B's traffic priority is below a configured threshold. On other aspects, the ratio of NACK could also be considered to improve PRR.

Proposal 1: UE-B generates and/or transmits an explicit request when enabled by configuration and

- Traffic priority is high enough or

- NACK ratio is high

Conclusion

In this contribution, we shared our views on the open issues regarding inter-UE coordination and proposed that:

Proposal 1: UE-B generates and/or transmits an explicit request when enabled by configuration and

- Traffic priority is high enough or

- NACK ratio is high

Reference

[1] Chairman's note of RAN1#107bis-e.