3GPP TSG RAN WG1 #108-e			R1-2201937

e-Meeting, February 21st – March 3rd, 2022

Source: 	     Xiaomi

Title:	Remaining issue on dynamic spectrum sharing

Agenda item:    8.13.3

Document for:  Discussion

Introduction

In RAN1#107e meeting, the efficient activation and de-activation mechanism for SCell was heatedly discussed. Good progress was achieved especially for the temporary RS triggering command. The following agreements on GAP determination between two RS bursts was achieved during last meeting:

Agreement For both Alt 1 and Alt 2 of temporary RS triggering, - For Alt 1, the gap between temporary RS bursts is explicitly configured. - A set of possible gap lengths from which the triggering MAC-CE can indicate one from RAN1 perspective. Up to RAN2 to decide details. - For Alt 2, a gap length is configured by RRC for each temporary RS having two bursts. For different temporary RS, the value of the gap length can be different based on RRC configuration. - the number of bursts is up to 2. It can be either explicitly configured, or implicitly indicated by the gap configuration ((Up to RAN2 to decide one)

In the current specification, there are some cases wherein symbols or slots is not available for temporary RS:

- UL symbol or UL slot configured by TDD UL DL configuration

- CRS from a co-existence LTE system

- SSB

In this contribution, we provide our views on collision between temporary RS and the invalid resources.

Discussion

In order to avoid potential conflicts with potential invalid symbols/slots, e.g. semi-static UL slots/symbols, symbols used for SSB/type0-CSS, the CRS symbols if LTE-NR co-existence is deployed, multiple GAP values need to be indicated. The key point for two bursts transmission/reception is that UE and gNB have a common understanding on the GAP between two indicated temporary RS bursts.

As mentioned above, there are several kinds of invalid symbol which are unavailable for temporary RS.

- UL symbol or UL slot configured by TDD UL DL configuration. The semi-static TDD UL-DL configuration can be obtained via SIB1 and/or UE dedicated RRC signalling. If temporary RS sample or temporary RS burst conflicts with semi-static UL symbols/slots, gNB cannot transmit it on those UL symbols.

- CRS from a co-existence LTE system. NR UE can recognize the CRS via RateMatchingPatternLTE-CRS. If the temporary RS burst conflicts with LTE-CRS, the performance of temporary RS would be degraded because of the strong interference from LTE CRS.

- SSB

It was agreed that the GAP value between two bursts can be indicated via the following mechanisms depending on how to design the SCell activation and TRS triggering MAC CE:

- For Alt 1, the gap between temporary RS bursts is explicitly configured.

- For Alt 2, a gap length is configured by RRC for each temporary RS having two bursts. For different temporary RS, the value of the gap length can be different based on RRC configuration

Although the GAP value between two bursts can be configured or indicated directly, there is possibility one of the TRS burst collides with the unavailable symbol or slots considering the complex scenarios in reality. Once the temporary RS conflicts with the unavailable symbol or slots, one straightforward mechanism is to postpone the conflicting temporary RS to the nearest available slots.  There are two benefits via this mechanism:

- Reduce the overhead on triggering temporary RS as less information is needed.

- Save the efforts at gNB side as it doesn’t need to consider the complicated issue between temporary RS and unavailable symbols.

Figure 1: Example on postponing the conflicting temporary RS burst to the available DL slots

Accordingly, we have the following proposal:

Proposal: When the temporary RS burst conflicts with the unavailable symbols, postpone the victim temporary RS burst to the nearest available slots.

Conclusion

In this contribution, we discuss the issues on collision between temporary RS and unavailable symbols.  Based on the discussion, our proposals are summarized as follows:

Proposal: When the temporary RS burst conflicts with the unavailable symbols, postpone the victim temporary RS burst to the nearest available slots.

Reference

  - R1-2109391, Discussion on efficient activation and de-activation mechanism for SCell in NR CA, Xiaomi

  - Chairman notes, RAN1#107 e-meeting