3GPP TSG RAN WG1 #108-e                                       	       R1-2201948

e-Meeting, February 21st – March 3rd, 2022

Agenda Item:	8.5.5

Source:	Xiaomi

Title:                     Remaining issues on potential enhancements for multipath/NLOS mitigation

Document for:	Discussion and Decision

Introduction

This contribution describes our view on the maintenance of enhancements on multipath/NLOS mitigation based on agreements in last RAN1 e-meetings.

Multipath/NLOS mitigation

TP for TS 38.214

The following is the text in the latest 38.214[1] in Rel-17, which describes the multipath reporting enhancements: support reporting relative timing (to the first detected path) in the measurement reports from UE to LMF for at least DL-TDOA and multi-RTT, for up to additional 8 paths. And since the DL PRS RSRPP means the DL PRS RSRP of a path, it is better to delete ‘path’. Thus we propose the following TP to 38.214.

- 5.1.6.5 PRS reception procedure … The UE may be configured to measure and report via higher layer parameter [AdditionalPath-relativeTiming-Request], subject to UE capability, the timing and the quality metrics of up to 8 additional detected paths, that are associated with each RSTD or UE Rx – Tx time difference. The timing of each additional path is reported relative to the path timing used for determining nr-RSTD or nr-UE-RxTxTimeDiff. For UE positioning measurement reporting in higher layer parameters NR-DL-TDOA-SignalMeasurementInformation or NR-Multi-RTT-SignalMeasurementInformation, the UE may be configured to measure and report, subject to UE capability, the path DL PRS RSRPP of the first path and the up to 8 additional paths that are associated with each RSTD or UE Rx – Tx time difference. …

Proposal 1: Adopt the following TP to 38.214

-------------------------- Start of Text Proposal for TS 38.214 -------------------------- <Unchanged parts omitted> - 5.1.5 5.1.6.5 PRS reception procedure … The UE may be configured to measure and report via higher layer parameter [AdditionalPath-relativeTiming-Request], subject to UE capability, the timing and the quality metrics of up to 8 additional detected paths, that are associated with each RSTD or UE Rx – Tx time difference. The timing of each additional path is reported relative to the path timing used for determining nr-RSTD or nr-UE-RxTxTimeDiff. For UE positioning measurement reporting in higher layer parameters NR-DL-TDOA-SignalMeasurementInformation or NR-Multi-RTT-SignalMeasurementInformation, the UE may be configured to measure and report, subject to UE capability, the path DL PRS RSRPP of the first path and the up to 8 additional paths that are associated with each RSTD or UE Rx – Tx time difference. … <Unchanged parts omitted> -------------------------- End of Text Proposal for TS 38.214 --------------------------

Conclusion

In this contribution, we discussed about remaining issues on enhancements of information reporting from UE and gNB for multipath/NLOS mitigation. Based on above discussion, we provide the following proposals.

Proposal 1: Adopt the following TP to 38.214

-------------------------- Start of Text Proposal for TS 38.214 -------------------------- <Unchanged parts omitted> - 5.1.5 5.1.6.5 PRS reception procedure … The UE may be configured to measure and report via higher layer parameter [AdditionalPath-relativeTiming-Request], subject to UE capability, the timing and the quality metrics of up to 8 additional detected paths, that are associated with each RSTD or UE Rx – Tx time difference. The timing of each additional path is reported relative to the path timing used for determining nr-RSTD or nr-UE-RxTxTimeDiff. For UE positioning measurement reporting in higher layer parameters NR-DL-TDOA-SignalMeasurementInformation or NR-Multi-RTT-SignalMeasurementInformation, the UE may be configured to measure and report, subject to UE capability, the path DL PRS RSRPP of the first path and the up to 8 additional paths that are associated with each RSTD or UE Rx – Tx time difference. … <Unchanged parts omitted> -------------------------- End of Text Proposal for TS 38.214 --------------------------

References

- 3GPP TS38.214 h00, V17.0.0 (2021-12)