3GPP TSG RAN WG1 #108e             	                                          R1-2201975

e-Meeting, February 21th – March 3rd, 2022

Agenda Item:		5

Source:			Samsung

Title:			Draft Reply to RAN2 LS on PDCCH Skipping in RRC_CONNECTED

Document for:		Discussion and decision

# Introduction

As requested in [1], RAN1 needs to provide response to RAN2’s questions about the impact of PDCCH skipping on SR and RA procedure.

# 2. Overall Description:

RAN2 has discussed the impact of PDCCH skipping on SR and RA procedure. Following agreements were made in RAN2#116bis-e:

- UE ignores PDCCH skipping while the SR is pending.

- If PDCCH skipping is applied to RNTI(s) monitored during RAR/MsgB window, the UE ignores PDCCH skipping during the RAR/MsgB window.

- UE ignores PDCCH skipping while contention resolution timer is running.

Question 1: RAN2 would like to know whether PDCCH skipping is applied to RNTI(s) monitored during RAR/MsgB window for RAR/MsgB reception.

For RAR/MsgB reception in NR, the following UE behaviors are supported:

- A) UE monitors PDCCH addressed to RA-RNTI/MsgB-RNTI in ra-SearchSpace for RAR/MsgB

- B) UE monitors PDCCH addressed to C-RNTI in a search space set associated with recoverySearchSpaceId for RAR/MsgB triggered for BFR

Based on current design in RAN1, PDCCH skipping is applied to USS and Type3-PDCCH CSS. So, A) is not impacted by PDCCH skipping. However, B) will be impacted as recoverySearchSpaceId can be configured for a USS set. Additional RAN1 specification work is needed to support UE ignores PDCCH skipping for RAR reception during BFR process per RAN2 agreement.

RAN1 Response 1: Based on current design in RAN1, PDCCH skipping is applied to C-RNTI monitored in a USS set associated with recoverySearchSpaceId for RAR triggered for BFR. Additional RAN1 specification work is needed to support ignoring of PDCCH skipping for RAR reception as per RAN2 agreement.

Question 2: RAN2 would like to know if RAN1 prefers to capture the above agreements in RAN1 specification or prefer these to be captured in MAC specification.

Since the agreements relates PDCCH monitoring behavior in respective search space sets. RAN1 prefer to capture them in RAN1 specification.

RAN1 Response 2: RAN1 prefers to capture the above agreements in RAN1 specification.

Question 3: RAN2 would like to ask RAN1 whether a) Physical layer of UE reports a value of 1 for Wake-up indication bit to higher layer or b) Physical layer of UE does not report Wake-up indication bit to higher layer, in case UE cannot monitor DCP due to PDCCH skipping; or if Approach 3 can be assumed.

RAN1 Response 3: PDCCH skipping only applies in Active Time and hence DCP cannot be missed due to PDCCH skipping.

Question 4: RAN2 would like to know whether UE should continue transmitting CSI/SRS during the PDCCH skipping duration or not?

PDCCH monitoring adaptation has no impact to UE behavior regarding CSI-RS reception or SRS transmission. UE should continue transmitting/receiving periodic/semi-static CSI-RS/SRS during PDCCH skipping duration.

RAN1 Response 4: UE should continue transmitting/receiving periodic/semi-static CSI-RS/SRS during PDCCH skipping duration.

# 6. Reference

[1] R1-2200884, LS on PDCCH Skipping in RRC_CONNECTED, 3GPP TSG-RAN WG1 Meeting #108e