3GPP TSG RAN WG1 #108                      	                                      R1-2202025

e-Meeting, February 21th – March 3rd, 2022

Agenda Item:		8.7.2

Source:			Samsung

Title:			Maintenance on DCI-based power saving techniques

Document for:		Discussion and decision

# Introduction

In RAN1#107bis e-meeting [1], the following were agreed:

Agreement

Remove the bracket from the following agreement made in RAN1#106bis-e

Agreement (extracted from RAN1#106bis-E)

- For Case 3 (i.e., 3 SSSG switching) , the following is supported

  - 2-bit in scheduling DCI is supported to indicate PDCCH monitoring adaptation UE behaviors

    - ‘00’ is Beh 2

    - ‘01’ is Beh 2A

    - ‘10’ is Beh 2B

    - [‘11’ is reserved]

Agreement

Confirm the following working assumption (extracted from RAN1#107-bis agreements)

Agreement

- For value X in Beh 1A, candidate skipping values are

  - Up to [100ms] length is supported,

    - The X is configured and indicated in the unit of slot.

      - Working assumption for candidate values for X

        - {1,2,3,…,20,30, 40, 50, 60, 80, 100} for 15 kHz SCS,

        - {1,2,3,…,40, 60, 80, 100, 120,160,200} for 30 kHz SCS,

        - {1,2,3,…,80, 120, 160, 200, 240, 320,400} for 60kHz SCS,

        - {1,2,3,…,160, 240, 320,400, 480, 640,800} for 120kHz SCS

  - FFS: Equal to or longer than the applicable minimum scheduling offset

  - FFS: additional symbol level / PDCCH monitoring period level skipping duration

Agreement

- The initial timer value for switching from SSSG#2 to SSSG#0 and from SSSG#1 to SSSG#0 is common and configured per BWP.

- The PDCCHSkippingDurationList-r17 is configured per BWP.

Working Assumption

- The bit length of the candidate skipping values and SSSG switching initial timer values in slots for 480kHz and 960kHz SCS are assumed to be the same as that for 120KHz SCS.

Agreement

PDCCH monitoring adaptation is only applied to the individual scheduling cell (including self-scheduling and cross-carrier scheduling)

Conclusion

For Rel-17 PDCCH monitoring adaptation, the case where single DCI indicates monitoring adaptation for multiple cells is not supported.

Agreement

For PDCCH monitoring adaptation case 1, support at most M = 3 for PDCCH skipping with 2 bits indication.

Agreement

The configuration of PDCCH monitoring adaptation is common for DCI format x-1 and x-2, neither Alt 1 nor Alt 2 is agreed,

- Alt 1: Support RRC parameter configured for the PDCCH monitoring adaptation indication field presence/absence in DCI format x-2.

- Alt 2: The configuration of PDCCH monitoring adaptation is common for DCI format x-1 and x-2, except skipping duration for DCI format x-2 (i.e., PDCCHSkippingDurationList-r17-x-2 for DCI format x-2). The PDCCH monitoring adaptation indication field for DCI format x-2 is dependent on PDCCHSkippingDurationList-r17-x-2 if configured.

Conclusion

No consensus to introduce non-scheduling DCI based PDCCH monitoring adaptation for Rel-17.

In this contribution, we discuss remaining issues to support DCI-based PDCCH monitoring adaptation including both SSSG switching and PDCCH skipping.

# Applicable search space sets

According to the discussion in RAN1#107-bis [2], the views on applicable search space sets for adaptation of PDCCH skipping is still controversial. A remaining issue is whether or not PDCCH skipping can impact monitoring of Type0/0A/1/2 CSS.

We are not convinced to support PDCCH skipping on search space sets or RNTIs other than USS sets and Type3-PDCCH CSS sets.  The additional power saving gain by applying PDCCH skipping to C-RNTI, MCS-C-RNTI, or CS-RNTI in Type0/0A/1/2 CSS is negligible, because UE may still monitor other RNTIs in the Type0/0A/1/2 CSS and be wake-up during the configured PDCCH MOs. More importantly, PDCCH scrambled by C-RNTI, MCS-C-RNTI or CS-RNTI in Type 0/1/1A/2 CSS are necessary for BFR or radio link failure recovery in RRC_CONNECTED state. PDCCH monitoring adaptation for power saving gain shouldn’t impact UE procedure for maintain the link quality.

Conclusion #1:  Monitoring of PDCCH candidates in Type0/0A/1 or 2 PDCCH CSS is not affected by PDCCH monitoring adaptation.

- Adopt proposed TP #1 for TS 38.213

Proposed TP #1 for TS 38.213

## 10.4 Search space set group switching and skipping of PDCCH monitoring ============================= Unchanged Text Omitted =============================== A UE can be provided a set of durations by PDCCHSkippingDurationList for PDCCH monitoring in Type3-PDCCH CSS set or USS set on a serving cell and, if the UE is not provided searchSpaceGroupIdList-r17, a DCI format 0_1, and/or DCI format 1_1, and/or DCI format 0_2, and/or DCI format 1_2 that schedules a PUSCH transmission or a PDSCH reception can include a PDCCH monitoring adaptation field of 1 bit or of 2 bits. ============================= Unchanged Text Omitted ===============================

# Impact of PDCCH skipping on SR and RA procedure

According to the LS from RAN2 [3], the following agreements were made in RAN2#116bis-e regarding impact of PDCCH skipping on SR and RA procedure:

- UE ignores PDCCH skipping while the SR is pending.

- If PDCCH skipping is applied to RNTI(s) monitored during RAR/MsgB window, the UE ignores PDCCH skipping during the RAR/MsgB window.

- UE ignores PDCCH skipping while contention resolution timer is running.

In our view, they have impact on RAN1 PDCCH monitoring adaptation in terms of PDCCH skipping. In order to support ignoring PDCCH skipping for SR, a UE can start monitoring PDCCH at least for DCI formats scheduling a PUSCH transmission in corresponding USS set(s). Also, UE can monitor PDCCH for DCI formats scheduling a PDSCH reception when the DCI formats have a same size as DCI format scheduling a PUSCH transmission. Alternatively, UE can perform full PDCCH monitoring for all DCI formats after the UE transmits a positive SR. For the benefit of power saving gain, we support to restrict the impact of SR only for the UL transmission.

Proposal #1: UE cancels PDCCH skipping adaptation after transmission of a postive SR.

For the impact on RAR reception, the following UE behaviors are supported for RAR/MsgB reception in NR:

- A) UE monitors PDCCH addressed to RA-RNTI/MsgB-RNTI in ra-SearchSpace for RAR/MsgB

- B) UE monitors PDCCH addressed to C-RNTI in a search space set associated with recoverySearchSpaceId for RAR/MsgB triggered for BFR

Based on current design in RAN1, PDCCH skipping is applied to USS and Type3-PDCCH CSS. So, A) is not impacted by PDCCH skipping. However, B) will be impacted as recoverySearchSpaceId can be configured for a USS set. Additional RAN1 specification work is needed to support UE ignores PDCCH skipping for RAR reception during BFR process per RAN2 agreement.

While contention resolution timer is running, for contention resolution:

- UE monitors PDCCH addressed to TC-RNTI

- UE monitors PDCCH addressed to C-RNTI, if C-RNTI was included in Msg3.

Additional RAN1 specification work is needed to support UE ignores PDCCH skipping for contention resolution addressed to C-RNTI in USS per RAN2 agreement.

Proposal #2: UE cancels PDCCH skipping adaptation after transmission of PRACH.

# UE behavior associated with searchSpaceSwitchTimer-r17 Timer

According to the discussion in RAN1#107bis, we need to resolve the remaining regarding UE behavior about searchSpaceSwitchTimer-r17 timer. The FL proposal [2] based on the latest discussion is copied in box below.

#### [High] Proposal 3-1 (v1) – Timer behavior for discussion If a UE is provided group indexes for a Type3-PDCCH CSS set or a USS set by searchSpaceGroupIdList-r17 and a timer value by searchSpaceSwitchTimer-r17 for PDCCH monitoring on a serving cell and the timer is running, the UE - resets the timer after a slot of the active DL BWP of the serving cell when the UE detects a DCI format in a PDCCH reception in the slot - Alt 2a: for the Type3-PDCCH CSS set or the USS set with group index of either 1 or 2 - Alt 2a-modified: with CRC scrambled by C-RNTI/CS-RNTI/MCS-C-RNTI in a PDCCH reception in the slot for the Type3-PDCCH CSS set or the USS set with group index of either 1 or 2 - Alt 2b: for the Type3-PDCCH CSS set or the USS set - Alt 2c: with CRC scrambled by C-RNTI/CS-RNTI/MCS-C-RNTI - otherwise, - Alt 3a: decrease the timer value by one after each slot. - Alt3a-modified: decrease the timer value by one after each slot, if UE does not apply PDCCH skipping - Alt 3c: decrements the timer after a slot of an active DL BWP of the serving cell when the UE does not detect a DCI format in a PDCCH reception in the slot for Type3-PDCCH CSS set or the USS set with group index of either 1 or 2 When the timer expires in a slot: - Alt 1a: the UE monitors PDCCH on the serving cell according to search space sets with group index 0 - Alt 1b: - If the UE has not been indicated skipping PDCCH monitoring for a duration overlapping in time with the slot, the UE monitors PDCCH on the serving cell according to search space sets with group index 0; - If the UE has been indicated skipping PDCCH monitoring for a duration overlapping in time with the slot, the UE starts to monitor PDCCH until the completion of the PDCCH skipping for the duration on the serving cell according to search space sets with group index 0.

Since the timer, searchSpaceSwitchTimer-r17, triggers PDCCH monitoring associated with SSSG#1 or SSSG#2, the reset of the timer should be triggered when UE receives any new indication associated with SSSG #1 or SSSG#2. Therefore, we support Alt 2a, where UE resets the timer after a slot of the active DL BWP of the serving cell when the UE detects a DCI format in a PDCCH reception in the slot. For decreasing timer, we think Alt3a and 3c is same when combined with Alt2a.

Proposal #3:  Down-select Alt 2a for resetting the timer value by searchSpaceSwitchTimer-r17.

- Adopt proposed TP #2 for TS 38.213

Proposed TP #2 for TS 38.213

## 10.4 Search space set group switching and skipping of PDCCH monitoring ============================= Unchanged Text Omitted =============================== If a UE is provided group indexes for a Type3-PDCCH CSS set or a USS set by searchSpaceGroupIdList-r17 and a timer value by searchSpaceSwitchTimer-r17 for PDCCH monitoring on a serving cell and the timer is running, the UE - decrements the timer after a slot of an active DL BWP of the serving cell when the UE does not detect a DCI format in a PDCCH reception in the slot for Type3-PDCCH CSS set or the USS set with group index of either 1 or 2 - resets the timer after a slot of the active DL BWP of the serving cell when the UE detects a DCI format in a PDCCH reception in the slot for Type3-PDCCH CSS set or the USS set with group index of either 1 or 2 ============================= Unchanged Text Omitted ===============================

Another remaining issue is UE behaviour when the timer expires. The following two alternatives were discussed in last RAN1 meeting according to FL summary [2].

When the timer expires in a slot

- Alt 1a: the UE monitors PDCCH on the serving cell according to search space sets with group index 0

- Alt 1b:

  - If the UE has not been indicated skipping PDCCH monitoring for a duration overlapping in time with the slot, the UE monitors PDCCH on the serving cell according to search space sets with group index 0;

  - If the UE has been indicated skipping PDCCH monitoring for a duration overlapping in time with the slot, the UE starts to monitor PDCCH until the completion of the PDCCH skipping for the duration on the serving cell according to search space sets with group index 0.

In our understanding, when the expiration of the timer and reception of PDCCH skipping indication happen in the same time slot, both SSSG switching and PDCCH skipping can be executed by UE without issue. The dynamic indication of PDCCH skipping should be executed first; otherwise the effort from gNB to provide PS singling is wasted. UE can monitor SSSG#0 after wake up from PDCCH skipping.

Proposal #4:  Down-select Alt 1b for UE behavior when the searchSpaceSwitchTimer-r17 timer expires in a slot.

# Application delay

According FL summary from RAN1#107bis-e meeting, the following proposal summarizes the latest discussion on application delay for PDCCH monitoring adaptation.

[High] Proposal 5-1 (v6) The following application delay is considered for baseline - Upon detecting a scheduling DCI format 1-1/1-2/0-1/0-2 indicating PDCCH skipping (i.e., Beh 1A), select one of the following schemes o Alt 1a: the UE applies Beh 1A on the serving cell at the first slot after the last OFDM symbol of the PDCCH transmission. o Alt 1b: the UE applies Beh 1A on the serving cell at the first slot after the application delay needed for PDCCH processing for Rel-16 minimum application delay for K0min/K2min indication is reused/extended, e.g., the indication is applied in the next Zµ slot, where Definition of Zµ is described in Table 5.3.1-1 in TS38.214 o Alt 1c: For PDCCH skipping indication via 0-1/0-2(/1-1/1-2), the UE applies Beh 1A on the serving cell after an RRC configured delay for UL (DL). - Upon detecting a scheduling DCI format 1-1/1-2/0-1/0-2 indicating SSSG switching (i.e., Beh 2/2A/2B), select one of the following schemes o Alt 1a: the UE applies SSSG switching on the serving cell at a first slot that is at least Pswitch symbols after the last symbol of the PDCCH o FFS application delay when cross-slot scheduling adaptation are jointly configured o Alt 1b: the UE applies SSSG switching on the serving cell at a first slot that is after the application delay needed for PDCCH processing for Rel-16 minimum application delay for K0min/K2min indication is reused/extended, e.g., the indication is applied in the next Zµ slot, where Definition of Zµ is described in Table 5.3.1-1 in TS38.214 o Alt 1c: the UE applies SSSG switching on the serving cell [TBD: Application delay] after the last OFDM symbol of HARQ-ACK transmission/PUSCH transmission - Upon SSSG timer exipry, UE performs Beh 2 at a first slot that is at least Pswitch symbols after a slot where the timer expires - FFS value of Pswitch, e.g., Table 10.4-1 in TS38.213 for search-space group switching Select from below how to handle the case of PDCCH skipping interaction with HARQ retransmission, - Alt 0: No additional UE behavior is specified. - Alt 1: During PDCCH skipping duration, the UE still performs PDCCH monitoring (i.e., Beh 1) when drx-RetransmissionTimerDL/UL is running - Alt 2: For PDCCH skipping indication via 1-1/1-2, If the UE fails to decode the associated PDSCH and/or transmit a NACK, UE performs Beh 1 (i.e., the PDCCH skipping is stopped) at the first slot after the last OFDM symbol of the NACK transmission. - Alt 4: the UE applies Beh 1A next slot after the last OFDM symbol of ACK transmission, otherwise the indication is not applied. - Other alternatives are not precluded - It is up to gNB to configure UE to follow the additional UE behaviour for PDCCH monitoring adaptation for PDCCH skipping interaction with HARQ retransmission

We are generally OK with the framework to consider a common design for both SSSG switching and PDCCH skipping.  For the first part of the proposal, we think a common design of applicable delay can be applicable to both DCI format x_1 and DCI format x_2 based PDCCH monitoring adaptation. Since the application delay is determined mostly by decoding time of PDCCH. According to Rel-16 PS work, it’s the common understanding that the PDCCH processing time is impacted by K0min/K2min. Therefore, we support Alt1b for determining the application delay for both PDCCH skipping and SSSG switching regardless of the triggering DCI format. Similar as Rel-16 cross-slot scheduling adaptation, UE should be able to receive scheduling PDCCH within an application delay. To avoid any ambiguity, UE doesn’t expect to receive different PDCCH adaptation indication during the application delay.

Proposal 5: For PDCCH monitoring adaptation indication, reuse Rel-16 minimum application delay for K0min/K2min indication.

- UE can continue receiving scheduling PDCCH during the application delay of a PDCCH monitoring adaptation indication, but the UE doesn’t expect to different PDCCH monitoring adaptation indication during the application delay.

- Adopt proposed TP #3 for 38.213

Proposed TP #3 for TS 38.213

## 10.4 Search space set group switching and skipping of PDCCH monitoring ============================= Unchanged Text Omitted =============================== If a UE detects a DCI format that include a PDCCH monitoring adaptation indication in slot n, the indication shall be applied in slot n + X of scheduling cell. The application delay, X, is determined in Clause 5.3.1 of TS 38. The UE does not expect to be scheduled with a DCI format with PDCCH monitoring adaptation indication field indication a different value of PDCCH monitoring adaptation indication for the same active BWP before slot n + X of the scheduling cell.

For the second part about impact of PDCCH skipping on retransmission. We support additional UE behavior to avoid delay of retransmission caused by PDCCH skipping. To avoid unnecessary scheduling delay, UE should continue monitoring PDCCH for scheduling DCI before applying the PDCCH skipping indication. The additional UE behavior should be supported based on retransmission timer from higher layer without additional RRC signaling.

Proposal #6: In the case of PDCCH skipping interaction with HARQ retransmission, support

- During PDCCH skipping duration, the UE still performs PDCCH monitoring (i.e., Beh 1) when drx-RetransmissionTimerDL/UL is running

# Conclusion

This contribution discussed remaining issue for supporting PDCCH monitoring adaptation within DRX active time. The following observation and proposals were made:

Conclusion #1:  Monitoring of PDCCH candidates in Type0/0A/1 or 2 PDCCH CSS is not affected by PDCCH monitoring adaptation.

- Adopt proposed TP #1 for TS 38.213

Proposal #1: UE cancels PDCCH skipping adaptation after transmission of a postive SR.

Proposal #2: UE cancels PDCCH skipping adaptation after transmission of PRACH.

Proposal #3:  Down-select Alt 2a for resetting the timer value by searchSpaceSwitchTimer-r17.

- Adopt proposed TP #2 for TS 38.213

Proposal #4:  Down-select Alt 1b for UE behavior when the searchSpaceSwitchTimer-r17 timer expires in a slot.

Proposal 5: For PDCCH monitoring adaptation indication, reuse Rel-16 minimum application delay for K0min/K2min indication.

- UE can continue receiving scheduling PDCCH during the application delay of a PDCCH monitoring adaptation indication, but the UE doesn’t expect to different PDCCH monitoring adaptation indication during the application delay.

- Adopt proposed TP #3 for 38.213

Proposal #6: In the case of PDCCH skipping interaction with HARQ retransmission, support

- During PDCCH skipping duration, the UE still performs PDCCH monitoring (i.e., Beh 1) when drx-RetransmissionTimerDL/UL is running

# References

- [1] RAN1 Chairman’s Note, 3GPP TSG RAN WG1 Meeting #107-e

- [2] R1-2112878,“Final FL summary of DCI-based power saving adaptation”,

- [3] R1-2200884, LS on PDCCH Skipping in RRC_CONNECTED, 3GPP TSG-RAN WG1 Meeting #108e