3GPP TSG RAN WG1 #108-e		R1-2202049

e-Meeting, Feb. 21st – Mar. 3rd, 2022

Title:	[draft] Reply LS on interruption for PUCCH SCell activation in invalid TA case

Response to:	R1-2200871 (R4-2120420)

Release:	Release 17

Work Item:	NR_RRM_enh2-Core

Source:	MediaTek, [RAN1]

To:	RAN4

Cc:	RAN2

Contact Person:

Name: Chi-Hsuan Hsieh

E-mail Address:	CH.Hsieh@mediatek.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

RAN1 thanks RAN4 for the LS and sharing the latest progress in RAN4 on interruption for PUCCH SCell activation in invalid TA case. Regarding RAN4’s questions to RAN1, the following includes RAN1’s answers.

- Q1: Can UE capability diffNumerologyAcrossPUCCH-Group defined in TS 38.306 be used to allow indication of UE support of parallel transmission of PRACH on PUCCH SCell and physical channels/signals with different SCS on other activated serving cells?

  - If YES, RAN4 would like to respectfully ask RAN2 to clarify that diffNumerologyAcrossPUCCH-Group applies to PRACH in the corresponding “Definitions for parameters” in TS 38.306.

  - If NO, are there any other existing capabilities that can serve this purpose? If so, RAN4 would like RAN1 to indicate the capabilities.

- Answer: As stated in TS 38.306, the UE capability diffNumerologyAcrossPUCCH-Group indicates whether UE supports different numerology across two NR PUCCH groups for data and control channel. It cannot be used to allow indication of UE support of parallel transmission of PRACH on PUCCH SCell and physical channels/signals with different SCS on other activated serving cells.

For inter-band CA, the UE capability parallelTxPRACH-SRS-PUCCH-PUSCH defined in 38.306 indicates whether the UE supports parallel transmission of PRACH and SRS/PUCCH/PUSCH across CCs in an inter-band CA band combination, irrespective of SCS configurations.

- Q2: If the answer in Q1 is NO, and there is no capabilities that can serve the purpose, what would be the expected UE behavior when a target PUCCH SCell PRACH has a different SCS from SpCell/SCell physical channels/signals according to RAN1/2 spec? e.g. Shall UE be able to simultaneously transmit PRACH on PUCCH SCell and physical channels/signals on other activated serving cells irrespective of SCS configurations?

- Answer: For inter-band CA case, the answer is provided in Q1. For intra-band CA case, according to 38.213 8.1:

  - “For single cell operation or for operation with carrier aggregation in a same frequency band, a UE does not transmit PRACH and PUSCH/PUCCH/SRS in a same slot or when a gap between the first or last symbol of a PRACH transmission in a first slot is separated by less than N symbols from the last or first symbol, respectively, of a PUSCH/PUCCH/SRS transmission in a second slot where N=2 for μ=0 or μ=1, N=4 for μ=2 or μ=3, and μ is the SCS configuration for the active UL BWP.”

It can be seen that UE does not support parallel transmission of PRACH on PUCCH SCell and physical channels/signals on other activated serving cells for intra-band CA, irrespective of SCS configurations.

2. Actions:

To RAN4:

RAN1 kindly asks RAN4 to take the above response into account for the corresponding future works.

3. Date of Next RAN1 Meetings:

TSG-RAN WG1 Meeting 109e 	16 May – 27 May 2022        		e-Meeting