3GPP TSG RAN WG1 #108-e			R1-2202054

e-Meeting, Feb. 21st – Mar. 3rd, 2022

Agenda item:	8.16.13

Source: 	MediaTek Inc.

Title: 	On UE features for DSS

Document for:	Discussion and decision

- Introduction

This paper provides our views on Rel-17 UE features in R1-2200781 (Updated RAN1 UE features list for Rel-17 NR after RAN1 #107bis-e) for DSS. Detailed proposals are described in the following sections.

- Views on UE features for DSS

In the Rel-17 DSS WID (RP-211345), it is noted that:

This work item is limited to FR1, and includes the following objectives for NR Dynamic Spectrum Sharing (DSS):

- PDCCH enhancements for cross-carrier scheduling including [RAN1, RAN2]

  - PDCCH of SCell scheduling PDSCH or PUSCH on P(S)Cell

- Note: The total PDCCH blind decoding budget should not be changed as a result of this work

- Note: These enhancements are not specific to DSS and are generally applicable to cross-carrier scheduling in carrier aggregation

Since the feature of “SCell scheduling PDSCH or PUSCH on P(S)Cell” is not specific to DSS and is generally applicable to cross-carrier scheduling in carrier aggregation, we think the following scenarios should be separately reported for both Type A and Type B UE:

- lower-numerology sSCell scheduling higher-numerology P(S)Cell

- higher-numerology sSCell scheduling lower-numerology P(S)Cell

- same numerology between sSCell and P(S)Cell

Furthermore, it is agreed in RAN1 #106-bis-e that:

Agreement

- When P(S)Cell SCS (μ) is larger than sSCell SCS (μ1), for CCS from sSCell to P(S)Cell and, it is not supported Rel-17 DSS.

It can be seen that “lower-numerology sSCell scheduling higher-numerology P(S)Cell” is not supported in Rel-17 DSS.

Observation 1: Since the feature of “SCell scheduling PDSCH or PUSCH on P(S)Cell” is not specific to DSS and is generally applicable to cross-carrier scheduling in carrier aggregation, and “lower-numerology sSCell scheduling higher-numerology P(S)Cell” is not supported in Rel-17 DSS as RAN1 #106-bis-e agreement, the following scenarios should be separately reported for both Type A and Type B UE:

- higher-numerology sSCell scheduling lower-numerology P(S)Cell

- same numerology between sSCell and P(S)Cell

- both

Proposal 1:

Adopt the following candidate values for FG 34-1 and FG 34-2

- One or more of supported SCS combinations ({P(S)Cell SCS in kHz, sSCell SCS in kHz}) from following set are indicated by the UE: {15,15}, {15,30}, (15, 60), {30,30}, {30,60},{60,60})

Besides, for #unicast DCI limits for PCell/PSCell scheduling (DCI processing capabilities), the related discussions are still pending in the main session, so they should be determined after the discussions are finished in the main session.

Proposal 2: For #unicast DCI limits for PCell/PSCell scheduling (DCI processing capabilities), the related discussions are still pending in the main session, so they should be determined after the discussions are finished in the main session.

# Proposed changes on current RAN1 UE features table

# 34. NR_DSS

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 34. NR_DSS | 34-2 | Cross-carrier scheduling from SCell to PCell/PSCell (Type B) | [Support of Cross-carrier scheduling (CCS) from sSCell to PCell/PSCell (Type B)] - Cross-carrier scheduling from sSCell to PCell/PSCell with CIF - sSCell USS set(s) (for CCS from sSCell to PCell/PSCell) and search space sets on PCell/PSCell can be configured so that the UE monitors them in overlapping [slot/symbol] of PCell/PSCell and sSCell - Configuration of scaling factor α for BD and CCE limit handling and PDCCH overbooking handling on P(S)Cell - FFS: #unicast DCI limits for PCell/PSCell scheduling - Processing one unicast DCI scheduling DL on PCell/PSCell per PCell/PSCell slot and its aligned N consecutive sSCell slot(s) - Processing one unicast DCI scheduling UL on PCell/PSCell per PCell/PSCell slot and its aligned N consecutive sSCell slot(s) - N is based on pair of (PCell/PSCell SCS, sSCell SCS): N=1 for(15,15), (30,30), (60,60) and N=2 for (15,30), (30,60) and N=4 for (15, 60) - Same numerology between sSCell and P(S)Cell or sSCell SCS is larger than P(S)Cell SCS - FFS: USS set(s) for DCI format 0_1,1_1,0_2,1_2 configured on sSCell for CCS from sSCell to PCell/PSCell - FFS: Support of sSCell deactivation/activation when sSCell cross carrier scheduling to PCell/PSCell is configured - FFS: Support of sSCell dormancy when sSCell cross carrier scheduling to PCell/PSCell is configured - FFS: PDCCH monitoring occasion(s) is within the first 3 OFDM symbols of a PCell/PSCell slot - FFS: Numbers of CORESET configurations and search space sets on sSCell (for PCell/PSCell cross-carrier scheduling) - FFS: frame boundary alignment between PCell/PSCell and sSCell - FFS: Precoder granularity of REG-bundle size when CCS from sSCell to PCell/PSCell is configured Note: The SCell configured with Cross-carrier scheduling to PCell/PSCell is referred to as ‘sSCell’ | 6-5 [, 34-1] | Yes |  |  | Per BC | No | Applicable to FR1 only |  | [Candidate value set 1: One or more of supported SCS combinations ({P(S)Cell SCS in kHz, sSCell SCS in kHz}) from following set are indicated by the UE: {15,15}, {15,30}, (15, 60) for N=4, {30,30}, {30,60},{60,60}) Candidate value set 2: frequency band pair(s) for {PCell/PSCell, sSCell}] Note: The CCS from sSCell to Pcell is applicable to FR1 only but there can be other Scells in FR2 configured for the UE | Optional with capability signalling |
| 34. NR_DSS | 34-1 | Cross-carrier scheduling from SCell to PCell/PSCell [with search space restrictions] (Type A) | Support of Cross-carrier scheduling from sSCell to PCell/PSCell [with search space restrictions] (Type A) - Cross-carrier scheduling from sSCell to PCell/PSCell with CIF - FFS: sSCell USS set(s) (for CCS from sSCell to PCell/PSCell) and at least following search space sets on PCell/PSCell can only be configured such that UE does not monitor them in same [slot/symbol] of PCell/PSCell and sSCell - USS sets for DCI formats 0_1,1_1,0_2,1_2 (if supported) - USS sets for DCI formats 0_0,1_0 - Type3-CSS set(s) for DCI formats 1_0/0_0 with C-RNTI/CS-RNTI/MCS-C-RNTI - FFS: BD limit handling and any configuration of associated parameters and UE reporting of any associated parameters - FFS: #unicast DCI limits for PCell/PSCell scheduling - Processing one unicast DCI scheduling DL on PCell/PSCell per PCell/PSCell slot and its aligned N consecutive sSCell slot(s) - Processing one unicast DCI scheduling UL on PCell/PSCell per PCell/PSCell slot and its aligned N consecutive sSCell slot(s) - N is based on pair of (PCell/PSCell SCS, sSCell SCS): N=1 for(15,15), (30,30), (60,60) and N=2 for (15,30), (30,60) and N=4 for (15, 60) - Same numerology between sSCell and P(S)Cell or sSCell SCS is larger than P(S)Cell SCS - FFS: USS set(s) for DCI format 0_1,1_1,0_2,1_2 configured on sSCell for CCS from sSCell to Pcell/PSCell - FFS: sSCell USS set(s) (for CCS from sSCell to Pcell/PSCell) and Type0/0A/1/2 CSS sets on Pcell/PSCell can be configured so that the UE monitors them in overlapping [slot/symbol] of Pcell/PSCell and sSCell. FFS overlap handling - FFS: Support of monitoring DCI formats 0_1,1_1,0_2,1_2 on PCell/PSCell USS set(s) - FFS: Support of sSCell deactivation/activation when sSCell cross carrier scheduling to PCell/PSCell is configured - FFS: Support of sSCell dormancy when sSCell cross carrier scheduling to PCell/PSCell is configured - FFS: PDCCH monitoring occasion(s) is within the first 3 OFDM symbols of a PCell/PSCell slot - FFS: Numbers of CORESET configurations and search space sets on sSCell (for PCell/PSCell cross-carrier scheduling) per BWP are 1 and 3, respectively - FFS: frame boundary alignment between PCell/PSCell and sSCell - FFS: Precoder granularity of REG-bundle size when CCS from sSCell to PCell/PSCell is configured | 6-5 | Yes |  |  | Per BC | No | Applicable to FR1 only |  | [Candidate value set 1: One or more of supported SCS combinations ({P(S)Cell SCS in kHz, sSCell SCS in kHz}) from following set are indicated by the UE: {15,15}, {15,30}, (15, 60) for N=4, {30,30}, {30,60},{60,60}) Candidate value set 2: frequency band pair(s) for {PCell/PSCell, sSCell}] Note: The CCS from sSCell to PCell is applicable to FR1 only but there can be other SCells in FR2 configured for the UE | Optional with capability signalling |

# 35. LTE_NR_DC_enh2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 35. LTE_NR_DC_enh2 | 35-1 | TRS RS for SCell activation | - TRS for SCell activation is aperiodic and triggered by MAC CE - Temporary RS is based on aperiodic TRS - Temporary RS is triggered within the BWP indicated by firstActiveDownlinkBWP-Id for the sSCell - A P-TRS of the to-be-activated Scell is indicated as a QCL source for the temporary RS in case of known Scell same as existing specification - FFS: Maximum number of temporary RS resource sets that can be configured to UE per CC {1 … 16} - FFS: Maximum number of temporary RS resource sets that can be configured to UE across CCs {1 … 256} - FFS: Maximum number of triggering states for temporary RS based Scell activation by a MAC-CE {1 … 64} - FFS: Maximum number of temporary RS resource sets that can be associated with a triggering state {1 … 16} - FFS: Support of temporary RS based SCell activation on one or more from {FR1 FDD, FR1 TDD, FR1 unlicensed, FR2} [Note: following are reported via the legacy feature, FG2-33 - Maximum number of configured NZP-CSI-RS resources per CC - Maximum total number of simultaneous NZP-CSI-RS resources in active BWPs across all CCs - Maximum number simultaneous NZP-CSI-RS resources per CC - Maximum total number of CSI-RS ports in simultaneous NZP-CSI-RS resources in active BWPs across all CCs] | 6-5 | Yes | N/A |  | [Per UE/Per BC/Per band] | [No/Yes] | [No/Yes] |  | [The NZP-CSI-RS configured as temporary RS for fast SCell activation are not considered when counting the maximum NZP-CSI-RS configurations of FG2-33] | Optional with capability signalling |