3GPP TSG RAN WG1 Meeting #108-e 	R1-2202075

e-Meeting, February 21st – March 3rd , 2022

Agenda Item: 8.16.2

Source: MediaTek Inc.

Title: 	Views on UE features for supporting NR from 52.6 GHz to 71 GHz

Document for: Discussion

# Introduction

This paper provides our views on Rel-17 UE features for supporting NR from 52.6 GHz to 71 GHz. Detailed proposals are described in the following sections and summarized in feature list table based on our proposals are presented in Section 3.

# Views on UE features for supporting NR from 52.6 GHz to 71 GHz

  - Views on UE features for initial access aspects

FG 24-1b and FG 24-4b Wideband PRACH

Whether the FG 24-1b and FG 24-4b should be included as basic FR2-2 UL FGs was discussed in RAN1 #107-e meeting. Some company mentioned that PRACH is a fundamental channel in initial access and supporting such feature as basic functionality is necessary. However, the motivation of introducing such FG is to comply with regulation, which varies based on different regions. Therefore, we prefer to allow UE to have the option on whether to support the FG based on different regulations.

Proposal 1: Modify FG 24-1b and FG24-4b as follows

| Features | Index | Feature group | Components | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- |
| 24. NR_ext_to_71GHz | 24-1b | Wideband PRACH for 120 kHz in FR2-2 [with/without shared spectrum channel access] | Enhanced PRACH design for operation by adopting a single long ZC sequence, with ZC sequence equal to 1151 for 120kHz and ZC sequence equal to 571 for 120kHz |  | Optional [with/without]capability signalling [Note: This FG is only supported in bands for shared spectrum operation] [A UE that supports 24-2 must indicate this FG is supported] |
| 24. NR_ext_to_71GHz | 24-4b | Wideband PRACH for 480 kHz in FR2-2 [with/without shared spectrum channel access] | PRACH with 480KHz and length 571 |  | Optional with capability signalling [Note: This FG is only supported in bands for shared spectrum operation] |

  - Views on UE features for PDCCH monitoring enhancements

FG 24-5f Enhanced PDCCH monitoring for 960kHz

There is a inconsistence between FG24-4f and FG24-5f on the FG naming where FR2-2 notion is missing in FG24-5f. We suggest to add such notion to align with FG24-4f.

Proposal 2: Update FG 24-5f as follows:

| Features | Index | Feature group | Components | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- |
| 24. NR_ext_to_71GHz | 24-5f | Enhanced PDCCH monitoring for 960KHz in FR2-2 | 1.) Multiple-slot PDCCH monitoring for 960KHz with (Xs,Ys)=(4,1) 2.) Multiple-slot PDCCH monitoring for 960KHz with (Xs,Ys)= (4,2) 1.) Multiple-slot PDCCH monitoring for 960KHz with (Xs,Ys)=(8,4) slots 2.) Within each of the Ys = 2 or 4 slots, monitoring of type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS in the first 3 OFDM symbols of each slot as in according to FG 3-1 (FFS: Monitoring capability within slots of type 1 CSS without dedicated RRC configuration and type0, 0A, and 2 CSS) |  | Optional |

  - Views on UE features for Enhancements for PUCCH formats 0/1/4

FGs for multi-RB PUCCH format 0/1/4

Similar to our comments on wideband PRACH, the multi-RB PUCCH FGs should be considered as optional FGs due to the different regulation requirements in different areas.

Proposal 3: Update FG 24-1c, FG24-4c, and FG24-5c as follows:

| Features | Index | Feature group | Components | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- |
| 24. NR_ext_to_71GHz | 24-1c | Multi-RB support PUCCH format 0/1/4 for 120 kHz in FR2-2 | 1. Support multi-RB PUCCH format 4 for 120 kHz 2. Support multi-RB PUCCH format 0/1 for 120 kHz |  | Optional with capability signalling [A UE that supports [24-1a/24-2/FR2-2] must indicate this FG is supported] This FG is only supported in bands under PSD limitation in shared spectrum operation |
| 24. NR_ext_to_71GHz | 24-4c | Multi-RB PUCCH format 0/1/4 for 480 kHz in FR2-2 | Support multi-RB PUCCH format 0/1/4 for 480 kHz |  | Optional with capability signalling This FG is only supported in bands under PSD limitation in shared spectrum operation |
| 24. NR_ext_to_71GHz | 24-5c | Multi-RB PUCCH format 0/1/4 for 960 kHz in FR2-2 | Support multi-RB PUCCH format 0/1/4 for 960 kHz |  | Optional with capability signalling This FG is only supported in bands under PSD limitation in shared spectrum operation |

  - Views on UE features for beam management for new SCSs

For the additional beam switching time delay FG, there was a discussion on the UE behavior when UE doesn’t signal to support the FG. Based on our understanding, the FG is used to specify which additional beam switching time delay UE needs among d=56 and d=112, instead of whether UE needs additional beam switching time delay for 480kHz or not. Therefore, we prefer to have d=112 as default value if UE didn’t signal such capability.

Proposal 4: Update FG24-10 as follows:

| 24. NR_ext_to_71GHz | 24-10 | Additional beam switching time delay | Supported additional beam switching time delay d=56 for 480 kHz SCS |  | Yes | N/A | Additional beam switching time delay d=112 is supported | Per UE | N/A | N/A | N/A | Candidate value set: 56 or 112 symbols | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

In RAN1 #106bis e meeting, the following agreement regarding m-TRP multi-PDSCH scheduling reception is achieved.

Agreement:

The working assumption in RAN1#106-e is confirmed with the following update:

For multi-PDSCH scheduling for multi-TRPs, support a single DCI field ‘Transmission Configuration Indication’ as in Rel-16 TCI state indication mechanism for multi-TRPs

- The single DCI field ‘Transmission Configuration Indication’ indicates one or two TCI states associated with a code point for single DCI based multi-TRP mechanism

  - When two TCI states are indicated, reuse Rel-16 association rules to apply the two TCI states for each PDSCH scheduled by a multi-PDSCH scheduling DCI

- The single DCI field ‘Transmission Configuration Indication’ indicates only one TCI state associated with a code point for multi-DCI based multi-TRP mechanism

- Reuse Rel-16 RRC configuration and MAC CE activation/deactivation methods for the one or two TCI states

- FFS: Details of multiple TCI state association with multiple PDSCHs

- Within the TDRA table for multi-PDSCH scheduling, the UE does not expect to be configured with the higher layer parameter repetitionNumber

To allow UE to support m-TRP single-PDSCH scheduling and only s-TRP multi-PDSCH scheduling, we suggest to introduce additional FGs for m-TRP multi-PDSCH scheduling.

Proposal 5: Add FGs for m-TRP multi-PDSCH scheduling as follows:

| Features | Index | Feature group | Components | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- |
| 24. NR_ext_to_71GHz | 24-1g | Single-DCI based SDM scheme multi-PDSCH DL grant for 120 kHz SCS in FR2-2 | - Support of single-DCI based SDM scheme for multi-PDSCH scheduling for 120kHz SCS in FR2-2 |  | Optional |
| 24. NR_ext_to_71GHz | 24-4g | Single-DCI based SDM scheme multi-PDSCH DL grant for 480kHz SCS in FR2-2 | - Support of single-DCI based SDM scheme for multi-PDSCH scheduling for 480kHz SCS in FR2-2 |  | Optional |
| 24. NR_ext_to_71GHz | 24-5g | Single-DCI based SDM scheme multi-PDSCH DL grant for 960kHz SCS in FR2-2 | - Support of single-DCI based SDM scheme for multi-PDSCH scheduling for 960kHz SCS in FR2-2 |  | Optional |
| 24. NR_ext_to_71GHz | 24-1h | Single-DCI based FDMSchemeA multi-PDSCH DL grant for 120 kHz SCS in FR2-2 | - Support of single-DCI based FDMSchemeA scheme for multi-PDSCH scheduling for 120kHz SCS in FR2-2 |  | Optional |
| 24. NR_ext_to_71GHz | 24-4h | Single-DCI based FDMSchemeA multi-PDSCH DL grant for 480kHz SCS in FR2-2 | - Support of single-DCI based FDMSchemeA scheme for multi-PDSCH scheduling for 480kHz SCS in FR2-2 |  | Optional |
| 24. NR_ext_to_71GHz | 24-5h | Single-DCI based FDMSchemeA multi-PDSCH DL grant for 960kHz SCS in FR2-2 | - Support of single-DCI based FDMSchemeA scheme for multi-PDSCH scheduling for 960kHz SCS in FR2-2 |  | Optional |
| 24. NR_ext_to_71GHz | 24-1i | Single-DCI based FDMSchemeB multi-PDSCH DL grant for 120 kHz SCS in FR2-2 | - Support of single-DCI based FDMSchemeB scheme for multi-PDSCH scheduling for 120kHz SCS in FR2-2 |  | Optional |
| 24. NR_ext_to_71GHz | 24-4i | Single-DCI based FDMSchemeB multi-PDSCH DL grant for 480kHz SCS in FR2-2 | - Support of single-DCI based FDMSchemeB scheme for multi-PDSCH scheduling for 480kHz SCS in FR2-2 |  | Optional |
| 24. NR_ext_to_71GHz | 24-5i | Single-DCI based FDMSchemeB multi-PDSCH DL grant for 960kHz SCS in FR2-2 | - Support of single-DCI based FDMSchemeB scheme for multi-PDSCH scheduling for 960kHz SCS in FR2-2 |  | Optional |
| 24. NR_ext_to_71GHz | 24-1j | Single-DCI based TDMSchemeA multi-PDSCH DL grant for 120 kHz SCS in FR2-2 | - Support of single-DCI based TDMSchemeA scheme for multi-PDSCH scheduling for 120kHz SCS in FR2-2 |  | Optional |
| 24. NR_ext_to_71GHz | 24-4j | Single-DCI based TDMSchemeA multi-PDSCH DL grant for 480kHz SCS in FR2-2 | - Support of single-DCI based TDMSchemeA scheme for multi-PDSCH scheduling for 480kHz SCS in FR2-2 |  | Optional |
| 24. NR_ext_to_71GHz | 24-5j | Single-DCI based TDMSchemeA multi-PDSCH DL grant for 960kHz SCS in FR2-2 | - Support of single-DCI based TDMSchemeA scheme for multi-PDSCH scheduling for 960kHz SCS in FR2-2 |  | Optional |

  - Views on UE features for PDSCH/PUSCH enhancements

FG for multi-PDSCH DL grant

We suggest to add separated FGs for the enhancements of both multi-PDSCH and multi-PUSCH scheduled by single DCI instead of including those FGs as basic FGs. We also suggest to add the notion of FR2-2 in this FG such that it can be differentiated from the existing multi-PUSCH feature introduced for Rel-16 NR-U and for FR2-1. Note that multi-PDSCH can’t be configured with legacy PDSCH repetition. Therefore, it is not desirable to include an enhanced feature as basic feature with the consequence of removing the legacy configuration.

Proposal 6: Remove multi-PDSCH scheduling from FG24-4 and FG24-5 and add FGs for multi-PDSCH scheduling as follows:

| Features | Index | Feature group | Components | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- |
| 24. NR_ext_to_71GHz | 24-4d | Multiple PDSCH scheduling by single DCI for 480 kHz in FR2-2 | - Multi- PDSCH scheduling by single DCI for the operation with 480 kHz SCS - HARQ enhancements |  | Optional |
| 24. NR_ext_to_71GHz | 24-5d | Multiple PDSCH scheduling by single DCI for 960 kHz in FR2-2 | - Multi- PDSCH scheduling by single DCI for the operation with 960 kHz SCS - HARQ enhancements |  | Optional |

Proposal 7: Remove multi-PUSCH scheduling from FG24-4a and FG24-5a and add FGs for multi-PUSCH scheduling as follows:

| Features | Index | Feature group | Components | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- |
| 24. NR_ext_to_71GHz | 24-4e | Multiple PUSCH scheduling by single DCI for 480 kHz in FR2-2 | - Multi- PUSCH scheduling by single DCI for the operation with 480 kHz SCS - HARQ enhancements |  | Optional |
| 24. NR_ext_to_71GHz | 24-5e | Multiple PUSCH scheduling by single DCI for 960 kHz in FR2-2 | - Multi- PUSCH scheduling by single DCI for the operation with 960 kHz SCS - HARQ enhancements |  | Optional |

# Conclusion

In summary, we have the following proposals for revising RAN1 UE feature list and the updated UE feature list table is provided accordingly. Revised parts are written in red.

Proposal 1: Modify FG 24-1b and FG24-4b as follows

| Features | Index | Feature group | Components | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- |
| 24. NR_ext_to_71GHz | 24-1b | Wideband PRACH for 120 kHz in FR2-2 [with/without shared spectrum channel access] | Enhanced PRACH design for operation by adopting a single long ZC sequence, with ZC sequence equal to 1151 for 120kHz and ZC sequence equal to 571 for 120kHz |  | Optional [with/without]capability signalling [Note: This FG is only supported in bands for shared spectrum operation] [A UE that supports 24-2 must indicate this FG is supported] |
| 24. NR_ext_to_71GHz | 24-4b | Wideband PRACH for 480 kHz in FR2-2 [with/without shared spectrum channel access] | PRACH with 480KHz and length 571 |  | Optional with capability signalling [Note: This FG is only supported in bands for shared spectrum operation] |

Proposal 2: Update FG 24-5f as follows:

| Features | Index | Feature group | Components | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- |
| 24. NR_ext_to_71GHz | 24-5f | Enhanced PDCCH monitoring for 960KHz in FR2-2 | 1.) Multiple-slot PDCCH monitoring for 960KHz with (Xs,Ys)=(4,1) 2.) Multiple-slot PDCCH monitoring for 960KHz with (Xs,Ys)= (4,2) 1.) Multiple-slot PDCCH monitoring for 960KHz with (Xs,Ys)=(8,4) slots 2.) Within each of the Ys = 2 or 4 slots, monitoring of type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS in the first 3 OFDM symbols of each slot as in according to FG 3-1 (FFS: Monitoring capability within slots of type 1 CSS without dedicated RRC configuration and type0, 0A, and 2 CSS) |  | Optional |

Proposal 3: Update FG 24-1c, FG24-4c, and FG24-5c as follows:

| Features | Index | Feature group | Components | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- |
| 24. NR_ext_to_71GHz | 24-1c | Multi-RB support PUCCH format 0/1/4 for 120 kHz in FR2-2 | 1. Support multi-RB PUCCH format 4 for 120 kHz 2. Support multi-RB PUCCH format 0/1 for 120 kHz |  | Optional with capability signalling [A UE that supports [24-1a/24-2/FR2-2] must indicate this FG is supported] This FG is only supported in bands under PSD limitation in shared spectrum operation |
| 24. NR_ext_to_71GHz | 24-4c | Multi-RB PUCCH format 0/1/4 for 480 kHz in FR2-2 | Support multi-RB PUCCH format 0/1/4 for 480 kHz |  | Optional with capability signalling This FG is only supported in bands under PSD limitation in shared spectrum operation |
| 24. NR_ext_to_71GHz | 24-5c | Multi-RB PUCCH format 0/1/4 for 960 kHz in FR2-2 | Support multi-RB PUCCH format 0/1/4 for 960 kHz |  | Optional with capability signalling This FG is only supported in bands under PSD limitation in shared spectrum operation |

Proposal 4: Update FG24-10 as follows:

| 24. NR_ext_to_71GHz | 24-10 | Additional beam switching time delay | Supported additional beam switching time delay d=56 for 480 kHz SCS |  | Yes | N/A | Additional beam switching time delay d=112 is supported | Per UE | N/A | N/A | N/A | Candidate value set: 56 or 112 symbols | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 5: Add FGs for m-TRP multi-PDSCH scheduling as follows:

| Features | Index | Feature group | Components | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- |
| 24. NR_ext_to_71GHz | 24-1g | Single-DCI based SDM scheme multi-PDSCH DL grant for 120 kHz SCS in FR2-2 | - Support of single-DCI based SDM scheme for multi-PDSCH scheduling for 120kHz SCS in FR2-2 |  | Optional |
| 24. NR_ext_to_71GHz | 24-4g | Single-DCI based SDM scheme multi-PDSCH DL grant for 480kHz SCS in FR2-2 | - Support of single-DCI based SDM scheme for multi-PDSCH scheduling for 480kHz SCS in FR2-2 |  | Optional |
| 24. NR_ext_to_71GHz | 24-5g | Single-DCI based SDM scheme multi-PDSCH DL grant for 960kHz SCS in FR2-2 | - Support of single-DCI based SDM scheme for multi-PDSCH scheduling for 960kHz SCS in FR2-2 |  | Optional |
| 24. NR_ext_to_71GHz | 24-1h | Single-DCI based FDMSchemeA multi-PDSCH DL grant for 120 kHz SCS in FR2-2 | - Support of single-DCI based FDMSchemeA scheme for multi-PDSCH scheduling for 120kHz SCS in FR2-2 |  | Optional |
| 24. NR_ext_to_71GHz | 24-4h | Single-DCI based FDMSchemeA multi-PDSCH DL grant for 480kHz SCS in FR2-2 | - Support of single-DCI based FDMSchemeA scheme for multi-PDSCH scheduling for 480kHz SCS in FR2-2 |  | Optional |
| 24. NR_ext_to_71GHz | 24-5h | Single-DCI based FDMSchemeA multi-PDSCH DL grant for 960kHz SCS in FR2-2 | - Support of single-DCI based FDMSchemeA scheme for multi-PDSCH scheduling for 960kHz SCS in FR2-2 |  | Optional |
| 24. NR_ext_to_71GHz | 24-1i | Single-DCI based FDMSchemeB multi-PDSCH DL grant for 120 kHz SCS in FR2-2 | - Support of single-DCI based FDMSchemeB scheme for multi-PDSCH scheduling for 120kHz SCS in FR2-2 |  | Optional |
| 24. NR_ext_to_71GHz | 24-4i | Single-DCI based FDMSchemeB multi-PDSCH DL grant for 480kHz SCS in FR2-2 | - Support of single-DCI based FDMSchemeB scheme for multi-PDSCH scheduling for 480kHz SCS in FR2-2 |  | Optional |
| 24. NR_ext_to_71GHz | 24-5i | Single-DCI based FDMSchemeB multi-PDSCH DL grant for 960kHz SCS in FR2-2 | - Support of single-DCI based FDMSchemeB scheme for multi-PDSCH scheduling for 960kHz SCS in FR2-2 |  | Optional |
| 24. NR_ext_to_71GHz | 24-1j | Single-DCI based TDMSchemeA multi-PDSCH DL grant for 120 kHz SCS in FR2-2 | - Support of single-DCI based TDMSchemeA scheme for multi-PDSCH scheduling for 120kHz SCS in FR2-2 |  | Optional |
| 24. NR_ext_to_71GHz | 24-4j | Single-DCI based TDMSchemeA multi-PDSCH DL grant for 480kHz SCS in FR2-2 | - Support of single-DCI based TDMSchemeA scheme for multi-PDSCH scheduling for 480kHz SCS in FR2-2 |  | Optional |
| 24. NR_ext_to_71GHz | 24-5j | Single-DCI based TDMSchemeA multi-PDSCH DL grant for 960kHz SCS in FR2-2 | - Support of single-DCI based TDMSchemeA scheme for multi-PDSCH scheduling for 960kHz SCS in FR2-2 |  | Optional |

Proposal 6: Remove multi-PDSCH scheduling from FG24-4 and FG24-5 and add FGs for multi-PDSCH scheduling as follows:

| Features | Index | Feature group | Components | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- |
| 24. NR_ext_to_71GHz | 24-4d | Multiple PDSCH scheduling by single DCI for 480 kHz in FR2-2 | - Multi- PDSCH scheduling by single DCI for the operation with 480 kHz SCS - HARQ enhancements |  | Optional |
| 24. NR_ext_to_71GHz | 24-5d | Multiple PDSCH scheduling by single DCI for 960 kHz in FR2-2 | - Multi- PDSCH scheduling by single DCI for the operation with 960 kHz SCS - HARQ enhancements |  | Optional |

Proposal 7: Remove multi-PUSCH scheduling from FG24-4a and FG24-5a and add FGs for multi-PUSCH scheduling as follows:

| Features | Index | Feature group | Components | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- |
| 24. NR_ext_to_71GHz | 24-4e | Multiple PUSCH scheduling by single DCI for 480 kHz in FR2-2 | - Multi- PUSCH scheduling by single DCI for the operation with 480 kHz SCS - HARQ enhancements |  | Optional |
| 24. NR_ext_to_71GHz | 24-5e | Multiple PUSCH scheduling by single DCI for 960 kHz in FR2-2 | - Multi- PUSCH scheduling by single DCI for the operation with 960 kHz SCS - HARQ enhancements |  | Optional |

The proposed FG table is summarized as follows:

| 24. NR_ext_to_71GHz | 24-1 | Basic FR2-2 DL support | 1. Support reception of 120kHz subcarrier spacing for DL data and control channels, SSB, and reference signals in FR2-2 for non-initial access |  |  |  | FR2-2 is not supported | per band |  |  |  |  | Optional with capability signalling [A UE that supports FR2-2 must indicate this FG is supported] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 24. NR_ext_to_71GHz | 24-1a | Basic FR2-2 UL support | 1. Support 120KHz SCS reception for non-initial access 2. Support multi-RB PUCCH format 0/1 for 120 kHz 3. PRACH with 120KHz SCS and length 139 4. Support transmission of 120kHz subcarrier spacing for UL data and control channels and reference signals in FR2-2 | 24-1 |  |  |  |  |  |  |  |  | Optional with capability signalling [A UE that supports FR2-2 must indicate this FG is supported] |
| 24. NR_ext_to_71GHz | 24-1b | Wideband PRACH for 120 kHz in FR2-2 | Enhanced PRACH design for operation by adopting a single long ZC sequence, with ZC sequence equal to 1151 for 120kHz and ZC sequence equal to 571 for 120kHz | 24-1a |  |  |  |  |  |  |  | [Note: This FG is only supported in bands for shared spectrum operation] [A UE that supports 24-2 must indicate this FG is supported] | Optional [with/without]capability signalling |
| 24. NR_ext_to_71GHz | 24-1c | Multi-RB support PUCCH format 0/1/4 for 120 kHz in FR2-2 | 1. Support multi-RB PUCCH format 4 for 120 kHz 2. Support multi-RB PUCCH format 0/1 for 120 kHz | 24-1a |  |  |  |  |  |  |  | [A UE that supports [24-1a/24-2/FR2-2] must indicate this FG is supported] This FG is only supported in bands under PSD limitation in shared spectrum operation | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-1d | Multiple PDSCH scheduling by single DCI for 120kHz | 1. Multi-PDSCH scheduling by single DCI for the operation with 120 kHz SCS 2. HARQ enhancements | 24-1 |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-1e | Multiple PUSCH scheduling by single DCI for 120kHz | 1. Multi-PUSCH scheduling by single DCI for the operation with 120 kHz SCS | 24-1a |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-1g | Single-DCI based SDM scheme multi-PDSCH DL grant for 120 kHz SCS in FR2-2 | Support of single-DCI based SDM scheme for multi-PDSCH scheduling for 120kHz SCS in FR2-2 |  |  |  |  |  |  |  |  |  |  |
| 24. NR_ext_to_71GHz | 24-1h | Single-DCI based FDMSchemeA multi-PDSCH DL grant for 120 kHz SCS in FR2-2 | Support of single-DCI based FDMSchemeA scheme for multi-PDSCH scheduling for 120kHz SCS in FR2-2 |  |  |  |  |  |  |  |  |  |  |
| 24. NR_ext_to_71GHz | 24-1i | Single-DCI based FDMSchemeB multi-PDSCH DL grant for 120 kHz SCS in FR2-2 | Support of single-DCI based FDMSchemeB scheme for multi-PDSCH scheduling for 120kHz SCS in FR2-2 |  |  |  |  |  |  |  |  |  |  |
| 24. NR_ext_to_71GHz | 24-1j | Single-DCI based TDMSchemeA multi-PDSCH DL grant for 120 kHz SCS in FR2-2 | Support of single-DCI based TDMSchemeA scheme for multi-PDSCH scheduling for 120kHz SCS in FR2-2 |  |  |  |  |  |  |  |  |  |  |
| 24. NR_ext_to_71GHz | 24-2 | 120KHz SSB support for SA/DCinitial access in FR2-2 | Support 120KHz SSB for SA/DCinitial access in FR2-2 | 24-1, 24-1a | N/A | N/A | 120KHz SSB based initial access in FR2-2 is not supported | per band |  | 24-1, 24-1a | N/A | N/A | 120KHz SSB based initial access in FR2-2 is not supported |

| 24. NR_ext_to_71GHz | 24-3 | 480KHz SSB support for initial access in FR2-2 | 1. Support 480KHz SSB for initial in FR2-2 | 24-2, 24-4, 24-4a | N/A | N/A | 480KHz SSB for initial access in FR2-2 is not supported | per band | N/A | N/A | N/A |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 24. NR_ext_to_71GHz | 24-4 | 480KHz SCS support for DL | 1. 480KH SCS for DL data and control channels, SSB, and reference signal reception in FR2-2 for non-initial access 2. Multiple-slot PDCCH monitoring for 480KHz with (Xs,Ys) = (4,1) FFS: 3. Multi- PDSCH scheduling by single DCI for the operation with 480 kHz SCS and corresponding HARQ enhancements 4. Within the Ys = 1 slot, monitoring of type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS with a maximum of two monitoring spans per slot with set2 = (4, 3) and (7, 3) symbols where set2 is defined in FG3-5b (FFS: Monitoring capability within slots of type 1 CSS without dedicated RRC configuration and type0, 0A, and 2 CSS) 5. Processing one unicast DCI scheduling DL and one unicast DCI scheduling UL per slot group of Xs slots per scheduled CC for FDD (This supersedes corresponding component of FG 3-5b) 6. Processing one unicast DCI scheduling DL and 2 unicast DCI scheduling UL per slot group of Xs slots per scheduled CC for TDD (This supersedes corresponding component of FG 3-5b) | 24-1 | Yes | N/A | 480KHz SCS for DL is not supported | Perband | N/A | N/A | N/A | FFS: component description without a reference to other R15 FGs | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-4a | 480KHz SCS support for UL | 1. PRACH with 480KHz and length 139 2. 480KHz SCS for UL data and control channels and reference signal transmission in FR2-2 3. Multi-PUSCH scheduling by single DCI for the operation with 480 kHz SCS | 24-1a, 24-4 | Yes | N/A | 480KHz SCS support for UL is not supported | Per band | N/A | N/A | N/A |  | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-4b | Wideband PRACH for 480 kHz in FR2-2 | PRACH with 480KHz and length 571 | 24-4a | Yes | N/A | Wideband PRACH for 480 kHz in FR2-2 is not supported | Per band | N/A | N/A | N/A | [Note: This FG is only supported in bands for shared spectrum operation] | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-4c | Multi-RB PUCCH format 0/1/4 for 480 kHz in FR2-2 | Support multi-RB PUCCH format 0/1/4 for 480 kHz | 24-4a | Yes | N/A | Multi-RB PUCCH format 0/1/4 for 480 kHz in FR2-2 is not supported | Per band | N/A | N/A | N/A | This FG is only supported in bands under PSD limitation in shared spectrum operation | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-4d | Multiple PDSCH scheduling by single DCI for 480 kHz | 1. Multi- PDSCH scheduling by single DCI for the operation with 480 kHz SCS 2. HARQ enhancements |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-4e | Multiple PUSCH scheduling by single DCI for 480 kHz | 1. Multi-PUSCH scheduling by single DCI for the operation with 480 kHz SCS |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-4f | Enhanced PDCCH monitoring for 480KHz in FR2-2 | 1. Multiple-slot PDCCH monitoring for 480KHz with (Xs,Ys) 2.) Within each of the Ys = 2 slots, monitoring of type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS in the first 3 OFDM symbols of each slot (FFS: Monitoring capability within slots of type 1 CSS without dedicated RRC configuration and type0, 0A, and 2 CSS) | 24-4 | Yes | N/A | Enhanced PDCCH monitoring for 480KHz in FR2-2 is not supported | Per band | N/A | N/A | N/A | Component 1 candidate values: [one or more of] {[(2,1),] (4,2) } Note: If (2,1) is not agreed, this FG will have no component candidate values and the component 1 description will be updated from (Xs,Ys) to (Xs,Ys)=(4,2) similar to FG 24-4 and 24-5 | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-4g | Single-DCI based SDM scheme multi-PDSCH DL grant for 480 kHz SCS in FR2-2 | Support of single-DCI based SDM scheme for multi-PDSCH scheduling for 480kHz SCS in FR2-2 |  |  |  |  |  |  |  |  |  |  |
| 24. NR_ext_to_71GHz | 24-4h | Single-DCI based FDMSchemeA multi-PDSCH DL grant for 480 kHz SCS in FR2-2 | Support of single-DCI based FDMSchemeA scheme for multi-PDSCH scheduling for 480kHz SCS in FR2-2 |  |  |  |  |  |  |  |  |  |  |
| 24. NR_ext_to_71GHz | 24-4i | Single-DCI based FDMSchemeB multi-PDSCH DL grant for 480 kHz SCS in FR2-2 | Support of single-DCI based FDMSchemeB scheme for multi-PDSCH scheduling for 480kHz SCS in FR2-2 |  |  |  |  |  |  |  |  |  |  |
| 24. NR_ext_to_71GHz | 24-4j | Single-DCI based TDMSchemeA multi-PDSCH DL grant for 480 kHz SCS in FR2-2 | Support of single-DCI based TDMSchemeA scheme for multi-PDSCH scheduling for 480kHz SCS in FR2-2 |  |  |  |  |  |  |  |  |  |  |

| 24. NR_ext_to_71GHz | 24-5 | 960KHz SCS support for DL | 1. 960KHz SCS for DL data and control channels, SSB, and reference signal reception in FR2-2 for non-initial access 2. Multiple-slot PDCCH monitoring for 960KHz with (Xs,Ys)=(8,1) FFS: 3. MultiPDSCH scheduling by single DCI for the operation with 960 kHz SCS and corresponding HARQ enhancements 3. Within the Ys = 1 slot, monitoring of type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS with set1 = (7, 3) symbols where set1 is defined in FG3-5b (FFS: Monitoring capability within slots of type 1 CSS without dedicated RRC configuration and type0, 0A, and 2 CSS) 4. Processing one unicast DCI scheduling DL and one unicast DCI scheduling UL per slot group of Xs slots per scheduled CC for FDD (This supersedes corresponding component of FG 3-5b) 5. Processing one unicast DCI scheduling DL and 2 unicast DCI scheduling UL per slot group of Xs slots per scheduled CC for TDD (This supersedes corresponding component of FG 3-5b) | 24-1 | Yes | N/A | 960KHz SCS support for DL is not supported | Perband | N/A | N/A | N/A | FFS: component description without a reference to other R15 FGs | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 24. NR_ext_to_71GHz | 24-5a | 960KHz SCS support for UL | 1. PRACH with 960KHz and length 139 2. 960KHz SCS for UL data and control channels and reference signal transmission in FR2-2 [3. Multi-PUSCH scheduling by single DCI for the operation with 960 kHz SCS] | 24-1a, 24-5 | Yes | N/A | 960KHz SCS support for UL is not supported | Per band | N/A | N/A | N/A |  | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-5c | Multi-RB PUCCH format 0/1/4 for 960 kHz in FR2-2 | Support multi-RB PUCCH format 0/1/4 for 960 kHz | 24-5a | Yes | N/A | Multi-RB PUCCH format 0/1/4 for 960 kHz in FR2-2 is not supported | Per band | N/A | N/A | N/A | This FG is only supported in bands under PSD limitation in shared spectrum operation | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-5d | Multiple PDSCH scheduling by single DCI for 960 kHz | 1. Multi-PDSCH scheduling by single DCI for the operation with 960 kHz SCS 2. HARQ enhancements |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-5e | Multiple PUSCH scheduling by single DCI for 960 kHz | 1. Multi-PUSCH scheduling by single DCI for the operation with 960 kHz SCS |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-5f | Enhanced PDCCH monitoring for 960KHz | 1. Multiple-slot PDCCH monitoring for 960KHz with (Xs,Ys) 2.) Within each of the Ys = 2 or 4 slots, monitoring of type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS in the first 3 OFDM symbols of each slot (FFS: Monitoring capability within slots of type 1 CSS without dedicated RRC configuration and type0, 0A, and 2 CSS) | 24-5 | Yes | N/A | Enhanced PDCCH monitoring for 960KHz is not supported | Per band | N/A | N/A | N/A | Component 1 candidate values: one or more of {(4,1), (4,2), (8,4)} | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-5g | Single-DCI based SDM scheme multi-PDSCH DL grant for 960 kHz SCS in FR2-2 | Support of single-DCI based SDM scheme for multi-PDSCH scheduling for 960kHz SCS in FR2-2 |  |  |  |  |  |  |  |  |  |  |
| 24. NR_ext_to_71GHz | 24-5h | Single-DCI based FDMSchemeA multi-PDSCH DL grant for 960 kHz SCS in FR2-2 | Support of single-DCI based FDMSchemeA scheme for multi-PDSCH scheduling for 960kHz SCS in FR2-2 |  |  |  |  |  |  |  |  |  |  |
| 24. NR_ext_to_71GHz | 24-5i | Single-DCI based FDMSchemeB multi-PDSCH DL grant for 960 kHz SCS in FR2-2 | Support of single-DCI based FDMSchemeB scheme for multi-PDSCH scheduling for 960kHz SCS in FR2-2 |  |  |  |  |  |  |  |  |  |  |
| 24. NR_ext_to_71GHz | 24-5j | Single-DCI based TDMSchemeA multi-PDSCH DL grant for 960 kHz SCS in FR2-2 | Support of single-DCI based TDMSchemeA scheme for multi-PDSCH scheduling for 960kHz SCS in FR2-2 |  |  |  |  |  |  |  |  |  |  |
| 24. NR_ext_to_71GHz | 24-10 | Additional beam switching time delay | Supported additional beam switching time delay d=56 for 480 kHz SCS |  | Yes | N/A | Additional beam switching time delay d=112 is supported | Per UE | N/A | N/A | N/A | Candidate value set: 56 or 112 symbols | Optional with capability signalling |