3GPP TSG RAN WG1 Meeting #108-e	R1-2202095

e-Meeting, February 21th – March 3rd, 2022

Title:	[Draft] Reply LS to RAN4 on UE capability for supporting single DCI transmission schemes for multi-TRP

Response to:	R4-2120652

Release:	Rel-16

Work Item:	NR_eMIMO-Perf

Source:	Qualcomm [RAN1]

To:	RAN4

Cc:

Contact person:

Name:		Mostafa Khoshnevisan

E-mail Address:	mostafak@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# Overall Description

RAN1 thanks RAN4 for their LS on UE capability for supporting single DCI transmission schemes for multi-TRP and would like to provide replies to the following issues:

RAN4 requests RAN1 to clarify if explicit signaling of maxNumberActiveTCI-PerBWP other than n1 is required for single DCI FDM and SDM transmission schemes or the UE capability of supportFDM-SchemeA-r16 or singleDCI-SDM-scheme-r16 is sufficient.

Answer: Yes, explicit signaling of maxNumberActiveTCI-PerBWP other than n1 is required for single DCI FDM and SDM transmission schemes.

Discussions: RAN1 has implicitly assumed that a UE capable of single-DCI based FDM and SDM schemes is also capable of supporting two active TCI states. However, this is not the case for Single DCI Inter-Slot TDM as UE capability is defined for both maxNumberTCI-states-r16=1 and maxNumberTCI-states-r16=2. The reason for Single DCI Inter-Slot TDM with maxNumberTCI-states-r16=1 is the additional functionality of dynamic number of repetitions introduced in Rel-16, which is applicable to single-TRP as well.

Regarding multi-DCI based mTRP, signaling is defined in UE capability to indicate max number of active TCI states per TRP and across both TRPs through maxNumberActivatedTCI-States-r16 {maxNumberPerCORESET-Pool-r16, maxTotalNumberAcrossCORESET-Pool-r16}.

# Actions

To RAN4:

Action: RAN1 kindly asks RAN4 to take the above reply into account for further work.

# Dates of Next TSG-RAN1 Meeting:

RAN1 #109-e			16 May – 27 May 2022			Electronic