3GPP TSG RAN WG1 #108-e									R1-2202106

e-Meeting, February 21st – March 3rd, 2022

Title:	Draft Reply to RAN2 LS on the reporting of the Tx TEG association information

Response to:       R2-2201776

Release:	Rel-17

Work Item:	NR_pos_enh-Core

Source:	Qualcomm Incorporated [RAN1]

To:	RAN2

CC:	RAN3, RAN4

# Contact Person:

# Name:	Alexandros Manolakos

E-mail Address:	amanolak@qti.qualcomm.com

# 1. Overall Description:

RAN1 thanks RAN2 for their response LS on the reporting of the Tx TEG association information and would like to provide a reply to the following questions:

RAN2 discussed the parameters on Mitigation of UE Rx/Tx timing delays and Mitigation of TRP Rx/Tx timing delays indicated in RAN1 LS (R1-2112976) and found the issues listed below which will need to be confirmed by RAN1: - These two duplicated parameters can be deleted in the RAN1 parameter table: - numOfUERxTEG-PerPRSResource: duplicated with MeasPRSwithDiffRxTEGs_Request_RSTD - numOfTRPRxTEG-PerPRSResource_RTOA: duplicated with MeasPosSRSwithDiffRxTEGs_Request, - The value range of maxNumOfUE-RxTEG is supposed to be 32, rather than as 8 in the table. But RAN2 does not conclude that if the srs-PosResourceSetId is required or not in UE TxTEG association report. RAN2 kindly asks RAN1 to confirm the issues identified by RAN2. And RAN2 respectfully asks RAN1 to respond if srs-PosResourceSetId is required in UE TxTEG association report.

RAN1 confirms the issues identified by RAN2.

Furthermore, RAN1 discussed and concluded that the srs-PosResourceSetId is not required in the UE TxTEG association report. If any additional information is required in the UE TxTEG report, RAN1 will inform RAN2 accordingly.

# 2. Actions to RAN2:

RAN1 respectfully requests RAN2 to take the abovementioned information into account.

# 3. Date of Next TSG-RAN WG1 Meetings:

TSG RAN WG1 Meeting #109-e	May 2022 		      		e-meeting

TSG RAN WG1 Meeting #110	August 2022 			     Toulouse, FR