3GPP TSG RAN WG1 #108-e			    R1- 2202115

e-Meeting, February 21st – March 3rd, 2022

Agenda item:	7.1

Source: 	Qualcomm Incorporated

Title: 	Remaining issues for HARQ-ACK multiplexing on PUSCH without PUCCH

Document for:	Discussion/Decision

# HARQ-ACK multiplexing on PUSCH without PUCCH

In RAN1 105e, an issue on HARQ-ACK multiplexing on PUSCH without PUCCH was identified [1]. The issue can be illustrated as follows.

As a background, a nominal scenario of HARQ-ACK multiplexing on PUSCH is illustrated in Fig 1. In a nominal scenario, when PUCCH HARQ-ACK is generated, UE will determine the PUCCH overlaps with which PUSCHs based on the boundary of the PUCCH. Then in the set of overlapping PUSCHs, UE run the rules defined in spec to pick a PUSCH in the set and multiplexing HARQ-ACK on it. The # multiplexed HARQ-ACK bits follow the tDAI associated with that PUSCH. The UE ignore the tDAIs of other PUSCHs.

Fig 1: A nominal scenario of HARQ-ACK multiplexing on PUSCH

However, as illustrated in Fig 2, in a problematic scenario where PUCCH-HARQ-ACK is not generated due to missing all DL DCIs scheduling the PDSCHs, the above procedure does not work. As shown in the below Fig 2, UE does not know where the PUCCH-HARQ-ACK starts and ends, it cannot decide which PUSCHs overlap with the PUCCH. Therefore, the UE cannot determine HARQ-ACK should be mux on which PUSCH. In other words, UE does not know follow which tDAI and ignore the rest tDAIs.

Fig 2: A problematic scenario of HARQ-ACK multiplexing on PUSCH without PUCCH

In RAN1 # 106e, the issue was discussed under Rel-15 scope. The conclusion is to leave it up to UE implementation to handle this case, to avoid introducing NBC changes to deployed UEs in the field.

For Rel-16 specification, the following agreements were made in RAN1 #107e.

Agreement

For Rel-16 with multiple overlapping PUSCHs with no overlapping PUCCH with HARQ-ACK within a span of one PUCCH slot, if the UL-TDAI n.e. 4 (for Type 2 codebook) or equal to 1 (for Type 1 codebook) there is no consensus in RAN1 on Rel-16 UE behavior. 
 
Agreement

For Rel-16 with one PUSCH and no overlapping PUCCH with HARQ-ACK within a span of one PUCCH slot (both single carrier and UL CA), if the UL-TDAI is not equal to 4 (for Type 2 codebook) or equal to 1 (for Type 1 codebook), the UE multiplexes HARQ-ACK following the UL-TDAI into the PUSCH.

It was also decided to continue discussion on UE behavior with respect to multiplexing HARQ-ACK in PUSCH for the following case in Rel-16.

- More than one non-overlapping PUSCH and no overlapping PUCCH with HARQ-ACK within a span on one PUCCH slot (both single carrier and UL CA), if for at least one of the PUSCHs the UL-TDAI is not equal to 4 (for Type 2 codebook) or equal to 1 (for Type 1 codebook)

For the above case, it is very naturally to follow the principle of the two agreements made in RAN1 #107e, and take the following solution.

- if there is more than one PUSCH in the more than one non-overlapping PUSCH has UL-TDAI not equal to 4 (for Type 2 codebook) or equal to 1 (for Type 1 codebook), it is up to UE implementation to handle this case.

- if there is only one PUSCH (denote as PUSCH A) in the more than one non-overlapping PUSCH has UL-TDAI not equal to 4 (for Type 2 codebook) or equal to 1 (for Type 1 codebook), multiplex HARQ-ACK following the UL-TDAI into that PUSCH A.

Therefore, we have the following proposal.

Proposal 1: for the case of more than one non-overlapping PUSCH and no overlapping PUCCH with HARQ-ACK within a span on one PUCCH slot (both single carrier and UL CA), if at least one of the PUSCHs the UL-TDAI is not equal to 4 (for Type 2 codebook) or equal to 1 (for Type 1 codebook)

- if there is more than one PUSCH in the more than one non-overlapping PUSCH have UL-TDAI not equal to 4 (for Type 2 codebook) or equal to 1 (for Type 1 codebook), it is up to UE implementation to handle this case.

- if there is only one PUSCH (denote as PUSCH A) in the more than one non-overlapping PUSCH has UL-TDAI not equal to 4 (for Type 2 codebook) or equal to 1 (for Type 1 codebook), a UE multiplex HARQ-ACK following the UL-TDAI into that PUSCH A.

# Conclusions

In summary, we make the following proposals for the issue of HARQ-ACK multiplexing on a group of PUSCHs without HARQ-ACK PUCCH.

Proposal 1: for the case of more than one non-overlapping PUSCH and no overlapping PUCCH with HARQ-ACK within a span on one PUCCH slot (both single carrier and UL CA), if at least one of the PUSCHs the UL-TDAI is not equal to 4 (for Type 2 codebook) or equal to 1 (for Type 1 codebook)

- if there is more than one PUSCH in the more than one non-overlapping PUSCH have UL-TDAI not equal to 4 (for Type 2 codebook) or equal to 1 (for Type 1 codebook), it is up to UE implementation to handle this case.

- if there is only one PUSCH (denote as PUSCH A) in the more than one non-overlapping PUSCH has UL-TDAI not equal to 4 (for Type 2 codebook) or equal to 1 (for Type 1 codebook), a UE multiplex HARQ-ACK following the UL-TDAI into that PUSCH A.

# References

- R1-2105079, “Discussions on PUSCH UCI Multiplexing without HARQ-ACK PUCCH in Rel-15”, 3GPP TSG RAN WG1 Meeting #105-e, May 10th – 27th 2021.