3GPP TSG RAN WG1 #108-e																  R1-2202142

e-Meeting, February 21st – March 3rd, 2022

Agenda item:	8.5.3

Source: 	Qualcomm Incorporated

Title: 	Maintenance on Potential Enhancements for DL-AoD positioning

Document for:		Discussion and Decision

# Introduction

At RAN1 #107-e meeting, the following agreements were reached with regards to this sub-agenda:

Conclusion The current definition of measured path DL PRS RSRP for ith path delay is sufficient and will not be discussed further in AI 8.5.3 - Note: discussion in other agenda items, if necessary, is not precluded. Agreement From the RAN1 perspective, for the TRP beam/antenna information to be optionally provided by the LMF to the UE for UE-based DL-AoD: - The LMF provides the quantized version of the relative Power between PRS resources per angle per TRP. - The relative power is defined with respect to the peak power in each angle - For each angle, at least two PRS resources are reported. - Note: the peak power per angle is not provided - Note: up to RAN3 to decide how the TRP beam information is provided to the LMF for both UE-assisted and UE-based - Send an LS to RAN2/RAN3 to decide on the signaling details R1-2112843 [DRAFT] LS on TRP beam/antenna information Moderator (Ericsson) Final LS endorsed in R1-2112844. Agreement For reporting of DL PRS RSRPP and PRS RSRP in UE-A DL-AOD - The maximum number of DL PRS RSRPP M is a UE capability and its candidate values include {2,4,8,16,24}. - The capabilities for DL PRS RSRPP (M value) and DL PRS RSRP (N values) are such that M is less than or equal to N Agreement For the purpose of both UE-B and UE-A DL-AoD, and with regards to the support of AOD measurements with an expected uncertainty window, the following is supported - Indication of expected angle value and uncertainty (of the expected azimuth and zenith angle value) range(s) is signaled by the LMF to the UE - The type of expected angle and uncertainty can be requested by the UE, between the following options - Option 1: Indication of expected DL-AoD/ZoD value and uncertainty (of the expected DL-AoD/ZoD value) range(s) is signaled by the LMF to the UE - Option 2: Indication of expected DL-AoA/ZoA value and uncertainty (of the expected DL-AoA/ZoA value) range(s) is signaled by the LMF to the UE Agreement For UE-assisted DL-AOD positioning method, to enhance the signaling to the UE for the purpose of PRS resource(s) reporting, the LMF may indicate in the assistance data (AD), one or both the following: - option 1: subject to UE capability, for each PRS resource, a subset of PRS resources for the purpose of prioritization of DL-AOD reporting: - a UE may include the requested PRS measurement for the subset of the PRS in the DL-AoD additional measurements if the requested PRS measurement of the associated PRS is reported - The requested PRS measurement can be DL PRS RSRP and/or path PRS RSRP. - UE may report PRS measurements only for the subset of PRS resources. - Note: The subset associated with a PRS resource can be in a same or different PRS resource set than the PRS resource - option 2: subject to UE capability, for each PRS resource, the boresight direction information. - Note: Either case does not imply any restriction on UE measurement - FFS: prioritization of the PRS resources and resource subsets to be measured

In this paper, we present our views on the maintenance of the potential enhancements related to DL-AoD positioning.

# RSRPP definition & related RAN4 LS

An LS (R1-2200905) was sent to RAN1 from RAN4 with regards to the RSRPP definition with the following content:

The PRS-RSRPP can be reported in ways below : - RAN4 understands that PRS-RSRPP should be defined as the path RSRP per RE, in the same way PRS-RSRP is defined as power per RE. RAN4 respectfully asks RAN1 to confirm if this understanding is correct. - PRS-RSRPP may be reported by reusing absolute and differential PRS-RSRP measurement report mapping tables in TS38.133 clause 10.1.24.3.1 and 10.1.24.3.2 respectively. - When differential reporting is used, PRS-RSRPP is reported as the difference in dB with respect to a reference measurement. RAN4 understands that it is up to RAN1/2 to decide what reference measurement would be for the PRS-RSRPP differential reporting. RAN4 concludes that these ways are possible to support PRS-RSRPP in Rel-17.

We make the following proposals with regards to the issues raised in the above LS by RAN4:

Proposal 1: RAN1 confirms that PRS-RSRPP should be defined as the path RSRP per RE.

Proposal 2: Support reporting first-path PRS-RSRPP by reusing absolute PRS-RSRP measurement report mapping for the first measurement (similar to PRS-RSRP in NR Rel-16).

Proposal 3: Support reporting first-path PRS-RSRPP for additional measurements by reusing differential PRS-RSRP measurement report mapping with respect to the first-path PRS-RSRPP of the first measurement (similar to the reporting of PRS-RSRP for the additional measurements in NR Rel-16).

Proposal 4: Support reporting additional path PRS-RSPP for a measurement (first or additional) by reusing differential reporting with respect to the PRS-RSRPP of the first-path of the measurement.

# Beam Information Signaling

The following agreement was reached with regards to the beam information signaling for UE-based Positioning:

Agreement From the RAN1 perspective, for the TRP beam/antenna information to be optionally provided by the LMF to the UE for UE-based DL-AoD: - The LMF provides the quantized version of the relative Power between PRS resources per angle per TRP. - The relative power is defined with respect to the peak power in each angle - For each angle, at least two PRS resources are reported. - Note: the peak power per angle is not provided - Note: up to RAN3 to decide how the TRP beam information is provided to the LMF for both UE-assisted and UE-based - Send an LS to RAN2/RAN3 to decide on the signaling details

One way to reduce overhead is to exploit the fact that different TRPs may have the same LCS relative beam pattern, i.e., the same digital/analog precoder weights, with similar RF components and antenna elements, being used across TRPs. In this case, it may be possible to provide the relative beam pattern only for one of the TRPs, whereas provide for the rest, the LCS-to-GCS translation parameters (each TRP is located in a different place and have a different orientation).

Observation: 1: Multiple TRPs may have the same relative beam pattern, when expressed in LCS, i.e., when the same digital/analog precoder weights, with similar RF components and antenna elements patterns.

We observe that there was a similar field introduced in NR Rel-16 for the reporting of the boresight directions to the UE. However, this field currently assumes that even the LCS-To-GCS translation needs to be the same, which actually eliminates almost completely the usefulness of this field. To be specific, the likelihood that 2 TRPs have exactly the same beam pattern in the GCS is really limited in realistic deployment; it has to be some kind of hexagonal/symmetric/academic-type of deployment.

associated-DL-PRS-ID This field specifies the dl-PRS-ID of the associated TRP from which the beam information and parameters for LCS to GCS translation are adopted. If the field is omitted, the beam information is provided via the dl-prs-BeamInfoSet field and the LCS to GCS translation parameter is provided via the lcs-GCS-TranslationParameter. If the field is present, the fields lcs-GCS-TranslationParameter and dl-PRS-BeamInfoSet shall be absent.

Proposal 5: In the beam antenna Assistance data element, support signaling that 2 TRPs have the same relative beam information when provided in Local Coordinate System (LCS). In other words, introduce an associated-DL-PRS-ID field with the following meaning:

- This field specifies the dl-PRS-ID of the associated TRP from which the beam antenna information is adopted: If the field is present, the field dl-PRS-BeamInfoSet shall be absent, but the field lcs-GCS-TranslationParameter can be provided.

With regards to the granularity of the beam information signaling we make the following proposals:

Proposal 6: In the beam antenna Assistance data element, support the beam antenna information to be able to be reported with a resolution of 1 degree, or 0.1 degrees (fine resolution), for the beam antenna information in both azimuth and zenith dimension.

Proposal 7: In the beam antenna Assistance data element, the relative power between 2 PRS resources should at least be reportable with 0.1 dB granularity with a range between 0 and -30 dB.

Proposal 8: In the beam antenna Assistance data element, for each (Azimuth, elevation) angle, the relative ratio of a maximum of 128 PRS resources can be provided in the assistance data, which could be from the same or different set of the same PFL.

# Expected DL-AoD Assistance Data for UE-A/UE-B DL-AoD

In the previous meeting, the following agreement was made with regards to expected AoA enhancements:

Agreement For the purpose of both UE-B and UE-A DL-AoD, and with regards to the support of AOD measurements with an expected uncertainty window, the following is supported - Indication of expected angle value and uncertainty (of the expected azimuth and zenith angle value) range(s) is signaled by the LMF to the UE - The type of expected angle and uncertainty can be requested by the UE, between the following options - Option 1: Indication of expected DL-AoD/ZoD value and uncertainty (of the expected DL-AoD/ZoD value) range(s) is signaled by the LMF to the UE - Option 2: Indication of expected DL-AoA/ZoA value and uncertainty (of the expected DL-AoA/ZoA value) range(s) is signaled by the LMF to the UE

For UL-AOA, the following agreements were made with regards to the granularity of the expectedAoA indication

Agreement: Granularity of 0.1 degrees is applied for the expected AoA (φAOA), expected ZoA (θZOA ) and the corresponding uncertainty values

Proposal 9: With regards to the expected angle indication for DL-AoD, the indication of the

- Expected Azimuth DL-AoD should be with range between 0 to 359.9 degrees, with a scale of 0.1 degrees.

- Expected Zenith DL-AoD should be with range between 0 to 180 degrees, with a scale of 0.1 degrees.

- Expected Azimuth DL-AoD/DL-AoA uncertainty range is [-60,60] with an step size of 1 degrees.

- Expected Zenith DL-AoD/DL-AoA uncertainty range is [-30,30] with an step size of 1 degrees.

# Associated PRS resource subset for UE-A DL-AoD

The following agreement was reached previous meeting with regards to the PRS resource reporting prioritization:

Agreement For UE-assisted DL-AOD positioning method, to enhance the signaling to the UE for the purpose of PRS resource(s) reporting, the LMF may indicate in the assistance data (AD), one or both the following: - option 1: subject to UE capability, for each PRS resource, a subset of PRS resources for the purpose of prioritization of DL-AOD reporting: - a UE may include the requested PRS measurement for the subset of the PRS in the DL-AoD additional measurements if the requested PRS measurement of the associated PRS is reported - The requested PRS measurement can be DL PRS RSRP and/or path PRS RSRP. - UE may report PRS measurements only for the subset of PRS resources. - Note: The subset associated with a PRS resource can be in a same or different PRS resource set than the PRS resource - option 2: subject to UE capability, for each PRS resource, the boresight direction information. - Note: Either case does not imply any restriction on UE measurement FFS: prioritization of the PRS resources and resource subsets to be measured

In option 1, for each PRS resource, the subset of PRS resources should be sorted according to decending priority where the first PRS resource ID in the subset has the highest priority.

Proposal 10: For UE-A DL-AoD, for each PRS resource, the subset of associated PRS resources should be sorted according to decending priority where the first PRS resource ID in each subset has the highest priority.

- Up to 24 PRS resources can be included in each subset, which can be from the same or different set of a same PFL of the TRP.

# Conclusions

Overall, we make the following Observations and proposals related to DL-AoD Enhancements:

Proposal 1: RAN1 confirms that PRS-RSRPP should be defined as the path RSRP per RE.

Proposal 2: Support reporting first-path PRS-RSRPP by reusing absolute PRS-RSRP measurement report mapping for the first measurement (similar to PRS-RSRP in NR Rel-16).

Proposal 3: Support reporting first-path PRS-RSRPP for additional measurements by reusing differential PRS-RSRP measurement report mapping with respect to the first-path PRS-RSRPP of the first measurement (similar to the reporting of PRS-RSRP for the additional measurements in NR Rel-16).

Proposal 4: Support reporting additional path PRS-RSPP for a measurement (first or additional) by reusing differential reporting with respect to the PRS-RSRPP of the first-path of the measurement.

Proposal 5: In the beam antenna Assistance data element, support signaling that 2 TRPs have the same relative beam information when provided in Local Coordinate System (LCS). In other words, introduce an associated-DL-PRS-ID field with the following meaning:

- This field specifies the dl-PRS-ID of the associated TRP from which the beam antenna information is adopted: If the field is present, the field dl-PRS-BeamInfoSet shall be absent, but the field lcs-GCS-TranslationParameter can be provided.

Proposal 6: In the beam antenna Assistance data element, support the beam antenna information to be able to be reported with a resolution of 1 degree, or 0.1 degrees (fine resolution), for the beam antenna information in both azimuth and zenith dimension.

Proposal 7: In the beam antenna Assistance data element, the relative power between 2 PRS resources should at least be reportable with 0.1 dB granularity with a range between 0 and -30 dB.

Proposal 8: In the beam antenna Assistance data element, for each (Azimuth, elevation) angle, the relative ratio of a maximum of 128 PRS resources can be provided in the assistance data, which could be from the same or different set of the same PFL.

Proposal 9: With regards to the expected angle indication for DL-AoD, the indication of the

- Expected Azimuth DL-AoD should be with range between 0 to 359.9 degrees, with a scale of 0.1 degrees.

- Expected Zenith DL-AoD should be with range between 0 to 180 degrees, with a scale of 0.1 degrees.

- Expected Azimuth DL-AoD/DL-AoA uncertainty range is [-60,60] with an step size of 1 degrees.

- Expected Zenith DL-AoD/DL-AoA uncertainty range is [-30,30] with an step size of 1 degrees.

Proposal 10: For UE-A DL-AoD, for each PRS resource, the subset of associated PRS resources should be sorted according to decending priority where the first PRS resource ID in each subset has the highest priority.

- Up to 24 PRS resources can be included in each subset, which can be from the same or different set of a same PFL of the TRP.

# References

- RP-202900, “New WID on NR Positioning Enhancements”, CATT, Intel Corporation, Ericsson; 3GPP TSG RAN Meeting #90e, Electronic Meeting, December 7 - 11, 2020.

- Bluetooth Direction Finding: A Technical Overview, Martin Wooley, Bluetooth SIG, March 2019.

[3] R1-2110625 FL summary #4 for AI 8.5.3 Accuracy improvements for DL-AoD positioning   solutions	Moderator (Ericsson