| 3GPP TSG-RAN WG1 Meeting #108-e e-Meeting, February 21st – March 3rd, 2022 | R1-2202154 |
| --- | --- |

Agenda item:	8.8.2

Source: 	Qualcomm Incorporated

Title: 	 PUCCH enhancements

Document for:	Discussion/Decision

# Introduction

Most of the critical issues regarding DMRS bundling for PUCCH repetitions were resolved in RAN1 107-e and 107bis-e meetings. This document discusses some of the remaining issues including interaction with inter-slot frequency hopping.

# Inter-slot frequency hopping

In RAN1 107-e meeting, the following agreement was reached:

Agreement For the interaction between inter-slot frequency hopping and DMRS bundling for PUCCH/PUSCH repetitions, a UE performs the “hopping intervals determination”, “configured TDW determination”, and “actual TDW determination” in a sequential ordering, based on the following option 1. - Option 1: “hopping intervals determination” -> “configured TDW determination” -> “actual TDW determination” - DMRS bundling shall be restarted at the beginning of each frequency hop - DMRS bunding is per actual TDW - FFS: Frequency hopping pattern is determined by physical slot indices. - FFS: different FH pattern determination for PUCCH and PUSCH - FFS: details of FH pattern design - Support separate RRC configuration(s) for hopping interval and configured TDW length. - if hopping interval is not configured, the default hopping interval is the same as the configured TDW length - FFS: if both hopping interval and TDW length are not configured - Note: hopping interval is only determined by the configuration of hopping interval if hopping interval is configured

In the following we discuss the determination of the frequency hopping pattern and its dependence on physical slot indices.

We propose the following procedure to describe how inter-slot frequency hopping and DMRS bundling may interact with each other and be enabled at the same time.

For the purposes of DMRS bundling, a gNB may configure a UE with a new frequency hopping pattern. This new frequency pattern defines groups of contiguous physical slots that are assigned to each frequency hop. For example, slots 0 to 3 could be assigned to first hop, slots 4 to 7 could be assigned to second hop, and so on. Note that this a predetermined/static mapping between hops to physical slots. This strives to prioritize frequency hopping and the efficient use of resources over DMRS bundling.

Thus, once UE determines the set of available slots for a sequence of PUSCH repetitions, it assigns hop indices to each slot that’s available based on the predetermined hopping patter that it is configured with. Once these two steps are completed, it proceeds to determining the placement of the time domain windows. Hop indices are factored into time domain window locations and two slots with different hop indices are not bundled together.

The three-step procedure can be summarized as follows:

- Identify available slots for PUSCH/PUCCH reps

- Determine frequency hopping index for each available slot based on the configured hopping pattern for DMRS bundling

- Determine configured TDWs for DMRS bundling assuming frequency hop allocations made in Step 2. UE ensures that no two slots with different hop indices are bundled together.

The above process is figuratively represented below, where a UE is scheduled to transmit PUSCH with 8 repetitions and a 4-slot hopping pattern. Note that the hopping pattern is decoupled from the start of PUSCH repetitions.

Figure 1 Inter-slot frequency hopping with DMRS bundling

We further consider the behavior if we were to determine hop index based on relative slot index and available slot index, rather than physical slot index. These alternatives are considered in the figure below, where 3-slot hop pattern are determined across a sequence of repetitions. As can be observed, determination of hop patterns based on physical slot indices or relative slot indices differs only by an offset. Given the complexities of multiplexing different UEs for better resource utilization, we don’t see relative slot index-based hopping as offering any significant benefits. On the other hand, determination based on available slot index results in a hop pattern that is far more likely to be well balanced across hops but is a significant departure from legacy operation  and comes at the expense of inefficient resource utilization across users.

Figure 2 Hop pattern determination based on physical slot indices, relative slot indices, and available slot indices.

With the above discussion in mind, we make the following proposal.

Proposal 1: Frequency hopping pattern for DMRS bundling across PUCCH transmissions is determined based on physical slot indices.

# Conclusion

This document presents our views on DMRS bundling across PUCCH repetitions. In particular, we make the following proposals:

Proposal 1: Frequency hopping pattern for DMRS bundling across PUCCH transmissions is determined based on physical slot indices.

# Reference

- Chairman notes, 3GPP TSG RAN1 meeting #107-e, e-Meeting, November 11th – 19th, 2021.

- Chairman notes, 3GPP TSG RAN1 meeting #108-e, e-Meeting, January 17th – 25th, 2022.

# Appendix A: Agreements made in RAN1-107bis-e [2]

Agreement In column J of RRC parameter “PUCCH-nrofSlots-r17”, add a note as the following: - Note: a PUCCH resource not configured with PUCCH-nrofSlots-r17 can attain the value of 1 according to when the Rel-15/16 parameter nrofSlots is not configured. Agreement For PUCCH repetitions and PUSCH repetitions with DMRS bundling, introduce the following two RRC parameters for frequency hopping interval configuration. - PUCCH-Frequencyhopping-Interval - PUSCH-Frequencyhopping-Interval Note: finalize the details (such as value range, parent IE, etc…) of these two RRC parameters in RAN1 107#bis-e. Agreement PUCCH repetitions with different sets of power control parameters in multi-TRP operation should be regarded as a [semi-static] event that causes power consistency and phase continuity not to be maintained across PUCCH repetitions. Agreement - The RRC parameter “PUCCH-DMRS-Bundling” is per UL BWP, and the RRC parameter “PUCCH-TimeDomainWindowLength” is per UL BWP. - PUCCH DMRS bundling is not supported for PUCCH format 0/2. Agreement Adopt the following table for the RRC parameters PUCCH-nrofSlots-r17, PUCCH-DMRS-Bundling, PUCCH-TimeDomainWindowLength, PUCCH-Window-Restart, PUCCH-Frequencyhopping-Interval, and PUSCH-Frequencyhopping-Interval. WI code; Sub-feature group; Parameter name in the spec; New or existing?; Description; Value range; Default value aspect; Per (UE, cell, TRP, …); UE-specific or Cell-specific. NR_cov_enh-Core; PUCCH enhancements; PUCCH-nrofSlots-r17; new; A new repetition parameter corresponding to Rel-17 dynamic PUCCH repetition factor indication. The new repetition parameter is configured per PUCCH resource and should be in PUCCH-Resource. Note: a PUCCH resource not configured with PUCCH-nrofSlots-r17 can attain the value of 1 according to when the Rel-15/16 parameter nrofSlots is not configured.; ENUMERATED {2, 4, 8}; In PUCCH-resource; UE-specific. NR_cov_enh-Core; DM-RS bundling for PUCCH; PUCCH-DMRS-Bundling; new; Enabling/disabling of DM-RS bundling and time domain window for PUCCH repetitions.; ENUMERATED {enabled, disable }; in PUCCH-Config Note: The RRC parameter “PUCCH-DMRS-Bundling” is per UL BWP. PUCCH DMRS Bundling is not supported for PUCCH format 0/2; UE-specific. NR_cov_enh-Core; DM-RS bundling for PUCCH; PUCCH-TimeDomainWindowLength; new; Length of a [nominal] time domain window in slots for DMRS bundling for PUCCH.; INTEGER (2..[8]) Note: the value shall not exceed the maximum duration; in PUCCH-Config Note: The RRC parameter “PUCCH-TimeDomainWindowLength” is per UL BWP. PUCCH DMRS Bundling is not supported for PUCCH format 0/2; UE-specific. NR_cov_enh-Core; DM-RS bundling for PUCCH; PUCCH-Window-Restart; new; UE bundles PUCCH DM-RS slots remaining in a bundling nominal time domain window after a slot for which events violate power consistency and phase continuity requirements UE bundles PUCCH DM-RS remaining in a nominal time domain window after dynamic event(s) triggered by DCI or MAC-CE that violate power consistency and phase continuity requirements Note: Events, which are triggered by DCI or MAC CE, but regarded as semi-static events, e.g. frequency hopping, UL beam switching for multi-TRP operation, or other if defined, are excluded.; ENUMERATED {enabled, disable }; in PUCCH-Config; UE-specific. NR_cov_enh-Core; DM-RS bundling for PUCCH; PUCCH-Frequencyhopping-Interval; New; Number of consecutive slots for UE to perform Rel-17 inter-slot frequency hopping with inter-slot bundling for PUCCH Note: When DMRS bundling for PUCCH is enabled by PUCCH-DMRS-Bundling, PUCCH frequency hopping interval is only determined by the configuration of PUCCH hopping interval if PUCCH hopping interval is configured.; FFS; PUCCH-TimeDomainWindowLength; in PUCCH-Config; UE-specific. NR_cov_enh-Core; DM-RS bundling for PUSCH; PUSCH-Frequencyhopping-Interval; New; Number of consecutive slots for UE to perform Rel-17 inter-slot frequency hopping with inter-slot bundling for PUSCH. Note 1: This RRC parameter is shared for both DG-PUSCH and CG-PUSCH Note 2: When DMRS bundling for PUSCH is enabled by PUSCH-DMRS-Bundling, PUSCH frequency hopping interval is only determined by the configuration of PUSCH hopping interval if PUSCH hopping interval is configured.; FFS; PUSCH-TimeDomainWindowLength; in PUSCH-Config; UE-specific. Agreement The same mechanism supporting inter-slot frequency hopping with inter-slot bundling for DMRS bundling for type-A PUSCH repetitions is reused for TBoMS with/without repetitions. - No optimization of FH mechanism specific to TBoMS with repetitions

# Appendix B: Agreements in RAN1 107-e e-meeting [1]

Agreement For a PUCCH resource to transmit a PUCCH without an associated scheduling DCI (e.g. P/SP-CSI or SR), if the PUCCH resource is configured with RRC parameter “nrofSlots-r17”, “nrofSlots-r17” is ignored and the RRC parameter “nrofSlots” is used for determining the repetition factor of the specific PUCCH resource. Agreement The following use case 5 of PUCCH DMRS bundling is not supported in Rel-17 - Use case 5: PUCCH repetitions across non-consecutive slots. - Use case 5a: no uplink transmission in the middle of two PUCCH repetitions - Use case 5b: other uplink transmissions in the middle of two PUCCH repetitions Agreement For PUCCH DMRS bundling, when appliable, reuse the procedure developed for PUSCH DMRS bundling to determine configured TDW(s) and actual TDW(s). - FFS: events for PUCCH actual TDW(s) Agreement For the interaction between inter-slot frequency hopping and DMRS bundling for PUCCH/PUSCH repetitions, a UE perform the “hopping intervals determination”, “configured TDW determination”, and “actual TDW determination” in a sequential ordering. One option of the following options is to be selected. - Option 1: “hopping intervals determination” -> “configured TDW determination” -> “actual TDW determination” - FFS: DMRS bundling should be restarted in case of frequency hopping event - FFS: whether same or separate RRC configuration(s) for hopping interval and configured TDW. - Option 2: “configured TDW determination” -> “hopping intervals determination” -> “actual TDW determination” Agreement For the interaction between inter-slot frequency hopping and DMRS bundling for PUCCH/PUSCH repetitions, a UE performs the “hopping intervals determination”, “configured TDW determination”, and “actual TDW determination” in a sequential ordering, based on the following option 1. - Option 1: “hopping intervals determination” -> “configured TDW determination” -> “actual TDW determination” - DMRS bundling shall be restarted at the beginning of each frequency hop - DMRS bunding is per actual TDW - FFS: Frequency hopping pattern is determined by physical slot indices. - FFS: different FH pattern determination for PUCCH and PUSCH - FFS: details of FH pattern design - Support separate RRC configuration(s) for hopping interval and configured TDW length. - if hopping interval is not configured, the default hopping interval is the same as the configured TDW length - FFS: if both hopping interval and TDW length are not configured - Note: hopping interval is only determined by the configuration of hopping interval if hopping interval is configured