3GPP TSG RAN WG1 Meeting #108-e	R1-2202158

February 21st – March 3rd, 2022

Agenda item:	8.11.1.1

Source: 	Qualcomm Incorporated

Title: 	Power Savings for Sidelink

Document for:	Discussion/Decision

# Introduction

RAN1 identified the following remaining open issues for sidelink power saving enhancements [1]:

- Physical layer aspects on resource allocation to reduce UE’s power consumption including:

  - Finalization of pre-emption/re-evaluation checking for aperiodic transmission

  - Finalization of selection/report of candidate resources in which at least its subset is within RX UE's active time

  - Finalization of SL CBR measurement in partial sensing

  - CPS monitoring window for aperiodic transmission when UE performs at least CPS in a Tx pool

  - T1 of RSW when UE performs only CPS in a Tx pool with periodic reservation for another TB disabled

  - Sensing and SL CBR measurement during its SL DRX inactive time

  - Re-evaluation and pre-emption checking after random selection

  - Resource pool segregation for periodically occurring resources

  - Random resource selection in pools with mixed RA schemes

  - Conditions in which CPS can be disabled in resource (re)selection

In RAN1 #107bis some of the open issues were addressed and resolved. In this contribution, we discuss the remaining open issues and provide proposals to address them.

# Random Selection

## Re-evaluation with Random Selection

Re-evaluation enables the UE to change selected, but not-yet-signalled resources, based on sensing information. This allows the UE to choose different resources and avoid collisions, thereby improving performance. Here, the FFS on whether re-evaluation can be supported by UEs that perform random selection is discussed:

Agreements:

- Re-evaluation and pre-emption checking are not supported by UEs that do not perform any sensing (i.e. PSCCH reception)

- Re-evaluation and pre-emption checking are supported by UEs that perform sensing

  - FFS details and any conditions(s) in which re-evaluation and pre-emption can be performed

- FFS whether/how re-evaluation and pre-emption can be supported by UEs performing random resource selection that do perform sensing

- Note: details about sensing in this context, including when it is performed, are not decided yet.

We propose a scheme, where a UE that performs random selection additionally starts sensing and performs re-evaluation after selecting resources. The sensing information used for re-evaluation would then begin at the resource selection trigger and the UE continues to perform sensing and re-evaluation until the last retransmission of the TB. This process is illustrated in Figure 1.

Figure 1: Performing random selection with subsequent revaluation

The performance of random selection with and without re-evaluation is shown in Figure 2 for an urban road scenario with a mix of 500 vehicular and 500 pedestrian UEs with other simulation assumptions as in Appendix A. It can be observed that performing re-evaluation significantly improves the performance of random selection and recovers most of the loss compared to the full sensing baseline.

Figure 2 Performance of methods using sensing after resource selection trigger

Observation 1: Performing re-evaluation after random resource selection significantly improves performance compared to random selection without sensing.

Proposal 1: Introduce random selection with subsequent re-evaluation in NR sidelink, at least by UE implementation.

## Random Selection in a Mixed Resource Pool

This section discussed how random selection by some UEs in the resource pool impacts the performance of both the UEs performing random selection and UEs performing full sensing in the system for NR traffic, studying the impact of resource collisions in such cases from the following agreement:

Agreement

For random resource selection in a resource pool (pre-)configured with full/partial sensing and random resource selection, down-select to one of the followings in RAN1#106bis-e

- Option 1: A priority threshold value or a range of priority levels is (pre-)configured for the resource pool, below or within which random resource selection is allowed

  - Note, lower value means higher priority

  - FFS whether resource pool partitioning can be additionally applied

- Option 2: Increase the priority for the transmission based on random selection and indicate the new priority value in the priority field in the 1st-stage SCI

  - FFS: An extra field is added in SCI for indicating the original priority value associated with QoS requirement,

  - FFS: A 1-bit field in the SCI indicates that the UE is performing random resource selection, or

  - FFS: An extra field is added in SCI for indicating the mapping to the original priority value associated with QoS requirement.

- Option 7: Exclude resources reserved by UE performing random selection without re-evaluation / pre-emption checking, regardless of their priorities. E.g. a 1-bit field in the SCI indicates that the UE is performing random resource selection and not performing re-evaluation and pre-emption checking

- Option 12: No special consideration

To separate the impact of sensing procedure from that of the traffic patterns used by power-sensitive UEs, we first compare the performance using UEs with identical traffic models, NR medium with 50ms+50ms packet arrival rate and 50ms PDB. Figure 3 shows the performance of a system composed of 1000 UEs. The proportion of UEs that perform full sensing is varied and the other UEs in the system use random selection without sensing.

The performance of UEs that perform full sensing is presented in Figure 3-a, where it can be observed that the performance of these UEs is not noticeably impacted when their proportion is reduced in the system and performance remains similar to the baseline where all UEs perform full sensing.

Figure 3-b shows the performance of UEs that do not perform sensing in the system compared to the baseline of all sensing UEs. It can be observed that the performance of UEs that do not perform sensing is degraded relative to the baseline performance.

Figure 3 Performance impact of disabling sensing on (a) UEs that perform sensing (b) UEs that do not when both have aperiodic traffic.

Similarly, we consider the scenario where a proportion of the UEs are performing full sensing, have periodic traffic, and are using periodic reservations, while the remaining UEs have aperiodic traffic with aperiodic transmissions. We compare the performance of the UE with periodic traffic in the case where the UEs with aperiodic traffic are performing sensing and the case where the UEs with aperiodic traffic are performing random selection without sensing. The performance of the full-sensing, periodic-traffic UEs is shown in Figure 4, where 50% of the UEs are performing periodic transmissions and 50% are performing aperiodic transmissions.

Figure 4 Highway scenario performance of full-sensing UEs with periodic traffic when the remaining UEs in the system perform aperiodic transmissions with and without sensing.

Based on these results, we note that not performing sensing in a proportion of the UEs degraded the performance of the UEs not performing sensing but did not noticeably impact UEs that perform sensing.

Observation 2: The performance of full-sensing UEs in the system is not noticeably impacted when a proportion of UEs is replaced with ones that do not perform sensing.

Proposal 2: No special consideration is needed for resources chosen using random selection in a resource pool with full and/or partial sensing.

# Partial Sensing for Power Saving

In this section, we present the design considerations required for supporting Mode 2 resource allocation with partial sensing in NR.

As a first step, a resource pool, configured with partial sensing, should be split into a disjoint set of resources, where each set comprises a set of contiguous slots and repeats with a given periodicity. This is illustrated in Figure 5 below.

Figure 5: An illustration of a set of disjoint resource sets defined on a given resource pool.

Each UE can perform sensing in one or multiple of the defined sets. For example, a UE can choose to perform sensing in set A only, or in set A and set B. However, both the periodic and aperiodic resource reservations are limited to the same or different instances of the same set. As an example, if a UE is performing partial sensing over both sets A and B, it can only reserve resources in set A from slots in set A. The reason for this is that if a UE selects a resource in set B from a slot in set A, its reservation may collide with a UE that only performs sensing in set B and, hence, has no knowledge about the reservations made from other sets. This, in fact, is also the reason for partitioning the resource pools using the resource sets and ensuring that they are completely non-overlapping. This can further be clarified using a scenario illustrated in Figure 6 below.

Figure 6: Two different pairs of partial sensing UEs communicating with each other; the activity durations are aligned within each pair of UEs but not across the two pairs.

As can be seen in the figure above, if the partial sensing resource sets are not completely disjoint, collision is expected between UE1’s and UE2’s transmissions since UE2 is not aware of the earlier reservation made by UE1.

Based on the discussion so far, we propose the following:

Proposal 3: For supporting mode 2 resource allocation with partial sensing, define a set of periodic partial sensing resource sets partitioning a resource pool.

Proposal 4: A UE can perform partial sensing over a single or multiple resource sets.

Proposal 5: The reservation of a resource in a given set can only be signalled from another slot associated with the same resource set.

Next, for a UE operating with partial sensing, we present further details on how the selection of Y candidate slots within the resource selection window should be done and which resources should be monitored for sensing under both periodic and aperiodic reservations.

For the selection of Y candidate slots, let us consider the scenarios depicted in Figure 7, and assume that a UE is performing partial sensing over set A. Let us further assume that the resource selection procedure is triggered in slot n.

Figure 7: Identifying Y candidate slots for a UE performing aperiodic resource reservation with partial sensing.

As mentioned earlier, a UE performs sensing and resource selection in the same resource set. For the UE in this example, partial sensing is done over slots associated with resource set A. The selection of candidate resources is also within the same set; however, some limitations are required, which can be explained via Case (a)-(c) shown above.

Under Case (a), the processing time for resource selection ends before the start of resource set A. The remaining PDB for the packet expires before the same instance of the resource set A. Hence, the set of candidate resources consists of all slots from the beginning of the resource set A until the end of the remaining PDB within the same instance of resource set A.

Under Case (b), the processing time for resource selection ends after the start of resource set A. Therefore, the slots at the beginning of the resource set are not within the candidate set. The remaining PDB of the packet expires after the end of the resource set A, but before the start of the next instance of resource set A. Hence, except the first few slots at the beginning of the resource set A, the remaining ones within the same instance are within the candidate resource set.

Under Case (c), similar to Case (b), the first few slots are not part of the candidate resource set. However, the remaining PDB extends more than one instance of a resource set A. Hence, as shown above, some of the slots in the next instance of resource set A are also considered as part of the candidate resource set.

Based on the observations above, it can be concluded that the resource selection window is determined within a set of resources formed by the intersection of resources in a given resource set for partial sensing and those in-between n+T1 and when the remaining PDB of a packet expires. In other words, similar to Rel. 16, the resource selection window consists of all resources within [n+T1,n+T2] confined within the set of periodic resources associated with a given partial sensing window.

Proposal 6: For a UE performing partial sensing, if the resource selection procedure is triggered in slot n, the resource selection window consists of the slots associated with a given partial sensing window that lie within  [n+T1,n+T2].

As a next step, we answer the question that which resources should be monitored for the purpose of resource selection. Let us first consider a case where periodic resource reservations are not enabled for a resource pool.  Let us consider again a UE that is performing partial sensing within resources associated with set A as shown in the figure below.

Figure 8: Sensing occasions for aperiodic resource reservation.

Under the full-sensing operation when periodic reservations are disabled, to identify the availability of resources in the selection window, it is sufficient to only monitor the slots within the last 32 slots that lie within the sensing window.  When a UE is performing partial sensing, a similar approach, with a minor modification, can be adopted. In particular, for selecting resources, only the slots that overlap with the resources associated with the partial sensing set A should be considered.

Proposal 7: For identifying the availability of resources in a selection window associated with a given partial sensing set in a resource pool with periodic reservation disabled, the sensing information from the past 32 slots that are within the intersection of the sensing window and the same partial sensing set is sufficient.

If the periodic reservations are further enabled, to determine the availability of slot tySL in a partial sensing set A, the sensing information from slot tySL-Preserve is needed only if the sensing slot is associated with the same partial sensing set.  This is shown in the figure below (note that the aperiodic sensing occasions are not shown in the figure below for simplicity.)

Figure 9: Sensing occasions for periodic reservations.

Proposal 8: For identifying the availability of a slot tySL associated with a given partial sensing set in a resource pool with periodic reservation enabled, in addition to the sensing information for aperiodic reservations as proposed in Proposal 6, the sensing information from tySL-Preserve,  ∀ Preserve ∈sl_ResourceReservePeriodList, is required if tySL-Preserve belongs to the same partial sensing set.

So far in this section, we focused on how mode 2 resource allocation with partial sensing should be designed from the SL transmitter point of view. In particular, we proposed that for a given UE, sensing and transmission operations are limited to a periodic set of resources defined by a partial sensing set(s).

Unlike LTE, for NR SL, it is desirable to allow the partial sensing UEs to not only transmit, but to also receive from other UEs. This could be helpful, e.g., for a pedestrian UE receiving warnings from the vehicles or public safety UEs communicating with each other in both directions. To communicate with partial sensing UEs, other UEs need to know when such UEs can perform reception.

To define the reception occasions for a UE performing partial sensing, it should first be noted that such a UE should not be required to receive at all times. Otherwise, no power saving gain due to partial sensing can be realized.

Observation 3: For a UE performing partial sensing, the reception occasions should be limited in time; otherwise, no power saving gain is expected.

Let us consider a scenario, where UE1 is performing partial sensing, UE2 is performing full sensing, and both UEs are trying to communicate with each other. One example with non-aligned sensing and reception occasions are illustrated in Figure 10 below.

Figure 10: Non-aligned sensing/transmission and reception occasions for a partial sensing UE.

As shown in the figure above, if the sensing/Tx and reception occasions are non-aligned, a UE1 has to perform decoding for a longer duration of time. An alternative, desirable, approach is to align the sensing/Tx and Rx occasions of UE1. This effectively means that a partial sensing UE can receive within the same partial sensing windows that are assigned for sensing/Tx.

Proposal 9: A partial sensing UE performs reception over the set of periodic resources assigned for sensing/transmission.

Finally, as also shown in Figure 10 above, to communicate with a partial sensing UE, its sensing/Tx/Rx occasions should be known by the other UEs.

Proposal 10: Other UEs should know when a partial sensing UE can receive in order to communicate with it.

## Contiguous Partial Sensing for Aperiodic Transmissions

The last identified open issue regarding CPS is when to disable CPS in resource (re)selection. One such case is when the UE does not have sufficient remaining packet delay budget to perform CPS. Another case would be when the UE is in a power constrained state, e.g., low battery. In those cases, the UE could opt to perform random selection instead of partial sensing:

Proposal 11: A UE can disable CPS and perform random selection when there is insufficient remaining PDB or it is low on power.

# Sidelink DRX

Sidelink DRX is agreed to be supported for all cast types by RAN2, and the details are currently being developed. In this section, we aim at addressing some of the PHY related aspects of sidelink DRX.

Similar to the argument made based on Figure 6, if the DRX configurations are non-orthogonal, i.e., if the DRX patterns do not partition the resource pool, the chance of collision, due to not receiving an SCI indicating reservations, increases. This can also be seen using the scenario depicted in Figure 11 below.

Figure 11: Partially overlapping DRX ON durations due to inactivity timer.

Let us consider two UE pairs: UE A is receiving data from UE B and UE C is receiving data from UE D. The DRX patterns for UE A and B are aligned so that they can communicate. The DRX patterns for UE C and D are also aligned. However, across the two pairs, the DRX patterns are partially overlapping. As a result, the earlier reservation made by UE B cannot be received by UE D. As a result, UE D may choose the same resource which leads to collision.

Observation 4: Partially overlapping DRX configurations increase collisions among power saving UEs' transmissions.

Based on the above observation, it is desirable to keep the DRX patterns of different UEs on a resource pool either non-overlapping or fully overlapping. However, as RAN2 has agreed to support inactivity timer for different cast types, some level of partial overlap across different DRX patterns is unavoidable. To keep the collision probability at a reasonable level, therefore, some restrictions on scheduling a transmission outside of an RX UE’s DRX ON duration would be needed. For example, in order to reduce the chance of collisions, only the packets with a numerical priority value below a given threshold could be scheduled outside of the DRX ON duration.

Proposal 12: In order to keep the collision probability manageable, RAN1 should discuss the conditions for scheduling a (re-)transmission outside of the DRX ON duration of the RX UE.

If an Rx UE is configured with DRX and a non-zero inactivity timer, its timer only starts running if it receives an SCI during its ON duration indicating a reservation during the OFF duration. In such a case, the Rx UE stays awake to receive the retransmissions during the extended ON duration. However, if the Rx UE misses the SCI with reservation indication, and if the Tx UE transmits over the reserved resources, not only the transmissions will not be received by the Rx UE, but also the chance of collision increases without any benefit.

Proposal 13: For unicast and groupcast, the Tx UE retransmits on the resources outside of the Rx UE's ON duration only if it receives a NAK in response to the (re)transmission inside the ON duration indicating reservations.

RAN1 agreed that MAC provides PHY with the receiver UE’s DRX active time so that a subset of resources would be selected within that duration:

Agreement

When SL DRX active time of Rx-UE is provided by the higher layer for candidate resource selection (including resource (re)selection and re-evaluation/pre-emption checking), the following working assumption is confirmed with option 2 as agreement (with modification in RED)

Working Assumption (RAN1#106bis-e)

When PHY layer is indicated with an active time of RX UE from MAC layer for candidate resource selection, a restriction is applied in PHY layer so that at least a subset of candidate resources reported to MAC layer is located within the indicated active time of the RX UE. The following options will be further discussed in RAN1 to restrict resources for candidate resource selection taking into account the indicated active time from MAC layer:

- Option 1: PHY layer selects and reports candidate resources only within the indicated active time of the RX UE

- Option 2: PHY layer selects and reports candidate resources in which at least a subset of the candidate resources is within the indicated active time of the RX UE

  - FFS: Details on when the number of subsets of candidate resource is less than the threshold

  - FFS: The subset of candidate resource outside of the active time should consider each inactive time period

  - FFS: UE selection of resource selection window to overlap with indicated RX UE active time

  - FFS: Whether it is up to UE implementation to report candidate resources only within the indicated active time of the RX UE

- Option 3: PHY layer selects and reports an additional candidate resource set of candidate resources within the indicated active time of the RX UE

An important consideration here is how to report only resources that can be utilized by MAC. For example, any resource reported prior to the beginning of the indicated active time cannot be reliably received by the receiver UE since it will be in inactive time and not receiving any signalling. At minimum, PHY should only report resources that start at or after the beginning of the receiver’s active time. Hence, the earliest resources in the reported candidate set should be in the provided active time. How to ensure that this condition is met can be left up to UE implementation.

Proposal 14: The earliest X resources in the candidate resource set SA are in the provided active time. X = [1] or a (pre-)configured value.

Another related issue is handling multiple indicated active times for the same resource selection trigger. A similar issue arises here is that resources between the active times might not be usable if a resource within the earlier active time is not reported to MAC and then selected. Hence to simplify design, the number of indicated active times should be limited to one per resource selection operation.

Proposal 15: MAC only indicates a single active time of a receiver UE to PHY layer for candidate resource selection.

The two remaining FFS items are to discuss how to handle the case when insufficient resources are found in the indicated active time and whether to allow the UE to only select resources within the indicated active time. For the first FFS, PHY could report both the set containing resources within the indicated active time and the set generated using Rel-16 procedure without accounting for the receiver’s active time. MAC would then select resources appropriately. For the second FFS, it is advantageous for the UE to select all resources within the indicated active time, for example when it is important to ensure that the receiver would receive all transmissions of the TB without relying on timer extensions.

Proposal 16: When the number of subsets of candidate resource is less than the threshold, PHY additionally reports the candidate resource set generated according to Rel-16 procedure.

Proposal 17: It is up to UE implementation to report candidate resources only within the indicated active time of the RX UE.

# Conclusion

Observation 1: Performing re-evaluation after random resource selection significantly improves performance compared to random selection without sensing.

Observation 2: The performance of full-sensing UEs in the system is not noticeably impacted when a proportion of UEs is replaced with ones that do not perform sensing.

Observation 3: For a UE performing partial sensing, the reception occasions should be limited in time; otherwise, no power saving gain is expected.

Observation 4: Partially overlapping DRX configurations increase collisions among power saving UEs' transmissions.

Proposal 1: Introduce random selection with subsequent re-evaluation in NR sidelink, at least by UE implementation.

Proposal 2: No special consideration is needed for resources chosen using random selection in a resource pool with full and/or partial sensing.

Proposal 3: For supporting mode 2 resource allocation with partial sensing, define a set of periodic partial sensing resource sets partitioning a resource pool.

Proposal 4: A UE can perform partial sensing over a single or multiple resource sets.

Proposal 5: The reservation of a resource in a given set can only be signalled from another slot associated with the same resource set.

Proposal 6: For a UE performing partial sensing, if the resource selection procedure is triggered in slot n, the resource selection window consists of the slots associated with a given partial sensing window that lie within  [n+T1,n+T2].

Proposal 7: For identifying the availability of resources in a selection window associated with a given partial sensing set in a resource pool with periodic reservation disabled, the sensing information from the past 32 slots that are within the intersection of the sensing window and the same partial sensing set is sufficient.

Proposal 8: For identifying the availability of a slot tySL associated with a given partial sensing set in a resource pool with periodic reservation enabled, in addition to the sensing information for aperiodic reservations as proposed in Proposal 6, the sensing information from tySL-Preserve,  ∀ Preserve ∈sl_ResourceReservePeriodList, is required if tySL-Preserve belongs to the same partial sensing set.

Proposal 9: A partial sensing UE performs reception over the set of periodic resources assigned for sensing/transmission.

Proposal 10: Other UEs should know when a partial sensing UE can receive in order to communicate with it.

Proposal 11: A UE can disable CPS and perform random selection when there is insufficient remaining PDB or it is low on power.

Proposal 12: In order to keep the collision probability manageable, RAN1 should discuss the conditions for scheduling a (re-)transmission outside of the DRX ON duration of the RX UE.

Proposal 13: For unicast and groupcast, the Tx UE retransmits on the resources outside of the Rx UE's ON duration only if it receives a NAK in response to the (re)transmission inside the ON duration indicating reservations.

Proposal 14: The earliest X resources in the candidate resource set SA are in the provided active time. X = [1] or a (pre-)configured value.

Proposal 15: MAC only indicates a single active time of a receiver UE to PHY layer for candidate resource selection.

Proposal 16: When the number of subsets of candidate resource is less than the threshold, PHY additionally reports the candidate resource set generated according to Rel-16 procedure.

Proposal 17: It is up to UE implementation to report candidate resources only within the indicated active time of the RX UE.

# References

- RP-212880, Status report of WI: NR Sidelink enhancement, RAN1.

- Chair’s notes, RAN1 103-e.

- Chair’s notes, RAN1 104-e.

- Chair’s notes, RAN1 104bis-e.

- Chair’s notes, RAN1 105-e.

- Chair’s notes, RAN1 106-e.

# Appendix A:

## Simulation Assumptions

| Sidelink Frequency | 6GHz |
| --- | --- |
| Traffic models | Vehicular UE traffic: Aperiodic traffic with medium Intensity Inter-packet arrival time: 50 ms + an exponential random variable with the mean of 50 ms Packet size: Uniformly distributed between [200, 400, 600, 800, 1000, 1200, 1400, 1600, 1800, 2000] bytes Latency requirement: 50 ms Reservation method: aperiodic. Pedestrian UE traffic: Periodic with 200ms period. Packet size: 300 bytes Latency requirement: 50 ms Reservation method: aperiodic. |
| Simulation Environment | Urban road |
| Number of Tx/Rx Antenna elements | 1Tx/2Rx |
| Antenna Models | Option 1 |
| SL Simulation BW | 40MHz |
| SCS | 30 kHz |
| Pathloss, shadowing, blocking and dual mobility models | Enabled (as per TR 37.885) |
| Number of Transmissions | 3 retransmissions |
| N_MAX | 2 |
| T_3 (timeline) | 2000 us |
| Initial RSRP Threshold | -78 dBm |
| Communication mode | Groupcast option 1 |
| Required Communication Range | 160 m |