3GPP TSG-RAN WG1 Meeting #108-e	R1-2202165

e-Meeting, February 21th – March 3rd, 2022

Agenda item:	8.16.1

Source: 	Qualcomm Incorporated

Title: 	Discussion on FeMIMO UE features

Document for:	Discussion/Decision

- Introduction

Updated RAN1 UE features list for Rel-17 NR after RAN1 #107-e is captured in R1-2200780. In this contribution, we discuss our views on various remaining details of FeMIMO UE features.

- Multi-Beam Operation

Proposal 2-1: For FG 23-1-1 on the joint TCI for intra-cell operation (R1-2200780), suggest to consider the following changes.

- Different combinations of TCI types and intra/inter-cell BM should have different FGs, because communication can work with only 1 combination, e.g. joint TCI + intra-cell BM

  - Separate FG for joint TCI + intra-cell BM

  - Separate FG for joint TCI + inter-cell BM

  - Separate FG for separate TCI + intra-cell BM

  - Separate FG for separate TCI + inter-cell BM

- For component 2, 3, 7, 8-b, 8-c, 9, 11, 12, 13, they are not must-have features, and can be in a separate FG

- For component 4, 5, 6, 8-a, they can be basic features, and can be merged into FG 23-1-1

- For component 4

  - Prefer “per BWP per CC” than “per CC”, which is inaccurate in legacy description to our understanding

  - Prefer “in a band”, which is same as legacy definition

- For component 5

  - Prefer “in a band”, which is same as legacy definition

- For component 8

  - Prefer to keep “mode”, since the 3 sub-components correspond to 3 modes to our understanding

  - Prefer to remove “in case of updates”, which seems redundant

  - Sub-component 8-a can be basic feature, while 8-b and 8-c should be optional

- For component 9, 12, 13, suggest to merge them into a single component as below

  - Support PDSCH-Config which contains a reference to another CC/BWP, in which the PDSCH-Config contains the TCI state list.

    - If support, UE also reports the maximum number of PDSCH-Configs containing TCI states if any PDSCH-Config contains a reference to another CC/BWP

Proposal 2-2: For FG 23-1-2 on the inter-cell beam measurement/report (R1-2200780), suggest to consider the following changes.

- Suggest to remove the bracket on “for both inter-cell BM and mTRP”, since the measurement should be applicable to both as in the updated WID

- For component 10, 11, they are not must-have features, and can be in a separate FG

- For component 2, 3, 7, 8, they can be basic features, and can be merged into FG 23-1-2

- For component 2

  - Prefer to keep current wording, can clarify K can have different candidate values, which is aligned with agreement with Kmax as UE capability

- For component 3

  - Prefer to keep “RRC configured” for clearer definition, or no change is also fine

- For component 4, 5

  - They may not be needed if we have component 12 , 13

- For component 6

  - It may not be needed, since today the serving cell SSB can be simultanouesly configured for L3 and L1 measurement

- For component 9

  - It may not be needed. SSB for L1 measurement can be within or outside SMTC without complexity impact

- For component 10, 11

  - They are needed, since UE cannot measure overlapped SSBs if not supporting two Rx beams simultaneously

  - In addition, 10 can be removed if we have 11 with canaidate value of 0.

- For component 12, 13

  - Suggest to include them to the FG 23-5-1a, or add a note to clarify the resources in FG 23-1-2 are counted in FG 16-1g

Proposal 2-3: For FG 23-1-3 on the MPE mitigation (R1-2200780), prefer to use “P-MPR” instead of “PHR” for the name in yellow

Proposal 2-4: For FG 23-1-4 on the MPUE support on UL (R1-2200780), suggest to consider the following changes.

- For component 1, prefer to keep “set”, which is the wording for agreement as well as current spec

- For the note, it can be revised as the report list only contains a unique candidate value per set

- For the candidate value, support the value of 0 to indicate a panel is DL only. Otherwise, UE cannot report the best DL panel if it is DL only, and that will degrade DL performance

- mTRP PDCCH

In Rel-15, PDCCH monitoring capability is defined for different cases including Case 1-1 for basic capability (FG 3-1), Case 1-2 for single occasion monitoring (FG 3-2), and Case 2 for PDCCH monitoring with span gap (FG 3-5b). Furthermore, Rel-16 introduced additional capabilities (FG 11-2 family). The support of Rel-17 mTRP PDCCH repetition should be separately indicated for each of these PDCCH monitoring capabilities. In other words, the basic FG for Rel-17 mTRP PDCCH repetition should only include the basic PDCCH monitoring capability. This is because PDCCH repetition can significantly increase the UE complexity and some of the advanced PDCCH monitoring features that the UE is able to support in the absence of PDCCH repetition need a separate capability when combined with PDCCH repetition. This enables a UE to support an advance PDCCH monitoring capability in absence of PDCCH repetition while support a more basic PDCCH monitoring capability in the presence of PDCCH repetition.

Furthermore, for the proposed FGs 23-2-1c and 23-2-1d, UE should be able to indicate the supported mode of PDCCH repetition wrt intra-span versus inter-span. In fact, such capability is already agreed:

Agreement

When 3 BDs are counted for two linked candidates

- The third BD is counted in the later span for inter-span PDCCH repetition when r16monitoringcapablityis configured.

- Note: Inter-span repetition is UE optional

Additionally, in RAN1 #107-e, the following was agreed to handle the UE complexity / memory requirement, which needs to be captured entirely by UE capability spec (since the editor of 38.213 clarified that this agreement can be completely taken care of by UE capability spec instead of by 38.213), and is specific to inter-span PDCCH repetition

Agreement

To handle UE complexity / memory requirements for linked PDCCH candidates, address the issue by UE capability, where UE indicates a limit (X) associated with the total number of linked candidates of which the first candidate is received and the second one has not been received at any given span.

- The limit X is indicated as a total count assuming count 1 for AL=1; 2 for AL=2; 4 for AL=4 or 8 or 16.

- The limit X is indicated per CC and also across all CCs

- Note: “received” and “not been received” is wrt the end of the corresponding span of PDCCH candidate.

- Above is applicable at least for the inter-span case (FFS: intra-span case)

Based on the discussions above, and the agreements during the WI mentioned above, we propose the following:

Proposal 3-1:

- The basic FG for Rel-17 mTRP PDCCH repetition (FG 23-2-1) should only include the basic PDCCH monitoring capability, i.e., the highlighted note in square bracket is required (square bracket should be removed).

- PDCCH repetition for more advanced PDCCH monitoring capabilities need a separate capability for each of the following

  - Add FG 23-2-1b for “PDCCH repetition with PDCCH monitoring on any span of up to 3 consecutive OFDM symbols of a slot”

  - Add FG 23-2-1c for “PDCCH repetition for Case 2 PDCCH monitoring with a span gap”

    - Add a component for “Supported mode of PDCCH repetition” with candidate values {intra-span, inter-span, both}

    - Add a component for “X per CC” and “X across all CCs”, where X is defined in the agreement above applicable to inter-span PDCCH repetition

  - Add FG 23-2-1d for “PDCCH repetition for Rel-16 PDCCH monitoring”

    - Add a component for “Supported mode of PDCCH repetition” with candidate values {intra-span, inter-span, both}

    - Add a component for “X per CC” and “X across all CCs”, where X is defined in the agreement above applicable to inter-span PDCCH repetition

Regarding FG 23-2-1a, we think there should not be any dependency on component 2 of FG 23-2-1. This is because for whether the UE performs soft-combining or not and the relationship to number of BDs was extensively discussed in RAN1 and no consensus was achieved. Hence, the note from FG 23-2-1a should be deleted. Furthermore, component 3 is already agreed as shown below, and it needs to be included that this max number is per scheduled CC per slot. However, this number (component 3) should be per scheduled CC “per span” for Rel-16 span-based PDCCH monitoring since BDs and CCEs are also defined per span in this case. Hence, a similar component is also needed for FG 23-2-1d (component 5).

Agreement

When one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET

- Interpretation of the detected DCI is based on Rel. 17 PDCCH repetition rules (wrt reference PDCCH candidate).

  - Whether the individual candidate is monitored or not is determined by a UE capability

    - FFS (In UE feature session): The details including reusing the reported number of BDs for this purpose, or relation to reported number of BDs

  - In both cases, the individual candidate is not counted toward the BD limit.

- UE capability for max number of such overlaps is introduced

  - FFS: Value of 0 is included as a candidate value for the UE capability

  - The details to be discussed as part of UE capability discussions

- FFS: When the individual candidate is monitored, the scenario where the other linked candidate is also “overlapping” (same CORESET, DCI size, CCEs, scrambling) with a second individual candidate

Hence, we propose the following:

Proposal 3-2:

- Remove the note from FG 23-2-1a.

- Remove the “FFS” from component 3 of FG 23-2-1 and add “per slot” at the end, i.e., “Supported max number of overlaps when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate per scheduled component carrier per slot”.

  - Also, the note should be revised to “for component 3, if N PDCCH candidates are overlapped, the number of overlaps each unique pair of overlaps is counted as one”

- A similar component (component 5 in the table below) is needed for FG 23-2-1d as “Supported max number of overlaps when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate per scheduled component carrier per span”

Based on the above discussions, we propose the following FGs for mTRP PDCCH:

Proposal 3-3: Adopt the following for Rel-17 mTRP PDCCH UE features (modifications in red).

| 23. NR_FeMIMO | 23-2-1 | PDCCH repetition | 1. Support of intra-slot PDCCH repetition based on two linked SS sets associated with corresponding CORESETs [with [non-SFN scheme] TDM and FDM (except FR2)] [including PDCCH repetition for Type 3 CSS] 2. Support of reporting one number as required number of BDs for the two PDCCH candidates FFS: 3. Support max number of overlaps when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate per scheduled component carrier per slot |  |  |  |  | Per FS |  |  |  | Component 2 candidate values: details 2 or 3 Component 4 3 candidate values: [{0,1,2,3}] {2,4,6,10,20,40} [Note: UE supports PDCCH repetition for the following (basic) PDCCH monitoring capability: For type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS, the monitoring occasion is within the first 3 OFDM symbols of a slot] [Note: for component 3, if N PDCCH candidates are overlapped, the number of overlaps each unique pair of overlaps is counted as one.] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-2-1a | Monitoring of individual candidates | Support of monitoring of individual candidates when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET | [23-2-1] |  |  |  | Per band |  |  |  | [Note: [If 2 is reported in component 2 of FG 23-2-1,] the individual candidate is monitored when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET for the case that the linked PDCCH candidates is 2 BDs. If 3 is reported in component 2, support of whether the individual candidate is monitored or not when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET] | Optional with capability signalling |
| 23. NR_FeMIMO | 23-2-1b | PDCCH repetition with PDCCH monitoring on any span of up to 3 consecutive OFDM symbols of a slot | Support of PDCCH repetition for PDCCH monitoring on any span of up to 3 consecutive OFDM symbols of a slot | 3-2 23-2-1 |  |  |  | Per Band |  |  | FR1 only |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-2-1c | PDCCH repetition for Case 2 PDCCH monitoring with a span gap | 1. Support of PDCCH repetition for PDCCH monitoring of any occasions with span gap as defined in FG 3-5b. 2. Supported mode of PDCCH repetition 3. X per CC 4. X across all CCs | 3-5b 23-2-1 |  |  |  | Per FS |  |  |  | This capability is necessary for each SCS. Component2: {intra-span, inter-span, both} Component3: FFS candidate values Component 4: FFS candidate values Note: - Components 3 and 4 are reported only if UE supports inter-span PDCCH repetition. - The limit (X) is associated with the total number of linked candidates of which the first candidate is received and the second one has not been received at any given span, where “received” and “not been received” is wrt the end of the corresponding span of PDCCH candidate. - The limit X is indicated as a total count assuming count 1 for AL=1; 2 for AL=2; 4 for AL=4 or 8 or 16. | Optional with capability signalling |
| 23. NR_FeMIMO | 23-2-1d | PDCCH repetition for Rel-16 PDCCH monitoring | 1. Support of PDCCH repetition with Rel-16 PDCCH monitoring capability as defined in FG 11-2 family. 2. Supported mode of PDCCH repetition 3. X per CC 4. X across all CCs 5. Supported max number of overlaps when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate per scheduled component carrier per span | 11-2 23-2-1 |  |  |  | Per FS |  |  |  | This capability is signalled for SCS 15 kHz and 30 kHz. Component2: {intra-span, inter-span, both} Component3: FFS candidate values Component 4: FFS candidate values Note: - Components 3 and 4 are reported only if UE supports inter-span PDCCH repetition. - The limit X is associated with the total number of linked candidates of which the first candidate is received and the second one has not been received at any given span, where “received” and “not been received” is wrt the end of the corresponding span of PDCCH candidate. - The limit X is indicated as a total count assuming count 1 for AL=1; 2 for AL=2; 4 for AL=4 or 8 or 16. Component5: {1,2,3,5,10,20} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-2-2 | Two QCL TypeD for CORESET monitoring in PDCCH repetition | Support of determining two QCL-TypeD for time-domain overlapping CORESETs in the same CC or for intra-band CA when UE is configured with PDCCH repetition [with non-SFN TDM and/or FDM sheme] | 23-2-1 |  |  |  | Per band |  |  | FR2 only |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-2-3 | PDCCH repetition for Type3 CSS | Support of PDCCH repetition for Type3 CSS |  |  |  |  |  |  |  |  |  | Optional with capability signalling |

- mTRP PUSCH

One important remaining aspect is related to CB versus NCB differentiation. A UE should be able to indicate it supports only one of these operations for mTRP while both may be supported for legacy / sTRP. In addition, the existing CB / NCB parameters in Rel-15 corresponding FGs are not enough as two SRS resource sets are introduced for both cases for mTRP PUSCH operation (i.e., for the same number of SRS resources per SRS resource set, the total number of SRS resources is doubled for mTRP PUSCH). Hence, we would like to emphasize the following points:

- Separate and independent capabilities (two FGs) are required for codebook-based and non-codebook based mTRP PUSCH.

  - For codebook-based mTRP PUSCH, the structure of Rel-15 capability (FG 2-14) including indication on max number of SRS resources need to be taken in to account, and should be extended for the case that two SRS resource sets are configured.

  - For non-codebook based mTRP PUSCH, the structure of Rel-15 capability (FGs 2-15, 2-15a, 2-15b) including indication on max number of SRS resources and various capabilities for associated CSI-RS (for precoding calculation) need to be taken in to account, and should be extended for the case that two SRS resource sets / two associated CSI-RS resources are configured.

  - These two FGs should be per FSPC similar to Rel-15 FGs 2-14 and 2-15.

- To ensure that UE capability overhead is not increased too much, the CB and NCB capabilities can be shared between PUSCH repetition Type A and PUSCH repetition Type B as well as for some of the other FGs. This means that either FG 23-3-1 (for CB) or new FG 23-3-1-2 (for NCB) can be a prerequisite for FGs 23-3-1-1 or FGs 23-3-1a, 23-3-1b, …, 23-3-1g.

In addition, we think it would be beneficial to allow the UE to indicated that for CG mTRP PUSCH repetition, whether CG Type 1, CG Type 2, or both are supported.

Based on the discussions above, we suggest the following:

Proposal 4-1: Adopt the following for Rel-17 mTRP PUSCH UE features (modifications in red).

| 23. NR_FeMIMO | 23-3-1 | Multi-TRP PUSCH repetition (type A) [-CB] | 1. Support of multi-TRP PUSCH repetition for codebook based PUSCH (based on PUSCH repetition type A) [for CB] - sequential mapping for repetitions larger than 2 [- cyclic mapping for 2 repetitions] 2. Support of two SRS resource sets with usage set to 'codebook' 3. Supported number of SRS resources in one SRS resource set [2. The maximum number of PHR reporting across all CCs (including those related to M-TRP PUSCH repetition and the legacy Rel-15/16 PUSCH transmission)] [3. Support dynamic switching between multi-TRP PUSCH scheme and single-TRP PUSCH transmission] FFS: Support PUSCH operations: CB based and NCB based and corresponding parameters including number of SRS resources | 2-14 |  |  |  | Per FSPC |  |  |  | [Candidate component values: {CB, non-CB, both}] Component 3: {1,2,4} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-3-1-2 | Multi-TRP PUSCH repetition (type A) - NCB | 1. Support of multi-TRP PUSCH repetition for non-codebook based PUSCH (based on PUSCH repetition type A) 2. Support of two SRS resource sets with usage set to 'nonCodebook' 3. Supported number of SRS resources in one SRS resource set | 2-15 |  |  |  | Per FSPC |  |  |  | Component 3: {1,2,3,4} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-1-2a | Two associated CSI-RS resources | Support of up to two CSI-RS resources associated with the two SRS resource sets for non-codebook-based mTRP PUSCH | 2-15a 23-3-1-2 |  |  |  | Per Band |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-1-2b | CSI-RS processing framework for SRS with two associated CSI-RS resources | 1. Maximum number of periodic SRS resources associated with first or second CSI-RS per BWP 2. Maximum number of aperiodic SRS resources associated with first or second CSI-RS per BWP 3. Maximum number of semi-persistent SRS resources associated with first or second CSI-RS per BWP 4. UE can process Y SRS resources associated with first or second CSI-RS resources simultaneously in a CC. Includes P/SP/A SRS. | 23-3-1-2a |  |  |  | Per Band |  |  |  | Component 1: {1 to 8} Component 2: {1 to 8} Component 3: {1 to 8} Component 4: {1 to 16} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-1a | Cyclic mapping | Support of cyclic mapping when the number of repetitions is larger than 2 for single DCI based M-TRP PUSCH repetition Type A | 23-3-1 or 23-3-1-2 |  |  |  | Per Band |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-1b | Second TPC field | Support of second TPC field for per TRP closed-loop power control for PUSCH with DCI formats 0_1 / 0_2 | 23-3-1 or 23-3-1-2 |  |  |  | Per Band |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-1c | Two PHR reporting | Support of PHR reporting related to M-TRP PUSCH repetition (calculate two PHRs (at least corresponding to the CC that applies m-TRP PUSCH repetitions), each associated with a first PUSCH occasion to each TRP, and report two PHRs.) | 23-3-1 or 23-3-1-2 |  |  |  | Per Band |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-1e | A-CSI report | Support of A-CSI report on two PUSCH repetitions | 23-3-1 or 23-3-1-2 |  |  |  | Per Band |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-1f | SP-CSI report | Support of SP-CSI report on two PUSCH repetitions | 23-3-1 or 23-3-1-2 |  |  |  | Per Band |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-1g | CG PUSCH transmission | 1. Support of CG PUSCH transmission towards M-TRPs using a single CG configuration (Use same beam mapping principals as dynamic grant PUSCH repetition scheme.) 2. Supported type of configured grant for multi-TRP PUSCH repetition | 23-3-1 or 23-3-1-2 |  |  |  | Per Band |  |  |  | Component 2: {Type1, Type2, both} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-1-1 [-CB] | Multi-TRP PUSCH repetition (type B) | 1. Support of multi-TRP PUSCH repetition (based on PUSCH repetition type B) [for CB] FFS: Support PUSCH operations: CB based and NCB based and corresponding parameters including number of SRS resources | 23-3-1 or 23-3-1-2 |  |  |  | Per FS |  |  |  | [Candidate component values: {CB, non-CB, both}] | Optional with capability signalling |

- mTRP PUCCH

The following aspects need to be considered for mTRP PUCCH repetition capability:

- Supported PUCCH format(s) need to be indicated for each of the Scheme 1 and Scheme 3, i.e., a bitmap of size 5 can be added as a component to indicate support for PUCCH formats 0-4 for both 23-3-2 and 23-3-3.

- Updating two beams / power control parameters for a group of PUCCH resources requires a UE capability with a pre-requisite of corresponding Rel-16 feature in the case of one beam (i.e., FG 16-1b-3)

Hence, we have the following proposal:

Proposal 5-1: Adopt the following for Rel-17 mTRP PUCCH UE features (modifications in red).

| 23. NR_FeMIMO | 23-3-2 | Multi-TRP PUCCH repetition | 1. Support of PUCCH repetition scheme 1 (inter-slot repetition) - sequential mapping for repetitions larger than 2 [- cyclic mapping for 2 repetitions] [2. Support of up to two PUCCH spatial relation per PUCCH resource] 2. Supported PUCCH formats for this scheme | FFS |  |  |  | Per FS |  |  |  | Component 2: A bit map of size 5 for PUCCH format 0-4. At least one value of the bitmap should be set to 1. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-3-2b | Cyclic mapping for multi-TRP PUCCH repetition | Support of cyclic mapping for beam mapping/power control parameter set mapping for PUCCH repetitions scheme 1 and/or 3 when the number of repetitions is larger than 2 | FFS 23-3-2 |  |  |  | Per Band |  |  |  | FFS: candidate values | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-2c | Second TPC field for multi-TRP PUCCH repetition | Support of second TPC field for per TRP closed-loop power control for PUCCH with DCI formats 1_1 / 1_2 | FFS 23-3-2 |  |  |  | Per Band |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-3 | Multi-TRP PUCCH repetition-intra-slot | Support of PUCCH repetition scheme 3 (intra-slot repetition) - sequential mapping for repetitions larger than 2 [- cyclic mapping for 2 repetitions] 2. Supported PUCCH formats for this scheme | FFS 23-3-2 25-3 |  |  |  | Per FS |  |  |  | Component 2: A bit map of size 5 for PUCCH format 0-4. At least one value of the bitmap should be set to 1. | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-2d | Updating two Spatial relation or two sets of power control parameters for PUCCH group | Support of updating two Spatial Relation Info’s / two sets of power control parameters for a group of PUCCH resources in a CC by MAC-CE | 16-1b-3 23-3-2 |  |  |  | Per Band |  |  |  |  | Optional with capability signalling |

- Inter-cell mTRP

The following agreement needs to be implemented, and the language of components 2 and 3 should be aligned with this agreement.

Agreement

Support two independent X values (X1, X2) are reported as a UE capability for two different assumptions on additional SSB time domain position and periodicity with respect to serving cell SSB.

- X1 (Case 1)= The maximum number of configured additional PCIs when each configuration of SSB time domain positions and periodicity of the additional PCIs is the same as SSB time domain positions and periodicity of the serving cell PCI

- X2 (Case 2)= The maximum number of configured additional PCIs when the configurations of SSB time domain positions and periodicity of the additional PCIs is not according to Case 1

- Note: By definition, Case 1 and Case 2 cannot be enabled simultaneously

- Supported values for X1 and X2 include at least 0,1,2,3 and 7. FFS on other values

- This UE capability has FR1 and FR2 differentiation (FFS : Whether this UE capability is per UE or per band)

Furthermore, we prefer to add value 0 to Component 2 and add a note that UE needs to indicate a non-zero value for at least one of component 2 or component 3. This allows UE to support either Case 1 or Case 2, which have different complexity with respect to rate matching as well as L1-RSRP measurements.

Hence, we have the following proposal:

Proposal 6-1: Adopt the following for Rel-17 inter-cell mTRP UE features (modifications in red).

| 23. NR_FeMIMO | 23-4 | IntCell-mTRP | 1. Support of RRC configuration of additional PCI different from serving cell associated with the TCI state and/or QCL-info [2. The maximum number of configured additional PCIs is X1 (Case 1) when each configuration of SSB time domain positions and periodicity of the additional PCIs is the same as SSB time domain positions and periodicity of the serving cell PCI] [3. The maximum number of configured additional PCIs is X2 (Case 2) when the configurations of SSB time domain positions and periodicity of the additional PCIs is not according to Case 1 different with SSB time domain positions and periodicity of the serving cell PCI] |  |  |  |  | Per Band |  |  |  | [Component 2 candidate values: {0,1,2,3,7}] [Component 3 candidate values: {0,1,2,3,7}] Note: UE indicates a non-zero value for at least one of component 2 or component 3 [Note: case1 and case2 cannot be enabled simultaneously] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- mTRP BM

Proposal 7-1: For FG 23-5-1 on the enhanced group based L1-RSRP report (R1-2200780), suggest to consider the following changes.

- For component 3, 4

  - Suggest to include them to the FG 23-5-1a, or add a note to clarify the resources in FG 23-5-1 are counted in FG 16-1g

Proposal 7-2: For FG 23-5-2 on the mTRP BFR enhancement (R1-2200780), suggest to consider the following changes.

- For component 2

  - Suggest to remove the bracket, and move it to be the 1st component

- For component 6

  - It can be removed and add candidate value of 0 for the 1st component of the FG 23-5-2a

Proposal 7-3: For FG 23-5-2a on the PUCCH-BFR resources for mTRP BFRQ (R1-2200780), suggest to consider the following changes.

- For component 2

  - Suggest to remove the bracket based on the agreement on UE capability for association. The agreement is simple and clear, and should not have any ambiguity. We can also check with companies involved in the lengthy WI online discussion

Agreement

Support to configure an association between a BFD-RS set on SpCell and a PUCCH-SR resource / SR configuration for per TRP BFR.

FFS: Configure an association between a BFD-RS set on SCell and a PUCCH-SR resource / SR configuration for per TRP BFR

A UE capability signaling is introduced for indicating the support of this association. Above applies only for multi-DCI case.

- mTRP HST-SFN

Table 1 is a summary of the supported combination of SFN transmission schemes for both HST and URLLC use-case. SFN scheme A (scheme 1) can be supported for both PDSCH + PDCCH, or SFN PDSCH + single TPR PDCCH.

Table 1: PDSCH and PDCCH transmission mode in SFN mode

|  |  | PDSCH |  |  |  |
| --- | --- | --- | --- | --- | --- |
| PDCCH |  | Rel-15 | SFN scheme A | SFN scheme B |
|  | Rel-15 | N/A | Supported | Supported |
|  | SFN scheme A | Supported (URLLC) | Supported | No supported |  |
|  | SFN scheme B | Not support | Not supported | Supported |

## FG 23-6-1

FG 23-6-1 indicates the support of SFN scheme A (scheme 1) for both PDCCH and PDSCH while FG 23-6-1b indicates the support of SFN scheme A only for PDSCH. The description of both FGs should be updated by removing most of the yellow text. In addition, a new FG 23-6-1c should be added for the indication of the support the combination for Rel-17 SNF PDCCH scheme 1 and single-TPR PDSCH based on RAN1 agreement in  RAN1 meeting #106bis-e.

Agreement Support combination of Rel-17 SFN PDCCH scheme 1 and single-TRP PDSCH - This is optional UE feature - Note: The support of such combination scheme is for URLLC use-case only.

Proposal 8-1: For FG 23-6-1, remove the yellow text of “[single TRP/]” and “[and default QCL assumption with one or two TCI states for PDCCH]”

- Support per FS reporting granularity.

Proposal 8-2: For FG 23-6-1b, remove the yellow text of “[and default QCL assumption with one or two TCI states for PDCCH]” and update the description to reflect the support of SFN PDSCH scheduled by single TRP PDCCH.

- Support per FS reporting granularity.

Proposal 8-3: Add new FG 23-6-1c for the support of SFN scheme A PDCCH and single TRP PDSCH.

- Support per FS reporting granularity.

Proposal 8-4: The prerequisite of FG 23-6-1a should be FG 23-6-1 or FG 23-6-1b

## FG 23-6-2

FG 23-6-2 indicates the UE support of SFN scheme B (TRP pre-compensation) for both PDCCH and PDSCH while FG 23-6-2b indicates the support of SFN scheme A only for PDSCH. The description of both FGs should be updated by removing most of the yellow text that is not clear. In addition, SFN scheme B was introduced to reduce UE demod complexity as compared to SFN scheme A. It doesn’t make to make sense to make SFN scheme A (high demod complexity) as prerequisite to support SFN scheme B (lower complexity and simpler demod).

Proposal 8-5: For FG 23-6-2, remove the yellow text of “[and default QCL assumption with one or two TCI states for PDCCH]”

- Support per FS reporting granularity.

- Remove the prerequisite of FG 23-6-1

Proposal 8-6: The prerequisite of FG 23-6-2a should be FG 23-6-2 or FG 23-6-2b.

Proposal 8-7: For FG 23-6-2b, remove the yellow text of “[and default QCL assumption with one or two TCI states for PDCCH]” and update the description to reflect the support of SFN scheme B PDSCH scheduled by single TRP PDCCH.

- Support per FS reporting granularity.

- Remove the prerequisite of FG 23-6-1

## FG 23-6-4

There is missing FG for the UE capability of supporting of two QCL TypeD prosperities for multiple overlapping CORESETs when a CORESET is activated with two TCI as per RAN1-agreement below

Agreement When a CORESET is activated with two TCI states which overlaps with another CORESET, support extension of Rel-15 prioritization rule for PDCCH monitoring of PDCCH candidates in overlapping monitoring occasions with different QCL-TypeD - FFS: Prioritization rule considers CORESETs indicated with 1 and/or 2 TCI states - Supports identifying two QCL-TypeD properties for multiple overlapping CORESETs - UE capability is introduced - FFS other details - FFS: Strive to have same / similar solution as discussed under AI 8.1.2.1

Proposal 8-8:  Add FG 23-6-4b for the indication of Two QCL TypeD properties determination for overlapping CORESETs.

Proposal 8-9: Adopt the following for Rel-17 HST-SFN UE features (with highlighted changes/edit in blue)

| 23. NR_FeMIMO | 23-6-1 | SFN scheme A (scheme 1) for PDSCH and PDCCH | 1. Support of SFN scheme A for PDCCH scheduling [single TRP/] SFN Scheme A PDSCH [and default QCL assumption with one or two TCI states for PDCCH] |  |  |  |  | Per FS |  |  |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-6-1a | Dynamic switching - scheme A | Support of dynamic switching between single-TRP and PDSCH SFN scheme A by TCI state field in DCI formats 1_1, 1_2 | 23-6-1 or 23-6-1b |  |  |  | Per FS |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-6-1b | SFN scheme A (scheme 1) for PDSCH only | 1. Support of SFN scheme A for PDSCH [only scheduled by [single TRP/ Scheme A] PDCCH] [and default QCL assumption with one or two TCI states for PDSCH] |  |  |  |  | Per FS |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-6-1c | SFN scheme A for PDCCH for URLLC | Support of SFN scheme A for PDCCH and single TRP PDSCH |  |  |  |  | Per FS |  |  |  |  |  |
| 23. NR_FeMIMO | 23-6-2 | SFN scheme B (TRP based pre-compensation) for PDSCH and PDCCH | 1. Support of SFN scheme B for PDCCH scheduling SFN Scheme B PDSCH [and default QCL assumption with one or two TCI states for PDCCH] | [23-6-1] |  |  |  | Per FS |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-6-2a | Dynamic switching - scheme B | Support of dynamic switching between single-TRP and PDSCH SFN scheme B by TCI state field in DCI formats 1_1, 1_2 | 23-6-2 or 23-6-2b |  |  |  | Per FS |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-6-2b | SFN scheme B (TRP based pre-compensation) for PDSCH only | 1. Support of SFN scheme B for PDSCH [only and default QCL assumption with two TCI states for PDSCH] [scheduled by [single TRP/Scheme B] PDCCH] | [23-6-1] |  |  |  | Per FS |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-6-3 | Simultaneous activation of two TCI states for PDCCH across multiple CCs (HST/URLLC) | Support of simultaneous activation of two TCI states for CORESETs with the same CORESET ID in all BWPs across a set of configured component carriers by single MAC-CE | 23-6-1 or 23-6-2 |  |  |  | Per UE |  | Yes |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-6-4 | Default DL beam setup for SFN | 1. Support of PDSCH reception using default beam for Rel-17 enhanced SFN scheme when PDSCH is scheduled with offset less than threshold 2. Support PDSCH reception using default beam for Rel-17 enhanced SFN scheme when TCI field is not present in DCI when PDSCH is scheduled with offset equal or larger than the threshold 3. Support aperiodic CSI-RS reception using default beam for Rel-17 enhanced SFN scheme when scheduling offset is less than threshold | [23-6-1, 23-6-2] |  |  |  | Per band |  | FR2 only |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-6-4a | Default UL beam setup for SFN | 1. Support of single-TRP PUCCH transmission using default beam when enhanced SFN PDCCH transmission scheme is configured 2. Support of single-TRP PUSCH transmission using default beam when enhanced SFN PDCCH transmission scheme is configured 3. Support of single-TRP SRS resource transmission using default beam when enhanced SFN PDCCH transmission scheme is configured | [23-6-1, 23-6-2] |  |  |  | Per band |  | FR2 only |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-6-4b | Two QCL TypeD properties for SFN PDCCH | Supports identifying two QCL-TypeD properties for multiple overlapping CORESETs when a CORESET is activated with two TCI states. |  |  |  |  | Per Band |  | FR2 only |  |  | Optional with capability signalling |

- SRS enhancement

## FG 23-8-1

The description of the feature could be clarified as “Support of the maximum number of configured available slots offsets for determining aperiodic SRS location based on available slot. The description of the second component is no clear whether it means the maximum configured value of available slot offset “t” or the maximum span of physical slot for the available slot.

Proposal 9-1: Remove the square bracket and update the description of FG 23-8-1 as “Support of the maximum number of configured available slots offsets for determining aperiodic SRS location based on available slot”

- Candidate values of {1, 2, 4}.

Proposal 9-2: Further clarity the description of the second component “Maximum actual slots offset” whether it means the maximum configured value of available slot ‘t’ or the maximum span of physical slots from the reference slot.

## FG 23-8-2

Proposal 9-3: Support per-band reporting granularity for FG 23-8-2.

## FG 23-8-3

The candidate values for component 1 should have independent xTyR configuration = {1T8R, 1T6R, 2T8R, 2T6R, 4T8R}. UE should report one of {1T6R, 1T8R, 2T6R, 2T8R, 4T6R, 4T8R} similar to basic feature FG 2-55. There is no RAN1 agreement to support a downgraded (or combo) SRS switching configuration. If the UE wants to support downgraded configuration, this can be done by the combination of this FG 38-8-3 and Rel-16 (supportedSRS-TxPortSwitchv1610).

Proposal 9-4: The candidate values for component 1 should be independent xTyR configuration of {1T8R, 1T6R, 2T8R, 2T6R, 4T8R}.

- Note: Downgraded SRS antenna switching configuration could be achieved by the combination of FG 38-8-3 and Rel-16 (supportedSRS-TxPortSwitchv1610).

- Support per-band perBC reporting granularity.

## FG 23-8-4

Proposal 9-5: Support reporting granularity of per-FS for FG 23-8-4 to align with reporting type of the prerequisite 2-53.

## FG 23-8-9

In RAN1 meeting #106bis-e, it was agreed to extend the number of aperiodic SRS sets for legacy xTyR configuration as separate UE feature. A separate UE feature 23-8-9 should be added to reflect the RAN1 agreement.

Agreement For extension of aperiodic antenna switching SRS configurations for <=4Rx, support N=4 for 1T4R and N=2 for 1T2R/2T4R. - The above extension is UE optional.

Proposal 9-6: Add FG 23-8-9 supporting the extension of maximum number of sets for aperiodic SRS antenna switching configuration for <= 4Rx

Proposal 9-7: Adopt the following for Rel-17 SRS-enhancement UE features (proposed changed/edits highlighted in blue).

| 23. NR_FeMIMO | 23-8-1 | SRS triggering offset enhancement | 1. [Support of / The maximum number of configured available slots offsets for] determining aperiodic SRS location based on available slot [2. Maximum actual slots offset] | 2-52 |  |  |  | Per band |  |  |  | Candidate 1 component values: {1, 2, [3,] 4} [Candidate 2 component values: FFS] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-8-2 | Triggering SRS only in DCI 0_1/0_2 | Support of triggering SRS in DCI 0_1/0_2 without data and without CSI | 2-52 |  |  |  | per band |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-3 | SRS Antenna switching for >4Rx | 1. Support of SRS antenna switching xTyR with y>4 2. Report whether the antenna switching impact to downlink receiving in a band 3. Report whether the antenna is switched together with UL Tx in another band | 2-55 |  |  |  | Per band per BC |  |  |  | Component 1 candidate values: {1T8R, 1T6R, 2T8R, 2T6R, 4T8R}. Component 2 candidate values: FFS Component 3 candidate values: FFS FFS for component 2&3: impact when UE reports component 1 as xTyR with x=y | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-4 | Maximum 2 SP and 1 periodic SRS sets for antenna switching | Support of maximum 2 SP SRS resource sets and maximum 1 periodic SRS resource set for antenna switching | 2-53 |  |  |  | [Per FS] |  |  |  | Note1: - Applies for all supported xTyR where y<=8 - For xTyR where y>4, if UE does NOT support this feature, support maximum one SRS resource set for periodic SRS and maximum one SRS resource set for semi-persistent SRS - For xTyR where y<=4, if UE does not support this feature, follow Rel-15 on the number of resource sets for periodic and semi-persistent SRS - The two SP-SRS resource sets are not activated at the same time | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-5 | Increased repetition for SRS | Support of increased repetition patterns (8, 10, 12, 14 symbols) for SRS resource | 10-11, 2-52 |  |  |  | Per band |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-6 | Partial frequency sounding of SRS | Support of partial frequency sounding for SRS | 2-52 |  |  |  | Per band |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-7 | Start RB location hopping for partial frequency SRS | Support of start RB location hopping in partial frequency SRS transmission across different SRS frequency hopping periods for periodic/semi-persistent/aperiodic SRS | 23-8-6 |  |  |  | Per band |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-8 | Comb-8 SRS | Support of comb-8 for SRS other than for positioning |  |  |  |  | Per band |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-9 | extension of maximum number of SRS sets for antenna switching | Support the extension of number of sets (N) for aperiodic SRS antenna switching to N = 4 for 1T4R, N = 2 for 1T2R/2T4R. | FG 2-55 |  |  |  | Per band per BC |  |  |  |  | Optional with capability signalling |

- mTRP CSI

The main issue to address based on the discussions in the previous meeting is that for a given NCJT measurement hypothesis, in order to properly take into account inter-layer interference, a joint search is needed across the codebooks associated with two CMRs. This complexity is obviously more than just double of a given sTRP measurement hypothesis. For example, if UE evaluates 8 PMI/RI hypos for a given sTRP CSI, UE may need to evaluate up to 8*8=64 hypos for a given NCJT CSI for optimal results and not just 16 hypos, where the exact additional complexity may depend on UE implementation. In order to address the UE complexity issue in the presence of NCJT CSI (across all CCs and all CSI report settings even when some of them are not configured with NCJT CSI), we see two possible Alts:

- Alt1: Separate the CPU/resource/port occupation budget for NCJT CSI versus sTRP CSI.

  - In component 4 of FG 23-7-1, the values of a, c, and f (currently in bracket) are included in each reported list

  - Components 5 and 6 (currently in bracket) are also needed.

- Alt2: Allow a UE to report a different list of triplets when CSI for both NCJT and sTRP CSIs are configured for different codebook combinations of sTRP CSI

  - The existing FG 23-7-1 is to report capability for NCJT only (i.e., in the absence of sTRP CSI). Hence, in component 4 of FG 23-7-1, the values of a, c, and f are removed, and component 5 and 6 are changed to report one value for NCJT only (instead of a list for both NCJT and sTRP).

  - Add a new FG 23-7-1b so that UE can report list of triplets for different codebook combinations of the form of (NCJT, one or more codebooks for sTRP).

    - In order to limit the number of codebook combinations, it is reasonable to bundle NCJT CSI (which is defined only for Type1 SP codebook) and sTRP CSI for Type1 SP codebook. This corresponds to codebook 1=” NCJT+Type 1 SP (for sTRP)”

    - To further limit the number of codebook combinations, the possible choice for {codebook2, codebook3} can be limited to more useful/practical cases. Also, a {Null, Null} possibility is needed, i.e., to report triplets for {NCJT+Type 1 SP (for sTRP), Null, Null}.

Even though our preference is still Alt1 due to more flexibility for UE capability signalling, Alt2 can be accepted as a compromise so that additional discussions wrt mixed codebooks can be avoided as Alt2 already captures it.

In addition to the discussions above (regarding Alt1 versus Alt2 to address the UE complexity associated with NCJT CSI), the following points need to be considered:

- Candidate values for component 2 of FG 23-7-1 needs to be {2,3,4,5,6,7,8}. A value smaller than 2 is not needed since at least 2 CMRs are needed for NCJT. Also, both {2,3} should be included as they are valid configuration.

- Candidate values for component 3 of FG 23-7-1 should be {mode 1 with X=0, mode 2, both}. In the corresponding agreement, we have “for the UE supporting option 1”, which implies that mode 1 is not mandatory for mTRP CSI.

- A component 7 should be added to FG 23-7-1, which is to indicate the supported codebook mode for NCJT CSI. Note that for sTRP Type1 SP, UE reports the supported codebook mode through component 2 of FG 2-36.

- FG 23-7-1 (and/or FG 23-7-1b in Alt2 discussed above) should be reported per band and separately per band combination. This is same as legacy behaviour for reporting triplets for each codebook / codebook combination per band and separately per band combination.

- FG 23-7-3 is not needed because it is same as the agreed component 2 of FG 23-7-1.

- FG 23-7-2 may not be needed assuming that the complexity issue for NCJT CSI is handled through one of Alt1 or Alt2 discussed above:

Proposal 10-1: Adopt the following for Rel-17 mTRP CSI UE features (modifications in red). Both Alt1 and Alt2 discussed above are included in the table below from which one of them needs to be selected.

| 23. NR_FeMIMO | 23-7-1 (Alt1) | Basic Features of CSI Enhancement for Multi-TRP | - Support of NZP CSI-RS resource pairs used as CMR (channel measurement resource) pairs for NCJT measurement hypothesis: Support of N=1 - Maximum number of NZP CSI-RS resources in one CSI-RS resource set: Ks,max - CSI report mode [selection] of mode 1 with X=0 [and/or] mode 2 - A list of [supported combinations, up to 16, across all CCs simultaneously, where each combination is] - [Maximum number of Tx ports in one NZP CSI-RS resource associated with a single-TRP measurement hypothesis] - Maximum number of Tx ports in one NZP CSI-RS resource associated with an NCJT measurement hypothesis - [Maximum total number of CMRs for single-TRP measurement] [per CC/across all CCs] - Maximum total number of CMRs for NCJT measurement [per CC/across all CCs] - [Maximum total number of Tx ports of NZP CSI-RS resources associated with single-TRP measurement hypotheses] [per CC/across all CCs] - Maximum total number of Tx ports of NZP CSI-RS resources associated with NCJT measurement hypotheses [per CC/across all CCs] - [Maximum total number of Tx ports of NZP CSI-RS resources associated with one NCJT measurement] - [A list of (Y1,Y2): UE can process Y1 NCJT CSI and Y2 sTRP CSI measurement hypothesis simultaneously in a CC] - [A list of (X1,X2): UE can process X1 NCJT CSI and X2 sTRP CSI measurement hypothesis simultaneously across all CCs] - Supported codebook modes for NCJT CSI | FFS |  |  |  | [Per band and per BC] |  |  |  | [Component 2 candidate value set: {[0, 2, 3,] 4, 5, 6, 7, 8}] [Component 3 candidate value set: { mode 1 with X=0, mode 2, both}] Component 5 4 candidate values: - [{2, 4, 8, 12, 16, 24, 32}] - {2, 4, 8, 12, 16[, 24, 32]} - [{1,2,3,4 … 64}] - {2,3,4 … 64} - [{4,5,6, …, 256}] - {2,3,4, …, 256} [Component 6 5: The list can have maximum of 16 pairs. - Y1: {1 to 4} - Y2: {1 to 8}] [Component 7 6: The list can have maximum of 16 pairs. - X1: {1 to 16} - X2: {1 to 32}] Component 7:{ mode 1, both mode 1 and mode 2} Note: ‘NCJT’ is not used in RAN1 specifications and will be aligned with 38.214 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-7-1 (Alt2) | Basic Features of CSI Enhancement for Multi-TRP | - Support of NZP CSI-RS resource pairs used as CMR (channel measurement resource) pairs for NCJT measurement hypothesis: Support of N=1 - Maximum number of NZP CSI-RS resources in one CSI-RS resource set: Ks,max - CSI report mode [selection] of mode 1 with X=0 [and/or] mode 2 - A list of [supported combinations, up to 16, across all CCs simultaneously, where each combination is] - [Maximum number of Tx ports in one NZP CSI-RS resource associated with a single-TRP measurement hypothesis] - Maximum number of Tx ports in one NZP CSI-RS resource associated with an NCJT measurement hypothesis - [Maximum total number of CMRs for single-TRP measurement] [per CC/across all CCs] - Maximum total number of CMRs for NCJT measurement [per CC/across all CCs] - [Maximum total number of Tx ports of NZP CSI-RS resources associated with single-TRP measurement hypotheses] [per CC/across all CCs] - Maximum total number of Tx ports of NZP CSI-RS resources associated with NCJT measurement hypotheses [per CC/across all CCs] - [Maximum total number of Tx ports of NZP CSI-RS resources associated with one NCJT measurement] - [A list of (Y1,Y2): UE can process Y1 NCJT CSI and Y2 sTRP CSI measurement hypothesis simultaneously in a CC] - [A list of (X1,X2): UE can process X1 NCJT CSI and X2 sTRP CSI measurement hypothesis simultaneously across all CCs] - Supported codebook modes for NCJT CSI | FFS |  |  |  | [Per band and per BC] |  |  |  | [Component 2 candidate value set: {[0, 2, 3,] 4, 5, 6, 7, 8}] [Component 3 candidate value set: { mode 1 with X=0, mode 2, both}] Component 5 4 candidate values: - [{2, 4, 8, 12, 16, 24, 32}] - {2, 4, 8, 12, 16[, 24, 32]} - [{1,2,3,4 … 64}] - {2,3,4 … 64} - [{4,5,6, …, 256}] - {2,3,4, …, 256} [Component 6 5: The list can have maximum of 16 pairs. - Y1: {1 to 4} - Y2: {1 to 8}] [Component 7 6: The list can have maximum of 16 pairs. - X1: {1 to 16} - X2: {1 to 32}] Component 7:{ mode 1, both mode 1 and mode 2} Note: ‘NCJT’ is not used in RAN1 specifications and will be aligned with 38.214 | Optional with capability signalling |
| 23. NR_FeMIMO | 23-7-1b (Alt2) | Active CSI-RS resources and ports in the presence of multi-TRP CSI | - List of codebook combinations - List of {max number of ports per resource, max number of resources, max number of total ports} for each codebook combination | 23-7-1 |  |  |  | Per band and per BC |  |  |  | Component 1 candidate values: Codebook 1 = {NCJT+Type 1 SP (for sTRP)} {Codebook 2, Codebook 3} = {(NULL, NULL}), (Type II, NULL), (eType II R=1, NULL), (eType II R=2, NULL), …, (FeType II PS M=1, NULL), (FeType II PS M=2 R=1, NULL), …} Component 2 candidate values: - Maximum 16 triplets for each codebook combination - Max # of Tx ports in one resource: {2, 4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256} Note 1: A CMR pair configured for NCJT will be counted as two activated resources, a CMR configured for sTRP will be counted as one activated resource for a triplet. Note2: This capability is relevant only when UE is configured with NCJT CSI in at least one CSI report setting in at least one CC in the band and/or band combination. | Optional with capability signalling |
| 23. NR_FeMIMO | 23-7-1a | Additional CSI report mode 1 selection | Maximum value of numberOfSingleTRP-CSI-Mode1 | 23-7-1 |  |  |  | Per Band |  |  |  | Component 1 candidate value set: { X=1, X=2} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-7-2 | Support of max # of Tx ports [per source/across two CMRs] [in a resource set for Multi-TRP CSI] [and max # resources] | [A list of supported combinations, each combination is {max # of Tx ports per source in a resource set for Multi-TRP CSI, max # resources in a resource set for Multi-TRP CSI}] [Note: same number of ports among CMRs] |  |  |  |  |  |  |  |  | [{4, 8, 12, 16, 24, 32}] | Optional with capability signalling |
| 23. NR_FeMIMO | 23-7-3 | More than two resources in a resource set for Multi-TRP CSI | FFS exact candidate values, Ks,max is up to 8 |  |  |  |  |  |  |  |  | [candidate values are {2, 3, 4, 5, 6, 7, 8}] | Optional with capability signalling |
| 23. NR_FeMIMO | 23-7-4 | Support of Nmax=2 for Multi-TRP CSI | Support of maximum number of CMR pairs Nmax=2 configured in NZP-CSI-RS-ResourceSet for a given CSI report setting | 23-7-1 |  |  |  | Per band |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-7-5 | CMR sharing | Support a NZP CSI-RS resource referred by both a CMR pair configured for NCJT measurement hypothesis and a CMR configured for Single-TRP measurement hypothesis | 23-7-1 |  |  |  | Per band |  | FR2 only |  | Note: ‘NCJT’ and ‘single-TRP’ are not used in RAN1 specifications and will be aligned with 38.214 | Optional with capability signalling |

- FDD CSI

The basic feature and sub features for FeType II PS codebook were agreed in last meeting. The remaining issue lies in the codebook concurrency capability, i.e., FG23-9-5. Regarding FG23-9-5, the following is agreed (except the yellow and red text) in RAN1 #107bis-e.

| 23-9-5 | Active CSI-RS resources and ports for mixed codebook types in any slot | - List of codebook combinations - List of {max number of ports per resource, max number of resources, max number of total ports} for each codebook combination | 23-9-1, 16-3a, 16-3b, 16-3a-1, 16-3b-1, 2-36, 2-40, 2-41, 2-43 |  |  |  | Per band and per BC |  |  |  | Component 1 candidate values: Codebook 1 = {Type I SP, Type I MP} {Codebook 2, Codebook 3} = {{FeType II PS M=1, NULL},{FeType II PS M=2 R=1, NULL}[, {FeType II PS M=2 R=2, NULL}], {Type II, FeType II PS M=1}, {Type II, FeType II PS M=2 R=1} ,{eType II R=1, FeType II PS M=1},{eType II R=1, FeType II PS M=2 R=1}} FFS: The list of {Codebook 2, Codebook 3} may be refined depending on outcome of discussion for FG 23-9-2 Component 1 and FG 23-9-4 Component 2 Component 2 candidate values: - Maximum 16 triplets for each codebook combination - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256} Note 1：if a UE reports one or more codebook combinations in 23-9-5, then usage of active CSI-RS resources and ports for multiple codebooks in any slot is allowed only within those combinations Note 2: For coexisting of mixed codebooks in any slot, gNB need to honor 16-8, 23-9-5 and per-codebook capability 2-36/40/41/43, 16-3a/b and 16-3a-1/16-3b-1, and 23-9-1/23-9-2/23-9-4 [Note: If the list of triplets for a codebook combination is empty then the list of triplets is reused from the codebook combination indicated previously] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

In our view, following aspects should be considered

  - Since CSI-RS triplets are reported in FG23-9-4, codebook combination of FeType II PS M=2 R=2 should be added to avoid underreporting. Besides, we don’t see strong motivation in adding combination of 3 codebooks in the list as it is not typical use case. The total number of candidate codebook combinations should be considered small and only important combinations should be added into the candidate list.

  - FG23-9-2 and FG2-9-4 should be added as prerequisite because the codebook combination list includes FeTypeII PS M=2 R=1 and FeType II PS M=2 R=2.

  - The last note in yellow text is not needed.

Proposal: Adopt the following for Rel-17 CSI codebook concurrency UE feature (modifications in red)

| 23-9-5 | Active CSI-RS resources and ports for mixed codebook types in any slot | List of codebook combinations List of {max number of ports per resource, max number of resources, max number of total ports} for each codebook combination | 23-9-1, 16-3a, 16-3b, 16-3a-1, 16-3b-1, 2-36, 2-40, 2-41, 2-43, 23-9-2, 23-9-4 |  |  |  | Per band and per BC |  |  |  | Component 1 candidate values: Codebook 1 = {Type I SP, Type I MP} {Codebook 2, Codebook 3} = {{FeType II PS M=1, NULL},{FeType II PS M=2 R=1, NULL}, {FeType II PS M=2 R=2, NULL}, {Type II, FeType II PS M=1}, {Type II, FeType II PS M=2 R=1} ,{eType II R=1, FeType II PS M=1},{eType II R=1, FeType II PS M=2 R=1}} FFS: The list of {Codebook 2, Codebook 3} may be refined depending on outcome of discussion for FG 23-9-2 Component 1 and FG 23-9-4 Component 2 Component 2 candidate values: - Maximum 16 triplets for each codebook combination - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256} Note 1：if a UE reports one or more codebook combinations in 23-9-5, then usage of active CSI-RS resources and ports for multiple codebooks in any slot is allowed only within those combinations Note 2: For coexisting of mixed codebooks in any slot, gNB need to honor 16-8, 23-9-5 and per-codebook capability 2-36/40/41/43, 16-3a/b and 16-3a-1/16-3b-1, and 23-9-1/23-9-2/23-9-4 [Note: If the list of triplets for a codebook combination is empty then the list of triplets is reused from the codebook combination indicated previously] | Optional with capability signalling |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- References

- R1-2200780, Updated RAN1 UE features list for Rel-17 NR after RAN1 #107bis-e, Moderators (AT&T, NTT DOCOMO, INC.).

- R1-2106431, LS on UL MIMO coherence for Tx switching between two carriers, RAN1-106e.