3GPP TSG RAN WG1 #108-e			R1-2202168

e-Meeting, February 21st – March 3rd, 2022

Agenda item:	8.16.4

Source: 	Qualcomm Incorporated

Title: 	UE features for NR NTN

Document for:	Discussion/Decision

# Introduction

This document includes the discussion about how to indicate legacy UE features in NTN and comments on the preliminary RAN1 UE features provided after RAN1 107bis-e [1].

# Discussions

How to indicate the support of legacy UE features in NTN is an important question concerning NTN UE features and capability signalling. NR NTN is developed with the assumption that any legacy NR feature, if needed, is supported. However, not all legacy UE features are applicable. Two options are:

- If a separate NTN container is used, legacy features applicable to NTN need to be identified. This approach however does not promote the sharing of UE features between TN and NTN networks.

- If NR container is used, all NTN features must be per band or per band combination by default. Some legacy per-UE features may require different capability indications in NTN or may not be applicable to NTN.  In such case, we may create a separate column in UE feature table such as /TN/NTN differentiation; or declare some of the features not applicable to NTN bands.

To answer the above questions, we have the following proposal:

Proposal 1:

- NTN uses the NR capability container.

- Legacy NR features are applicable to NTN by default and exceptions are handled only when requested.

- NTN UE features should be at least per band differentiated so that NTN and non-NTN capabilities can be independently set.

# UE Features

After RAN1 107bis-e, preliminary RAN1 UE features are summarized in [1]. Based on the latest summary, additional comments on UE features for NR-NTN are provided below.

FG Granularity

Since NTN is designed with the assumption that legacy NR features are supported in NTN whenever needed, NR container should be used for NTN features. As such, NTN features should by default be per band or per band combination.

FG 26-1

- UE specific TA and common TA cannot be calculated separately. Consequently, items 1 and 2 should be merged as:

- UE specific TA and common TA calculation calculation based on its GNSS-acquired position, the serving satellite ephemeris, and common TA parameters. UE considers common TA as 0 if the parameter is not provided.

- Existing Item 4 is not needed as Item 3 already implies it.

- The FG should be identified as “ UL time & frequency pre-compensation”.

FG 26-3

The FG does not have pre-requisites.

FG 26-6

The FG is needed only when UE supports DL HARQ feedback disabling, which is a RAN2 feature.  We may have the FG description as:

- Support of type-2 HARQ codebook enhancements when per HARQ process feedback disabling is supported

FG 26-6a

- The FG is needed only when UE supports DL HARQ feedback disabling, which is a RAN2 feature.  We may have the FG description as:

- Enhancement on Type-1 HARQ codebook when per HARQ processes feedback disabling is supported

- This FG should not merge with FG 26-6 as a UE may support one but not the other.

- This FG does not have pre-requisites.

FG 26-8

- The feature is only intended for enhancement of operation and is not needed for basic operation. The consequence if the feature is not supported should be:

UE is not able to take the advantage of polarization information to save power.

- UE does not have to support it to access NTN. The related statement should be removed.

# Conclusions

We have provided inputs of UE features for NR NTN based on updated features list after RAN1107 bis-e. We also have the flowing proposal concerning UE feature and capability signaling.

Proposal 1:

- NTN uses the NR capability container.

- Legacy NR features are applicable to NTN by default and exceptions are handled only when requested.

- NTN UE features should be at least per band differentiated so that NTN and non-NTN capabilities can be independently set.

# References

- R1-2200780, “Updated RAN1 UE features list for Rel-17 NR after RAN1 #107bis-e”, 3GPP RAN1#107bis-e, Jan. 2022.