3GPP TSG RAN WG1 #108-emeeting	     					R1-2202181

e-Meeting, Feb. 21st – Mar. 3rd, 2022

Agenda item:	8.16.17

Source: 	Qualcomm Incorporated

Title: 	UE features for Rel-17 UL Tx switching

Document for:	Discussion/Decision

# Introduction

RAN plenary approved the work item R17 NR FR1 which includes the R17 UL Tx switching [1]. In this paper, we discussed the UE capabilities and express our views.

# Overview of Rel-16 UL Tx switching capabilities

For Rel-17 UL Tx switching, RAN2 made the following agreement [2].

Based on the following RAN2 agreements made in RAN2 #115 meeting, the R16 UE capability reporting should be extended to cover R17 scenarios. No need to introduce Rel-17 UE capability of DL interruption for 2Tx-2Tx switching. The Rel-16 UE capability of DL interruption for 1Tx-2Tx switching applies to 2Tx-2Tx switching as well. To introduce Rel-17 per-band pair UE capability to indicate a different switching time for 2Tx-2Tx switching for a given BC (Option 1). The Rel-16 filter uplinkTxSwitchRequest-r16 can be reused to request Rel-17 UL Tx switching UE capability. For R17 1Tx-2Tx/2Tx-2Tx switching between 1 carrier on band A and 2 contiguous aggregated carriers on band B for SUL and UL CA, RAN2 takes the following way-forward as RAN2 understanding. - Way-forward: the UE should report corresponding CA bandwidth class and UL MIMO layers in the UL featureSetPerCCs for 2 continuous CCs on band B in the legacy way. No new UE capability is needed specific to the case with 2CCs on band B. On band B, the fallback capability from 2 CCs to 1 CC can be supported in the legacy way.

Based on the agreement, we made following observation and share our initial considerations.

| Capability name |  | R16 | Views on R17 |
| --- | --- | --- | --- |
| ULTxSwitchingBandPair | bandIndexUL1, bandIndexUL2 | Y | Agreed |
|  | uplinkTxSwitchingPeriod | Y | Agreed |
|  | uplinkTxSwitching-DL-Interruption | Y | No need |
| supportedBandCombinationList-UplinkTxSwitch |  | Y | Agreed |
| uplinkTxSwitching-OptionSupport | Option 1 or 2 | Y | Need a new capability |
| uplinkTxSwitching-PowerBoosting |  | Y | No need |
| supportedBandCombinationList-UplinkTxSwitch |  | Y | Agreed |

# New UE capability - uplinkTxSwitching-OptionSupport17

In RAN1 #106b-emeeting, FL made the following proposal based on the discussion [3] which was further discussed in RAN1#107e. In RAN1-107e [4], we feel the majority are supportive to introduce a new UE capability to differentiate Rel-17 and Rel-16 CA UL Tx switching behaviors.

- FG 37-1 is added as “Indicating supported option for UL Tx switching for 2Tx-2Tx inter-band UL CA” as follows 37. [NR_RF_FR1_enh]; 37-1; Indicating supported option for UL Tx switching for 2Tx-2Tx inter-band UL CA; Indicating supported option for 2Tx-2Tx UL Tx switching for inter-band UL CA - Candidate values set is {option1, option2, both option 1 and option 2}; FFS; Yes; N/A; Per BC; N/A; N/A (FR1 only); N/A; FFS details. Note that any contents highlighted in yellow mean FFS and to be discussed further.

Rel-17 UL Tx switching mainly has two sub-features – a new switching case (Case 3) and intra-band switching (2 carrier on band B). Some key issues requires ASN.1 impact as follows.

- Switching between the new Case (Case 3) and Case 1 & 2

- A new RRC IE to indicate the prioritized target case between Case 1 and 2Tx on the other band/carrier.

- One new switching time – 2Tx-2Tx and potential switch between the 2 switch modes (1Tx-2Tx & 2Tx-2Tx)

Given the above new UE & network behaviors would require changes of physical and upper layer specs, We feel it would be good to define an new UE capability for Rel-17 inter-band CA UL Tx switching..

Proposal: Introduce a new UE capability to indicate supported option for 2Tx-2Tx UL Tx switching for inter-band UL CA

# Conclusions

For Rel-17 UL Tx switching UE capability, we have the following proposals:

Proposal: Introduce a new UE capability to indicate supported option for 2Tx-2Tx UL Tx switching for inter-band UL CA

# Reference

- RP-210899, revised WID: RF requirements enhancement for NR frequency range 1 (FR1), Mar. 2021

- R2-2109226, CR to TS 38.306 Introduction of Rel-17 Tx switching enhancements, Aug. 2021

- R1-2109720, Summary on other UE feature related discussions	Moderator (NTT DOCOMO, INC.)

- R1-2112147, Summary on other UE feature related discussions	Moderator (NTT DOCOMO, INC.)