3GPP TSG-RAN WG1 Meeting # 108-e 	R1-2202183

# e-Meeting, February 21st - March 3rd, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.211 | CR | - | rev | - | Current version: | 15.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on Intra-slot frequency hopping for PUCCH transmission |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Sharp |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR newRAT-Core |  |  |  |  |  | Date: |  |  | 2022-02-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - According to clause 9.2.1 of TS38.213, there are two PUCCH transmission behaviours relating to intra-slot frequency hopping, i.e. cell-specific PUCCH transmission always using intra-slot frequency hopping and dedicated PUCCH transmission for which the intra-slot frequency hopping is enabled or disabled by the RRC parameter intraSlotFrequencyHopping. However, Clause 6.3.2.2.1 of TS 38.211 describes hop-level base sequence hopping for PUCCH transmission and how to define the frequency hopping index nhop only for intra-slot frequency hopping enabled or disabled by an RRC parameter intraSlotFrequencyHopping. Therefore, the current 6.3.2.2.1 of TS 38.211 would cause the ambiguity on whether hop-level base sequence hopping is applied to cell-specific PUCCH transmission and how to clarify the nhop for cell-specific PUCCH transmission. - The intra-slot frequency hopping related description, i.e. ‘Intra-slot frequency hopping shall be assumed when the higher-layer parameter intraSlotFrequencyHopping is provided, regardless of whether the frequency-hop distance is zero or not, otherwise no intra-slot frequency hopping shall be assumed’, in clauses 6.3.2.4.1 and 6.4.1.3.1.1 is intended to clarify that phase continuity is not needed to keep even if two hops have zero distance. However, ‘otherwise no intra-slot frequency hopping shall be assumed’ would be misleading as it seems that the cell-specific PUCCH transmission always using intra-slot frequency hopping would be covered by the ‘otherwise’ case. Consequently, ambiguity would be caused on sequence generation for PUCCH format 1 and its associated DMRS. Moreover, the intention of the related description in clauses 6.3.2.4.1 and 6.4.1.3.1.1 have been already reflected in the clause 6.2. Therefore, the related description which is misleading can be removed with no harm. Clause 6.2 in TS 38.211 If intra-slot frequency hopping is enabled for a physical channel, the UE transmission shall be such that the channel over which a symbol on the antenna port used for uplink transmission is conveyed can be inferred from the channel over which another symbol on the same antenna port is conveyed only if the two symbols correspond to the same frequency hop, regardless of whether the frequency hop distance is zero or not. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Replacing ‘by the higher-layer parameter intraSlotFrequencyHopping’ with ‘according to clause 9.2.1 of [5, TS38.213]’ in clause 6.3.2.2.1 of TS 38.211 to clarify the nhop for cell-specific PUCCH transmission. - Deleting ‘Intra-slot frequency hopping shall be assumed when the higher-layer parameter intraSlotFrequencyHopping is provided, regardless of whether the frequency-hop distance is zero or not, otherwise no intra-slot frequency hopping shall be assumed’ in clauses 6.3.2.4.1 and 6.4.1.3.1.1 to remove misleading wording. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The UE/gNB behaviour is not clear as there is no clarification of nhop described in clause 6.3.2.2.1 of TS 38.211 for cell-specific PUCCH transmission. - The UE/gNB behaviour is not clear as the related description is not precise and misleading. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2.2.1, 6.3.2.4.1, 6.4.1.3.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated impact analysis: This CR has isolated impact on intra-slot frequency hopping for cell-specific PUCCH transmission in terms of base sequence selection and generation. And the corrections are based on RAN1 common understanding. It is expected that the UE and gNB have been implemented in accordance to this CR. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

6.3.2.2.1	Group and sequence hopping

The sequence group u=fgh+fss mod 30 and the sequence number v within the group depends on the higher-layer parameter pucch-GroupHopping:

-	if pucch-GroupHopping equals 'neither'

where  is given by the higher-layer parameter hoppingId if configured, otherwise nID=NIDcell.

-	if pucch-GroupHopping equals 'enable'

where the pseudo-random sequence  is defined by clause 5.2.1 and shall be initialized at the beginning of each radio frame with  where  is given by the higher-layer parameter hoppingId if configured, otherwise nID=NIDcell.

-	if pucch-GroupHopping equals 'disable'

where the pseudo-random sequence  is defined by clause 5.2.1 and shall be initialized at the beginning of each radio frame with cinit=25nID30+nID mod 30 where  is given by the higher-layer parameter hoppingId if configured, otherwise nID=NIDcell.

The frequency hopping index nhop=0 if intra-slot frequency hopping is disabled. If frequency hopping is enabled,  for the first hop and  for the second hop.

6.3.2.4.1	Sequence modulation

The block of bits  shall be modulated as described in clause 5.1 using BPSK if  and QPSK if , resulting in a complex-valued symbol . 
The complex-valued symbol  shall be multiplied with a sequence ru,vα,δ(n) according to

where ru,vα,δ(n) is given by clause 6.3.2.2. The block of complex-valued symbols  shall be block-wise spread with the orthogonal sequence  according to

where NSF,m'PUCCH,1 is given by Table 6.3.2.4.1-1.

6.4.1.3.1.1	Sequence generation

The reference signal sequence is defined by

where  is given by Table 6.4.1.3.1.1-1 and the sequence ru,vα,δ(n) is given by clause 5.2.2.