3GPP TSG-RAN WG1 Meeting # 108-e 	R1-2202188

# e-Meeting, February 21st – March 3rd , 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | - | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on time domain resource allocation procedure for a PUSCH scheduled by RAR UL grant or by DCI format 0_0 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Sharp |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_L1enh_URLLC-Core |  |  |  |  |  | Date: |  |  | 2022-02-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In Rel-16 URLLC WI, the term ‘PUSCH repetition type B’ was introduced to represent the new Rel-16 PUSCH transmission scheme, while the term ‘PUSCH repetition Type A’ was introduced to represent the PUSCH transmission scheme according to Rel-15 behavior. In terms of time domain resource allocation, the starting symbol S and the number of consecutive symbols L directly provided by higher layer parameters are applied to the PUSCH repetition Type B. On the other hand, the starting symbol S and the number of consecutive symbols L determined by SLIV are still applied to the PUSCH repetition Type A. However, clause 6.1.2.1 of TS 38.214 only describes how to determine the time domain resource allocation for PUSCH scheduled by DCI formats 0_1/0_2, which would cause the ambiguity on determination of the time domain resource allocation for PUSCH scheduled by RAR UL grant and for PUSCH scheduled by DCI format 0_0. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding ‘for PUSCH scheduled by RAR UL grant or for PUSCH scheduled by DCI format 0_0, the UE applies PUSCH repetition Type A procedure when determining the time domain resource allocation’ to clarify that when determining time domain resource allocation for PUSCH scheduled by RAR UL grant or for PUSCH scheduled by DCI format 0_0, PUSCH repetition type A procedure is applied. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | How to determine time resource allocation for PUSCH scheduled by RAR UL grant or for PUSCH scheduled by DCI format 0_0 is not defined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated impact analysis: This CR has isolated impact on determining time resource allocation for PUSCH scheduled by RAR UL grant and PUSCH scheduled by DCI format 0_0. The correction is based on RAN1 common understanding. It is expected that the UE and gNB have been implemented in accordance to this CR. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 6.1.2.1	Resource allocation in time domain

When the UE is scheduled to transmit a transport block and no CSI report, or the UE is scheduled to transmit a transport block and a CSI report(s) on PUSCH by a DCI, the 'Time domain resource assignment' field value m of the DCI provides a row index m + 1 to an allocated table. The determination of the used resource allocation table is defined in Clause 6.1.2.1.1. The indexed row defines the slot offset K2, the start and length indicator SLIV, or directly the start symbol S and the allocation length L, the PUSCH mapping type, and the number of repetitions (if numberOfRepetitions is present in the resource allocation table) to be applied in the PUSCH transmission.

When the UE is scheduled to transmit a PUSCH with no transport block and with a CSI report(s) by a 'CSI request' field on a DCI, the 'Time domain resource assignment' field value m of the DCI provides a row index m + 1 to the allocated table as defined in Clause 6.1.2.1.1. The indexed row defines the start and length indicator SLIV, or directly the start symbol S and the allocation length L, and the PUSCH mapping type to be applied in the PUSCH transmission and the K2 value is determined as , where  are the corresponding list entries of the higher layer parameter

-	reportSlotOffsetListDCI-0-2, if PUSCH is scheduled by DCI format 0_2 and reportSlotOffsetListDCI-0-2 is configured;

-	reportSlotOffsetListDCI-0-1, if PUSCH is scheduled by DCI format 0_1 and reportSlotOffsetListDCI-0-1 is configured;

-	reportSlotOffsetList, otherwise;

in CSI-ReportConfig for the  triggered CSI Reporting Settings and  is the (m+1)th entry of .

-	The slot Ks where the UE shall transmit the PUSCH is determined by K2 as Ks =, if UE is configured with ca-SlotOffset for at least one of the scheduled and scheduling cell, Ks =, otherwise, and where n is the slot with the scheduling DCI, K2 is based on the numerology of PUSCH, and  and  are the subcarrier spacing configurations for PUSCH and PDCCH, respectively,

-	Nslot, offset, PDCCHCA and μoffset,PDCCH are the Nslot, offsetCA and the, respectively, which are determined by higher-layer configured ca-SlotOffset for the cell receiving the PDCCH, Nslot, offset, PUSCHCA and μoffset,PUSCH are the Nslot, offsetCA and the,respectively, which are determined by higher-layer configured ca-SlotOffset for the cell transmitting the PUSCH, as defined in clause 4.5 of [4, TS 38.211],

-    for PUSCH scheduled by RAR UL grant or for PUSCH scheduled by DCI format 0_0, the UE applies PUSCH repetition Type A procedure when determining the time domain resource allocation, and

-	for PUSCH scheduled by DCI format 0_1, if pusch-RepTypeIndicatorDCI-0-1  is set to 'pusch-RepTypeB', the UE applies PUSCH repetition Type B procedure when determining the time domain resource allocation. For PUSCH scheduled by DCI format 0_2, if pusch-RepTypeIndicatorDCI-0-2 is set to 'pusch-RepTypeB', the UE applies PUSCH repetition Type B procedure when determining the time domain resource allocation. Otherwise, the UE applies PUSCH repetition Type A procedure when determining the time domain resource allocation for PUSCH scheduled by PDCCH.

-	For PUSCH repetition Type A, the starting symbol S relative to the start of the slot, and the number of consecutive symbols L counting from the symbol S allocated for the PUSCH are determined from the start and length indicator SLIV of the indexed row:

if  then

else

where, and

-	For PUSCH repetition Type B, the starting symbol S relative to the start of the slot, and the number of consecutive symbols L counting from the symbol S allocated for the PUSCH are provided by startSymbol and length of the indexed row of the resource allocation table, respectively.

-	For PUSCH repetition Type A, the PUSCH mapping type is set to Type A or Type B as defined in Clause 6.4.1.1.3 of [4, TS 38.211] as given by the indexed row.

-	For PUSCH repetition Type B, the PUSCH mapping type is set to Type B.

The UE shall consider the S and L combinations defined in table 6.1.2.1-1 as valid PUSCH allocations

Table 6.1.2.1-1: Valid S and L combinations

| PUSCH mapping type | Normal cyclic prefix |  |  | Extended cyclic prefix |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | S | L | S+L | S | L | S+L |
| Type A (repetition Type A only) | 0 | {4,…,14} | {4,…,14} | 0 | {4,…,12} | {4,…,12} |
| Type B | {0,…,13} | {1,…,14} | {1,…,14} for repetition Type A, {1,…,27} for repetition Type B | {0,…, 11} | {1,…,12} | {1,…,12} for repetition Type A, {1,…,23} for repetition Type B |

For PUSCH repetition Type A, when transmitting PUSCH scheduled by DCI format 0_1 or 0_2 in PDCCH with CRC scrambled with C-RNTI, MCS-C-RNTI, or CS-RNTI with NDI=1, the number of repetitions K is determined as

-	if numberOfRepetitions is present in the resource allocation table, the number of repetitions K is equal to numberOfRepetitions;

-	elseif the UE is configured with pusch-AggregationFactor, the number of repetitions K is equal to pusch-AggregationFactor;

-	otherwise K=1.

If a UE is configured with higher layer parameter pusch-TimeDomainAllocationListForMultiPUSCH, the UE does not expect to be configured with pusch-AggregationFactor.

For PUSCH repetition Type A, in case K>1, the same symbol allocation is applied across the K consecutive slots and the PUSCH is limited to a single transmission layer. The UE shall repeat the TB across the K consecutive slots applying the same symbol allocation in each slot. The redundancy version to be applied on the nth transmission occasion of the TB, where n = 0, 1, … K-1, is determined according to table 6.1.2.1-2.

Table 6.1.2.1-2: Redundancy version for PUSCH transmission

| rvid indicated by the DCI scheduling the PUSCH | rvid to be applied to nth transmission occasion (repetition Type A) or nth actual repetition (repetition Type B) |  |  |  |
| --- | --- | --- | --- | --- |
|  | n mod 4 = 0 | n mod 4 = 1 | n mod 4 = 2 | n mod 4 = 3 |
| 0 | 0 | 2 | 3 | 1 |
| 2 | 2 | 3 | 1 | 0 |
| 3 | 3 | 1 | 0 | 2 |
| 1 | 1 | 0 | 2 | 3 |

When transmitting MsgA PUSCH on a non-initial UL BWP, if the UE is configured with startSymbolAndLengthMsgA-PO, the UE shall determine the S and L from startSymbolAndLengthMsgA-PO.

When transmitting MsgA PUSCH, if the UE is not configured with startSymbolAndLengthMsgA-PO, and if the TDRA list PUSCH-TimeDomainResourceAllocationList is provided in PUSCH-ConfigCommon, the UE shall use msgA-PUSCH-TimeDomainAllocation to indicate which values are used in the list. If PUSCH-TimeDomainResourceAllocationList is not provided in PUSCH-ConfigCommon, the UE shall use parameters S and L from table 6.1.2.1.1-2 or table 6.1.2.1.1-3 where msgA-PUSCH-TimeDomainAllocation indicates which values are used in the list. The time offset for PUSCH transmission is described in [6, TS38.213].

For PUSCH repetition Type A, a PUSCH transmission in a slot of a multi-slot PUSCH transmission is omitted according to the conditions in Clause 9, Clause 11.1 and Clause 11.2A of [6, TS38.213].

For PUSCH repetition Type B, except for PUSCH transmitting CSI report(s) with no transport block, the number of nominal repetitions is given by numberOfRepetitions. For the n-th nominal repetition, n = 0, …, numberOfRepetitions - 1,

-	The slot where the nominal repetition starts is given by , and the starting symbol relative to the start of the slot is given by .

-	The slot where the nominal repetition ends is given by , and the ending symbol relative to the start of the slot is given by .

Here is the slot where the PUSCH transmission starts, and is the number of symbols per slot as defined in Clause 4.3.2 of [4, TS38.211].

For PUSCH repetition Type B, the UE determines invalid symbol(s) for PUSCH repetition Type B transmission as follows:

-	A symbol that is indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated is considered as an invalid symbol for PUSCH repetition Type B transmission.

-	For operation in unpaired spectrum, symbols indicated by ssb-PositionsInBurst in SIB1 or ssb-PositionsInBurst in ServingCellConfigCommon for reception of SS/PBCH blocks are considered as invalid symbols for PUSCH repetition Type B transmission.

-	For operation in unpaired spectrum, symbol(s) indicated by pdcch-ConfigSIB1 in MIB for a CORESET for Type0-PDCCH CSS set are considered as invalid symbol(s) for PUSCH repetition Type B transmission.

-	For operation in unpaired spectrum, if numberOfInvalidSymbolsForDL-UL-Switching is configured, numberOfInvalidSymbolsForDL-UL-Switching symbol(s) after the last symbol that is indicated as downlink in each consecutive set of all symbols that are indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated are considered as invalid symbol(s) for PUSCH repetition Type B transmission. The symbol(s) given by numberOfInvalidSymbolsForDL-UL-Switching are defined using the reference SCS configuration referenceSubcarrierSpacing provided in tdd-UL-DL-ConfigurationCommon.

-	The UE may be configured with the higher layer parameter invalidSymbolPattern, which provides a symbol level bitmap spanning one or two slots (higher layer parameter symbols given by invalidSymbolPattern). A bit value equal to 1 in the symbol level bitmap symbols indicates that the corresponding symbol is an invalid symbol for PUSCH repetition Type B transmission. The UE may be additionally configured with a time-domain pattern (higher layer parameter periodicityAndPattern given by invalidSymbolPattern), where each bit of periodicityAndPattern corresponds to a unit equal to a duration of the symbol level bitmap symbols, and a bit value equal to 1 indicates that the symbol level bitmap symbols is present in the unit. The periodicityAndPattern can be {1, 2, 4, 5, 8, 10, 20 or 40} units long, but maximum of 40 msec. The first symbol of periodicityAndPattern every 40 msec/P periods is a first symbol in frame 𝑛𝑓 mod 4 = 0, where P is the duration of periodicityAndPattern-r16 in units of msec. When periodicityAndPattern is not configured, for a symbol level bitmap spanning two slots, the bits of the first and second slots correspond respectively to even and odd slots of a radio frame, and for a symbol level bitmap spanning one slot, the bits of the slot correspond to every slot of a radio frame. If invalidSymbolPattern is configured, when the UE applies the invalid symbol pattern is determined as follows:

-	if the PUSCH is scheduled by DCI format 0_1, or corresponds to a Type 2 configured grant activated by DCI format 0_1, and if invalidSymbolPatternIndicatorDCI-0-1  is configured,

-	if invalid symbol pattern indicator field is set 1, the UE applies the invalid symbol pattern;

-	otherwise, the UE does not apply the invalid symbol pattern;

-	if the PUSCH is scheduled by DCI format 0_2, or corresponds to a Type 2 configured grant activated by DCI format 0_2, and if invalidSymbolPatternIndicatorDCI-0-2  is configured,

-	if invalid symbol pattern indicator field is set 1, the UE applies the invalid symbol pattern;

-	otherwise, the UE does not apply the invalid symbol pattern;

-	otherwise, the UE applies the invalid symbol pattern.

-	If the UE

-	is configured with multiple serving cells within a cell group and is provided with directionalCollisionHandling-r16 = 'enabled' for a set of serving cell(s) among the multiple serving cells, and

-	indicates support of half-DuplexTDD-CA-SameSCS-r16 capability, and

-	is not configured to monitor PDCCH for detection of DCI format 2-0 on any of the multiple serving cells,

-	a symbol indicated to the UE for reception of SS/PBCH blocks in a first cell of the multiple serving cells by ssb-PositionsInBurst in SIB1 or ssb-PositionsInBurst in ServingCellConfigCommon is considered as an invalid symbol for PUSCH repetition Type B transmission in

-	any of the multiple serving cells if the UE is not capable of simultaneous transmission and reception as indicated by simultaneousRxTxInterBandCA among the multiple serving cells, and

-	any one of the cells corresponding to the same band as the first cell, irrespective of any capability indicated by simultaneousRxTxInterBandCA

and

-	a symbol is considered as an invalid symbol in another cell among the set of serving  cell(s) provided with directionalCollisionHandling-r16 for PUSCH repetition Type B transmission with Type 1 or Type 2 configured grant except for the first Type 2 PUSCH transmission (including all repetitions) after activation if the symbol is indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated on the reference cell as defined in Clause 11.1 of [6, TS 38.213], or the UE is configured by higher layers to receive PDCCH, PDSCH, or CSI-RS on the reference cell in the symbol.

For PUSCH repetition Type B, after determining the invalid symbol(s) for PUSCH repetition type B transmission for each of the K nominal repetitions, the remaining symbols are considered as potentially valid symbols for PUSCH repetition Type B transmission. If the number of potentially valid symbols for PUSCH repetition type B transmission is greater than zero for a nominal repetition, the nominal repetition consists of one or more actual repetitions, where each actual repetition consists of a consecutive set of all potentially valid symbols that can be used for PUSCH repetition Type B transmission within a slot. An actual repetition with a single symbol is omitted except for the case of L=1. An actual repetition is omitted according to the conditions in Clause 9, Clause 11.1 and Clause 11.2A of [6, TS38.213]. The UE shall repeat the TB across actual repetitions. The redundancy version to be applied on the nth actual repetition (with the counting including the actual repetitions that are omitted) is determined according to table 6.1.2.1-2.

For PUSCH repetition Type B, when a UE receives a DCI that schedules aperiodic CSI report(s) or activates semi-persistent CSI report(s) on PUSCH with no transport block by a 'CSI request' field on a DCI, the number of nominal repetitions is always assumed to be 1, regardless of the value of numberOfRepetitions. When the UE is scheduled to transmit a PUSCH repetition Type B with no transport block and with aperiodic or semi-persistent CSI report(s) by a 'CSI request' field on a DCI, the first nominal repetition is expected to be the same as the first actual repetition. For PUSCH repetition Type B carrying semi-persistent CSI report(s) without a corresponding PDCCH after being activated on PUSCH by a 'CSI request' field on a DCI, if the first nominal repetition is not the same as the first actual repetition, the first nominal repetition is omitted; otherwise, the first nominal repetition is omitted according to the conditions in Clause 9, Clause 11.1 and Clause 11.2A of [6, TS38.213].

For PUSCH repetition Type B, when a UE is scheduled to transmit a transport block and aperiodic CSI report(s) on PUSCH by a 'CSI request' field on a DCI, the CSI report(s) is multiplexed only on the first actual repetition. The UE does not expect that the first actual repetition has a single symbol duration.

If pusch-TimeDomainAllocationListForMultiPUSCH in pusch-Config contains row indicating resource allocation for two to eight contiguous PUSCHs, K2 indicates the slot where UE shall transmit the first PUSCH of the multiple PUSCHs. Each PUSCH has a separate SLIV and mapping type. The number of scheduled PUSCHs is signalled by the number of indicated valid SLIVs in the row of the pusch-TimeDomainAllocationListForMultiPUSCH signalled in DCI format 0_1.

When the UE is configured with minimumSchedulingOffsetK2 in an active UL BWP it applies a minimum scheduling offset restriction indicated by the 'Minimum applicable scheduling offset indicator' field in DCI format 0_1 or DCI format 1_1 if the same field is available. When the UE is configured with minimumSchedulingOffsetK2 in an active UL BWP and it has not received 'Minimum applicable scheduling offset indicator' field in DCI format 0_1 or 1_1, the UE shall apply a minimum scheduling offset restriction indicated based on 'Minimum applicable scheduling offset indicator' value '0'. When the minimum scheduling offset restriction is applied the UE is not expected to be scheduled with a DCI in slot n to transmit a PUSCH scheduled with C-RNTI, CS-RNTI, MCS-C-RNTI or SP-CSI-RNTI with K2 smaller than K2min⋅2μ'2μ, where K2min and μ are the applied minimum scheduling offset restriction and the numerology of the active UL BWP of the scheduled cell when receiving the DCI in slot n, respectively, and μ' is the numerology of the new active UL BWP in case of active UL BWP change in the scheduled cell and is equal to μ, otherwise. The minimum scheduling offset restriction is not applied when PUSCH transmission is scheduled by RAR UL grant or fallbackRAR UL grant for RACH procedure, or when PUSCH is scheduled with TC-RNTI. The application delay of the change of the minimum scheduling offset restriction is determined in Clause 5.3.1.

##### 6.1.2.1.1	Determination of the resource allocation table to be used for PUSCH

Table 6.1.2.1.1-1, Table 6.1.2.1.1-1A and Table 6.1.2.1.1-1B define which PUSCH time domain resource allocation configuration to apply.

Table 6.1.2.1.1-4 defines the subcarrier spacing specific values j. j is used in determination of K2 in conjunction to table 6.1.2.1.1-2, for normal CP or table 6.1.2.1.1.-3 for extended CP, where µPUSCH is the subcarrier spacing configurations for PUSCH.

Table 6.1.2.1.1-5 defines the additional subcarrier spacing specific slot delay value for the first transmission of PUSCH scheduled by the RAR or by the fallbackRAR. When the UE transmits a PUSCH scheduled by RAR or by the fallbackRAR, the Δ value specific to the PUSCH subcarrier spacing µPUSCH is applied in addition to the K2 value.

Table 6.1.2.1.1-1: Applicable PUSCH time domain resource allocation for common search space and DCI format 0_0 in UE specific search space

| RNTI | PDCCH search space | pusch-ConfigCommon includes pusch-TimeDomainAllocationList | pusch-Config includes pusch-TimeDomainAllocationList | PUSCH time domain resource allocation to apply |
| --- | --- | --- | --- | --- |
| PUSCH scheduled by MAC RAR as described in clause 8.2 of [6, TS 38.213] or MAC fallbackRAR as described in clause 8.2A of [6, 38.213] or for MsgA PUSCH transmission |  | No | - | Default A |
|  |  | Yes |  | pusch-TimeDomainAllocationList provided in pusch-ConfigCommon |
| C-RNTI, MCS-C-RNTI, TC-RNTI, CS-RNTI | Any common search space associated with CORESET 0 | No | - | Default A |
|  |  | Yes |  | pusch-TimeDomainAllocationList provided in pusch-ConfigCommon |
| C-RNTI, MCS-C-RNTI, TC-RNTI, CS-RNTI | Any common search space not associated with CORESET 0, DCI format 0_0 in UE specific search space | No | No | Default A |
|  |  | Yes | No | pusch-TimeDomainAllocationList provided in pusch-ConfigCommon |
|  |  | No/Yes | Yes | pusch-TimeDomainAllocationList provided in pusch-Config |

Table 6.1.2.1.1-1A: Applicable PUSCH time domain resource allocation for DCI format 0_1 in UE specific search space scrambled with C-RNTI, MCS-C-RNTI, CS-RNTI or SP-CSI-RNTI

| pusch-ConfigCommon includes pusch-TimeDomainAllocationList | pusch-Config includes pusch-TimeDomainAllocationList | pusch-Config includes pusch-TimeDomainAllocationListDCI-0-1 | pusch-Config includes pusch-TimeDomainAllocationList-ForMultiPUSCH | PUSCH time domain resource allocation to apply |
| --- | --- | --- | --- | --- |
| No | No | No | No | Default A |
| Yes | No | No | No | pusch-TimeDomainAllocationList provided in pusch-ConfigCommon |
| No/Yes | Yes | No | No | pusch-TimeDomainAllocationList provided in pusch-Config |
| No/Yes | No/Yes | Yes | - | pusch-TimeDomainAllocationListDCI-0-1 provided in pusch-Config |
| No/Yes | No/Yes | - | Yes | pusch-TimeDomainAllocationList-ForMultiPUSCH provided in pusch-Config |

Table 6.1.2.1.1-1B: Applicable PUSCH time domain resource allocation for DCI format 0_2 in UE specific search space scrambled with C-RNTI, MCS-C-RNTI, CS-RNTI or SP-CSI-RNTI

| pusch-ConfigCommon includes pusch-TimeDomainAllocationList | pusch-Config includes pusch-TimeDomainAllocationList | pusch-Config includes pusch-TimeDomainAllocationListDCI-0-2 | PUSCH time domain resource allocation to apply |
| --- | --- | --- | --- |
| No | No | No | Default A |
| Yes | No | No | pusch-TimeDomainAllocationList provided in pusch-ConfigCommon |
| No/Yes | Yes | No | pusch-TimeDomainAllocationList provided in pusch-Config |
| No/Yes | No/Yes | Yes | pusch-TimeDomainAllocationListDCI-0-2 provided in pusch-Config |

Table 6.1.2.1.1-2: Default PUSCH time domain resource allocation A for normal CP

| Row index | PUSCH mapping type | K2 | S | L |
| --- | --- | --- | --- | --- |
| 1 | Type A | j | 0 | 14 |
| 2 | Type A | j | 0 | 12 |
| 3 | Type A | j | 0 | 10 |
| 4 | Type B | j | 2 | 10 |
| 5 | Type B | j | 4 | 10 |
| 6 | Type B | j | 4 | 8 |
| 7 | Type B | j | 4 | 6 |
| 8 | Type A | j+1 | 0 | 14 |
| 9 | Type A | j+1 | 0 | 12 |
| 10 | Type A | j+1 | 0 | 10 |
| 11 | Type A | j+2 | 0 | 14 |
| 12 | Type A | j+2 | 0 | 12 |
| 13 | Type A | j+2 | 0 | 10 |
| 14 | Type B | j | 8 | 6 |
| 15 | Type A | j+3 | 0 | 14 |
| 16 | Type A | j+3 | 0 | 10 |

Table 6.1.2.1.1-3: Default PUSCH time domain resource allocation A for extended CP

| Row index | PUSCH mapping type | K2 | S | L |
| --- | --- | --- | --- | --- |
| 1 | Type A | j | 0 | 8 |
| 2 | Type A | j | 0 | 12 |
| 3 | Type A | j | 0 | 10 |
| 4 | Type B | j | 2 | 10 |
| 5 | Type B | j | 4 | 4 |
| 6 | Type B | j | 4 | 8 |
| 7 | Type B | j | 4 | 6 |
| 8 | Type A | j+1 | 0 | 8 |
| 9 | Type A | j+1 | 0 | 12 |
| 10 | Type A | j+1 | 0 | 10 |
| 11 | Type A | j+2 | 0 | 6 |
| 12 | Type A | j+2 | 0 | 12 |
| 13 | Type A | j+2 | 0 | 10 |
| 14 | Type B | j | 8 | 4 |
| 15 | Type A | j+3 | 0 | 8 |
| 16 | Type A | j+3 | 0 | 10 |

Table 6.1.2.1.1-4: Definition of value j

| µPUSCH | j |
| --- | --- |
| 0 | 1 |
| 1 | 1 |
| 2 | 2 |
| 3 | 3 |

Table 6.1.2.1.1-5: Definition of value Δ

| µPUSCH | Δ |
| --- | --- |
| 0 | 2 |
| 1 | 3 |
| 2 | 4 |
| 3 | 6 |