3GPP TSG RAN WG1 #108-e							R1-2202224

e-Meeting, February 21st – March 3rd, 2022

Agenda Item:	8.16.7

Source:	Ericsson

Title:	UE features for UEPS

Document for:	Discussion/Decision

# 1	Introduction

This document discusses UE features related to Rel-17 UE power savings work item.

# 2	Discussion and proposals

Below are some of the main changes proposed for the FGs (using track changes in below table).

- For FG 29-1 (paging enhancements),

  - Component description should be updated to reflect that paging early indication is supported via DCI format 2_7.

  - In last RAN1 meeting, an LS was sent to RAN2 informing consequences of separating the component 2 into a separate FG. Thus, it is preferable to wait for further RAN2 input on this and continue with other aspects of finalizing FG 29-1.

  - It may be helpful if RAN1 can also indicate the reporting granularity (as per UE or per Band) in case ‘optional with capability signalling’ is identified as essential by RAN2. The granularity can be per UE or at most per Band, OK to use per band to avoid differentiation FR1/FR2, licensed/unlicensed, etc.

- For FG 29-2 (TRS occasions),

  - ‘Consequence column’: The current sentence (Lose of power saving gain on AGC, time/frequency tracking in idle/inactive mode) should be removed. OK to add “UE does not support TRS occasions for idle/inactive UEs” or it can be left empty.

  - There was some discussion in last meeting on whether to add PEI (FG 29-1) as a prerequisite for receiving TRS occasions (FG 29-2). PEI is not necessary to receive TRS occasions, for which the availability indication can be received via Paging DCI instead. Therefore, we do not see need to add 29-2 as prerequisite. On the other hand, it makes sense that a UE supporting both PEI and TRS occasions should be able to receive availability indication via PEI. Components need to be updated accordingly to indicate support via DCI 1_0 and via DCI 2_7 (if UE supports FG29-1).

  - Allowing optional UE capability signalling can be useful for NW to know when to turn on this feature, but it is not essential to have capability signalling for this or any additional separate capabilities (for reception of L1 signalling). TRS occasion configuration and L1 availability configuration is not UE-specific. Idle/Inactive UEs can ignore any TRS occasion-related information they are not interested in/capable of receiving. If ‘optional with capability’ signalling is identified as essential, it should be per UE granularity or at most per Band, OK to use per band to avoid differentiation FR1/FR2, licensed/unlicensed, etc. As suggested by some companies, it would be also OK to leave optional with/without capability signalling to RAN2 decision.

- For FG29-3a/3b/3c/3d (PDCCH monitoring adaptation)

  - The FGs should be per UE or at most per Band, OK to use per band to avoid differentiation FR1/FR2, licensed/unlicensed, etc.

  - They should be optional with capability signalling.

  - ‘Consequence column’ can be left empty – there is no need to say that the feature is not supported as a consequence.

Based on above discussion, below are proposed updates to the UEPS FG table.

Proposal 1: Update the UE feature list for UEPS as shown in below Table (in red text).

# 29. NR_UE_pow_sav_enh

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 29. NR_UE_pow_sav_enh | 29-1 | Paging enhancement | 1. Support paging early indication via DCI format 2_7 2. Support UE subgroup indication |  |  |  | UE does not support paging enhancement | Per UE Per Band | N/A | N/A | N/A | For component 2, it is up to RAN2 whether/how to separate the capability for UE subgroup indication Leave RAN2 to decide whether ‘optional with capability signalling’ or ‘optional without capability signalling’ Leave RAN2 to decide whether Need for the gNB to know if the feature is supported is Yes or No | Optional |
| 29. NR_UE_pow_sav_enh | 29-2 | TRS resources for idle/inactive UEs | TRS occassions for idle/inactive UEs 1. Support reading TRS configuration from SIB 2. Support receving L1 indication for TRS availability via DCI format 1_0 3. Support receiving L1 indication for TRS availability via DCI format 2_7 (when UE supports FG 29-1) |  | N |  | Lose of power saving gain on AGC, time/frequency tracking in idle/inactive mode UE does not support TRS occasions for idle/inactive UEs | Per UE | N/A | N/A | N/A |  | Optional without capability signalling |
| 29. NR_UE_pow_sav_enh | 29-3a | PDCCH skipping | Support of up to 2-bit indication of PDCCH skipping by scheduling DCI if SSSG is not configured |  | Y |  |  | Per UE Per Band | N/A | N/A | N/A |  | Optional with capability signaling |
| 29. NR_UE_pow_sav_enh | 29-3b | 2 search space sets group switching | Support of 1-bit indication of SSSG switching between 2 SSSGs by scheduling DCI, and timer based search space set group switching, without PDCCH skipping |  | Y |  |  | Per UE Per Band | N/A | N/A | N/A |  | Optional with capability signaling |
| 29. NR_UE_pow_sav_enh | 29-3c | 3 search space sets group switching | Support of 2-bit indication of SSSG switching among 3 SSSGs by scheduling DCI and timer based search space set group switching | 29-3b | Y |  |  | Per UE Per Band | N/A | N/A | N/A |  | Optional with capability signaling |
| 29. NR_UE_pow_sav_enh | 29-3d | 2 search space sets group switching with PDCCH skipping | Support of 2-bit indication of SSSG switching between 2 SSSGs with PDCCH skipping by scheduling DCI and timer based search space set group switching | 29-3a, 29-3b | Y |  |  | Per UE Per Band | N/A | N/A | N/A |  | Optional with capability signaling |

# References

[1] R1-2200780, Updated RAN1 UE features list for Rel-17 NR after RAN1 #107bis-e, Moderators (AT&T, NTT DOCOMO, INC.), RAN1#107b-e, Jan 2022.

[2] R1-2109713	Summary on UE features for UE power saving enhancements	Moderator (NTT DOCOMO, INC.), RAN WG1#106bis-e, Oct 2021.

[3] R1-2112139	Summary on UE features for UE power saving enhancements	Moderator (NTT DOCOMO, INC.) , RAN WG1#107-e, Nov 2021.

[4] R1-2200678	Summary on UE features for UE power saving enhancements	Moderator (NTT DOCOMO, INC.) , RAN WG1#107b-e, Jan 2022.