3GPP TSG RAN WG1 #108-e		                                    R1-2202250

e-Meeting, February 21st – March 3rd, 2022

Agenda Item:	8.6.1.1

Source:	InterDigital, Inc.

Title:	Remaining issues on reduced maximum UE bandwidth

Document for:	Discussion

# Introduction

The following were agreed in RAN1 #107-e regarding reduced maximum UE bandwidth [1]:

Agreement

- In Rel-17, up to 1 separate initial UL BWP for RedCap can be configured.

Agreement

- For both FR1 and FR2, for a cell that allows a RedCap UE to access, network can configure a separate initial DL BWP for RedCap UEs in SIB. At least the case when the separate initial DL BWP includes CD-SSB and the entire CORESET#0 is supported

  - It can be used in idle/inactive mode (including paging) and during and after initial access, when applicable

  - It is no wider than the maximum RedCap UE bandwidth.

  - This applies to both TDD and FDD (including FD FDD and HD FDD) cases.

Agreement

- For FR1,

  - For a separate initial DL BWP (if it does not include CD-SSB and the entire CORESET#0) from RAN1 perspective,

    - If it is configured for random access while not for paging in idle/inactive mode, RedCap UE does NOT expect it to contain SSB/CORESET#0/SIB.

    - Note: RAN1 assumes REDCAP UE performing Random access in the separate DL BWP does not need to monitor paging in a BWP containing CORESET#0

    - Working assumption: If it is configured for paging, RedCap UE expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB from RAN1 perspective

  - For an RRC-configured active DL BWP in connected mode (if it does not include CD-SSB and the entire CORESET#0) from RAN1 perspective,

    - A RedCap UE supporting mandatory FG 6-1 (but not optional FG 6-1a) expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB

    - A RedCap UE can indicate the following as optional capability:

      - Not need NCD-SSB: A RedCap UE can in addition optionally support relevant operation (except for standalone use for RRM measurement) based on for CSI-RS (working assumption) and/or FG 6-1a by reporting optional capabilities.

  - Note: if a separate initial/RRC configured DL BWP is configured to contain the entire CORESET#0, CD-SSB is expected by RedCap UE.

  - Note: The network may choose to configure SSB or MIB-configured CORESET#0 or SIB1 to be within the respective DL BWP.

  - Note: If a separate SIB-configured initial DL BWP for RedCap UEs contains the entire CORESET#0, the RedCap UE shall use the bandwidth and location of the CORESET#0 in DL during initial access.

  - Note: NCD-SSB periodicity is not required to be configured the same as that of CD-SSB

  - Note: Periodicity of NCD-SSB shall be not less than periodicity of CD-SSB

Agreement

- For FR2,

  - For a separate initial DL BWP (if it does not include CD-SSB and the entire CORESET#0) from RAN1 perspective,

    - If it is configured for random access while not for paging in idle/inactive mode, RedCap UE does NOT expect it to contain SSB/CORESET#0/SIB.

    - Note: RAN1 assumes REDCAP UE performing Random access in the separate DL BWP does not need to monitor paging in a BWP containing CORESET#0

    - Working assumption: If it is configured for paging, RedCap UE expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB from RAN1 perspective

  - For an RRC-configured active DL BWP in connected mode (if it does not include CD-SSB and the entire CORESET#0) from RAN1 perspective,

    - A RedCap UE supporting mandatory FG 6-1 (but not optional FG 6-1a) expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB

    - A RedCap UE can indicate the following as optional capability:

      - Not need NCD-SSB: A RedCap UE can in addition optionally support relevant operation based on for CSI-RS (working assumption) and/or FG 6-1a by reporting optional capabilities.

  - Note: For SSB and CORESET#0 multiplexing pattern 1, if a separate initial/RRC configured DL BWP is configured to contain the entire CORESET#0, CD-SSB is expected by RedCap UE.

  - Note: The network may choose to configure SSB or MIB-configured CORESET#0 or SIB1 to be within the respective DL BWP.

  - Note: If a separate SIB-configured initial DL BWP for RedCap UEs contains the entire CORESET#0, the RedCap UE shall use the bandwidth and location of the CORESET#0 in DL during initial access.

  - Note: NCD-SSB periodicity is not required to be configured the same as that of CD-SSB

  - Note: Periodicity of NCD-SSB shall be not less than periodicity of CD-SSB

Agreement

- Send an LS to RAN2 and RAN4 to inform them about relevant RAN1 agreement on FR1 and corresponding agreement on FR2, as well as the working assumption, and ask them whether the working assumption reasonable or not:

  - For a separate initial DL BWP (if it does not include CD-SSB and the entire CORESET#0) from RAN1 perspective,

    - If it is configured for random access while not for paging in idle/inactive mode, RedCap UE does NOT expect it to contain SSB/CORESET#0/SIB.

    - Note: RAN1 assumes REDCAP UE performing Random access in the separate DL BWP does not need to monitor paging in a BWP containing CORESET#0

    - Working assumption: If it is configured for paging, RedCap UE expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB from RAN1 perspective

- Indicate in the LS that RAN1 does not expect any further RAN1 specification impact from the above working assumption.

- Also include the following RAN1 agreement in the LS as background information:

  - For both FR1 and FR2, for a cell that allows a RedCap UE to access, network can configure a separate initial DL BWP for RedCap UEs in SIB. At least the case when the separate initial DL BWP includes CD-SSB and the entire CORESET#0 is supported

    - It can be used in idle/inactive mode (including paging) and during and after initial access, when applicable

    - It is no wider than the maximum RedCap UE bandwidth.

    - This applies to both TDD and FDD (including FD FDD and HD FDD) cases.

Agreement

- When the frequency hopping for the RedCap PUCCH resources (for HARQ feedback for Msg4/MsgB) is deactivated,

  - Each PUCCH resource is mapped to a single PRB.

  - What side[(s)] of the RedCap UL BWP center frequency to which PUCCH resources are mapped is[/are] configurable by the network, including SIB-configurable [additional] offset (with no more than [4] candidate values) from edge using the existing equations for determining the PRB index of the PUCCH transmission as a starting point.

- RedCap and non-RedCap can be configured with the same or different PUCCH resource set indices (see TS 38.213 Table 9.2.1-1).

Agreement

- For a separate initial DL BWP for RedCap UEs,

  - The supported bandwidths for the separate initial DL BWP for RedCap UEs can have any values up to the maximum UE bandwidth (as in legacy operation).

Then, in RAN #94-e, it was agreed that [2]:

Scheme 1 (UE in IDLE and INACTIVE monitors paging in an initial BWP associated with CD-SSB) is adopted for further work in Rel-17. Scheme 2 (UE in IDLE and INACTIVE monitors paging in an initial BWP associated with NCD-SSB) is not considered further in Rel-17.

Furthermore, one of the agreements made in RAN1 #106bis-e is as follows [3]:

Agreement

For FR1,

- For TDD, center frequencies are assumed to be the same for the initial DL (FFS: if it does not include CD-SSB and the entire CORESET#0) and UL BWPs used during random access for RedCap UEs.

  - FFS: For Option 1 and Option 2, whether the case that the center frequencies are different is also supported, and whether RedCap UE can expect CD-SSB and CORESET#0 in this case

- For TDD, center frequencies are assumed to be the same for non-initial DL and UL BWPs with the same BWP id for a RedCap UE.

This contribution discusses remaining issues for reduced maximum UE bandwidth.

# Discussion

One issue that was heavily discussed during the WI was whether the Redcap UE can expect to have NCD-SSBs in the separate initial DL BWP. According to the agreement made in RAN #94-e, Scheme 2 (UE in IDLE and INACTIVE monitors paging in an initial BWP associated with NCD-SSB) is not further considered. So, the following observation can be made:

Observation: The following working assumptions are not confirmed:

- For FR1,

  - For a separate initial DL BWP (if it does not include CD-SSB and the entire CORESET#0) from RAN1 perspective,

    - Working assumption: If it is configured for paging, RedCap UE expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB from RAN1 perspective.

- For FR2,

  - For a separate initial DL BWP (if it does not include CD-SSB and the entire CORESET#0) from RAN1 perspective,

    - Working assumption: If it is configured for paging, RedCap UE expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB from RAN1 perspective.

In RAN1 #106bis-e, the following was agreed:

Agreement For FR1, - For TDD, center frequencies are assumed to be the same for the initial DL (FFS: if it does not include CD-SSB and the entire CORESET#0) and UL BWPs used during random access for RedCap UEs. - FFS: For Option 1 and Option 2, whether the case that the center frequencies are different is also supported, and whether RedCap UE can expect CD-SSB and CORESET#0 in this case - For TDD, center frequencies are assumed to be the same for non-initial DL and UL BWPs with the same BWP id for a RedCap UE.

According to this agreement, it is still FFS whether center frequencies of the initial DL BWP and UL BWP are the same if the initial DL BWP does not include CD-SSB and the entire CORESET#0. A compelling reason not to support aligned center frequencies of the separate initial DL and UL BWPs does not exist. Although the complexity due to RF retuning is manageable during initial access, it is still advantageous perform RF retuning when not needed; so, it would be beneficial for the center frequencies to be aligned (Figure 1a).

Proposal 1: For TDD, if the initial DL BWP does not include CD-SSB and the entire CORESET#0, center frequencies are assumed to be the same for the initial DL BWP and UL BWPs used during random access.

On the other hand, if the initial DL BWP contains the CD-SSB and the entire CORESET #0 (e.g., in the middle of the carrier), it may be preferable to place the initial UL BWP at the carrier edge to minimize PUSCH resource fragmentation (Figure 1b). In this case, alignment may not be possible. So, the following is proposed:

Proposal 2: For Option 1 and Option 2, support that the center frequencies are different if the separate initial DL BWP includes CD-SSB and the entire CORESET#0.

Figure 1 Alignment of separate initial DL and UL BWPs

# Conclusion

This contribution proposed the following for the treatment of remaining issues for reduced maximum UE bandwidth for RedCap UEs:

Proposal 1: For TDD, if the initial DL BWP does not include CD-SSB and the entire CORESET#0, center frequencies are assumed to be the same for the initial DL BWP and UL BWPs used during random access.

Proposal 2: For Option 1 and Option 2, support that the center frequencies are different if the separate initial DL BWP includes CD-SSB and the entire CORESET#0.

# References

- [1] Chairman’s Notes, 3GPP RAN1 #107-e

- [2] Chairman’s Notes, 3GPP RAN #94-e

- [3] Chairman’s Notes, 3GPP RAN1 #106bis-e