3GPP TSG RAN WG1 Meeting #108-e  	          	     		 R1-2202253

e-Meeting, February 21st – March 3rd, 2022

Agenda Item:	8.11.1.2

Source: 	LG Electronics

Title: 	Discussion on inter-UE coordination for Mode 2 enhancements

Document for:	Discussion and decision

- Introduction

In RAN1#107bis-e meeting [1], followings are agreed for the inter-UE coordination for resource allocation enhancement:

For Scheme 1: Agreement For Scheme 1, when the inter-UE coordination information transmission is triggered by UE-B’s explicit request, - Starting/Ending time locations of resource selection window is provided by UE-B’s explicit request - Starting/Ending time locations of resource selection window is a form of combination of DFN index and slot index Agreement For Scheme 1, a resource pool level (pre-)configuration can enable one of the following alternatives: - (Working Assumption) Alt1: MAC CE and 2nd SCI are used as the container of an explicit request transmission from UE-B to UE-A - A single format SCI 2-C is used for inter-UE coordination information and request - 1 bit in format 2-C is used to indicate whether the SCI is used for request to coordination information or for conveying coordination information - SCI 2-C is UE RX optional - It is up to UE implementation to additionally use 2nd SCI (for UE-B). - Alt2: MAC CE is used as the container of an explicit request transmission from UE-B to UE-A Agreement For Scheme 1, unicast is supported for an explicit request transmission for inter-UE coordination information - Unicast is used for the inter-UE coordination information transmission triggered by the explicit request Working Assumption For Scheme 1, following cast type(s) are supported for inter-UE coordination information transmission triggered by a condition other than explicit request reception - Groupcast/Broadcast for non-preferred resource set, FFS for preferred resource set - FFS: Under which conditions groupcast/broadcast can be supported - Unicast - FFS: Under which conditions unicast can be supported Agreement For determining preferred resource set in Scheme 1, the value of Cresel is determined by UE-A according to Rel-16 procedure. - This information is not conveyed to/from UE-B - When inter-UE coordination information is triggered by UE-B’s request, P_rsvp_TX used for determining SL_RESOURCE_RESELECTION_COUNTER according to Rel-16 procedure is provided by resource reservation interval indicated by UE-B’s request Agreement For the indication of resource set in Scheme 1, the value of Sl-MaxNumPerReserve is fixed to 3. Agreement The following working assumption is confirmed with modification in RED. - MAC CE or 2nd SCI are used as the container of inter-UE coordination information transmission from UE A to UE B. - For the indication of resource set, the following is supported: - N combinations of TRIV, FRIV, resource reservation period as specified in Rel-16 TS 38.214 Section 8.1.5 with following modification. The value of resource reservation period is omitted at least when the transmission of preferred resource set is triggered by UE-B’s explicit request. - First resource location of each TRIV is separately indicated by the inter-UE coordination information - If [N <= 3], MAC CE is used and it is up to UE implementation to additionally use 2nd SCI. When 2nd SCI and MAC CE are both used, the same resource set is indicated in the 2nd SCI and the MAC CE. If [N > 3], only MAC CE is used. - FFS: UE capability details - 2nd SCI is UE RX optional - The field size of the indication of resource set in a SCI format 2-C is determined by [N=3] Agreement - For inter-UE coordination information transmission in Scheme 1, - Inter-UE coordination information can be multiplexed with other data only if the source/destination ID pair is the same - Retransmission of the TB carrying inter-UE coordination information is supported - For explicit request transmission in Scheme 1, - Explicit request can be multiplexed with other data only if the source/destination ID pair is the same - Retransmission of the TB carrying request is supported Agreement For inter-UE coordination triggered by an explicit request in Scheme 1, whether or not to transmit the inter-UE coordination information upon the request reception is determined by UE-A’s implementation subject to the following procedures. - Rel-16 procedure of UL/SL prioritization, LTE SL/NR SL prioritization, and congestion control Agreement For inter-UE coordination triggered by a condition rather than request reception in Scheme 1, - A resource pool level (pre-)configuration can enable one of the following alternatives: - Alt 1: it is up to UE-A’s implementation whether or not to trigger the inter-UE coordination information generation. - Alt 2: the inter-UE coordination information generation can be triggered only when UE-A has data to be transmitted together with the inter-UE coordination information to UE-B - Note: Rel-16 procedure of UL/SL prioritization, LTE SL/NR SL prioritization, and congestion control is applied to the transmission of the inter-UE coordination information triggered by a condition. Agreement For inter-UE coordination triggered by UE-B’s explicit request in Scheme 1, - A resource pool level (pre-)configuration can enable one of the following alternatives: - Alt 1: it is up to UE-B’s implementation whether or not to trigger the request generation - Alt 2: the request generation can be triggered only when UE-B has data to be transmitted to UE-A - Note: Rel-16 procedure of UL/SL prioritization, LTE SL/NR SL prioritization, and congestion control is applied to the transmission of the request transmission. Agreement For Scheme 1 with preferred resource set Option A, - MAC layer selects resources using S_A and the received preferred resource set - MAC layer firstly selects resources for transmissions within the intersection of S_A and the preferred resource set until it becomes impossible to select a resource within the intersection under the constraint defined in Rel-16. - It is up to the UE whether to use the preferred resource set from SCI format 2-C and/or MAC CE - After this, if the number of selected resources is smaller than the required number of transmissions for a TB, MAC layer selects resources for the remaining transmissions outside the intersection but inside S_A under the constraint defined in Rel-16. Agreement For Scheme 1 with preferred resource set Option B, - MAC layer selects resources belonging to the received preferred resource set under the constraint defined in Rel-16 - It is up to the UE whether to use the preferred resource set from SCI format 2-C and/or MAC CE Agreement For inter-UE coordination information triggered by an explicit request in Scheme 1, the priority value of the inter-UE coordination information is (pre)configured priority value if it is provided by (pre)configuration. Otherwise, the priority value is the same as indicated by UE-B’s explicit request. - For the case when inter-UE coordination information is transmitted together with other data, the priority value of the multiplexed sidelink transmission is determined by the smallest priority value between the inter-UE coordination information and data Agreement For inter-UE coordination information triggered by an explicit request in Scheme 1, the priority value of explicit request is (pre)configured priority value if it is provided by (pre)configuration. Otherwise, the priority value is the same as that of a TB to be transmitted by UE-B. - For the case when the explicit request is transmitted together with other data, the priority value of the multiplexed sidelink transmission is determined by the smallest priority value between the explicit request and data Agreement For inter-UE coordination information triggered by a condition other than explicit request reception in Scheme 1, the priority value of the inter-UE coordination information is (pre)configured priority value if it is provided by (pre)configuration. - FFS: Otherwise, the priority value is determined by UE-A’s implementation. - For the case when inter-UE coordination information is transmitted together with other data, the priority value of the multiplexed sidelink transmission is determined by the smallest priority value between the inter-UE coordination information and data Agreement - For sidelink transmission carrying inter-UE coordination information in Scheme 1, - UE-A performs its resource (re)selection according to the same procedure in TS 38.214 Section 8.1.4 to transmit the inter-UE coordination information to UE-B. - For sidelink transmission carrying request in Scheme 1, - UE-B performs its resource (re)selection according to the same procedure in TS 38.214 Section 8.1.4 to transmit the request for the inter-UE coordination information to UE-A if UE-B performs sensing/resource exclusion. Otherwise, at least UE-B can perform random selection - Note: RAN1 does not pursue specific enhancement of Rel-17 resource (re)selection for the transmission of inter-UE coordination information and its request. Working Assumption First resource location of each TRIV is a slot offset with respect to a reference slot - The slot offset is the number of logical slots from the reference slot - The value range of slot offsets is from 0 to maximum value that is (pre)configurable up to [256] - FFS: The detailed value range including granularity - Slot offset for each TRIV to indicate the set of resources is separately indicated by inter-UE coordination information - For the reference slot, - The reference slot is the slot indicated by the inter-UE coordination information in a form of combination of DFN index and slot index Agreement For determining preferred resource set in Scheme 1, when inter-UE coordination information transmission is triggered by a condition other than explicit request reception, - Values of following parameters are (pre)configured for a resource pool. If there is no (pre)configuration, UE-A determines by its implementation the values of the following parameters - prio_TX - L_subCH - P_rsvp_TX - UE-A determines by its implementation values of following parameters - n+T_1, n+T_2 - FFS: Whether/how to support (pre)configuration of n+T_1 and n+T_2 - Note that it is up to RAN2 decision whether/how the values of these parameters are provided by PC5-RRC signaling from UE-B to UE-A and UE-A uses the received information to determine the preferred resource set Agreement For inter-UE coordination information is triggered by UE-B’s request, - A resource pool level (pre-)configuration can enable one of the following alternatives: - Alt 1: - Resource set type to be provided by inter-UE coordination information transmission is determined by UE-A’s implementation and its information is indicated by UE-A’s inter-UE coordination information - UE-A’s inter-UE coordination information indicates either preferred resource set or non-preferred resource set - Alt 2: - Resource set type to be provided by inter-UE coordination information transmission is indicated by UE-B’s request - UE-B’s request indicates either preferred resource set or non-preferred resource set - Note that it is up to RAN2 decision whether/how UE-B provides its support of sensing/resource exclusion to UE-A via PC5-RRC signaling and UE-A uses the received information to determine the type of resource set to be transmitted to UE-B Agreement For inter-UE coordination information is triggered by a condition other than explicit request reception, - Resource set type to be provided by inter-UE coordination information transmission is determined by UE-A’s implementation and its information is indicated by UE-A’s inter-UE coordination information - UE-A’s inter-UE coordination information indicates either preferred resource set or non-preferred resource set For Scheme 2: Conclusion For Scheme 2, there is no consensus to support the indication of the following - Condition type of a resource conflict - Time location of a resource conflict Agreement - When PSFCH occasion is derived by a slot where expected/potential resource conflict occurs on PSSCH resource indicated by UE-B’s SCI, time gap between the PSFCH and SCI(s) scheduling conflicting TBs is larger than or equal to X value - X = sl-MinTimeGapPSFCH - UE does not transmit the conflict indicator or receive the conflict indicator if the timeline is not satisfied Agreement For Scheme 2, - The PHY layer reports S_A after Step 7) of TS 38.214 Section 8.1.4 to higher layer. - When UE-B receives a conflict indicator for resource(s) indicated by its SCI, - PHY layer at UE-B reports resources overlapping with the next reserved resource indicated by the corresponding UE-B’s SCI for current TB transmission to higher layer. - If (pre)configured, the PHY layer reports resources in a slot including the next reserved resource indicated by the corresponding UE-B’s SCI for current TB transmission to higher layer. - Higher layer at UE-B re-selects the resource(s) indicated by the conflict indicator among the S_A excluding the reported resources. - FFS: Whether/How the conflict in periodic transmission is indicated by UE-A and handled by UE-B Agreement For PSFCH TX/RX or TX/TX prioritization in Scheme 2, - Priority value of PSFCH TX for a resource conflict indication is the smallest priority value of the conflicting TBs - Priority value of PSFCH RX for a resource conflict indication is priority value indicated by UE-B’s SCI - For PSFCH TX/RX or TX/TX prioritization between SL HARQ-ACK feedback(s) and resource conflict indication(s), PSFCH TX/RX for SL HARQ-ACK feedback is always prioritized over PSFCH TX/RX for a resource conflict indication Working Assumption For Scheme 2, (pre)configuration is supported to enable or disable that 1 LSB of reserved bits of a SCI format 1-A is used to indicate of whether UE scheduling a conflict TB can be UE-B or not. - FFS: UE-A's behavior for the case when at least one of UEs scheduling conflicting TBs is not capable of receiving the conflict indication

In FL’s summary made in RAN1#107bis-e meeting [2], following issues were discussed for inter-UE coordination information design:

Draft proposal 17 for conclusion: - For cast type(s) of inter-UE coordination information with preferred resource set triggered by a condition other than explicit request reception - Neither groupcast nor broadcast for preferred resource set is supported Draft proposal 18: - (Working Assumption) A SCI format 2-C includes all the fields present in SCI format 2-A - If (pre)configured, the SCI format 2-C includes zone ID field, and communication range requirement field Draft proposal 21: - For Scheme 2, - m_0 for a resource conflict indication is derived in the same way as specified for HARQ-ACK information in TS 38.213 Section 16.3 - m_CS for a resource conflict indication for UE-B’s current TB transmission is 0 - FFS: Whether/How the conflict in periodic transmission is indicated by UE-A and handled by UE-B - a UE expects that different PRBs are (pre)configured between for conflict indication and HARQ-ACK information Draft proposal 2-6: - Confirm the following working assumption with red-color changes: - Working Assumption: - For Condition 2-A-1 in Scheme 2, when “a non-destination UE of a TB transmitted by UE-B can be UE-A” is enabled or when “a non-destination UE of a TB transmitted by UE-B can be UE-A” is disabled and the destination UE of the conflicting TBs is UE-A, - for each pair of UEs scheduling the conflicting TBs whose PSFCH occasions for resource conflict indication are not yet passed, a UE with the higher priority value is UE-B. - if PSFCH occasion for conflict indication has not passed only for one of paired UEs scheduling the conflicting TBs, that UE is UE-B.

In this contribution, we continue to discuss remaining issues on the inter-UE coordination mechanism in mode 2 resource allocation for enhanced reliability and reduced latency.

- Discussion

To finalize RAN1 specification work, it would be necessary to focus on essential issues upon the agreed parts. Following is the list of the remaining essential issues:

- Finalization of contents and containers of UE-A’s inter-UE coordination information and UE-B’s explicit request, including determination of destination UE(s) for UE-A’s inter-UE coordination information and UE-B’s explicit request

- Finalization of behaviour of UE-B receiving resource set(s) from UE-A(s)

- Finalization of when and with which information UE-A generates and/or transmits an inter-UE coordination information, including triggering based on condition(s) other than an explicit request

- Finalization of determination of PSFCH resource/index for conflict indication

- Finalization of how to determine UE-B among UEs scheduling conflicting TBs, including whether/how to handle, or differently handle, the case when at least one of UEs scheduling conflicting TBs doesn’t support Scheme 2

  - Scheme 1

    - Finalization of contents and containers of UE-A’s inter-UE coordination information and UE-B’s explicit request, including determination of destination UE(s) for UE-A’s inter-UE coordination information and UE-B’s explicit request

      - Remaining issues on resource indication provided by UE-A’s inter-UE coordination information

On first resource location of each TRIV, the maximum value of the slot offset should be increased into 8000 to cover the case of resource reservation period of 1ms with SCS of 120kHz. Meanwhile, when the maximum resource reservation period configured in a resource pool is small, the maximum value of the slot offset would be (pre)configured to be small as well. In this case, the total payload size for indicating first resource location indication(s) would be also small.

In the last meeting, it was discussed that the possibility of that the reference slot of TRIV(s) other than first TRIV is the first resource location of immediately previous TRIV. However, in this case, depending on the form the set of resources, it will cause a large number of N and it will increase total payload size unnecessarily. For instance, if the number of resources is small but the time gap between them is larger than maximum slot offset value, the value of N combinations could be large to cover the wide range of time gap. In this case, most combinations of TRIV and FRIV would not be used to indicate actual resource. Instead, these combinations will be used to extend the value range of first resource location. On the other hand, when all the TRIVs have the same reference slot and a wide range of slot offset as per the working assumption, then the flexibility to indicate a variety combinations of sets of resources will be maximized.

Considering the lack of payload size of a SCI format 2-C, it would be necessary to efficiently reduce the payload size with minimizing the loss of flexibility. First of all, slot durations covered by different TRIV does not need to be partially overlapping. In other words, it would be beneficial in terms of overhead to allow only full overlapping and non-overlapping between set of slots covered by different TRIVs. In this case, the granularity of slot offset could be a multiple(s) of 31 logical slots. When the maximum value of the slot offset is 8000, then the number of slot offsets would be roughly 256, and then the maximum bit field size to indicate the first resource location of each TRIV would be reduced into 8 bits. This approach will reduce 5*N bits where N is the number of combinations to indicate the set of resources.

Proposal 1: Confirm the following working assumption with modification in RED

Working Assumption

First resource location of each TRIV is a slot offset with respect to a reference slot

- The slot offset is the number of logical slots from the reference slot

  - The value range of slot offsets is from 0 to maximum value that is (pre)configurable up to [256] 8000

    - FFS: The detailed value range including granularity

    - Granularity of slot offset is 31 (e.g. values of slot offsets is 0, 31, 62, 93, …)

  - Slot offset for each TRIV to indicate the set of resources is separately indicated by inter-UE coordination information

- For the reference slot,

  - The reference slot is the slot indicated by the inter-UE coordination information in a form of combination of DFN index and slot index

In RAN1#107bis E-meeting, it was discussed that the possibility of indicating lowest subchannel index of first resource via inter-UE coordination information. According to agreement, the set of resources will be expressed by N combinations of TRIV, FRIV, and resource reservation period. In other words, using “lowest index of the subcahnnel allocation to the initial transmission” is not part of the agreement. Moreover, since it will increase at most 5 bits for each combination, it may affect the decision on the condition of using a SCI format 2-C as a container of inter-UE coordination information. In our analysis, the maximum required size of a SCI format 2-C would be increased into 167 bits. Even though the value of N is decreased by 1, the maximum required size of a SCI format 2-C would be 148 bits. On the other hand, this approach can additionally indicate N resources by using N combinations at the expense of 5*N bits increase on the payload size.

Observation 1: Indicating and using lowest subchannel index of first resource(s) for N combinations of TRIV, FRIV, resource reservation period could additionally provide N resources at the expense of 5*N bits increase on payload size of inter-UE coordination information.

Observation 2: Indicating and using lowest subchannel index of first resource(s) for N combinations of TRIV, FRIV, resource reservation period will have an impact on decision on other discussion (e.g., a SCI format 2-C design).

      - Remaining issues on a SCI format 2-C design

As per working assumption, inter-UE coordination information will indicate a reference slot to interpret N combinations of TRIV, FRIV, and resource reservation period. Considering the form of the reference slot, it will requires at most 10 bits for indicating DFN index and 7 bits for indicating slot index within the indicated DFN. In this case, depending on the number of subchannels in a resource pool or maximum value of slot offset, N=3 would not always ensure that the size of a SCI format 2-C is not greater than 140 bits. In our analysis, even though a SCI format 2-A is a baseline for a SCI format 2-C design, the maximum required payload size of a SCI format 2-C is 152 bits (=4+1+2+8+16+1+2+1+1+3*(13+9+4)+3*8+10+7+1). Since there is the possibility of disabling a SCI format 2-C via (pre)configuration or the possibility of using MAC CE only via UE implementation, it would be fine to remove square brackets in the following agreement with adding a note. The added note could ensure that there is no UE implementation supporting a SCI format 2-C of which size exceeds 140 bits.

Proposal 2: Remove square brackets in the following agreement and add note in Blue.

Agreement

The following working assumption is confirmed with modification in RED.

- MAC CE or 2nd SCI are used as the container of inter-UE coordination information transmission from UE A to UE B.

      - For the indication of resource set, the following is supported:

        - N combinations of TRIV, FRIV, resource reservation period as specified in Rel-16 TS 38.214 Section 8.1.5 with following modification. The value of resource reservation period is omitted at least when the transmission of preferred resource set is triggered by UE-B’s explicit request.

          - First resource location of each TRIV is separately indicated by the inter-UE coordination information

        - If [N <= 3], MAC CE is used and it is up to UE implementation to additionally use 2nd SCI. When 2nd SCI and MAC CE are both used, the same resource set is indicated in the 2nd SCI and the MAC CE. If [N > 3], only MAC CE is used.

          - FFS: UE capability details

          - 2nd SCI is UE RX optional

          - The field size of the indication of resource set in a SCI format 2-C is determined by [N=3]

          - Note: UE does not expect that the total payload size of a SCI format 2-C with N=3 exceeds 140 bits

Regarding the contents of a SCI format 2-C for inter-UE coordination information transmission, cast type is necessary at least for the case when the inter-UE coordination information is triggered by a condition other than explicit request reception since multiple cast types could be used for inter-UE coordination information transmission.

According to Rel-16 procedure, zone ID and communication range requirement are used to determine whether or not to transmit SL HARQ-ACK feedback in a response of groupcast PSSCH with SL HARQ-ACK feedback option 1. In other words, RX UE will try to decode the received PSSCH even though its location is outside the indicated communicated range requirement from the TX UE. In this point of view, it seem that there is no difference in UE-B’s behavior depending on whether inter-UE coordination information includes zone ID and communication range requirement.

Next, even though zone ID and communication range requirement are not included in a SCI format 2-C, the inter-UE coordination information can be transmitted together with a TB with distance-based HARQ-ACK feedback when MAC CE conveying inter-UE coordination information is scheduled by a SCI format 2-B. In other words, it is still possible that all the types of data can be multiplexed with inter-UE coordination information even though a SCI format 2-C does not support distance-based HARQ-ACK feedback.

Observation 3: Inter-UE coordination information can be transmitted together with a TB with distance-based HARQ-ACK feedback when MAC CE conveying inter-UE coordination information is scheduled by a SCI format 2-B.

Proposal 3: A SCI format 2-C includes all the fields present in SCI format 2-A

In this case, the contents of a SCI format 2-C conveying inter-UE coordination information can be summarized as the following proposal.

Proposal 4: Support following SCI format 2-C for Scheme 1

- The following information is transmitted by means of the SCI format 2-C:

  - HARQ process number – 4 bits.

  - New data indicator – 1 bit.

  - Redundancy version – 2 bits as defined in Table 7.3.1.1.1-2.

  - Source ID – 8 bits as defined in clause 8.1 of [TS 38.214].

  - Destination ID – 16 bits as defined in clause 8.1 of [TS 38.214].

  - HARQ feedback enabled/disabled indicator – 1 bit as defined in clause 16.3 of [TS 38.213].

  - Cast type indicator – 2 bits as defined in Table 8.4.1.1-1 and in clause 8.1 of [TS 38.214].

  - CSI request – 1 bit as defined in clause 8.2.1 of [6, TS 38.214] and in clause 8.1 of [TS 38.214].

  - Providing/Requesting indicator – 1 bit

    - ‘0’ indicates inter-UE coordination information

    - ‘1’ indicates an explicit request for inter-UE coordination information

  - Resource combination(s) – 3×log2(N subChannel SLN subChannel SL + 12N subChannel SL + 16)+9+log2Nrsv_period  bits,

    - where

      - Nrsv_period is the number of entries in the higher layer parameter sl-ResourceReservePeriodList, if higher layer parameter sl-MultiReserveResource is configured; 1 otherwise

  - Slot offset(s) for first resource location(s) – 3×NmaxSlotOffsetTRIV31 bits,

    - where

      - NmaxSlotOffsetTRIV is provided by the higher layer parameter maxSlotOffsetTRIVScheme1

      - Slot offset for each first resource location is 31*n where n is the decimal number of the bit values

  - Reference slot location

    - DFN index – 10 bits

    - Slot index – log2(10∙2μ) bits

  - Resource set type – 1 bit

    - ‘0’ indicates preferred resource set

    - ‘1’ indicates non-preferred resource set

      - Remaining issues on cast type(s) of inter-UE coordination information transmission

When inter-UE coordination information is triggered by a condition other than explicit request reception, groupcast and broadcast are supported for inter-UE coordination information transmission with non-preferred resource set. Remaining RAN1 issue is whether groupcast and broadcast are supported for preferred resource set or not. Considering that the multiple UE-Bs receives the same set of preferred resources, resource collision among the UE-Bs would occur frequently especially when UE-Bs don’t have their own sensing results. In this point of view, we suggest to support unicast only for the inter-UE coordination information transmission with preferred resource set.

Proposal 5: For cast type(s) of inter-UE coordination information transmission with preferred resource set triggered by a condition other than explicit request reception

  - Neither groupcast nor broadcast for preferred resource set is supported

    - Finalization of behaviour of UE-B receiving resource set(s) from UE-A(s)

According to agreements made in this WI, UE-B’s behavior focus on the case when UE-B receives only one inter-UE coordination information from UE-A. It would be necessary to determine whether or how to support the case when UE-B receives multiple inter-UE coordination information from a single UE-A or different UE-As.

First for all, since the preferred resource set determined by UE-A is basically the set of resources with low interference at UE-A side, UE-A should be a destination of a TB to be transmitted by UE-B based on the preferred resource set. When UE-B’s PSSCH transmission is unicast, since UE-B will have different unicast link for different UE-As, UE-B will not use inter-UE coordination information from UE-A for PSCCH/PSSCH transmission to another UE. Instead, if UE-B receives more than one inter-UE coordination information from the same UE-A, it can be considered that UE-B uses the latest inter-UE coordination information in its resource (re)selection subject to processing time budget. In short, UE-B will use only one preferred resource set for a certain unicast link in a time.

When UE-B’s transmission is groupcast and UE-B knows multiple UE-As are interesting groupcast destination ID to be used for UE-B’s transmission, UE-B may need to use intersection of all the preferred resource set transmitted by multiple destination UE-As. However, in this case, the amount of candidate single-slot resources would not be sufficient, so most inter-UE coordination information would be not be actually used by UE-B in its resource (re)selection. In our view, rather than skipping most preferred resource set provided by the inter-UE coordination information, it would be much better to focus on a certain preferred resource set to boost up a certain UE-A’s performance.

Similarly, for non-preferred resource set, when UE-B uses all the received inter-UE coordination information, UE-B may not find sufficient number of candidate single-slot resources. It will make UE-B use high interference resources or collide with UE not supporting inter-UE coordination. In other words, UE-B’s transmission would not be protected.

Considering unclear benefit of using multiple inter-UE coordination information and huge specification work, we suggest that when UE-B receives multiple inter-UE coordination information, UE-B determines one of them by implementation to use in its resource (re)selection.

Proposal 6: When UE-B receives multiple inter-UE coordination information from the same UE-A or different UEs, UE-B determines one of them by implementation to use in its resource (re)selection.

    - Finalization of when and with which information UE-A generates and/or transmits an inter-UE coordination information, including triggering based on condition(s) other than an explicit request

For inter-UE coordination information triggered by UE-B’s explicit request, the resource selection window is provided by UE-B’s request. In this case, according to TS38.214 section 8.1.4, the associated sensing window could be determined. To be specific, UE-A will monitor logical slots until a slot T_proc,0+T_1 before the beginning of UE-B’s resource selection window provided by UE-B’s request. In this stage, it would have a huge specification work and impact to extend UE-A’s sensing window for determining the set of resources considering both UE-A’s resource selection window for inter-UE coordination information transmission and UE-B’s resource selection window in a time.

For inter-UE coordination information triggered by a condition other than explicit request reception, the resource selection window is determined by UE-A. According to the existing MAC procedure, once SL data is available in a logical channel in MAC layer of UE-A, UE-A can trigger resource (re)selection procedure. In this case, the sensing window for determining the set of resources would be independent on the actual transmission time of inter-UE coordination information. In other words, there is no need to have further specification change for defining sensing window for determining the set of resources.

Observation 4: There is no need to update the sensing window for determining the set of resources further for both inter-UE coordination information triggered by an explicit request and inter-UE coordination information triggered by a condition other than explicit request reception.

In the perspective of UE-B, it would be possible that the remaining processing time budget is not enough to use the received inter-UE coordination information in its resource (re)selection. It will take the processing time of T_proc,0 to interpret the received inter-UE coordination information, and it will take the processing time of T_proc,1 or T_1 to perform resource (re)selection based on the received inter-UE coordination information and to prepare SL transmission. When UE-B performs resource (re)selection based on the received inter-UE coordination information, the effective resource selection window size would needs to be still larger than or equal to T_2,min. In those points of views, UE-B may use the inter-UE coordination information in its own resource (re)selection at least when the inter-UE coordination information arrives at UE-B side to cover those processing time budgets. In the perspective of UE-A, when UE-A knows the ending time of UE-B’s resource selection window, UE-A needs to transmit inter-UE coordination information so that UE-B’s effective resource selection window size is larger than or equal to T_2,min subject to processing time budget.

Proposal 7: In Scheme 1, UE-B can use the received inter-UE coordination information from UE-A until UE-B’s remaining resource selection window is larger than or equal to T_2,min subject to processing time budget.

  - Scheme 2

    - Finalization of determination of PSFCH resource/index for conflict indication

Differentiating m_0 to indicate some information is a kind of channel selection mechanism which is not supported in NR. Moreover, this channel selection mechanism is already prevented in Rel-16 design due to some high UE complexity. In this case, it seem not clear benefit to introduce new PSFCH resource determination rule compared to SL HARQ-ACK feedback. Moreover, the total number of cyclic shifts (m_0+m_CS) is related to the gap between different cyclic shift values, and the gap is highly related to the target delay spread of SL transmission. In this case, other values for m_0 and m_CS except for values specified in TS38.213 16.3 seems not necessary.

On the resource conflict in periodic transmission, when UE-A transmits different m_CS value to indicate resource conflict for the next TB or the next period, it will require duplicated discussion regarding the following conclusion made in RAN1#107bis E-meeting.

Conclusion For Scheme 2, there is no consensus to support the indication of the following - Condition type of a resource conflict - Time location of a resource conflict

Furthermore, considering principles of re-evaluation/pre-emption, it can be considered that resource reselection triggered by a resource conflict indication is also applied to the current TB or the current period.

Proposal 8: For Scheme 2,

  - m_0 for a resource conflict indication is derived in the same way as specified for HARQ-ACK information in TS 38.213 Section 16.3

  - m_CS for a resource conflict indication for UE-B’s current TB transmission is 0

  - a UE expects that different PRBs are (pre)configured between for conflict indication and HARQ-ACK information

    - Finalization of how to determine UE-B among UEs scheduling conflicting TBs, including whether/how to handle, or differently handle, the case when at least one of UEs scheduling conflicting TBs doesn’t support Scheme 2

In Scheme 2, UE-A can observe two or more reserved resources indicated by SCIs transmitted by different UEs in different slots are overlapping in time-and-frequency domain, and then UE-A can decide which UE(s) scheduling the conflict TBs will receive the resource conflict indication depending on the corresponding priority values. If PSFCH occasion is derived by a slot where UE-B’s SCI is transmitted, there is a case where UE-A determines the existence of the resource conflict after some portion of PSFCH occasion(s) are passed. In this case, it is necessary to decide whether or not to transmit the resource conflict when the remaining PSFCH occasion is associated with reserved resources with high priority. When UE-A does not support transmitting the resource conflict for this case, the resource conflict will not be resolved and the reserved resource with high priority itself will suffer from high interference. On the other hand, when UE-A supports transmitting the resource conflict for this case, the observed resource conflict will be resolved but the reserved resource with high priority needs to be re-selected, which may cause another pre-emption or resource collision. In our view, it would be more beneficial to handle the expected problem first.

In addition, in RAN1#107bis E-meeting, it is agreed as a working assumption that the possibility of that UE scheduling a conflict TB indicates whether the UE can be UE-B or not. In this case, UE-B selection mechanism also needs to be modified considering the UE type indication. For simplicity, we suggest that the UE paring for determining UE-B considers only UEs scheduling the conflicting TBs with second UE flag of 1 whose PSFCH occasions for resource conflict indication are not yet passed.

Proposal 9: Confirm the following working assumption with modification in RED:

  - Working Assumption:

    - For Condition 2-A-1 in Scheme 2, when “a non-destination UE of a TB transmitted by UE-B can be UE-A” is enabled or when “a non-destination UE of a TB transmitted by UE-B can be UE-A” is disabled and the destination UE of the conflicting TBs is UE-A,

      - for each pair of UEs scheduling the conflicting TBs with second UE flag of 1 whose PSFCH occasions for resource conflict indication are not yet passed, a UE with the higher priority value is UE-B.

      - if PSFCH occasion for conflict indication has not passed only for one of paired UEs scheduling the conflicting TBs with second UE flag of 1, that UE is UE-B.

- Conclusions

In this contribution, we discussed inter-UE coordination for mode 2 resource allocation enhancement. Based on the above discussion, our observations and proposals are given as follows:

Observation 1: Indicating and using lowest subchannel index of first resource(s) for N combinations of TRIV, FRIV, resource reservation period could additionally provide N resources at the expense of 5*N bits increase on payload size of inter-UE coordination information.

Observation 2: Indicating and using lowest subchannel index of first resource(s) for N combinations of TRIV, FRIV, resource reservation period will have an impact on decision on other discussion (e.g., a SCI format 2-C design).

Observation 3: Inter-UE coordination information can be transmitted together with a TB with distance-based HARQ-ACK feedback when MAC CE conveying inter-UE coordination information is scheduled by a SCI format 2-B.

Observation 4: There is no need to update the sensing window for determining the set of resources further for both inter-UE coordination information triggered by an explicit request and inter-UE coordination information triggered by a condition other than explicit request reception.

Proposal 1: Confirm the following working assumption with modification in RED

Working Assumption

First resource location of each TRIV is a slot offset with respect to a reference slot

- The slot offset is the number of logical slots from the reference slot

  - The value range of slot offsets is from 0 to maximum value that is (pre)configurable up to [256] 8000

    - FFS: The detailed value range including granularity

    - Granularity of slot offset is 31 (e.g. values of slot offsets is 0, 31, 62, 93, …)

  - Slot offset for each TRIV to indicate the set of resources is separately indicated by inter-UE coordination information

- For the reference slot,

  - The reference slot is the slot indicated by the inter-UE coordination information in a form of combination of DFN index and slot index

Proposal 2: Remove square brackets in the following agreement and add note in Blue.

Agreement

The following working assumption is confirmed with modification in RED.

- MAC CE or 2nd SCI are used as the container of inter-UE coordination information transmission from UE A to UE B.

      - For the indication of resource set, the following is supported:

        - N combinations of TRIV, FRIV, resource reservation period as specified in Rel-16 TS 38.214 Section 8.1.5 with following modification. The value of resource reservation period is omitted at least when the transmission of preferred resource set is triggered by UE-B’s explicit request.

          - First resource location of each TRIV is separately indicated by the inter-UE coordination information

        - If [N <= 3], MAC CE is used and it is up to UE implementation to additionally use 2nd SCI. When 2nd SCI and MAC CE are both used, the same resource set is indicated in the 2nd SCI and the MAC CE. If [N > 3], only MAC CE is used.

          - FFS: UE capability details

          - 2nd SCI is UE RX optional

          - The field size of the indication of resource set in a SCI format 2-C is determined by [N=3]

          - Note: UE does not expect that the total payload size of a SCI format 2-C with N=3 exceeds 140 bits

Proposal 3: A SCI format 2-C includes all the fields present in SCI format 2-A

Proposal 4: Support following SCI format 2-C for Scheme 1

- The following information is transmitted by means of the SCI format 2-C:

  - HARQ process number – 4 bits.

  - New data indicator – 1 bit.

  - Redundancy version – 2 bits as defined in Table 7.3.1.1.1-2.

  - Source ID – 8 bits as defined in clause 8.1 of [TS 38.214].

  - Destination ID – 16 bits as defined in clause 8.1 of [TS 38.214].

  - HARQ feedback enabled/disabled indicator – 1 bit as defined in clause 16.3 of [TS 38.213].

  - Cast type indicator – 2 bits as defined in Table 8.4.1.1-1 and in clause 8.1 of [TS 38.214].

  - CSI request – 1 bit as defined in clause 8.2.1 of [6, TS 38.214] and in clause 8.1 of [TS 38.214].

  - Providing/Requesting indicator – 1 bit

    - ‘0’ indicates inter-UE coordination information

    - ‘1’ indicates an explicit request for inter-UE coordination information

  - Resource combination(s) – 3×log2(N subChannel SLN subChannel SL + 12N subChannel SL + 16)+9+log2Nrsv_period  bits,

    - where

      - Nrsv_period is the number of entries in the higher layer parameter sl-ResourceReservePeriodList, if higher layer parameter sl-MultiReserveResource is configured; 1 otherwise

  - Slot offset(s) for first resource location(s) – 3×NmaxSlotOffsetTRIV31 bits,

    - where

      - NmaxSlotOffsetTRIV is provided by the higher layer parameter maxSlotOffsetTRIVScheme1

      - Slot offset for each first resource location is 31*n where n is the decimal number of the bit values

  - Reference slot location

    - DFN index – 10 bits

    - Slot index – log2(10∙2μ) bits

  - Resource set type – 1 bit

    - ‘0’ indicates preferred resource set

    - ‘1’ indicates non-preferred resource set

Proposal 5: For cast type(s) of inter-UE coordination information transmission with preferred resource set triggered by a condition other than explicit request reception

  - Neither groupcast nor broadcast for preferred resource set is supported

Proposal 6: When UE-B receives multiple inter-UE coordination information from the same UE-A or different UEs, UE-B determines one of them by implementation to use in its resource (re)selection.

Proposal 7: In Scheme 1, UE-B can use the received inter-UE coordination information from UE-A until UE-B’s remaining resource selection window is larger than or equal to T_2,min subject to processing time budget.

Proposal 8: For Scheme 2,

  - m_0 for a resource conflict indication is derived in the same way as specified for HARQ-ACK information in TS 38.213 Section 16.3

  - m_CS for a resource conflict indication for UE-B’s current TB transmission is 0

  - a UE expects that different PRBs are (pre)configured between for conflict indication and HARQ-ACK information

Proposal 9: Confirm the following working assumption with modification in RED:

  - Working Assumption:

    - For Condition 2-A-1 in Scheme 2, when “a non-destination UE of a TB transmitted by UE-B can be UE-A” is enabled or when “a non-destination UE of a TB transmitted by UE-B can be UE-A” is disabled and the destination UE of the conflicting TBs is UE-A,

      - for each pair of UEs scheduling the conflicting TBs with second UE flag of 1 whose PSFCH occasions for resource conflict indication are not yet passed, a UE with the higher priority value is UE-B.

      - if PSFCH occasion for conflict indication has not passed only for one of paired UEs scheduling the conflicting TBs with second UE flag of 1, that UE is UE-B.

Reference

[1] 3GPP RAN1#107bis-e meeting chairman’s note.

[2] R1-2200748, “Feature lead summary #7 for AI 8.11.1.2 Inter-UE coordination for Mode 2 enhancements,” Moderator (LG Electronics).