3GPP TSG RAN WG1 Meeting #108-e					           R1-2202257

e-Meeting, February 21st – March 3rd, 2022

______________________________________________________________________Agenda item: 8.11.1.2

Source: Moderator (LG Electronics)

Title: Feature lead summary #4 for AI 8.11.1.2 Inter-UE coordination for Mode 2 enhancements

Document for: Discussion and information

- Draft proposals for Monday’s GTW (February 28)

  - Scheme 2

FL’s observation of 4th email discussion (for UE-B’s behavior for the received conflict indication for next TB transmission)

- Alt 1: Intel, CATT, Ericsson, Spreadtrum, InterDigital, (6)

- Alt 1 with modification: Apple, (1)

  - Replace “UE-B’s last SCI for current TB transmission” to “UE-B’s SCI with TRIV=0”: Apple, (1)

- Alt 2: Fraunhofer, Nokia, LGE, Qualcomm, Samsung, Futurewei, Huawei, OPPO, vivo, Sharp, xiaomi, Panasonic, NEC, MediaTek, Fujitsu, ZTE (16)

Draft Proposal 4-1:

Alt 1:

- For UE-B’s periodic transmission in Scheme 2,

  - When PSFCH occasion is derived by a slot where UE-B’s SCI is transmitted,

    - When UE-B receives a conflict indicator in a PSFCH occasion derived by UE-B’s SCI with TRIV=0 UE-B’s last SCI for current TB transmission,

      - PHY layer at UE-B reports resources overlapping with the next earliest reserved resource for next TB transmission to higher layer.

        - If (pre)configured, the PHY layer reports resources in a slot including the next earliest reserved resource for next TB transmission to higher layer.

      - Higher layer at UE-B re-selects the resource(s) indicated by the conflict indicator among the S_A excluding the reported resources.

    - Note: In case of UE-B’s periodic transmission, when UE-B receives a conflict indicator for resource(s) indicated by its SCI with nonzero TRIV, UE-B’s behavior agreed in RAN1#107bis-e meeting is applied

  - When PSFCH occasion is derived by a slot where expected/potential resource conflict occurs on PSSCH resource indicated by UE-B’s SCI,

    - When UE-B receives a conflict indicator in a PSFCH occasion derived by the reserved resource for next TB transmission,

      - PHY layer at UE-B reports resources overlapping with the reserved resource for next TB transmission to higher layer.

        - If (pre)configured, the PHY layer reports resources in a slot including the reserved resource for next TB transmission to higher layer.

      - Higher layer at UE-B re-selects the resource(s) indicated by the conflict indicator among the S_A excluding the reported resources.

  - Note: the existing higher layer parameter of “slotLevelResourceExclusionScheme2” is reused for the (pre)configuration

  - Note: In case of UE-B’s periodic transmission, when UE-B receives a conflict indicator for resource(s) indicated by its SCI for current TB, UE-B’s behavior agreed in RAN1#107bis-e meeting is applied

Alt 2 (Note: red-marked part is the change from the previous agreement):

- For Scheme 2,

  - The PHY layer reports S_A after Step 7) of TS 38.214 Section 8.1.4 to higher layer.

  - When UE-B receives a conflict indicator for resource(s) indicated by its SCI,

    - PHY layer at UE-B reports resources overlapping with the next reserved resource indicated by the corresponding UE-B’s SCI for current TB transmission to higher layer.

      - If (pre)configured, the PHY layer reports resources in a slot including the next reserved resource indicated by the corresponding UE-B’s SCI for current TB transmission to higher layer.

    - Higher layer at UE-B re-selects the resource(s) indicated by the conflict indicator among the S_A excluding the reported resources.

FL’s observation of 4th email discussion (clarification on the meaning of “next reserved resource indicated by the corresponding UE-B’s SCI for current TB transmission”)

- Support: Fraunhofer, Intel, Apple, LGE, Qualcomm, Futurewei, CATT, OPPO, Ericsson, Fujitsu, Spreadtrum, vivo, xiaomi, Panasonic, NEC, MediaTek (16)

- Not support: DCM, Sharp, InterDigital, (3)

- Comments:

  - No additional agreement is needed: Samsung, (1)

Draft conclusion 4-2 (Note: it will be discussed after making a decision on Draft Proposal 4-1):

When PSFCH occasion is derived by a slot where UE-B’s SCI is transmitted,

- if there is a PSFCH occasion satisfying “the minimum time gap (sl-MinTimeGapPSFCH) between the PSFCH occasion and a slot where the SCI is transmitted” but not satisfying “the minimum time gap (T_3) between the PSFCH occasion and a slot of the earliest reserved PSSCH resource indicated by the corresponding SCI after the PSFCH occasion”,

  - the PSFCH occasion cannot be used by UE-A for a conflict indication for reserved PSSCH resource other than the earliest reserved PSSCH resource indicated by the corresponding SCI after the PSFCH occasion

FL’s observation of 4th email discussion (for UE-B determination)

- Draft proposal 4-3:

  - Support: Fraunhofer, Intel, Nokia, LGE, Qualcomm, DCM, CATT, Huawei, OPPO, Ericsson, xiaomi, InterDigital, Panasonic, NEC, MediaTek (15)

  - Not support: Apple, Samsung, Fujitsu, Spreadtrum, (4)

- Update note (i.e., adding “if it has higher priority value” at the end of the note):

  - Support: Fraunhofer, Intel, Nokia, CATT, OPPO, vivo (6)

  - Not support: LGE, Qualcomm, DCM, Futurewei, Huawei, Ericsson, Fujitsu, Spreadtrum, InterDigital, Panasonic, (10)

- Comments:

  - Add “or pre-emption is disabled in the resource pool” to the note: Nokia, CATT, (2)

  - Add “UE-A does not consider the SCIs received later than “sl-MinTimeGapPSFCH” before the PSFCH occasion when determining UE-B” as a note: Apple, Sparedtrum, (2)

  - Keep the original WA: Samsung, (1)

  - Additional enhancement for UE-B determination in addition to draft proposal 3-2: Futurewei, (1)

  - Consider only UEs satisfying the timeline of PSFCH (i.e., adding a note that if there is only one UE scheduling the conflicting TB whose PSFCH occasion for resource conflict indication satisfies the timeline and indicationUEB flag is set to 1 if the higher parameter of indicationUEBScheme2 is (pre)configured to ‘Enabled’, that UE is UE-B): Fujitsu, (1)

  - Add “if either of the following conditions is true (1) it has higher priority value and preemption is enabled or (2) pre-emption is disabled in the resource pool” to a note: Intel, (1)

Draft proposal 4-3:

- Confirm the following working assumption with modification in RED. Note that the terminology of “indicationUEB flag” means the indication of whether UE scheduling a conflict TB can be UE-B or not.

  - Working Assumption:

    - For Condition 2-A-1 in Scheme 2, when “a non-destination UE of a TB transmitted by UE-B can be UE-A” is enabled or when “a non-destination UE of a TB transmitted by UE-B can be UE-A” is disabled and the destination UE of the conflicting TBs is UE-A,

      - for each pair of UEs scheduling the conflicting TBs whose PSFCH occasions for resource conflict indication are not yet passed and indicationUEB flag is set to 1 if the higher parameter of indicationUEBScheme2 is (pre)configured to ‘Enabled’, a UE with the higher priority value is UE-B. When the UEs in the pair have the same priority value, UE-A determines which one of the UEs is UE-B by its implementation.

        - Note: if there is only one UE scheduling the conflicting TB whose PSFCH occasion for resource conflict indication is not yet passed and indicationUEB flag is set to 1 if the higher parameter of indicationUEBScheme2 is (pre)configured to ‘Enabled’, that UE is UE-B.

        - Note: UE-A considers the SCIs received earlier than or equal to sl-MinTimeGapPSFCH before the PSFCH occasion for conflict indication when determining UE-B.

FL’s observation of 4th email discussion (conflict indication prioritization rule for overlapping with LTE SL or UL )

- Support: Intel, Nokia, Apple, LGE, Qualcomm, Samsung, DCM, Futurewei, CATT, Huawei, OPPO, Panasonic, Ericsson, Fujitsu, Spreadtrum, vivo, Sharp, xiaomi, InterDigital, NEC, MediaTek (21)

Draft proposal 4-4:

A UE performs PSFCH TX/RX or TX/TX prioritization between SL HARQ-ACK feedback(s) and resource conflict indication(s) first, and then the UE performs prioritization between prioritized PSFCH TX(s) or RX(s) and LTE SL TX/RX or UL by reusing prioritization rule as specified in TS 38.213 Section 16.2.4.1 and 16.2.4.3.1.

FL’s observation of 4th email discussion (conflict indication prioritization rule for overlapping with UL containing SL HARQ-ACK information)

- Support: Intel, Apple, LGE, Qualcomm, Samsung, DCM, Futurewei, CATT, Huawei, OPPO, Ericsson, Fujitsu, Spreadtrum, Sharp, xiaomi, InterDigital, Panasonic, NEC, MediaTek (19)

- Not support: vivo, (1)

Draft conclusion 4-5:

RAN1 does not pursue specific enhancement of Rel-17 inter-UE coordination operation for handling the overlapping between UL with SL-HARQ-ACK information and PSFCH for a conflict indication, i.e., there is no case in Rel-17 where the overlapping between UL with SL-HARQ-ACK information and PSFCH for a conflict indication occur at a UE performing inter-UE coordination operation

FL’s observation of 4th email discussion (Enhancement on resource selection procedure based on the timeline of a conflict indication)

- Support: Fraunhofer, Intel, Apple, LGE, Qualcomm, Samsung, DCM, Futurewei, Huawei (w/ removing Note), OPPO (w/ removing Note), Ericsson, Fujitsu, Spreadtrum, vivo, xiaomi (w/ removing Note), InterDigital, Panasonic, NEC, MediaTek (19)

- Not support: CATT, (1)

Updated Draft conclusion 4-6:

RAN1 does not pursue specific enhancement in Rel-17 on Mode 2 resource selection procedure to ensure the timeline (i.e., minimum time gap between PSFCH and a slot where a SCI is transmitted of sl-MinTimeGapPSFCH, minimum time gap between PSFCH and a slot where expected/potential resource conflict occurs on PSSCH resource indicated by a SCI of T_3) for a conflict indication.

- Note: It does not preclude the possibility for UE-B to perform resource selection satisfying the above timeline by its implementation.

  - Scheme 1

FL’s observation of 4th email discussion (Whether or not to indicating actual number of resource combination in a SCI format 2-C)

- Support: Intel, Apple, LGE, CATT, Huawei, OPPO, Spreadtrum, InterDigital, Panasonic, (9)

- Not support: Fraunhofer, Qualcomm, Samsung, DCM, Ericsson, Fujitsu, vivo, Sharp, xiaomi, (9)

Updated Draft conclusion 4-7:

No consensus on indicating actual number of resource combination in a SCI format 2-C for inter-UE coordination information.

- Note : different resource combinations can indicate the same set of resources for the case when only one resource combination is actually used

FL’s observation of 4th email discussion (Bit field size of MAC CE for inter-UE coordination information when both MAC CE and a SCI format 2-C are used)

- Support: Apple, LGE, Qualcomm, DCM, Futurewei, CATT, Ericsson, Fujitsu, Spreadtrum, Fujitsu, xiaomi, InterDigital, Panasonic, (13)

- Not support: Samsung, Sharp, (2)

- Comments:

  - Add “Bit field size of “actual number of combinations” on MAC CE may be different from that on SCI format 2-C”: Apple, (1)

  - No need to discuss it: Huawei, (1)

  - Up to RAN2: Samsung, Sharp, (2)

Updated Draft proposal 3-6:

Alt 1:

- For Scheme 1, when both SCI format 2-C and MAC CE are used as the container of inter-UE coordination information, the same bit field size for inter-UE coordination information in a SCI format 2-C is applied to MAC CE except for first resource location(s) from RAN1’s perspective

  - Bit field size of the first resource location(s) on MAC CE is
log2(X) where X is provided by the (pre)configured maximum value of slot offset for the case when MAC CE only is used as a container of inter-UE coordination information

    - When both SCI format 2-C and MAC CE can be used as the container for inter-UE coordination information, UE does not expect that X is (pre)configured to be smaller than 255

  - Details (e.g., how to put these fields into MAC CE and the related field sizes in MAC CE) are up to RAN2

Alt 2:

- For Scheme 1, when both SCI format 2-C and MAC CE are used as the container of inter-UE coordination information, the same inter-UE coordination information is indicated in the SCI format 2-C and the MAC CE

  - Details (e.g., how to put the fields of SCI format 2C for inter-UE coordination information into MAC CE and the related field sizes in MAC CE) are up to RAN2

FL’s observation of 4th email discussion (Cast type of inter-UE coordination information transmission triggered by a condition other than explicit request reception)

- Support: Nokia, Apple, LGE, Qualcomm, DCM, Futurewei, CATT, OPPO, Fujitsu, Spreadtrum, Sharp, xiaomi, InterDigital, Panasonic, (14)

- Not support: Fraunhofer, Intel, Samsung, Huawei, (4)

  - Apply the same principle for the case when it is multiplexed with other data: Fraunhofer, Huawei, (2)

  - Groupcast set can be (pre)configured: Samsung, (1)

  - It is up to RAN2: Huawei, (1)

Updated Draft conclusion 3-12:

- For transmission of inter-UE coordination information with non-preferred resource set triggered by a condition other than explicit request reception,

  - When it is not multiplexed with other data, UE-A determines its cast type by implementation

FL’s observation of 4th email discussion (UE-B’s behavior when UE-B receives multiple resource sets from the same UE-A)

- Alt 1: Fraunhofer, Nokia, LGE, DCM, Ericsson, Panasonic, (6)

- Alt 1 with modification:

  - Remove sub-bullet of Option 1: Intel, Futurewei, (2)

  - Replace “all” with “applicable” in 2nd bullet: Intel, (1)

  - Remove 3rd bullet: Intel, (1)

  - Remove “to be transmitted to the UE-A” in 2nd bullet: Qualcomm, (1)

  - Use the latest IUC for all the cases: Samsung, CATT, (2)

  - Up to UE implementation for 3rd bullet: vivo, Apple, (2)

- Alt 2: LGE, Huawei, InterDigital, (3)

Updated Draft proposal 3-13:

Alt 1:

- For UE-B’s behavior when UE-B receives multiple preferred resource sets from the same UE-A,

  - Option 1: UE-B uses the latest received preferred resource set from the same UE-A subject to UE processing timeline for its resource selection for a TB to be transmitted to the UE-A.

    - It is up to UE-B's implementation on how to determine the latest received preferred resource set

- For UE-B’s behavior when UE-B receives multiple non-preferred resource sets from the same UE-A,

  - Option 3: UE-B determines a final non-preferred resource set by making union of all the received non-preferred resource sets from the same UE-A subject to UE processing timeline. UE-B uses the final non-preferred resource set for its resource selection for a TB to be transmitted to the UE-A.

- For UE-B’s behavior when UE-B receives both a single preferred resource set and a single non-preferred resource set from the same UE-A,

  - Option 3: UE-B uses both the received preferred resource set and non-preferred resource set from the same UE-A for its resource selection for a TB to be transmitted to the UE-A.

  - It is up to UE-B implementation to use one or multiple of them in its resource (re)selection

Alt 2:

- When UE-B receives multiple inter-UE coordination information from the same UE-A, it is up to UE-B implementation to use one or multiple of them in its resource (re)selection.

FL’s observation of 4th email discussion (UE-B’s behavior when UE-B receives multiple resource sets from the different UE-As)

- Alt 1: LGE, DCM, Ericsson, Panasonic, (4)

- Alt 1 with modification:

  - Remove 2nd bulllet: Fraunhofer, (1)

  - Add “Applicable“ before “received“: Intel, (1)

  - Add “In case of groupcast to the multiple UE-As, UE-B selects resources from the intersection of the received preferred resource sets“: Nokia, CATT, (2)

  - Remove “to these different UE-As providing the non-preferred resource set”: Qualcomm, Samsung, (2)

  - Use each non-preferred resource set for each UE-A: Futurewei, (1)

  - Up to UE implementation for 3rd bullet: Apple, vivo, (2)

- Alt 2: LGE, Huawei, InterDigital, (3)

Draft proposal 3-14:

Alt 1:

- For UE-B’s behaviour when UE-B receives multiple preferred resource sets from the different UE-As,

  - Option 1: UE-B uses each received preferred resource set subject to UE processing timeline for its resource selection for each TB to be transmitted to each UE-A providing the preferred resource set.

- For UE-B’s behaviour when UE-B receives multiple non-preferred resource sets from the different UE-As.

  - Option 1: UE-B determines a final non-preferred resource set by making union of all the received non-preferred resource sets from different UE-As subject to UE processing timeline. UE-B uses the final non-preferred resource set for its resource selection for TB(s) to be transmitted to these different UE-As providing the non-preferred resource sets.

- For UE-B’s behaviour when UE-B receives both a single preferred resource set and a single non-preferred resource set from the different UE-As,

  - Option 2: UE-B uses both the received preferred resource set and non-preferred resource set from different UE-As for its resource selection for a TB to be transmitted to the UE-A providing the preferred resource set. UE-B uses the received non-preferred resource set for its resource selection for a TB to be transmitted to the UE-A providing the non-preferred resource set.

  - it is up to UE-B implementation to use one or multiple of them in its resource (re)selection

Alt 2:

- When UE-B receives multiple inter-UE coordination information from the different UE-As, it is up to UE-B implementation to use one or multiple of them in its resource (re)selection.

FL’s observation of 4th email discussion ((pre)configuration of parameters related to n+T_1 and n+T_2 for determining the preferred resource set)

- Support: Fraunhofer, Intel, Apple, LGE, Qualcomm, Samsung, DCM, Futurewei, CATT, Huawei, OPPO, Ericsson, Fujitsu, Spreadtrum, vivo, Sharp, xiaomi, InterDigital, Panasonic, (19)

- Comments:

  - Add “T_1 <= Tproc,1” as note: Intel, LGE, vivo, (3)

  - Remove note: Huawei, (1)

Updated Draft conclusion 3-1:

Not support (pre)configuration of parameters related to n+T_1 and n+T_2 for determining the set of preferred resources in inter-UE coordination information triggered by a condition other than explicit request reception.

- Note that T_2 is no smaller than T_2,min and 0<=T_1 <= Tproc,1 as specified in TS 38.214 section 8.1.4.

FL’s observation of 4th email discussion (UE-A’s behavior of determining a priority value of inter-UE coordination information transmission triggered by a condition other than explicit request reception if the priority value is not (pre)configured)

- Support: Intel, Apple, LGE, Qualcomm, Samsung, CATT, Huawei, OPPO, Ericsson, Fujitsu, Spreadtrum, vivo, Sharp, xiaomi, InterDigital, Panasonic, (16)

- Not support: Fraunhofer, Nokia, DCM, Futurewei, (4)

- Comments:

  - Remover sub-bullet: Fraunhofer, Intel, Nokia, DCM, Futurewei, (5)

Updated Draft conclusion 3-11:

In RAN1, no further decision is necessary for UE-A’s behavior of determining a priority value of inter-UE coordination information transmission triggered by a condition other than explicit request reception if the priority value is not (pre)configured

- It is up to RAN2 whether/how to additionally handle this case (e.g., defining default priority value)

FL’s observation of 4th email discussion (Cast type of inter-UE coordination information when both a SCI format 2-C and MAC CE are used)

- Support: Intel, LGE, Qualcomm, DCM, CATT, Huawei, OPPO, Spreadtrum, vivo, Sharp, InterDigital, Panasonic, (12)

- Not support: Fraunhofer, Nokia, Samsung, Futurewei, (4)

Updated Draft proposal 3-8:

- For inter-UE coordination information transmission, a SCI format 2-C  can be used in addition to MAC CE only when its cast type is unicast regardless of whether or not it is multiplexed with other data

FL’s observation of 4th email discussion (Additional criteria on which received preferred non-preferred resource set(s) can be actually taken into account in UE-B’s resource selection)

- Support: Fraunhofer, LGE, Qualcomm, DCM, Futurewei, Huawei, OPPO, Fujitsu, Spreadtrum, Fujitsu, Sharp, xiaomi, InterDigital, Panasonic, (14)

- Not support: Intel, Ericsson, vivo, (3)

- FFS: Apple, (1)

- Comments:

  - Remove “additional”: Huawei, (1)

Draft conclusion 3-15:

RAN1 does not pursue defining additional criteria on filtering the received preferred or non-preferred resource set(s) to be taken into account in UE-B’s resource selection

FL’s observation of 4th email discussion (Sensing window for determining the set of resources)

- Support: Fraunhofer, LGE, Qualcomm, DCM, CATT, OPPO, Fujitsu, Spreadtrum, Sharp, InterDigital, Panasonic, (11)

- Not support: Intel, Futurewei, Huawei, vivo, (4)

- Comments:

  - Add relationship between a resource selection window for determining the set of resources and a resource selection window for transmitting the set of resources: Intel, (1)

  - Clarification on re-evaluation for the set of resources as per Rel-16 procedure: LGE, Qualcomm, OPPO, Ericsson, Spreadtrum, xiaomi, (6)

    - UE-A is required to perform at least one mandatory reevaluation at slot n’-T_3. It is up to UE-A to perform any extra reevaluation before slot n’-T_3: Qualcomm, (1)

    - It is up to UE-A to further update the set of resources before it is transmitted: OPPO, (1)

  - Remove last bullet: CATT, vivo, InterDigital, (3)

  - Remove n’ part: Huawei, (1)

  - Replace “n>=n’” with “n>n’”: vivo,

Updated Draft conclusion 3-10:

For sensing window for determining the set of resources in Scheme 1,

- No further change is supported. Note that the sensing window for determining the set of resources is already derived based on the location n+T_1 and n+T_2 used for determining the set of resources in TS38.214 section 8.1.4, i.e., sensing window is defined by the range of slots [ (n+T_1) - T_0 - T_1 determined by UE-A, (n+T_1) - T_proc,0 - T_1 determined by UE-A ).

  - For inter-UE coordination information triggered by UE-B’s explicit request, n+T_1 and n+T_2 are provided by the request.

  - For inter-UE coordination information triggered by a condition other than explicit request reception, n+T_1 and n+T_2 are determined by UE-A’s implementation. With n>= n’, where n’ is the slot in which inter-UE coordination information generation is triggered.

- Re-evaluation for the set of resources is supported as per Rel-16 procedures.

FL’s observation of 4th email discussion (Additional UE-B behavior to handle the case when it is not possible that the number of candidate single-slot resources after applying the received non-preferred resource set as per the existing agreement meets the requirement of X*M_total)

- Support: LGE, Qualcomm, DCM, Futurewei, CATT, OPPO, Huawei, Spreadtrum, vivo, xiaomi, InterDigital, Panasonic, (12)

- Not support: Fraunhofer, Intel, Nokia, Samsung, Ericsson, (5)

- Comments:

  - Fallback to TX candidate resource set: Intel, LGE, Ericsson, OPPO, xiaomi, (5)

  - Allowing partial overlapping between candidate single-slot resources and non-preferred resources: Nokia, (1)

  - Different preference levels are indicated for non-preferred resources: Samsung, (1)

  - Replace “how to meet this requirement” with “whether or not to use the received non-preferred resource set”: Huawei, (1)

  - Add “to use none or part of the non-preferred resource(s) to meet the requirement of X*M_total, when applying all the non-preferred resource(s) cannot meet the requirement of X*M_total”: vivo, (1)

Updated Draft proposal 3-16:

Alt 1:

For the case when it is not possible that the number of candidate single-slot resources after applying the received non-preferred resource set as per the existing agreement meets the requirement of X*M_total,

- It is up to UE-B’s implementation whether/how to take the received non-preferred resource set in its resource selection to meet this requirement

Alt 2:

For the case when it is not possible that the number of candidate single-slot resources after applying the received non-preferred resource set as per the existing agreement meets the requirement of X*M_total,

- UE-B does not take the received non-preferred resource set in its resource selection.

FL’s observation of 4th email discussion (Condition when Option B can be used for preferred resource set)

- Support: Fraunhofer, Apple, Qualcomm, Futurewei, CATT, Huawei, OPPO, Fujitsu, Spreadtrum, vivo, InterDigital, Panasonic, (12)

- Not support: Intel, LGE, Samsung, DCM, Ericsson, Sharp, (6)

  - Remove Option 3: Intel, LGE, Samsung, DCM, Ericsson, Sharp, (6)

  - Remove Option 2: Intel, Samsung, Ericsson, Sharp, (4)

Draft proposal 3-17:

For Scheme 1, Option B can be used for preferred resource set when at least one of following condition is met:

- Option 1: UE-B does not have a capability of performing sensing/resource exclusion.

- Option 2: UE-B performs random resource selection.

- Option 3: UE-B has a capability of performing sensing/resource exclusion, but UE-B determines not to perform sensing/resource exclusion by its implementation.

FL’s observation of 4th email discussion (Latency bound of inter-UE coordination information)

- Support: LGE, Qualcomm, Samsung, DCM, Huawei, OPPO, Ericsson, Fujitsu, InterDigital, Panasonic, (10)

- Not support: Futurewei, vivo, xiaomi, (3)

- Comments:

  - No need for conclusion: Intel, CATT, (2)

Draft conclusion 3-9:

- For latency bound of inter-UE coordination information transmission, RAN1 relies on RAN2’s decision as per LS R1-2200880 from RAN2

FL’s observation of 4th email discussion (2nd SCI format(s) than can be used to schedule retransmission of a TB containing inter-UE coordination information initially scheduled by a SCI format 2-C)

- Support: Nokia, Apple, LGE, DCM, OPPO, Fujitsu, Spreadtrum, vivo, xiaomi, InterDigital, Panasonic, (11)

- Not support: Qualcomm, Futurewei, CATT, Huawei, Ericsson, Sharp, (6)

- Comments:

  - Add “The same TBS should be ensured across (re)transmission(s) of inter-UE coordination information”: Qualcomm, Sharp, (2)

Updated Draft conclusion 3-20:

- No consensus on any restriction on a 2nd SCI format that can be used for retransmission of inter-UE coordination information MAC CE initially scheduled by a SCI format 2-C.

  - Note: At least, the same TBS should be ensured between initial transmission and retransmission(s) for the same inter-UE coordination information

FL’s observation of 4th email discussion (Restrictions to the IUC mechanism to address the power saving operation)

- Support: Fraunhofer, LGE, Qualcomm, Futurewei, Huawei, OPPO, Fujitsu, Spreadtrum, vivo, xiaomi, InterDigital, Panasonic, (12)

- Not support: Intel, CATT, Ericsson, (3)

- Comments:

  - the intention is not to cancel but rather not to trigger generation: Intel, CATT, (2)

  - Support approach with no RRC impact: Apple, DCM, (2)

Updated Draft conclusion 3-19:

No consensus on RAN1 does not pursue supporting specific enhancement on UE-A’s behavior canceling not triggering inter-UE coordination information transmission generation when the amount of sensing performed by UE-A is below a certain threshold

FL’s observation of 3rd email discussion (UE-B’s behavior to consider “the slot(s) overlapped with UE-A’s reserved resource(s) by 1st stage SCI” as non-preferred resource(s) in its resource selection)

- Support: Nokia, Apple, LGE, DCM, Futurewei, CATT, Huawei, OPPO, Ericsson, Fujitsu, Spreadtrum, Sharp, xiaomi, InterDigital, Panasonic, (15)

- Not support: Qualcomm, vivo, (2)

Draft conclusion 3-18:

No consensus on UE-B’s behavior using the slot(s) overlapped with UE-A’s reserved resource(s) by 1st stage SCI as non-preferred resource(s) in its resource selection for unicast/groupcast transmission

FL’s observation of 3rd email discussion (conclusion for the reply LS to RAN2)

- Support: Intel, LGE, DCM, Futurewei, Huawei, OPPO, Ericsson, Fujitsu, xiaomi, InterDigital, (10)

- Not support: Apple, Samsung, (2)

Draft conclusion 3-21:

- No consensus for RAN1 to send a reply LS of R1-2200880 to RAN2.

- Draft proposals for Friday’s GTW (February 25)

  - Scheme 1

FL’s observation of 3rd email discussion (WA for indicating slot offset for each TRIV)

- Support:  DCM, ZTE, LGE, OPPO, Fujitsu, Spreadtrum, NEC, xiaomi, Fraunhofer, ETRI, Huawei, Intel, Apple, Ericsson, Futurewei, Nokia, Qualcomm, Sharp, InterDigital, (20)

- Not support: vivo, Samsung,

- Comments:

  - The slot offset to the first TRIV is 0, and it not signaled: vivo, Samsung,

Updated Draft proposal 3-2:

Confirm the following working assumption with modification in RED

- Working assumption made in RAN1#107bis-e:

  - First resource location of each TRIV is a slot offset with respect to a reference slot

    - Alt 2:

      - The slot offset is the number of logical slots from the reference slot

        - The value range of slot offsets is from 0 to maximum value that is (pre)configurable up to [8000256]

          - FFS: The detailed value range including granularity

        - Slot offset for each TRIV except for first TRIV to indicate the set of resources is separately indicated by inter-UE coordination information

          - Slot offset for first TRIV is 0

    - For the reference slot,

      - The reference slot is the slot indicated by the inter-UE coordination information in a form of combination of DFN index and slot index

FL’s observation of 3rd email discussion (Maximum number of resource combinations in a SCI format 2-C)

- Support: DCM, LGE, Fujitsu, Spreadtrum, NEC, xiaomi, ETRI, Huawei, Intel, Apple, Samsung, Ericsson, Qualcomm, Sharp, InterDigital, (15)

- Not support: OPPO, Fraunhofer, Futurewei, Nokia, (4)

  - Replace “2” with “Y” where Y is the maximum integer that ensures the size of a SCI format 2-C not larger than 140: OPPO, Fraunhofer, Nokia, Qualcomm, (4)

  - Keep 3: Futurewei, (1)

Draft proposal 3-3:

- For following agreement,

  - Replace “[N<=3]” with “N<=2”

  - Replace “[N>3]” with “N>2”

  - Replace “[N=3]” with “N=2”

Agreement made in RAN1#107bis-e: The following working assumption is confirmed with modification in RED. - MAC CE or 2nd SCI are used as the container of inter-UE coordination information transmission from UE A to UE B. - For the indication of resource set, the following is supported: - N combinations of TRIV, FRIV, resource reservation period as specified in Rel-16 TS 38.214 Section 8.1.5 with following modification. The value of resource reservation period is omitted at least when the transmission of preferred resource set is triggered by UE-B’s explicit request. - First resource location of each TRIV is separately indicated by the inter-UE coordination information - If [N <= 3], MAC CE is used and it is up to UE implementation to additionally use 2nd SCI. When 2nd SCI and MAC CE are both used, the same resource set is indicated in the 2nd SCI and the MAC CE. If [N > 3], only MAC CE is used. - FFS: UE capability details - 2nd SCI is UE RX optional - The field size of the indication of resource set in a SCI format 2-C is determined by [N=3]

FL’s observation of 3rd email discussion (bit field size of a SCI format 2-C for inter-UE coordination infomration)

- Support: DCM, LGE, OPPO, Fujitsu, Spreadtrum, NEC, xiaomi, Fraunhofer, ETRI, Intel, Apple, Ericsson, Nokia, Qualcomm, Sharp, InterDigital, (16)

- Not support: vivo, Huawei, Samsung, Futurewei, (4)

  - The size for first resource location is log2(255): vivo, Fraunhofer, Samsung, (3)

  - Add actual number of resource combinations: Huawei, Apple, (2)

  - Remove the lowest subchannel index for the first resource location: Samsung, Futurewei, (2)

Updated Draft proposal 3-4:

- For Scheme 1, each bit field size of a SCI format 2-C for inter-UE coordination information is given by following table:

  - Note that lowest subchannel index for the first resource location of each TRIV is separately indicated by inter-UE coordination information

| Field name | Field size (in bits) |
| --- | --- |
| Providing/requesting indicator | 1 |
| Resource combination(s) | 2*log2(N ubchannel SLN ubchannel SL + 12N ubchannel SL + 16)+9+Y Where N ubchannel SL is provided by the higher layer parameter sl-NumSubchannel, Y=log2Nrsv_periodwith that Nrsv_period is the number of entries in the higher layer parameter sl-ResourceReservePeriodList, if higher layer parameter sl-MultiReserveResoure is configured; Y=0 otherwise. |
| First resource location(s) | log2(255) |
| Reference slot location | 10+log2(10∙2μ) Where μ is 0, 1, 2, 3 for SCS of 15kHz, 30kHz, 60kHz, 120kHz, respectively. |
| Resource set type | 1 |
| Lowest subchannel indices for the first resource location of each TRIV | [2]*log2NsubchannelSL where NsubchannelSL is provided by the higher layer parameter sl-NumSubchannel |

FL’s observation of 3rd email discussion (bit field size of a SCI format 2-C for an explicit request)

- Support: DCM, LGE, OPPO, Fujitsu, Spreadtrum, NEC, xiaomi, Fraunhofer, ETRI, Huawei, Intel, Samsung, Ericsson, Futurewei, Qualcomm, Sharp, InterDigital, (17)

- Not support: vivo, Nokia, (2)

  - Reduce bit field size of end time of a resource selection window: vivo,

  - Add “resource combination(s)”: Nokia,

- Comments:

  - Add “Note: FFS field related to latency bound of inter-UE coordination information”: Apple,

Draft proposal 3-5:

- For Scheme 1, each bit field size of a SCI format 2-C for an explicit request for inter-UE coordination information is given by following table:

| Field name | Field size (in bits) |
| --- | --- |
| Providing/requesting indicator | 1 |
| Priority | 3 |
| Number of subchannels | log2(N subChannel SL) Where N subChannel SL is provided by the higher layer parameter sl-NumSubchannel |
| Resource reservation period | Y Where Y=log2Nrsv_periodwith that Nrsv_period is the number of entries in the higher layer parameter sl-ResourceReservePeriodList, if higher layer parameter sl-MultiReserveResoure is configured; Y=0 otherwise. |
| Resource selection window location | 210+log2(10∙2μ) Where μ is 0, 1, 2, 3 for SCS of 15kHz, 30kHz, 60kHz, 120kHz, respectively. |
| Resource set type | 1 bit if determineResourceSetTypeScheme1 is set to ‘UE-B’s request’, otherwise, 0 bit |

FL’s observation of 3rd email discussion (Bit field size of MAC CE for inter-UE coordination information when both MAC CE and a SCI format 2-C are used)

- Support: DCM, LGE, Fujitsu, Spreadtrum, NEC, xiaomi, ETRI, Samsung, Ericsson, Futurewei, Qualcomm, InterDigital, (12)

- Not support: OPPO, Apple, Nokia, (3)

  - Remove sub-bullet: OPPO, Nokia, (2)

  - Add a field to indicate the number of resource combinations: Apple, (1)

- Comment:

  - The size for first resource location is log2(X): vivo, (1)

  - Leave MAC CE details up to RAN2: Huawei, Sharp, (2)

Draft proposal 3-6:

- For Scheme 1, when both SCI format 2-C and MAC CE are used as the container of inter-UE coordination information, the same bit field size for inter-UE coordination information in a SCI format 2-C is applied to MAC CE except for first resource location(s)

  - Bit field size of the first resource location(s) on MAC CE is
log2(X) where X is provided by the (pre)configured maximum value of slot offset for the case when MAC CE only is used as a container of inter-UE coordination information

    - When both SCI format 2-C and MAC CE can be used as the container for inter-UE coordination information, UE does not expect that X is (pre)configured to be smaller than 255

FL’s observation of 3rd email discussion (Bit field size of MAC CE for an explicit request inter-UE coordination information when only MAC CE is used)

- Support: DCM, vivo, LGE, OPPO, Fujitsu, Spreadtrum, NEC, xiaomi, Fraunhofer, ETRI, Huawei, Intel, Apple, Ericsson, Futurewei, Nokia, Qualcomm, InterDigital, (18)

- Not support: Samsung, (1)

  - Bit field size for first resource location of first TRIV is 0: Samsung, (1)

Draft proposal 3-7:

- For Scheme 1, when MAC CE only is used as the container of inter-UE coordination information, each bit field size for inter-UE coordination information is given by following table from RAN1’s perspective, and RAN1 understands that the maximum value of N resource combinations to be conveyed in inter-UE coordination information is bounded so that the total payload size of inter-UE coordination information leads not to exceed the size of TB including the MAC CE

  - Details (e.g., whether/how to separately indicate the value of N in the inter-UE coordination information, how to put the following fields into MAC CE and the related field sizes in MAC CE) are up to RAN2

| Field name | Field size (in bits) |
| --- | --- |
| Providing/requesting indicator | 1 |
| Resource combination(s) | N*log2(N subChannel SLN subChannel SL + 12N subChannel SL + 16)+9+Y Where N subChannel SL is provided by the higher layer parameter sl-NumSubchannel, Y=log2Nrsv_periodwith that Nrsv_period is the number of entries in the higher layer parameter sl-ResourceReservePeriodList, if higher layer parameter sl-MultiReserveResoure is configured; Y=0 otherwise. |
| First resource location(s) | N-1*log2(X) Where X is provided by the (pre)configured maximum value of slot offset for the case when MAC CE only is used as a container of inter-UE coordination information |
| Reference slot location | 10+log2(10∙2μ) Where μ is 0, 1, 2, 3 for SCS of 15kHz, 30kHz, 60kHz, 120kHz, respectively. |
| Resource set type | 1 |
| Lowest subchannel indices for the first resource location of each TRIV | N*log2NsubchannelSL Where NsubchannelSL is provided by the higher layer parameter sl-NumSubchannel. |

FL’s observation of 3rd email discussion (Cast type of inter-UE coordination information transmission triggered by a condition other than explicit request reception)

- Support: DCM, LGE, OPPO, Fujitsu, Spreadtrum, NEC, xiaomi, Fraunhofer, Huawei, Futurewei, Nokia, Qualcomm, Sharp, InterDigital, (14)

- Not support: vivo, Intel, Samsung, (3)

Draft conclusion 3-12:

- For inter-UE coordination information transmission triggered by a condition other than explicit request reception, UE-A determines its cast type by implementation

FL’s observation of 3rd email discussion (UE-B’s behavior when UE-B receives multiple resource sets from the same UE-A)

- Alt 1: DCM, OPPO, Spreadtrum, NEC, xiaomi, Fraunhofer, Intel, Ericsson, Futurewei, Nokia, Qualcomm, (11)

- Alt 2: LGE, Fujitsu, Spreadtrum, Huawei, Sharp, InterDigital, (6)

- Others: vivo, Apple, Samsung, (3)

Draft proposal 3-13:

Alt 1:

- For UE-B’s behavior when UE-B receives multiple preferred resource sets from the same UE-A,

  - Option 1: UE-B uses the latest received preferred resource set from the same UE-A for its resource selection for a TB to be transmitted to the UE-A.

- For UE-B’s behavior when UE-B receives multiple non-preferred resource sets from the same UE-A,

  - Option 3: UE-B determines a final non-preferred resource set by making union of all the received non-preferred resource sets from the same UE-A. UE-B uses the final non-preferred resource set for its resource selection for a TB to be transmitted to the UE-A.

- For UE-B’s behavior when UE-B receives both a single preferred resource set and a single non-preferred resource set from the same UE-A,

  - Option 3: UE-B uses both the received preferred resource set and non-preferred resource set from the same UE-A for its resource selection for a TB to be transmitted to the UE-A.

Alt 2:

- When UE-B receives multiple inter-UE coordination information from the same UE-A, it is up to UE-B implementation to use one or multiple of them in its resource (re)selection.

FL’s observation of 3rd email discussion (UE-B’s behavior when UE-B receives multiple resource sets from the different UE-As)

- Alt 1: DCM, Spreadtrum, NEC, xiaomi, Ericsson, Futurewei, Qualcomm, (7)

- Alt 2: LGE, Fujitsu, Huawei, Sharp, InterDigital, (5)

- Others: vivo, OPPO, Fraunhofer, Apple, Samsung, Nokia, (6)

- Comments:

  - First discuss combinations of cast types for inter-UE coordination information transmissions: Intel, (1)

Draft proposal 3-14:

Alt 1:

- For UE-B’s behaviour when UE-B receives multiple preferred resource sets from the different UE-As,

  - Option 1: UE-B uses each received preferred resource set for its resource selection for each TB to be transmitted to each UE-A providing the preferred resource set.

- For UE-B’s behaviour when UE-B receives multiple non-preferred resource sets from the different UE-As.

  - Option 1: UE-B determines a final non-preferred resource set by making union of all the received non-preferred resource sets from different UE-As. UE-B uses the final non-preferred resource set for its resource selection for TB(s) to be transmitted to these different UE-As providing the non-preferred resource sets.

- For UE-B’s behaviour when UE-B receives both a single preferred resource set and a single non-preferred resource set from the different UE-As,

  - Option 2: UE-B uses both the received preferred resource set and non-preferred resource set from different UE-As for its resource selection for a TB to be transmitted to the UE-A providing the preferred resource set. UE-B uses the received non-preferred resource set for its resource selection for a TB to be transmitted to the UE-A providing the non-preferred resource set.

Alt 2:

- When UE-B receives multiple inter-UE coordination information from the different UE-As, it is up to UE-B implementation to use one or multiple of them in its resource (re)selection.

FL’s observation of 3rd email discussion ((pre)configuration of parameters related to n+T_1 and n+T_2 for determining the preferred resource set)

- Support: DCM, ZTE, vivo, LGE, OPPO, Fujitsu, Spreadtrum, NEC, xiaomi, Fraunhofer, ETRI, Intel, Apple, Samsung, Futurewei, Nokia, Qualcomm, Sharp, InterDigital, (19)

- Comments:

  - Remove “-T_1 for determining the set of preferred resources”: Huawei, Futurewei, (2)

  - Remove “Note that”: Intel, (1)

Updated Draft conclusion 3-1:

Not support (pre)configuration of parameters related to n+T_1 and n+T_2 for determining the set of preferred resources in inter-UE coordination information triggered by a condition other than explicit request reception.

- Note that T_2 is no smaller than T_2,min as specified in TS 38.214 section 8.1.4.

FL’s observation of 3rd email discussion (UE-A’s behavior of determining a priority value of inter-UE coordination information transmission triggered by a condition other than explicit request reception if the priority value is not (pre)configured)

- Support: vivo, LGE, OPPO, Fujitsu, Spreadtrum, NEC, xiaomi, ETRI, Huawei, Apple, Samsung, Ericsson, Qualcomm, Sharp, InterDigital, (15)

- Not support: Intel, Futurewei, Nokia, (3)

- Comments:

  - Remove sub-bullet: DCM, Fraunhofer, Intel, (3)

Draft conclusion 3-11:

No further decision is necessary for UE-A’s behavior of determining a priority value of inter-UE coordination information transmission triggered by a condition other than explicit request reception if the priority value is not (pre)configured

- It is up to RAN2 whether/how to additionally handle this case

FL’s observation of 3rd email discussion (Cast type of inter-UE coordination information when both a SCI format 2-C and MAC CE are used)

- Support: DCM, vivo, LGE, OPPO, Fujitsu, Spreadtrum, NEC, ETRI, Huawei, Intel, Samsung, Qualcomm, Sharp, InterDigital, (14)

- Not support: Fraunhofer, Futurewei, Nokia, (3)

- Comments:

  - Consider possibility of using a SCI format 2-C only: Samsung, (1)

Updated Draft proposal 3-8:

- For inter-UE coordination information transmission, a SCI format 2-C  can be used in addition to MAC CE only when its cast type is unicast regardless of whether or not it is multiplexed with other data

FL’s observation of 3rd email discussion (Additional criteria on which received preferred non-preferred resource set(s) can be actually taken into account in UE-B’s resource selection)

- Option 1: DCM, LGE, OPPO, Fujitsu, Spreadtrum, NEC, xiaomi, Fraunhofer, Huawei, Nokia, Qualcomm, Sharp, InterDigital, (13)

- Option 2: Intel, Apple, (2)

- Option 3:

  - Consider distance between UE-A and UE-B: Ericsson, (1)

  - Option 2 with a specified time gap instead of (pre)configured value: Futurewei, (1)

Draft conclusion 3-15:

RAN1 does not pursue defining additional criteria on filtering the received preferred or non-preferred resource set(s) to be taken into account in UE-B’s resource selection

FL’s observation of 3rd email discussion (Sensing window for determining the set of resources)

- Alt 1: DCM, vivo, LGE, OPPO, Fujitsu, Spreadtrum, NEC, xiaomi, Fraunhofer, ETRI, Huawei, Apple, Samsung, Ericsson, Nokia, Qualcomm, Sharp, InterDigital, (18)

- Alt 2: LGE, Intel, Futurewei, (3)

- Comments:

  - Add “Re-evaluation for the set of resources is supported as per Rel-16 procedure”: Huawei, (1)

  - Add “with n>= n’, where n’ is the slot in which inter-UE coordination information generation is triggered” for condition-based IUC: Qualcomm,

Updated Draft conclusion 3-10:

For sensing window for determining the set of resources in Scheme 1,

- No further change is supported. Note that the sensing window for determining the set of resources is already derived based on the location n+T_1 and n+T_2 used for determining the set of resources in TS38.214 section 8.1.4, i.e., sensing window is defined by the range of slots [ (n+T_1) - T_0 - T_1 determined by UE-A, (n+T_1) - T_proc,0 - T_1 determined by UE-A ).

  - For inter-UE coordination information triggered by UE-B’s explicit request, n+T_1 and n+T_2 are provided by the request.

  - For inter-UE coordination information triggered by a condition other than explicit request reception, n+T_1 and n+T_2 are determined by UE-A’s implementation. With n>= n’, where n’ is the slot in which inter-UE coordination information generation is triggered.

- Re-evaluation for the set of resources is supported as per Rel-16 procedures.

FL’s observation of 3rd email discussion (Additional UE-B behavior to handle the case when it is not possible that the number of candidate single-slot resources after applying the received non-preferred resource set as per the existing agreement meets the requirement of X*M_total)

- Option 1: DCM, vivo, LGE, OPPO, Fujitsu, Spreadtrum, NEC, xiaomi, Qualcomm, Sharp, InterDigital, (11)

- Option 2:

  - UE-B does not use the received non-preferred resource sets in its resource selection: DCM, OPPO, Intel, (3)

  - UE-B does not used a subset of the received non-preferred resource sets in its resource selection until the requirement is met: Fujitsu, (1)

  - Increasing RSRP threshold: Fraunhofer, (1)

  - Different preference levels are indicated for non-preferred resources: Samsung, (1)

  - It is up to UE-B’s implementation on how to satisfy the requirement of X*M_total but UE-B should at least apply the whole slot(s) that is appeared in non-preferred resource set: Futurewei, (1)

  - Allowing partial overlapping with non-preferred resources: Nokia, (1)

- Comments:

  - Discuss draft proposal 3-20, 3-21 first: Huawei, (1)

Draft proposal 3-16:

For the case when it is not possible that the number of candidate single-slot resources after applying the received non-preferred resource set as per the existing agreement meets the requirement of X*M_total,

- It is up to UE-B’s implementation how to meet the requirement

FL’s observation of 3rd email discussion (Condition when Option B can be used for preferred resource set)

- Option 1: DCM, vivo, OPPO, Fujitsu, Spreadtrum, NEC, xiaomi, Franhofer, Huawei, Intel, Samsung, Ericsson, Futurewei, (13)

- Option 2: DCM, vivo, LGE, OPPO, NEC, xiaomi, Franhofer, Huawei, Apple, Futurewei, InterDigital, (11)

- Option 3: vivo, OPPO, NEC, xiaomi, Franhofer, Huawei, Apple, Futurewei, Qualcomm, InterDigital, (10)

- Option 4:

  - UE-B has a capability of performing sensing/resource exclusion, but UE-B is (pre)-configured not to perform sensing/resource exclusion in SL DRX : InterDigital, (1)

Draft proposal 3-17:

For Scheme 1, Option B can be used for preferred resource set when one of following condition is met:

- Option 1: UE-B does not have a capability of performing sensing/resource exclusion.

- Option 2: UE-B performs random resource selection.

- Option 3: UE-B has a capability of performing sensing/resource exclusion, but UE-B determines not to perform sensing/resource exclusion by its implementation.

FL’s observation of 3rd email discussion (Latency bound of inter-UE coordination information)

- Support: DCM, LGE, OPPO, Huawei, Samsung, Ericsson, Nokia, Qualcomm, InterDigital, (9)

- Not support: xiaomi, Intel, Futurewei, (3)

Draft conclusion 3-9:

- For latency bound of inter-UE coordination information transmission, RAN1 relies on RAN2’s decision as per LS R1-2200880 from RAN2

FL’s observation of 3rd email discussion (2nd SCI format(s) than can be used to schedule retransmission of a TB containing inter-UE coordination information initially scheduled by a SCI format 2-C)

- Support: DCM, vivo, LGE, OPPO, Fujitsu, Spreadtrum, NEC, xiaomi, Fraunhofer, Nokia, (10)

- Not support: Huawei, Intel, Apple, Samsung, Ericsson, Futurewei, Qualcomm, Sharp, (8)

  - Use only  SCI format 2-C for the retransmission: Huawei, Intel, Apple, Ericsson, Qualcomm, Sharp, (6)

  - Not use a SCI format 2-C for the retransmission: Samsung, (1)

  - Not support retransmission of inter-UE coordination information: Futurewei, (1)

Draft conclusion 3-20:

- No consensus on any restriction on a 2nd SCI format that can be used for retransmission of inter-UE coordination information MAC CE initially scheduled by a SCI format 2-C.

FL’s observation of 3rd email discussion (Restrictions to the IUC mechanism to address the power saving operation)

- Support: Intel, Ericsson, (2)

- Not support: vivo, LGE, OPPO, Fujitsu, Spreadtrum, NEC, xiaomi, Fraunhofer, Huawei, Samsung, Qualcomm, Sharp, InterDigital, (13)

- Comments:

  - Consider RAN2 impact: Apple, Futurewei, (2)

Draft conclusion 3-19:

RAN1 does not pursue specific enhancement on UE-A’s behavior canceling inter-UE coordination information transmission when the amount of sensing performed by UE-A is below a certain threshold

FL’s observation of 3rd email discussion (UE-B’s behavior to consider “the slot(s) overlapped with UE-A’s reserved resource(s) by 1st stage SCI” as non-preferred resource(s) in its resource selection)

- Support: vivo, OPPO, Spreadtrum, xiaomi, Intel, Nokia, Qualcomm, Sharp , InterDigital, (9)

- Not support: DCM, Fujitsu, NEC, Huawei, Samsung, Ericsson, LGE, (7)

- Comments:

  - Not support the case when a SCI format 1-A is transmitted without PSSCH: LGE, (1)

  - Define similar behavior for unicast and groupcast to target RX UE: Intel, (1)

  - UE-A is the destination UE of the TB to be transmitted by UE-B: Apple, Futurewei, (2)

Draft conclusion 3-18:

No consensus on UE-B’s behavior using the slot(s) overlapped with UE-A’s reserved resource(s) by 1st stage SCI as non-preferred resource(s) in its resource selection for unicast/groupcast transmission

  - Others

FL’s observation of 3rd email discussion (conclusion for the reply LS to RAN2)

- Support: DCM, LGE, OPPO, Fujitsu, xiaomi, Huawei, Ericsson, Futurewei, Qualcomm, Sharp, InterDigital, (11)

- Not support: NEC, Intel, (2)

Draft conclusion 3-21:

- No consensus for RAN1 to send a reply LS of R1-2200880 to RAN2.

- Draft proposals for 4th email discussion (Due: February 28th 4:59am UTC)

  - Scheme 2

FL’s observation of 3rd email discussion (for UE-B’s behavior for the received conflict indication for next TB transmission)

- Support: LGE, Fujitsu, Fraunhofer, Intel, Samsung, Ericsson, InterDigital, (7)

- Comments:

  - 1st sub-bullet:

    - Clarify that UE-B reports to higher layer “collision” for both aperiodic resource and periodic resource: DCM, (1)

    - Remove “latest”: OPPO, Spreadtrum, Apple, Qualcomm, (4)

    - Remove “for current TB transmission”: OPPO, Spreadtrum, Apple, Qualcomm, (4)

    - Remove “before next reserved resource for next TB transmission”: OPPO, Spreadtrum, Apple, Ericsson, Qualcomm, (5)

    - Add “earliest” before reserved resource: OPPO, Spreadtrum, Apple, Qualcomm, (4)

    - Add “either current TB or” before the next TB transmission: OPPO, Spreadtrum, Apple, Qualcomm, (4)

    - Add “where the next reserved resource is” before for next TB transmission: Samsung, Ericsson, (2)

    - Replace “latest” with “last”: Nokia, (1)

  - 2nd sub-bullet:

    - Remove “earliest”: DCM, ZTE, (2)

    - Remove “for next TB transmission”: ZTE, Apple, (2)

    - Remove 2nd sub-bullet: vivo, (1)

    - Replace “earliest” with “next”: Apple, (1)

  - Other aspects:

    - UE-B reselects reserved resources for next TB transmission when next TB is available: vivo, LGE, xiaomi, (3)

    - Remove “for current TB transmission” in the previous agreement for UE-B’s behavior for the received conflict indication: Huawei, (1)

    - Add “when UE-B has periodic resource reservation”: Apple, (1)

    - Consider at least one of UEs scheduling conflicting TBs is not capable of receiving the conflict indication: Futurewei, (1)

    - Consider UE-B’s SCI missing: Qualcomm, (1)

    - Add “the same RRC parameter as for the first part of the above agreement is used”: Ericsson, (1)

Q4-1: FL understands that there are two interpretations of the agreement for m_CS when PSFCH occasion is derived by a slot where UE-B’s SCI is transmitted.		One is that UE-A can transmit a conflict indication for reserved resource for next TB transmission in PSFCH occasion derived by UE-B’s last SCI for current TB transmission regardless of whether or not the SCI for current TB transmission directly indicates the reserved resource for next TB transmission. 		The other is that UE-A can transmit a conflict indication for next reserved resource for next TB transmission in PSFCH occasion derived by UE-B’s SCI for current TB transmission only when the next reserved resource indicated by the SCI for current TB transmission is for next TB transmission. 		 For a formal case, as several companies already commented, UE-A could fail to identify which received SCI is the last SCI for current TB transmission due to SCI missing. This lead to a misalignment between UE-A and UE-B on the next reserved resource for next TB transmission associated with the actual last SCI. 		FL prepares two alternatives of UE-B’s behaviour based on different interpretations mentioned above. Company provides which alternative is supported.

Draft Proposal 4-1:

Alt 1:

- For UE-B’s periodic transmission in Scheme 2,

  - When PSFCH occasion is derived by a slot where UE-B’s SCI is transmitted,

    - When UE-B receives a conflict indicator in a PSFCH occasion derived by UE-B’s last SCI for current TB transmission,

      - PHY layer at UE-B reports resources overlapping with the next earliest reserved resource for next TB transmission to higher layer.

        - If (pre)configured, the PHY layer reports resources in a slot including the next earliest reserved resource for next TB transmission to higher layer.

      - Higher layer at UE-B re-selects the resource(s) indicated by the conflict indicator among the S_A excluding the reported resources.

  - When PSFCH occasion is derived by a slot where expected/potential resource conflict occurs on PSSCH resource indicated by UE-B’s SCI,

    - When UE-B receives a conflict indicator in a PSFCH occasion derived by the reserved resource for next TB transmission,

      - PHY layer at UE-B reports resources overlapping with the reserved resource for next TB transmission to higher layer.

        - If (pre)configured, the PHY layer reports resources in a slot including the reserved resource for next TB transmission to higher layer.

      - Higher layer at UE-B re-selects the resource(s) indicated by the conflict indicator among the S_A excluding the reported resources.

  - Note: the existing higher layer parameter of “slotLevelResourceExclusionScheme2” is reused for the (pre)configuration

  - Note: In case of UE-B’s periodic transmission, when UE-B receives a conflict indicator for resource(s) indicated by its SCI for current TB, UE-B’s behavior agreed in RAN1#107bis-e meeting is applied

Alt 2 (red-marked part is the change from the previous agreement):

- For Scheme 2,

  - The PHY layer reports S_A after Step 7) of TS 38.214 Section 8.1.4 to higher layer.

  - When UE-B receives a conflict indicator for resource(s) indicated by its SCI,

    - PHY layer at UE-B reports resources overlapping with the next reserved resource indicated by the corresponding UE-B’s SCI for current TB transmission to higher layer.

      - If (pre)configured, the PHY layer reports resources in a slot including the next reserved resource indicated by the corresponding UE-B’s SCI for current TB transmission to higher layer.

    - Higher layer at UE-B re-selects the resource(s) indicated by the conflict indicator among the S_A excluding the reported resources.

| Company | Alt | Comments |
| --- | --- | --- |
| Fraunhofer | Alt 2 | We are fine with Alt 2 due to UE-A not being able to identify the last resource reserved for the current TB (or periodic transmission). |
| Intel | Alt.1 |  |
| Nokia, NSB | Alt 2 |  |
| Apple | Alt 1 with modifications | The ambiguity may exist in “UE-B’s last SCI for current TB transmission”. In chained retransmissions where the chain is broken, it is not guaranteed to say which is the last SCI for current TB. We may modify “UE-B’s last SCI for current TB transmission” to “UE-B’s SCI with TRIV=0”, which implies no more reservation of the same TB at the current stage. We think in Alt 2, “the next reserved resource indicated by the corresponding UE-B’s SCI” may not be very accurate. Consider the case where 1 retransmission of the same TB is supported for periodic transmission. To indicate the potential resource collision of the initial transmission of period 2, if PSFCH occasion corresponds to the initial transmission of period 1, then it may lead to ambiguity whether this indication is for the initial transmission of period 2 or is for the retransmission of period 1. This is shown in the following figure. To avoid resource collision indication ambiguity, it is better to send PSFCH corresponding to the retransmission of period 1 to indicate the initial transmission of period 2. (Although the SCI for the retransmission of period 1 does not indicate the resource of the initial transmission of period 2, UE-B knows what the next reserved resource is based on its resource selection.) This is shown in the following figure. |
| LGE | Alt 2 | Considering error propagation issue (i.e., SCI for determining a resource conflict and SCI for determining PSFCH resource can be different), we support Alt 2. |
| Qualcomm | Alt 2 | We think that it is beneficial to clarify the meaning of “next reserved resource”. In our understanding, for periodic transmissions both the next resource at slot T+a as well as the periodic reservation for the next TB at slot T+Tperiod is indicated to the upper layer when a conflict indicator is received. For example, if SPS periodicity is 50 slots and SCI is transmitted at slot 9 and reserves the next re-transmission of the TB at slot 15; then UE-B report both Slot 15 and 59 to upper layer if conflict indicator is received for the reservation at Slot 15. |
| Samsung | Alt 2 |  |
| NTT DOCOMO | Comment | We think Alt 1 has a lot of ambiguous texts. - For the 1st bullet, UE-B cannot distinguish collision at periodic reserved or that at aperiodic reserved resource from the received collision indication. Then if UE-B reserved from a SCI both periodic resource and aperiodic resource, and when UE-B receives corresponding collision indication, what is the expected behavior? This should be clarified under the 1st bullet. - For the 2nd bullet, periodic reservation/aperiodic reservation does not matter for this option. When collision indication is received for a resource, then the reserved resource is reported to MAC layer. Why do we need “current” “next” for this option? - For the last note, what is the intention of this note? This intends a case where UE-B reserved from a SCI both periodic resource and aperiodic resource? If correct, then the 1st bullet should be restricted by using “ONLY” like, “when the UE-B’s last SCI for current TB transmission reserved only resource for next TB transmission”. In addition, if correct, this note is unnecessary for the 2nd bullet. In short, we think this discussion/proposal should be handled separately between the options of PSFCH occasion determination. |
| Futurewei | Alt. 2 | We think that alt 2 with updates from previous agreement is sufficient. |
| CATT, GOHIGH | Alt 1 | For alt 1, UE-A can know which SCI transmitted from UE-B will be the potential last SCI, for example, if a SCI has not reserve next reserved resource is deemed as a last SCI. when a UE-A doesn’t detect a such SCI, it can not transmit the conflict indication for next TB. For alt 2, if retransmission is configured, we don’t know how it can address the resource conflict of initial transmission for periodic transmission. If it can not support resource conflict for initial transmission, this is definitely different from the motivation of the agreement in GTW session. |
| Huawei, HiSilicon | Alt 2 | On the newly made agreement about m_CS, our understanding is “m_CS for a resource conflict indication for the next reserved resource indicated by the corresponding UE-B’s SCI is 0 regardless the next reserved resource is for current TB transmission or for next TB transmission”. Some examples are given in the Fig 1/2/3 below. Alt 2 correctly reflects this intention, and is thus supported. Alt 1 is unclear and has technical issues, thus Alt 1 cannot be agreed in its current form. Technical issues of Alt 1 are as below: - As FL already mentioned, how does UE-A know which received SCI is the last SCI for UE-B’s current TB transmission? - UE-A may miss some SCIs for initial or re-transmissions. - In addition, it seems some company thinks “TRIV=0 in a SCI” is equivalent to “this SCI is the last SCI for current TB”. However, this is not true! - Because chain reservation cannot be always guaranteed during the re-transmissions of a TB, i.e., in some re-transmission, UE-B may set TRIV=0 if UE-B cannot find a nearby resource for next retransmission, and UE-B may continue future retransmission using unreserved resource. - For example, as Fig 3 below shows, it’s possible that TRIV=0 for SCI in A2, but SCI in A10 is the last SCI for current TB. - In general, the term “last SCI” is very unclear. We suggest proponents of Alt 1 to use more accurate terms (e.g., TRIV=0 or not) to clearly explain their understanding. - “… the next earliest reserved resource for next TB transmission …”: this part is also unclear. - As Fig 3 below shows, if UE-A receives SCI in A10, which resource is “the next earliest reserved resource for next TB transmission”? It’s B1 or B10? - It seems some company thinks B1 is the answer. But how can UE-A knows where B1 is by decoding SCI in A10? Note that UE-A could miss some SCIs for initial/re-retransmission (e.g., may miss SCI in A1), and chain reservation cannot always be guaranteed (e.g., maybe TRIV=0 for SCI in A2). == Fig 1 Fig 2 Fig 3 |
| OPPO | Alt 2 | As we commented in the last round, if go with Alt 1, the previous agreement has also to be modified, otherwise, they conflict with each other (as the previous agreement says only report resource for current TB to higher layer). |
| Ericsson | Alt 1 |  |
| Spreadtrum | Alt .1 |  |
| Fujitsu | Alt 1 | In our understanding, Alt 1 is the intention of the previous agreement on indicating the conflict of the next TB. For Alt 2, since the last SCI of a current TB does not have to reserve the next TB for the next period, the conflict of the next TB will not be indicated in most cases. |
| Vivo | Alt2 | Alt2 is aligned with the agreement of conflict indication, i.e., the confclit on the next reserved resource will be indicated. |
| Sharp | Alt 2 |  |
| xiaomi | Alt2 | We share the similar view with Fraunhofer, it is more clear and simple. |
| Panasonic | Alt2 | We see alt 2 cannot indicate conflict of initial transmission of next TB when aperiodic transmission is also allocated. However, alt 2 has no issue on SCI missing. So, we support alt 2. |

FL’s observation of 3rd email discussion (clarification on the meaning of “next reserved resource indicated by the corresponding UE-B’s SCI for current TB transmission”)

- Support: ZTE, vivo, LGE, Fujitsu, Spreadtrum, NEC, xiaomi, Fraunhofer, ETRI, Apple, Samsung, Qualcomm, InterDigital, (13)

- Not support: DCM, Intel, Nokia, (3)

- Comments:

  - Delay this discussion after deciding draft proposal 3-1: OPPO, Huawei, (2)

  - Remove “for current TB transmission”: Futurewei, (1)

  - Remove “for receiving a conflict indicator for resource(s) indicated by the SCI”: InterDigital, (1)

  - Do not need to discuss it: Ericsson, (1)

Q4-2: FL understands that based companies’ feedback on the meaning of “next reserved resource indicated by the corresponding UE-B’s SCI for current TB transmission” during 3rd email discussion, the point needs to be clarified further is UE-A’s behaviour that if there is a PSFCH occasion satisfying “the minimum time gap (sl-MinTimeGapPSFCH) between the PSFCH occasion and a slot where the SCI is transmitted” but not satisfying “the minimum time gap (T_3) between the PSFCH occasion and a slot of the earliest reserved PSSCH resource after the PSFCH occasion”, whether or not to allow for UE-A to use the PSFCH occasion for a conflict indication for reserved PSSCH resource other than the earliest reserved PSSCH resource after the PSFCH occasion. FL thinks that after having this clarification, no additional clarification is necessary for UE-B’s behavior. Do you agree the following draft conclusion 4-2?

Draft conclusion 4-2:

When PSFCH occasion is derived by a slot where UE-B’s SCI is transmitted,

- if there is a PSFCH occasion satisfying “the minimum time gap (sl-MinTimeGapPSFCH) between the PSFCH occasion and a slot where the SCI is transmitted” but not satisfying “the minimum time gap (T_3) between the PSFCH occasion and a slot of the earliest reserved PSSCH resource after the PSFCH occasion”,

  - the PSFCH occasion cannot be used by UE-A for a conflict indication for reserved PSSCH resource other than the earliest reserved PSSCH resource after the PSFCH occasion

| Company | Yes or no | Comments |
| --- | --- | --- |
| Fraunhofer | Yes |  |
| Intel | Yes |  |
| Apple | Yes | If our understanding of the proposal is as illustrated as the following figure, we support the proposal. |
| LGE | Yes |  |
| Qualcomm | Yes |  |
| Samsung | No | We think that no more agreements are necessary based on the yellow marked in the previous agreement as Agreement - When PSFCH occasion is derived by a slot where expected/potential resource conflict occurs on PSSCH resource indicated by UE-B’s SCI, time gap between the PSFCH and SCI(s) scheduling conflicting TBs is larger than or equal to X value - X = sl-MinTimeGapPSFCH - UE does not transmit the conflict indicator or receive the conflict indicator if the timeline is not satisfied |
| NTT DOCOMO | No | We think the situation is a bit incorrect. The situation is, there is a PSFCH occasion - satisfying “the minimum time gap (sl-MinTimeGapPSFCH) between the PSFCH occasion and a slot where the SCI is transmitted”, and - satisfying “the minimum time gap (T_3) between the PSFCH occasion and a slot of the reserved PSSCH resource with resource collision after the PSFCH occasion”, and - not satisfying “the minimum time gap (T_3) between the PSFCH occasion and a slot of the earliest reserved PSSCH resource after the PSFCH occasion”. Note that agreed text for T_3 is the following. Not with the earliest resource. Then why such a new restriction is necessary? Time gap between the PSFCH and a slot where expected/potential resource conflict occurs is larger than or equal to T_3 |
| Futurewei | Yes | We agree. |
| CATT, Yes |  |  |
| Huawei, HiSilicon | Need more clarification | - As commented for proposal 4-1 above, the meaning of “earliest reserved PSSCH resource” is not clear. - As Fig 3 in our reply for proposal 4-1 (also copied below) shows, if UE-A receives SCI in A10, which resource is “the next earliest reserved resource for next TB transmission”? It’s B1 or B10? - We are not clear about the scenario of this conclusion. Does it refer to UE-B transmits single or multiple SCIs to reserve multiple resources? Does the reserved resources refer to current TB, or next TB, or does not matter? - Will it also happen when PSFCH occasion is derived by a slot where expected/potential resource conflict occurs In general, more clarifications are needed. Fig 3 |
| OPPO | Yes | We share the same understanding as Apple illustrated. |
| Ericsson | Yes | We can accept this conclusion. |
| Spreadtrum | Yes |  |
| Fujitsu | Yes |  |
| Vivo | Yes | In our understanding, it means that UE-A does not transmit PSFCH, which is aligned with privious agreement |
| Sharp | Comment | We share DCM’s understanding on existing RAN1 agreements. On the other hand, since there was no consensus on indicating the time location of resource conflict we are also fine with what the proposed conclusion says. To avoid confusions, it seems better if RAN1 can update the definition of the timeline in the yellow highlighted bullet below (rather than making the proposed conclusion): - RAN1#107-e Agreement: - A resource pool level (pre-)configuration uses either of the following options - Option 1: PSFCH occasion is derived by a slot where UE-B’s SCI is transmitted - Reuse PSSCH-to-PSFCH timing as specified in TS 38.213 Section 16.3 to determine the PSFCH occasion for resource conflict indication Time gap between the PSFCH and a slot where expected/potential resource conflict occurs is larger than or equal to T_3 |
| xiaomi | Yes |  |
| Panasonic | Yes |  |

FL’s observation of 3rd email discussion (for UE-B determination)

- Support: DCM, ZTE, LGE, OPPO, Fujitsu, Spreadtrum, NEC, xiaomi, Fraunhofer, ETRI, Apple, Ericsson, Nokia, Qualcomm, InterDigital, (15)

- Not support: Intel, Samsung, Futurewei, (3)

- Comments:

  - Clarify the value of the minimum time gap between PSFCH and a slot where expected/potential resource conflict: vivo, (1)

  - Last note is unnecessary since it is already agreed: xiaomi, Ericsson, (2)

  - Clarify whether/how the last note affect to UE-B’s determination: Huawei, (1)

  - Add “if it has higher priority value” to the 1st note: Intel, (1)

  - Consider at least one of Ues scheduling conflicting TBs is not capable of receiving the conflict indication: Futurewei, (1)

FL’s observation of 2nd email discussion: - Additional enhancement for UE-B determination in addition to draft proposal 3-2: Apple, Futurewei, Spreadtrum, (3) - Apple: Further consider a case where both Ues, scheduling the conflict TB, do not have the capability of receiving IUC scheme 2 - Futurewei: If at least one of Ues scheduling conflicting TBs is not capable of receiving the conflict indication, except the Ues not capable of receiving the conflict indication, all other Ues scheduling the conflicting TBs whose PSFCH occasions for resource conflict indication are not yet passed are UE-B - Spreadtrum: Clarification on “Capable of receiving the conflict indication”. - No additional enhancement for UE-B determination in addition to draft proposal 3-2: DCM, Panasonic, ETRI, LGE, Qualcomm, Sharp, ZTE, Fujitsu, OPPO, Samsung, xiaomi, Ericsson, CATT, Fraunhofer, Huawei, Intel, (16)

Q4-3: FL observed that companies’ view were divergent on whether or not to keep the last note. FL understands that the intention of the agreement of “UE does not transmit the conflict indicator or receive the conflict indicator if the timeline is not satisfied” is that UE(s) not satisfying the timeline are finally precluded after applying UE-B determination procedure based on the draft proposal 4-3. Assuming that the blue-marked part is common understanding, companies provide whether the following updated proposal is supported, including whether or not it is acceptable to add “if it has higher priority value” at the end of the note. In the 2nd round of email discussion, FL understands that it is clear that majority companies does not prefer to have additional enhancement for UE-B determination in addition to draft proposal 4-3. So, FL encourages companies to focus on discussing the current draft proposal 4-3 itself.

Draft proposal 4-3:

- Confirm the following working assumption with modification in RED. Note that the terminology of “indicationUEB flag” means the indication of whether UE scheduling a conflict TB can be UE-B or not.

  - Working Assumption:

    - For Condition 2-A-1 in Scheme 2, when “a non-destination UE of a TB transmitted by UE-B can be UE-A” is enabled or when “a non-destination UE of a TB transmitted by UE-B can be UE-A” is disabled and the destination UE of the conflicting TBs is UE-A,

      - for each pair of Ues scheduling the conflicting TBs whose PSFCH occasions for resource conflict indication are not yet passed and indicationUEB flag is set to 1 if the higher parameter of indicationUEBScheme2 is (pre)configured to ‘Enabled’, a UE with the higher priority value is UE-B.

        - Note: if there is only one UE scheduling the conflicting TB whose PSFCH occasion for resource conflict indication is not yet passed and indicationUEB flag is set to 1 if the higher parameter of indicationUEBScheme2 is (pre)configured to ‘Enabled’, that UE is UE-B.

| Company | Yes or no | Yes or no for adding “if it has higher priority value” at the end of the note | Comments |
| --- | --- | --- | --- |
| Fraunhofer | Yes | Yes |  |
| Intel | YES | YES |  |
| Nokia, NSB | Yes | Yes, with comment | Add “if it has higher priority value or pre-emption is disabled in the resource pool” Reason for proposed addition: If pre-emption is disabled, the other UE (e.g., for whom the PSFCH occasion has passed) will not perform pre-emption even if it has higher priority value (e.g., 8), thus the UE for whom the PSFCH occasion has not passed should be UE-B even if it has lower priority value (e.g., 1), in order to protect it from the other UE. |
| Apple | No |  | We still think it is necessary to clarify “PSFCH occasion for resource conflict indication are not yet passed.” In the example we mentioned in the previous round: UE1’s SCI is sent in slot 1 whose PSFCH occasion is in slot 3 and UE2’s SCI is sent in slot 3 whose PSFCH occasion is in slot 5. Near the end of slot 3, both PSFCH occasions have not been passed. However, due to processing limitation, UE-A may not decode UE2’s SCI before the end of slot 3. Hence, UE2’s SCI is considered when UE-A determines UE-B at the end of slot 3. We need to make this point clear since otherwise, UE-A is mandatory to decode UE2’s SCI before the end of slot 3 when determining UE1 is UE-B or not. We suggest adding another note to the proposal: Note: UE-A does not consider the SCIs received later than “sl-MinTimeGapPSFCH” before the PSFCH occasion when determining UE-B. |
| LGE | Yes | No | First of all, it seems necessary to clarify that the UE has higher priority value among conflicting TBs. Next, it seems not consistent with the modified 1st subbullet of the WA. To be specific, if there is a SCI format 1-A with indicationUE flag of 0 and higher priority value, a UE with indicationUE flag of 0 and lower priority value can transmit a conflict indication. Technically, we see a benefit of reselecting resources of the UE in the note regardless of its priority value. The reselection will help to protect UE-B’s transmission. |
| Qualcomm | Yes | No | In our understanding, it will be beneficial to notify UE-B of a potential collision even if it of a lower priority value. Otherwise, the collision may still happen in the system leading to both the receptions being degraded. |
| Samsung | No |  | We should not change the WA expect if it is clearly broken. We think that the WA is sufficient. Setting conflict indication based on the timeline or whether the UE supports or doesn’t support conflict feedback leads to the following drawbacks: - This could lead to conflicting results with Rel-16 pre-emption leading to wasted resources. - It disincentives Ues from indicating that they support conflict indication. Based on the agreement made in the Wednesday GTW, a conflict can happen for a reservation of the next TB. The SCI reserving such a resource for the next TB is sent much earlier than the resource being indicated. This proposal gives an advantage to these resources as the PSFCH occasion for indication would have most likely already past. This can be problematic when the colliding resource has an SCI that is sent later in time and is of higher priority, and potentially with a tighter PDB. |
| NTT DOCOMO | Yes | No | We feel that Apple’s clarification is good. “not yet passed” is ambiguous for spec text. We do not think “if it has higher priority value” is necessary for the last note. Regardless of priority value, the UE should be UE-B; otherwise, collision cannot be avoided due to UE not supporting scheme 2 and having higher priority value. |
| Futurewei | comment | No | We think we need to consider the case that if at least one of Ues scheduling conflicting TBs has indicationUEB flag is set to 1 or is not capable of receiving the conflict indication. We also need to consider the case of the priority value being the same. Otherwise, we do not have a complete specification on Scheme 2. Therefore, we do not think these cases are for enhancement purpose. At least some conclusions or agreements are needed for these cases. Since most companies do not want to an independent proposal to consider the case, we then prefer to work on this proposal to cover other necessary scenarios, with updated proposal below. - Working Assumption: - For Condition 2-A-1 in Scheme 2, when “a non-destination UE of a TB transmitted by UE-B can be UE-A” is enabled or when “a non-destination UE of a TB transmitted by UE-B can be UE-A” is disabled and the destination UE of the conflicting TBs is UE-A, - for each pair of Ues scheduling the conflicting TBs whose PSFCH occasions for resource conflict indication are not yet passed and indicationUEB flag is set to 1 if the higher parameter of indicationUEBScheme2 is (pre)configured to ‘Enabled’, a UE with the higher priority value is UE-B. If both Ues have the same priority, UE-A selects one UE to be UE-B by implementation. - Note: if there is only one UE scheduling the conflicting TB whose PSFCH occasion for resource conflict indication is not yet passed and or indicationUEB flag is set to 1 if the higher parameter of indicationUEBScheme2 is (pre)configured to ‘Enabled’, that UE is UE-B. Note that in UE feature discussions, for scheme 2 Rx (FG 32-5b-2), almost all companies support the capability signalling exchange between Ues. So it is not clear then how to determine the UE-B, if one of Ues signaling capable of Scheme 2 Rx but with indicationUEB set to 0. |
| CATT, GOHIGH | Yes | Comment | Regarding whether adding “if it has higher priority value”, we share similar views as Nokia, I,e. “if it has higher priority value or pre-emption is disabled in the resource pool” |
| Huawei, HiSilicon | Yes | No | We are generally fine with the proposal. There is no need to add “if it has higher priority value” at the end of the note. Because there is only one UE whose PSFCH occasion is not passed and supports being UE-B, so there is no need to compare priorities anymore. Otherwise, it’s possible that no UE will be UE-B, and the collision cannot be avoided. |
| OPPO | Yes | Yes with comments | We think it should be “highest” rather than “higher”, as there may be more than 2 Ues scheduling the conflicting TB. “if it has higher highest priority value” |
| Ericsson | Yes | No |  |
| Spreadtrum | No | No | For the clarification that “PSFCH occasions for resource conflict indication are not yet passed”, We support the comment of Apple. |
| Fujitsu | Comments | No | We think a Note on the timeline is needed. For example, the conflict in the following figure cannot be solved by proposal 4-3 without a timeline note. In the figure, it is assumed that PSFCH is derived by the slot where UE-B’s SCI is transmitted. For the conflict between PSSCH3 (UE1) and PSSCH5 (UE2), PSFCH2 and PSFCH4 can indicate the conflict to UE1 and UE2 respectively. Since PSFCH2 does not satisfy the timeline, only UE2 can be UE-B. More specifically, UE2 is UE-B no matter whether UE2 has the higher priority value or not. UE-B is determined due to “the timeline is satisfied”, but not due to “the priority is higher” or “PSFCH is not yet passed”. This should be additionally clarified. Actually, another parallel sub-bullet is more preferred as marked by the bule color. - Note: if there is only one UE scheduling the conflicting TB whose PSFCH occasion for resource conflict indication is not yet passed and indicationUEB flag is set to 1 if the higher parameter of indicationUEBScheme2 is (pre)configured to ‘Enabled’, that UE is UE-B. - Note: if there is only one UE scheduling the conflicting TB whose PSFCH occasion for resource conflict indication satisfies the timeline and indicationUEB flag is set to 1 if the higher parameter of indicationUEBScheme2 is (pre)configured to ‘Enabled’, that UE is UE-B. |
| vivo |  | Yes, Comment | We prefer to remove the whole note, if we add “if it has higher priority value” at the end of the note. |
| Xiaomi | Yes | Yes |  |
| Panasonic | Yes | No | When only one of paired UEs scheduling the conflicting TBs has inter-UE coordination capability, the UE with inter- UE coordination capability should be UE-B regardless of priority value to avoid collision. |

Q4-4: FL understands that main motivation of draft proposal 4-4 is to clarify the executing order of “PSFCH TX/RX or TX/TX prioritization between SL HARQ-ACK feedback(s) and resource conflict indication(s)”. So, the current full sentence is needed. With this understanding, companies provide their views on draft proposal 4-4.

FL’s observation of 3rd email discussion (conflict indication prioritization rule for overlapping with LTE SL or UL )

- Support: DCM, ZTE, vivo, LGE, OPPO, Fujitsu, Spreadtrum, NEC, xiaomi, ETRI, Huawei, Intel, Samsung, Ericsson, Futurewei, Nokia, Qualcomm, InterDigital, (18)

- Not support:

- Comments:

  - Make an agreement instead of a conclusion: Apple, (1)

Draft proposal 4-4:

A UE performs PSFCH TX/RX or TX/TX prioritization between SL HARQ-ACK feedback(s) and resource conflict indication(s) first, and then the UE performs prioritization between prioritized PSFCH TX(s) or RX(s) and LTE SL TX/RX or UL by reusing prioritization rule as specified in TS 38.213 Section 16.2.4.1 and 16.2.4.3.1.

| Company | Yes or no | Comments |
| --- | --- | --- |
| Intel | Yes | Support P#4-4 |
| Nokia, NSB | Yes |  |
| Apple | Yes | We think this proposal should be agreement based on the following reasons: - In Rel 16/17 sidelink, we do not have any agreement about prioritization order for the prioritization among PSFCH, the prioritization between NR SL and LTE SL/Uu. Hence, an agreement is necessary, especially considering different prioritization orders will lead to different prioritization results. - In the current specification, it is unclear which prioritization order is taken. Actually, our understanding of the current specification is that the prioritization between NR SL and LTE SL is performed before the prioritization among PSFCH. In TS38.213, Section 16.2.4.1, it is mentioned that “If a UE- would transmit a first channel/signal using E-UTRA radio access and second channels/signals using NR radio access, and…” This implies that multiple NR channels could be compared with LTE channel. Based on the above reasons, we think an agreement is needed here which will have spec. impact. |
| LGE | Yes | This does not always imply that the spec updated is needed. |
| Qualcomm | Yes |  |
| Samsung | Yes |  |
| NTT DOCOMO | Yes | And should be discussed/approved in email reflector since there is no company not supporting this proposal. |
| Futurewei | Yes | We are fine with the proposal although we think the second part “and then…” is not necessary. |
| CATT, GOHIGH | Yes |  |
| Huawei, HiSilicon | Yes |  |
| OPPO | Yes |  |
| Ericsson | Yes |  |
| Spreadtrum | Yes |  |
| Fujitsu | Yes |  |
| Vivo | Yes |  |
| Sharp | Yes |  |
| Xiaomi | Yes |  |
| Panasonic | Yes |  |

Q4-5: FL understands that the simultaneous mode 1 and 2 operation is not supported from a single UE perspective in Rel-16. Moreover, RAN1 already agreed that “For inter-UE coordination operation in Rel-17, RAN1 understands that only UE(s) in mode 2 can be UE-A”. Companies provide their views on draft conclusion 4-5.

FL’s observation of 3rd email discussion (conflict indication prioritization rule for overlapping with UL containing SL HARQ-ACK information)

- Support: DCM, ZTE, LGE, Fujitsu,  Spreadtrum, NEC, xiaomi, ETRI, Huawei, Intel, Apple, Samsung, Ericsson, Nokia, Qualcomm, InterDigital, (16)

- Not support: vivo, Futurewei, (2)

Draft conclusion 4-5:

RAN1 does not pursue specific enhancement of Rel-17 inter-UE coordination operation for handling the overlapping between UL with SL-HARQ-ACK information and PSFCH for a conflict indication.

| Company | Yes or no | Comments |
| --- | --- | --- |
| Intel | Yes | Only Mode-2 is considered |
| Apple | Yes |  |
| LGE | Yes |  |
| Qualcomm | Yes |  |
| Samsung |  | OK for this but we do not need to spend time for this conclusion. |
| NTT DOCOMO | Yes |  |
| Futurewei | Yes with comment | We are fine with the proposal although we are also ok to discuss the prioritization with UL with SL-HARQ-ACK information if a UE can be configured with both modes, e.g. in different pools but slot overlap. |
| CATT, GOHIGH | Yes | No need to involve mode 1. |
| Huawei, HiSilicon | Yes |  |
| OPPO | Yes |  |
| Ericsson | Yes |  |
| Spreadtrum | Yes |  |
| Fujitsu | Yes |  |
| Vivo | See comment | The discussion point is how to handle the conflict between Rel-17 PSFCH and UL. I wonder why companies says the scenario does not exist. Actually, we cannot preclude co-existence of mode 1 and mode 2 in the same carrier in different resource pools. For the conclusion 4-5, the consequence of the conclusion is not clear. If there is not enhancement, what would be the UE behavior if Rel-17 PSFCH is conflict with UL containing SL-HARQ, we need some clarification on this point. Alt.1:UE does not expect overlap between Rel-17 PSFCH and UL containing SL HARQ. Alt.2: when there is overlap between Rel-17 PSFCH and UL containing SL HARQ, it is up to implementation to prioritize either Rel-17 PSFCH or UL containing SL HARQ. Alt.3: overlap between Rel-17 PSFCH and UL containing SL HARQ is an error case which is not addressed by specification. |
| Sharp | Yes |  |
| Xiaomi | Yes |  |
| Panasonic | Yes |  |

Q4-6: Do you agree following updated draft conclusion?

FL’s observation of 3rd email discussion (Enhancement on resource selection procedure based on the timeline of a conflict indication)

- Support: DCM, ZTE, vivo, LGE, OPPO, Fujitsu, Spreadtrum, NEC, xiaomi, ETRI, Huawei, Intel, Samsung, Ericsson, Nokia, Qualcomm, InterDigital, (17)

- Not support: Futurewei, (1)

Updated Draft conclusion 4-6:

RAN1 does not pursue specific enhancement in Rel-17 on Mode 2 resource selection procedure to ensure the timeline (i.e., minimum time gap between PSFCH and a slot where a SCI is transmitted of sl-MinTimeGapPSFCH, minimum time gap between PSFCH and a slot where expected/potential resource conflict occurs on PSSCH resource indicated by a SCI of T_3) for a conflict indication.

| Company | Yes or no | Comments |
| --- | --- | --- |
| Fraunhofer | Yes |  |
| Intel | Yes |  |
| Apple | Yes |  |
| LGE | Yes |  |
| Qualcomm | Yes |  |
| Samsung |  | OK for this but we do not need to spend time for this conclusion. |
| NTT DOCOMO | Yes |  |
| Futurewei | Yes | Since majority does not prefer, we are ok with the proposal without the enhancement on the time gap. |
| CATT, GOHIGH | No | We think this is related to how to feedback the conflict indication effectively. If there is no resource selection enhancement to ensure timeline, then the resource conflict can not be feedback. This will degrade the performance of scheme 2, even not useful for scheme 2. We are concerning going this direction. |
| Huawei, HiSilicon | Yes |  |
| OPPO | Yes |  |
| Ericsson | Yes |  |
| Spreadtrum | Yes |  |
| Fujitsu | Yes |  |
| Vivo | Yes |  |
| Xiaomi | Yes |  |
| Panasonic | Yes |  |

  - Scheme 1

Q4-7: For the following agreement made in Friday’s GTW session, companies provide their views on whether or not to support defining additional field of “Actual number of resource combination” in SCI format 2-C for inter-UE coordination information?

- Agreement

  - For Scheme 1, each bit field size of a SCI format 2-C for inter-UE coordination information is given by following table:

    - Note that lowest subchannel index for the first resource location of each TRIV is separately indicated by inter-UE coordination information

| Field name | Field size (in bits) |
| --- | --- |
| Providing/requesting indicator | 1 |
| Resource combination(s) | 2*log2(N subchannel SLNsubchannel SL + 12Nsubchannel SL + 16)+9+Y Where N subchannel SL is provided by the higher layer parameter sl-NumSubchannel, Y=log2Nrsv_period with that Nrsv_period is the number of entries in the higher layer parameter sl-ResourceReservePeriodList, if higher layer parameter sl-MultiReserveResource is configured; Y=0 otherwise. |
| First resource location(s) | 8 |
| Reference slot location | 10+log2(10∙2μ) Where μ is 0, 1, 2, 3 for SCS of 15kHz, 30kHz, 60kHz, 120kHz, respectively. |
| Resource set type | 1 |
| Lowest subchannel indices for the first resource location of each TRIV | 2*log2NsubchannelSL where NsubchannelSL is provided by the higher layer parameter sl-NumSubchannel |
| (FFS) Actual number of resource combination | 1 Note: Support of this field is to be concluded by Feb 28. |

| Company | Yes or no | Comments |
| --- | --- | --- |
| Fraunhofer | No |  |
| Intel | Yes | We are open how to encode this information as a separate field or a codepoint with another field. Information seems needed at least for indication of non-preferred resource set in SCI format 2C |
| Apple | Yes | We think this field is needed to align with MAC CE. In general, we expect that MAC CE has a field of “Actual number of resource combination”. Also, this 1-bit field is not a big signaling overhead, and the upper bound of 140 bits is still satisfied with this field. |
| LGE | Yes | It would be helpful to have unified design of inter-UE coordination information regardless of a container, and it will be also helpful to reduce RAN2 workload considering very limited timeline. |
| Qualcomm | Comment | The field is needed in MAC-CE, otherwise, the MAC-CE size could be unnecessarily increased. If the same fields are to be used in both SCI-2C and the MAC-CE, which is not necessary in our view, then this field would also be included in SCI-2C. Our preference is to not include the field in SCI-2C and only include it in the MAC-CE. However, we can accept the field in SCI-2C is RAN1 determines that it is a pre-requisite to including it in the MAC-CE. |
| Samsung | No | This field is not essential. The same functionality can be done without adding this field. |
| NTT DOCOMO | No | The same resource can be indicated when actual number of resource comb. is one. In addition in this case also MAC can indicate only one resource comb. in the same way. We do not see issue on unified design between PHY and MAC. |
| Futurewei | Comments | Although we are not clear the benefit or usage of the bit, we are open to hearing how it will operate with and without this field (or a codepoint). |
| CATT, GOHIGH | Yes | We are open to use a code point of other field, or a separate field. |
| Huawei, HiSilicon | Yes | We assume the question may need some updates. E.g., a question as in red below could be given. The important thing here is RAN1 needs to clarify how UE-A indicates the actual number of resource combinations? If this is not clarified, the spec might be unclear and RAN1 may need CRs in the maintenance phase. During the discussions before, we assume there are 3 possible ways mentioned by companies: - Alt 1: introduce 1 bit field in SCI 2C (as shown in the current proposal) - Alt 2: use an unused codepoint of TRIV in SCI 2C to indicate this resource combination is not used as preferred/non-preferred resources - Alt 3: different resource combinations indicate the same set of resources. RAN1 needs to discuss and decide on one of the above Alternatives to avoid many CRs later. Among the above 3 alternatives, we think Alt 1 is more typical and straightforward. In addition, current size of SCI 2C for IUC is 122 bits. So there should be no problem to include this 1 bit field. == Question: On how UE-A indicates the actual number of resource combinations, which one of the following alternative do you support? - Alt 1: introduce 1 bit field in SCI 2C (as shown in the current proposal) - Alt 2: use an unused codepoint of TRIV in SCI 2C to indicate this resource combination is not used as preferred/non-preferred resources - Alt 3: different resource combinations indicate the same set of resources. |
| OPPO | Yes |  |
| Ericsson | No | We think that in order to include this functionality a bit is not needed. |
| Spreadtrum | Yes |  |
| Fujitsu | No |  |
| vivo | No | We do not see the motivation to support the field in format 2-C, since many UE implementations have been proposed by companies in GTW discussion. Whether MAC CE support such field or not is up to RAN2. We do not see the motivation to always align MAC CE field and format 2-C field |
| Sharp | No | Even for MAC CE this is not essential. There are a number of existing MAC CEs indicating “N” entries without the need to indicate the N. |
| xiaomi | No | SCI 2-C does’t need this information field like MAC CE, UE-B just need know the a set of resource indicated by resource combination by decode SCI 2-C. |
| Panasonic | Yes | At least this field is necessary in MAC-CE and same field is indicated by SCI-2C is preferable. |

Q4-8: According to RAN2 LS R1-2200880, RAN2 agreed that “Inter-UE coordination (IUC) issues (on which) RAN2 mainly relies on RAN1: Information and length of information of IUC MAC CE. The information indicated in RAN1 LS should be taken into account as baseline”. Also RAN1 already made the following agreements on the bit filed size for MAC CE of explicit request in this meeting. When considering these aspects, FL thinks that there would be no problem for RAN1 to make a recommendation on the bit field size for MAC CE of inter-UE coordination information with FFS point of “Details (e.g., how to put the recommended fields into MAC CE and the related field sizes in MAC CE) are up to RAN2". Do you agree following updated draft proposal?

- Agreement:

  - For Scheme 1, when both SCI format 2-C and MAC CE are used as the container of an explicit request for inter-UE coordination information, the same bit field size for the request in a SCI format 2-C is applied to MAC CE

- Agreement:

  - For Scheme 1, when MAC CE only is used as the container of an explicit request for inter-UE coordination information, the same bit field size for the request in a SCI format 2-C is applied to MAC CE

FL’s observation of 3rd email discussion (Bit field size of MAC CE for inter-UE coordination information when both MAC CE and a SCI format 2-C are used) - Support: DCM, LGE, Fujitsu, Spreadtrum, NEC, xiaomi, ETRI, Samsung, Ericsson, Futurewei, Qualcomm, InterDigital, (12) - Not support: OPPO, Apple, Nokia, (3) - Remove sub-bullet: OPPO, Nokia, (2) - Add a field to indicate the number of resource combinations: Apple, (1) - Comment: - The size for first resource location is log2(X): vivo, (1) - Leave MAC CE details up to RAN2: Huawei, Sharp, (2)

Updated Draft proposal 3-6:

- For Scheme 1, when both SCI format 2-C and MAC CE are used as the container of inter-UE coordination information, the same bit field size for inter-UE coordination information in a SCI format 2-C is applied to MAC CE except for first resource location(s) from RAN1’s perspective

  - Bit field size of the first resource location(s) on MAC CE is
log2(X) where X is provided by the (pre)configured maximum value of slot offset for the case when MAC CE only is used as a container of inter-UE coordination information

    - When both SCI format 2-C and MAC CE can be used as the container for inter-UE coordination information, UE does not expect that X is (pre)configured to be smaller than 255

  - Details (e.g., how to put these fields into MAC CE and the related field sizes in MAC CE) are up to RAN2

| Company | Yes or no | Comments |
| --- | --- | --- |
| Apple | No | The MAC CE may have the field of “actual number of combinations”, which may have a different size from SCI 2-C. We think this could be another exceptional case, besides the field of first resource location. - For Scheme 1, when both SCI format 2-C and MAC CE are used as the container of inter-UE coordination information, the same bit field size for inter-UE coordination information in a SCI format 2-C is applied to MAC CE except for first resource location(s) from RAN1’s perspective - Bit field size of the first resource location(s) on MAC CE is log2(X) where X is provided by the (pre)configured maximum value of slot offset for the case when MAC CE only is used as a container of inter-UE coordination information - When both SCI format 2-C and MAC CE can be used as the container for inter-UE coordination information, UE does not expect that X is (pre)configured to be smaller than 255 - Bit field size of “actual number of combinations” on MAC CE may be different from that on SCI format 2-C. - Details (e.g., how to put these fields into MAC CE and the related field sizes in MAC CE) are up to RAN2 |
| LGE | Yes | If we consider that container(s) can be different across (re)transmission(s) of inter-UE coordination information, sub-bullet part is essentially needed to ensure having the same TBS between initial transmission and retransmission. To be specific, if both SCI format 2-C and MAC CE are used for initial transmission of IUC, and if only MAC CE is used for retransmission of IUC, each bit field size of MAC CE should be the same to ensure the same TBS between them. Even though we support only the same container(s) are used for (re)transmission of inter-UE coordination information, it would be helpful to design MAC CE commonly regardless of whether a SCI format 2-C is used or not. |
| Qualcomm | Yes |  |
| Samsung |  | According to the last discussion, there was no consensus on this issue We are O.K to drop this proposal and to handle in RAN2. It is sufficient to provide RAN2 with the agreements RAN1 has made for the SCI Format 2-C fields and the size of each field. RAN2 can use this bit-size or modify them. |
| NTT DOCOMO | Yes |  |
| Futurewei | Yes | We support this proposal |
| CATT, GOHIGH | Yes |  |
| Huawei, HiSilicon | See comments | This issue should be closed already given the discussions during last GTW. For example, as already mentioned by some companies, the following agreement already said “…the same resource set is indicated in the 2nd SCI and the MAC CE …”, so RAN2 should be able to design the MAC-CE given RAN1’s agreements so far. RAN1 does not need to spend time designing MAC-CE, especially considering RAN1 has so many proposals to be discussed within the next few days. If companies really want to draw a conclusion here, we suggest the purples changes below: == Updated Draft proposal 3-6: - For Scheme 1, when both SCI format 2-C and MAC CE are used as the container of inter-UE coordination information, the same bit field size for inter-UE coordination information in a SCI format 2-C is applied to MAC CE except for first resource location(s) from RAN1’s perspective the same resource set is indicated in the SCI format 2C and the MAC CE as per RAN1’s previous agreement - Bit field size of the first resource location(s) on MAC CE is log2(X) where X is provided by the (pre)configured maximum value of slot offset for the case when MAC CE only is used as a container of inter-UE coordination information - When both SCI format 2-C and MAC CE can be used as the container for inter-UE coordination information, UE does not expect that X is (pre)configured to be smaller than 255 - Details (e.g., how to put these the fields of SCI format 2C into MAC CE and the related field sizes in MAC CE) are up to RAN2 == Agreement MAC CE or 2nd SCI are used as the container of inter-UE coordination information transmission from UE A to UE B. - For the indication of resource set, the following is supported: - N combinations of TRIV, FRIV, resource reservation period as specified in Rel-16 TS 38.214 Section 8.1.5 with following modification. The value of resource reservation period is omitted at least when the transmission of preferred resource set is triggered by UE-B’s explicit request. - First resource location of each TRIV is separately indicated by the inter-UE coordination information - If N <= 2, MAC CE is used and it is up to UE implementation to additionally use 2nd SCI. When 2nd SCI and MAC CE are both used, the same resource set is indicated in the 2nd SCI and the MAC CE. If N > 2, only MAC CE is used. - FFS: UE capability details - 2nd SCI is UE RX optional - The field size of the indication of resource set in a SCI format 2-C is determined by N=2 |
| Ericsson | Yes |  |
| Spreadtrum | Yes |  |
| Fujitsu | Yes |  |
| Vivo | See comment | As comment to Q4-7, “actual number of combinations” for SCI format 2-C and for MAC CE may be different. We do not need to force RAN2 to follow format 2-C design regarding “actual number of combinations”. |
| Sharp | No | Same view as Samsung. |
| Xiaomi | Yes |  |
| Panasonic | Yes |  |

Q4-9: FL understands that as per RAN1 agreement, only unicast can be used for transmission of inter-UE coordination information with preferred resource set even when it is triggered by a condition other than explicit request reception. Also as it was already agreed that inter-UE coordination information can be multiplexed with other data only if the source/destination ID pair is the same, it is clear that the cast type is aligned between inter-UE coordination information and other data when they are multiplexed. One remaining issue is how UE-A determines cast type for transmission of inter-UE coordination information with non-preferred resource set when it is not multiplexed with other data. Do you agree following updated draft conclusion?

FL’s observation of 3rd email discussion (Cast type of inter-UE coordination information transmission triggered by a condition other than explicit request reception) - Support: DCM, LGE, OPPO, Fujitsu, Spreadtrum, NEC, xiaomi, Fraunhofer, Huawei, Futurewei, Nokia, Qualcomm, Sharp, InterDigital, (14) - Not support: vivo, Intel, Samsung, (3)

Updated Draft conclusion 3-12:

- For transmission of inter-UE coordination information with non-preferred resource set triggered by a condition other than explicit request reception,

  - When it is not multiplexed with other data, UE-A determines its cast type by implementation

| Company | Yes or no | Comments |
| --- | --- | --- |
| Fraunhofer | Comments | This conclusion is different from what was proposed earlier. We do not see the need to add the additional information on whether it is multiplexed with other data or not in this proposal. We prefer the original wording of the proposal. |
| Intel | Comments | For condition-based standalone feedback transmission (w/o multiplexing with data) the IUC feedback should be available to all UEs. We do not support leaving it up to UE implementation. |
| Nokia, NSB | Yes |  |
| Apple | Yes |  |
| LGE | Yes | Since we already agreed to support unicast/groupcast/broadcast for inter-UE coordination information with non-preferred resource set triggered by a condition, UE-A choose any cast type by its implementation if the relevant information (e.g. source/destination IDs) are available. |
| Qualcomm | Yes | We prefer this to be broadcast but can accept this to be up to UE implementation for progress. |
| Samsung |  | We think that deciding cast type is not RAN1 scope. Even though our suggestion below has higher layer impact but we think this is the most desirable way to conclude this issue. So, we suggest the followings: - Groupcast/Broadcast for non-preferred resource set, - A Groupcast set for the transmission of condition-based RSAI information to can be (pre-)configured, if not (pre-)configured, the condition-based RSAI information is broadcast to surrounding UEs. - The period of the condition-based RSAI information is (pre-)configured to one of [{100, 500, 1000, 2000}] - Unicast: Only when UE-A has data send to UE-B, and the inter-UE co-ordination information is included in the same SL transmission with the data |
| NTT DOCOMO | Yes |  |
| Futurewei | Yes | We support this proposal. |
| CATT, GOHIGH | Yes |  |
| Huawei, HiSilicon | No | The proposal seems incomplete. What happens “when it is multiplexed with other data”? In general, RAN1 does not need to discuss whether/how UE-A multiplex data with IUC and the corresponding cast type, these are RAN2 issue. We can fully reply on RAN2 to discuss/decide such things. We suggest the following red changes (either is ok for us). Btw: we are also ok with no further discussion/conclusion here, since we assume RAN2 will anyway discuss/decide such things. == Updated Draft conclusion 3-12: - For transmission of inter-UE coordination information with non-preferred resource set triggered by a condition other than explicit request reception, - When it is not multiplexed with other data, UE-A determines its cast type by implementation == Updated Draft conclusion 3-12: - For transmission of inter-UE coordination information with non-preferred resource set triggered by a condition other than explicit request reception, - When it is not multiplexed with other data, how UE-A determines its cast type by implementation is up to RAN2 |
| OPPO | Yes |  |
| Spreadtrum | Yes |  |
| Fujitsu | Yes |  |
| Sharp | Yes |  |
| Xiaomi | Yes |  |
| Panasonic | Yes |  |

Q4-10: Company provides which alternative is supported for UE-B’s behavior when UE-B receives multiple resource sets from the same UE-A. FL would like to strongly encourage the proponents of Alt 1 to provide answers on concerns raised up by the proponents of Alt 2 (e.g., see Huawei's concern captured below). By doing so, we can efficiently make a decision on this issue in GTW session.

- Alt1, sub-bullet of Option 1:

  - Some companies mentioned the latest one is more accurate and is thus used. But this may not be true. Because it’s possible that a single IUC information (e.g., SCI 2C) cannot include all the preferred resources at UE-A side, so that UE-A may decide to transmit another IUC information to include another set, i.e., the set of preferred resources in different IUCs can be different or even orthogonal. In this case, use the latest one does not make sense.

  - Will RAN1 further consider an earliest and latest bound due to the newly introduced idea of “ … latest received …”, which will even have RRC impact

  - Will UE-B further consider the different priorities of different IUCs from the same UE-A? E.g., assume UE-A1 sends IUC with priority value 1 at slot n, and sends IUC with priority value 8 at slot n+50. The latter IUC is the latest one. However, the former IUC seems to be more important. Which one should UE-B consider?

- Alt 1, sub-bullet of 1st Option 3

  - If UE-B takes union of the non-preferred resources, the remaining resources in S_A could be very limited, causing RSRP increment and increasing interference. Thus, the performance could be even worse compared with Rel-16.

  - Some previously received non-preferred resource set may be no longer valid and thus should not be considered.

- Alt 1, sub-bullet of 2nd Option 3

  - If the same resource is marked as preferred in one IUC and marked as non-preferred in another IUC, what’s the UE-B’s behavior?

FL’s observation of 3rd email discussion (UE-B’s behavior when UE-B receives multiple resource sets from the same UE-A) - Alt 1: DCM, OPPO, Spreadtrum, NEC, xiaomi, Fraunhofer, Intel, Ericsson, Futurewei, Nokia, Qualcomm, (11) - Alt 2: LGE, Fujitsu, Spreadtrum, Huawei, Sharp, InterDigital, (6) - Others: vivo, Apple, Samsung, (3)

Draft proposal 3-13:

Alt 1:

- For UE-B’s behavior when UE-B receives multiple preferred resource sets from the same UE-A,

  - Option 1: UE-B uses the latest received preferred resource set from the same UE-A for its resource selection for a TB to be transmitted to the UE-A.

    - It is up to UE-B's implementation on how to determine the latest received preferred resource set

- For UE-B’s behavior when UE-B receives multiple non-preferred resource sets from the same UE-A,

  - Option 3: UE-B determines a final non-preferred resource set by making union of all the received non-preferred resource sets from the same UE-A. UE-B uses the final non-preferred resource set for its resource selection for a TB to be transmitted to the UE-A.

- For UE-B’s behavior when UE-B receives both a single preferred resource set and a single non-preferred resource set from the same UE-A,

  - Option 3: UE-B uses both the received preferred resource set and non-preferred resource set from the same UE-A for its resource selection for a TB to be transmitted to the UE-A.

Alt 2:

- When UE-B receives multiple inter-UE coordination information from the same UE-A, it is up to UE-B implementation to use one or multiple of them in its resource (re)selection.

| Company | Alt | Comments |
| --- | --- | --- |
| Fraunhofer | Alt 1 | Both UE-A and UE-B would need these rules defined for mutual understanding of the IUCs being transmitted/received. Hence we do not prefer Alt 2. |
| Intel | Alt.1 with comments | For Alt.1 Option 1 - Remove - It is up to UE-B's implementation on how to determine the latest received preferred resource set - Replace above with - RAN1 to finalize what “latest” mean at RAN1#108e For Alt.1 Option 3 - For UE-B’s behavior when UE-B receives multiple non-preferred resource sets from the same UE-A, - Option 3: UE-B determines a final non-preferred resource set by making union of all applicable the received non-preferred resource sets from the same UE-A. UE-B uses the final non-preferred resource set for its resource selection for a TB to be transmitted to the UE-A. - RAN1 to finalize meaning of “applicable” at RAN1#108e The following seems not needed given that exclusion is performed before selection (i.e., such resources are effectively treated as non-preferred) We can discuss whether such resources should be precluded by IUC feedback further - For UE-B’s behavior when UE-B receives both a single preferred resource set and a single non-preferred resource set from the same UE-A, - Option 3: UE-B uses both the received preferred resource set and non-preferred resource set from the same UE-A for its resource selection for a TB to be transmitted to the UE-A. |
| Nokia, NSB | Alt 1 |  |
| Apple | Mixed Alt 1 and Alt 2 | For first bullet (preferred resource set) with Option 1, and the third bullet (mixed resource set) with Option 3, we have another question: How does UE-B do if the TB is not sent to UE-A? It seems the solution is not complete. It seems the solution is incomplete. For second bullet (non-preferred resource set) with Option 3. We think it is needed. If UE-A detects the number of non-preferred resources more than the MAC CE container capacity, UE-A has to send these non-preferred resources in multiple transmissions. A simple way is to take the union of these resources at UE-B. Regarding HW’s comments - If UE-B takes union of the non-preferred resources, the remaining resources in S_A could be very limited, causing RSRP increment and increasing interference. Thus, the performance could be even worse compared with Rel-16. We think this is unavoidable if UE-A already detected so many non-preferred resources for UE-B. UE-B has to exclude them in its resource selection procedure. Otherwise, proposal 3-16 addresses the issue. Regarding HW’s comments - Some previously received non-preferred resource set may be no longer valid and thus should not be considered. We are fine to define the timeline of the gap between multiple IUC transmissions. For example, if the time gap between two IUCs are longer than X slots, then the previously received IUCs are not used. |
| LGE | Either Alt 2 or Alt 1 (without any change) | For progress, we can accept alt 1 without any changes if majority companies support it. Regarding the determination of the set of resources, as per agreements, they are all independent, and there is no associations between different sets of resources across different inter-UE coordination information transmissions. |
| Qualcomm | Alt 1 with corrections | For condition 1-B-1, in the case of non-preferred set of resource, UE-B should use the inter-UE coordination for a TB to be transmitted to any UE. We propose the following wording change - For UE-B’s behavior when UE-B receives multiple non-preferred resource sets from the same UE-A, - Option 3: UE-B determines a final non-preferred resource set by making union of all the received non-preferred resource sets from the same UE-A. UE-B uses the final non-preferred resource set for its resource selection for a TB to be transmitted to the UE-A. Without the above change, the working assumption agreed upon during RAN 106bis-e is violated: - For Condition 1-B-1 of Scheme 1, the following two options are supported - Option 1: Reserved resource(s) of other UE(s) identified by UE-A whose RSRP measurement is larger than a (pre)configured RSRP threshold which is determined by at least priority value indicated by SCI of the UE(s) - Option 2: Reserved resource(s) of other UE identified by UE-A whose RSRP measurement is smaller than a (pre)configured RSRP threshold which is determined by at least priority value indicated by SCI of the UE(s) when UE-A is a destination of a TB transmitted by the UE(s) Below, we provide responses to some of the raised concerns. Comments on Alt 1, sub-bullet of Option 1: - The case mentioned in the first sub-bullet is not supported by current agreement: if the number of resources in the preferred resource set is greater than a threshold, only MAC CE should be used for inter-UE coordination message. - If UE-A decides to indicate a preferred resource too far away in the future, it may happen that this resource is later reserved by another UE. Thus UE-A will need to update the preferred resource set in a new preferred resource set which excludes the concerned resource. - We do not understand the comment about the RRC impact. Latest mean that the inter-UE coordination message that is received most recently. We’re ok replacing “latest” with “most recent” if that addresses companies concerns. - For the comment on the difference in priority of the inter-UE coordination information. It is our understanding that all inter-UE coordination messages have the same priority from UE-B point of view. Differentiating inter-UE coordination message based on priority level has not been discussed in RAN 1 so far and we do not think it is beneficial to open this discussion at this late stage. Alt 1, sub-bullet of 1st Option 3: - We provided simulation results with this option implemented and observed performance gains from inter-UE coordination even for systems with high traffic load - The reason the resource is non-preferred is that it is reserved by another UE. Since there is no mechanism for a UE to un-reserve a reserved resource; a non-preferred resource will not stop being non-preferred at later time. Alt 1, sub-bullet of 2nd Option 3 When a resource is indicated at both preferred and non-preferred by UE-A, the UE-B will treat this resource as non-preferred. |
| Samsung | Neither | A simple solution is that UE-B uses the latest IUC information it receives from UE-A, whether it is preferred or non-preferred. It is not clear why UE-B would use any information rather than the latest. |
| NTT DOCOMO | Alt 1 | For the option 1, - we think “it’s possible that a single IUC information (e.g., SCI 2C) cannot include all the preferred resources at UE-A side, so that UE-A may decide to transmit another IUC information to include another set, i.e., the set of preferred resources in different IUCs can be different or even orthogonal.” is not valid. For more resources, MAC-CE should be used. The motivation of SCI 2C is lower latency, so separate IUCs in SCI 2C are not aligned with the motivation. - We do not understand why this has RRC impact. - We are also OK to consider priority perspective if majority want. But this should not be the reason to object Alt 1. For the 1st option 3, - If there are issues on available resource amount and old information, corresponding rule can be proposed. But this should not be the reason to object Alt 1. For the 2nd Option 3, - If there is an issue of overlap between preferred and non-preferred, corresponding rule can be proposed. But this should not be the reason to object Alt 1. In short, if UE-B is allowed to perform any for received IUCs, UE-A does not know the behavior; thus UE-A cannot decide which/when/whether IUC should be transmitted. Without clear answer for this perspective, we do not support Alt 2. |
| Futurewei | Alt 1 with comments | We support Alt 1 in general. For preferred set, it is possible that some available resources were later occupied other UEs. Even with priority value change, the latest would still be most reliable one. It is preferable to use the latest preferred resource set. For non-preferred set, it is highly probable that the resources occupied by others would be released later before the time passed. Therefore, it is technical sound to use union of the nonpreferred sets. For the case of both preferred and nonpreferred resource set, if the same resource is marked as preferred in one IUC and marked as non-preferred in another IUC (although we think it is rare if not impossible), following agreed UE-B’s behavior, UE-B will exclude it as it in the nonpreferred resource set and the S_A sent to MAC will not contain the resource. So the UE-B behavior in existing agreement works fine for this corner case. We however do not support the newly added subbullet in Alt 1 on UE-B’s implementation how to determine the latest received preferred resource. The latest can be defined the latest one received before UE-B performs resource selection. We propose the following updated on Alt 1 - For UE-B’s behavior when UE-B receives multiple preferred resource sets from the same UE-A, - Option 1: UE-B uses the latest received preferred resource set from the same UE-A for its resource selection for a TB to be transmitted to the UE-A. - It is up to UE-B's implementation on how to determine the latest received preferred resource set The latest received preferred resource set is latest one received before UE-B’s resource selection. - For UE-B’s behavior when UE-B receives multiple non-preferred resource sets from the same UE-A, - Option 3: UE-B determines a final non-preferred resource set by making union of all the received non-preferred resource sets from the same UE-A. UE-B uses the final non-preferred resource set for its resource selection for a TB to be transmitted to the UE-A. - For UE-B’s behavior when UE-B receives both a single preferred resource set and a single non-preferred resource set from the same UE-A, - Option 3: UE-B uses both the received preferred resource set and non-preferred resource set from the same UE-A for its resource selection for a TB to be transmitted to the UE-A. |
| CATT, GOHIGH | Alt 1 with comments | Since the IUC messages are from the same UE, we prefer to reuse the same principle for both preferred resource set and non-preferred resource set, i.e. the latest received IUC message. From our understanding, the latest received IUC is more important than others, since it reflect the most recent resource occupation status. - For UE-B’s behavior when UE-B receives multiple non-preferred resource sets from the same UE-A, - Option 3: UE-B determines a final non-preferred resource set by making union of all the received non-preferred resource sets from the same UE-A. UE-B uses the final non-preferred resource set for its resource selection for a TB to be transmitted to the UE-A. - UE-B uses the latest received preferred resource set from the same UE-A for its resource selection for a TB to be transmitted to the UE-A. |
| Huawei, HiSilicon | Alt2 | The baseline situation for all these cases is that if there is not consensus for another solution, it is left with no specified UE behavior, i.e. up to UE implementation (Alt 2). Our technical concern on the Options in Alt 1 are not addressed yet, despite multiple rounds of discussion. (Thanks to FL for highlighting them). If RAN1 does not have enough time to have very careful technical discussions on each of the options under each case, we expect there will be many CRs in maintenance phase to fix the issues we mentioned above or even more issues. For simplicity, we suggest to take a unified solution to handle all the cases, i.e., Alt2. In addition, if Alt 2 is taken, Q4-11 can also be quickly resolved using similar solution, which can save a lot of RAN1 time. |
| Ericson | Alt. 1 |  |
| Vivo | Alt.1 | For the 1st bullet, we are fine to use the latest or any received resource set. However, the resource set should match the UE-B’s TB transmission. E.g., resource set#1 is for TB#1, while a later set#2 is for TB#2. for TB#1 transmission, only set#1 can match the TB transmission requirement, which can be used for TB#1 transmission. - Option 1: UE-B uses the latest received preferred resource set from the same UE-A for its resource selection for a TB to be transmitted to the UE-A. - For request based IUC, the latest preferred resource set matching the requirement indicated by the request information should be used. - Option 1’: UE-B uses the latest any received preferred resource set from the same UE-A for its resource selection for a TB to be transmitted to the UE-A. - For request based IUC, the preferred resource set matching the requirement indicated by the request information should be used. Regarding the time bound to define the “latest received”, we think RAN2 has already perform the discussion on latency bound. We are fine to further discuss it in RAN1 … For 2nd bullet, we share with apple that the issue mentioned by Huawei has been discussed by proposal 3-16 For 3rd bullet, UE-B should have flexibility to use either of preferred or non-preferred resource set or both. Especially when UE-B does not performs sensing, UE-B only use the preferred resource set. - For UE-B’s behavior when UE-B receives both a single preferred resource set and a single non-preferred resource set from the same UE-A, - Option 3: it is up to UE-B’s implementation to use preferred resource set, non-preferred resource set, or UE-B uses both the received preferred resource set and non-preferred resource set from the same UE-A for its resource selection for a TB to be transmitted to the UE-A. |
| Panasonic | Alt 1 | When UE-A transmits multiple preferred resource set in a different time, the preferred resources are updated in UE-A. UE-B should use the latest received preferred resource set. |

Q4-11: Company provides which alternative is supported for UE-B’s behavior when UE-B receives multiple resource sets from the different UE-As. FL would like to strongly encourage the proponents of Alt 1 to provide answers on concerns raised up by the proponents of Alt 2 (e.g., see Huawei's concern captured below). By doing so, we can efficiently make a decision on this issue in GTW session.

- Alt1, sub-bullet of 1st Option 1:

  - Preferred resources sets from different UE-As may overlap. For example, maybe multiple UE-A indicate the same resource R1 as preferred resource and send it to UE-B. However, UE-B cannot transmit to multiple UE-As on the same resource R1. Then, how does Option 1 work in this case? Will UE-B consider different priorities of different UE-As? Or up to UE-B implementation?

- Alt 1, sub-bullet of 2nd Option 1:

  - As mentioned above, if UE-B takes union of the non-preferred resources, the remaining resources in S_A could be very limited, causing RSRP increment and increasing interference. Thus, the performance could be even worse compared with Rel-16.

  - If non-preferred resources of different UE-A are due to half-duplex. Then, UE-A1’s half-duplex slot has no relationship with UE-A2’s half-duplex slot. Why UE-B needs to take union of the non-preferred resources.

  - Some previously received non-preferred resource set may be no longer valid and thus should not be considered.

- Alt 1, sub-bullet of Option 2

  - For example, assume UE-A1 indicates R1 as non-preferred due to half-duplex and provides it to UE-B, UE-A2 provides preferred resource to UE-B. Then, when UE-B chooses resource to transmit to UE-A2, why UE-B needs to consider R1? R1 is UE-A1’s non-preferred resource due to half-duplex, and has no relationship with UE-A2.

FL’s observation of 3rd email discussion (UE-B’s behavior when UE-B receives multiple resource sets from the different UE-As) - Alt 1: DCM, Spreadtrum, NEC, xiaomi, Ericsson, Futurewei, Qualcomm, (7) - Alt 2: LGE, Fujitsu, Huawei, Sharp, InterDigital, (5) - Others: vivo, OPPO, Fraunhofer, Apple, Samsung, Nokia, (6) - Comments: - First discuss combinations of cast types for inter-UE coordination information transmissions: Intel, (1)

Draft proposal 3-14:

Alt 1:

- For UE-B’s behaviour when UE-B receives multiple preferred resource sets from the different UE-As,

  - Option 1: UE-B uses each received preferred resource set for its resource selection for each TB to be transmitted to each UE-A providing the preferred resource set.

- For UE-B’s behaviour when UE-B receives multiple non-preferred resource sets from the different UE-As.

  - Option 1: UE-B determines a final non-preferred resource set by making union of all the received non-preferred resource sets from different UE-As. UE-B uses the final non-preferred resource set for its resource selection for TB(s) to be transmitted to these different UE-As providing the non-preferred resource sets.

- For UE-B’s behaviour when UE-B receives both a single preferred resource set and a single non-preferred resource set from the different UE-As,

  - Option 2: UE-B uses both the received preferred resource set and non-preferred resource set from different UE-As for its resource selection for a TB to be transmitted to the UE-A providing the preferred resource set. UE-B uses the received non-preferred resource set for its resource selection for a TB to be transmitted to the UE-A providing the non-preferred resource set.

Alt 2:

- When UE-B receives multiple inter-UE coordination information from the different UE-As, it is up to UE-B implementation to use one or multiple of them in its resource (re)selection.

| Company | Alt | Comments |
| --- | --- | --- |
| Fraunhofer | Alt 1, with comments | For the second bullet, we do not agree. It is unclear why non-preferred resources from UE-A1 that is diagonally located to UE-A2 would be relevant for the selection of resources for a transmission by UE-B to both UE-As. Both UE-A and UE-B would need these rules defined for mutual understanding of the IUCs being transmitted/received. Hence, we do not prefer Alt 2. |
| Intel | Alt.1 | Comment #1: We can accept Alt.1 but the definition of the “received” needs to be clarified. The “received” may be a several seconds away. We assume there is no intention to use such feedback. Comment #2: Change “received” to “applicable received” Alt 1: - For UE-B’s behaviour when UE-B receives multiple preferred resource sets from the different UE-As, - Option 1: UE-B uses each applicable received preferred resource set for its resource selection for each TB to be transmitted to each UE-A providing the preferred resource set. - For UE-B’s behaviour when UE-B receives multiple non-preferred resource sets from the different UE-As. - Option 1: UE-B determines a final non-preferred resource set by making union of all the applicable received non-preferred resource sets from different UE-As. UE-B uses the final non-preferred resource set for its resource selection for TB(s) to be transmitted to these different UE-As providing the non-preferred resource sets. - For UE-B’s behaviour when UE-B receives both a single preferred resource set and a single non-preferred resource set from the different UE-As, - Option 2: UE-B uses both the applicable received preferred resource set and applicable non-preferred resource set from different UE-As for its resource selection for a TB to be transmitted to the UE-A providing the preferred resource set. UE-B uses the applicable received non-preferred resource set for its resource selection for a TB to be transmitted to the UE-A providing the non-preferred resource set. We cannot accept Alt.2 as it diminishes all RAN1 efforts to enable IUC framework and cannot be considered as way forward. |
| Nokia, NSB | Alt 1, with comments | Regarding the first bullet of Alt 1, Option 1 works for the case when UE-B has different unicast links with different UE-As. If UE-B requests multiple UE-As to provide IUC information for a single TB, then UE-B should use the intersection of the received multiple preferred resource sets from different UE-As. - For UE-B’s behaviour when UE-B receives multiple preferred resource sets from the different UE-As, - Option 1: UE-B uses each received preferred resource set for its resource selection for each TB to be transmitted to each UE-A providing the preferred resource set. - In case of groupcast to the multiple UE-As, UE-B selects resources from the intersection of the received preferred resource sets |
| Apple | Mixed Alt 1 and Alt 2 | For first bullet (preferred resource set) with Option 1, and the third bullet (mixed resource set) with Option 2, we have another question: How does UE-B do if the TB is not sent to UE-A? It seems the solution is not complete. It seems the solution is incomplete. For second bullet (non-preferred resource set) with Option 3. We think it is needed. If UE-A detects the number of non-preferred resources more than the MAC CE container capacity, UE-A has to send these non-preferred resources in multiple transmissions. A simple way is to take the union of these resources at UE-B. Regarding HW’s comments - As mentioned above, if UE-B takes union of the non-preferred resources, the remaining resources in S_A could be very limited, causing RSRP increment and increasing interference. Thus, the performance could be even worse compared with Rel-16. We think this is unavoidable if UE-As already detected so many non-preferred resources for UE-B. UE-B has to exclude them in its resource selection procedure. Otherwise, proposal 3-16 addresses the issue. Regarding HW’s comments - Some previously received non-preferred resource set may be no longer valid and thus should not be considered. We are fine to define the timeline of the gap between multiple IUC transmissions. For example, if the time gap between two IUCs are longer than X slots, then the previously received IUCs are not used. |
| LGE | Either Alt 2 or Alt 1 (without any change) | For progress, we can accept alt 1 without any changes if majority companies support it. As per agreements, only unicast is supported for inter-UE coordination information transmission with preferred resource set. In this case, UE-B only receives unicast source IDs of different UE-As. On the other hand, UE-B would not know which UE-A(s) are interested in the same groupcast destination ID(s). We prefer not to add description for groupcast case. |
| Qualcomm | Alt 1 with corrections | For condition 1-B-1, in the case of non-preferred set of resource, UE-B should use the inter-UE coordination for a TB to be transmitted to any UE. We propose the following wording change: - For UE-B’s behaviour when UE-B receives multiple non-preferred resource sets from the different UE-As. - Option 1: UE-B determines a final non-preferred resource set by making union of all the received non-preferred resource sets from different UE-As. UE-B uses the final non-preferred resource set for its resource selection for TB(s) to be transmitted to these different UE-As providing the non-preferred resource sets. - For UE-B’s behaviour when UE-B receives both a single preferred resource set and a single non-preferred resource set from the different UE-As, - Option 2: UE-B uses both the received preferred resource set and non-preferred resource set from different UE-As for its resource selection for a TB to be transmitted to the UE-A providing the preferred resource set. UE-B uses the received non-preferred resource set for its resource selection for a TB TB(s) to be transmitted to the UE-A providing the non-preferred resource set. Without the above change, the working assumption agreed upon during RAN 106bis-e is violated: - For Condition 1-B-1 of Scheme 1, the following two options are supported - Option 1: Reserved resource(s) of other UE(s) identified by UE-A whose RSRP measurement is larger than a (pre)configured RSRP threshold which is determined by at least priority value indicated by SCI of the UE(s) - Option 2: Reserved resource(s) of other UE identified by UE-A whose RSRP measurement is smaller than a (pre)configured RSRP threshold which is determined by at least priority value indicated by SCI of the UE(s) when UE-A is a destination of a TB transmitted by the UE(s) Below, we provide responses to some of the raised concerns. Alt1, sub-bullet of 1st Option 1 - This should be up to implementation. The same scenario happens in Release 16 where a UE needs to select resource to transmit multiple TB at the same time and it is up to UE implementation to determine the resource(s) to be selected for each of the TB-s such that the selected resources are not overlapping in time. Similar approach can be applied here. Alt1, sub-bullet of 2nd Option 1: - As noted in Q 4-10, we provided simulation results with this option implemented and observed performance gains from inter-UE coordination even for systems with high traffic load. - In our view in order to address half-duplex issue using condition 1-B-2 there has to be a mechanism to differentiate between non-preferred resource based on condition 1-B-1 and non-preferred resource based on condition 1-B-2. However, such discussion is separate from this proposal. - As noted in Q 4-10, the reason the resource is non-preferred is that it is reserved by another UE. Since there is no mechanism for a UE to un-reserve a reserved resource; there is no reason that a non-preferred resource will stop being non-preferred at later time. Alt 1, sub-bullet of Option 2 In our view in order to address half-duplex issue using condition 1-B-2 there has to be a mechanism to differentiate between non-preferred resource based on condition 1-B-1 and non-preferred resource based on condition 1-B-2. However, such discussion is separate from this proposal. |
| Samsung | Modified Alt1 | - For UE-B’s behaviour when UE-B receives multiple preferred resource sets from the different UE-As, - Option 1: UE-B uses each received preferred resource set for its resource selection for each TB to be transmitted to each UE-A providing the preferred resource set. - For UE-B’s behaviour when UE-B receives multiple non-preferred resource sets from the different UE-As. - Option 1: UE-B determines a final non-preferred resource set by making union of all the received non-preferred resource sets from different UE-As. UE-B uses the final non-preferred resource set for its resource selection for TB(s) to be transmitted to these different UE-As providing the non-preferred resource sets any UE. - For UE-B’s behaviour when UE-B receives both a single preferred resource set and a single non-preferred resource set from the different UE-As, - Option 2: UE-B uses both the received preferred resource set and non-preferred resource set from different UE-As for its resource selection for a TB to be transmitted to the UE-A providing the preferred resource set. In addition, UE-B uses the received non-preferred resource set for its resource selection for a TB to be transmitted to the UE-A providing the non-preferred resource set any UE. Alternatively, a simpler solution (which is our first preference), is: UE-B uses the latest IUC information it receives from each UE-A, whether it is preferred or non-preferred when transmitting to any UE. |
| NTT DOCOMO | Alt 1 | Similar comment for proposal 3-13. |
| Futurewei | Alt-1 with comment | For preferred resource set, we think there shouldn’t be an issue if two preferred resource sets are overlap with each other. If UE-B selects the resource for one UE, it cannot select the resource overlapped with the selected resource to another UE. For nonpreferred set, since the transmissions are all from the same UE-B, UE-B should select resource to avoid the non-preferred resources from all UE-A’s even the final S_A is small. If the final S_A is smaller than the threshold, we have the proposal later for case of one UE-A, which can be applied here too for the union of non-preferred set. Also for nonpreferred set, if the time already passed, the existing resource exclusion procedure can take care of this as the Rxy will not include these resources. For the nonpreferred resources resulting from half-duplex, we agree that union may not be efficient. We are open to do discuss this case. Since UE-B does not know which one is from half-duplex, it can only guess that the whole slot which is included in the non-preferred resource set is from half duplex. We are then ok to exclude the whole slot non-preferred resource set for each individual UE. This may have potential risk that the whole slot could be from condition 1-B-1. So we are fine with the Alt-1 from FL or with some update on the nonpreferred resource set as - For UE-B’s behaviour when UE-B receives multiple preferred resource sets from the different UE-As, - Option 1: UE-B uses each received preferred resource set for its resource selection for each TB to be transmitted to each UE-A providing the preferred resource set. - For UE-B’s behaviour when UE-B receives multiple non-preferred resource sets from the different UE-As. - Option 1: UE-B determines a final non-preferred resource set by making union of all the received non-preferred resource sets from different UE-As except the whole slot non-preferred resources. UE-B uses union of the final non-preferred resource set and each received nonpreferred resource set for its resource selection for TB(s) to be transmitted to these different each UE-As providing the non-preferred resource sets. - For UE-B’s behaviour when UE-B receives both a single preferred resource set and a single non-preferred resource set from the different UE-As, - Option 2: UE-B uses both the received preferred resource set and non-preferred resource set from different UE-As for its resource selection for a TB to be transmitted to the UE-A providing the preferred resource set. UE-B uses the received non-preferred resource set for its resource selection for a TB to be transmitted to the UE-A providing the non-preferred resource set. |
| CATT, GOHIGH | Alt 1 with comment | Regarding to the second bullet for non-preferred resource set. We think this is related to the cast type, if this is for unicast, no need to make union of all the received non-preferred resource sets from different UE-As. |
| Huawei, HiSilicon | Alt2 | Same view as for Q4-10. The baseline situation for all these cases is that if there is not consensus for another solution, it is left with no specified UE behavior, i.e. up to UE implementation (Alt 2). Our technical concern on the Options in Alt 1 are not addressed yet, despite multiple rounds of discussion. (Thanks to FL for highlighting them). If RAN1 does not have enough time to have very careful technical discussions on each of the options under each case, we expect there will be many CRs in maintenance phase to fix the issues we mentioned above or even more issues. For simplicity, we suggest to take a unified solution to handle all the cases, i.e., Alt2. In addition, if Alt 2 is taken, Q4-11 can also be quickly resolved using similar solution, which can save a lot of RAN1 time. |
| Ericssson | Alt. 1 |  |
| Vivo | Alt.1 | For 3rd bullet, UE-B should have flexibility to use either of preferred or non-preferred resource set or both. Especially when UE-B does not performs sensing, UE-B only use the preferred resource set. - For UE-B’s behaviour when UE-B receives both a single preferred resource set and a single non-preferred resource set from the different UE-As, Option 2: it is up to UE-B’s implementation to use preferred resource set, non-preferred resource set, or UE-B uses both the received preferred resource set and non-preferred resource set from different UE-As for its resource selection for a TB to be transmitted to the UE-A providing the preferred resource set. UE-B uses the received non-preferred resource set for its resource selection for a TB to be transmitted to the UE-A providing the non-preferred resource set. |
| Panasonic | Alt 1 |  |

Q4-12: Do you agree following updated draft conclusion on (pre)configuration of parameters related to n+T_1 and n+T_2 for determining the preferred resource set?

FL’s observation of 3rd email discussion ((pre)configuration of parameters related to n+T_1 and n+T_2 for determining the preferred resource set) - Support: DCM, ZTE, vivo, LGE, OPPO, Fujitsu, Spreadtrum, NEC, xiaomi, Fraunhofer, ETRI, Intel, Apple, Samsung, Futurewei, Nokia, Qualcomm, Sharp, InterDigital, (19) - Comments: - Remove “-T_1 for determining the set of preferred resources”: Huawei, Futurewei, (2) - Remove “Note that”: Intel, (1)

Updated Draft conclusion 3-1:

Not support (pre)configuration of parameters related to n+T_1 and n+T_2 for determining the set of preferred resources in inter-UE coordination information triggered by a condition other than explicit request reception.

- Note that T_2 is no smaller than T_2,min as specified in TS 38.214 section 8.1.4.

| Company | Yes or no | Comments |
| --- | --- | --- |
| Fraunhofer | Yes |  |
| Intel | Yes | We can accept if additional condition is added to the note - T1 < Tproc,1 |
| Apple | Yes |  |
| LGE | Yes | We are OK with Intel’s suggestion with minor chagne. It is also part of the existing spec. - T1 <= Tproc,1 |
| Qualcomm | Yes | We support he proposal |
| Samsung |  | OK for this but we do not need to spend time for this conclusion. Also, we think that the note above is not necessary. |
| NTT DOCOMO | Yes |  |
| Futurewei | Yes |  |
| CATT, GOHIGH | Yes |  |
| Huawei, HiSilicon | Yes | We are generally fine with this proposal. We suggest to remove the sub-bullet. Because we assume there is no specific reason to highlight T_2, T_2,min part. All the following restrictions are specified in Rel-16 (copied from TS 38.214): - 0 ≤ T1≤ Tproc,1SL - if T2min is shorter than the remaining packet delay budget (in slots) then T2 is up to UE implementation subject to T2min ≤ T2 ≤ remaining packet delay budget (in slots); otherwise T2 is set to the remaining packet delay budget (in slots). So RAN1 does not need to have the Note, otherwise it’ll be strange why other restrictions in Rel-16 are not mentioned. If RAN1 does not have a new agreement to change existing restrictions in current spec, that restriction will be followed by default. This is common understanding, no need to highlight this. == Updated Draft conclusion 3-1: Not support (pre)configuration of parameters related to n+T_1 and n+T_2 for determining the set of preferred resources in inter-UE coordination information triggered by a condition other than explicit request reception. - Note that T_2 is no smaller than T_2,min as specified in TS 38.214 section 8.1.4. |
| OPPO | yes |  |
| Ericsson | Yes |  |
| Spreadtrum | Yes |  |
| Fujitsu | Yes |  |
| vivo | Yes | We are fine to set a minimum length for selection window length by using T2_min. Note that T_1 should be determined as specified in TS 38.214 section 8.1.4 |
| Sharp | Yes |  |
| xiaomi | Yes |  |
| Panasonic | Yes |  |

Q4-13: Do you agree following updated draft conclusion for UE-A’s behavior of determining a priority value of inter-UE coordination information transmission triggered by a condition other than explicit request reception if the priority value is not (pre)configured?

FL’s observation of 3rd email discussion (UE-A’s behavior of determining a priority value of inter-UE coordination information transmission triggered by a condition other than explicit request reception if the priority value is not (pre)configured) - Support: vivo, LGE, OPPO, Fujitsu, Spreadtrum, NEC, xiaomi, ETRI, Huawei, Apple, Samsung, Ericsson, Qualcomm, Sharp, InterDigital, (15) - Not support: Intel, Futurewei, Nokia, (3) - Comments: - Remove sub-bullet: DCM, Fraunhofer, Intel, (3)

Updated Draft conclusion 3-11:

In RAN1, no further decision is necessary for UE-A’s behavior of determining a priority value of inter-UE coordination information transmission triggered by a condition other than explicit request reception if the priority value is not (pre)configured

- It is up to RAN2 whether/how to additionally handle this case (e.g., defining default priority value)

| Company | Yes or no | Comments |
| --- | --- | --- |
| Fraunhofer | Comment | We are fine with the main bullet, and prefer that the priority value be left up to UE implementation. |
| Intel | Yes | Prefer to remove sub-bullet |
| Nokia, NSB | No | A decision is necessary – and this aspect should be specified in the standard. If the prioritization is left up to UE-A implementation, this leaves the door open for UE-As to abuse their freedom by setting the highest priority to their IUC transmissions, and in so doing unfairly promote their own data transmissions when multiplexed with the IUC. Our preference is Option 5: The priority value is the same as the priority value indicated by other UE’s SCI that is used to determine the non-preferred resource set. |
| Apple | Yes | We are also fine with the removal of the sub-bullet. |
| LGE | Yes | If network does not want to have unexpected setting, network can (pre)configure its priority value. |
| Qualcomm | Yes |  |
| Samsung | Yes | BTW, do we need to have this conclusion? We think that we do not need to spend time for this conclusion. |
| NTT DOCOMO | No | Remove the sub-bullet. |
| Futurewei | Comment | We prefer to include the option with the priority value determined by UE implementation if the priority is not preconfigured. We do not support the subbullet which leave the discussion to RAN2. |
| CATT, GOHIGH | Yes |  |
| Huawei, HiSilicon | Yes | Although our 1st preference is leaving it to UE-A implementation, we can live with this for the sake of progress. |
| OPPO | yes |  |
| Ericsson | Yes |  |
| Spreadtrum | Yes |  |
| Fujitsu | Yes |  |
| Vivo | Yes |  |
| Sharp | Yes |  |
| xiaomi | Yes |  |
| Panasonic | Yes |  |

Q4-14: RAN1 already agreed that SCI format 2-C is UE RX optional. FL understands that in the case of GC/BC, it would be difficult for a UE transmitting inter-UE coordination information using SCI format 2-C to know whether or not target receiver(s) have the capability of SCI format 2-C reception. Note that in the Rel-17 UE feature discussion, the singling exchange of capability of SCI format 2-C reception between UEs was agreed, and RAN2 will work on the design of higher layer signaling (e.g., PC5 RRC) for this purpose in case of UC. Do you agree following updated draft conclusion for cast type of inter-UE coordination information when both a SCI format 2-C and MAC CE are used?

FL’s observation of 3rd email discussion (Cast type of inter-UE coordination information when both a SCI format 2-C and MAC CE are used) - Support: DCM, vivo, LGE, OPPO, Fujitsu, Spreadtrum, NEC, ETRI, Huawei, Intel, Samsung, Qualcomm, Sharp, InterDigital, (14) - Not support: Fraunhofer, Futurewei, Nokia, (3) - Comments: - Consider possibility of using a SCI format 2-C only: Samsung, (1)

Updated Draft proposal 3-8:

- For inter-UE coordination information transmission, a SCI format 2-C  can be used in addition to MAC CE only when its cast type is unicast regardless of whether or not it is multiplexed with other data

| Company | Yes or no | Comments |
| --- | --- | --- |
| Fraunhofer | No | As mentioned by Nokia and Futurewei in the previous rounds, if the destination ID can be a groupcast ID, we do not see the need to restrict the use of SCI 2-C to unicast alone, and can support groupcast. |
| Intel | Yes | For the sake of progress and simplicity |
| Nokia, NSB | No | Broadcast can be supported by setting Destination ID to a broadcast identity. Likewise, groupcast can be supported when HARQ feedback is not requested, by setting Destination ID to a groupcast ID. There may be cases where broadcasting SCI 2-C is beneficial given its lower latency. For example, if UE-A has some non-preferred resource(s) in the very near future due to Condition 1-B-1 Option 2, then, due to latency, SCI format 2-C may be the only option to make surrounding UEs aware of those non-preferred resource(s). If UE-A now broadcasts these non-preferred resource(s) using SCI format 2-C, of course some surrounding UEs won't be able to decode it - but that is a better outcome than not being able to inform any surrounding UEs except those with which UE-A currently has a unicast link. Moreover, if SCI 2-C is constrained to unicast, but UE-A has groupcast or broadcast data to transmit, UE-A won’t be able to multiplex/piggyback the SCI 2-C with the groupcast/broadcast data, which would be more efficient. |
| LGE | Yes | In our understanding, some similar discussion was made in Rel-16. To be specific, this issue was about L1 destination ID is sufficient to distinguish cast type. In RAN1 perspective, it was important to decide whether or not to introduce HARQ-ACK feedback option indication in 2nd SCI format. As we know, the result was to add cast type indicator. In this point of view, destination ID would not be sufficient to distinguish cast type in MAC layer, so, similar approach should be adopted. In short, we need to tie some cast type with a SCI format 2-C without cast type indicator. |
| Qualcomm | Yes |  |
| Samsung | No | It is not clear ‘in addition to MAC CE’. What is the UE behavior whether for UE to use MAC CE or 2nd SCI? When coordination message or coordination request is multiplexed with data, it would be beneficial to use the 2nd SCI for RSAI request rather than MAC CE in the latency aspect. Therefore, we propose: When a resource pool level configuration enables that MAC CE or 2nd SCI are used as the container for RSAI message, - If N<= 3 and RSAI is multiplexed with data, only 2nd SCI is used for container. - Otherwise, only MAC-CE is used for container. When a resource pool level configuration enables that MAC CE and 2nd SCI are used as the container for RSAI request, - If RSAI request is multiplexed with data, only 2nd SCI is used for container, Otherwise, only MAC-CE is used for container. |
| NTT DOCOMO | Yes |  |
| Futurewei | No | We do not see any necessities to restrict it to unicast only, while groupcast can be supported. |
| CATT, GOHIGH | Yes |  |
| Huawei, HiSilicon | Yes | We share similar view with FL. |
| OPPO | yes |  |
| Spreadtrum | Yes |  |
| Vivo | Yes |  |
| Sharp | Yes |  |
| Panasonic | Yes |  |

Q4-15: Do you agree following draft conclusion for additional criteria on which received preferred non-preferred resource set(s) are taken into account in UE-B’s resource selection?

FL’s observation of 3rd email discussion (Additional criteria on which received preferred non-preferred resource set(s) can be actually taken into account in UE-B’s resource selection) - Option 1: DCM, LGE, OPPO, Fujitsu, Spreadtrum, NEC, xiaomi, Fraunhofer, Huawei, Nokia, Qualcomm, Sharp, InterDigital, (13) - Option 2: Intel, Apple, (2) - Option 3: - Consider distance between UE-A and UE-B: Ericsson, (1) - Option 2 with a specified time gap instead of (pre)configured value: Futurewei, (1)

Draft conclusion 3-15:

RAN1 does not pursue defining additional criteria on filtering the received preferred or non-preferred resource set(s) to be taken into account in UE-B’s resource selection

| Company | Yes or no | Comments |
| --- | --- | --- |
| Fraunhofer | Yes, with comment | While we understand the benefit of considering the distance between UE-A and UE-B (as suggested by Ericsson), at this point in time, we prefer to go with the FL’s proposal. |
| Intel | No | In this case it is unclear whether/how to handle feedback from different UEs and different types of feedback, including feedback generated with different parameters comparing to the ones used by TX UE |
| Apple | Comments | Maybe we do not need any conclusion at this moment. If in maintenance phase, companies think some additional criteria is necessary, we could still discuss them. |
| LGE | Yes |  |
| Qualcomm | Yes |  |
| Samsung |  | We think that we do not need to spend time for this conclusion. |
| NTT DOCOMO | Yes |  |
| Futurewei | Yes | We can accept the proposal although we feel a certain timing gap is needed. |
| CATT, GOHIGH | Comment | At least, how to identify the which UE is the effect UE-A should be discussed. |
| Huawei, HiSilicon | See comments | So far, RAN1 has not agreed any filtering mechanism, right? So “additional” should be removed. The term “filtering” is not defined yet, the meaning might be unclear and cause confusion. A more accurate term is appreciated. == Draft conclusion 3-15: RAN1 does not pursue defining additional criteria on filtering the received preferred or non-preferred resource set(s) to be taken into account in UE-B’s resource selection |
| OPPO | yes |  |
| Ericsson | No | We think that some sort of filtering is needed in order to avoid having too much unnecessary signalling. |
| Spreadtrum | Yes |  |
| Fujitsu | Yes |  |
| Vivo | Comment | We tend to agree with Intel |
| Sharp | Yes |  |
| xiaomi | Yes |  |
| Panasonic | Yes |  |

Q4-16: Do you agree following updated draft conclusion for sensing window for determining the set of resources? Note that the updated parts of Alt1 are marked with red.

FL’s observation of 3rd email discussion (Sensing window for determining the set of resources) - Alt 1: DCM, vivo, LGE, OPPO, Fujitsu, Spreadtrum, NEC, xiaomi, Fraunhofer, ETRI, Huawei, Apple, Samsung, Ericsson, Nokia, Qualcomm, Sharp, InterDigital, (18) - Alt 2: LGE, Intel, Futurewei, (3) - Comments: - Add “Re-evaluation for the set of resources is supported as per Rel-16 procedure”: Huawei, (1) - Add “with n>= n’, where n’ is the slot in which inter-UE coordination information generation is triggered” for condition-based IUC: Qualcomm,

Updated Draft conclusion 3-10:

For sensing window for determining the set of resources in Scheme 1,

- No further change is supported. Note that the sensing window for determining the set of resources is already derived based on the location n+T_1 and n+T_2 used for determining the set of resources in TS38.214 section 8.1.4, i.e., sensing window is defined by the range of slots [ (n+T_1) - T_0 - T_1 determined by UE-A, (n+T_1) - T_proc,0 - T_1 determined by UE-A ).

  - For inter-UE coordination information triggered by UE-B’s explicit request, n+T_1 and n+T_2 are provided by the request.

  - For inter-UE coordination information triggered by a condition other than explicit request reception, n+T_1 and n+T_2 are determined by UE-A’s implementation. With n>= n’, where n’ is the slot in which inter-UE coordination information generation is triggered.

- Re-evaluation for the set of resources is supported as per Rel-16 procedures.

| Company | Yes or no | Comments |
| --- | --- | --- |
| Fraunhofer | Yes |  |
| Intel | No | Proposed revision: - For inter-UE coordination information triggered by UE-B’s explicit request, n+T_1 and n+T_2 are provided by the request. - If inter-UE coordination information is triggered by an explicit request, - (n+T_1) ≤ (n'+T’_1) - (n'+T’_2) ≤ (n+T_2) where - (n+T_1) – Start slot of resource selection window for determining the set of resources - (n+T_2) – End slot of resource selection window for determining the set of resources - (n’+T’_1) – Start slot of resource selection window used for inter-UE coordination information transmission - (n’+T’_2) – End slot of resource selection window used for inter-UE coordination information transmission |
| Apple | Comments | We have a clarification question on the proposal. If UE-A have the resource selection window of (n+T1, n+T2) as in explicit request, and its sensing window is defined by (n+T1-T_0-T_{proc,1}, n+T1-T_{proc,0}-T_{proc,1}), then UE-A determines the contents of IUC at slot n. Then what is UE-A’s resource selection window for IUC transmission? We think it has to be later than slot n, but earlier than the first indicated resources in IUC (since otherwise, the IUC information is outdated). Do we need to agree on the restriction of the resource selection window (or PDB) for IUC transmission? |
| LGE | Yes with comment | It seems necessary to clarify the meaning of the last sub-bullet. As we know, re-evaluation is performed before the UE transmit the selected resource(s). Now, since it is about determining the set of resources, there are two possibilities. One is that UE-A can perform re-evaluation for the set of resources until the earliest timing of the set of resources. The other is that UE-A can perform re-evaluation for the set of resources until its inter-UE coordination information transmission. |
| Qualcomm | Yes, with comments | Regarding the last bullet, we think that the meaning is a bit unclear. In Rel-16 a mandatory re-evaluation is performed at least in slot m-T_3 where m is the slot of TB transmission. It is up to UE to perform any extra reevaluation before m-T_3. We think the same principle can be applied here. We propose the following wording: - Re-evaluation for the set of resources is supported as per Rel-16 procedures. - UE-A is required to perform at least one mandatory reevaluation at slot n’-T_3. It is up to UE-A to perform any extra reevaluation before slot n’-T_3. |
| Samsung |  | BTW, do we need to have this conclusion? We think that we do not need to spend time for this conclusion. |
| NTT DOCOMO | Yes | Tend to agree with LGE. The last bullet is ambiguous. |
| Futurewei | No | For IUC triggered by explicit request, UE-B does not wait after n+T1-Tproc,1 to perform resource selection as we have agreed that UE-A may not transmit coordination information per the following agreement. Agreement For inter-UE coordination triggered by an explicit request in Scheme 1, whether or not to transmit the inter-UE coordination information upon the request reception is determined by UE-A’s implementation subject to the following procedures. - Rel-16 procedure of UL/SL prioritization, LTE SL/NR SL prioritization, and congestion control Therefore, we think the sensing ending time should be specified consider the resource selection (RSW) at UE-A and UE-B’s resource selection time and behaviour. For explicit request IUC, the deadline for UE-B receiving coordination information is (n+T1)-Tproc,1-1, where 1 slot included is for transmission propagation and decoding of coordination information. Since UE-A needs to select the resource for coordination, therefore additional timing gap Tr needed. So sensing should be ended by (n+T1)-Tproc,1-1 -Tr-Tproc,0. We can set Tr = T2min or larger which includes T1 timing per R16 definition. Therefore, we propose change sensing ending time as (n+T_1) - T_proc,0 – T_proc,1-Tr-1, where Tr can be T2min or larger than T2min. The sensing window also applies to the IUC triggered by condition where n+T_1 is determined by UE-A’s implementation. |
| CATT, GOHIGH | Yes with comments | We are generally fine with the proposal, but think some clarification is necessary. First, the sensing window range “(n+T_1) - T_0 - T_1 determined by UE-A, (n+T_1) - T_proc,0 - T_1” need to further clarify. The T_1 in “n+T_1” should be different from T_1 determined by UE-A. we suggest to change n+T_1 to n1, and n+T_2 to n2. Second, we are not fully convinced the new added last bullet, it is unclear what’s the checking timing of re-evaluation, does it need to update the IUC message after re-evaluation? If yes, it is different from that of Rel-16, which doesn’t to reconstruct the TB. |
| Huawei, HiSilicon | See comments | On the last sub-bullet of re-evaluation: we support it. Same behavior as per Rel-16 procedures is followed. Btw: the last sub-bullet refers to “UE-A performs re-evaluation for the set of resources before IUC transmitting” (same as Rel-16 re-evaluation). Because after IUC is transmitted, the contents of MAC-CE cannot be changed due to HARQ combining, so there is no need to update the set of resources. For “With n>= n’, where n’ is the slot in which inter-UE coordination information generation is triggered” part: we are unclear why this is needed. In this case, inter-UE coordination information generation is triggered by UE-A’s implementation (i.e., n’ is determined by UE-A implementation), and n+T_1 and n+T_2 are also determined by UE-A’s implementation. Therefore, the newly added red part is already covered by the sentence before it, so it’s not necessary. == Updated Draft conclusion 3-10: For sensing window for determining the set of resources in Scheme 1, - No further change is supported. Note that the sensing window for determining the set of resources is already derived based on the location n+T_1 and n+T_2 used for determining the set of resources in TS38.214 section 8.1.4, i.e., sensing window is defined by the range of slots [ (n+T_1) - T_0 - T_1 determined by UE-A, (n+T_1) - T_proc,0 - T_1 determined by UE-A ). - For inter-UE coordination information triggered by UE-B’s explicit request, n+T_1 and n+T_2 are provided by the request. - For inter-UE coordination information triggered by a condition other than explicit request reception, n+T_1 and n+T_2 are determined by UE-A’s implementation. With n>= n’, where n’ is the slot in which inter-UE coordination information generation is triggered. - Re-evaluation for the set of resources is supported as per Rel-16 procedures. |
| OPPO | Yes with comments | For the last bullet, we support the intention behind, but re-evaluation defined in Rel-16 is for the resources selected for transmission, not for the contents to be transmitted. How about the following: - It is up to UE-A to further update Re-evaluation for the set of resources before it is transmitted is supported as per Rel-16 procedures. |
| Ericssson | Comment | We need more clarification in the last bullet. Is the intention that a UE will perform re-evaluation/pre-emption of the resources included in the IUC message? |
| Spreadtrum | Yes with comment | The mechanism of the last sub-bullet needs to be clarified. It is not clear which UE will perform re-evaluation. If UE-A, is it means that UE-A will transmit a new resource set to UE-B for supporting this mechanism? If UE-B, how to guarantee it can support sensing procedure? |
| Fujitsu | Yes |  |
| Vivo | No for the red colored statement | We think slot n for preferred resource selection should be located after IUC transmission slot, thus re-evaluation for the preferred resource is not needed, since IUC has informed the preferred resource already. With n>= n’, where n’ is the slot in which inter-UE coordination information generation is triggered. With n> n’, where n’ is the slot where IUC is transmitted. Re-evaluation for the set of resources is supported as per Rel-16 procedures |
| Sharp | Yes |  |
| xiaomi | comment | For the last bullet, we think the re-evaluation is performed at UE-B other than UE-A, this need be clarified. |
| Panasonic | Yes |  |

Q4-17: Do you agree following draft conclusion for additional UE-B behavior to handle the case when it is not possible that the number of candidate single-slot resources after applying the received non-preferred resource set as per the existing agreement meets the requirement of X*M_total?

FL’s observation of 3rd email discussion (Additional UE-B behavior to handle the case when it is not possible that the number of candidate single-slot resources after applying the received non-preferred resource set as per the existing agreement meets the requirement of X*M_total) - Option 1: DCM, vivo, LGE, OPPO, Fujitsu, Spreadtrum, NEC, xiaomi, Qualcomm, Sharp, InterDigital, (11) - Option 2: - UE-B does not use the received non-preferred resource sets in its resource selection: DCM, OPPO, Intel, (3) - UE-B does not used a subset of the received non-preferred resource sets in its resource selection until the requirement is met: Fujitsu, (1) - Increasing RSRP threshold: Fraunhofer, (1) - Different preference levels are indicated for non-preferred resources: Samsung, (1) - It is up to UE-B’s implementation on how to satisfy the requirement of X*M_total but UE-B should at least apply the whole slot(s) that is appeared in non-preferred resource set: Futurewei, (1) - Allowing partial overlapping with non-preferred resources: Nokia, (1) - Comments: - Discuss draft proposal 3-20, 3-21 first: Huawei, (1)

Draft proposal 3-16:

For the case when it is not possible that the number of candidate single-slot resources after applying the received non-preferred resource set as per the existing agreement meets the requirement of X*M_total,

- It is up to UE-B’s implementation how to meet this requirement

| Company | Yes or no | Comments |
| --- | --- | --- |
| Fraunhofer | Comment | We do not think UE-B discarding the non-preferred resource set is ideal because UE-B would be then including resources where collisions are possible to its candidate resource set. We are fine with UE-B increasing the threshold and repeating the process, as described in the current specifications (step 7). If the group cannot converge, we can accept the proposal as a compromise. |
| Intel | NO | As a simple solution we propose to fallback to TX candidate resource set |
| Nokia, NSB | No | We don’t think this issue should be left up to UE-B implementation. The infinite loop issue needs to be addressed by the specification. We suggest to relax UE-B’s resource exclusion by increasing the allowed overlap in Step 7. When evaluating whether or not to exclude a candidate single-slot resource that overlaps with non-preferred resource(s), the allowed overlap (initially 0%) may be successively increased (e.g., first to 10%, then to 20%, and so on) in Step 7. A candidate single-slot resource is only excluded if its overlap with non-preferred resource(s) is greater than the allowed overap. In this way, the infinite loop issue can be resolved. 7) If the number of candidate single-slot resources remaining in the set SA is smaller than X ⋅ Mtotal, then Th(pi , pj) is increased by 3 dB for each priority value Th(pi , pj) and allowedOverlapNonPreferredResources is increased by [Y] percentage points [alternatively, increased by Z subchannels] and the procedure continues with step 4. |
| LGE | Yes | We are also fine with fallbacking to S_A determined by UE-B’s sensing results. |
| Qualcomm | Yes |  |
| Samsung | No | Different preference levels are indicated for the non-preferred resources. If there isn’t enough candidate resources, the levels corresponding to the least of the non-preferred resources is not excluded. |
| NTT DOCOMO | OK | Or we are fine with some rule. |
| Futurewei | Yes with Comment | We think it is similar to the infinite loop scenario, so we may clarify the case which happens after repeating the process with RSRP increasing. Second although it is up to UE-B’s implementation, we may need to clarify that it is allowed that some non-preferred resources are not excluded. The updated proposal is For the case when it is not possible that the number of candidate single-slot resources after applying the received non-preferred resource set as per the existing agreement meets the requirement of X*M_total with the repeating processed with RSRP increasing, - It is up to UE-B’s implementation how to meet this requirement. It is allowed that some resources in nonpreferred resource set are not excluded. |
| CATT, GOHIGH | Yes | We can accept this proposal if group can not converge. |
| Huawei, HiSilicon | No | The sub-bullet is too broad, it means UE-B can do whatever it wants, e.g., UE-B may totally skip Mode 2 sensing procedure, etc. To be more accurate, we suggest the following red changes. == Draft proposal 3-16: For the case when it is not possible that the number of candidate single-slot resources after applying the received non-preferred resource set as per the existing agreement meets the requirement of X*M_total, It is up to UE-B’s implementation how to meet this requirement whether or not to use the received non-preferred resource set |
| OPPO | yes | We are also fine with “UE-B does not use the received non-preferred resource sets in its resource selection” |
| Ericsson | No | The Tx UE can use its own resource set if there are no enough resources. |
| Spreadtrum | Yes |  |
| Vivo | No | Clarify which implementation is allowed, e.g., UE can give up the non-preferred resources, or selects part of the non-preferred resource, to meet the requirement. - It is up to UE-B’s implementation to use none or part of the non-preferred resource(s) to meet the requirement of X*M_total, when applying all the non-preferred resource(s) cannot meet the requirement of X*M_total. |
| xiaomi | comment | We prefer to support that UE-B does not use the received non-preferred resource sets in its resource selection，but we can accept the FL’s proposal. |
| Panasonic | Yes |  |

Q4-18: Do you agree following draft conclusion for condition when Option B can be used for preferred resource set?

FL’s observation of 3rd email discussion (Condition when Option B can be used for preferred resource set) - Option 1: DCM, vivo, OPPO, Fujitsu, Spreadtrum, NEC, xiaomi, Franhofer, Huawei, Intel, Samsung, Ericsson, Futurewei, (13) - Option 2: DCM, vivo, LGE, OPPO, NEC, xiaomi, Franhofer, Huawei, Apple, Futurewei, InterDigital, (11) - Option 3: vivo, OPPO, NEC, xiaomi, Franhofer, Huawei, Apple, Futurewei, Qualcomm, InterDigital, (10) - Option 4: - UE-B has a capability of performing sensing/resource exclusion, but UE-B is (pre)-configured not to perform sensing/resource exclusion in SL DRX : InterDigital, (1)

Draft proposal 3-17:

For Scheme 1, Option B can be used for preferred resource set when one of following condition is met:

- Option 1: UE-B does not have a capability of performing sensing/resource exclusion.

- Option 2: UE-B performs random resource selection.

- Option 3: UE-B has a capability of performing sensing/resource exclusion, but UE-B determines not to perform sensing/resource exclusion by its implementation.

| Company | Yes or no | Comments |
| --- | --- | --- |
| Fraunhofer | Yes | All the 3 cases should be considered in the case where UE-B does not have its own sensing results. For Option 1 and 2, the UE should be capable of receiving the IUCs. |
| Intel | No | We support Option 1 only. Other options require more discussion and details if there is an intention to support it. |
| Apple | Yes |  |
| LGE | No | Currently, we support Option 1 and Option 2. When Rel-16 UE and Rel-17 UE coexist in the same resource pool, Option 3 will cause performance degradation on Rel-16 UEs. |
| Qualcomm | Comment | Option 3, in our view, is the most important use case for power saving. As we understand the proposal, all three options will be supported in Rel-17. In which case, we support the proposal. |
| Samsung | Comment | Only Option 1 |
| NTT DOCOMO | No | Issue here is whether Option 3 (and/or Option 2) is allowed or not. Now this proposal means that Option 3 is allowed; we do not support this proposal. As commented, UE in Option 3 ignores its surrounding UEs. Collision will increase. |
| Futurewei | Yes | We support all three options. |
| CATT, GOHIGH | Yes with comments | From our understanding, the three options are all possible if UE-B is capable of NR-SL reception. If this is the situation, it would be better to add a note. Note: the pre-requisite of UE-B is capable of NR-SL reception. |
| Huawei, HiSilicon | Yes with comments | We support all three options. Option 3 has the benefit of UE-B power saving, and also has the benefits of reduced interference when one UE-A can help multiple UE-Bs (e.g., RSU scenario). We suggest to add “at least” as below, because some Options might be satisfied simultaneously, e.g., Option 1 and 2, etc. == Draft proposal 3-17: For Scheme 1, Option B can be used for preferred resource set when at least one of following condition is met: - Option 1: UE-B does not have a capability of performing sensing/resource exclusion. - Option 2: UE-B performs random resource selection. - Option 3: UE-B has a capability of performing sensing/resource exclusion, but UE-B determines not to perform sensing/resource exclusion by its implementation. |
| OPPO | yes |  |
| Ericsson | No | Only Option 1. |
| Spreadtrum | Yes | At least Option 1 should be supported. We are fine will Option 2 and Option 3. |
| vivo | Comment | All the options are feasible. |
| Sharp |  | Only Option 1 |
| Panasonic | Yes |  |

Q4-19: Do you agree following draft conclusion for latency bound of inter-UE coordination information?

FL’s observation of 3rd email discussion (Latency bound of inter-UE coordination information) - Support: DCM, LGE, OPPO, Huawei, Samsung, Ericsson, Nokia, Qualcomm, InterDigital, (9) - Not support: xiaomi, Intel, Futurewei, (3)

Draft conclusion 3-9:

- For latency bound of inter-UE coordination information transmission, RAN1 relies on RAN2’s decision as per LS R1-2200880 from RAN2

| Company | Yes or no | Comments |
| --- | --- | --- |
| Intel |  | No need for conclusion. RAN2 was not responsible for design of IUC and resource allocation. Offloading discussion to RAN2 may delay progress as there are many details |
| LGE | Yes | It would be helpful to clarify RAN1’s plan or remaining issues. |
| Qualcomm | Yes |  |
| Samsung | Yes |  |
| NTT DOCOMO | Yes |  |
| Futurewei | No | We do not think it is appropriate for RAN2 to define the bound as a lot of parameters are PHY parameters, e.g., n+T1, n+T2 signalled in PHY and most timing requirements are specified in RAN1, e.g., Tproc,0, Tproc,1 etc. We prefer RAN1 to discuss the latency bound as RAN1. |
| CATT, GOHIGH |  | No need for conclusion. From our understanding, we think the impacts on resource allocation due to latency bound of inter-UE coordination should be discussed in RAN1. |
| Huawei, HiSilicon | Yes | As shown in RAN2’s summary R2-2203159 (see “Issue 4. Timer to handle latency bound for inter-UE coordination”), RAN2 already had quite in-depth discussions on the latency bound issue and will continue discussing it. For example, as shown in R2-2203159, RAN2 raised 8 questions and come up with 15 proposals (see Proposal 4-x in R2-2203159)! RAN1 should avoid such duplicated discussions to save time. |
| OPPO | yes |  |
| Ericsson | Yes |  |
| Fujitsu | Yes |  |
| Vivo | No | Even when RAN2 does not support, RAN1 can further discuss to address RAN1 issues |
| xiaomi | no | From our understanding, RAN2 would only define the timer for the delay bound. Issues on how the bound would have impact on resource (re)selection and how the bound is determined can only be discussed in RAN1. Meanwhile, the RAN2 only discuss the timer for request-based inter-UE coordination, but the timer for condition-based inter-UE coordination also need be discussed, so the latency bound need be discussed in RAN 1. |
| Panasonic | Yes |  |

Q4-20: Do you agree following draft conclusion on 2nd SCI format(s) than can be used to schedule retransmission of a TB containing inter-UE coordination information initially scheduled by a SCI format 2-C?

FL’s observation of 3rd email discussion (2nd SCI format(s) than can be used to schedule retransmission of a TB containing inter-UE coordination information initially scheduled by a SCI format 2-C) - Support: DCM, vivo, LGE, OPPO, Fujitsu, Spreadtrum, NEC, xiaomi, Fraunhofer, Nokia, (10) - Not support: Huawei, Intel, Apple, Samsung, Ericsson, Futurewei, Qualcomm, Sharp, (8) - Use only SCI format 2-C for the retransmission: Huawei, Intel, Apple, Ericsson, Qualcomm, Sharp, (6) - Not use a SCI format 2-C for the retransmission: Samsung, (1) - Not support retransmission of inter-UE coordination information: Futurewei, (1)

Draft conclusion 3-20:

- No consensus on any restriction on a 2nd SCI format that can be used for retransmission of inter-UE coordination information MAC CE initially scheduled by a SCI format 2-C.

| Company | Yes or no | Comments |
| --- | --- | --- |
| Fraunhofer | Comment | While we are fine with using any 2nd SCI format for retransmissions, we are open to using only SCI format 2-C. If the group cannot converge, we can accept the proposal as a compromise. |
| Intel |  | Prefer to understand what it means / Feature is supported? |
| Nokia, NSB | Yes |  |
| Apple | Yes | We can accept the proposal as a compromise. |
| LGE | Yes |  |
| Qualcomm | No | While the current status is that there’s no consensus, the issue of maintaining the same TB sizes if different SCI-2 formats are used for retransmission remains. At the very least, the following additional restriction is needed: The calculated TB size cannot be changed between retransmissions. |
| Samsung | Comment | While it is true that there is currently no consensus, we would like to point out that re-transmission of SCI Format 2-C with the same information is redundant and degrades air interface capacity when the UE-A receives a NACK. A NACK reception is an indication that pervious SCI Format 2-C has been successfully received. |
| NTT DOCOMO | Yes |  |
| Futurewei | No | We are not clear on the meaning or outcome of the conclusion, whether any 2nd SCI format is allowed or it is restricted to be SCI 2-C. We prefer the same SCI 2-C is used for retransmission. |
| CATT, GOHIGH | No | We are not fully understanding this proposal. Since when 2nd SCI format 2C is used, it is only for unicast. But SCI format 2B is only used for groupcast. We think it is not aligned between the cast type. |
| Huawei, HiSilicon | See comment | We assume it is common understanding that at least SCI 2-C can be used for re-retransmission if initially scheduled by a SCI format 2-C. - If companies have different view on this aspect (i.e., SCI 2C cannot be used in this case), we would be surprised and would like to know why. So the issue here is if 2nd SCI formats other than SCI 2C can be used for re-transmission in this case. To our understanding, only SCI 2C can be used in this case. Because as we pointed out previously, different 2nd SCI format have different size, thus occupy different number of REs. As per current spec, this will further impact TBS determination, resulting in different TBS for initial and retransmission(s) of the same TB and causing HARQ combining infeasible. This technical comment still stands since we do not see any different understandings here. So we suggest the following red changes. == Draft conclusion 3-20: - No consensus on any restriction on a 2nd SCI format other than SCI format 2-C that can be used for retransmission of inter-UE coordination information MAC CE initially scheduled by a SCI format 2-C. |
| OPPO | yes |  |
| Ericsson | No | We think that only SCI format 2-C can be used to re-transmit the inter-UE coordination information. |
| Spreadtrum | Yes |  |
| Fujitsu | Yes |  |
| Vivo | Yes |  |
| Sharp | No | Same view as QC. |
| xiaomi | Yes |  |
| Panasonic | Yes |  |

Q4-21: Do you agree following draft conclusion for restriction(s) to the inter-UE coordination mechanism to address the power saving operation?

FL’s observation of 3rd email discussion (Restrictions to the IUC mechanism to address the power saving operation) - Support: Intel, Ericsson, (2) - Not support: vivo, LGE, OPPO, Fujitsu, Spreadtrum, NEC, xiaomi, Fraunhofer, Huawei, Samsung, Qualcomm, Sharp, InterDigital, (13) - Comments: - Consider RAN2 impact: Apple, Futurewei, (2)

Draft conclusion 3-19:

RAN1 does not pursue specific enhancement on UE-A’s behavior canceling inter-UE coordination information transmission when the amount of sensing performed by UE-A is below a certain threshold

| Company | Yes or no | Comments |
| --- | --- | --- |
| Fraunhofer | Yes | Our understanding is that UE-A is a full sensing UE, not sure how this scenario would present itself. Hence we are fine with the proposal. |
| Intel | No | Our understanding the intention is not to cancel but rather not to trigger generation. For sidelink transmission, sensing was introduced in R16. In R17, IUC is defined to improve sensing. If feedback is not based on sufficient sensing information it may degrade R16. |
| Apple |  | We are fine with the direction of pursuing specific enhancement, if the threshold has no RRC impact. Otherwise, we can live with the proposed conclusion. |
| LGE | Yes | As per agreement, there are possibilities that UE-A does not trigger inter-UE coordination information generation. |
| Qualcomm | Yes |  |
| Samsung |  | We think that we do not need to spend time for this conclusion. |
| NTT DOCOMO |  | Same view with Apple. |
| Futurewei | Yes with comments | We are ok with the conclusion. But we feel it is ok to have an agreement that IUC is only transmitted when the UE performs full sensing. |
| CATT, GOHIGH |  | We share similar views as Intel, if UE-A doesn’t have sufficient sensing results, it can not be triggered to transmit IUC information in scheme 1. |
| Huawei, HiSilicon | Yes | There is no time for RAN1 to discuss additional optimization issues. |
| OPPO | yes | Agree with Fraunhofer’s understanding. |
| Ericsson | No | We have a similar view as Intel. If the UE does not perform enough sensing the resources sent are likely not reliable or not useful for the receiving UE. |
| Spreadtrum | Yes |  |
| Fujitsu | Yes |  |
| Vivo | YES |  |
| Xiaomi | Yes | Further enhancement is not necessary at this stage. |
| Panasonic | Yes |  |

Q4-22: Do you agree following draft conclusion for UE-B’s behavior to consider “the slot(s) overlapped with UE-A’s reserved resource(s) by 1st stage SCI” as non-preferred resource(s) in its resource selection?

FL’s observation of 3rd email discussion (UE-B’s behavior to consider “the slot(s) overlapped with UE-A’s reserved resource(s) by 1st stage SCI” as non-preferred resource(s) in its resource selection) - Support: vivo, OPPO, Spreadtrum, xiaomi, Intel, Nokia, Qualcomm, Sharp , InterDigital, (9) - Not support: DCM, Fujitsu, NEC, Huawei, Samsung, Ericsson, LGE, (7) - Comments: - Not support the case when a SCI format 1-A is transmitted without PSSCH: LGE, (1) - Define similar behavior for unicast and groupcast to target RX UE: Intel, (1) - UE-A is the destination UE of the TB to be transmitted by UE-B: Apple, Futurewei, (2)

Draft conclusion 3-18:

No consensus on UE-B’s behavior using the slot(s) overlapped with UE-A’s reserved resource(s) by 1st stage SCI as non-preferred resource(s) in its resource selection for unicast/groupcast transmission

| Company | Yes or no | Comments |
| --- | --- | --- |
| Intel | Comment | We can accept it if the similar behavior is defined for unicast and groupcast transmissions to target RX |
| Nokia, NSB | Yes |  |
| Apple | Yes |  |
| LGE | Yes |  |
| Qualcomm | No | We agree that this issue exists in Rel. 16 and that is the reason why we have this work item in Rel. 17 with the objective to handle the half-duplex issue. RAN 1 has agree that this objective should be fulfilled by the following working assumption: - For Scheme 1 with non-preferred resource set, support following condition: - Condition 1-B-2: - Resource(s) (e.g., slot(s)) where UE-A, when it is intended receiver of UE-B, does not expect to perform SL reception from UE-B due to half duplex operation. As there is no differentiation between 1-B-1 and 1-B-2 in the set of non-preferred resource indicated in SCI-2 or MAC-CE. We have not had any discussion so far on this as this, supporting this behavior is the only way to implement condition 1-B-2 in a meaningful way. In this proposal, SCI-1 is always transmitted with the corresponding data or MAC-CE carrying inter-UE coordination information from UE-A. UE-A is the intended receiver of UE-B. So, UE-B should avoid the slot containing the resource reserved for transmission by UE-A as indicated in SCI-1 to fulfill condition 1-B-2. |
| Samsung |  | We think that we do not need to spend time for this conclusion. |
| NTT DOCOMO | Yes |  |
| Futurewei | Yes with comment | We are ok with the conclusion, but we are open to discuss FL proposal in previous round. |
| CATT, GOHIGH | Yes |  |
| Huawei, HiSilicon | Yes | There is no time for RAN1 to discuss additional optimization issues. |
| OPPO | yes | For progress we can accept the conclusion. |
| Ericsson | Yes | No need to discuss it further |
| Spreadtrum | Yes |  |
| Fujitsu | Yes |  |
| Vivo | No | We have not received any technical reason, why not support it. Prefer to continue discussion in GTW. [vivo2] 1.To companies who have concern on the signaling. Regarding whether reservation signaling can be regarded as IUC, this is FFS in previous meeting, we think it can be without additional modification on signaling design. The slots overlapped with reserved resource is non-preferred resource 2.To Huawei’s comment as following, we think UE-B can selects either Rel-16 or Rel-17 resource exclusion behaviour only scheme 1 non-preferred resource is allowed in the pool. “For R16 sensing procedures, reserved resources by 1st SCI will be excluded using RSRP (i.e., in step 6 of TS 38.214 clause 8.1.4). However, if this proposal is agreed, reserved resources by 1st SCI will be excluded after step 6) directly, i.e., regardless of their RSRP. This is very different from legacy sensing procedures.” |
| Sharp | Yes |  |
| Xiaomi | Yes | We are fine with the FL’s proposal. |
| Panasonic | Yes |  |

  - Others

Q4-23: Do you agree following draft conclusion for the reply LS to RAN2?

FL’s observation of 3rd email discussion (conclusion for the reply LS to RAN2) - Support: DCM, LGE, OPPO, Fujitsu, xiaomi, Huawei, Ericsson, Futurewei, Qualcomm, Sharp, InterDigital, (11) - Not support: NEC, Intel, (2)

Draft conclusion 3-21:

- No consensus for RAN1 to send a reply LS of R1-2200880 to RAN2.

| Company | Yes or no | Comments |
| --- | --- | --- |
| Intel | OK |  |
| Apple | No | If the MAC CE field or field size is up to RAN2, we need to notify RAN2 about this conclusion. Otherwise, RAN2 is still relying on RAN1 on the design. |
| LGE | Yes |  |
| Samsung | Comment | We can send list of agreements that impact RAN2 especially the MAC CE contents. |
| NTT DOCOMO | OK |  |
| Futurewei | Yes | We are ok with the conclusion. |
| Huawei, HiSilicon | Yes |  |
| OPPO | yes |  |
| Ericsson | Ok |  |
| Fujitsu | Yes |  |
| xiaomi | Yes |  |

- Draft proposals for 3rd discussion (Due: February 24th 11:59pm UTC)

  - Scheme 2

Q3-1: FL understands that the agreement of “m_CS for a resource conflict indication for the next reserved resource indicated by the corresponding UE-B’s SCI for either current TB transmission or next TB transmission is 0” was made in Wednesday’s GTW session, so further clarification is necessary on UE-B’s behaviour when UE-B receives a conflict indication for reserved resource for next TB transmission. It is understood that the intention of the agreement is that UE-A can transmit a conflict indication for next TB transmission after the last PSSCH transmission indicated by UE-B’s SCI for current TB. Do you agree the following draft proposal?

Agreement For Scheme 2, - m_CS for a resource conflict indication for the next reserved resource indicated by the corresponding UE-B’s SCI for either current TB transmission or next TB transmission is 0 Agreement - Alt 2-1 - For Scheme 2, - The PHY layer reports S_A after Step 7) of TS 38.214 Section 8.1.4 to higher layer. - When UE-B receives a conflict indicator for resource(s) indicated by its SCI, - PHY layer at UE-B reports resources overlapping with the next reserved resource indicated by the corresponding UE-B’s SCI for current TB transmission to higher layer. - If (pre)configured, the PHY layer reports resources in a slot including the next reserved resource indicated by the corresponding UE-B’s SCI for current TB transmission to higher layer. - Higher layer at UE-B re-selects the resource(s) indicated by the conflict indicator among the S_A excluding the reported resources. - FFS: Whether/How the conflict in periodic transmission is indicated by UE-A and handled by UE-B

Draft Proposal 3-1:

- For Scheme 2,

  - When PSFCH occasion is derived by a slot where UE-B’s SCI is transmitted,

    - When UE-B receives a conflict indicator in a PSFCH occasion derived by UE-B’s latest SCI for current TB transmission before next reserved resource for next TB transmission,

      - PHY layer at UE-B reports resources overlapping with the next reserved resource for next TB transmission to higher layer.

        - If (pre)configured, the PHY layer reports resources in a slot including the next reserved resource for next TB transmission to higher layer.

      - Higher layer at UE-B re-selects the resource(s) indicated by the conflict indicator among the S_A excluding the reported resources.

  - When PSFCH occasion is derived by a slot where expected/potential resource conflict occurs on PSSCH resource indicated by UE-B’s SCI,

    - When UE-B receives a conflict indicator in a PSFCH occasion derived by the earliest reserved resource for next TB transmission,

      - PHY layer at UE-B reports resources overlapping with the earliest reserved resource for next TB transmission to higher layer.

        - If (pre)configured, the PHY layer reports resources in a slot including the earliest reserved resource for next TB transmission to higher layer.

      - Higher layer at UE-B re-selects the resource(s) indicated by the conflict indicator among the S_A excluding the reported resources.

| Company | Yes or no | Comments |
| --- | --- | --- |
| NTT DOCOMO |  | For the 1st sub-bullet, is it correct understanding that UE-B reports to higher layer “collision” for both aperiodic resource and periodic resource? Then we are fine with it. For the 2nd sub-bullet, why “earliest” is there? This option of PSFCH determination is just derived by a resource with collision. That’s all. “earliest” should be removed. |
| ZTE, Sanechips | comment | For this case, it is one-to -one mapping b/w the reserved resource and PSFCH occasion, so ‘earliest’ is not needed here, and the current proposal only covered the Tx for next TB, we prefer a unified procedure for both reTx of current TB and next TB. We propose to change the second bullet as following: - When PSFCH occasion is derived by a slot where expected/potential resource conflict occurs on PSSCH resource indicated by UE-B’s SCI, - When UE-B receives a conflict indicator in a PSFCH occasion derived by the earliest a reserved resource for next TB transmission, - PHY layer at UE-B reports resources overlapping with the earliest reserved resource for next TB transmission to higher layer. - If (pre)configured, the PHY layer reports resources in a slot including the earliest reserved resource for next TB transmission to higher layer. - Higher layer at UE-B re-selects the resource(s) indicated by the conflict indicator among the S_A excluding the reported resources. |
| Vivo |  | For 1st bullet, it is not clear whether the resource reselection is triggered at current period or at next period, e.g., when TB transmission on next period is ready. Our preference is that the reselection is triggered at next period. For 2nd bullet, the agreement made in prior meeting can be applied as well. So we think 2nd bullet is not needed. |
| LGE | Yes | In our understanding, the intention of the agreement for m_CS setting is to protect initial transmission of next TB. Meanwhile, retransmission of next TB will be protected by UE-B’s behavior for current TB transmission. In this case, for option 1 timing (Based on slot where SCI is transmitted), the last SCI for current TB transmission is no longer have aperiodic reserved resources. When UE-B receives a conflict indication in a PSFCH occasion derived by this last SCI time location, UE-B will find the first reserved resources for next TB transmission based on UE-B’s first SCI in the current TB transmission. In short, in this time, UE-B will report only the first reserved resource for the next TB transmission as collided resources. For the second bullet, we also need to have “earliest”. We think that reserved resources other than the earliest one is already covered by UE-B’s behavior for current TB transmission case. We also fine to have duplicated agreement if majority companies want. Anyway, at least, we need to have new agreement for this earliest reserved resource for next TB since it is not covered by the previous agreement (UE-B’s behavior for current TB transmission). We share similar view with the timing when UE-B knows the next TB is available or not. For simplicity, it can be considered to add that UE-B use the received conflict indication for next TB transmission if the next TB is available at UE-B side. |
| OPPO | Comment | We are fine with the 2nd sub-bullet (PSFCH occasion is derived by conflicting resource). For the first sub-bullet, if it was agreed, it contradicts with the agreement below, where only resource reserved for the current TB is reported. The agreement below is made based on the assumption that only conflict indication for next reserved resource for current TB is transmitted, under the new agreement made on Wed. GTW, we are wondering whether the agreement below is applicable any more. So maybe we should expand the proposal to cover PSFCH occasion derived by UE-B’s any SCI, and override the agreement below. To the end following modifications for the first sub-bullet are suggested: Agreement - Alt 2-1 - For Scheme 2, - The PHY layer reports S_A after Step 7) of TS 38.214 Section 8.1.4 to higher layer. - When UE-B receives a conflict indicator for resource(s) indicated by its SCI, - PHY layer at UE-B reports resources overlapping with the next reserved resource indicated by the corresponding UE-B’s SCI for current TB transmission to higher layer. - <omitted> Draft Proposal 3-1: - For Scheme 2, - When PSFCH occasion is derived by a slot where UE-B’s SCI is transmitted, - When UE-B receives a conflict indicator in a PSFCH occasion derived by UE-B’s latest SCI for current TB transmission before next reserved resource for next TB transmission, - PHY layer at UE-B reports resources overlapping with the next earliest reserved resource for either current TB or the next TB transmission to higher layer. - If (pre)configured, the PHY layer reports resources in a slot including the next earliest reserved resource for either current TB or next TB transmission to higher layer. - Higher layer at UE-B re-selects the resource(s) indicated by the conflict indicator among the S_A excluding the reported resources. |
| Fujitsu | Yes | Since there is no difference between “next reserved resource for next TB transmission” in the 1st sub-bullet and “the earliest reserved resource for next TB transmission” in the 2nd sub-bullet, it is suggested to use “the earliest reserved resource for next TB transmission” for consistency. |
| Spreadtrum | comment | We think the purpose of this draft proposal should be clarified, i.e., whether it is only targeting the UE-B’s behavior when it receives a conflict indicator of next TB rather than current TB. If yes, we agree with 2nd sub-bullet in this draft proposal. Otherwise, the modification of ZTE looks better. For the 1st sub-bullet, we share the same concerns as OPPO. Besides, we still has a concern that how UE-B know whether the conflict is current TB transmission or next TB transmission in the 1st sub-bullet. |
| NEC |  | From our reading, the proposal has an assumption that when the conflict indication is for next TB transmission, it only could be the initial transmission of next TB. That’s the reason the proposal captures “latest SCI” “earliest reserved resource” in our understanding. However, we have doubt this, do we have common understanding or agreement saying that the next TB should be the initial transmission of next TB? From our perspective, regardless of the PSFCH is derived by SCI or potential conflicted resource, the resource conflict indication is targeting for the next reserved resource which could be either a resources for current TB or next TB as the agreement said. Agreement For Scheme 2, - m_CS for a resource conflict indication for the next reserved resource indicated by the corresponding UE-B’s SCI for either current TB transmission or next TB transmission is 0 Given that, may be a simple way is to revise the previous agreement by Agreement - Alt 2-1 - For Scheme 2, - The PHY layer reports S_A after Step 7) of TS 38.214 Section 8.1.4 to higher layer. - When UE-B receives a conflict indicator for resource(s) indicated by its SCI, - PHY layer at UE-B reports resources overlapping with the next reserved resource indicated by the corresponding UE-B’s SCI to higher layer. - If (pre)configured, the PHY layer reports resources in a slot including the next reserved resource indicated by the corresponding UE-B’s SCI to higher layer. - Higher layer at UE-B re-selects the resource(s) indicated by the conflict indicator among the S_A excluding the reported resources. FFS: Whether/How the conflict in periodic transmission is indicated by UE-A and handled by UE-B |
| Xiaomi | Comment | We share the similar view with vivo, for the first bullet, the PSFCH occasion conveys the resource conflict for next TB in the current period, so it is not clear that resource reselection is triggered in the current period or in the next period, this issue needs to be clarified. |
| Fraunhofer | Yes | We are fine with the proposal, with the understanding that this is an extension to include the case for the next TB transmission (periodic transmissions), since the case for current TB transmission was already agreed. |
| ETRI |  | Similar as other companies, we also think the reason FL try to make above proposal as agreement is that we do NOT have an agreement for the next TB transmission while we already have an agreement for the current TB transmission. With this understanding, NEC’s proposal which revises the previous agreement looks better and much simpler. |
| Huawei, HiSilicon | No | We have different understanding with FL. On the newly made agreement about m_CS, our understanding is “m_CS for a resource conflict indication for the next reserved resource indicated by the corresponding UE-B’s SCI is 0 regardless the next reserved resource is for current TB transmission or for next TB transmission” For example: - As Fig 1 below shows, assume UE-B transmits SCI in resource A1 with “TRIV=0, period>0”, then resource in B1 is the next reserved resource as per agreement, which is for next TB transmission. - As Fig 2 below shows, assume UE-B transmits SCI in resource A1 with “TRIV>0, period>0”, then resource in A2 is the next reserved resource as per agreement, which is for current TB transmission. Regarding FL’s understanding that “It is understood that the intention of the agreement is that UE-A can transmit a conflict indication for next TB transmission after the last PSSCH transmission indicated by UE-B’s SCI for current TB.” We see some problems here: - Problem1: how does UE-B know a PSSCH transmission is the last PSSCH transmission for current TB? - Note that chain reservation cannot be always guaranteed during the transmission of a TB, i.e., in some re-transmission, UE-B may set TRIV=0 if UE-B cannot find a nearby resource for retransmission, and UE-B may continue future retransmission using unreserved resource. - Problem2: as Fig 3 below shows, assume PSSCH transmission in A10 is the last PSSCH transmission, then SCI in A10 reserves resources in B10, not B1 (where B1 is the initial transmission of next TB). So UE-A cannot indicate the conflict for initial transmission of next TB. In summary, regarding the change on the UE-B behavior to reflect the m_CS agreement, we feel RAN1 only needs to remove “for current TB transmission” in previous agreement as below. == Agreement For Scheme 2, - The PHY layer reports S_A after Step 7) of TS 38.214 Section 8.1.4 to higher layer. - When UE-B receives a conflict indicator for resource(s) indicated by its SCI, - PHY layer at UE-B reports resources overlapping with the next reserved resource indicated by the corresponding UE-B’s SCI for current TB transmission to higher layer. - If (pre)configured, the PHY layer reports resources in a slot including the next reserved resource indicated by the corresponding UE-B’s SCI for current TB transmission to higher layer. - Higher layer at UE-B re-selects the resource(s) indicated by the conflict indicator among the S_A excluding the reported resources. - FFS: Whether/How the conflict in periodic transmission is indicated by UE-A and handled by UE-B == Fig 1 Fig 2 Fig 3 |
| Intel | OK | Intel |
| Apple | Comment | In our understanding, UE-B’s behavior when it only has aperiodic transmission is agreed in last meeting. Here, the discussion is UE-B’s behavior when it has periodic resource reservation, which is open from last meeting. Hence, we could make it clear that the new proposal is for “when UE-B has periodic resource reservation”. Under this condition, we could make the proposal in a general way, i.e., no matter it is for the initial transmission of a period, or it is for the retransmission of a period. For Scheme 2 when UE-B has periodic resource reservation, - When PSFCH occasion is derived by a slot where UE-B’s SCI is transmitted, - When UE-B receives a conflict indicator in a PSFCH occasion derived by UE-B’s latest SCI for current TB transmission before next reserved resource for next TB transmission, - PHY layer at UE-B reports resources overlapping with the next reserved resource, either for the current TB transmission or for next TB transmission, to higher layer. - If (pre)configured, the PHY layer reports resources in a slot including the next reserved resource, either for the current TB transmission or for next TB transmission to higher layer. - Higher layer at UE-B re-selects the resource(s) indicated by the conflict indicator among the S_A excluding the reported resources. - When PSFCH occasion is derived by a slot where expected/potential resource conflict occurs on PSSCH resource indicated by UE-B’s SCI, - When UE-B receives a conflict indicator in a PSFCH occasion derived by the earliest reserved resource for next TB transmission, - PHY layer at UE-B reports resources overlapping with the earliest next reserved resource for next TB transmission to higher layer. - If (pre)configured, the PHY layer reports resources in a slot including the earliest next reserved resource for next TB transmission to higher layer. - Higher layer at UE-B re-selects the resource(s) indicated by the conflict indicator among the S_A excluding the reported resources. |
| Samsung | Yes | We suggest the following modification for clarification: - For Scheme 2, - When PSFCH occasion is derived by a slot where UE-B’s SCI is transmitted, - When UE-B receives a conflict indicator in a PSFCH occasion derived by UE-B’s latest SCI for current TB transmission before next reserved resource, where the next reserved resource is for next TB transmission, … |
| Ericsson | Yes but clarification is needed | We are fine with the approach, but think that the wording must be clarified. For Scheme 2, - When PSFCH occasion is derived by a slot where UE-B’s SCI is transmitted, - When UE-B receives a conflict indicator in a PSFCH occasion derived by UE-B’s latest SCI for current TB transmission, and the next reserved resource corresponds to a next TB, before next reserved resource for next TB transmission, - PHY layer at UE-B reports resources overlapping with the next reserved resource for next TB transmission to higher layer. - If (pre)configured, the PHY layer reports resources in a slot including the next reserved resource for next TB transmission to higher layer. - The same RRC parameter as for the first part of the above agreement is used. - Higher layer at UE-B re-selects the resource(s) indicated by the conflict indicator among the S_A excluding the reported resources. - When PSFCH occasion is derived by a slot where expected/potential resource conflict occurs on PSSCH resource indicated by UE-B’s SCI, - When UE-B receives a conflict indicator in a PSFCH occasion derived by the earliest reserved resource for next TB transmission, - PHY layer at UE-B reports resources overlapping with the earliest reserved resource for next TB transmission to higher layer. - If (pre)configured, the PHY layer reports resources in a slot including the earliest reserved resource for next TB transmission to higher layer. - The same RRC parameter as for the second part of the above agreement is used. - Higher layer at UE-B re-selects the resource(s) indicated by the conflict indicator among the S_A excluding the reported resources. |
| Futurewei | Comment | We need to consider the case that if at least one of UEs scheduling conflicting TBs is not capable of receiving the conflict indication. |
| Nokia, NSB | Comments | For the case “When PSFCH occasion is derived by a slot where UE-B’s SCI is transmitted”, it is not clear to us what is meant by “latest SCI”: If this said “last SCI”, then it would be clear what the intention is; but if the intention is latest SCI in the sense of “most recent SCI”, then it is unclear how UE-B will interpret this, since this will then indicate both next reservation of current TB and next reservation of next TB. |
| Qualcomm | Comment | We share the same view as Oppo regarding the case when PSFCH occasion is derived by the slot when UE-B-s SCI is transmitted. We are not clear as to how the UE knows which is the latest SCI when ACK-NACK feedback is enabled. Furthermore, the current wording does not cover the case where the receiving UE misses some of the SCI due to decoding failure. As a result, the association between the latest SCI and the initial resource reservation for the next TB may be incorrect. For example, in the case when a UE is transmitting 2 TBs with 2 SCI-s for each TB. A receiver UE may not receive the latest SCI of the 1st TB and the initial SCI of the 2nd TB. It may incorrectly associate the latest SCI of the 2nd TB to the initial SCI of the 1st TB. |
| InterDigital | Yes |  |

Q3-2: Do you agree for following draft proposal for UE-B determination?

FL’s observation of 2nd email discussion: - Support: DCM, Panasonic, InterDigital, ETRI, LGE, Qualcomm, Futurewei, Spreadtrum, ZTE, Fujitsu, NEC, OPPO, xiaomi, Ericsson, CATT, Fraunhofer, Nokia, Intel, MediaTek (19) - Not support: Samsung, vivo, Huawei, (3) - No additional change on the WA: Samsung, (1) - Vivo, Huawei: Remove last note. - Comments: - Apple: Replace “(i.e., minimum time gap between PSFCH and SCI(s) scheduling conflicting TBs, minimum time gap between PSFCH and a slot where expected/potential resource conflict occurs)” with “(i.e., sl-MinTimeGapPSFCH, T_3)” FL’s observation of 2nd email discussion: - Additional enhancement for UE-B determination in addition to draft proposal 4: Apple, Futurewei, Spreadtrum, (3) - Apple: Further consider a case where both UEs, scheduling the conflict TB, do not have the capability of receiving IUC scheme 2 - Futurewei: If at least one of UEs scheduling conflicting TBs is not capable of receiving the conflict indication, except the UEs not capable of receiving the conflict indication, all other UEs scheduling the conflicting TBs whose PSFCH occasions for resource conflict indication are not yet passed are UE-B - Spreadtrum: Clarification on “Capable of receiving the conflict indication”. - No additional enhancement for UE-B determination in addition to draft proposal 4: DCM, Panasonic, ETRI, LGE, Qualcomm, Sharp, ZTE, Fujitsu, OPPO, Samsung, xiaomi, Ericsson, CATT, Fraunhofer, Huawei, Intel, (16)

Draft proposal 3-2:

- Confirm the following working assumption with modification in RED. Note that the terminology of “indicationUEB flag” means the indication of whether UE scheduling a conflict TB can be UE-B or not.

  - Working Assumption:

    - For Condition 2-A-1 in Scheme 2, when “a non-destination UE of a TB transmitted by UE-B can be UE-A” is enabled or when “a non-destination UE of a TB transmitted by UE-B can be UE-A” is disabled and the destination UE of the conflicting TBs is UE-A,

      - for each pair of UEs scheduling the conflicting TBs whose PSFCH occasions for resource conflict indication are not yet passed and indicationUEB flag is set to 1 if the higher parameter of indicationUEBScheme2 is (pre)configured to ‘Enabled’, a UE with the higher priority value is UE-B.

        - Note: if there is only one UE scheduling the conflicting TB whose PSFCH occasion for resource conflict indication is not yet passed and indicationUEB flag is set to 1 if the higher parameter of indicationUEBScheme2 is (pre)configured to ‘Enabled’, that UE is UE-B.

      - Note: A UE not satisfying the timeline (i.e., minimum time gap between PSFCH and a slot where a SCI is transmitted of sl-MinTimeGapPSFCH, minimum time gap between PSFCH and a slot where expected/potential resource conflict occurs on PSSCH resource indicated by a SCI of T_3) is not considered as UE-B.

| Company | Yes or no | Comments |
| --- | --- | --- |
| NTT DOCOMO | Yes |  |
| ZTE, Sanechips | Yes |  |
| vivo | See comment | For the last bullet, we are going to define the minimum time gap between PSFCH and a slot where expected/potential resource conflict as T3. Basically, T_3 is used for PSSCH resource selection preparation, but now we also need to define PSFCH decoding time, T_3 may be not enough, is it correct understanding. |
| LGE | Yes | In our understanding, T_3 part is a part of agreement. We do not need to discuss for other value for this purposes. - Agreement: - A resource pool level (pre-)configuration uses either of the following options - Option 1: PSFCH occasion is derived by a slot where UE-B’s SCI is transmitted - Reuse PSSCH-to-PSFCH timing as specified in TS 38.213 Section 16.3 to determine the PSFCH occasion for resource conflict indication - Time gap between the PSFCH and a slot where expected/potential resource conflict occurs is larger than or equal to T_3 - Option 2: PSFCH occasion is derived by a slot where expected/potential resource conflict occurs on PSSCH resource indicated by UE-B’s SCI - UE-A transmits the PSFCH in a latest slot that includes PSFCH resources for inter-UE coordination information and is at least T_3 slots of the resource pool before the PSSCH resource indicated by UE-B’s SCI in which expected/potential resource conflict occurs - FFS: How to account for processing timeline - Note that it is possible not to configure either option1 or option 2. |
| OPPO | yes |  |
| Fujitsu | Yes | From our understanding, the last Note about the timeline clarifies the follows. If UE1 satisfies the timeline but UE2 does not, then UE1 is UE-B. |
| Spreadtrum | Yes |  |
| NEC | Yes |  |
| xiaomi | Yes with comment | We think the part of note has been agreed in the last meeting, so the note is not necessary. |
| Fraunhofer | Yes |  |
| ETRI | Yes |  |
| Huawei, HiSilicon | Need to clarify last Note | We are generally fine with the proposal except the last Note. More clarifications on the last Note are appreciated. According to previous agreements (copied below, especially cyan part), it was already agreed that “…UE does not transmit the conflict indicator or receive the conflict indicator if the timeline is not satisfied”. So if the timeline is not satisfied, the UE does not receive the conflict indicator as per agreement and thus is not considered as UE-B. To our understanding, the previous agreements below are complete. So we are unclear what is the intention of the last Note. What’s the new information from the last Note compared to the previous agreements below? == Agreement A resource pool level (pre-)configuration uses either of the following options - Option 1: PSFCH occasion is derived by a slot where UE-B’s SCI is transmitted - Reuse PSSCH-to-PSFCH timing as specified in TS 38.213 Section 16.3 to determine the PSFCH occasion for resource conflict indication - Time gap between the PSFCH and a slot where expected/potential resource conflict occurs is larger than or equal to T_3 - Option 2: PSFCH occasion is derived by a slot where expected/potential resource conflict occurs on PSSCH resource indicated by UE-B’s SCI - UE-A transmits the PSFCH in a latest slot that includes PSFCH resources for inter-UE coordination information and is at least T_3 slots of the resource pool before the PSSCH resource indicated by UE-B’s SCI in which expected/potential resource conflict occurs - FFS: How to account for processing timeline Note that it is possible not to configure either option1 or option 2. Agreement - When PSFCH occasion is derived by a slot where expected/potential resource conflict occurs on PSSCH resource indicated by UE-B’s SCI, time gap between the PSFCH and SCI(s) scheduling conflicting TBs is larger than or equal to X value - X = sl-MinTimeGapPSFCH - UE does not transmit the conflict indicator or receive the conflict indicator if the timeline is not satisfied |
| Intel | No with comments | Small change is needed. Change 1: Our motivation is to respect priority when UE-B is determined in the first note. Proposed rewording: Note: if there is only one UE scheduling the conflicting TB whose PSFCH occasion for resource conflict indication is not yet passed and indicationUEB flag is set to 1 if the higher parameter of indicationUEBScheme2 is (pre)configured to ‘Enabled’, that UE is UE-B if it has higher priority value |
| Apple | Yes with comments | We agree that the last note is necessary, with the similar view as Fujitsu. The last Note is to ensure if UE1 satisfies the timeline but UE2 does not, then UE2’s SCI is not considered when determining whether UE1 is UE-B. Consider UE2’s SCI is received on the same slot as UE1’s PSFCH occasion. Since UE-A has not gotten the time to decode/process UE2’s SCI, the information in UE2’s SCI is not considered when determining whether UE1 is UE-B. In case it turns out UE2’s SCI has potential collision with UE1’s SCI in a later time, then UE-A can only treat UE2 as UE-B, since PSFCH occasion for UE1 is passed. For completeness, we may cover the case where both UEs scheduling conflicting TBs are unable to receive IUC. In this case, no UE is UE-B. |
| Samsung | No | We should not change the WA expect if it is clearly broken. We think that the WA is sufficient. Setting conflict indication based on the timeline or whether the UE supports or doesn’t support conflict feedback leads to the following drawbacks: - This could lead to conflicting results with Rel-16 pre-emption leading to wasted resources. - It disincentives UEs from indicating that they support conflict indication. Based on the agreement made in the Wednesday GTW, a conflict can happen for a reservation of the next TB. The SCI reserving such a resource for the next TB is sent much earlier than the resource being indicated. This proposal gives an advantage to these resources as the PSFCH occasion for indication would have most likely already past. This can be problematic when the colliding resource has an SCI that is sent later in time and is of higher priority, and potentially with a tighter PDB. |
| Ericsson | Yes, without the note | The note is redundant and will likely lead to confusion. The relevant agreement was made in RAN1#107bis-e |
| Futurewei | Comment | We need to consider the case that if at least one of UEs scheduling conflicting TBs is not capable of receiving the conflict indication. |
| Nokia, NSB | Yes |  |
| Qualcomm | Yes | Agree with the working assumption. |
| InterDigital | Yes |  |

Q3-3-1: Do you agree following draft conclusion for PSFCH TX/TX or TX/RX prioritization for a conflict indication?

FL’s observation of 2nd email discussion: - Same understanding: DCM, Panasonic, ETRI, InterDigital, LGE, Qualcomm, Futurewei, Sharp, Spreadtrum, Fujitsu, NEC, vivo, OPPO, xiaomi, Huawei, Intel, MediaTek (17) - Different understanding: Apple, Ericsson, (2) - Add an additional step to prioritize PSFCH with SL HARQ-ACK information over PSFCH with a conflict indication in a specification after prioritization with LTE SL TX/RX or UL: Apple, (1) - Comments: - For prioritization between Rel-17 PSFCH and UL with SL-HARQ, UL with SL-HARQ is always prioritized to protect the SL HARQ. This point should be clarified: vivo, (1)

Draft conclusion 3-3:

RAN1 understands that a UE performs PSFCH TX/RX or TX/TX prioritization between SL HARQ-ACK feedback(s) and resource conflict indication(s) first, and then the UE performs prioritization between prioritized PSFCH TX(s) or RX(s) and LTE SL TX/RX or UL by reusing prioritization rule as specified in TS 38.213 Section 16.2.4.1 and 16.2.4.3.1.

| Company | Yes or no | Comments |
| --- | --- | --- |
| NTT DOCOMO | Yes |  |
| ZTE,Sanechips | Yes |  |
| Vivo | Yes |  |
| LGE | Yes | The current specification is already clear. Even for PSFCH of SL HARQ-ACK information, LTE SL TX/RX or UL prioritization is performed after UE decide whether PSFCH TX or RX is prioritized and which PSFCH TX(s) are prioritized. |
| OPPO | yes |  |
| Fujitsu | Yes |  |
| Spreadtrum | Yes |  |
| NEC | Yes |  |
| xiaomi | Yes |  |
| ETRI | Yes |  |
| Huawei, HiSilicon | Yes |  |
| Intel | Yes | Intel |
| Apple |  | We can accept the direction of this proposal, if this is the majority companies’ view. However, we think the current specification is ambiguous on this point. It does not specify that the prioritization among PSFCH with HARQ or with IUC is performed earlier than the prioritization between PSFCH and LTE sidelink/Uu. Hence, we think the “conclusion” should be changed to “agreement” to ensure it is captured in the specifications properly. Also, we should remove the wording “RAN1 understands that” in the proposal. |
| Samsung | Yes |  |
| Ericsson |  | We can accept this conclusion if this is the view of the majority of companies for the sake of progress. |
| Futurewei | Yes with comments | We are generally fine with the conclusion. However we are not clear whether the second part “and then…” is necessary. Since we agreed PSFCH TX/RX for SL HARQ-ACK feedback is always prioritized over PSFCH TX/RX for a resource conflict indication, after the prioritization, the PSFCH TX/RX for SL HARQ-ACK is always kept. Then legacy prioritization rule applies. If there is no PSFCH TX/RX for HARQ-ARQ, we do not need this conclusion. |
| Nokia, NSB | Yes |  |
| Qualcomm | Yes |  |
| Sharp | Yes |  |
| InterDigital | Yes |  |

Q3-3-2: There was a comment that RAN1 needs to discuss whether UL with SL-HARQ-ACK information is always prioritized over PSFCH for a conflict indication to protect the SL HARQ. However, FL understands that SL HARQ-ACK reporting on UL is supported only for Mode 1 RA UE, so this issue is not necessary to be considered in Rel-17 inter-UE coordination operation (i.e., only Mode RA 2 UE can transmit or receive PSFCH for a conflict indication). Do you have the same understanding?

| Company |  | Yes or no | Comments |
| --- | --- | --- | --- |
| NTT DOCOMO |  | Yes |  |
| ZTE,Sanechips | Same understanding |  |  |  |
| vivo |  | Do not agree with FL guidance | For a given UE, mode 1 and mode 2 may coexist, so even we only enhance mode 2, the mode 1 SL-HARQ on UL will appear. Actually, mode 1 operation is the same as operation in another RAT, e.g., LTE SL. We did not say we are going to enhance LTE SL, but the prioritization rule also involves LTE SL. |
| LGE |  | Yes | Since UE-A will perform sensing-like operation for determining a resource conflict, this UE-A is in Mode 2 RA. Then, we do not need to consider the case when UL containing SL HARQ-ACK feedback is overlapping with PSFCH of a conflict indication. |
| Fujitsu |  | Yes |  |
| Spreadtrum |  | Yes |  |
| NEC |  | Yes |  |
| xiaomi |  | Yes |  |
| ETRI |  | Yes |  |
| Huawei, HiSilicon |  | Yes | Same understanding as FL. No need to discuss this. |
| Intel |  | Yes |  |
| Apple |  | Yes | Unless a resource pool supports both mode 1 and mode 2, the prioritization between PUCCH with SL HARQ and PSFCH with IUC does not occur. |
| Samsung |  | Yes |  |
| Ericsson |  | Yes |  |
| Futurewei |  | Comment | We are ok to discuss the prioritization with UL with SL-HARQ-ACK information if a UE can be configured with both modes, e.g. in different pools but slot overlap. |
| Nokia, NSB |  | Yes |  |
| Qualcomm |  | Yes | We are okay to not discuss this issue. |
| Sharp |  | Yes |  |
| InterDigital |  | Yes |  |

Q3-4: Do you agree following draft conclusion for clarification on “next reserved resource indicated by the corresponding UE-B’s SCI for current TB transmission”? FL understands that as per RAN1 agreement, a UE will not transmit the conflict indicator or receive the conflict indicator if the timeline is not satisfied.

FL’s observation of 2nd email discussion: - Same understanding: InterDigital, LGE, Qualcomm, Futurewei, Sharp, ZTE, NEC, vivo, Fujitsu, Samsung, xiaomi, Ericsson, CATT, Fraunhofer, Huawei, MediaTek (16) - Different understanding: DCM, ETRI, Spreadtrum, OPPO, Nokia, Intel, (6) - When PSFCH occasion is derived by a slot where UE-B’s SCI is transmitted, the earliest reserved resource indicated by the SCI and at least T_3 after the PSFCH occasion: DCM, Spreadtrum, OPPO, Nokia, (4)

Draft conclusion 3-4:

RAN1 understands that the meaning of “next reserved resource indicated by the corresponding UE-B’s SCI for current TB transmission” is as follows:

- When PSFCH occasion is derived by a slot where UE-B’s SCI is transmitted, the earliest reserved resource indicated by the SCI for current TB transmission

- When PSFCH occasion is derived by a slot where expected/potential resource conflict occurs on PSSCH resource indicated by UE-B’s SCI, the reserved resource indicated by UE-B’s SCI for current TB transmission associated with PSFCH occasion for receiving a conflict indicator for resource(s) indicated by the SCI

| Company | Yes or no | Comments |
| --- | --- | --- |
| NTT DOCOMO | No | For 1st bullet, we guess FL has some misunderstanding. In the following figure, still UE-A transmits collision indication and UE-B receives collision indication since required time gap is between PSFCH and slot where resource conflict will occur. NOT with the earliest slot among reserved slots. Please see the agreement at 107-e meeting. For 2nd bullet, OK. |
| ZTE,Sanechips | Yes |  |
| vivo | Yes |  |
| LGE | Yes |  |
| OPPO |  | This proposal is related to Draft Proposal 3-1, it should be deferred. |
| Fujitsu | Yes with comments | Agree in principle. To avoid any ambiguity on the T_3 timeline, the following is suggested for the 1st sub-bullet. It has a similar format with that of 2nd sub-bullet. When PSFCH occasion is derived by a slot where UE-B’s SCI is transmitted, the earliest reserved resource indicated by the SCI for current TB transmission associated with PSFCH for deriving the timeline |
| Spreadtrum | Yes |  |
| NEC | Yes |  |
| xiaomi | Yes |  |
| Fraunhofer | Yes |  |
| ETRI | Yes |  |
| Huawei, HiSilicon | Discuss Draft Proposal 3-1 first | Due to the newly made agreement below, indication for next TB is also supported. So RAN1 needs to discuss Draft Proposal 3-1 first to make sure companies are on the same page. == Agreement For Scheme 2, - m_CS for a resource conflict indication for the next reserved resource indicated by the corresponding UE-B’s SCI for either current TB transmission or next TB transmission is 0 |
| Intel | No | Since UE-B is not aware which resource is in collision it is expected to reselect reserved resources for current TB. |
| Apple | Yes |  |
| Samsung | Yes | This conclusion should be updated based on the agreements made in the Wednesday GTW (include next TB transmission) |
| Ericsson |  | Due to the new agreements from last GTW we do not think this is needed anymore |
| Futurewei | Comment | We are not clear why we need to add “for current TB transmission”. Without it, based on the latest agreement in GTW, the agreement can be applied for both current and next TB transmissions. |
| Nokia, NSB | No | For 1st bullet agree with DOCOMO; anyway, given recent discussions about next TB, we are not sure that this discussion is still that relevant. |
| Qualcomm | Yes |  |
| InterDigital | Yes (with comments) | We are okay with the conclusion. The wording of “associated with PSFCH occasion” implies the PSFCH occasion is for receiving a conflict indicator corresponding to the resource associated with PSFCH occasion. Therefore, the conclusion can be simplified as below. - When PSFCH occasion is derived by a slot where expected/potential resource conflict occurs on PSSCH resource indicated by UE-B’s SCI, the reserved resource indicated by UE-B’s SCI for current TB transmission associated with PSFCH occasion for receiving a conflict indicator for resource(s) indicated by the SCI |

Q3-5: Do you agree following draft conclusion for additional enhancement on Mode 2 resource selection procedure to ensure the timeline in Rel-17 inter-UE coordination operation?

FL’s observation of 2nd email discussion: - Additional enhancement on Mode 2 RA to ensure the timeline - Support: Apple, InterDigital, Futurewei, Sharp, NEC, Ericsson, CATT, (7) - Not support: DCM, ETRI, LGE, Qualcomm, Spreadtrum, Fujitsu, vivo, OPPO, Samsung, xiaomi, Huawei, Intel, MediaTek (13)

Draft conclusion 3-5:

No consensus on any specific enhancement in Rel-17 on Mode 2 resource selection procedure to ensure the timeline (i.e., minimum time gap between PSFCH and a slot where a SCI is transmitted of sl-MinTimeGapPSFCH, minimum time gap between PSFCH and a slot where expected/potential resource conflict occurs on PSSCH resource indicated by a SCI of T_3) for a conflict indication.

| Company | Yes or no | Comments |
| --- | --- | --- |
| NTT DOCOMO | Yes |  |
| ZTE,Sanechips | Yes |  |
| Vivo | Yes |  |
| LGE | Yes |  |
| OPPO | yes |  |
| Fujitsu | Yes |  |
| Spreadtrum | Yes |  |
| NEC | Yes |  |
| xiaomi | Yes |  |
| ETRI | Yes |  |
| Huawei, HiSilicon | Yes | No need for further enhancements at this late stage. |
| Intel | Yes |  |
| Samsung |  | OK for this but we do not need to spend time for this conclusion. |
| Ericsson | Yes |  |
| Futurewei | Comment | We prefer to discuss the time line (minimum time gap) for the conflict indication. We are not clear on the meaning of no consensus conclusion. Does mean no addition enhancement (no timeline specification)? |
| Nokia, NSB | Yes |  |
| Qualcomm | Yes | As long as the timeline is ensured. |
| InterDigital | Yes |  |

  - Scheme 1

Q3-6: Do you agree following draft conclusion for (pre)configuration of parameters related to n+T_1 and n+T_2 for determining the set of preferred resources in inter-UE coordination information triggered by a condition other than explicit request reception?

FL’s observation of 2nd email discussion: - Support: DCM, Apple, Panasonic, ETRI, InterDigital, LGE, Qualcomm, CMCC, ZTE, Fujitsu, NEC, OPPO, Samsung, vivo, xiaomi, CATT, Fraunhofer, Huawei, Nokia, MediaTek (20) - Not support: Ericsson, Intel, (2) - Intel: As a compromise, T_2,min is satisfied for feedback generation

Draft conclusion 3-6:

Not support (pre)configuration of parameters related to n+T_1 and n+T_2 for determining the set of preferred resources in inter-UE coordination information triggered by a condition other than explicit request reception.

- Note that T_2 – T_1 for determining the set of preferred resources is no smaller than T_2,min as specified in TS 38.214 section 8.1.4.

| Company | Yes or no | Comments |
| --- | --- | --- |
| NTT DOCOMO | Yes |  |
| ZTE,Sanechips | Yes |  |
| Vivo | Yes |  |
| LGE | Yes |  |
| OPPO | Yes |  |
| Fujitsu | Yes |  |
| Spreadtrum | Yes |  |
| NEC | Yes |  |
| Xiaomi | Yes | We think resource selection window is determined by UE-A’s implementation is enough. |
| Fraunhofer | Yes |  |
| ETRI | Yes |  |
| Huawei, HiSilicon | Sub-bullet is inaccurate | We are generally fine with this proposal. But the sub-bullet seems inaccurate. Current TS 38.214 says “T2min ≤ T2” (copied below, cyan part), it does not say “T2-T1 >= T2,min”. So the following red changes on the sub-bullet is needed. We also suggest to remove the sub-bullet since it’s strange to emphasize “T2min ≤ T2” separately. == Draft conclusion 3-6: Not support (pre)configuration of parameters related to n+T_1 and n+T_2 for determining the set of preferred resources in inter-UE coordination information triggered by a condition other than explicit request reception. - Note that T_2 – T_1 for determining the set of preferred resources is no smaller than T_2,min as specified in TS 38.214 section 8.1.4. == (…below is copied from TS 38.214…) The following steps are used: 1) … - selection of T1 is up to UE implementation under 0 ≤ T1≤ Tproc,1SL , where Tproc,1SL is defined in slots in Table 8.1.4-2 where μSL is the SCS configuration of the SL BWP; - if T2min is shorter than the remaining packet delay budget (in slots) then T2 is up to UE implementation subject to T2min ≤ T2 ≤ remaining packet delay budget (in slots); otherwise T2 is set to the remaining packet delay budget (in slots). |
| Intel | Yes with minor change | - Note that T_2 – T_1 for determining the set of preferred resources is no smaller than T_2,min as specified in TS 38.214 section 8.1.4. |
| Apple | Yes |  |
| Samsung |  | OK for this but we do not need to spend time for this conclusion. |
| Ericsson |  | Although it is not our preference, we can accept the conclusion for the sake of progress. |
| Futurewei | Yes with comment | On the note, in TS 38.214 section 8.1.4, it is T_2>= T_2,min, not T_2 – T_1>= T_2,min |
| Nokia, NSB | Yes |  |
| Qualcomm | Yes |  |
| Sharp | Yes |  |
| InterDigital | Yes |  |

Q3-7: Do you agree following draft proposal for confirming the WA?

FL’s observation of 2nd email discussion: - Support: DCM, Apple, Panasonic, ETRI, InterDigital, LGE, Qualcomm, Futurewei, CMCC, ZTE, Fujitsu, NEC, OPPO, Samsung, xiaomi, Ericsson, CATT, Fraunhofer, Huawei, Nokia, Intel, MediaTek (22) - Not support: vivo, (1) - Slot offset for first TRIV is always 0: vivo, (1)

Draft proposal 3-7:

Confirm the following working assumption with modification in RED

- Working assumption made in RAN1#107bis-e:

  - First resource location of each TRIV is a slot offset with respect to a reference slot

    - Alt 2:

      - The slot offset is the number of logical slots from the reference slot

        - The value range of slot offsets is from 0 to maximum value that is (pre)configurable up to [8000256]

          - FFS: The detailed value range including granularity

        - Slot offset for each TRIV to indicate the set of resources is separately indicated by inter-UE coordination information

    - For the reference slot,

      - The reference slot is the slot indicated by the inter-UE coordination information in a form of combination of DFN index and slot index

| Company | Yes or no | Comments |
| --- | --- | --- |
| NTT DOCOMO | Yes |  |
| ZTE,Sanechips | Yes |  |
| Vivo | Yes for compromise | If majority prefer no optimization, we are fine. |
| LGE | Yes |  |
| OPPO | yes |  |
| Fujitsu | Yes |  |
| Spreadtrum | Yes |  |
| NEC | Yes |  |
| xiaomi | Yes |  |
| Fraunhofer | Yes |  |
| ETRI | Yes |  |
| Huawei, HiSilicon | Yes |  |
| Intel | Yes | Intel |
| Apple | Yes |  |
| Samsung | Comment | The proposal is fine in general with the following changes: - The reference slot is the location of the first resource location of the first TRIV. The slot offset to the first TRIV is 0, and is not signaled. |
| Ericsson | Yes |  |
| Futurewei | Yes |  |
| Nokia, NSB | Yes |  |
| Qualcomm | Yes |  |
| Sharp | Yes |  |
| InterDigital | Yes |  |

Q3-8: Do you agree following draft proposal for the maximum value of N for a SCI format 2-C?

FL’s observation of 2nd email discussion: - Support: DCM, Panasonic, ETRI, InterDigital, LGE, Sharp, Spreadtrum, Fujitsu, NEC, OPPO, Samsung, vivo, xiaomi, Ericsson, CATT, Huawei, Intel, MediaTek (18) - Not support: Qualcomm, Futurewei, ZTE, Fraunhofer, Nokia, (5) - Allowing N=3 depending on (pre)configurations: Qualcomm, Futurewei, Fraunhofer, Nokia, (4) - First resource location value for first TRIV is always 0: ZTE, (1)

Draft proposal 3-8:

- For following agreement,

  - Replace “[N<=3]” with “N<=2”

  - Replace “[N>3]” with “N>2”

  - Replace “[N=3]” with “N=2”

Agreement made in RAN1#107bis-e: The following working assumption is confirmed with modification in RED. - MAC CE or 2nd SCI are used as the container of inter-UE coordination information transmission from UE A to UE B. - For the indication of resource set, the following is supported: - N combinations of TRIV, FRIV, resource reservation period as specified in Rel-16 TS 38.214 Section 8.1.5 with following modification. The value of resource reservation period is omitted at least when the transmission of preferred resource set is triggered by UE-B’s explicit request. - First resource location of each TRIV is separately indicated by the inter-UE coordination information - If [N <= 3], MAC CE is used and it is up to UE implementation to additionally use 2nd SCI. When 2nd SCI and MAC CE are both used, the same resource set is indicated in the 2nd SCI and the MAC CE. If [N > 3], only MAC CE is used. - FFS: UE capability details - 2nd SCI is UE RX optional - The field size of the indication of resource set in a SCI format 2-C is determined by [N=3]

| Company | Yes or no | Comments |
| --- | --- | --- |
| NTT DOCOMO | Yes |  |
| vivo | No | I note that the following comment for proposal 3-9: 0 bit for First resource location for first TRIV: Qualcomm, Samsung, vivo, Fraunhofer, MediaTek (5) Based on that, we can modify the WA: First resource location value for first TRIV is always 0 |
| LGE | Yes |  |
| OPPO | yes | If companies still want to support N=3 when number of sub-channels is small, we are fine with following changes: - Replace “[N<=3]” with “N<=2 Y” - Replace “[N>3]” with “N>2 Y” - Replace “[N=3]” with “N=2 Y” - Replace “[N=3]” with “N=2 Y” - Y is the maximum integer that ensures the size of SCI 2-C(not including the CRC) not larger than 140. |
| Fujitsu | Yes |  |
| Spreadtrum | Yes |  |
| NEC | Yes |  |
| xiaomi | Yes | If the proposal on information field of IUC and request can be agreed first, we support this proposal. |
| Fraunhofer | Comment | As stated in the previous round, we would still prefer N=3 if feasible, else we can support this proposal. We are also fine with OPPO’s suggestion. |
| ETRI | Yes |  |
| Huawei, HiSilicon | Yes | “N=2” seems to be the simplest way. |
| Intel | Yes |  |
| Apple | Yes |  |
| Samsung | Yes |  |
| Ericsson | Yes |  |
| Futurewei | Comment | Since it is up-to UE implementation to use 2nd SCI, the upper bound of N should be the maximum pairs are possibly allowed in SCI. If there is one configuration allows 3 pairs in SCI, our preference is to still allow 3 in that case. |
| Nokia, NSB | No | We see no harm in allowing the maximum possible value of N, as long as the SCI format 2-C is not larger than 140 bits. We support OPPO’s proposal above. |
| Qualcomm | Yes | We agree with OPPO’s suggestion with the understanding that Y depends only on (pre)configuration. |
| Sharp | Yes |  |
| InterDigital | Yes |  |

Q3-9: Do you agree following draft proposal for bit field sizes of a SCI format 2-C for inter-UE coordination information? There was a comment that when the lowest subchannel indices for first resource location for each TRIV is separately indicated, defining additional field or mechanism is necessary to indicate unused resource combination(s). However, FL understands that this issue can be simply resolved by UE implementation without having further specification work, e.g., different resource combinations indicate the same set of resources. I would like to emphasize that there is no officially approved CR which includes the bit field sizes of the SCI format 2-C. In other words, by making the relevant agreement, we can avoid unnecessary discussion in RAN1 CR phase and also give RAN2 sufficient time to proceed their related work in the next week or in RAN2 CR phase.

FL’s observation of 2nd email discussion (bit field sizes of a SCI format 2-C for inter-UE coordination information except for the indication of the lowest subchannel index for the first resource location): - Support: DCM, Panasonic, ETRI, InterDigital, LGE, Qualcomm, Futurewei, Sharp, Spreadtrum, ZTE, Fujitsu, NEC, OPPO, Samsung, vivo, xiaomi, Ericsson, CATT, Fraunhofer, Huawei, Nokia, Intel, (22) - Not support: Apple, (1) - Comments: - 0 bit for First resource location for first TRIV: Qualcomm, Samsung, vivo, Fraunhofer, MediaTek (5) - 0 bit for resource reservation period if periodic reservation is disabled in the pool: Qualcomm, Nokia, (2) FL’s observation of 2nd email discussion (indicating the lowest subchannel index for the first resource location of each TRIV): - Support: DCM, Apple, Panasonic, ETRI, ZTE, Fujitsu, NEC, OPPO, vivo, xiaomi, Ericsson, CATT, Fraunhofer, Nokia, Intel, MediaTek (16) - Not support: LGE, Qualcomm, Futurewei, CMCC, Sharp, Samsung, (6) - Discuss it after deciding the condition of using a SCI format 2-C: Huawei, (1)

Draft proposal 3-9:

- For Scheme 1, each bit field size of a SCI format 2-C for inter-UE coordination information is given by following table:

  - Note that lowest subchannel index for the first resource location of each TRIV is separately indicated by inter-UE coordination information

| Row | Field name | Field size (in bits) |
| --- | --- | --- |
| 0 | Providing/requesting indicator | 1 |
| 1 | Resource combination(s) | 2*log2(N ubchannel SLN ubchannel SL + 12N ubchannel SL + 16)+9+Y Where N ubchannel SL is provided by the higher layer parameter sl-NumSubchannel, Y=log2Nrsv_periodwith that Nrsv_period is the number of entries in the higher layer parameter sl-ResourceReservePeriodList, if higher layer parameter sl-MultiReserveResoure is configured; Y=0 otherwise. |
| 2 | First resource location(s) | [2]*log2(255) |
| 4 | Reference slot location | 10+log2(10∙2μ) Where μ is 0, 1, 2, 3 for SCS of 15kHz, 30kHz, 60kHz, 120kHz, respectively. |
| 5 | Resource set type | 1 |
| 6 | Lowest subchannel indices for the first resource location of each TRIV | [2]*log2NsubchannelSL where NsubchannelSL is provided by the higher layer parameter sl-NumSubchannel |

| Company | Yes or no | Comments |
| --- | --- | --- |
| NTT DOCOMO | Yes |  |
| Vivo | See comment | The size for first resource location is log2(255) |
| LGE | Yes | For progress, we can accept it if no additional field will be introduced. |
| OPPO | OK |  |
| Fujitsu | Yes |  |
| Spreadtrum | Yes |  |
| NEC | Yes |  |
| xiaomi | Yes |  |
| Fraunhofer | Yes, with comment | We agree with the content of the SCI format 2-C. For “Lowest subchannel indices for the first resource location of each TRIV”, it can be [N-1]*log2(255), where N is dependent on proposal 3-8. |
| ETRI | Yes |  |
| Huawei, HiSilicon | See comments | It isn’t necessary to introduce discussions that can be avoided by handling the next update to the 212 editor CR. It’s the whole point of the CR process after 107bis-e. If draft proposal 3-8 is agreed (i.e., N=2), then it is feasible to include row 6. On another point, if the above fields are included in SCI 2C and N=2, the current SCI size is 130 bits. So it’s feasible and quite easy to include another 1 bit field to indicate the actual number of resource combinations conveyed by this SCI 2C. We assume this is more typical way than FL’s suggestion that “different resource combinations indicate the same set of resources”. So we suggest adding the following row: Row; Field name; Field size (in bits). …; …; …. 7; Actual number of resource combinations; 1. Btw: it seems row index needs to be re-ordered since row index 3 is not used. |
| Intel | Yes |  |
| Apple |  | We can accept the proposal for the sake of progress. Huawei’s suggestion is also fine to us. |
| Samsung | No | At first, we do not support to indicate the lowest subchannel index for the first resource location of each TRIV since its benefit was not clearly investigated considering trade-off between signaling overhead and performance. In addition, for the suggested fields, it is OK with one change for the first locations only indicate that of second TRIV. The first resource location of the first TRIV can be that of the reference slot location. |
| Ericsson | Yes |  |
| Futurewei | Comment | We prefer not to use the first resource location. We may need to discuss this with upper bound of N for SCI-2C. If the bit-size is ok for SCI 2-C is ok with agreed N, we can accept it. |
| Nokia, NSB | Yes |  |
| Qualcomm | Yes | Regarding the concern raised by Huawei, HiSilicon: One simple way is to set all the bits in the TRIV to 1 to indicate that the corresponding TRIV field is not indicating any valid resource reservation. This also avoids using additional bits in the SCI 2. |
| Sharp | Yes |  |
| InterDigital | Yes |  |

Q3-10: Do you agree following draft proposal for bit field sizes of a SCI format 2-C for an explicit request?

FL’s observation of 2nd email discussion: - Support: DCM, Panasonic, ETRI, InterDigital, LGE, Qualcomm, Futurewei, Sharp, Spreadtrum, ZTE, Fujitsu, NEC, Samsung, xiaomi, Ericsson, CATT, Fraunhofer, Huawei, Intel, MediaTek (20) - Not support: Apple, vivo, Nokia, (3) - Latency bound is indicated by an explicit request: Apple, - Modify the definition of ending time of a resource selection window: vivo, Nokia,

Draft proposal 3-10:

- For Scheme 1, each bit field size of a SCI format 2-C for an explicit request for inter-UE coordination information is given by following table:

| Row | Field name | Field size (in bits) |
| --- | --- | --- |
| 0 | Providing/requesting indicator | 1 |
| 1 | Priority | 3 |
| 2 | Number of subchannels | log2(N subChannel SL) Where N subChannel SL is provided by the higher layer parameter sl-NumSubchannel |
| 3 | Resource reservation period | Y Where Y=log2Nrsv_periodwith that Nrsv_period is the number of entries in the higher layer parameter sl-ResourceReservePeriodList, if higher layer parameter sl-MultiReserveResoure is configured; Y=0 otherwise. |
| 4 | Resource selection window location | 210+log2(10∙2μ) Where μ is 0, 1, 2, 3 for SCS of 15kHz, 30kHz, 60kHz, 120kHz, respectively. |
| 5 | Resource set type | 1 bit if determineResourceSetTypeScheme1 is set to ‘UE-B’s request’, otherwise, 0 bit |

| Company | Yes or no | Comments |
| --- | --- | --- |
| NTT DOCOMO | Yes |  |
| Vivo | See comment | We are not sure whether it has been agreed or not, both starting and ending time of selection window are indicated by DFN/slot index. The prior agreement is not clear. We understand only one of starting or ending time is indicated by DFN/slot index. While the other is indicated in the form of offset, e.g., 10 bits offset to indicate from 0-800 logical slots (i.e., 100ms) |
| LGE | Yes | We have explicit agreement that the ending time of a resource selection window location is provided by DFN index and slot index. Violating the existing agreement should be avoided. - Agreement: - For Scheme 1, when the inter-UE coordination information transmission is triggered by UE-B’s explicit request, - Starting/Ending time locations of resource selection window is provided by UE-B’s explicit request - Starting/Ending time locations of resource selection window is a form of combination of DFN index and slot index |
| OPPO | yes |  |
| Fujitsu | Yes |  |
| Spreadtrum | Yes |  |
| NEC | Yes |  |
| xiaomi | Yes |  |
| Fraunhofer | Yes |  |
| ETRI | Yes |  |
| Huawei, HiSilicon | Yes | Though, it isn’t necessary to introduce discussions that can be avoided by handling the next update to the 212 editor CR. It’s the whole point of the CR process after 107bis-e. |
| Intel | Yes |  |
| Apple |  | Since RAN 2 is discussing the latency bound of IUC transmission, we could add a note: “Note: FFS field related to latency bound of IUC transmission.” Then, we are fine with the proposal for progress. |
| Samsung | Yes | Even though 4 (Resource selection window location) is not our preference, we can accept for progress |
| Ericsson | Yes |  |
| Futurewei | Yes |  |
| Nokia, NSB | Comment | Add “Resource combination(s)” field in SCI 2-C used for explicit request. When the request is sent using SCI 2-C, that SCI 2-C has to be padded with zeros until its payload size is equal to the payload size of SCI 2-C used for transmitting IUC information. Instead of using padding, these bits can be exploited to convey preferred resources (using the “Resource combination(s)” field in SCI 2-C) to UE-A to increase IUC reception reliability. This is especially important since the IUC reception at UE-B may otherwise be interfered by a data transmission from a UE that is hidden from UE-A’s perspective. |
| Qualcomm | Yes |  |
| Sharp | Yes |  |
| InterDigital | Yes |  |

Q3-11: Do you agree following draft proposal for bit field sizes of a MAC CE for inter-UE coordination information when both MAC CE and a SCI format 2-C are used? Please the proponents of 1st sub-bullet clarify the technical reason why it is needed.

FL’s observation of 2nd email discussion: - Support: Panasonic, ETRI, InterDigital, LGE, Futurewei, CMCC, Spreadtrum, ZTE, Fujitsu, Samsung, xiaomi, Ericsson, CATT, Fraunhofer, MediaTek (15) - Not support: DCM, Apple, OPPO, vivo, Huawei, (5) - Remove exception part: DCM, Apple, OPPO, Huawei, (4) - Remove first resource location for first TRIV: vivo, (1)

Draft proposal 3-11:

- For Scheme 1, when both SCI format 2-C and MAC CE are used as the container of inter-UE coordination information, the same bit field size for inter-UE coordination information in a SCI format 2-C is applied to MAC CE except for first resource location(s)

  - Bit field size of the first resource location(s) on MAC CE is
2*log2(X) where X is provided by the (pre)configured maximum value of slot offset for the case when MAC CE only is used as a container of inter-UE coordination information

| Company | Yes or no | Comments |
| --- | --- | --- |
| NTT DOCOMO | Yes | We understand the intention: for the case when MAC CE only is used. Then we are fine with the proposal. |
| vivo | See comment | The size for first resource location is log2(X) |
| LGE | Yes |  |
| OPPO | NO | The sub-bullet should be removed, we did not see the benefit to use different X for MAC CE, moreover it is conflict with following agreement. Agreement For a slot offset that is (pre)configured to indicate the first resource location of each TRIV with respect to a reference slot, - Granularity of the slot offset is 1 logical slot - (Pre)configured maximum value of the slot offset is up to 8000 - When both SCI format 2-C and MAC CE are used as the container of inter-UE coordination information, the maximum value of the slot offset is 255 |
| Fujitsu | Yes |  |
| Spreadtrum | Yes |  |
| NEC | Yes |  |
| xiaomi | Yes with comment | The subbullet seems to be contrary to the main bullet. In main bullet it is said “when both SCI format 2-C and MAC CE are used as the container”, but in subbullet it is said “when MAC CE only is used as a container”. The subbulet should be at the same level of main bullet. |
| ETRI | Yes |  |
| Huawei, HiSilicon | Leave MAC CE details up to RAN2 | We feel the current proposal with the newly added sub-bullet is over-designing in RAN1. RAN1 does not need to spend time discussing SL MAC-CE design, RAN1 can fully reply on RAN2 to discuss/decide them, and RAN2 may need to perform other design such as alignment of field sizes that make this less relevant to spend time on for RAN1. Draft proposal 3-13 is enough for RAN2 to know the contents/sizes of the MAC-CE. So we suggest the following red changes: == Draft proposal 3-11: - For Scheme 1, when both SCI format 2-C and MAC CE are used as the container of inter-UE coordination information, the same bit field size for inter-UE coordination information in a SCI format 2-C is applied to MAC CE except for first resource location(s) - Bit field size of the first resource location(s) on MAC CE is 2*log2(X) where X is provided by the (pre)configured maximum value of slot offset for the case when MAC CE only is used as a container of inter-UE coordination information - Details are up to RAN2, e.g., the field sizes of first resource location(s) in MAC CE |
| Apple | no | If IUC is carried only in MAC CE, then MAC CE may have a field to indicate the number N of combinations of (TRIV, FRIV, periodicity) in MAC CE. If IUC is carried in both SCI 2-C and MAC CE, will the MAC CE have a field to indicate the number N of combinations? If so, what is the field size for that? We prefer a unified design of MAC CE in the above two cases. In other words, MAC CE always has a field to indicate the number N of combinations. Then, the indication of number N of combinations is not in SCI 2C, but in MAC CE. If the indication of number N of combinations in MAC CE is still under RAN2 discussion (as in proposal 3-12), then we may add a note: “FFS on the indication of the value of N of combinations”. |
| Samsung | Yes |  |
| Ericsson | Yes |  |
| Futurewei | yes | We are ok with the proposal |
| Nokia, NSB | Comment | The subbullet probably contradicts our earlier agreement, as pointed out by OPPO. On the other hand, following the subbullet will probably simplify implementation, since the MAC CE can then be decoded without having to keep track of whether SCI format 2-C was used or not. If we agree the subbullet, then there is a potential problem if we allow maxSlotOffsetTRIVScheme1 to take values less than 255 – then it can happen that 2nd stage SCI can indicate resources which MAC CE cannot, which would contradict an earlier agreement. |
| Qualcomm | Yes | This ensures that the MAC CE size remains the same regardless whether SCI 2-C is used or not and addresses the concerns raised during GTW. |
| Sharp |  | Same view as Huawei |
| InterDigital | Yes |  |

Q3-13: Do you agree following draft proposal for bit field sizes of a MAC CE for inter-UE coordination information when only MAC CE is used?

FL’s observation of 2nd email discussion: - Support: DCM, Apple, Panasonic, ETRI, InterDigital, LGE, Qualcomm, Spreadtrum, ZTE, Fujitsu, NEC, OPPO, vivo, xiaomi, CATT, Fraunhofer, Huawei, Nokia, Intel, MediaTek (20) - Not support: Futurewei, Samsung, Ericsson, (3) - Add indicator to indicate N value: Futurewei, (1) - RAN2 check is needed to have variable size MAC CE: Samsung, (1) - 0 bit for First resource location for first TRIV: Samsung, (1) - Up to RAN2 decision: Ericsson, (1) - Comments: - 0 bit for resource reservation period if periodic reservation is disabled in the pool: Qualcomm, Nokia,

Draft proposal 3-13:

- For Scheme 1, when MAC CE only is used as the container of inter-UE coordination information, each bit field size for inter-UE coordination information is given by following table from RAN1’s perspective, and RAN1 understands that the maximum value of N resource combinations to be conveyed in inter-UE coordination information is bounded so that the total payload size of inter-UE coordination information leads not to exceed the size of TB including the MAC CE

  - Details (e.g., whether/how to separately indicate the value of N in the inter-UE coordination information, how to put the following fields into MAC CE and the related field sizes in MAC CE) are up to RAN2

| Row | Field name | Field size (in bits) |
| --- | --- | --- |
| 0 | Providing/requesting indicator | 1 |
| 1 | Resource combination(s) | N*log2(N subChannel SLN subChannel SL + 12N subChannel SL + 16)+9+Y Where N subChannel SL is provided by the higher layer parameter sl-NumSubchannel, Y=log2Nrsv_periodwith that Nrsv_period is the number of entries in the higher layer parameter sl-ResourceReservePeriodList, if higher layer parameter sl-MultiReserveResoure is configured; Y=0 otherwise. |
| 2 | First resource location(s) | N*log2(X) Where X is provided by the (pre)configured maximum value of slot offset for the case when MAC CE only is used as a container of inter-UE coordination information |
| 4 | Reference slot location | 10+log2(10∙2μ) Where μ is 0, 1, 2, 3 for SCS of 15kHz, 30kHz, 60kHz, 120kHz, respectively. |
| 5 | Resource set type | 1 |
| 6 | Lowest subchannel indices for first resource location(s) | N*log2NsubchannelSL Where NsubchannelSL is provided by the higher layer parameter sl-NumSubchannel. |

| Company | Yes or no | Comments |
| --- | --- | --- |
| NTT DOCOMO | Yes |  |
| Vivo | Yes |  |
| LGE | Yes |  |
| OPPO | yes |  |
| Fujitsu | Yes |  |
| Spreadtrum | Yes |  |
| NEC | Yes |  |
| xiaomi | Yes |  |
| Fraunhofer | Yes |  |
| ETRI | Yes |  |
| Huawei, HiSilicon | Yes | Field name in row 6 needs to be aligned with that in draft proposal 3-9 Btw: it seems row index needs to be re-ordered since row index 3 is not used. |
| Intel | Yes |  |
| Apple | Yes |  |
| Samsung | No | As we responded Draft proposal 3-9, we are OK with one change for the first locations only indicate that of second and later TRIVs. The first TRIV can be that of the first location. |
| Ericsson |  | We can accept this proposal. |
| Futurewei | Yes | Given the majority support, we are ok to accept it and leave the indication of N value to RAN2. |
| Nokia, NSB | Yes |  |
| Qualcomm | Yes |  |
| InterDigital | Yes |  |

Q3-15: Do you agree following draft proposal for cast type of inter-UE coordination information when a SCI format 2-C is used?

FL’s observation of 2nd email discussion: - Unicast: DCM, Panasonic, ETRI, InterDigital, LGE, Qualcomm, CMCC, Sharp, Spreadtrum, ZTE, Fujitsu, NEC, OPPO, Samsung, vivo, CATT, Fraunhofer, Huawei, Intel, MediaTek (20) - Unicast and groupcast by using ID setting: Futurewei, (1) - All cast type: Ericsson, Nokia, (2)

Draft proposal 3-15:

- For inter-UE coordination information transmission, a SCI format 2-C  can be used only when its cast type is unicast regardless of whether it is multiplexed with other data or not

| Company | Yes or no | Comments |
| --- | --- | --- |
| NTT DOCOMO | Yes |  |
| Vivo | Yes |  |
| LGE | Yes | In our understanding, cast type indicator is made based RAN2’s decision. If the cast type indicator is not present, it would be necessary to tie with a certain cast type as if a SCI format 2-B is tied with groupcast only. |
| OPPO | yes |  |
| Fujitsu | Yes |  |
| Spreadtrum | Yes |  |
| NEC | Yes |  |
| Fraunhofer | No | As mentioned by Nokia and Futurewei in the previous round, if the destination ID can be a groupcast ID, we do not see the need to restrict the use of SCI 2-C to unicast alone. |
| ETRI | Yes |  |
| Huawei, HiSilicon | Yes |  |
| Intel | OK | Motivation: standalone SCI format 2C is not supported Proposed rewording: - For inter-UE coordination information transmission, a SCI format 2-C can be used only when its cast type is unicast regardless of whether it is multiplexed with other data or not MAC CE only |
| Samsung | Comment | O.K for the proposal but we suggest to discuss the followings together. When coordination message or coordination request is multiplexed with data, it would be beneficial to use the 2nd SCI for RSAI request rather than MAC CE in the latency aspect. Therefore, we propose: When a resource pool level configuration enables that MAC CE or 2nd SCI are used as the container for RSAI message, - If N<= 3 and RSAI is multiplexed with data, only 2nd SCI is used for container. - Otherwise, only MAC-CE is used for container. When a resource pool level configuration enables that MAC CE and 2nd SCI are used as the container for RSAI request, - If RSAI request is multiplexed with data, only 2nd SCI is used for container, Otherwise, only MAC-CE is used for container. |
| Futurewei | Comments | We do not see the reason why groupcast cannot be supported. |
| Nokia, NSB | No, all cast types | Broadcast can be supported by setting Destination ID to a broadcast identity. Likewise, groupcast can be supported when HARQ feedback is not requested, by setting Destination ID to a groupcast ID. There may be cases where broadcasting SCI 2-C is beneficial given its lower latency. For example, if UE-A has some non-preferred resource(s) in the very near future due to Condition 1-B-1 Option 2, then, due to latency, SCI format 2-C may be the only option to make surrounding UEs aware of those non-preferred resource(s). If UE-A now broadcasts these non-preferred resource(s) using SCI format 2-C, of course some surrounding UEs won't be able to decode it - but that is a better outcome than not being able to inform any surrounding UEs except those with which UE-A currently has a unicast link. Moreover, if SCI 2-C is constrained to unicast, but UE-A has groupcast or broadcast data to transmit, UE-A won’t be able to multiplex/piggyback the SCI 2-C with the groupcast/broadcast data, which would be more efficient. |
| Qualcomm | Yes |  |
| Sharp | Yes |  |
| InterDigital | Yes |  |

Q3-16: Do you agree following draft proposal for latency bound of inter-UE coordination information? On the other hand, considering a few companies continue insisting this issue should be handled by RAN1, FL asks these proponents to provide complete proposals in the column of “Comments” in order for other companies to check whether those can be easily agreeable or there is a thing that cannot be covered by RAN2 decision/discussion.

FL’s observation of 2nd email discussion: - Support: DCM, Panasonic, InterDigital, LGE, CMCC, ZTE, Fujitsu, NEC, OPPO, Samsung, Ericsson, Huawei, MediaTek (13) - Not support: Apple, Futurewei, vivo, xiaomi, CATT, Intel, (6)

Draft conclusion 3-16:

- For latency bound of inter-UE coordination information transmission, RAN1 relies on RAN2’s decision as per LS R1-2200880 from RAN2

| Company | Yes or no | Comments |
| --- | --- | --- |
| NTT DOCOMO | Yes |  |
| LGE | Yes |  |
| OPPO | yes |  |
| Fujitsu | Yes |  |
| NEC | Yes |  |
| xiaomi | no | From our understanding, RAN2 would only define the timer for the delay bound. Issues on how the bound would have impact on resource (re)selection and how the bound is determined can only be discussed in RAN1. |
| Huawei, HiSilicon | Yes | As shown in RAN2’s summary R2-2203159 (see “Issue 4. Timer to handle latency bound for inter-UE coordination”), RAN2 already had quite in-depth discussions on the latency bound issue and will continue discussing it. For example, as shown in R2-2203159, RAN2 raised 8 questions and come up with 15 proposals (see Proposal 4-x in R2-2203159)! RAN1 should avoid such duplicated discussions to save time. |
| Intel | No |  |
| Apple |  | We could let RAN2 to determine latency bound of IUC transmission, if it is the majority companies’ view. However, this latency bound is related to SCI 2-C design, we should ensure the SCI 2-C design is not closed before RAN2 makes final decision. |
| Samsung | Yes |  |
| Ericsson | Yes |  |
| Futurewei | No | We prefer to discuss the latency bound or deadline of UE-A transmission of coordination in RAN1 which is easier than RAN2 as sensing, resource selection, transmission, the related processes having timing impact, are all done in PHY. RAN2 have their limitations on PHY timing parameters. We can specify the latency bound for both request-based and condition based IUE, while RAN2 only considers request-based IUC. |
| Nokia, NSB | yes |  |
| Qualcomm | Yes | We agree with Xiaomi on the point that this can have impact on the resource selection procedure. A UE which has very tight time delay for inter-UE coordination resource section may cause excessive collisions with other data transmissions. |
| InterDigital | Yes |  |

Q3-17: Which alternative is supported for defining sensing window for determining the set of resources?

FL’s observation of 2nd email discussion: - Support: DCM, Apple, Panasonic, ETRI, InterDigital, LGE, Sharp, Spreadtrum, ZTE, Fujitsu, NEC, OPPO, Samsung, xiaomi, Ericsson, CATT, Nokia, (17) - Not support: Qualcomm, Futurewei, CMCC, vivo, Huawei, Intel, MediaTek (7) - n is the slot where inter-UE coordination information is transmitted: Qualcomm, CMCC, Intel, MediaTek (4) - Additional margin is needed to ensure inter-UE coordination information transmission before n+T_1-1: Futurewei, (1) - n and remaining PDB are determined by UE-A’s implementation: vivo, (1) - Comments: - Ericsson: Skipping inter-UE coordination information transmission based on sensing status with respect to SL DRX operation. - Huawei: Re-evaluation for the set of resources is supported as per Rel-16 procedures. - Intel: Define restriction on a resource selection window for transmission.

Draft conclusion 3-17:

Alt 1:

For sensing window for determining the set of resources in Scheme 1,

- No further change is supported. Note that the sensing window for determining the set of resources is already derived based on the location n+T_1 and n+T_2 used for determining the set of resources in TS38.214 section 8.1.4, i.e., sensing window is defined by the range of slots [ (n+T_1) - T_0 - T_1 determined by UE-A, (n+T_1) - T_proc,0 - T_1 determined by UE-A ).

  - For inter-UE coordination information triggered by UE-B’s explicit request, n+T_1 and n+T_2 are provided by the request

  - For inter-UE coordination information triggered by a condition other than explicit request reception, n+T_1 and n+T_2 are determined by UE-A’s implementation

Alt 2:

For sensing window for determining the set of resources in Scheme 1,

- Sensing window for determining the set of resources is derived based on the location n’+T’_1 and n’+T’_2 where n’ is the slot in which inter-UE coordination information generation is triggered and T’_1/T’_2 are determined by UE-A, i.e., sensing window is defined by the range of slots [ n’ - T_0, n’ - T_proc,0 ).

  - (n’+T1’) and (n’+T2’) are determined by UE-A subject to the following conditions:

    - If inter-UE coordination information is triggered by an explicit request,

      - (n+T_1) ≤ (n'+T’_1)

      - (n'+T’_2) ≤ (n+T_2)

    - If inter-UE coordination information is triggered by a condition other than explicit request reception,

      - n = n’

    - T2,min ≤ (T’_2-T’_1)

where

    - (n+T_1) – Start slot of resource selection window for determining the set of resources

    - (n+T_2) – End slot of resource selection window for determining the set of resources

    - (n’+T’_1) – Start slot of resource selection window used for inter-UE coordination information transmission

    - (n’+T’_2) – End slot of resource selection window used for inter-UE coordination information transmission

| Company | Alt | Comments |
| --- | --- | --- |
| NTT DOCOMO | Alt 1 |  |
| Vivo | Alt.1 |  |
| LGE | Alt 1 or Alt 2 | For Alt 2, it might be difficult to specify the association between a resource selection window for determining the set of resources and a source selection window for its transmission in the specification. |
| OPPO | Alt 1 | Firstly, it should be clarified that the value of T_1 is up to UE-A implementation. Given that, it seems Alt 1 and Alt 2 are fundamentally the same, i.e., the meaning of n in Alt 1 is where UE-A starts generating the information based on sensing results. For Alt 2, the sensing window is derived from [n’+T’_1 and n’+T’_2], this may no exist if inter-UE coordination information is transmitted with other data. Furthermore, RAN1 had following agreement last meeting, we do not think more discussion on [n’+T’_1 and n’+T’_2] is necessary. Agreement - For sidelink transmission carrying inter-UE coordination information in Scheme 1, - UE-A performs its resource (re)selection according to the same procedure in TS 38.214 Section 8.1.4 to transmit the inter-UE coordination information to UE-B. - For sidelink transmission carrying request in Scheme 1, - UE-B performs its resource (re)selection according to the same procedure in TS 38.214 Section 8.1.4 to transmit the request for the inter-UE coordination information to UE-A if UE-B performs sensing/resource exclusion. Otherwise, at least UE-B can perform random selection - Note: RAN1 does not pursue specific enhancement of Rel-17 resource (re)selection for the transmission of inter-UE coordination information and its request. |
| Fujitsu | Alt 1 |  |
| Spreadtrum | Alt 1 |  |
| NEC | Alt.1 |  |
| xiaomi | Alt1 |  |
| Fraunhofer | Alt 1 | At this stage, we prefer to not to carry out any further changes. |
| ETRI | Alt 1 |  |
| Huawei, HiSilicon | Alt 1 with supporting re-evaluation | We raised some technical concern in previous round, i.e., in both Alt1 and Alt 2, UE-A will not use the latest sensing results to determine the set of resource and thus inaccurate. This technical issue still stands for both Alt 1 and Alt 2. For example, in Alt 1, assume UE-A transmits IUC at slot n+T_1+200, i.e., far away from n+T_1 (this is possible if “n+T2” is large and because resource is selected randomly). Based on the current draft conclusion, the sensing results between the time window [(n+T_1) - T_proc,0 - T_1 determined by UE-A, n+T_1+200 – Tproc,0 – Tproc,1) will not be used to determine the set of resources. This will be very inaccurate since the latest sensing results are not used. In Rel-16, re-evaluation mechanism is introduced to ensure UE-A can update the resource using the latest sensing results before transmitting. We think a similar mechanism is needed. So we suggest the following red changes. == Alt 1: For sensing window for determining the set of resources in Scheme 1, - No further change is supported. Note that the sensing window for determining the set of resources is already derived based on the location n+T_1 and n+T_2 used for determining the set of resources in TS38.214 section 8.1.4, i.e., sensing window is defined by the range of slots [ (n+T_1) - T_0 - T_1 determined by UE-A, (n+T_1) - T_proc,0 - T_1 determined by UE-A ). - For inter-UE coordination information triggered by UE-B’s explicit request, n+T_1 and n+T_2 are provided by the request - For inter-UE coordination information triggered by a condition other than explicit request reception, n+T_1 and n+T_2 are determined by UE-A’s implementation - Re-evaluation for the set of resources is supported as per Rel-16 procedures. |
| Intel | Alt.2 |  |
| Apple | Alt 1 |  |
| Samsung | Alt1 | With Alt 1, do we need to have this conclusion? |
| Ericsson | Alt. 1 |  |
| Futurewei | Comments | We prefer Alt 2 direction in general. However, we think the coordination information should be transmitted before n+T_1. UE-B needs to have a certain processing time for resource selection. Including one slot for transmission, UE-A has to transmit coordination by n+T_1-Tproc,1-1. Therefore, for request based IUC, it should be n’+T’_1, n’+T’_2<=n+T1-Tproc,1-1. On the other hand, we do not think it is necessary to specify the RSW for UE-A’s resource selection of transmitting coordination information. Instead, we only specify a deadline on sensing for generate coordination information. Based on above discussions, the sensing ending time can be restricted by n+T1-Tproc,1-1-Tr or n+T1-Tproc,1-Tr include the 1 slot is included. If considering T2,min for resource selection at UE-A, we then specify Tr>=T2,min +1 . Then the rest can be up-to UE-implementation. Above specification can be applied to IUC triggered by condition, just n+T1, n+T2 for coordination generation are up-to UE implementation or from RRC signaling upon RAN2 decision. So in summary Sensing ends the latest by n+T1-Tproc,1-Tr where Tr>=T2,min+1. Coordination information is transmitted by n+T1-Tproc,1-1. |
| Nokia, NSB | Alt 1 |  |
| Qualcomm | Alt. 1 with modifications | We propose the following changes to Alt. 1 to make sure that the UE always report the set of resource using the latest sensing information when triggered by a condition other than explicit request: Alt 1: For sensing window for determining the set of resources in Scheme 1, - No further change is supported. Note that the sensing window for determining the set of resources is already derived based on the location n+T_1 and n+T_2 used for determining the set of resources in TS38.214 section 8.1.4, i.e., sensing window is defined by the range of slots [ (n+T_1) - T_0 - T_1 determined by UE-A, (n+T_1) - T_proc,0 - T_1 determined by UE-A ). - For inter-UE coordination information triggered by UE-B’s explicit request, n+T_1 and n+T_2 are provided by the request - For inter-UE coordination information triggered by a condition other than explicit request reception, n+T_1 and n+T_2 are determined by UE-A’s implementation with n ≥ n’, where n’ is the slot in which inter-UE coordination information generation is triggered. |
| Sharp | Alt 1 |  |
| InterDigital | Alt 1 |  |

Q3-18: Do you agree following draft conclusion for UE-A’s behavior of determining a priority value of inter-UE coordination information transmission triggered by a condition other than explicit request reception if the priority value is not (pre)configured?

FL’s observation of 2nd email discussion: - Support: DCM, Apple, Panasonic, ETRI, InterDigital, LGE, Qualcomm, CMCC, Sharp, Spreadtrum, ZTE, NEC, OPPO, Samsung, vivo, xiaomi, Ericsson, CATT, Intel, MediaTek (20) - Not support: Futurewei, Fraunhofer, Huawei, Nokia, (4) - Up to UE-A’s implementation: Futurewei, Fraunhofer, Huawei, (3) - The priority value is the same as the priority value indicated by other UE’s SCI that is used to determine the non-preferred resource set: Nokia, (1)

Draft conclusion 3-18:

No further decision is necessary for UE-A’s behavior of determining a priority value of inter-UE coordination information transmission triggered by a condition other than explicit request reception if the priority value is not (pre)configured

- It is up to RAN2 whether/how to additionally handle this case

| Company | Yes or no | Comments |
| --- | --- | --- |
| NTT DOCOMO |  | The sub-bullet is unnecessary. (Pre-)configuration shall be provided for the feature. For the case where (pre-)configuration is not provided, the behavior is undefined, which is the original intention in our understanding. |
| Vivo | Yes |  |
| LGE | Yes |  |
| OPPO | yes |  |
| Fujitsu | Yes | We can accept the proposal. |
| Spreadtrum | Yes |  |
| NEC | Yes |  |
| xiaomi | Yes |  |
| Fraunhofer |  | We are fine with the main bullet, and prefer that the priority value be left up to UE implementation. |
| ETRI | Yes |  |
| Huawei, HiSilicon | Yes | For the sake of progress, we can live with this. The sub-bullet is necessary to ensure the specification is complete. |
| Intel | NO | Please remove sub-bullet or clarify it as follows - It is up to RAN2 to set default value for priority value |
| Apple | Yes |  |
| Samsung | Yes | BTW, do we need to have this conclusion? |
| Ericsson | Yes |  |
| Futurewei | Comment | We think it is not necessary to leave the issue to RAN2. It is a minor issue in RAN1. UE-A determining the priority value by its implementation has a flexibility for efficient design by the UE vendors. However, if majority are ok, we are also fine to leave it to RAN2 although we do not prefer this conclusion. |
| Nokia, NSB | No | A decision is necessary – and this aspect should be specified in the standard. If the prioritization is left up to UE-A implementation, this leaves the door open for UE-As to abuse their freedom by setting the highest priority to their IUC transmissions, and in so doing unfairly promote their own data transmissions when multiplexed with the IUC. Our preference is Option 5: The priority value is the same as the priority value indicated by other UE’s SCI that is used to determine the non-preferred resource set. |
| Qualcomm | Yes | We are also okay with the updated wording proposed by Intel. |
| Sharp | Yes |  |
| InterDigital | Yes |  |

Q3-19: Do you agree following draft conclusion for cast type(s) of inter-UE coordination information triggered by a condition other than explicit request reception? FL observed that companies views are still divergent, so it is suggested that the cast type is determined by UE-A’s implementation.

FL’s observation of 2nd email discussion: Draft conclusion: - Only cast type(s) available at UE-A for other data transmission can be used for cast type(s) for the inter-UE coordination information transmission triggered by a condition other than explicit request reception - Note: it is applied to both when the inter-UE coordination information is multiplexed with other data and when the inter-UE coordination information is not multiplexed with other data - Note: UE-A determines the cast type(s) of inter-UE coordination information by its implementation among the available cast type(s) - Support: Panasonic, InterDigital, LGE, Sharp, Spreadtrum, ZTE, Fujitsu, NEC, OPPO, xiaomi, Ericsson, CATT, (12) - Not support: Apple, Qualcomm, Futurewei, CMCC, Samsung, vivo, Fraunhofer, Huawei, Intel, Nokia, (10) - No need to have a conclusion: Apple, Qualcomm, Futurewei, Fraunhofer, Huawei, (5) - Tie with Condition for determining non-preferred resource set: CMCC, vivo, Nokia, (3) - Based on (pre)configuration: Samsung, (1) - Based on data multiplexing: Intel, (1)

Draft conclusion 3-19:

- For inter-UE coordination information transmission triggered by a condition other than explicit request reception, UE-A determines its cast type by implementation

| Company | Yes or no | Comments |
| --- | --- | --- |
| NTT DOCOMO | Yes |  |
| vivo | See comment | For broadcast IUC, no need to convey non-preferred resource determined based on Condition 1-B-2, i.e., HD issue |
| LGE | Yes | For progress, we can accept it. |
| OPPO | Yes with comments | Only for non-preferred resource, preferred resource can only be transmitted with unicast. |
| Fujitsu | Yes |  |
| Spreadtrum | Yes |  |
| NEC | Yes |  |
| xiaomi | Yes |  |
| Fraunhofer | Yes |  |
| Huawei, HiSilicon | Yes | We think RAN1 does not need to discuss such issue. Such discussions should be better taken in RAN2. If RAN1 really wants to take a conclusion, we can live with the current proposal for the sake of progress. |
| Intel | No | It is unclear which cases are considered with or w/o multiplexing with data. If w/o multiplexing it should be broadcast. For the case with multiplexing it should be aligned with cast type. |
| Apple |  | We do not see any RAN1 spec. impact of this proposal. |
| Samsung | No | We think that deciding cast type is not RAN1 scope. Our suggestion has higher layer impact but we think this is the most desirable way to conclude this issue. So, we suggest the followings: - Groupcast/Broadcast for non-preferred resource set, - A Groupcast set for the transmission of condition-based RSAI information to can be (pre-)configured, if not (pre-)configured, the condition-based RSAI information is broadcast to surrounding UEs. - The period of the condition-based RSAI information is (pre-)configured to one of [{100, 500, 1000, 2000}] Unicast: Only when UE-A has data send to UE-B, and the inter-UE co-ordination information is included in the same SL transmission with the data |
| Ericsson |  | What is the difference between agreeing and not agreeing? Unless some restriction is specified, of course. But that looks unlikely. |
| Futurewei | Yes | We support the proposal. |
| Nokia, NSB | Yes |  |
| Qualcomm | Yes |  |
| Sharp | Yes |  |
| InterDigital | Yes |  |

Q3-20: Which alternative is supported for UE-B’s behaviors when UE-B receives multiple resource sets from the same UE-A? FL observed that even proponents of Alt 1 have slight different details, so if it is difficult to make a consensus for Alt 1, Alt 2 could be a way to move forward this issue.

FL’s observation of 2nd email discussion (for UE-B’s behavior when UE-B receives multiple preferred resource sets from the same UE-A): > Option 1: UE-B uses the latest received preferred resource set from the same UE-A for its resource selection for a TB to be transmitted to the UE-A. > Option 3: UE-B does not expect to receive more than one preferred resource sets from the same UE-A for its resource selection for the same TB transmission to be transmitted to the UE-A. - Option 1: DCM, Panasonic, InterDigital, Qualcomm, Futurewei, Sharp, Spreadtrum, Fujitsu, NEC, OPPO, Samsung, Ericsson, Fraunhofer, Nokia, (14) - Option 3: Apple, CMCC, NEC, xiaomi, (4) - Option 1 and 3: Futurewei, CATT, MediaTek (3) - Others: Intel, Huawei, (2) - Comments: - vivo: Clarification on clear rule to associate a given TB with corresponding inter-UE coordination information - Huawei: Option 1 and 3 may have technical issues when UE-A sends a subset of preferred resource set in a time FL’s observation of 2nd email discussion (for UE-B’s behavior when UE-B receives multiple non-preferred resource sets from the same UE-A): > Option 1: UE-B uses the latest received non-preferred resource set from the same UE-A for its resource selection for a TB to be transmitted to the UE-A. > Option 3: UE-B determines a final non-preferred resource set by combining all the received non-preferred resource sets from the same UE-A. UE-B uses the final non-preferred resource set for its resource selection for a TB to be transmitted to the UE-A. - Option 1: InterDigital, Futurewei, Spreadtrum, Fujitsu, Samsung, Nokia, MediaTek (7) - Option 3: DCM, Apple, Panasonic, LGE, Qualcomm, CMCC, Sharp, NEC, OPPO, vivo, Ericsson, Fraunhofer, (12) - Option 1 and 3: CATT, (1) - Others: Intel, Huawei, (2) - Comments: - Huawei: Option 1 and 3 may have technical issues when UE-A sends a subset of non-preferred resource set in a time FL’s observation of 2nd email discussion (for UE-B’s behavior when UE-B receives both a single preferred resource set and a single non-preferred resource set from the same UE-A): > Option 3: UE-B uses both the received preferred resource set and non-preferred resource set from the same UE-A for its resource selection for a TB to be transmitted to the UE-A. > Option 4: UE-B does not expect to receive both preferred resource set and non-preferred resource set from the same UE-A for its resource selection for the same TB transmission to be transmitted to the UE-A. - Option 3: DCM, Panasonic, InterDigital, LGE, Futurewei, Spreadtrum, Fujitsu, NEC, OPPO, xiaomi, Ericsson, Fraunhofer, Nokia, Intel, (14) - Option 4: Apple, Qualcomm, CMCC, Sharp, xiaomi, CATT, (6) - Others: Samsung, vivo, Huawei, MediaTek (4) - Comments: - Huawei: Option 1 and 3 may have technical issues when UE-A sends a subset of resource set in a time

Draft proposal 3-20:

Alt 1:

- For UE-B’s behavior when UE-B receives multiple preferred resource sets from the same UE-A,

  - Option 1: UE-B uses the latest received preferred resource set from the same UE-A for its resource selection for a TB to be transmitted to the UE-A.

- For UE-B’s behavior when UE-B receives multiple non-preferred resource sets from the same UE-A,

  - Option 3: UE-B determines a final non-preferred resource set by making union of all the received non-preferred resource sets from the same UE-A. UE-B uses the final non-preferred resource set for its resource selection for a TB to be transmitted to the UE-A.

- For UE-B’s behavior when UE-B receives both a single preferred resource set and a single non-preferred resource set from the same UE-A,

  - Option 3: UE-B uses both the received preferred resource set and non-preferred resource set from the same UE-A for its resource selection for a TB to be transmitted to the UE-A.

Alt 2:

- When UE-B receives multiple inter-UE coordination information from the same UE-A, it is up to UE-B implementation to use one or multiple of them in its resource (re)selection.

| Company | Alt | Comments |
| --- | --- | --- |
| NTT DOCOMO | Alt 1 | If we go with Alt 2, UE-B might not work as intended at UE-A. Then it becomes difficult for UE-A to decide which/what/whether should be transmitted to UE-B. To have same understanding between UE-A and UE-B, certain rule should be defined. |
| vivo | Direction of Alt1 is fine. | For 1st bullet, FFS How to guarantee that the latest IUC can match UE-B’s current TB. For 3rd bullet, UE-B should have flexibility to use either of preferred or non-preferred resource set or both. Especially when UE-B does not performs sensing, UE-B only use the preferred resource set. |
| LGE | Alt 2 | It would be useful to resolve the over-exclusion problem in UE-B’s resource selection procedure considering the received non-preferred resource set(s). |
| OPPO | Alt 1 | Our understanding is that all the received resource sets are within the latency budget to be defined by RAN2. We are also fine with Alt 2 is Alt 1 cannot be agreed. |
| Fujitsu | Alt 2 |  |
| Spreadtrum | Alt 1 | We are also fine with Alt 2. |
| NEC | Alt.1 | Multiple resource set is for better performance at UE-B. Up to UE implementation is a waste of singling exchange. |
| xiaomi | Alt1 |  |
| Fraunhofer | Alt 1 | Agree with DCM that both UE-A and UE-B would need these rules defined for mutual understanding of the IUCs being transmitted/received. |
| Huawei, HiSilicon | Alt 2 | The baseline situation for all these cases is that if there is not consensus for another solution, it is left with no specified UE behavior, i.e. up to UE implementation. Our technical concern on the Options in Alt 1 are not addressed yet, despite multiple rounds of discussion. At this stage, it seems perhaps they cannot be resolved in the scope of the final e-meeting for the WI. After further thinking, we found there could be more issues, which are summarized below: - Alt1, sub-bullet of Option 1: - Some companies mentioned the latest one is more accurate and is thus used. But this may not be true. Because it’s possible that a single IUC information (e.g., SCI 2C) cannot include all the preferred resources at UE-A side, so that UE-A may decide to transmit another IUC information to include another set, i.e., the set of preferred resources in different IUCs can be different or even orthogonal. In this case, use the latest one does not make sense. - Will RAN1 further consider an earliest and latest bound due to the newly introduced idea of “ … latest received …”, which will even have RRC impact - Will UE-B further consider the different priorities of different IUCs from the same UE-A? E.g., assume UE-A1 sends IUC with priority value 1 at slot n, and sends IUC with priority value 8 at slot n+50. The latter IUC is the latest one. However, the former IUC seems to be more important. Which one should UE-B consider? - Alt 1, sub-bullet of 1st Option 3 - If UE-B takes union of the non-preferred resources, the remaining resources in S_A could be very limited, causing RSRP increment and increasing interference. Thus, the performance could be even worse compared with Rel-16. - Some previously received non-preferred resource set may be no longer valid and thus should not be considered. - Alt 1, sub-bullet of 2nd Option 3 - If the same resource is marked as preferred in one IUC and marked as non-preferred in another IUC, what’s the UE-B’s behavior? As FL already mentioned, even proponents of Alt 1 have different details. We feel RAN1 does not have enough time to have very careful technical discussions on each of the options under each case. If Alt 1 is taken, we expect there will be many CRs in maintenance phase to fix the issues we mentioned above or even more issues. For simplicity, we suggest to take a unified solution to handle all the cases, i.e., Alt2. In addition, if Alt 2 is taken, Q3-21 can also be quickly resolved using similar solution, which can save a lot of RAN1 time. |
| Intel | Alt.1 | We can accept Alt.1 but the definition of the “latest received” needs to be clarified. The “latest received” may be a several seconds away. We assume there is no intention to use such feedback. We cannot accept Alt.2 as it diminishes all RAN1 efforts to enable IUC framework and cannot be considered as wayforward. |
| Apple |  | Actually, we are in the middle of Alt 1 and Alt 2. For the first and the last bullet in Alt 1, we think it is up to UE-B implementation. But for the second bullet in Alt 1, we support Option 3 as in Alt 1. |
| Samsung | Neither | A simple solution is that UE-B uses the latest IUC information it receives from UE-A, whether it is preferred or non-preferred. It is not clear why UE-B would use any information rather than the latest. |
| Ericsson | Alt 1 |  |
| Futurewei | Alt 1 | We prefer Alt 1 in general. For non-preferred set, we prefer to use the latest, but ok to accept the union set. |
| Nokia, NSB | Alt 1 |  |
| Qualcomm | Alt. 1 | We can accept Alt. 1 for compromise but we still have a concern for the following case: - UE-B receives both a single preferred resource set and a single non-preferred resource set from the same UE-A. We feel preferred and non-preferred resource set indication should not be enabled in the same pool. |
| Sharp | Alt 2 | We are fine with Alt 2 if details in Alt 1 are controversial. |
| InterDigital | Alt 2 | We agree with FL that Alt 2 may be the best way-forward to conclude this discussion. We are also okay with Alt 1 if majority agree with Alt 1. |

Q3-21: Which alternative is supported for UE-B’s behaviors when UE-B receives multiple resource sets from the different UE-As? FL observed that even proponents of Alt 1 have slight different details, so if it is difficult to make a consensus for Alt 1, Alt 2 could be a way to move forward this issue.

FL’s observation of 2nd email discussion (for UE-B’s behavior when UE-B receives multiple preferred resource sets from the different UE-As): > Draft proposal: - For UE-B’s behavior when UE-B receives multiple preferred resource sets from the different UE-As, - Option 1: UE-B uses each received preferred resource set for its resource selection for a TB to be transmitted to each UE-A providing the preferred resource set. - Support: DCM, Panasonic, InterDigital, LGE, Qualcomm, Futurewei, Sharp, Spreadtrum, Fujitsu, NEC, OPPO, vivo, xiaomi, CATT, MediaTek (15) - Not support: Apple, CMCC, Samsung, Ericsson, Nokia, Fraunhofer, Huawei, Intel, (8) - UE-B uses all received preferred resource set for its resource selection for a TB to be transmitted to any UE: Apple, Samsung, Ericsson, (3) - UE-B uses all received preferred resource set from target RX UEs for its resource selection for a TB to be transmitted to the target RX UEs: CMCC, Nokia, (2) - When UE-B receives multiple inter-UE coordination information from the same UE-A or different UE-As, it is up to UE-B implementation to use one or multiple of them in its resource (re)selection: Huawei, (1) - Subject to aging condition UE-B uses each valid received preferred resource set for its resource selection for a TB to be transmitted: Intel, (1) FL’s observation of 2nd email discussion (for UE-B’s behavior when UE-B receives multiple non-preferred resource sets from the different UE-As): > Draft proposal: - For UE-B’s behavior when UE-B receives multiple non-preferred resource sets from the different UE-As. - Option 1: UE-B determines a final non-preferred resource set by combining all the received non-preferred resource sets from different UE-As. UE-B uses the final non-preferred resource set for its resource selection for TB(s) to be transmitted to these different UE-As providing the non-preferred resource sets. - Support: DCM, Apple, Panasonic, InterDigital, LGE, Futurewei, CMCC, Sharp, Spreadtrum, Fujitsu, NEC, OPPO, Samsung, xiaomi, Ericsson, Intel, Ericsson, vivo, MediaTek (19) - Not support: Qualcomm, CATT, Fraunhofer, Huawei, (4) - UE-B determines a final non-preferred resource set by combining all the received non-preferred resource sets from different UE-As. UE-B uses the final non-preferred resource set for its resource selection for TB(s) to be transmitted to any UE(s): Qualcomm, (1) - Different behaviour across different cast type of inter-UE coordination information transmission: CATT, (1) - When UE-B receives multiple inter-UE coordination information from the same UE-A or different UE-As, it is up to UE-B implementation to use one or multiple of them in its resource (re)selection: Huawei, (1) FL’s observation of 2nd email discussion (for UE-B’s behavior when UE-B receives both a single preferred resource set and a single non-preferred resource set from the different UE-As): > Option 1: UE-B uses the received preferred resource set for its resource selection for a TB to be transmitted to the UE-A providing the preferred resource set. UE-B uses the received non-preferred resource set for its resource selection for a TB to be transmitted to the UE-A providing the non-preferred resource set. > Option 2: UE-B uses both the received preferred resource set and non-preferred resource set from different UE-As for its resource selection for a TB to be transmitted to the UE-A providing the preferred resource set. UE-B uses the received non-preferred resource set for its resource selection for a TB to be transmitted to the UE-A providing the non-preferred resource set. > Option 3: UE-B uses both the received preferred resource set and non-preferred resource set from different UE-As for its resource selection for a TB to be transmitted to any UE > Option 4: UE-B uses all or a subset of the received preferred resource set and non-preferred resource set from different UE-As by its implementation for its resource selection for TB(s) to be transmitted to UE-A(s) providing the preferred resource set or non-preferred resource set - Option 1: InterDigital, LGE, Spreadtrum, NEC, xiaomi, Fraunhofer, (6) - Option 2: DCM, Panasonic, LGE, Qualcomm, Futurewei, OPPO, CATT, Intel, Nokia, MediaTek (10) - Option 3: CMCC, NEC, Samsung, (3) - Option 4: Apple, Sharp, ZTE, Fujitsu, Huawei, (5) - Other: vivo, Ericsson, (2)

Draft proposal 3-21:

Alt 1:

- For UE-B’s behaviour when UE-B receives multiple preferred resource sets from the different UE-As,

  - Option 1: UE-B uses each received preferred resource set for its resource selection for each TB to be transmitted to each UE-A providing the preferred resource set.

- For UE-B’s behaviour when UE-B receives multiple non-preferred resource sets from the different UE-As.

  - Option 1: UE-B determines a final non-preferred resource set by making union of all the received non-preferred resource sets from different UE-As. UE-B uses the final non-preferred resource set for its resource selection for TB(s) to be transmitted to these different UE-As providing the non-preferred resource sets.

- For UE-B’s behaviour when UE-B receives both a single preferred resource set and a single non-preferred resource set from the different UE-As,

  - Option 2: UE-B uses both the received preferred resource set and non-preferred resource set from different UE-As for its resource selection for a TB to be transmitted to the UE-A providing the preferred resource set. UE-B uses the received non-preferred resource set for its resource selection for a TB to be transmitted to the UE-A providing the non-preferred resource set.

Alt 2:

- When UE-B receives multiple inter-UE coordination information from the different UE-As, it is up to UE-B implementation to use one or multiple of them in its resource (re)selection.

| Company | Alt | Comments |
| --- | --- | --- |
| NTT DOCOMO | Alt 1 | Same comment as for proposal 3-20. |
| vivo | Direction of Alt1 is fine. | For 3rd bullet, UE-B should have flexibility to use either of preferred or non-preferred resource set or both. Especially when UE-B does not performs sensing, UE-B only use the preferred resource set. |
| LGE | Alt 2 | It would be useful to resolve the over-exclusion problem in UE-B’s resource selection procedure considering the received non-preferred resource set(s). |
| OPPO |  | Preferred resource set is transmitted by unicast, it does not make sense to use preferred resource set from another UE-A, Alt 1 should be adopted for this case. For other cases we are fine to leave them to UE-B implementation. |
| Fujitsu | Alt 2 |  |
| Spreadtrum | Alt 1 |  |
| NEC | Alt.1 |  |
| xiaomi | Alt1 |  |
| Fraunhofer | Alt 1, with comments | For the second bullet, we do not agree. It is unclear why non-preferred resources from UE-A1 that is diagonally located to UE-A2 would be relevant for the selection of resources for a transmission by UE-B to both UE-As. |
| Huawei, HiSilicon | Alt 2 | The baseline situation for all these cases is that if there is not consensus for another solution, it is left with no specified UE behavior, i.e. up to UE implementation. Our technical concern on the Options in Alt 1 are not addressed yet, despite multiple rounds of discussion. At this stage, it seems perhaps they cannot be resolved in the scope of the final e-meeting for the WI. After further thinking, we found there could be more issues, which are summarized below: - Alt1, sub-bullet of 1st Option 1: - Preferred resources sets from different UE-As may overlap. For example, maybe multiple UE-A indicate the same resource R1 as preferred resource and send it to UE-B. However, UE-B cannot transmit to multiple UE-As on the same resource R1. Then, how does Option 1 work in this case? Will UE-B consider different priorities of different UE-As? Or up to UE-B implementation? - Alt 1, sub-bullet of 2nd Option 1: - As mentioned above, if UE-B takes union of the non-preferred resources, the remaining resources in S_A could be very limited, causing RSRP increment and increasing interference. Thus, the performance could be even worse compared with Rel-16. - If non-preferred resources of different UE-A are due to half-duplex. Then, UE-A1’s half-duplex slot has no relationship with UE-A2’s half-duplex slot. Why UE-B needs to take union of the non-preferred resources. - Some previously received non-preferred resource set may be no longer valid and thus should not be considered. - Alt 1, sub-bullet of Option 2 - For example, assume UE-A1 indicates R1 as non-preferred due to half-duplex and provides it to UE-B, UE-A2 provides preferred resource to UE-B. Then, when UE-B chooses resource to transmit to UE-A2, why UE-B needs to consider R1? R1 is UE-A1’s non-preferred resource due to half-duplex, and has no relationship with UE-A2. As FL already mentioned, even proponents of Alt 1 have different details. We feel RAN1 does not have enough time to have very careful technical discussions on each of the options under each case. If Alt 1 is taken, we expect there will be many CRs in maintenance phase to fix the issues we mentioned above or even more issues. For simplicity, we suggest to take a unified solution to handle all the cases, i.e., Alt2. |
| Intel | Comments | RAN1 should decide first on which cast combinations of feedback of IUC information and sidelink transmission are supported. In addition, it is desirable to agree whether feedback from non-target receiver should be considered for sidelink transmissions. After that it may be easier to agree on supported options. |
| Apple |  | Actually, we are in the middle of Alt 1 and Alt 2. For the first and the last bullet in Alt 1, we think it is up to UE-B implementation. But for the second bullet in Alt 1, we support Option 1 as in Alt 1. |
| Samsung | Modified Alt1 | - For UE-B’s behaviour when UE-B receives multiple preferred resource sets from the different UE-As, - Option 1: UE-B uses each received preferred resource set for its resource selection for each TB to be transmitted to each UE-A providing the preferred resource set. - For UE-B’s behaviour when UE-B receives multiple non-preferred resource sets from the different UE-As. - Option 1: UE-B determines a final non-preferred resource set by making union of all the received non-preferred resource sets from different UE-As. UE-B uses the final non-preferred resource set for its resource selection for TB(s) to be transmitted to these different UE-As providing the non-preferred resource sets any UE. - For UE-B’s behaviour when UE-B receives both a single preferred resource set and a single non-preferred resource set from the different UE-As, - Option 2: UE-B uses both the received preferred resource set and non-preferred resource set from different UE-As for its resource selection for a TB to be transmitted to the UE-A providing the preferred resource set. In addition, UE-B uses the received non-preferred resource set for its resource selection for a TB to be transmitted to the UE-A providing the non-preferred resource set any UE. Alternatively, a simpler solution (which is our first preference), is: UE-B uses the latest IUC information it receives from each UE-A, whether it is preferred or non-preferred when transmitting to any UE. |
| Ericsson | Alt 1 |  |
| Futurewei | Alt 1 | We prefer Alt 1. |
| Nokia, NSB | Alt 1, with comments | Regarding the first bullet of Alt 1, Option 1 works for the case when UE-B has different unicast links with different UE-As. If UE-B requests multiple UE-As to provide IUC information for a single TB, then UE-B should use the intersection of the received multiple preferred resource sets from different UE-As. - For UE-B’s behaviour when UE-B receives multiple preferred resource sets from the different UE-As, - Option 1: UE-B uses each received preferred resource set for its resource selection for each TB to be transmitted to each UE-A providing the preferred resource set. - In case of groupcast to the multiple UE-As, UE-B selects resources from the intersection of the received preferred resource sets |
| Qualcomm | Alt 1 | We can accept Alt. 1 for progress with the modification below to avoid conflicting interpretation of the second and third bullets: - For UE-B’s behaviour when UE-B receives both a single preferred resource set and a single non-preferred resource set from the different UE-As, - Option 2: UE-B uses both the received preferred resource set and non-preferred resource set from different UE-As for its resource selection for a TB to be transmitted to the UE-A providing the preferred resource set. UE-B uses the received non-preferred resource set for its resource selection for a TB to be transmitted at least to the UE-A providing the non-preferred resource set. |
| Sharp | Alt 2 |  |
| InterDigital | Alt 2 |  |

Q3-22: There was a comment that defining additional criteria on which received preferred or non-preferred resource set(s) can be actually taken into account in UE-B’s resource selection is necessary. Which option is supported for this issue?

- Option 1: Up to UE-B’s implementation.

- Option 2: Only if a gap between the reception time of the resource set and the time when UE-B triggers a resource selection procedure for its data transmission is smaller than (pre)configured value, UE-B uses the received resource set in its resource selection.

- Option 3: Others (please specify it)

| Company | Option(s) | Comments |
| --- | --- | --- |
| NTT DOCOMO | Option 1 | No criteria other than those discussed above would be necessary. In our understanding. Gap-based rule like Option 2 is unnecessary since UE-B does not know when IUC message is received. If UE-B receives/decodes IUC message at timing X, then the information is used for data TX after timing X. That’s all. |
| LGE | Option 1 |  |
| OPPO | Option 1 | We think it could be up to UE-B implementation. |
| Fujitsu | Option 1 |  |
| Spreadtrum | Option 1 |  |
| NEC | Option 1 | Agree with DCM |
| xiaomi | Option1 |  |
| Fraunhofer | Option 1 |  |
| Huawei, HiSilicon | Option 1 | If which IUC is used is up to UE-B, there is little or no motivation to then specify how they will be used, since which, and how many, will even be considered cannot be predicted. If option 1 is taken, the rest of the discussion can be closed without specification impact. |
| Intel | Option 2 | Criteria is needed to avoid the use of outdated feedback and different UE behaviors. There should be common understanding across UEs which feedback can be considered for resource selection. |
| Apple | Option 2 |  |
| Samsung |  | Not clear why the additional criteria is necessary. |
| Ericsson | Option 3 | Only the resources of the UE-A(s) which are within a certain distance to UE-B are taken into consideration for the UE-B’s resource selection. |
| Futurewei | Option 2 with Comments | We think a time gap is needed. We are not sure how UE-B uses the coordination information after n+T1 except for re-evaluation/pre-emption. For IUC triggered by an explicit request, UE-B sends the starting/ending time to UE-A which can be viewed as UE-B’s RSW. UE-B expects to receive the coordination information before the starting time with a certain gap, e.g., Tproc,1 or Tproc,0+Tproc,1, which can be specified without a pre-configuration RRC parameter. UE-B cannot wait further. Similarly for IUC triggered by a condition. Although UE-B does not know when UE-A transmits coordination information except that RAN2 decides to use some PC5-RRC signaling from UE-B to UE-A(which the scenario IUC triggered by an explicit request applies), UE-B has set its own RSW [n+T1,n+T2] and it can only use the received set before [n+T1] with a certain time gap for resource selection processing. So we prefer option 2 with a specified time gap, e.g., Tproc,1 or Tproc,0+Tproc,1. |
| Nokia, NSB | Option 1 |  |
| Qualcomm | Option 1 |  |
| Sharp | Option 1 |  |
| InterDigital | Option 1 |  |

Q3-23: There were comments that it is necessary to define additional UE-B behavior to handle the case when it is not possible that the number of candidate single-slot resources after applying the received non-preferred resource set as per the existing agreement meets the requirement of X*M_total. Which option is supported for this issue?

- Option 1: Up to UE-B’s implementation, e.g., UE-B does not use the received non-preferred resource sets in its resource selection.

- Option 2: Others (please specify it)

| Company | Option(s) | Comments |
| --- | --- | --- |
| NTT DOCOMO |  | From technical perspective, there is no reason to use non-preferred resource for transmission to UE-A, so some rule can be considered. But considering the late stage, currently we are fine with either way. |
| Vivo | Option 1 | Clarify which implementation is allowed, e.g., UE can give up the non-preferred resources, or selects part of the non-preferred resource, to meet the requirement. |
| LGE | Option 1 |  |
| OPPO | Comments | Option 1 or specify that “UE-B does not use the received non-preferred resource sets in its resource selection.” |
| Fujitsu |  | Our first preference is that some non-preferred sources can be brought back until meeting the requirement. If not convergent, Option1 is the second preference. |
| Spreadtrum | Option 1 |  |
| NEC | Option 1 |  |
| xiaomi | Option 1 |  |
| Fraunhofer | Option 2 | We do not think UE-B discarding the non-preferred resource set is ideal because UE-B would be then including resources where collisions are possible to its candidate resource set. We are fine with UE-B increasing the threshold and repeating the process, as described in the current specifications (step 7). If the group cannot converge, Option 1 is our second preference. |
| Huawei, HiSilicon | Discuss proposal 3-20, 3-21 first | If Alt 2 (UE implementation) is taken for Proposal 3-20, 3-21, then there might be no need to discuss this question. |
| Intel | Comments | In our view, this is a critical issue. We can accept the simple behavior – fallback to TX candidate resource set. Leaving it up to UE implementation diminishes RAN1 efforts on definition of IUC framework. |
| Samsung | Option 2 | Different preference levels are indicated for the non-preferred resources. If there isn’t enough candidate resources, the levels corresponding to the least of the non-preferred resources is not excluded. |
| Futurewei | Comments | Some clarification is needed on the up-to UE-B’s implementation, e.g., whether it includes that UE-B ignore the requirement of X*M_total, which we do not prefer. We think some specification is needed. For example, if non-preferred resources are due to half-duplex issue. We think it should be applied in the procedure. So we prefer the following modification. - It is up to UE-B’s implementation on how to satisfy the requirement of X*M_total but UE-B should at least apply the whole slot(s) that is appeared in non-preferred resource set. |
| Nokia, NSB | Option 2 | We don’t think this issue should be left up to UE-B implementation. The infinite loop issue needs to be addressed by the specification. We suggest to relax UE-B’s resource exclusion by increasing the allowed overlap in Step 7. When evaluating whether or not to exclude a candidate single-slot resource that overlaps with non-preferred resource(s), the allowed overlap (initially 0%) may be successively increased (e.g., first to 10%, then to 20%, and so on) in Step 7. A candidate single-slot resource is only excluded if its overlap with non-preferred resource(s) is greater than the allowed overap. In this way, the infinite loop issue can be resolved. 7) If the number of candidate single-slot resources remaining in the set SA is smaller than X ⋅ Mtotal, then Th(pi , pj) is increased by 3 dB for each priority value Th(pi , pj) and allowedOverlapNonPreferredResources is increased by [Y] percentage points [alternatively, increased by Z subchannels] and the procedure continues with step 4. |
| Qualcomm | Option 1 |  |
| Sharp | Option 1 |  |
| InterDigital | Option 1 |  |

Q3-24: There was a comment that further clarification is necessary on which 2nd SCI format can be used for retransmission of inter-UE coordination information MAC CE initially scheduled by a SCI format 2-C. Do you agree following conclusion for this issue?

Draft conclusion 3-24:

- Any 2nd SCI formats can be used for retransmission of inter-UE coordination information MAC CE initially scheduled by a SCI format 2-C.

| Company | Yes or no | Comments |
| --- | --- | --- |
| NTT DOCOMO | Yes | And no spec impact is assumed. |
| Vivo | Yes |  |
| LGE | Yes | It might not need to have specification change. |
| OPPO | yes |  |
| Fujitsu | Yes |  |
| Spreadtrum | Yes |  |
| NEC | Yes |  |
| Xiaomi | Yes |  |
| Fraunhofer | Yes |  |
| Huawei, HiSilicon | No | Different 2nd SCI format have different size, thus occupy different number of REs. As per current spec (copied below, especially cyan part), this will further impact TBS determination, resulting in different TBS for initial and retransmission(s) of the same TB and causing HARQ combining infeasible. In general, we think there is no need to discuss this proposal. UE will behave as per spec. == (… below is copied from TS 38.214… ) #### 8.1.3.2 Transport block size determination For the PSSCH assigned by SCI, if Table 5.1.3.1-2 is used and 0 ≤ IMCS ≤27, or a table other than Table 5.1.3.1-2 is used and 0 ≤ IMCS ≤28, the UE shall first determine the TBS as specified below: The UE shall first determine the number of REs (NRE) within the slot. - A UE first determines the number of REs allocated for PSSCH within a PRB (NRE') by NRE'=NscRBNsymbsh-NsymbPSFCH-NohPRB-NREDMRS, where … - A UE determines the total number of REs allocated for PSSCH ()NRE) by NRE=NRE'∙nPRB-NRESCI,1-NRESCI,2NRE= NRE'* nPRB, where - nPRB is the total number of allocated PRBs for the PSSCH, - NRESCI,1 is the total number of REs occupied by the PSCCH and PSCCH DM-RS. - NRESCI,2 is the number of coded modulation symbols generated for 2nd-stage SCI transmission (prior to duplication for the 2nd layer, if present) according to Clause 8.4.4 of [5, TS 38.212], with the assumption of γ=0. |
| Intel | No | Motivation and benefits are unclear. |
| Apple | No | If the IUC MAC CE is initially scheduled by a SCI 2-C, then the MAC CE contains less than Nmax combinations of (TRIV, FRIV, periodicity). Hence, the SCI 2-C has to be used for delivering this IUC MAC CE for the retransmission. |
| Samsung | No | Format 2-C should not be used for re-transmissions when a NACK is received. The reception of a NACK implies that the pervious Format 2-C has been received, there is no need to re-transmit IUC in format 2-C. Using format 2-A, leaves more resources for the SL-SCH. We should have a proposal to use format 2-A for retransmission when a NACK is received. |
| Ericsson | No | Only SCI format 2-C can be used to re-transmit the inter-UE coordination information. |
| Futurewei | No | We did not support retx, and if it is to be used it should be as simple as possible and use the same format as the initial transmission |
| Nokia, NSB | Yes |  |
| Qualcomm | No | The motivation and potential benefits are unclear. This also brings up additional issues as mentioned by Huawei, HiSilicon. |
| Sharp | No | Same view as Huawei |

Q3-25: There was a comment that further clarification is necessary on the condition when Option B can be used for preferred resource set (including clarifying the meaning of “when UE-B does not support sensing/resource exclusion”). Which option is supported for this issue?

- Option 1: UE-B does not have a capability of performing sensing/resource exclusion.

- Option 2: UE-B performs random resource selection.

- Option 3: UE-B has a capability of performing sensing/resource exclusion, but UE-B determines not to perform sensing/resource exclusion by its implementation.

- Option 4: Others (please specify it)

| Company | Option(s) | Comments |
| --- | --- | --- |
| NTT DOCOMO | 1 only; or 1+2 | If UE-B ignores reservation from its surrounding UEs, collision increase at the surrounding UEs is assumed. This is not aligned with purpose of IUC. In other words, Option 3 means even in resource pool with full-sensing only, UE-B can perform random selection-like behavior from its surrounding UE’s perspective. |
| vivo | Option 1/2/3 |  |
| LGE | Option 2 | At least UE-B needs to have SL RX capability to receive inter-UE coordination information from UE-A. |
| OPPO | Option 1,Option 2 and Option 3 | All option 1, 2 and 3 can lead to a result that UE-B has not sensing results to combine with the preferred resource set, Option B can be applied. |
| Fujitsu | Option 1 |  |
| Spreadtrum | At least Option 1 | We are fine with option 1 or 2 or 3. |
| NEC | 1/2/3 | Agree with OPPO |
| Xiaomi | Option 1/2/3 | we are fine with either one at this stage. |
| Fraunhofer | Option 1, 2, 3 | All the 3 cases should be considered in the case where UE-B does not have its own sensing results. For Option 2, as mentioned by LG, the UE should be capable of receiving the IUCs. |
| Huawei, HiSilicon | Option 1, 2, 3 | All of Option 1, 2, 3 are possible cases. It is not essential to impose new constraints, because TS 38.214 is already general in capturing the possibilities in the wording “8.1.4C: …. when the UE has no own sensing result…”. |
| Intel | Option 1 |  |
| Apple | Option 2 or 3 | Regarding Option 1, we have a clarification question. If UE-B does not have the capability of sensing, how could UE-B receive IUC? To receive IUC, UE-B at least needs to decode SCI and decode PDSCH. For sensing, UE-B decodes SCI and measure RSRP. To support Option 1, does it mean UE-B has the capability of only SCI decoding, but has no capability of RSRP measurement (and hence no capability of sensing)? |
| Samsung | Option 1 |  |
| Ericsson | Option 1 | Option B should only be used when UE-B does not have the capability of performing sensing. |
| Futurewei | Option 1,2,3 | We think all Options 1,2,3 are applicable. |
| Qualcomm | Option 3 |  |
| InterDigital | Option 2 Option 3 Option 4 | For Option 1, UE-B without sensing capability may not be able to receive IUC information and support Scheme 1. We prefer also to use Option B for power saving purpose, e.g. for UE in SL DRX, so we suggest Option 4 below: Option 4: UE-B has a capability of performing sensing/resource exclusion, but UE-B is (pre)-configured not to perform sensing/resource exclusion in SL DRX. |

Q3-26: There was a comment that further enhancement on UE-B’s behavior to consider “the slot(s) overlapped with UE-A’s reserved resource(s) by 1st stage SCI” as non-preferred resource(s) in its resource selection. Company provide their view on following draft proposal.

Draft proposal 3-26:

For unicast/groupcast TB transmission of UE-B, it is up to UE-B’s implementation to use the slot(s) overlapped with UE-A’s reserved resource(s) by 1st stage SCI as non-preferred resource(s) in its resource selection

- Note that UE-A sends 1st stage SCI only when UE-A has TB transmission

| Company | Yes or no | Comments |
| --- | --- | --- |
| NTT DOCOMO |  | This is not related to IUC, right? We think this proposal is unnecessary. |
| Vivo | Yes | As discussed in previous meeting, legacy resource reservation information is interpreted as coordination information. Thus, even there is no MAC CE multiplexed with TB transmission, the non-preferred resource determined based on condition 1-B-2 can be informed, which can save plenty of redundant MAC CE signaling overhead. |
| LGE | Comment | At least, we do not support the case when only a SCI format 1-A is transmitted without PSSCH. |
| OPPO | yes | It should be supported to reduce half duplex between UE-A and UE-B |
| Fujitsu | Comment | Similar issues exist in Rel-16 mode 2. Not sure whether it should be solved in Rel-17. |
| Spreadtrum | Yes | The half-duplex issue should be considered. |
| NEC |  | Same view as DCM |
| xiaomi | Yes with comment | We think that there have agreement that the resource used by UE-A for its transmission is the non-preferred resource. Agreement - For Condition 1-A-2 of Scheme 1, the set of resources preferred for UE-B’s transmission is a form of candidate single-slot resource as specified in Rel-16 TS 38.214 Section 8.1.4 - UE-A excludes candidate single-slot candidate(s) belonging to “slot(s) where UE-A, when it is intended receiver of UE-B, does not expect to perform SL reception from UE-B due to half duplex operation” after Step 6) of TS 38.214 Section 8.1.4 |
| Huawei, HiSilicon | No | This is defining new Condition for non-preferred resources in addition to Condition 1-B-1 and 1-B-2. Such optimization should be avoided at this late stage. RAN1 should not spend time discussing it. We share similar view with other companies that this has no relationship with inter-UE coordination, because UE-A may not transmit SCI format 2C or new MAC-CE at all in this case. In addition, this will cause legacy sensing procedures not aligned between R16 UEs and R17 UEs. For R16 sensing procedures, reserved resources by 1st SCI will be excluded using RSRP (i.e., in step 6 of TS 38.214 clause 8.1.4). However, if this proposal is agreed, reserved resources by 1st SCI will be excluded after step 6) directly, i.e., regardless of their RSRP. This is very different from legacy sensing procedures. |
| Intel | Comment | We can accept it if the similar behavior is defined for unicast and groupcast transmissions to target RX |
| Apple |  | Clarification question: is it assumed that UE-B’s TB is to be sent to UE-A in this case? |
| Samsung | No | No further enhancement is necessary |
| Ericsson |  | We are not sure we need this proposal. Is the intention to have something different than legacy procedure? |
| Futurewei | comment | We think if UE-A is not the Rx of UE-B for the data to be transmitted, UE-B does not need to use the slots of UE-A’s reserved resource as non-preferred resource. We suggest the following update. For unicast/groupcast TB transmission of UE-B, it is up to UE-B’s implementation to use the slot(s) overlapped with UE-A’s reserved resource(s) by 1st stage SCI as non-preferred resource(s) in its resource selection if UE-A is the destination UE of the TB to be transmitted by UE-B - Note that UE-A sends 1st stage SCI only when UE-A has TB transmission |
| Nokia, NSB | Yes |  |
| Qualcomm | Yes | We support the proposal. |
| Sharp | Yes |  |
| InterDigital | Yes |  |

Q3-27: There was a comment that considering RAN2 agreed that “IUC in SL DRX is deprioritized in Rel-17 from RAN2 point of view”, RAN1 should include the restrictions to the IUC mechanism to address the power saving operation. Do you agree following draft proposal for this issue?

Draft proposal 3-27:

When the amount of sensing performed by UE-A is below a (pre)configured threshold, inter-UE coordination information is not transmitted by UE-A

| Company | Yes or no | Comments |
| --- | --- | --- |
| NTT DOCOMO |  | We are fine to have discussions, but sufficient discussions are preferred since there would be other solutions. |
| Vivo |  | UE-A’s behavior is clearly defined including the sensing result acquisition, thus no need to have such discussion |
| LGE | No | Regarding the following RAN2 agreement, our understanding is that RAN2 will not optimize inter-UE coordination operation considering SL DRX operation. IUC in SL DRX is deprioritized in Rel-17 from RAN2 point of view Moreover, it is possible that UE-A determines whether or not to transmit inter-UE coordination information by its implementation. No further condition seems necessary. |
| OPPO | NO | Our understanding is that UE-A is performing full sensing, the issue may not happen. Furthermore, whether to transmit inter-UE coordination information is up to UE-A implementation, there is no need to define further restrictions. |
| Fujitsu | No | The issue only occurs in DRX? Maybe we first discuss whether inter-UE coordination is specified for DRX. |
| Spreadtrum | No | Similar view with Fujitsu. We should first discuss whether this question will exist or not. |
| NEC | No |  |
| Xiaomi | no | Further optimization is not necessary. |
| Fraunhofer | No | Our understanding is that UE-A is a full sensing UE, not sure how this scenario would present itself. |
| Huawei, HiSilicon | No | There is no time/room for additional enhancements. RAN1 should avoid such discussion. |
| Intel | Yes | Reliability of feedback depends on sensing behavior amount of monitored slots. This discussion seems unavoidable. |
| Apple |  | We are fine with the direction of this proposal. But we are not sure whether this has RAN2 impact. |
| Samsung | No | The proposal has high layer impact and this is not an essential issue. |
| Ericsson | Yes | Same view as Intel |
| Futurewei | Comment | We do feel we need to follow ran2 lead (I think it is do). But we do not think it is necessary to discuss about a preconfigured threshold in this late stage. We suggest a simpler proposal that IUC is only transmitted when the UE performs full sensing. |
| Qualcomm | No | No further optimization is necessary. |
| Sharp | No |  |
| InterDigital | No | We have agreed the triggering conditions for UE-A is UE implementation and it is thus quite conceivable that a UE in SL DRX decides not to trigger IUC transmission at all. Also, for explicit-triggered IUC transmission, a UE in SL DRX may decide by implementation not to become UE-A and UE-B will not receive IUC information within the latency bound. So we think it can be left to UE implementation for UE in SL DRX whether or not to transmit IUC. |

  - Others

Q3-29: Do you agree following draft conclusion for the reply LS to RAN2?

FL’s observation of 2nd email discussion: - Support: DCM, InterDigital, LGE, Qualcomm, Futurewei, Sharp, Fujitsu, NEC, OPPO, Samsung, Ericsson, Huawei, Nokia, (13) - Not support: Intel, (1)

Draft conclusion 3-29:

- No consensus for RAN1 to send a reply LS of R1-2200880 to RAN2.

| Company | Yes or no | Comments |
| --- | --- | --- |
| NTT DOCOMO | Yes |  |
| LGE | Yes |  |
| OPPO | yes |  |
| Fujitsu | Yes |  |
| NEC | No |  |
| xiaomi | Yes |  |
| Huawei, HiSilicon | Yes | No consensus / no need. |
| Intel | Comments | No consensus means scope in each group is unclear. We prefer to have an LS with clear split of RAN1 and RAN2 objectives. Otherwise, we should inform RAN2 that there is no consensus on the list. |
| Ericsson | Yes | No need to send an LS right now. |
| Futurewei | Yes | We are ok with the conclusion |
| Qualcomm | Yes |  |
| Sharp | Yes |  |
| InterDigital | Yes |  |

- Summary of contributions

  - Scheme 1

- Finalization of contents and containers of UE-A’s inter-UE coordination information and UE-B’s explicit request, including determination of destination UE(s) for UE-A’s inter-UE coordination information and UE-B’s explicit request

  - Remaining details on determining preferred resource set

    - If inter-UE coordination information is triggered by a condition rather than request reception

      - Setting of resource selection window

        - T_1 and T_2 are (pre)configured and slot n is a slot when UE-A start to process the sensing and resource selection [Futurewei,3] (1)

        - T_2-T_1 is (pre)configured [Intel,14] (1)

      - Minimum number of candidate single-slot resources for feedback [Intel,14] (1)

      - No further change is supported [OPPO,6] [CMCC,17] [ZTE,29] (3)

    - Further consideration on modification of T_scal [Sharp,23] (1)

  - Remaining details on bit field size of contents of inter-UE coordination information

    - Reference slot indication

      - 10+ ceil( log2(10*2^u)) where u is 0, 1, 2, 3 for SCS of 15, 30, 60, 120, respectively

        - Supported by [Huawei,1] [CATT,7] [Apple,15] [Samsung,20] [LGE,26] [ZTE,29] (6)

    - Slot offset for first resource location

      - Ceil(log2(N_slot_offset)) where N_slot_offset is the number of entries in the (pre)configured values set from [0, 255]

        - Supported by [Huawei,1] (1)

      - Ceil(log2(maximum value of slot offset))

        - Supported by [DCM,9] [Apple,15] (2)

      - 8 bits

        - Supported by [Samsung,20](for TRIV other than first TRIV) [ZTE,29](for TRIV other than first TRIV in a SCI format 2-C) (2)

      - 0 bit

        - Supported by [Samsung,20](for first TRIV) [ZTE,29](for first TRIV) (2)

      - Ceil(log2(maximum value of slot offset/31))

        - Supported by [LGE,26] (1)

    - Resource set indication for each combination

      - Up to 26 bits [Huawei,1] [Panasonic,5] [CATT,7] [DCM,9] [Apple,15] [Samsung,20](for non-preferred resource set) [LGE,26] (7)

      - Up to 22 bits [Samsung,20](for preferred resource set) [ZTE,29] (2)

    - Resource set type

      - Always 1 bit

        - Supported by [Huawei,1] [CATT,7] [LGE,26] (3)

      - 0 bit if request contains “resource set type indication” and if condition-based IUC is disabled. Otherwise, 1 bit.

        - Supported by [Apple,15] (1)

  - Remaining details on first resource location indication of each TRIV

    - Maximum value of slot offset for the first resource location indication

      - 16

        - Supported by [Apple,15](for SCI format 2-A as a baseline) (1)

      - 32

        - Supported by [Qualcomm,22](when SCI format 2-C is used) (1)

      - 256

        - Supported by [Huawei,1] [CATT,7](for 2nd SCI) [DCM,9] [Apple,15](for SCI format 2-A as a baseline) (4)

      - 1023

        - Supported by [ZTE,29] (1)

      - 4092

        - Supported by [OPPO,6] (1)

      - 8000

        - Supported by [CATT,7](for MAC CE) [LGE,26] (2)

      - 8192

        - Supported by [Futurewei,3] [Samsung,20] (2)

          - Possible values of (pre)configured maximum value is form of 2^k -1 [Futurewei,3] [Samsung,20]

      - Maximum reservation periodicity configured in the pool * 2^u

        - Supported by [Qualcomm,22](when MAC CE only is used) (1)

    - Granularity of slot offset

      - 1

        - Supported by [CATT,7] [DCM,9] [Apple,15] [Qualcomm,22] (4)

      - 31

        - Supported by [LGE,26] (1)

      - Candidates themselves are (pre)configured

        - Supported by [Huawei,1] (1)

      - Determined by the bit field size for indicating slot offset and SCS (e.g., 1, 2, 4, 8, 16, 32)

        - Supported by [Samsung,20] (1)

    - Whether or not UE-A provide preferred or non-preferred resources for each first resource location

      - Supported with additional indicating the lowest subchannel index of each first resource

        - [OPPO,6] [ETRI,13] [Apple,15] (3)

      - Supported with additional indicating the lowest subchannel index of first resource of a first combination

        - Supported by [Intel,14] (1)

      - Not support

        - [Huawei,1] (1)

    - Further consideration on modifying the definition of reference slot [ETRI,13] [Intel,14] (2)

  - Remaining details on bit field size of contents of an explicit request

    - Starting and ending time locations of a resource selection window

      - 2*{10+ ceil( log2(10*2^u))} where u is 0, 1, 2, 3 for SCS of 15, 30, 60, 120, respectively

        - Supported by [Huawei,1] [CATT,7] (2)

    - Resource set type

      - 0 or 1 bit as per (pre)configuration

        - Supported by [Apple,15] (1)

  - Details on a SCI format 2-C

    - SCI fields design

      - SCI fields for a SCI format 2-A

        - Supported by [Huawei,1] [DCM,9] [Apple,15] [Xiaomi,19] [ITL,25] [LGE,26] [Ericsson,27] [ZTE,29] (8)

          - [vivo,4] [Panasonic,5]: Cast type is not included for an explicit request

      - SCI fields for both a SCI format 2-A and 2-B

        - Supported by [Panasonic,5] [CATT,7] [Intel,14] [Samsung,20] (4)

    - Condition of that a SCI format 2-C can be used as container of inter-UE coordination information

      - Keep N<=3 (i.e., remove square brackets)

        - Supported by [LGE,26] [Ericsson,27] (2)

          - [LGE,26]: Add “UE does not expect that the total payload size of a SCI format 2-C with N=3 exceeds 140 bits” as a note

      - N<=2

        - Supported by [CATT,7] [DCM,9] [Apple,15] (3)

      - Remove N parts

        - Supported by [Intel,14] (1)

      - Both N<=3 and N<=2

        - Supported by [Samsung,20] (1)

      - Further consideration on additional condition of that a SCI format 2-C can be used [Intle,14] [Qualcomm,22]

        - [Intel,14]: a SCI format 2-C can be used for preferred resource set

        - [Qualcomm,22]: a SCI format 2-C can be used for the case when other data is not multiplexed with inter-UE coordination information

  - Cast type(s) of inter-UE coordination information transmission with preferred resource set triggered by a condition other than explicit request reception on top of unicast

    - Neither groupcast nor broadcast

      - Supported by [vivo,4] [Panasonic,5] [OPPO,6] [DCM,9] [Spreadtrum,11] [CMCC,17] [Samsung,20] [LGE,26] [Ericsson,27] [Mitsubishi,28] [ZTE,29] (11)

    - Groupcast

      - Supported by [Futurewei,3] [Fraunhofer,30] (2)

    - Groupcast and broadcast

      - Supported by [Intel,14] (1)

    - Up to RAN2/SA2 decision

      - Supported by [Huawei,1] (1)

  - Latency bound of inter-UE coordination information transmission triggered by UE-B’s explicit request

    - Supported by [vivo,4] [CATT,7] [Intel,14] [Apple,15] [Xiaomi,19] [Qualcomm,22] [Sharp,23] [ITL,25] [Fraunhofer,30] (9)

      - PC5-RRC configured

        - Supported by [vivo,4] (1)

      - (pre)configured

        - Supported by [CATT,7] [Intel,14] [Xiaomi,19] (3)

      - Indicated by UE-B’s request

        - Supported by [CATT,7] [Apple,15] [Sharp,23] [ITL,25] [Fraunhofer,30] (5)

      - 8 slots

        - Supported by [Qualcomm,22](for standalone inter-UE coordination information) (1)

      - Derived based on the starting time of resource selection window provided by UE-B’s request

        - Supported by [Sharp,23] (1)

  - Further consideration on modifying UE-A’s procedure for determining a set of resources [Nokia,2] [vivo,4] [CATT,7] [Intel,14] [ASUSTeK,16] [Fraunhofer,30] (6)

  - Further consideration on additional contents of the inter-UE coordination information in Scheme 1 [InterDigital,10] [Intel,14] [ASUSTeK,16] (3)

  - Further consideration on differentiating supported cast type for each condition of non-preferred resource set [OPPO,6] [CMCC,17] [Mitsubishi,28] (3)

    - Up to UE-A’s implementation [DCM,9] (1)

  - Further consideration on additional contents of the request for the inter-UE coordination information in Scheme 1 [Nokia,2] [Fujitsu,8] (2)

  - Further consideration on specifying additional details on Condition 1-A-2/1-B-2/2-A-2 [vivo,4] [Intel,14] (2)

  - Further consideration on additional condition for determining a set of resources [Nokia,2]

  - Further consideration on parameter setting for determining the non-preferred resource set [Futurewei,3]

  - Further consideration on modifying re-evaluation/pre-emption operation considering the received non-preferred resource set [vivo,4]

  - Further consideration on using UE-A’s resource reservation period as coordination information [vivo,4]

  - Further consideration on modifying the cast type of request signaling [Intel,14]

  - Further consideration on modifying the cast type of inter-UE coordination information transmission triggered by an explicit request [Intel,14]

  - Further consideration on modifying the cast type of inter-UE coordination information transmission triggered by a condition other than explicit request reception [Xiaomi,19]

  - Further consideration on the case when only a SCI format 2-C is used as a container of inter-UE coordination information and/or its request [Samsung,20]

  - Further consideration on the possibility of that different parameters of the request are transmitted by a SCI format 2-C and MAC CE [Intel,14]

  - Further consideration on modifying interpretation rule for TRIV [ASUSTeK,16]

- Finalization of behavior of UE-B receiving resource set(s) from UE-A(s)

  - UE-B’s behavior when UE-B receives multiple inter-UE coordination information from the same UE-A

    - UE-B uses the latest inter-UE coordination information in its resource selection

      - Supported by [Panasonic,5] [LGE,26](for preferred resource set) (2)

    - UE-B determines one of them by implementation to use in its resource selection

      - Supported by [LGE,26] (1)

  - UE-B’s behavior when UE-B receives multiple inter-UE coordination information from the different UE-As

    - For preferred resource set,

      - UE-B uses one inter-UE coordination information for each UE-A

        - Supported by [Panasonic,5] [DCM,9] (2)

      - UE-B uses multiple inter-UE coordination information in its resource selection

        - Supported by [Apple,15] [Samsung,20] (2)

      - UE-B determines one of them by implementation to use in its resource selection

        - Supported by [LGE,26] (1)

    - For non-preferred resource set,

      - UE-B uses multiple inter-UE coordination information in its resource selection

        - Supported by [Panasonic,5] [DCM,9] [Apple,15] [Samsung,20] [Qualcomm,22] (5)

      - UE-B determines one of them by implementation to use in its resource selection

        - Supported by [LGE,26] (1)

    - For preferred resource set and non-preferred resource set,

      - UE-B uses preferred resource later

        - Supported by [DCM,9] (1)

      - UE-B determines one of them by implementation to use in its resource selection

        - Supported by [LGE,26] (1)

      - Not supported by [Samsung,20] (1)

  - Further consideration on modifying UE-B’s resource selection procedure based on the received set of resources [Nokia,2] [vivo,4] [CATT,7] [Fujitsu,8] [ITL,25] (5)

    - [Nokia,2]: Overlapping portion dependent resource exclusion

    - [vivo,4]: Restrict maximum number of resource exclusion, change the definition of M_total

    - [CATT,7]: Additional candidate single-slot resource ratio

    - [Fujitsu,8] [ITL,25]: Canceling a subset of resource exclusion

  - Further clarification on the condition for using Option B [DCM,9] [Qualcomm,22] [Ericsson,27] (3)

    - [DCM,9]: UE that does not support sensing/resource exclusion, UE that supports sensing/resource exclusion but performs random selection for the corresponding transmission

    - [Qualcomm,22]: UE that supports sensing/resource exclusion but does not perform sensing/resource exclusion

    - [Ericsson,27]: UE that does not support sensing

  - Further consideration on specifying cast type(s) of UE-B’s transmission that can use inter-UE coordination information [CATT,7] [Qualcomm,22] [Mitsubishi,28] (3)

  - Further considering on specifying a condition of skipping the received inter-UE coordination information [Intel,14] [Sharp,23]

  - Further consideration on specifying format translation from the received set of resources to candidate single-slot resources [Intel,14]

- Finalization of when and with which information UE-A generates and/or transmits an inter-UE coordination information, including triggering based on condition(s) other than an explicit request

  - Sensing window for determining the set of resources

    - Sensing window prior to the transmission time (slot n) of UE-A’s inter-UE coordination information

      - Supported by [Huawei,1] [OPPO,6] [CATT,7] [Xiaomi,19] (4)

        - [n-T_0-T_proc,1, n-T_proc,0-T_proc,1]: [Huawei,1] [CATT,7] [Xiaomi,19] (3)

        - [n-T_0-T_ 3, n-T_proc,0-T_ 3]: [OPPO,6] (1)

    - Sensing window prior to the resource selection window for transmitting UE-A’s inter-UE coordination information

      - Supported by [Intel,14]

        - [?, n-T_proc,0-T_proc1] where n is the beginning of the resource selection window: [Intel,14]

    - No additional spec change is needed for sensing window for determining the set of resources

      - Supported by [LGE,26]

  - Further consideration on additional condition triggering inter-UE coordination information [Nokia,2] [Intle,14] [Samsung,20] [Ericsson,27] [Fraunhofer,30] (5)

- Finalization of when UE-B generates and/or transmits an explicit request

  - Further consideration on additional condition triggering an explicit request for inter-UE coordination information [vivo,4] [Intel,14] [NEC,18] [Ericsson,27] (4)

- Finalization of resource selection and/or multiplexing with sidelink transmissions for UE-A’s inter-UE coordination information and UE-B’s explicit request

  - Further consideration on additional restriction on inter-UE coordination information transmission [Intel,14] [Qualcomm,22] [Lenovo,24] [Ericsson,27] (4)

    - [Intel,14]: Resource selection window for inter-UE coordination information transmission is inside of a resource selection window for determining the set of resources

    - [Qualcomm,22] [Ericsson,27]: For inter-UE coordination information transmission without multiplexing with other data, retransmission is not supported

    - [Qualcomm,22]: For inter-UE coordination information transmission without multiplexing with other data, the number of subchanel is 1 and a remaining PDB is 8 slots

    - [Lenovo,24]: The ending time of a resource selection window for inter-UE coordination information transmission is not after the starting time of a resource selection window for determining the set of resources

  - Further consideration on multiplexing inter-UE coordination information, an explicit request, and data in a PSSCH [Intel,14]

  - Further consideration on updating UE-A’s resource (re)selection procedure for its transmission based on UE-A’s inter-UE coordination information [ASUSTeK,16]

  - Further consideration on dedicated resources for inter-UE coordination information transmission [ITL,25]

- Finalization of prioritization of inter-UE coordination information and explicit request

  - Further consideration on default priority value for inter-UE coordination information triggered by a condition rather than request reception [Huawei,1] [Futurewei,3] [DCM,9] [Intel,14] [CMCC,17] (5)

    - Up to UE-A’s implementation [Huawei,1] [Futurewei,3] [CMCC,17](for preferred resource set) (3)

    - Up to UE-A’s implementation with (pre)configured lower limit of priority value [Panasonic,5] (1)

    - Fixed to 8 [DCM,9] (1)

    - Same as priority value of indicated by other UE’s SCI [CMCC,17](for non-preferred resource set) (1)

    - Not supported by [OPPO,6] (1)

  - Scheme 2

- Finalization of determination of PSFCH resource/index for conflict indication

  - Frequency and code domain resources derived by

    - m_CS

      - 0

        - Supported by [Futurewei,3] [OPPO,6] [DCM,9] [Intel,14] [Samsung,20] [Qualcomm,22] [LGE,26] [Ericsson,27] [ZTE,29] (9)

          - [Intel,14]: it up to UE implementations whether/how to set the reservation period in the re-selected resource

          - [Qualcomm,22]: UE A sends PSFCH conflict indicator to UE B if a resource conflict is detected in the next SPS period

      - Based on target TB (0 for current TB, 6 for next TB(s))

        - Supported by [Huawei,1] [Nokia,2] [CATT,7] [InterDigital,10] [Spreadtrum,11] [Apple,15] (6)

    - m_0 determination based on PSFCH resource index

      - In the same way as specified in TS 38.213 Section 16.3

        - Supported by [Huawei,1] [Panasonic,5](when different PRB is used) [CATT,7] [DCM,9] [Spreadtrum,11] [Intel,14] [Apple,15] [Samsung,20](when the different PRB is used) [Qualcomm,22] [LGE,26] [Ericsson,27] [ZTE,29] (12)

      - A value of m_0 is (pre)configured

        - Supported by [Panasonic,5](when the same PRB is used) (1)

      - Circular offset is additionally applied to values of m_0 as specified in TS38.213 Section 16.3

        - Supported by [Samsung,20](when the same PRB is used) (1)

    - Case when the same PRB is used for both SL HARQ-ACK feedback and a resource conflict indication

      - Supported by [Futurewei,3] [Samsung,20] (2)

      - UE does not expect it [Huawei,1] [CATT,7] [Qualcomm,22] [ZTE,29] (4)

- Finalization of behavior of UE-B receiving a conflict indication from UE-A

  - Further consideration on UE-B’s behavior for handling a resource conflict in periodic reserved resources [Huawei,1] [Nokia,2] [CATT,7] [InterDigital,10] [Spreadtrum,11] [Apple,15] (6)

    - Not supported by [Futurewei,3] [OPPO,6] [DCM,9] [Intel,14] [Samsung,20] [LGE,26] [Ericsson,27] (7)

  - Further consideration on skipping the received resource conflict indication [Nokia,2] [OPPO,6] [Fujitsu,8] [Ericsson,27] (4)

  - Further consideration on specifying conditions to skip a transmission of a resource conflict indication [Nokia,2] [Fujitsu,8] [Intel,14] (3)

  - Further clarification on the next reserved resources subject to processing time budget [DCM,9] (1)

- Finalization of prioritization of conflict indication

  - Further consideration on modifying executing order of prioritization of PSFCH for a resource conflict [ETRI,13] [Apple,15] [Xiaomi,19] (3)

    - [ETRI,13] [Xiaomi,19]: PSFCH TX/TX or TX/RX prioritization is performed first

    - [Apple,15]: prioritization between PSFCH TX or RX and other channel(s) is performed first

  - Further consideration on modifying prioritization rule for PSFCH TX of SL HARQ-ACK feedback and a resource conflict indication [ETRI,13] (1)

  - Further consideration on the issue due to imbalanced prioritization between PSFCH TX and RX for a resource conflict indication [Apple,15] (1)

- Finalization of how to determine UE-B among UEs scheduling conflicting TBs, including whether/how to handle, or differently handle, the case when at least one of UEs scheduling conflicting TBs doesn’t support Scheme 2

  - Based on a second UE flag (i.e., whether UE scheduling a conflict TB can be UE-B or not)  indicated by UE-B’s SCI format 1-A: [Huawei,1] [Futurewei,3] [Panasonic,5] [OPPO,6] [CATT,7] [DCM,9] [InterDigital,10] [Apple,15] [Sharp,23] [LGE,26] (10)

    - UE pairing for selecting UE-B considers only UEs transmitting SCI format 1-A with Second UE flag of 1

      - Supported by [Huawei,1] [Panasonic,5] [OPPO,6] [CATT,7] [DCM,9] [LGE,26] (6)

    - Drop PSFCH TX when the selected UE-B does not support Scheme 2 after applying the existing WA for selecting UE-B

      - Supported by [Nokia,2] [Sharp,23] [Ericsson,27] (3)

    - At least one of UEs scheduling conflicting TBs does not support scheme 2, all other UEs transmitting SCI format 1-A with a second flag of 1 are UE-Bs

      - Supported by [Futurewei,3] [Apple,15] (2)

  - Based on whether PSFCH occasion(s) for resource conflict indication is passed or not: [Huawei,1] [OPPO,6] [Fujitsu,7] [LGE,26] (4)

    - UE pairing for selecting UE-B considers only UEs whose PSFCH occasions for a resource conflict indication are not yet passed

      - Supported by [Huawei,1] [OPPO,6] [Fujitsu,7] [LGE,26] (4)

  - Based on priority value of UE-B’s transmission [InterDigital,10] (1)

  - Further consideration on specifying cast type of UE-B’s transmission that can receive a resource conflict indication [Futurewei,3] [CATT,7] [Fujitsu,8] (3)

  - Further consideration on tie-breaking for the case when conflicting TBs have the same priority [Futurewei,3] [Fujitsu,8] (2)

    - Up to UE implementation [Intel,14]

- Others

  - Further restrict or expand on the condition to be UE-A and/or UE-B [Nokia,2] [vivo,4] [Intel,14] [Ericsson,27] [Mitsubishi,28] (5)

  - Further consideration on modifying condition for determining a resource conflict [Nokia,2] [Fujitsu,8] [Intel,14] [Lenovo,24] (4)

  - Further consideration on ID sharing mechanism between UE-A and UE-B [Nokia,2]

  - Further consideration on modifying signaling granularity of enabling/disabling/controlling inter-UE coordination scheme [vivo,4]

  - Further consideration on specifying executing order for the case when multiple UE-B’s reserved resources are collided [vivo,4]

  - Further consideration on ensuring the time difference between successive UE-B’s reserved resources fulfil the processing time budget [CATT,7]

  - Further clarification on UE-A’s behavior when the case when one of SCI(s) scheduling the same reserved resources does not fulfill the processing time budget [Fujitsu,8]

  - Further consideration on modifying re-evaluation/pre-emption procedure without using inter-UE coordination information [Intel,14]

  - Further consideration on modifying UE-B’s resource (re)selection procedure based on a SCI format 1-A [Qualcomm,22]

  - Further consideration on inter-UE coordination with mode 1 operation [Lenovo,24]

  - Further consideration on power-saving UE with inter-UE coordination information [Ericsson,27]

- Reference

- R1-2200964	Inter-UE coordination in sidelink resource allocation	Huawei, HiSilicon

- R1-2200981	Inter-UE coordination for Mode 2 enhancements	Nokia, Nokia Shanghai Bell

- R1-2200983	Discussion on techniques for inter-UE coordination	FUTUREWEI

- R1-2201112	Remaining issues on mode-2 enhancements	vivo

- R1-2201182	Inter-UE coordination for Mode 2 enhancements	Panasonic Corporation

- R1-2201255	Inter-UE coordination in mode 2 of NR sidelink	OPPO

- R1-2201336	Remaining issues on Inter-UE coordination for Mode 2 enhancements	CATT, GOHIGH

- R1-2201438	Discussion on inter-UE coordination for Mode 2 enhancements	Fujitsu

- R1-2201495	Remaining issues on sidelink resource allocation for reliability and latency	NTT DOCOMO, INC.

- R1-2201531	Discussions on remaining issues for Mode 2 inter-UE coordination	InterDigital, Inc.

- R1-2201558	Discussion on inter-UE coordination in sidelink resource allocation	Spreadtrum Communications

- R1-2201585	Discussion on inter-UE coordination for Mode 2 enhancements	Sony

- R1-2201617	Discussion on inter-UE coordination for Mode 2 enhancements	ETRI

- R1-2201716	Remaining opens of sidelink inter-UE coordination schemes	Intel Corporation

- R1-2201785	Remaining Issues of Inter-UE Coordination	Apple

- R1-2201820	Remaining issues on V2X mode 2 enhancements	ASUSTeK

- R1-2201874	Remaining issues on inter-UE coordination for mode 2 enhancement	CMCC

- R1-2201907	Discussion on mode 2 enhancements	NEC

- R1-2201920	Discussion on inter-UE coordination	Xiaomi

- R1-2202032	On Inter-UE Coordination for Mode2 Enhancements	Samsung

- R1-2202086	Discussion on Mode 2 enhancements	MediaTek Inc.

- R1-2202159	Reliability and Latency Enhancements for Mode 2	Qualcomm Incorporated

- R1-2202202	Discussion on inter-UE coordination for mode 2 enhancements	Sharp

- R1-2202231	Inter-UE coordination for Mode 2 enhancements	Lenovo, Motorola Mobility

- R1-2202245	Inter-UE coordination for mode 2 enhancements	ITL

- R1-2202253	Discussion on inter-UE coordination for Mode 2 enhancements	LG Electronics

- R1-2202263	Details on mode 2 enhancements for inter-UE coordination	Ericsson

- R1-2202356	Inter-UE coordination for enhanced resource allocation	Mitsubishi Electric RCE

- R1-2202377	Remaining issues on the inter-UE coordination	ZTE, Sanechips

- R1-2202483	Inter-UE coordination for Mode 2 enhancements	Fraunhofer HHI

- Appendix

  - Conclusions made in RAN1#103-e meeting

- Conclusion:

  - The schemes of inter-UE coordination in Mode 2 are categorized as being based on the following types of “A set of resources” sent by UE-A to UE-B:

    - UE-A sends to UE-B the set of resources preferred for UE-B’s transmission

        - e.g., based on its sensing result

    - UE-A sends to UE-B the set of resources not preferred for UE-B’s transmission

        - e.g., based on its sensing result and/or expected/potential resource conflict

    - UE-A sends to UE-B the set of resource where the resource conflict is detected

    - FFS: details of resource conflict, e.g., including type of resource conflict

    - FFS: details of sensing operation at UE-A side

    - FFS: which type(s) of resource set information is(are) beneficial/feasible to which cast type(s)

    - Note: these different types may be used in combination with each other

  - From RAN1 perspective, further study on the feasibility/benefit of inter-UE coordination is required

  - Send an LS to RAN plenary

    - Final LS in R1-2009841

- Conclusion:

  - For the schemes of inter-UE coordination identified as feasible/beneficial, at least the following aspects are further discussed.

    - How/when UE-A determines the contents of ”A set of resources”, including consideration of UL scheduling

    - When UE-A sends ”A set of resources” to UE-B, including which UE(s) sends it

    - How UE-A and UE-B are determined

    - How UE-A sends ”A set of resources” to UE-B, including container used for carrying it, implicitly or explicitly or both

    - How/when/whether UE-B receives “A set of resources” and takes it into account in the resource selection for its own transmission

    - How/whether to define the relationship between support/signaling of inter-UE coordination and cast type

  - Conclusions made in RAN1#104-e meeting

- Conclusion:

  - RAN1 concludes that the inter-UE coordination in Mode 2 is feasible, and is beneficial (e.g., reliability, etc.) compared to Rel-16 Mode 2 RA, and thus recommends specification of the feature.

    - The detailed observations can be found in the attachment of the LS

- Draft LS in R1-2102165, along with the attachment R1-2102166, is approved (with a typo fix)

  - Final LS in R1-2102168

  - Agreements made in RAN1#104bis-e meeting

- Agreement:

  - Support the following schemes of inter-UE coordination in Mode 2:

    - Inter-UE Coordination Scheme 1:

      - The coordination information sent from UE-A to UE-B is the set of resources preferred and/or non-preferred for UE-B’s transmission

        - FFS details including a possibility of down-selection between the preferred resource set and the non-preferred resource set, whether or not to include any additional information other than indicating time/frequency of the resources within the set in the coordination information

      - FFS condition(s) in which Scheme 1 is used

    - Inter-UE Coordination Scheme 2:

      - The coordination information sent from UE-A to UE-B is the presence of expected/potential and/or detected resource conflict on the resources indicated by UE-B’s SCI

        - FFS details including a possibility of down-selection between the expected/potential conflict and the detected resource conflict

      - FFS condition(s) in which Scheme 2 is used

- Agreement:

  - Study further to determine the conditions for UEs to be UE-A(s)/UE-B(s) for inter-UE coordination:

    - Details include applicable scenario(s)/inter-UE coordination scheme(s)

    - E.g., only UE(s) among the intended receiver(s) of UE-B can be a UE-A, any UE can be a UE-A, high-layer configured, etc.

      - Including the possibility of being subject to certain conditions and/or capability

- Agreement:

  - When UE-B receives the inter-UE coordination information from UE-A, consider at least one of the following options (with details FFS including possibly down-selecting/merging one or more of the options below, applicable scenario(s)/condition(s) for each option, UE behavior) for UE-B’s to take it into account in the resource (re)-selection for its own transmission

    - For scheme 1:

      - Option 1-1: UE-B’s resource(s) to be used for its transmission resource (re)-selection is based on both UE-B’s sensing result (if available) and the received coordination information

      - Option 1-2: UE-B’s resource(s) to be used for its transmission resource (re)-selection is based only on the received coordination information

      - Option 1-3: UE-B’s resource(s) to be re-selected based on the received coordination information

      - Option 1-4: UE-B’s resource(s) to be used for its transmission resource (re)-selection is based on the received coordination information

    - For scheme 2:

      - Option 2-1: UE-B can determine resource(s) to be re-selected based on the received coordination information

      - Option 2-2: UE-B can determine a necessity of retransmission based on the received coordination information

  - Agreements made in RAN1#106-e meeting

- Agreement:

  - For scheme 1, the following inter-UE coordination information signalling from UE-A is supported. FFS details including condition(s)/scenario(s) under which each information is enabled to be sent by UE-A and used by UE-B.

    - Set of resources preferred for UE-B’s transmission

    - Set of resources non-preferred for UE-B’s transmission

- Agreement:

  - For scheme 2, the following inter-UE coordination information signalling from UE-A is supported. FFS details including condition(s)/scenario(s) under which each information is enabled to be sent by UE-A and used by UE-B

    - Presence of expected/potential resource conflict on the resources indicated by UE-B’s SCI

      - FFS: UE behaviour when the presence of expected/potential resource conflict is detected by the transmitter

    - FFS: Whether to additionally support the presence of detected resource conflict on the resources indicated by UE-B’s SCI

- Agreement:

  - In scheme 1, the following is supported for UE(s) to be UE-A(s)/UE-B(s) in the inter-UE coordination information transmission triggered by an explicit request in Mode 2:

    - A UE that sends an explicit request for inter-UE coordination information can be UE-B

    - A UE that received an explicit request from UE-B and sends inter-UE coordination information to the UE-B can be UE-A

    - (Working assumption) At least a destination UE of a TB transmitted by UE-B can be UE A

    - The above feature can be enabled or disabled or controlled by (pre-)configuration

      - FFS: Details on how to support this, including (pre-)configuration signaling granularity

    - FFS: Additional details and conditions on UE-A and UE-B

  - (Working Assumption) In scheme 1, the following is supported for UE(s) to be UE-A(s)/UE-B(s) in the inter-UE coordination information transmission triggered by a condition other than explicit request reception in Mode 2:

    - A UE that satisfies the condition mentioned in the main bullet and sends inter-UE coordination information is UE-A

    - A UE that received inter-UE coordination information from UE-A and uses it for resource (re-)selection is UE-B

    - The above feature can be enabled or disabled or controlled by (pre-)configuration

      - FFS: Details on how to support this, including (pre-)configuration signaling granularity

    - FFS: Additional details and conditions on UE-A and UE-B

- Agreement:

  - In scheme 2, at least the following is supported for UE(s) to be UE-A(s)/UE-B(s) in the inter-UE coordination transmission triggered by a detection of expected/potential resource conflict(s) in Mode 2:

    - A UE that transmitted PSCCH/PSSCH with SCI indicating reserved resource(s) to be used for its transmission, received inter-UE coordination information from UE-A indicating expected/potential resource conflict(s) for the reserved resource(s), and uses it to determine resource re-selection is UE-B

    - A UE that detects expected/potential resource conflict(s) on resource(s) indicated by UE-B’s SCI sends inter-UE coordination information to UE-B, subject to satisfy one of the following conditions, is UE-A

      - (Working assumption) At least a destination UE of one of the conflicting TBs, i.e., TBs to be transmitted in the expected/potential conflicting resource(s)

        - Whether a non-destination UE of a TB transmitted by UE-B can be UE-A is (pre-)configured

      - FFS: Additional details and condition(s) on UE-A and UE-B

    - The above feature can be enabled or disabled or controlled by (pre-)configuration

      - FFS: Details on how to support this, including (pre-)configuration signaling granularity

    - FFS: Definition of expected/potential resource conflict(s) and other details (if any)

- Agreement:

  - In scheme 2, the following UE-B’s behavior in its resource (re)selection is supported when it receives inter-UE coordination information from UE-A:

    - UE-B can determine resource(s) to be re-selected based on the received coordination information

      - UE-B can reselect resource(s) reserved for its transmission when expected/potential resource conflict on the resource(s) is indicated

        - FFS: Other details (if any)

- Agreement:

  - In scheme 1, at least following UE-B’s behavior in its resource (re-)selection is supported when it receives inter-UE coordination information from UE-A:

    - For preferred resource set, the following two options are supported:

      - Option A): UE-B’s resource(s) to be used for its transmission resource (re-)selection is based on both UE-B’s sensing result (if available) and the received coordination information

        - UE-B uses in its resource (re-)selection, resource(s) belonging to the preferred resource set in combination with its own sensing result

          - UE-B uses in its resource (re-)selection, resource(s) not belonging to the preferred resource set when condition(s) are met

            - FFS: Details of condition(s)

          - This option is supported when UE-B performs sensing/resource exclusion

          - FFS: Other details (if any)

      - Option B): UE-B’s resource(s) to be used for its transmission resource (re-)selection is based only on the received coordination information

        - UE-B uses in its resource (re-)selection, resource(s) belonging to the preferred resource set

          - This option is supported at least when UE-B does not support sensing/resource exclusion

            - FFS: Whether the support is conditional or UE capability

          - FFS: Other details (if any)

      - FFS: Other option(s), and other details (if any)

    - For non-preferred resource set,

      - UE-B’s resource(s) to be used for its transmission resource (re-)selection is based on both UE-B’s sensing result (if available) and the received coordination information

        - UE-B excludes in its resource (re-)selection, resource(s) overlapping with the non-preferred resource set

          - FFS: Details including

            - Whether/how UE-B can use in its resource (re-)selection, resource(s) overlapping with the non-preferred resource set, definition of the overlap, and other details (if any)

            - When UE-B excludes in its resource (re-)selection, resource(s) overlapping with the non-preferred resource set

        - FFS: UE-B reselects in its resource (re-)selection, resource(s) to be used for its transmission when the resource(s) are fully/partially overlapping with the non-preferred resource set

      - FFS: Other option(s), and other details (if any)

- Agreement:

  - In scheme 2, at least the following is supported to determine inter-UE coordination information:

    - Among resource(s) indicated by UE-B’s SCI, UE-A considers that expected/potential resource conflict occurs on the resource(s) satisfying at least one of the following condition(s):

      - Condition 2-A-1:

        - Other UE’s reserved resource(s) identified by UE-A are fully/partially overlapping with resource(s) indicated by UE-B’s SCI in time-and-frequency

        - FFS: Other details (if any)

        - FFS: Whether/how to specify additional criteria and other details (if any) including signaling details of conflict indication

      - (Working Assumption) Condition 2-A-2:

        - Resource(s) (e.g., slot(s)) where UE-A, when it is intended receiver of UE-B, does not expect to perform SL reception from UE-B due to half duplex operation

          - FFS: Other details (if any)

      - FFS: Other condition(s)

    - FFS: Other details (if any)

- Agreement:

  - In scheme 1, at least the following is supported to determine inter-UE coordination information of preferred resource set:

    - UE-A considers any resource(s) satisfying all the following condition(s) as set of resource(s) preferred for UE-B’s transmission

      - Condition 1-A-1:

        - Resource(s) excluding those overlapping with reserved resource(s) of other UE identified by UE-A whose RSRP measurement is larger than a RSRP threshold

          - FFS: Other details (if any)

      - FFS: Condition 1-A-2:

        - Resource(s) excluding slot(s) where UE-A, when it is intended receiver of UE-B, does not expect to perform SL reception from UE-B

          - FFS: Other details (if any)

      - FFS: Condition 1-A-3:

        - Resource(s) satisfying UE-B’s traffic requirement (if available)

          - FFS: Other details (if any)

      - FFS: Other condition(s)

    - FFS: Other details (if any)

- Agreement:

  - In scheme 1, at least the following is supported to determine inter-UE coordination information of non-preferred resource set:

    - UE-A considers any resource(s) satisfying at least one of the following condition(s) as set of resource(s) non-preferred for UE-B’s transmission

      - Condition 1-B-1:

        - Reserved resource(s) of other UE identified by UE-A from other UEs’ SCI (including priority field) and RSRP measurement

          - FFS: Other details (if any)

      - FFS: Condition 1-B-2:

        - Resource(s) (e.g., slot(s)) where UE-A, when it is intended receiver of UE-B, does not expect to perform SL reception from UE-B

          - FFS: Other details (if any)

      - FFS: Other condition(s)

    - FFS: Other details (if any)

  - Agreements made in RAN1#106bis-e meeting

- Agreement:

  - For Scheme 2, PSFCH format 0 is used to convey the presence of expected/potential resource conflict on reserved resource(s) indicated by UE-B’s SCI

- Agreement:

  - For Condition 2-A-1 of Scheme 2, down-select one or more of following additional criteria to determine resource(s) where expected/potential resource conflict occurs

    - Option 1: The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) whose RSRP measurement is larger than a RSRP threshold according to the priorities included in the SCI:

      - prio_TX and prio_RX are the priorities indicated in the SCI making the overlapping reservations

      - Strive to reuse Rel-16 specification wherever possible

    - Option 2: The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) whose RSRP measurement is within a (pre)configured RSRP threshold compared to the RSRP measurement of UE-B’s reserved resource.

      - FFS: Whether the threshold depends on priority

    - Option 3: The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) and the other UE is within a distance threshold of UE-B as determined by both UEs’ SCIs.

    - Option 4: The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) whose RSRP measurement is larger a (pre)configured RSRP threshold compared to the RSRP measurement of UE-B’s reserved resource.

      - FFS: Whether the threshold depends on priority

    - FFS: In case of collisions of resources for two UEs having TBs with UE A as destination UE, if needed

- Working Assumption

  - For Condition 1-B-1 of Scheme 1, the following two options are supported

    - Option 1: Reserved resource(s) of other UE(s) identified by UE-A whose RSRP measurement is larger than a (pre)configured RSRP threshold which is determined by at least priority value indicated by SCI of the UE(s)

    - Option 2: Reserved resource(s) of other UE identified by UE-A whose RSRP measurement is smaller than a (pre)configured RSRP threshold which is determined by at least priority value indicated by SCI of the UE(s) when UE-A is a destination of a TB transmitted by the UE(s)

- Working Assumption

  - For Scheme 1 with non-preferred resource set, support following condition:

    - Condition 1-B-2:

      - Resource(s) (e.g., slot(s)) where UE-A, when it is intended receiver of UE-B, does not expect to perform SL reception from UE-B due to half duplex operation

- Agreement:

  - For Condition 1-A-1 of Scheme 1, the set of resources preferred for UE-B’s transmission is a form of candidate single-slot resource as specified in Rel-16 TS 38.214 Section 8.1.4

    - When the inter-UE coordination information transmission is triggered by UE-B’s explicit request, the candidate single-slot resource(s) are determined in the same way according to Rel-16 TS 38.214 Section 8.1.4 with at least following parameters provided by signaling from UE-B. FFS whether or not to apply RSRP threshold increase in Step 7) of Rel-16 TS 38.214 Section 8.1.4.

      - Priority value to be used for PSCCH/PSSCH transmission

        - It replaces prio_TX

      - Number of sub-channels to be used for PSSCH/PSCCH transmission in a slot

        - It replaces L_subCH

      - Resource reservation interval

        - It replaces P_rsvp_TX

      - FFS: Starting/ending time location of resource selection window

    - FFS : In addition to Rel-16 procedure, use inter-UE coordination information from other UEs

      - If there is no consensus in RAN1#106bis-e, no further discussions for Rel-17

- Conclusion:

  - No consensus that UE-A uses inter-UE coordination information from other UEs when it determines the preferred resource set for Condition 1-A-1 of Scheme 1.

- Working Assumption

  - For Scheme 1 with preferred resource set, support following condition:

    - Condition 1-A-2:

      - Resource(s) excluding slot(s) where UE-A, when it is intended receiver of UE-B, does not expect to perform SL reception from UE-B due to half duplex operation

      - This can be disabled by RRC (pre-)configuration

- Agreement:

  - For allocating PSFCH resources in Scheme 2, at least following can be (pre)configured separately from those for SL HARQ-ACK feedback.

    - Set of PRBs for PSFCH transmission/reception (sl-PSFCH-RB-Set)

- Agreement:

  - For Scheme 2,

    - Index of a PSFCH resource for inter-UE coordination information transmission is determined in the same way according to Rel-16 TS 38.213 Section 16.3 with at least following modification

      - P_ID is L1-Source ID indicated by UE-B’s SCI

      - M_ID is 0

    - FFS: How to set m_CS

    - FFS: How to set m_0

    - FFS: Whether M_ID can be (pre)configured

  - Agreements made in RAN1#107-e meeting

- Agreement:

  - A resource pool level (pre-)configuration uses either of the following options

    - Option 1: PSFCH occasion is derived by a slot where UE-B’s SCI is transmitted

      - Reuse PSSCH-to-PSFCH timing as specified in TS 38.213 Section 16.3 to determine the PSFCH occasion for resource conflict indication

      - Time gap between the PSFCH and a slot where expected/potential resource conflict occurs is larger than or equal to T_3

    - Option 2: PSFCH occasion is derived by a slot where expected/potential resource conflict occurs on PSSCH resource indicated by UE-B’s SCI

      - UE-A transmits the PSFCH in a latest slot that includes PSFCH resources for inter-UE coordination information and is at least T_3 slots of the resource pool before the PSSCH resource indicated by UE-B’s SCI in which expected/potential resource conflict occurs

      - FFS: How to account for processing timeline

    - Note that it is possible not to configure either option1 or option 2.

- Agreement:

  - For Condition 1-A-2 of Scheme 1, the set of resources preferred for UE-B’s transmission is a form of candidate single-slot resource as specified in Rel-16 TS 38.214 Section 8.1.4

    - UE-A excludes candidate single-slot candidate(s) belonging to “slot(s) where UE-A, when it is intended receiver of UE-B, does not expect to perform SL reception from UE-B due to half duplex operation” after Step 6) of TS 38.214 Section 8.1.4

- Agreement:

  - When PSFCH TX/RX for Scheme 2 is overlapping with LTE SL TX/RX and/or UL in a UE, reuse prioritization rule as specified in TS 38.213 Section 16.2.4.1 and 16.2.4.3.1.

- Conclusion:

  - For Scheme 2, the values of the following parameters are the same as those for SL HARQ-ACK feedback in the same resource pool

    - Period of PSFCH resources (sl-PSFCH-Period)

    - Number of cyclic shift pairs used for a PSFCH transmission that can be multiplexed in a PRB (sl-NumMuxCS-Pair)

    - Number of PSFCH resources available for multiplexing information in a PSFCH transmission (sl-PSFCH-CandidateResourceType)

- Agreement:

  - For Scheme 1, a resource pool level (pre-)configuration can enable one of the following alternatives:

    - Alt 1 (Working Assumption): MAC CE or 2nd SCI are used as the container of inter-UE coordination information transmission from UE A to UE B.

      - For the indication of resource set, the following is supported:

        - N combinations of TRIV, FRIV, resource reservation period as specified in Rel-16 TS 38.214 Section 8.1.5 with following modification. The value of resource reservation period is omitted at least when the transmission of preferred resource set is triggered by UE-B’s explicit request.

          - First resource location of each TRIV is separately indicated by the inter-UE coordination information

        - If [N <= 3], MAC CE is used and it is up to UE implementation to additionally use 2nd SCI. When 2nd SCI and MAC CE are both used, the same resource set is indicated in the 2nd SCI and the MAC CE. If [N > 3], only MAC CE is used.

          - FFS: UE capability details

          - 2nd SCI is UE RX optional

    - Alt 2: MAC CE is used as the container of inter-UE coordination information transmission from UE A to UE B.

      - For the indication of resource set, the following is supported:

        - N combinations of TRIV, FRIV, resource reservation period as specified in Rel-16 TS 38.214 Section 8.1.5 with following modification. The value of resource reservation period is omitted at least when the transmission of preferred resource set is triggered by UE-B’s explicit request.

          - First resource location of each TRIV is separately indicated by the inter-UE coordination information

    - FFS: Whether/How to use resource reservation information as coordination information

- Working Assumption:

  - A resource pool level (pre-)configuration can enable one of the following options:

    - Option 1:

      - For Condition 2-A-1 of Scheme 2, support following additional criteria to determine resource(s) where expected/potential resource conflict occurs

        - For the case when UE-A is a destination UE of a TB transmitted by UE-B

          - The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) whose RSRP measurement is larger than a RSRP threshold according to the priorities included in the SCI:

            - prio_TX and prio_RX are the priorities indicated in the SCI making the overlapping reservations for UE-B and other UE respectively

        - For the case when UE-A is a destination UE of a TB transmitted by another UE

          - The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) when RSRP measurement of UE-B’s reserved resource is larger than a RSRP threshold according to the priorities included in the SCI:

            - prio_TX and prio_RX are the priorities indicated in the SCI making the overlapping reservations for other UE and UE-B respectively

    - Option 4:

      - For Condition 2-A-1 of Scheme 2, support following additional criteria to determine resource(s) where expected/potential resource conflict occurs

        - For the case when UE-A is a destination UE of a TB transmitted by UE-B

          - The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) whose RSRP measurement is larger than a (pre)configured RSRP threshold compared to the RSRP measurement of UE-B’s reserved resource.

        - For the case when UE-A is a destination UE of a TB transmitted by another UE

          - The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) when RSRP measurement of UE-B’s reserved resource is larger than a (pre)configured RSRP threshold compared to the RSRP measurement of the resource(s).

      - Support of Option 4 is subject to UE capability

    - FFS: Whether/how RSRP threshold depends on priority, MCS, overlap

- Agreement:

  - For Scheme 1 with non-preferred resource set,

    - Physical layer at UE-B excludes in its resource (re-)selection, candidate single-slot resource(s) obtained after Step 6) of Rel-16 TS 38.214 Section 8.1.4 overlapping with the non-preferred resource set

- Agreement:

  - For Condition 1-A-1 of Scheme 1, when UE-A determines the set of resources preferred for UE-B’s transmission, apply RSRP threshold increase in the same way according to Rel-16 TS 38.214 Section 8.1.4.

    - FFS: Whether/how to introduce the maximum limit of RSRP threshold increase

- Agreement:

  - For Scheme 1, at least following parameters are provided by UE-B’s request:

    - Priority value to be used for PSCCH/PSSCH transmission

    - Number of sub-channels to be used for PSSCH/PSCCH transmission in a slot

    - Resource reservation interval

- Agreement:

  - For Scheme 2, when PSFCH occasion is derived by a slot where expected/potential resource conflict occurs on PSSCH resource indicated by UE-B’s SCI,

    - Time gap between the PSFCH and SCI(s) scheduling conflicting TBs is larger than or equal to X value.

      - FFS: Details of X

- Working Assumption:

  - For Condition 2-A-1 in Scheme 2, when “a non-destination UE of a TB transmitted by UE-B can be UE-A” is enabled or when “a non-destination UE of a TB transmitted by UE-B can be UE-A” is disabled and the destination UE of the conflicting TBs is UE-A, for each pair of UEs scheduling the conflicting TBs, a UE with the higher priority value is UE-B.

    - FFS whether/how to set additional condition for UE-A to send PSFCH.

    - Conclude on whether/how to handle, or differently handle, the case when at least one of UEs scheduling conflicting TBs doesn’t support Scheme 2 at the subsequent meetings

- Agreement:

  - For inter-UE coordination information triggered by an explicit request in Scheme 1,

    - UE-A uses a TX resource pool used for UE-B’s request transmission to determine the set of resources and to transmit the set of resources to UE-B

- Agreement:

  - For inter-UE coordination information triggered by a condition rather than request reception in Scheme 1,

    - UE-A transmitting in a resource pool provides inter-UE coordination information associated with the same resource pool

  - Agreements made in RAN#94-e meeting

- Agreement:

  - RAN1 is tasked to complete the remaining normative work for Rel-17 NR sidelink enhancement by Q1 of 2022

    - All RAN1 decisions that impact other WGs should be finalized in RAN1#107bis-e

  - Use the list of open issues provided RP-212880 (status report of WI: NR sidelink enhancement) as a starting point for technical discussions in RAN1.

    - This does not mean that all the issues included in the list are considered essential or the list is complete

    - RAN1 should not spend additional effort to further refine the list

  - Agreements made in RAN1#107bis-e meeting

- Agreement:

  - For Scheme 1, when the inter-UE coordination information transmission is triggered by UE-B’s explicit request,

    - Starting/Ending time locations of resource selection window is provided by UE-B’s explicit request

      - Starting/Ending time locations of resource selection window is a form of combination of DFN index and slot index

- Agreement:

  - When PSFCH occasion is derived by a slot where expected/potential resource conflict occurs on PSSCH resource indicated by UE-B’s SCI, time gap between the PSFCH and SCI(s) scheduling conflicting TBs is larger than or equal to X value

    - X = sl-MinTimeGapPSFCH

  - UE does not transmit the conflict indicator or receive the conflict indicator if the timeline is not satisfied

- Agreement:

  - For Scheme 1, a resource pool level (pre-)configuration can enable one of the following alternatives:

    - (Working assumption) Alt1: MAC CE and 2nd SCI are used as the container of an explicit request transmission from UE-B to UE-A

      - A single format SCI 2-C is used for inter-UE coordination information and request

        - 1 bit in format 2-C is used to indicate whether the SCI is used for request to coordination information or for conveying coordination information

      - SCI 2-C is UE RX optional

      - It is up to UE implementation to additionally use 2nd SCI (for UE-B).

    - Alt2: MAC CE is used as the container of an explicit request transmission from UE-B to UE-A

- Conclusion:

  - For Scheme 2, there is no consensus to support indication of the following

    - Condition type of a resource conflict

    - Time location of a resource conflict

- Agreement:

  - Alt 2-1

    - For Scheme 2,

      - The PHY layer reports S_A after Step 7) of TS 38.214 Section 8.1.4 to higher layer.

      - When UE-B receives a conflict indicator for resource(s) indicated by its SCI,

        - PHY layer at UE-B reports resources overlapping with the next reserved resource indicated by the corresponding UE-B’s SCI for current TB transmission to higher layer.

          - If (pre)configured, the PHY layer reports resources in a slot including the next reserved resource indicated by the corresponding UE-B’s SCI for current TB transmission to higher layer.

        - Higher layer at UE-B re-selects the resource(s) indicated by the conflict indicator among the S_A excluding the reported resources.

      - FFS: Whether/How the conflict in periodic transmission is indicated by UE-A and handled by UE-B

- Agreement:

  - For PSFCH TX/RX or TX/TX prioritization in Scheme 2,

    - Priority value of PSFCH TX for a resource conflict indication is the smallest priority value of the conflicting TBs

    - Priority value of PSFCH RX for a resource conflict indication is priority value indicated by UE-B’s SCI

    - For PSFCH TX/RX or TX/TX prioritization between SL HARQ-ACK feedback(s) and resource conflict indication(s), PSFCH TX/RX for SL HARQ-ACK feedback is always prioritized over PSFCH TX/RX for a resource conflict indication

- Agreement:

  - For Scheme 1, unicast is supported for an explicit request transmission for inter-UE coordination information

    - Unicast is used for the inter-UE coordination information transmission triggered by the explicit request

- Working Assumption:

  - For Scheme 1, following cast type(s) are supported for inter-UE coordination information transmission triggered by a condition other than explicit request reception

    - Groupcast/Broadcast for non-preferred resource set, FFS for preferred resource set

      - FFS: Under which conditions groupcast/broadcast can be supported

    - Unicast

      - FFS: Under which conditions unicast can be supported

- Agreement:

  - For determining preferred resource set in Scheme 1, the value of Cresel is determined by UE-A according to Rel-16 procedure.

    - This information is not conveyed to/from UE-B

    - When inter-UE coordination information is triggered by UE-B’s request, P_rsvp_TX used for determining SL_RESOURCE_RESELECTION_COUNTER according to Rel-16 procedure is provided by resource reservation interval indicated by UE-B’s request

- Agreement:

  - For the indication of resource set in Scheme 1, the value of Sl-MaxNumPerReserve is fixed to 3.

- Agreement:

  - The following working assumption is confirmed with modification in RED.

    - MAC CE or 2nd SCI are used as the container of inter-UE coordination information transmission from UE A to UE B.

      - For the indication of resource set, the following is supported:

        - N combinations of TRIV, FRIV, resource reservation period as specified in Rel-16 TS 38.214 Section 8.1.5 with following modification. The value of resource reservation period is omitted at least when the transmission of preferred resource set is triggered by UE-B’s explicit request.

          - First resource location of each TRIV is separately indicated by the inter-UE coordination information

        - If [N <= 3], MAC CE is used and it is up to UE implementation to additionally use 2nd SCI. When 2nd SCI and MAC CE are both used, the same resource set is indicated in the 2nd SCI and the MAC CE. If [N > 3], only MAC CE is used.

          - FFS: UE capability details

          - 2nd SCI is UE RX optional

          - The field size of the indication of resource set in a SCI format 2-C is determined by [N=3]

- Agreement:

  - For inter-UE coordination information transmission in Scheme 1,

    - Inter-UE coordination information can be multiplexed with other data only if the source/destination ID pair is the same

      - Retransmission of the TB carrying inter-UE coordination information is supported

  - For explicit request transmission in Scheme 1,

    - Explicit request can be multiplexed with other data only if the source/destination ID pair is the same

      - Retransmission of the TB carrying request is supported

- Agreement:

  - For inter-UE coordination triggered by an explicit request in Scheme 1, whether or not to transmit the inter-UE coordination information upon the request reception is determined by UE-A’s implementation subject to the following procedures.

    - Rel-16 procedure of UL/SL prioritization, LTE SL/NR SL prioritization, and congestion control

- Agreement:

  - For inter-UE coordination triggered by a condition rather than request reception in Scheme 1,

    - A resource pool level (pre-)configuration can enable one of the following alternatives:

      - Alt 1: it is up to UE-A’s implementation whether or not to trigger the inter-UE coordination information generation.

      - Alt 2: the inter-UE coordination information generation can be triggered only when UE-A has data to be transmitted together with the inter-UE coordination information to UE-B

    - Note: Rel-16 procedure of UL/SL prioritization, LTE SL/NR SL prioritization, and congestion control is applied to the transmission of the inter-UE coordination information triggered by a condition.

- Agreement:

  - For inter-UE coordination triggered by UE-B’s explicit request in Scheme 1,

    - A resource pool level (pre-)configuration can enable one of the following alternatives:

      - Alt 1: it is up to UE-B’s implementation whether or not to trigger the request generation

      - Alt 2: the request generation can be triggered only when UE-B has data to be transmitted to UE-A

    - Note: Rel-16 procedure of UL/SL prioritization, LTE SL/NR SL prioritization, and congestion control is applied to the transmission of the request transmission.

- Agreement:

  - For Scheme 1 with preferred resource set Option A,

    - MAC layer selects resources using S_A and the received preferred resource set

      - MAC layer firstly selects resources for transmissions within the intersection of S_A and the preferred resource set until it becomes impossible to select a resource within the intersection under the constraint defined in Rel-16.

        - It is up to the UE whether to use the preferred resource set from SCI format 2-C and/or MAC CE

      - After this, if the number of selected resources is smaller than the required number of transmissions for a TB, MAC layer selects resources for the remaining transmissions outside the intersection but inside S_A under the constraint defined in Rel-16.

- Agreement:

  - For Scheme 1 with preferred resource set Option B,

    - MAC layer selects resources belonging to the received preferred resource set under the constraint defined in Rel-16

      - It is up to the UE whether to use the preferred resource set from SCI format 2-C and/or MAC CE

- Agreement:

  - For inter-UE coordination information triggered by an explicit request in Scheme 1, the priority value of the inter-UE coordination information is (pre)configured priority value if it is provided by (pre)configuration. Otherwise, the priority value is the same as indicated by UE-B’s explicit request.

    - For the case when inter-UE coordination information is transmitted together with other data, the priority value of the multiplexed sidelink transmission is determined by the smallest priority value between the inter-UE coordination information and data

- Agreement:

  - For inter-UE coordination information triggered by an explicit request in Scheme 1, the priority value of explicit request is (pre)configured priority value if it is provided by (pre)configuration. Otherwise, the priority value is the same as that of a TB to be transmitted by UE-B.

    - For the case when the explicit request is transmitted together with other data, the priority value of the multiplexed sidelink transmission is determined by the smallest priority value between the explicit request and data

- Agreement:

  - For inter-UE coordination information triggered by a condition other than explicit request reception in Scheme 1, the priority value of the inter-UE coordination information is (pre)configured priority value if it is provided by (pre)configuration.

    - FFS: Otherwise, the priority value is determined by UE-A’s implementation.

    - For the case when inter-UE coordination information is transmitted together with other data, the priority value of the multiplexed sidelink transmission is determined by the smallest priority value between the inter-UE coordination information and data

- Agreement:

  - For sidelink transmission carrying inter-UE coordination information in Scheme 1,

    - UE-A performs its resource (re)selection according to the same procedure in TS 38.214 Section 8.1.4 to transmit the inter-UE coordination information to UE-B.

  - For sidelink transmission carrying request in Scheme 1,

    - UE-B performs its resource (re)selection according to the same procedure in TS 38.214 Section 8.1.4 to transmit the request for the inter-UE coordination information to UE-A if UE-B performs sensing/resource exclusion. Otherwise, at least UE-B can perform random selection

  - Note: RAN1 does not pursue specific enhancement of Rel-17 resource (re)selection for the transmission of inter-UE coordination information and its request.

- Working assumption:

  - First resource location of each TRIV is a slot offset with respect to a reference slot

    - Alt 2:

      - The slot offset is the number of logical slots from the reference slot

        - The value range of slot offsets is from 0 to maximum value that is (pre)configurable up to [256]

          - FFS: The detailed value range including granularity

        - Slot offset for each TRIV to indicate the set of resources is separately indicated by inter-UE coordination information

    - For the reference slot,

      - The reference slot is the slot indicated by the inter-UE coordination information in a form of combination of DFN index and slot index

- Agreement:

  - For determining preferred resource set in Scheme 1, when inter-UE coordination information transmission is triggered by a condition other than explicit request reception,

    - Values of following parameters are (pre)configured for a resource pool. If there is no (pre)configuration, UE-A determines by its implementation the values of the following parameters

      - prio_TX

      - L_subCH

      - P_rsvp_TX

    - UE-A determines by its implementation values of following parameters

      - n+T_1, n+T_2

    - FFS: Whether/how to support (pre)configuration of n+T_1 and n+T_2

    - Note that it is up to RAN2 decision whether/how the values of these parameters are provided by PC5-RRC signaling from UE-B to UE-A and UE-A uses the received information to determine the preferred resource set

- Agreement:

  - For inter-UE coordination information is triggered by UE-B’s request,

    - A resource pool level (pre-)configuration can enable one of the following alternatives:

      - Alt 1:

        - Resource set type to be provided by inter-UE coordination information transmission is determined by UE-A’s implementation and its information is indicated by UE-A’s inter-UE coordination information

          - UE-A’s inter-UE coordination information indicates either preferred resource set or non-preferred resource set

      - Alt 2:

        - Resource set type to be provided by inter-UE coordination information transmission is indicated by UE-B’s request

          - UE-B’s request indicates either preferred resource set or non-preferred resource set

    - Note that it is up to RAN2 decision whether/how UE-B provides its support of sensing/resource exclusion to UE-A via PC5-RRC signaling and UE-A uses the received information to determine the type of resource set to be transmitted to UE-B

- Agreement:

  - For inter-UE coordination information is triggered by a condition other than explicit request reception,

    - Resource set type to be provided by inter-UE coordination information transmission is determined by UE-A’s implementation and its information is indicated by UE-A’s inter-UE coordination information

      - UE-A’s inter-UE coordination information indicates either preferred resource set or non-preferred resource set

- Working assumption:

  - For Scheme 2, (pre)configuration is supported to enable or disable that 1 LSB of reserved bits of a SCI format 1-A is used to indicate of whether UE scheduling a conflict TB can be UE-B or not.

    - FFS: UE-A's behavior for the case when at least one of UEs scheduling conflicting TBs is not capable of receiving the conflict indication

  - Agreements made in RAN1#108-e meeting

- Agreement:

  - For a slot offset that is (pre)configured to indicate the first resource location of each TRIV with respect to a reference slot,

    - Granularity of the slot offset is 1 logical slot

    - (Pre)configured maximum value of the slot offset is up to 8000

      - When both SCI format 2-C and MAC CE are used as the container of inter-UE coordination information, the maximum value of the slot offset is 255

      - When MAC CE only is used as the container of inter-UE coordination information, the maximum value of the slot offset is the (pre)configured maximum value

- Agreement:

  - A SCI format 2-C includes all the fields present in SCI format 2-A except cast type indicator

- Conclusion:

  - For cast type(s) of inter-UE coordination information with preferred resource set triggered by a condition other than explicit request reception

    - There is no consensus in RAN1 on the support of groupcast or broadcast for preferred resource set

- Agreement

  - For Scheme 2,

    - m_CS for a resource conflict indication for the next reserved resource indicated by the corresponding UE-B’s SCI for either current TB transmission or next TB transmission is 0

- Agreement

  - For Scheme 2, when UE-B receives a conflict indicator for resource(s) indicated by its SCI, it up to UE-B’s implementation whether/how to set the reservation periodicity in the re-selected resource.

- Agreement

  - For Scheme 2,

    - m_0 for a resource conflict indication is derived in the same way as specified for HARQ-ACK information in TS 38.213 Section 16.3

    - A UE expects that different PRBs are (pre)configured between conflict indication and HARQ-ACK information

- Agreement:

  - For Scheme 1, when both SCI format 2-C and MAC CE are used as the container of an explicit request for inter-UE coordination information, the same bit field size for the request in a SCI format 2-C is applied to MAC CE

- Agreement:

  - For Scheme 1, when MAC CE only is used as the container of an explicit request for inter-UE coordination information, the same bit field size for the request in a SCI format 2-C is applied to MAC CE

- Conclusion:

  - For inter-UE coordination operation in Rel-17, RAN1 understands that only UE(s) in mode 2 can be UE-A

    - Note that RAN1 does not pursue specific enhancement of Rel-17 inter-UE coordination operation for handling the case where UE(s) in mode 1 can be UE-A

- Agreement

  - Confirm the following working assumption with modification in RED

    - Working assumption made in RAN1#107bis-e:

      - First resource location of each TRIV is a slot offset with respect to a reference slot

        - Alt 2:

          - The slot offset is the number of logical slots from the reference slot

            - The value range of slot offsets is from 0 to maximum value that is (pre)configurable up to [8000256]

              - FFS: The detailed value range including granularity

          - Slot offset for each TRIV except for first TRIV to indicate the set of resources is separately indicated by inter-UE coordination information

            - Slot offset for first TRIV is 0

        - For the reference slot,

          - The reference slot is the slot indicated by the inter-UE coordination information in a form of combination of DFN index and slot index

- Agreement

  - MAC CE or 2nd SCI are used as the container of inter-UE coordination information transmission from UE A to UE B.

    - For the indication of resource set, the following is supported:

      - N combinations of TRIV, FRIV, resource reservation period as specified in Rel-16 TS 38.214 Section 8.1.5 with following modification. The value of resource reservation period is omitted at least when the transmission of preferred resource set is triggered by UE-B’s explicit request.

        - First resource location of each TRIV is separately indicated by the inter-UE coordination information

      - If N <= 2, MAC CE is used and it is up to UE implementation to additionally use 2nd SCI. When 2nd SCI and MAC CE are both used, the same resource set is indicated in the 2nd SCI and the MAC CE. If N > 2, only MAC CE is used.

        - FFS: UE capability details

        - 2nd SCI is UE RX optional

        - The field size of the indication of resource set in a SCI format 2-C is determined by N=2

- Agreement

  - For Scheme 1, each bit field size of a SCI format 2-C for inter-UE coordination information is given by following table:

    - Note that lowest subchannel index for the first resource location of each TRIV is separately indicated by inter-UE coordination information

| Field name | Field size (in bits) |
| --- | --- |
| Providing/requesting indicator | 1 |
| Resource combination(s) | 2*log2(N subchannel SLNsubchannel SL + 12Nsubchannel SL + 16)+9+Y Where N subchannel SL is provided by the higher layer parameter sl-NumSubchannel, Y=log2Nrsv_period with that Nrsv_period is the number of entries in the higher layer parameter sl-ResourceReservePeriodList, if higher layer parameter sl-MultiReserveResource is configured; Y=0 otherwise. |
| First resource location(s) | 8 |
| Reference slot location | 10+log2(10∙2μ) Where μ is 0, 1, 2, 3 for SCS of 15kHz, 30kHz, 60kHz, 120kHz, respectively. |
| Resource set type | 1 |
| Lowest subchannel indices for the first resource location of each TRIV | 2*log2NsubchannelSL where NsubchannelSL is provided by the higher layer parameter sl-NumSubchannel |
| (FFS) Actual number of resource combination | 1 Note: Support of this field is to be concluded by Feb 28. |

Agreement

  - For Scheme 1, each bit field size of a SCI format 2-C for an explicit request for inter-UE coordination information is given by following table:

| Field name | Field size (in bits) |
| --- | --- |
| Providing/requesting indicator | 1 |
| Priority | 3 |
| Number of subchannels | log2(N subChannel SL) Where N subChannel SL is provided by the higher layer parameter sl-NumSubchannel |
| Resource reservation period | Y Where Y=log2Nrsv_periodwith that Nrsv_period is the number of entries in the higher layer parameter sl-ResourceReservePeriodList, if higher layer parameter sl-MultiReserveResoure is configured; Y=0 otherwise. |
| Resource selection window location | 210+log2(10∙2μ) Where μ is 0, 1, 2, 3 for SCS of 15kHz, 30kHz, 60kHz, 120kHz, respectively. |
| Resource set type | 1 bit if determineResourceSetTypeScheme1 is set to ‘UE-B’s request’, otherwise, 0 bit |

- This agreement does not imply that new field requested by RAN2 cannot be further added.

Agreement

  - For Scheme 1, when MAC CE only is used as the container of inter-UE coordination information, each bit field size for inter-UE coordination information is given by following table from RAN1’s perspective, and RAN1 understands that the maximum value of N resource combinations to be conveyed in inter-UE coordination information is bounded so that the total payload size of inter-UE coordination information leads not to exceed the size of TB including the MAC CE

    - Details (e.g., whether/how to separately indicate the value of N in the inter-UE coordination information, how to put the following fields into MAC CE and the related field sizes in MAC CE) are up to RAN2

| Field name | Field size (in bits) |
| --- | --- |
| Providing/requesting indicator | 1 |
| Resource combination(s) | N*log2(N subChannel SLN subChannel SL + 12N subChannel SL + 16)+9+Y Where N subChannel SL is provided by the higher layer parameter sl-NumSubchannel, Y=log2Nrsv_periodwith that Nrsv_period is the number of entries in the higher layer parameter sl-ResourceReservePeriodList, if higher layer parameter sl-MultiReserveResoure is configured; Y=0 otherwise. |
| First resource location(s) | N-1*log2(X) Where X is provided by the (pre)configured maximum value of slot offset for the case when MAC CE only is used as a container of inter-UE coordination information |
| Reference slot location | 10+log2(10∙2μ) Where μ is 0, 1, 2, 3 for SCS of 15kHz, 30kHz, 60kHz, 120kHz, respectively. |
| Resource set type | 1 |
| Lowest subchannel indices for the first resource location of each TRIV | N*log2NsubchannelSL Where NsubchannelSL is provided by the higher layer parameter sl-NumSubchannel. |