3GPP TSG-RAN WG1 Meeting #108-e	Tdoc R1-2202272

e-Meeting, February 21st– March 3rd 2022

Agenda Item:	8.1.1

Source:	Ericsson

Title:	Remaining issues on beam management enhancements for multi-TRP

Document for:	Discussion

# 1	Introduction

The Rel-17 feMIMO WID [1] objective 2c states:

- Evaluate and, if needed, specify beam-management-related enhancements for simultaneous multi-TRP transmission with multi-panel reception

In this paper, we provide Ericsson’s views on the remaining issues related to this objective considering the progress made on this topic until RAN1#107-e.

# 2	Discussion

## 2.1	RRC parameter for number of BFD RS for implicit determination

In [2], for implicit BFD-RS determination, the latest Rel-17 specifications for 38.213 specifies the following in Clause 6:

“The UE expects the set q0,0 or the set q0,1 to include up to a number of NBFD RS indexes indicated by capabilityparametername.”

In the above sentence, the number of RS indices to be included in each of q0,0 and q0,1 is directly specified in terms of the corresponding capability reported by the UE.  However, this violates the following guidance provided by RAN2 in an incoming LS [3]:

5 	Avoid defining functionality that has no RRC configuration but is dependent on capability bits.

The specification should not be written so that the network determines what configuration it can use for a UE implicitly by the reported UE capabilities. Instead, the gNB should always configure the UE explicitly by DL RRC signalling, respecting  the reported capabilities.

A problematic case in Rel-15 was the UL/DL MIMO layers, which resulted in a late-stage introduction of explicit MIMO signalling support by RAN2 (maxLayersMIMO-Indication).

According to the above guidance from RAN2, the gNB should explicitly configure the UE with the number NBFD of RS indices to be included in each of the sets  q0,0 and q0,1, respecting the capability reported by the UE.  Hence, a new RRC parameter is needed for the gNB to configure number NBFD of RS indices to be included in the BFD-RS sets for implicit BFD RS determination.  The supported values for this RRC parameter can be determined after convergence is reached in the UE feature discussion.

According to RAN2 guidance, RAN1 should avoid defining functionality of how many BFD-RSs should be included in the BFD-RS purely based on UE capability bits without any RRC configuration.

Introduce a new RRC parameter for the gNB to configure the number NBFD of RS indices to be included in the BFD-RS sets for implicit BFD RS determination.

In addition, the following text proposal should be adopted in 38.213 Clause 6.

************************* Start of Text Proposal 1 for 38.213 Clause 6 *******************************************

6	Link recovery procedures

************************************ Unchanged Text Omitted ******************************************************

The UE expects the set q0,0 or the set q0,1 to include up to a number of NBFD RS indexes configured by higher layer parameter numBFD-RSImplicit, subject to UE capability.

************************************* End of Text Proposal 1 *******************************************************

Adopt Text Proposal 1 in Clause 6 of TS 38.213 V17.0.0.

## 2.2	Configuration of two BFD-RS sets

One open issue being discussed in RAN2 is how to configure the two BFD-RS sets [4].  It should be noted that in the current RRC excel sheet provided by RAN1, the RRC parameters are not provided for how to configure the two BFD-RS sets.  There are two options being currently discussed by RAN2:

- Option 1:  Introducing the second BFD-RS set within radioLinkMonitoringConfig parameter in BWP-DownlinkDedicated

- Option 2:  Introducing two radioLinkMonitoringConfig parameters (i.e., radioLinkMonitoringConfig and radioLinkMonitoringConfig2) within BWP-DownlinkDedicated

While down-selection among one of the two options can be left up to RAN2, configuration parameters for the two BFD-RS sets need to be reflected in 38.213 V17.0.0.  Hence, the following text proposal should be adopted in 38.213 Clause 6.

************************* Start of Text Proposal 2 for 38.213 Clause 6 *******************************************

6	Link recovery procedures

************************************ Unchanged Text Omitted ******************************************************

Instead of the sets q0 and q1, for each BWP of a serving cell, the UE can be provided respective two sets q0,0 and q0,1 of periodic CSI-RS resource configuration indexes by failureDetectionResourcesToAddModList and failureDetectionResourcesToAddModList2 and corresponding two sets q1,0 and q1,1 of periodic CSI-RS resource configuration indexes and/or SS/PBCH block indexes by candidateBeamRSList1 and candidateBeamRSList2, respectively, for radio link quality measurements on the BWP of the serving cell. The set q0,0 is associated with the set q1,0 and the set q0,1 is associated with the the set q1,1.

************************************ Unchanged Text Omitted ******************************************************

If the UE is not provided q0 by failureDetectionResourcesToAddModList for a BWP of the serving cell, the UE determines the set q0 to include periodic CSI-RS resource configuration indexes with same values as the RS indexes in the RS sets indicated by TCI-State for respective CORESETs that the UE uses for monitoring PDCCH. If the UE is not provided q0,0 by failureDetectionResourcesToAddModList or  q0,1 by failureDetectionResourcesToAddModList2 for a BWP of the serving cell, the UE determines the set q0,0 or  q0,1 to include periodic CSI-RS resource configuration indexes with same values as the RS indexes in the RS sets indicated by TCI-State for first and second CORESETs that the UE uses for monitoring PDCCH, where the UE is provided two coresetPoolIndex values 0 and 1 for the first and second CORESETs, or is not provided coresetPoolIndex value for the first CORESETs and is provided coresetPoolIndex value of 1 for the second CORESETs, respectively.

************************************* End of Text Proposal 2 *******************************************************

Adopt Text Proposal 2 in Clause 6 of TS 38.213 V17.0.0.

# Conclusion

In the previous sections we made the following observations:

Observation 1	According to RAN2 guidance, RAN1 should avoid defining functionality of how many BFD-RSs should be included in the BFD-RS purely based on UE capability bits without any RRC configuration.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Introduce a new RRC parameter for the gNB to configure the number NBFD of RS indices to be included in the BFD-RS sets for implicit BFD RS determination.

Proposal 2	Adopt Text Proposal 1 in Clause 6 of TS 38.213 V17.0.0.

Proposal 3	Adopt Text Proposal 2 in Clause 6 of TS 38.213 V17.0.0.

# References

RP-211586, Revised WID: Further enhancements on MIMO for NR, Samsung, RAN#92e, June 2021.

3GPP TS 38.213 V17.0.0, NR; Physical layer procedures for control (Release 17), December 2021.

R1-2001513, Guidelines for UE capability definitions, RAN2.

RAN2 email discussion on feMIMO RRC Open Issues (Link)