3GPP TSG RAN WG1 Meeting #108-e	R1-2202307

e-Meeting, February 21th – March 3rd, 2022

Title:	Draft reply LS on range of power control parameters for eIAB

Response to:	R4-2203020

Release:	Rel-17

Work Item:	NR_IAB_enh -Core

Source:	LG Electronics [RAN1]

To:	RAN4

Contact Person:

Name:

E-mail Address:

Attachments:	None

Overall description

RAN1 would like to provide response on the following question from RAN4 in R4-2203020:

RAN4 kindly asks RAN1 to consider above answers in future discussion and feedback on RAN4 question. Besides above reply, there is question raised by company for clarification to desired DL Tx power adjustment from an IAB-node to a parent node as: there are RAN4 requirements for IAB-DU receiver by which the RX PSD difference between two FDM signals can be derived. Such RX PSD difference range could be regarded as basic PSD difference can be tolerated by child IAB receiver and thus the desired DL TX power adjustment could be derived to avoid higher PSD difference to be tolerated by child IAB receiver compared to the basic PSD difference. Whether this information of basic PSD difference is needed in addition to desired DL TX power adjustment range from RAN1 perspective?

Response:

It is RAN1’s understanding that desired DL TX power adjustment does not mandate transmit power of parent IAB-DU, and the range of desired DL TX power is determined by IAB-MT within the given range. Therefore the basic PSD difference, mentioned by RAN4, is not needed for DL TX power adjustment range if the basic PSD difference is the RAN4 requirements for IAB-MT. In that perspective, there are several questions regarding the basic PSD difference for further clarification.

Q1. Is the basic PSD difference RAN4 requirements for IAB-MT?

Q2. If the basic PSD difference is not RAN4 requirement for IAB-MT, under what circumstance is it applied for IAB-MT? (For example, when IAB-MT and IAB-DU shares same RF)

Q3. What is the impact of IAB-MT if the transmit power of parent IAB-DU exceeds basic PSD difference?

Actions

To RAN4

Actions: RAN1 respectfully asks RAN4 to take the above information into account in their further work.

Date of next TSG-RAN WG1 meetings:

TSG-RAN WG1 Meeting#109-e,			16th  – 27th May 2022			Electronic Meeting