3GPP TSG RAN WG1 #108-e			R1-2202353

e-Meeting, February 21st – March 3rd, 2022

Agenda Item:	8.13.1

Source: 	LG Electronics

Title: 	Discussion on cross-carrier scheduling from SCell to PCell

Document for:	Discussion and decision

- Introduction

In RAN1#107-e meeting [1], the following agreements were made for cross-carrier scheduling from sSCell to PCell/PSCell.

Agreement If no additional set of (s1, s2) is introduced, - For Option A BD/CCE limit handling and (s1=1, s2=0) agreed in RAN1#106bis-e, down-select from - Option 2 - On sSCell (for cross-carrier scheduling to P(S)Cell) - UE is not required to monitor more than MPDCCHmax,slot,μ1 PDCCH BD candidates per sSCell slot Agreement - BD/CCE limits for Type B UEs are applicable for Type A UEs supporting cross-carrier scheduling from sSCell to P(S)Cell Agreement - Following approaches for PDCCH monitoring and BD limit handling is supported for Type A UE - Additional simplifications to PDCCH monitoring - Type A UE as per RAN1#105-e agreement and - no simultaneous monitoring between ‘USS sets (for P(S)Cell scheduling) on sSCell’ and ‘Type 0/0A/1/2/CSS sets on P(S)Cell for DCI formats with CRC scrambled by C-RNTI/MCS-C-RNTI/CS-RNTI’ - simultaneous monitoring of ‘USS sets (for P(S)Cell scheduling) on sSCell’ and ‘Type 0/0A/1/2/CSS sets on P(S)Cell for DCI formats with CRC not scrambled by C-RNTI/MCS-C-RNTI/CS-RNTI’ Agreement Confirm the WA from RAN1#106bis-e with addition of below Note (shown in blue) Working Assumption - When CIF for sSCell to Pcell cross-carrier scheduling is configured, non-fallback DCI formats on P(S)Cell include same number of CIF bits as the corresponding non-fallback DCI formats on sSCell that are used for sSCell to P(S)Cell scheduling - Note: per RAN1#102-e agreement, when sSCell to P(S)Cell scheduling is configured for the UE, cross-carrier scheduling from P(S)Cell to another cell is not allowed. The CIF bits included in non-fallback DCI formats on P(S)Cell are considered reserved. Agreement - When CCS from sSCell to P(S)Cell is configured for the UE, - Multiple CORESET pools are not configured for PDCCH monitoring on P(S)Cell and not configured for PDCCH monitoring on sSCell; - Other Scells can be configured with multiple CORESET pools Agreement - For Option A BD/CCE limit handling agreed in RAN1#106bis-e - Value of parameter α for BD limit handling is configured via RRC from a set of - 8 possible values Agreement - For Option A BD/CCE limit handling agreed in RAN1#106bis-e - For CCE limit handling (i.e., scaling of maximum number of non-overlapping CCEs) - same parameter α agreed for BD limit handling is used Agreement - Alt1: When CCS from sSCell to P(S)Cell is configured for the UE, - r16monitoringcapability is not configured for PDCCH monitoring on P(S)Cell and not configured for PDCCH monitoring on sSCell; - r16monitoringcapability can be configured for PDCCH monitoring on Scells other than sSCell

In this contribution, we discuss and provide our views on the support of cross-carrier scheduling from sSCell to P(S)Cell in Rel-17.

- Discussion

DSS WI is mainly motivated from the scenario where gNB operates LTE and NR on the same carrier and that carrier is configured as NR P(S)Cell to a UE. In this case, if always-on signals such as LTE CRS and reserved region for LTE PDCCH are taken into account, a resource to carry NR PDCCH on that carrier may not be sufficient. To overcome this problem, it would be beneficial to allow cross-carrier scheduling from SCell to P(S)Cell. However, since NR system hasn’t supported yet cross-carrier scheduling where P(S)Cell is configured as scheduled cell, several issues need to be resolved to support P(S)Cell as scheduled cell.

The first aspect to be considered to support cross-carrier scheduling where P(S)Cell is configured as scheduled cell is which type of search space sets configured on P(S)Cell can be offloaded to sSCell. Our view is summarized as follows:

- Type0/0A/2-PDCCH CSS set: As in Rel-15, that is, can be configured only in PCell, as agreed in RAN1#103-e

- Type1-PDCCH CSS set: As in Rel-15/16, that is, can be configured only in P(S)Cell, as agreed in RAN1#103-e

- Type3-PDCCH CSS set: As in Rel-15/16, that is, can be configured in either of P(S)Cell and SCell, for DCI formats 2_0, 2_1, 2_2, 2_3, 2_4 as agreed in RAN1#103-e, configured only in P(S)Cell for DCI format 2_6 as agreed in RAN1#106bis-e, for DCI format 2_5, applicability of the information in DCI format 2_5 is the same as in Rel-15/Rel-16

- USS set associated with fallback DCI formats (i.e., DCI format 0_0/1_0): FFS whether it remains in P(S)Cell regardless of cross-carrier scheduling, or it can be configured in SCell scheduling P(S)Cell (e.g., by replacing one of the USS sets configured on SCell)

- USS set associated with non-fallback DCI formats (e.g., DCI format 0_1/0_2/1_1/1_2): Alt 2-1, i.e., UE can monitor DCI formats 0_1,1_1,0_2,1_2 on both PCell USS set(s) and sSCell USS sets simultaneously based on SS set configuration for which cross-carrier scheduling or self-carrier scheduling can be configured per each USS set

Furthermore, if USS sets are configured on the active DL BWP of the scheduling SCell and their indices exist in the USS sets configured on scheduled P(S)Cell, the USS sets can be monitored for cross-carrier scheduling from SCell to P(S)Cell. However, it is necessary to discuss further whether to also support PDCCH monitoring for USS sets, if the USS sets are configured on the active DL BWP of the scheduling SCell but their indices exist in the CSS sets configured on scheduled P(S)Cell. In addition, it can be considered how to handle the case where USS set index for the active DL BWP of scheduling SCell corresponding to a search space set configured for P(S)Cell does not exist.

Proposal 1: When cross-carrier scheduling from sSCell to P(S)Cell is configured,

- For Type3-PDCCH CSS set, the configuration for DCI format 2_5 and applicability of the information in the DCI format are the same as in Rel-15/Rel-16.

- For USS set associated with fallback DCI formats (i.e., DCI format 0_0/1_0) scheduling P(S)Cell: FFS whether it remains in P(S)Cell regardless of cross-carrier scheduling configuration, or it can be monitored in the sSCell

- For USS set associated with non-fallback DCI formats (e.g., DCI format 0_1/0_2/1_1/1_2) scheduling P(S)Cell, UE can monitor DCI formats 0_1, 1_1, 0_2, 1_2 on both PCell USS set(s) and sSCell USS sets simultaneously (i.e., Alt 2-1 as per previous agreement).

Proposal 2: When cross-carrier scheduling from sSCell to P(S)Cell is configured, the scheduling cell to be monitored for a USS set scheduling P(S)Cell is configured per each USS set index.

Next, regarding the aspects of PDCCH monitoring and BD/CCE limit handling, [based on Option C] was adopted in RAN1#106bis-e. One of remaining issues is whether to support other s1/s2 values than agreed ones (i.e., s1=1, s2=0). It would be beneficial to introduce other s1/s2 values (e.g., s1=0.5, s2=0.5), in terms of PDCCH offloading from P(S)Cell to sSCell.

Proposal 3: For BD/CCE limit handling at least for Type B UE, introduce additional s1 and s2 values (e.g., s1=0.5, s2=0.5) based on RRC configuration.

On top of the modification for above steps, fallback operation can be considered especially when any CORESETs are not assigned for P(S)Cell slot as shown by slot k+1 in Figure 1. In this case, since any CORESETs are not assigned for PCell slot k+1, each of y(n+2) and y(n+3) needs to be less than BD/CCE limit that is determined based on 30 kHz as defined in Rel-15/16. It is noted that this operation could be also applicable to the above Alt 2 for type A UE PDCCH monitoring, since the type A UE would monitor only one of P(S)Cell and sSCell.

Figure 1. Example of search space set configuration for PCell and sSCell

(for example, PCell = 15 kHz, sSCell = 30 kHz)

Proposal 4: Consider to apply Rel-15/16 BD/CCE limit for a slot where CORESET(s) are assigned on only one of P(S)Cell and sSCell.

Furthermore, it may need to consider the case when the sSCell configured as scheduling cell for P(S)Cell can be deactivated (or dormant). UE is supposed to stop any PDCCH monitoring on a deactivated SCell. However, if the deactivated SCell was configured for cross-carrier scheduling from sSCell to P(S)Cell and PDCCH monitoring on the sSCell was turned off, gNB may not have sufficient resource for PDCCH scheduling PDSCH/PUSCH on P(S)Cell. To figure out this issue, the following three options can be considered and Option 1 is preferred since it doesn’t require additional signalling.

- Option 1: Fallback to P(S)Cell self-carrier scheduling as if cross-carrier scheduling from sSCell to P(S)Cell is not configured

- Option 2: New sSCell indication via DCI or MAC CE

- Option 3: Separate USS set configurations for each of activated sSCell case and deactivated sSCell case

Proposal 5: After cross-carrier scheduling from sSCell to P(S)Cell is configured, if the sSCell is deactivated or in dormant state, the following options can be considered to provide sufficient PDCCH resource and Option 1 is preferred since it doesn’t require additional signalling.

- Option 1: Fallback to P(S)Cell self-carrier scheduling as if cross-carrier scheduling from sSCell to P(S)Cell is not configured

- Option 2: New sSCell indication via DCI or MAC CE

- Option 3: Separate USS set configurations for each of activated sSCell case and deactivated sSCell case

- Conclusion

In this contribution, we discussed and provided our views on the support of cross-carrier scheduling from sSCell to P(S)Cell, and the proposals are as follows.

Proposal 1: When cross-carrier scheduling from sSCell to P(S)Cell is configured,

- For Type3-PDCCH CSS set, the configuration for DCI format 2_5 and applicability of the information in the DCI format are the same as in Rel-15/Rel-16.

- For USS set associated with fallback DCI formats (i.e., DCI format 0_0/1_0) scheduling P(S)Cell: FFS whether it remains in P(S)Cell regardless of cross-carrier scheduling configuration, or it can be monitored in the sSCell

- For USS set associated with non-fallback DCI formats (e.g., DCI format 0_1/0_2/1_1/1_2) scheduling P(S)Cell, UE can monitor DCI formats 0_1, 1_1, 0_2, 1_2 on both PCell USS set(s) and sSCell USS sets simultaneously (i.e., Alt 2-1 as per previous agreement).

Proposal 2: When cross-carrier scheduling from sSCell to P(S)Cell is configured, the scheduling cell to be monitored for a USS set scheduling P(S)Cell is configured per each USS set index.

Proposal 3: For BD/CCE limit handling at least for Type B UE, introduce additional s1 and s2 values (e.g., s1=0.5, s2=0.5) based on RRC configuration.

Proposal 4: Consider to apply Rel-15/16 BD/CCE limit for a slot where CORESET(s) are assigned on only one of P(S)Cell and sSCell.

Proposal 5: After cross-carrier scheduling from sSCell to P(S)Cell is configured, if the sSCell is deactivated or in dormant state, the following options can be considered to provide sufficient PDCCH resource and Option 1 is preferred since it doesn’t require additional signalling.

- Option 1: Fallback to P(S)Cell self-carrier scheduling as if cross-carrier scheduling from sSCell to P(S)Cell is not configured

- Option 2: New sSCell indication via DCI or MAC CE

- Option 3: Separate USS set configurations for each of activated sSCell case and deactivated sSCell case

- References

RAN1#107-e chairman’s note