3GPP TSG-RAN WG1 Meeting #108-e	R1-2202390

e-Meeting, February 21th – March 3rd , 2022

Agenda Item:	8.5.2

Source:	Ericsson

Title:	Enhancements of UL-AoA positioning solutions

Document for:	Discussion, Decision

- Introduction

This contribution provides text proposal regarding the specifications for UL AoA positioning in release 17.

- Reference point for UL SRS-RSRPP

During RAN1#107e, RAN1 sent an LS to RAN4 to check on the reference point used to measure the UL SRS-RSRPP. Based on the response by RAN4 in [1], the same reference points used for time-based measurements can be included. It should be noted that the working assumption in the LS refers the the WA made during the RAN1 agreements, and there is no working assumption pending in RAN4.

Endorse TP 2.1 for inclusion in 38.215

TP 2.1:

### 5.2.6 UL SRS reference signal received path power (UL SRS-RSRPP) Definition; UL SRS reference signal received path power (UL SRS-RSRPP) is defined as the power of the received UL SRS signal configured for the measurement at the i-th path delay of the channel response, where UL SRS-RSRPP for 1st path delay is the power corresponding to the first detected path. The reference point for UL SRS-RSRPP shall be: - for type 1-C base station TS 38.104 [1]: the Rx antenna connector, - for type 1-O or 2-O base station TS 38.104 [1]: based on the combined signal from antenna elements corresponding to a given receiver branch, - for type 1-H base station TS 38.104 [1]: the Rx Transceiver Array Boundary connector. For frequency range 1 and 2, if receiver diversity is in use by the gNB for UL SRS-RSRPP measurements: - The reported UL SRS-RSRPP value for the first and additional paths shall be provided for the same receiver branch(es) as applied for UL SRS-RSRP measurements, or - The reported UL SRS-RSRPP value for the first path shall not be lower than the corresponding UL SRS-RSRPP for the first path of any of the individual receiver branches and the reported UL SRS-RSRPP for the additional paths shall be provided for the same receiver branch(es) as applied UL SRS-RSRPP for the first path..

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Endorse TP 2.1 for inclusion in 38.215

- References

- R4-2202682, LS reply on UL SRS-RSRPP definition , RAN4.