3GPP TSG-RAN WG1 Meeting #108-e	R1- 2202396

e-Meeting, February 21th – March 3rd , 2022

Agenda Item:	8.12.1

Source:	Ericsson

Title:	Mechanisms to support MBS group scheduling for RRC_CONNECTED UEs

Document for:	Discussion

# Introduction

In this contribution, we provide our view on the leftover issues for group scheduling for NR MBS.

# Discussion of remaining issues

## Support of Carrier Aggregation in MBS

CA capabilities for MBS has not been discussed in detail. It seems reasonable not to require the UE to support group scheduling in all CCs if unicast CA are supported and MBS features are supported. Therefore, a separate MBS CA capability indicating the number of CCs that can be configured for MBS should be defined. The network will then configure a CFR for each of the Scell separately. Within a CC, group scheduling will function the same in Scell as in Pcell. For HARQ feedback, the procedure to generate the HARQ codebook in CA for unicast can be reused, where the feedback for each CC in the PUCCH group is concatenated to generate the transmitted harq feedback codebook. The procedure already described in the specification is sufficient.

Introduce a UE capability for the number of CCs supported for MBS by the UE.

The total number of PDSCH for PCell and Scell are defined with m1 and m2 in 38.202, respectively, in release 16. MBS introduces m3 and m4 for PCell and SCell multicast PDSCH capability. The maximum value for the total number m1+m3 and m2+m4 of PDSCH for CCs should also be discussed as part of the capability discussion.

Discuss whether the number of simultaneous unicast and multicast PDSCH for a CC should be unchanged from release 16 maximum number of simultaneous PDSCH for a CC.

## Downlink reception types for MBS in 38.202

In 38.202, D2 defines the downlink reception type where only the PDCCH is received in the slot. This corresponds to SPS release (PDCCH scrambled with CS-RNTI), or cross-slot scheduling (PDCCH scrambled with CS-RNTI for activation, PDCCH scrambled with C-RNTI or MCS RNTI with a TDRA corresponding to a slot offset from PDCCH).  However, D4 for MBS only includes G-CS-RNTI. It is proposed to discuss the inclusion of G-RNTI. Similarly, broadcast reception types do not include the case of reception of broadcast PDCCH alone in a slot, which should be considered.

Include G-RNTI as part of reception type D4 in 38.202 and add DL-SCH as the associated channel.

Include G-RNTI as part of a separate reception type for broadcast PDCCH in 38.202 and add DL-SCH as the associated channel.

The following TP for 38.202 is proposed:

## 6.2 Downlink The tables 6.2-1, 6.2-2 describe the possible combinations of physical channels that can be received simultaneously in the downlink by one UE. Table 6.2-1 introduces notation for a "Reception Type" which represents a physical channel and any associated transport channel. Table 6.2-2 describes the combinations of these "Reception Types" which are supported by the UE depending on capabilities [8, TS 38.306], and enumerates how many of each can be received simultaneously. The UE shall be able to receive all TBs according to the indication on PDCCH. Any subset of the combinations specified in table 6.2-2 is also supported. - Table 6.2-1: Downlink "Reception Types" "Reception Type"; Physical Channel(s); Monitored RNTI; Associated Transport Channel; Comment. A; PBCH; N/A; BCH. B; PDCCH+PDSCH; SI-RNTI; DL-SCH; Note 1. C0; PDCCH; P-RNTI; N/A; Note 1, Note 2. C1; PDCCH+PDSCH; P-RNTI; PCH; Note 1. D0; PDCCH+PDSCH; RA-RNTI or Temporary C-RNTI or MsgB-RNTI; DL-SCH; Note 3. D1; PDCCH+PDSCH; C-RNTI, CS-RNTI, MCS-C-RNTI; DL-SCH. D2; PDCCH; C-RNTI, CS-RNTI, MCS-C-RNTI; DL-SCH. D3; PDCCH+PDSCH; G-RNTI, G-CS-RNTI; DL-SCH; Note 6. D4; PDCCH; G-CS-RNTI, G-RNTI; N/A; Note 7. D5; PDCCH+PDSCH; MCCH-RNTI; DL-SCH; Note 8. D6; PDCCH+PDSCH; G-RNTI; DL-SCH; Note 9. E; PDCCH; C-RNTI; N/A; Note 4. F0; PDCCH; Temporary C-RNTI; UL-SCH; Note 3. F1; PDCCH; C-RNTI, CS-RNTI, MCS-C-RNTI; UL-SCH. G; PDCCH; SFI-RNTI; N/A. H; PDCCH; INT-RNTI; N/A. J0; PDCCH; TPC-PUSCH-RNTI; N/A. J1; PDCCH; TPC-PUCCH-RNTI; N/A. J2; PDCCH; TPC-SRS-RNTI; N/A. K; PDCCH; SP-CSI-RNTI; N/A. L0; PDCCH; SL-RNTI; SL-SCH. L1; PDCCH; SL-CS-RNTI; SL-SCH. M; PDCCH; SL Semi-Persistent Scheduling V-RNTI; SL-SCH; Note 5. N; PDCCH; PS-RNTI; N/A. O; PDCCH; AI-RNTI; N/A. P; PDCCH; CI-RNTI; N/A. Note 1: These are received from PCell only. Note 2: In some cases UE is only required to monitor the short message within the DCI for P-RNTI. Note 3: These are received from PCell or PSCell. Note 4: This corresponds to PDCCH-ordered PRACH. Note 5: This corresponds to PDCCH scheduling LTE PC5. Note 6: This is for multicast in RRC connected state Note 7: This corresponds to DL Semi-Persistent Scheduling release for multicast in RRC connected state. Note 8: This is for broadcast MCCH Note 9: This is for broadcast MTCH. - Table 6.2-2: Downlink "Reception Type" combinations

## Configuration details of the CFR

In order to efficiently reallocate the CFR between BWPs, we propose to introduce a “CFR ID”. In this way a cell configured with multiple BWPs could use a single CFR without having to duplicate the same CFR signaling per BWP.

CFR is configured per cell, with a CFR ID. A BWP configured for multicast is indicated a CFR ID.

## PDCCH configuration for MBS

### Content of New DCI formats for multicast and broadcast:

In RAN1#107-e, the following was agreed:

Agreement The following information is transmitted by means of the DCI format 1_0 with CRC scrambled by G-RNTI for multicast: - Frequency domain resource assignment - Time domain resource assignment – 4 bits as defined in Clause 5.1.2.1 of TS38.214 - VRB-to-PRB mapping – 1 bit according to Table 7.3.1.2.2-5 in TS38.212 - Modulation and coding scheme – 5 bits as defined in Clause 5.1.3 of TS38.214 - New data indicator – 1 bit - Redundancy version – 2 bits as defined in Table 7.3.1.1.1-2 in TS38.212 - HARQ process number – [4 or 5] bits - Downlink assignment index – 2 bits as defined in Clause 9.1.3 of TS 38.213, as counter DAI - PUCCH resource indicator – 3 bits as defined in Clause 9.2.3 of TS38.213 - PDSCH-to-HARQ_feedback timing indicator – 3 bits as defined in Clause 9.2.3 of TS38.213 - Reserved bits –3 bits - FFS: Some of the fields may be not useful and can be reserved in some conditions, and FFS the details of the conditions - FFS: other fields, e.g. for HARQ enabling/disabling Note: Whether new fields are defined for multicast DCI format 1_0 can be discussed separately. The reserved bits can be used for new fields if needed.

Agreement Multicast DCI format 1_1 includes all configurable fields of unicast DCI format 1_1 except - Identifier for DCI formats, TPC command for scheduled PUCCH, SRS request - FFS: Scell dormancy indication - One-shot HARQ-ACK request, PDSCH group index, New feedback indicator, Number of requested PDSCH group(s), ChannelAccess-Cpext - CBGTI, CBGFI - Minimum applicable scheduling offset indicator - FFS: Carrier indicator, BWP indicator, ZP CSI-RS trigger - FFS: MCS/NDI/RV for TB2

#### Use of the carrier indicator in DCI

Since use of a group carrier indicator for group cross carrier scheduling presupposes that the different UE in the group have the same CA configuration (same number of CCs, same carrier location), the applicability of cross carrier scheduling is small, therefore, we propose not to support it for this release.

Cross carrier scheduling is not supported for group scheduling PDSCH in release 17.

### DCI alignment and Counting of G-RNTI in the DCI budget

Regarding the 3+1 budget for DCI sizes, the following was agreed in RAN1#105e:

Agreement: Confirm the working assumption: Keep the “3+1” DCI size budget defined in Rel-15 for Rel-17 MBS. - FFS: Whether the G-RNTI is counted as “C-RNTI” or as “other RNTI” when considering the “3+1” DCI size budget rule for group-common PDCCH.

The DCI size for formats 4_0 and 4_1 have been agreed to be aligned with DCI 1_0. An extension to the alignment procedure to include DCI 4_2 remains to be agreed. When both DCI 1_1/0_1 and 1_2/0_2 are present in USS the procedure in step 4 is as follow:

- Aligns DCIs  1_1 and 0_1 to the largest size of the two,

- Align DCIs 1_2 and 0_2 to the largest size of the two

- Align fallback DCIs 0_0 and 1_0 for USS and CSS.

This step results in 3 different DCI sizes for unicast scheduling.  The multicast DCI sizes should also be counted toward the budget of 3 DCI sizes for scheduling, in order to free the last 4th DCI size to be used by the “other” DCI format purposes. Adding multicast DCIs to the budget will require further alignment. Since DCI 1_2 and 0_2 are meant to be compact DCIs for URLLC, it is proposed to align the DCIs for multicast with DCI 1_1/0_1.

The G-RNTI is counted as “C-RNTI”  when considering the “3+1” DCI size budget rule for group-common PDCCH.

Add the following steps 2B and 4D in the DCI alignment procedure:

Step 2B: Determine DCI format 4_2 monitored in a UE-specific search space according to clause 7.3.1.5.3 Step 4D: If the total number of different DCI sizes configured to monitor is more than 4 for the cell after applying the above steps, or if the total number of different DCI sizes with C-RNTI or G-RNTI configured to monitor is more than 3 for the cell after applying the above steps - If the number of information bits in the DCI format 0_1 is less than the payload size of the DCI format 4_2 for scheduling the same serving cell, a number of zero padding bits are generated for the DCI format 0_1 until the payload size equals that of the DCI format 4_2. - If the number of information bits in the DCI format 1_1 is less than the payload size of the DCI format 4_2 for scheduling the same serving cell, a number of zero padding bits are generated for the DCI format 1_1 until the payload size equals that of the DCI format 4_2.

During RAN1#107b-e It was agreed to have the DCI size of DCI 4_2 to be configurable. In order to make the DCI alignment procedure work, it is necessary that this size is equal or larger to the DCI 1_1/0_1 since the DCI size for format 4_2 must be the same across the UEs in the G-RNTI.

The UE does not expect to be configured in a way where the DCI size for format 1_1 is larger than DCI format 4_2.

## Configuration of TCI states for multicast

The 38.214 specification states that a separate list of M’ TCI states is configured for multicast within PDSCH-config-multicast. The remaining question is how to address the TCI states in the multicast DCI.

We propose to re-use the existing MAC CE “TCI States Activation/Deactivation for UE-specific PDSCH”, described in clause 6.1.3.14 in specification 38.321.  the MAC CE contains N octets to configure up to 8 TCI states for unicast PDSCH.

We note that if the TCI states IDs are non-overlapping between unicast and multicast, and the sum of TCI states in unicast and multicast does not exceed the existing maximum number of TCI states (maxNrofTCI-States=128),  both of the unicast and multicast TCI states can be activated by the same MAC CE. The only condition to change is that the MAC CE may activate at most 8 states for unicast and 8 states for multicast, totaling 16 states.

The TCI states codepoints in DCI 4_2 are configured by a “TCI States Activation/Deactivation for UE-specific PDSCH” MAC CE

The  “TCI States Activation/Deactivation for UE-specific PDSCH” MAC CE may activate up to 8 TCI states for multicast PDSCH, in addition to up to 8 states in unicast PDSCH

The TCI states IDs for unicast and multicast TCI state lists do not overlap, and the total number of TCI states across unicast and multicast in a cell does not exceed the currently specified maxNrofTCI-States=128

During RAN1#107e it was discussed whether group scheduling should be used to convey the MAC CE to the UE group. Using the proposed method, we believe the gNB can have the flexibility to send the MAC CE either over unicast or multicast.

Whether the MAC CE configuring the TCI states for multicast is sent over unicast or multicast, is left to the gNB implementation.

The problem of configuring the TCI state for multicast PDCCH may be more complex. The MAC CE configuring the PDCCH TCI state only contains to allocation a single state. To configure the TCI state for multicast PDCCH, the following cases can be foreseen:

- Multicast PDCCH uses the same TCI state as unicast PDCCH. This makes sense when PDCCH for multicast and unicast share the same coreset and are in the same slot. This can already be supported by the current specification.

- Multicast PDCCH use the same coreset, but is not configured in the same slot (that is, unicast and multicast search spaces are offset).  It is currently not possible to configure different TCI states for that case.

- Multicast PDCCH uses a different coreset from unicast. The current specification supports this solution to have a different TCI state for unicast and multicast. However, this would divide the coresets between multicast and unicast, which is not efficient.

In order to have the possibility to configure individually the TCI states for multicast and unicast, while maintaining the possibility to keep unicast and multicast in the same coreset, a solution must be designed. A new MAC CE is one solution but requires additional work for RAN2. Instead, we propose to reuse the existing MAC CE and use G-RNTI to identify the MAC CE aimed at group PDCCH TCI state configuration.

The TCI state for PDCCH is configured via the TCI State Indication for UE-specific PDCCH MAC CE, using a group PDSCH scrambled by G-RNTI.

## Further details on SPS multicast

2.5.1 Feedback type for SPS multicast activation

In RAN1#107e, the following was agreed:

Agreement For multicast SPS activation/deactivation, only ACK/NACK based feedback is supported.

The 38.213 specification capture the agreement as follow

A UE monitors PDCCH for scheduling PDSCH receptions or for activation/release of SPS PDSCH receptions for a corresponding SPS PDSCH configuration as described in clause 10.1.A UE can be configured by harq-Feedback-Option-Multicast for a G-RNTI or by sps-HARQ-Feedback-Option-Multicast for a G-CS-RNTI to provide HARQ-ACK information for a transport block reception associated with the G-RNTI or with the G-CS-RNTI, respectively, according to the first HARQ-ACK reporting mode or according to the second HARQ-ACK reporting mode. The second HARQ-ACK reporting mode is not applicable for DCI formats having associated HARQ-ACK information without scheduling a PDSCH reception. For the first HARQ-ACK reporting mode, the UE generates HARQ-ACK information with ACK value when a UE correctly decodes a transport block or detects a DCI format indicating an SPS PDSCH release; otherwise, the UE generates HARQ-ACK information with NACK value, as described in clauses 9 and 9.1 through 9.3. For the second HARQ-ACK reporting mode, the UE does not transmit a PUCCH that would include only HARQ-ACK information with ACK values.

SPS is activated by a PDCCH order, and this order is implicitly acknowledged by the reception of the first SPS PDSCH’s HARQ feedback. That is to say, the PDCCH for activation does not have associated HARQ-ACK information, contrary to SPS release where the PDCCH message is acknowledged by the UE to terminate SPS reception.

The first issue is to clarify that the ACK/NACK based feedback activation should be sent as the first SPS PDSCH transmission feedback.

Feedback for multicast SPS activation is always using ACK/NACK based feedback, and is transmitted as a response to the first occurrence of the SPS PDSCH.

The second issue is which PUCCH resource should be used to send feedback for the activation and release when the SPS HARQ feedback mode is not HARQ-ACK/NACK (mode1). The SPS PDSCH configuration contains PUCCH resources for HARQ feedback according to the feedback mode configured for SPS. However, when NACK only (second HARQ-ACK reporting mode ) is configured, or if HARQ feedback has been disabled for SPS, the UE does not have a PUCCH resource to send the ACK/NACK based feedback that has been agreed to use for multicast SPS activation.

In order to resolve the issue, it is propose to discuss the following options:

- Option 1: SPS-config-Multicast includes a PUCCH resource to be used in activation and release

- Option 2: SPS-config-Multicast includes a PUCCH resource ID pointing at a resource to be used within the PUCCH resources configured for dynamic multicast or unicast.

- Option 3: The PRI field in the PDCCH activation or release is used to point at a PUCCH resource from the dynamic scheduling pool of PUCCH resources configured in PUCCH-config.

Option 1 adds complexity since an additional PUCCH resource need to be configured on top of the option unicast and multicast resources. It offers the advantage of not being dependent of any PUCCH configuration from multicast or unicast. However, since it is expected that at least unicast will be configured an independent configuration may not be a clear advantage.

Option 2 is the simplest solution, as it relies on a pre-existing PUCCH resource already configured in either unicast or multicast.

Option 3 is a more flexible version of option 2, where the PRI can be used to point at what resource to use, but still relying in the pre-existing PUCCH configuration in unicast or multicast.

For SPS configured with NACK-only feedback, for configuration of the resource to use to acknowledge the first PDSCH of an SPS configuration following activation,  downselect the solution to configure the resource between:

    - Option 1 SPS-config-Multicast includes a PUCCH resource to be used in activation and release

    - Option 2: SPS-config-Multicast includes a PUCCH resource ID pointing at a resource to be used within the PUCCH resources configured for dynamic multicast or unicast.

    - Option 3: The PRI field in the PDCCH activation or release is used to point at a PUCCH resource within the PUCCH resources configured for dynamic multicast or unicast.

# Conclusion

In the previous sections we made the following observations:

Observation 1	For broadcast services to UEs in RRC CONNECTED, where the UE has not sent an MII, broadcast reception is best effort.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Introduce a UE capability for the number of CCs supported for MBS by the UE.

Proposal 2	Discuss whether the number of simultaneous unicast and multicast PDSCH for a CC should be unchanged from release 16 maximum number of simultaneous PDSCH for a CC.

Proposal 3	Include G-RNTI as part of reception type D4 in 38.202 and add DL-SCH as the associated channel.

Proposal 4	Include G-RNTI as part of a separate reception type for broadcast PDCCH in 38.202 and add DL-SCH as the associated channel.

Proposal 5	Group common PDCCH for multicast can be configured in CORESET#0 if CORESET#0 is partly or fully within a multicast CFR.

Proposal 6	Support a modified option 1 from RAN1#104b regarding using CORESETs from unicast with multicast: If a CFR is configured in a dedicated unicast BWP for multicast in RRC-CONNECTED state, the CORESET configured in PDCCH-config for unicast in the dedicated unicast BWP can be used for PTM-1 transmission

Proposal 7	Cross carrier scheduling is not supported for group scheduling PDSCH in release 17.

Proposal 8	Multicast dynamic BWP switching is supported with a BWP indicator field in DCI 4_2

Proposal 9	CFR is configured per cell, with a CFR ID. A BWP configured for multicast is indicated a CFR ID.

Proposal 10	The full ZP CSI-RS framework is supported in multicast, including:

a.	Periodic ZP CSI-RS configuration in PDSCH-config-multicast

b.	Semi-persisitent ZP CSI-RS configuration in PDSCH-config-multicast

c.	Activation and release of semi persistent ZP-CSI RS via group scheduling MAC CE

d.	Configuration of aperiodic ZP CSI-RS in PDSCH-config-multicast

e.	Triggering of aperiodic ZP CSI RS in DCI 4_2

Proposal 11	The DCI fields for TB#2 in DCI 1_1 are not included in DCI 4_2

Proposal 12	The G-RNTI is counted as “C-RNTI”  when considering the “3+1” DCI size budget rule for group-common PDCCH.

Proposal 13	The determination of non-fallback multicast/broadcast DCI size, monitored in the common search space is inserted as step ”2B” in the DCI alignment procedure

Proposal 14	Multicast reuses the TCI state list tci-StatesToAddModList configured in unicast PDSCH-config if it is configured.

Proposal 15	The TCI codepoints in DCI 4_2 are configured by a TCI States Activation/Deactivation for UE-specific PDSCH MAC CE sent over group PDSCH scrambled with G-RNTI.  The TCI codepoints for unicast are configured by a TCI States Activation/Deactivation for UE-specific PDSCH MAC CE sent over unicast PDSCH.

Proposal 16	if no MAC CE has been received by the UE to configure the TCI codepoints for multicast, The default TCI state is the one used for multicast PDCCH.

Proposal 17	The TCI state for multicast group PDCCH is configured by a TCI State Indication for UE-specific PDCCH MAC CE received over group PDSCH scrambled by G-RNTI.

Proposal 18	When no MAC CE has been received to configure the TCI state of multicast PDCCH, the UE assumes that the DM-RS antenna port associated with multicast PDCCH receptions is quasi co-located with the SS/PBCH block the UE identified during the initial access procedure.

Proposal 19	For UEs in RRC CONNECTED, the CFRs for multicast and broadcast may be independently configured, i.e. could use arbitrary different frequency resources, within the active BWP.

Proposal 20

Proposal 21	UEs in RRC CONNECTED are expected to receive unicast within the active BWP, in parallel with multicast and broadcast, on independent CFRs within the active BWP, based on UE capabilities.

Proposal 22	For broadcast services, where the UE has sent an MII, broadcast is counted as multicast in the number of supported multicast PDSCHs, without increasing the total number of supported PDSCHs.

Proposal 23	For UEs in RRC CONNECTED receiving broadcast, downselect between the following two solutions:

a.	Add DL signaling support to allow the UE to reuse one HARQ process buffer for broadcast

i.	Adding HARQ process ID and NDI in the broadcast DCI

ii.	Not excluding other methods

b.	Buffering for broadcast is independent of HARQ buffering for unicast/multicast, i.e. addition of broadcast has no impact on HARQ buffers for unicast/multicast

i.	Note: This may require dedicated additional HW for broadcast buffering to support PDSCH repetition

# References

Chairman Notes for RAN1#102-e meeting, 2020

R1-2009306, Discussion reliability mechanisms for NR-MBS, Ericsson, RAN1#103-e meeting, 2020

3GPP TS 38.331, NR; Radio Resource Control (RRC); Protocol specification, 2020

3GPP TS 38.822, NR; User Equipment (UE) feature list, 2020.

TS 38.321 - 5.15 Bandwidth Part (BWP) operation

R1-2200667, Support for NR broadcast reception in RRC Inactive/Idle, Ericsson, RAN1#107b-e, 2022