3GPP TSG-RAN WG1 Meeting #108-e	R1-2202401

e-Meeting, February 21th – March 3rd , 2022

Agenda Item:	5

Source:	Ericsson

Title:	views on LS on the applicability of PRS processing window in RRC_INACTIVE state

Document for:	Discussion, Decision

- Introduction

In this contribution, we discuss potential responses to RAN4 on the  LS on the applicability of PRS processing window in RRC_INACTIVE state[1].

- Discussion

## LS content

The following was received by RAN1 in R1-2200903[1]:

Based on previous RAN1 LS, RAN1 has introduced a PRS processing window to support PRS measurements outside measurement gaps. But when RAN4 is working on the PRS measurement requirements in RRC_INACTIVE state, it is unclear whether the PRS processing window can be applied in RRC_INACTIVE state, which may have impact on the measurement requirements. So RAN4 would like to ask RAN1 for the clarification on the following question: Q1: Whether the PRS processing window defined for PRS measurements outside measurement gaps can be also applied for PRS measurements in RRC_INACTIVE state?

## Views on PPW for PRS measurements in RRC_INACTIVE state

The PPW is designed to enable multiplexing of UE serving cell traffic with positioning signals, thus improving the latency of the whole positioning framework. UEs in RRC_INACTIVE do not receive data, therefore the use of PPW is not necessary in that case. The UE may receive paging, SSBs and other DL signals, but is not expected to prioritize PRS in that case according to the RAN1 agreement:

Agreement: - From RAN1 perspective, in RRC_INACTIVE state, reception of DL PRS has lower priority than other DL signals/channels (SSB, SIB1, CORESET0, MSG2/MSGB, paging, DL SDT) - FFS how to determine conflicts in DL PRS and other DL signals/channels reception by UE - FFS how to handle retuning time for the case when DL PRS and other DL signals/channels are allocated in different BW and/or have the same or different SCS as initial DL BWP - Send LS to RAN4 (cc RAN2) and ask if there is any feedback

Based on the existing agreements, the following is proposed:

The PRS processing window for measurement outside measurement gaps is not applicable to measurements made in RRC_INACTIVE

- Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	The PRS processing window for measurement outside measurement gaps is not applicable to measurements made in RRC_INACTIVE

- References

R1-2200903, LS on the applicability of PRS processing window in RRC_INACTIVE state, RAN4.