3GPP TSG-RAN WG1 Meeting #108-e 	R1-2202449

e-Meeting, February 21 – March 3, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | - | rev | - | Current version: | 15.14.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on effect time of absolute TPC for PUSCH transmissions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2022-02-21 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the discussion of Rel-17 CovEnh WI, it was identified that for a given PUSCH transmission occasion i the cut-off time to take into account a received absolute TPC is missing. Without such cut-off time, it is unclear which absolute TPC received from DCI format 0_x and DCI format 2_2 can impact a given configured PUSCH transmission occasion,nor is for a given PUSCH transmission occasion of PUSCH repetitions. A simple clarification is that, absolute TPC has similar timeline as accumulated TPC commands, i.e., the closed-loop state for PUSCH transmission occasion i is determined by the latest TPC command received K_PUSCH(i) symbols before the transmission occasion i, where the determination of K_PUSCH(i) is the same as that for accumulated TPC commands. |  |  |  |  |  |  |  |  |
| T |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify the cut-off time for a PUSCH transmission occasion to take into account received abosolute TPC commands. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | In case of absolute TPC commands for closed-loop power control, the cut-off time is not clearly specificed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR…CR… |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR…CR… |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR…CR… |  |  |  |
| Other comments: |  | Isolated Impact Analysis: This CR has isolated impact on PUSCH power control with absolute TPC command. It reuses the existing timeline that has been defined for accumulating TPC, it is not expected to impact on gNB/UE implementation. |  |  |  |  |  |  |  |  |

| This CR's revision history: |  |
| --- | --- |

### 7.1.1	UE behaviour

< Unchanged parts are omitted >

-	 is the PUSCH power control adjustment state for active UL BWP  of carrier  of serving cell  and PUSCH transmission occasion  if the UE is provided tpc-Accumulation, where

-	 absolute values are given in Table 7.1.1-1

-    δPUSCH,b,f,ci,l is the latest TPC command value that is received KPUSCH(i) symbols before the PUSCH transmission occasion i on active UL BWP b of carrier f of serving cell c for PUSCH power control adjustment state l

-	KPUSCH(i) is the same as that defined for the case where the UE is not provided tpc-Accumulation

< Unchanged parts are omitted >