3GPP TSG-RAN WG1 Meeting #108-e 	R1-2202454

e-Meeting, February 21 – March 3, 2022

Agenda Item:	5

Source:	Huawei, HiSilicon

Title:	Discussion on the functionality of PRU

Document for:	Discussion and decision

Introduction

- In this paper, we provided our view on the incoming LS from [1] [2] [3]. The incoming reply LSs are all originated from RAN1 LS [4].

RAN1 LS [4] in May (Arrival in Aug.)

RAN2 LS [1] in Nov. (Arrival in Feb.)

SA2 LS [2] in Nov. (Arrival in Feb.)

SA2 LS [3] in Nov. (Arrival in Feb.)

- Quote from RAN2 LS [1]

1. Overall Description: RAN2 thanks RAN1 for their LS on Positioning Reference Units (PRUs) for enhancing positioning performance. RAN2 discussed support of PRUs as described in the RAN1 LS R2-2106920 (R1-2106326, S2-2105263). Based on the information provided by RAN1, RAN2's current understanding is that a PRU is a device with a priori known location (to some degree of accuracy at least) and which perfoms location measurements at this known location. In addition, the PRU can transmit SRS to enable TRPs to measure and report UL positioning measurements from PRUs at known location. The PRU or TRP measurements can then be compared by a location server with the measurements expected to determine "correction information" for other nearby target devices. The location measurements for other target devices may then be corrected based on the previously determined "correction information". Further, RAN2 discussed the modelling of PRUs and agreed that the PRU can be considered (at least) as a UE (from LMF perspective). However, since the PRU location measurements are needed by an LMF itself different views exist in RAN2 on how this can be enabled in the current LCS architecture. Some companies in RAN2 believe that PRU support has no impact on SA2 and CT specifications and think that the MT-LR procedures as currently specified in TS 23.273 can also be instigated by an LMF (e.g., an LMF may initiate a location request to a GMLC) or existing MO-LR procedures as specified in TS 23.273 can be instigated by a PRU for enabling an LMF to obtain PRU location measurements or transmit UL-SRS. Other companies in RAN2 believe that an LMF needs to be enabled to instigate location procedures for a PRU (e.g., LPP, NRPPa procedures) without receiving a location request for the PRU from an AMF (i.e., in the absence of an MT-LR or MO-LR for the PRU). These companies also believe that this effectively means that the LMF should act as an "LCS Client" for PRUs. However, since the LCS procedures and overall architecture are in the realm of SA2, RAN2 would like to confirm with SA2 whether PRU support as described by RAN1 can be provided with the current LCS framework as specified by SA2 (e.g., TS 23.273) or whether any changes would be required. In addition, RAN2 discussed whether the LMF determined "correction information" obtained from PRU measurements needs to be provided to target UEs for UE-based mode of operation. As mentioned by RAN1 in the LS, a PRU may provide "antenna orientation information" to an LMF and RAN2 would like to ask RAN1 to provide further details of the "PRU antenna orientation information". 2. Actions: To SA2 group. ACTION: RAN2 kindly asks SA2 whether the MT-LR or MO-LR location procedures as currently specified in TS 23.273 can be used to enable an LMF obtaining location measurements from PRUs (via LPP) and to trigger SRS transmission of PRUs (via NRPPa), or whether an LMF needs to be enabled to instigate location procedures for a PRU (e.g., LPP, NRPPa procedures) without receiving a location request for the PRU from an AMF (i.e., in the absence of an MT-LR or MO-LR for the PRU), and if so, whether support can be provided as part of Release 17. To RAN1 group. ACTION: RAN2 kindly asks RAN1 whether the LMF determined "correction information" obtained from PRU measurements need to be provided to target UEs for UE-based mode of operation, and if so, kindly asks RAN1 to provide further details on the specific "correction information" which need to be provided to target UEs. RAN2 also kindly asks RAN1 to provide further details on the "PRU antenna orientation information" which should be provided to an LMF.

- Quote from SA2 LS [2]

1. Overall Description: SA2 thanks RAN1 for their LS on Positioning Reference Units (PRUs) for enhancing positioning performance. SA2 discussed the PRU and agreed to include the objective of how to support the PRU in Rel-18 eLCS_Ph3 Study Item, considering SA2 Rel-17 is already frozen and more time is needed to study how to support the PRU in SA2. SA2 also notice d that RAN2 is under discussion of how to support PRUs in Rel-17 and is considering solutions which may or may not have impacts to SA2. SA2 does not have enough time in Rel-17 to comment on such solutions at the present time but expects that a solution or solutions preferable to SA2 should be possible in Release 18. 2. Actions: To RAN1 and RAN2 group. ACTION: SA2 kindly asks RAN1 and RAN2 to take the above information into account.

- Quote from SA2 LS [3]

1. Overall Description: SA2 thanks RAN2 for providing the progress on the PRU discussion in RAN WG (S2-2108999/R2-2111488). Regarding the questions listed in the LS: “RAN2 kindly asks SA2 whether the MT-LR or MO-LR location procedures as currently specified in TS 23.273 can be used to enable an LMF obtaining location measurements from PRUs (via LPP) and to trigger SRS transmission of PRUs (via NRPPa), or whether an LMF needs to be enabled to instigate location procedures for a PRU (e.g., LPP, NRPPa procedures) without receiving a location request for the PRU from an AMF (i.e., in the absence of an MT-LR or MO-LR for the PRU), and if so, whether support can be provided as part of Release 17.” SA2 would like to provide the following answers. If a PRU is considered as a normal UE and has registered to the network, then a normal MO-LR, MT-LR, or NI-LR could be used to locate the PRU, where the LMF will initiate the positioning procedure, as defined in TS 23.273, clause 6.11. The procedure will trigger the UE to enter into CM-connected mode, i.e. page the UE if it is currently unreachable. However, the LMF may have no knowledge of which UEs act as PRUs and may therefore be unable to control and manage PRUs or make use of measurements provided by PRUs. Furthermore, in release 17 and previous releases, SA2 has not specified a procedure for LMF to initiate a positioning procedure for a UE without an external trigger. SA2 has finished R17 work on 5G_eLCS_Ph2, therefore it is not possible to study this topic in this release. SA2 may study support of PRUs in Release 18 in which case a better solution may be available later. 2. Actions: To RAN2 group. ACTION: SA2 asks RAN2 to take above information into account.

Discussion

Based on the input from RAN2 and SA2, the following observation can be drawn.

- Observation 1: LMF autonomously instigating the location procedure for a PRU is not specified in SA2.

- Observation 2: MO-LR from the PRU as a UE can be supported in Rel-17 without any SA/CT specification change.

Observation 3: MT-LR for the PRU as a UE or LMF autonomous location for the PRU, including PRU management will be studied by SA2 in Rel-18 within the SI FS_eLCS_Ph3.

Since there is already a solution in Rel-17, i.e. MO-LR from the PRU, from RAN1 perspective, we may conclude that PRU functionality can be specified in RAN in Rel-17 time frame.

- Proposal 1: RAN1 confirm that PRU functionality as a UE can be specified in RAN in Rel-17 time frame.

Reply to RAN2

On the first question in the action points from RAN2 [1]:

RAN2 kindly asks RAN1 whether the LMF determined "correction information" obtained from PRU measurements need to be provided to target UEs for UE-based mode of operation, and if so, kindly asks RAN1 to provide further details on the specific "correction information" which need to be provided to target UEs.

We think that the correction information may include both timing-based method correction and angle-based method correction for UE-based DL-TDOA and DL-AoD, respectively.

Primarily the timing-based correction is targeting the group delay based on the discussion earlier in RAN1, in which the PRU measurement in DL PRS could help LMF retrieve the group delay difference between different TRPs. In Rel-16 specification for UE-based positioning, the group delay difference between TRPs actually could be converted to RTD (real time difference), which defines the offset between subframe boundary between the target TPR and the reference TRP.

NR-RTD-Info-r16 ::= SEQUENCE {

referenceTRP-RTD-Info-r16		ReferenceTRP-RTD-Info-r16,

rtd-InfoList-r16				RTD-InfoList-r16,

...

}

ReferenceTRP-RTD-Info-r16 ::= SEQUENCE {

dl-PRS-ID-Ref-r16				INTEGER (0..255),

nr-PhysCellID-Ref-r16			NR-PhysCellID-r16		OPTIONAL,	-- Need ON

nr-CellGlobalID-Ref-r16			NCGI-r15				OPTIONAL,	-- Need ON

nr-ARFCN-Ref-r16				ARFCN-ValueNR-r15		OPTIONAL,	-- Need ON

refTime-r16						CHOICE {

systemFrameNumber-r16		BIT STRING (SIZE (10)),

utc-r16						UTCTime,

...

},

rtd-RefQuality-r16				NR-TimingQuality-r16	OPTIONAL,	-- Need ON

...

}

RTD-InfoList-r16 ::= SEQUENCE (SIZE (1..nrMaxFreqLayers-r16)) OF RTD-InfoListPerFreqLayer-r16

RTD-InfoListPerFreqLayer-r16 ::= SEQUENCE (SIZE(1..nrMaxTRPsPerFreq-r16)) OF RTD-InfoElement-r16

RTD-InfoElement-r16 ::= SEQUENCE {

dl-PRS-ID-r16					INTEGER (0..255),

nr-PhysCellID-r16				NR-PhysCellID-r16		OPTIONAL,	-- Need ON

nr-CellGlobalID-r16				NCGI-r15				OPTIONAL,	-- Need ON

nr-ARFCN-r16					ARFCN-ValueNR-r15		OPTIONAL,	-- Need ON

subframeOffset-r16				INTEGER (0..1966079),

rtd-Quality-r16					NR-TimingQuality-r16,

...

}

subframeOffset This field specifies the subframe boundary offset at the TRP antenna location between the reference TRP and this neighbour TRP in time units where Δfmax=480∙103 Hz and (TS 38.211 [41]). The offset is counted from the beginning of a subframe #0 of the reference TRP to the beginning of the closest subsequent subframe of this neighbour TRP. Scale factor 1 Tc.

rtd-Quality This field specifies the quality of the RTD.

We understand the current RTD granularity of 1 Tc is already sufficient to cover the correction information without any enhancements.

Observation 4: LMF could adjust the RTD in the assistance data for the purpose of providing the correction information from the PRU in UE-based DL-TDOA.

For angle-based correction, in Rel-16, DL-AoD is based on boresight direction of each beam, and it is not clear how a PRU can obtain the boresight direction of each DL PRS. In Rel-17, DL-AoD can be further based on power difference between PRS on each angle, and the correction data can be obtained based on RSRP/RSRPP measurement report from the PRU for a given angle corresponding to a PRU.

Agreement From the RAN1 perspective, for the TRP beam/antenna information to be optionally provided by the LMF to the UE for UE-based DL-AoD: - The LMF provides the quantized version of the relative Power between PRS resources per angle per TRP. - The relative power is defined with respect to the peak power in each angle - For each angle, at least two PRS resources are reported. - Note: the peak power per angle is not provided - Note: up to RAN3 to decide how the TRP beam information is provided to the LMF for both UE-assisted and UE-based - Send an LS to RAN2/RAN3 to decide on the signaling details

Observation 5: LMF could adjust the DL-AoD beam pattern in Rel-17 in the assistance data for the purpose of providing the correction information from the PRU in UE-based DL-AoD.

With the discussion above, we have the following proposal on the reply to the first question.

Proposal 2: RAN1 to reply to RAN2 with the question of correction information for UE-based mode:

It is RAN1 understanding that the correction information for UE-based DL-TDOA can be provided via NR-RTD-Info in Rel-16 without change of specification, and that the correction information for UE-based DL-AoD can be provided via the Rel-17 TRP beam/antenna information already agreed.

On the second question in the action points from RAN2 [1]:

RAN2 also kindly asks RAN1 to provide further details on the "PRU antenna orientation information" which should be provided to an LMF.

The antenna orientation in RAN1 consideration is usually reflected by the LCS-GCS translation, which is already supported at the TRP side, for angle based methods, e.g. UL-AoA or DL-AoD.

In addition, RAN1 already made the following agreement on UE requesting and LMF provision of the expected DL AoA if UE is aware of its orientation.

Agreement For the purpose of both UE-B and UE-A DL-AoD, and with regards to the support of AOD measurements with an expected uncertainty window, the following is supported - Indication of expected angle value and uncertainty (of the expected azimuth and zenith angle value) range(s) is signaled by the LMF to the UE - The type of expected angle and uncertainty can be requested by the UE, between the following options - Option 1: Indication of expected DL-AoD/ZoD value and uncertainty (of the expected DL-AoD/ZoD value) range(s) is signaled by the LMF to the UE - Option 2: Indication of expected DL-AoA/ZoA value and uncertainty (of the expected DL-AoA/ZoA value) range(s) is signaled by the LMF to the UE

Since we do not have per-antenna measurement report or per-antenna SRS transmission specified, which is different from the current TEG framework, we do not think the orientation defined per antenna for the PRU is helpful, and thus a single antenna orientation per UE is sufficient in this release.

Observation 6: No need to have multiple antenna orientations for a PRU, since the measurement report or the SRS transmission from the PRU cannot be associated with an antenna.

Proposal 3: RAN1 to reply to RAN2 with the question of antenna orientation information:

It is RAN1 understanding that a single LCS-GCS translation per UE is sufficient to convey the antenna orientation of a PRU.

Conclusion

In this contribution, we have the following observations and proposals for PRU.

- Observation 1: LMF autonomously instigating the location procedure for a PRU is not specified in SA2.

- Observation 2: MO-LR from the PRU as a UE can be supported in Rel-17 without any SA/CT specification change.

Observation 3: MT-LR for the PRU as a UE or LMF autonomous location for the PRU, including PRU management will be studied by SA2 in Rel-18 within the SI FS_eLCS_Ph3.

Observation 4: LMF could adjust the RTD in the assistance data for the purpose of providing the correction information from the PRU in UE-based DL-TDOA.

Observation 5: LMF could adjust the DL-AoD beam pattern in Rel-17 in the assistance data for the purpose of providing the correction information from the PRU in UE-based DL-AoD.

Observation 6: No need to have multiple antenna orientations for a PRU, since the measurement report or the SRS transmission from the PRU cannot be associated with an antenna.

- Proposal 1: RAN1 confirm that PRU functionality as a UE can be specified in RAN in Rel-17 time frame.

Proposal 2: RAN1 to reply to RAN2 with the question of correction information for UE-based mode:

It is RAN1 understanding that the correction information for UE-based DL-TDOA can be provided via NR-RTD-Info in Rel-16 without change of specification, and that the correction information for UE-based DL-AoD can be provided via the Rel-17 TRP beam/antenna information already agreed.

Proposal 3: RAN1 to reply to RAN2 with the question of antenna orientation information:

It is RAN1 understanding that a single LCS-GCS translation per UE is sufficient to convey the antenna orientation of a PRU.

- Reference

- R1-2200857, Response LS on Positioning Reference Units (PRUs) for enhancing positioning performance, RAN2 (Qualcomm, CATT), RAN1#108-e, e-Meeting, 21 Feb. – 3 Mar. 2022.

- R1-2200862, Response LS on Positioning Reference Units (PRUs) for enhancing positioning performance, SA2 (CATT), RAN1#108-e, e-Meeting, 21 Feb. – 3 Mar. 2022.

- R1-2200863, Reply LS on Response LS on Positioning Reference Units (PRUs) for enhancing positioning performance, SA2 (Huawei), RAN1#108-e, e-Meeting, 21 Feb. – 3 Mar. 2022.

- R1-2106326, LS on Positioning Reference Units (PRUs) for enhancing positioning performance, RAN1 (CATT),