3GPP TSG RAN WG1 Meeting #108-e	R1-22002471

e-Meeting, February 21 – March 3, 2022

Agenda Item:	8.16.17

Source:	Huawei, HiSilicon

Title:	Rel-17 UE features for UL Tx switching

Document for:	Discussion and Decision

# Introduction

In RAN1#107-e, discussions regarding Rel-17 UE features for UL Tx switching been made as summarized in [1]. In this contribution, our view on Rel-17 UE features for UL Tx switching are provided.

# Discussion

Based on RAN1 discussions on Rel-17 Tx switching, Rel-16 UE behaviors are fully reused to Rel-17. For example, for UL CA Option 1, the mechanism of Rel-16 uplink switching specified in S6.1.6.2 of TS 38.214 is reused, with the additional clarification that a switching between two carriers also covers the case of 2-port transmission to 2-port transmission in addition to the existing cases of 2-port to 1-port and 1-port to 2-port transmissions [2], as shown by the following agreement.

Agreements: - For a UE configured with UL CA Option 1 and with 2Tx-2Tx UL Tx switching between two uplink carriers, the mechanism of uplink switching specified in S6.1.6.2 of TS 38.214 is reused with the following add-on. - When the UE is to transmit a 2-port transmission on one uplink carrier and if the preceding uplink transmission is a 2-port transmission on another uplink carrier, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the two carriers.

- 6.1.6.2 Uplink switching for carrier aggregation For a UE indicating a capability for uplink switching with BandCombination-UplinkTxSwitch for a band combination, and if it is for that band combination configured with uplink carrier aggregation: - If the UE is configured with uplink switching with parameter uplinkTxSwitching, when the UE is to transmit in the uplink based on DCI(s) received before T0-Toffset or based on a higher layer configuration(s): - When the UE is to transmit a 2-port transmission on one uplink carrier and if the preceding uplink transmission is a 1-port transmission on another uplink carrier, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the two carriers. - When the UE is to transmit a 1-port transmission on one uplink carrier and if the preceding uplink transmission is a 2-port transmission on another uplink carrier, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the two carriers. - For the UE configured with uplinkTxSwitchingOption set to 'switchedUL', when the UE is to transmit a 1-port transmission on one uplink carrier and if the preceding uplink transmission was a 1-port transmission on another uplink carrier, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the two carriers. - For the UE configured with uplinkTxSwitchingOption set to 'dualUL', when the UE is to transmit a 2-port transmission on one uplink carrier and if the preceding uplink transmission was a 1-port transmission on the same uplink carrier and the UE is under the operation state in which 2-port transmission cannot be supported in the same uplink carrier, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the two carriers. - For the UE configured with uplinkTxSwitchingOption set to 'dualUL', when the UE is to transmit a 1-port transmission on one uplink carrier and if the preceding uplink transmission was a 1-port transmission on another uplink carrier and the UE is under the operation state in which 2-port transmission can be supported on the same uplink carrier, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the two carriers. - The UE is not expected to be scheduled or configured with uplink transmissions that result in simultaneous transmission on two antenna ports on one uplink carrier, and any transmission on another uplink carrier. - In all other cases the UE is expected to transmit normally all uplink transmissions without interruptions.

Compared with the existing UE behaviors as highlighted above, the new add-on in the agreement is obviously a simple clarification for UL-CA Option 1, which causes no unrealistic UE implementation issue. Therefore, we don’t see a need to have new UE capability for UL-CA Option 1.

For UL CA Option 2, based on the Rel-17 agreement [2] and Rel-16 agreement [3], the only new UE behavior for the additional switching state is a simple clarification on the state ambiguity issue [4]. It does not cause any unrealistic UE implementation, as shown by the following agreements.

Rel-17 [2][4]:

Agreements: - For inter-band UL CA, if 2Tx-2Tx UL Tx switching between two uplink carriers is configured: - For option 2 of mapping between UL transmission ports and Tx chain - The switching period is only applicable in the following cases: - If the current state of Tx chains is 1Tx on carrier 1 and 1Tx on carrier 2, the next UL transmission has a 2-port transmission on either carrier 1 or carrier 2. - If the current state of Tx chains is 0Tx on carrier 1 and 2Tx on carrier 2, the next UL transmission has a 1-port or 2-port transmission on carrier 1. - If the current state of Tx chains is 2Tx on carrier 1 and 0Tx on carrier 2, the next UL transmission has a 1-port or 2-port transmission on carrier 2. - For other cases, the state of Tx chains of last UL transmission is assumed. - Note: For SUL, UL CA option 1 and UL CA option 2, in RAN1 understanding, no spec change to power configuration and power control.

Agreement: - For UL-CA Option2, if UL Tx switching is triggered for 1-port transmission on a carrier and the state of Tx chains after the UL Tx switching is not unique, introduce a new RRC parameter to configure between 1) and 2) - 1) The state of Tx chains supporting 2Tx transmission on the carrier is assumed. - 2) 1Tx on carrier 1 and 1Tx on carrier 2 is assumed.

Rel-16 [3]:

Agreements: - For inter-band UL CA, if uplink Tx switching is configured: - For option 1 of mapping between UL transmission ports and Tx chain, the switching period is only applicable when the UL transmissions are switched between 1Tx carrier 1 and 2Tx carrier 2. - Note: 2Tx carrier 2 refers to an UL carrier capable of 2 Tx chains and both 1-port and 2-port UL transmissions. - For option 2 of mapping between UL transmission ports and Tx chain - The switching period is only applicable in the following cases: - If the current state of Tx chains is 1 Tx on carrier 1 and 1Tx on carrier 2, the next UL transmission has a 2-port transmission on carrier 2. - If the current state of Tx chains is 0 Tx on carrier 1 and 2Tx on carrier 2, the next UL transmission has a 1-port transmission on carrier 1. - For other cases, the state of Tx chains of last UL transmission is assumed. - Note: No spec change to power configuration and power control.

Therefore, for UL-CA Option 2, the only difference of UE behavior is also very small, which obviously cause no unrealistic implementation issue. We don't see a need to have new UE capability for UL-CA Option 2.

There is existing FG 22-1 to indicate Option 1/Option 2 and it is per BC capability since Rel-16, as shown below.

| Features | Index | Feature group | Components |
| --- | --- | --- | --- |
| 22. NR Others | 22-1 | Indicating supported option for UL Tx switching for inter-band UL CA | Indicating supported option for UL Tx switching for inter-band UL CA Candidate values set is {option1, option2, both option 1 and option 2} |

In RAN1#107-e, new UE capability FG 37-1 is proposed as “Indicating supported option for UL Tx switching for 2Tx-2Tx inter-band UL CA”  [1]. The reasoning seemed that Rel-17 UL Tx switching is new so that new UE capability is needed.

| 37. NR_RF_FR1_enh | 37-1 | Indicating supported option for UL Tx switching for 2Tx-2Tx inter-band UL CA | Indicating supported option for 2Tx-2Tx UL Tx switching for inter-band UL CA - Candidate values set is {option1, option2, both option 1 and option 2} | FFS | Yes | N/A |  | Per BC | N/A | N/A (FR1 only) | N/A | If UE supports Option1 for FG 37-1, the UE also needs to support Option1 for FG 22-1; If UE supports Option2 for FG 37-1, the UE also needs to support Option2 for FG 22-1; If UE supports both Option1 and Option2 for FG 37-1, the UE also needs to support both Option1 and Option2 for FG 22-1; | FFS details |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

However, in RAN2#116bis-e, the following agreed CR[5] has shown that the “uplinkTxSwitchingPeriod2T2T” is used to indicated whether 2Tx-2Tx switching is supported by UE supporting 1Tx-2Tx switching. There is no need to add FG 37-1 to indicate whether UE support R17 UL Tx switching.

ULTxSwitchingBandPair-v17xx ::= SEQUENCE { uplinkTxSwitchingPeriod2T2T-r17 ENUMERATED {n35us, n140us, n210us} OPTIONAL }

supportedBandPairListNR-r16, supportedBandPairListNR-v17xx Indicates a list of band pair supporting UL Tx switching as defined in TS 38.101-1 [15] for a given band combination. A UE supporting 2Tx-2Tx switching should include both of supportedBandPairListNR-r16 and supportedBandPairListNR-v17xx. And the UE shall include the same number of entries listed in the same order as in supportedBandPairListNR-r16. If the UE does not support 2Tx-2Tx switching for a given band pair, the field of uplinkTxSwitchingPeriod2T2T in the corresponding entry is absent.

Besides, the proposed FG37-1 overlaps with FG 22-1 and even does not take it as a prerequisite, which seems equivalent to changing the existing FG 22-1 from per BC to per feature set, and thus causes unnecessary troubles for gNBs during network operation.

Another motivation for FG 37-1 seemed to grant a UE the flexibility to report Option 1 only (or Option 2 only) for 2Tx-2Tx even the UE has reported a support of {both Option 1 and Option 2} for 1Tx-2Tx. However, Rel-17 UL CA Option1 is exact the same as Rel-16 UL CA Option1, which both triggers UL Tx switching only when UL carrier is switched. It’s unreasonable that a UE capable of Rel-16 Option 1 and 2Tx-2Tx cannot support Rel-17 Option 1 by the new capability FG 37-1. This would cause unnecessary troubles for gNB only capable of Rel-16 Option 1 and Rel-17 Option 1. Therefore, the new FG37-1 is not justified and unnecessary.

Observation: Since Rel-16 UE behaviors of UL Tx switching is reused with small add-ons for Rel-17, the existing FG 22-1 is sufficient for Rel-17 UL Tx switching while new UE capability FG37-1 is unnecessary.

# Conclusion

In this contribution, analysis of the issues of Rel-17 UE features for UL Tx switching are provided. The following observation is given:

Observation: Since Rel-16 UE behaviors of UL Tx switching is reused with small add-ons for Rel-17, the existing FG 22-1 is sufficient for Rel-17 UL Tx switching while new UE capability FG37-1 is unnecessary.

# References

R1-2112147, “Summary on other UE feature related discussions”, Moderator (NTT DOCOMO, INC.), 3GPP RAN1 #107-e, November 11th – 19 th, 2021.

R1-2106150, “[105-e-NR-R17-TxSwitching-01] Summary of email discussion on Rel-17 uplink Tx switching”, Moderator (China Telecom), 3GPP RAN1 #105-e, May 10th – 27th, 2021.

R1-2002964, “[100b-e-LS-TxSwitching-02] Email discussion/approval of the remaining issues for inter-band UL CA”, Moderator (China Telecom), 3GPP RAN1 #100bis-e, April 20th – 30th, 2020.

R1-2110566, “[106bis-e-NR-R17-TxSwitching-01] Summary of email discussion on Rel-17 uplink Tx switching”, Moderator (China Telecom), 3GPP RAN1 #106bis-e, October 11th – 19th, 2021.

R2-2201501, “Running CR to TS38.331 to support Tx switching enhancements”, Huawei, HiSilicon, China Telecom, Apple, CATT.