3GPP TSG-RAN WG1 Meeting #108-e	R1-2202472

e-Meeting, February 21 – March 3, 2022

Agenda Item:	5

Source:	Huawei, HiSilicon

Title:	On CORESET#0 impact of smaller CBW of n79

Document for:	Discussion and Decision

Introduction

In RAN4#101bis-e, the updated channel bandwidth (CBW) support to the band n79 for the new scenarios (e.g., RedCap) was discussed and the CORESET#0 impact which may cause non-backward compatible issue was identified. An LS from RAN4 was sent to RAN1 [1] about this issue.

On CORSET#0 impact

In TS 38.213, upon detection of SS/PBCH block, UE determines CORESET#0 from MIB by looking up the table for controlResourceSetZero. The table is determined according to subcarrier spacing of SSB, subcarrier spacing of PDCCH, and minimum channel bandwidth of the frequency band where UE located. For example, a new UE supporting 10 MHz the minimum channel bandwidth for n79 will have a different table (table 13.4 in TS 38.213) for CORESET#0 configuration with a legacy UE still supporting 40 MHz (table 13.6 in TS 38.213). Thus, how a UE determine the table in this scenario needs further clarifications.

Table 13-4: Set of resource blocks and slot symbols of CORESET for Type0-PDCCH search space set when {SS/PBCH block, PDCCH} SCS is {30, 30} kHz for frequency bands with minimum channel bandwidth 5 MHz or 10 MHz

| Index | SS/PBCH block and CORESET multiplexing pattern | Number of RBs | Number of Symbols | Offset (RBs) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 24 | 2 | 0 |
| 1 | 1 | 24 | 2 | 1 |
| 2 | 1 | 24 | 2 | 2 |
| 3 | 1 | 24 | 2 | 3 |
| 4 | 1 | 24 | 2 | 4 |
| 5 | 1 | 24 | 3 | 0 |
| 6 | 1 | 24 | 3 | 1 |
| 7 | 1 | 24 | 3 | 2 |
| 8 | 1 | 24 | 3 | 3 |
| 9 | 1 | 24 | 3 | 4 |
| 10 | 1 | 48 | 1 | 12 |
| 11 | 1 | 48 | 1 | 14 |
| 12 | 1 | 48 | 1 | 16 |
| 13 | 1 | 48 | 2 | 12 |
| 14 | 1 | 48 | 2 | 14 |
| 15 | 1 | 48 | 2 | 16 |

Table 13-6: Set of resource blocks and slot symbols of CORESET for Type0-PDCCH search space set when {SS/PBCH block, PDCCH} SCS is {30, 30} kHz for frequency bands with minimum channel bandwidth 40MHz

| Index | SS/PBCH block and CORESET multiplexing pattern | Number of RBs | Number of Symbols | Offset (RBs) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 24 | 2 | 0 |
| 1 | 1 | 24 | 2 | 4 |
| 2 | 1 | 24 | 3 | 0 |
| 3 | 1 | 24 | 3 | 4 |
| 4 | 1 | 48 | 1 | 0 |
| 5 | 1 | 48 | 1 | 28 |
| 6 | 1 | 48 | 2 | 0 |
| 7 | 1 | 48 | 2 | 28 |
| 8 | 1 | 48 | 3 | 0 |
| 9 | 1 | 48 | 3 | 28 |
| 10 | Reserved |  |  |  |
| 11 | Reserved |  |  |  |
| 12 | Reserved |  |  |  |
| 13 | Reserved |  |  |  |
| 14 | Reserved |  |  |  |
| 15 | Reserved |  |  |  |

In the LS, RAN4 has come up with four potential solutions as follows:

- Alt-1: Add narrower channel bandwidth to n79, and indicate different table to legacy UE and new UE for CORESET#0 configuration. - Alt-1a: Add narrower channel bandwidth to n79, and indicate the same table 13.6 to legacy UE and new UE for CORESET#0 configuration. - Rationale: Legacy UE not supporting channel bandwidth lower than 40MHz will always look at table 13.6, this table shall then be the common one. - Alt-1b: Add narrower channel bandwidth to n79, and indicate: - A UE synchronizing on a GSCN with step 16 shall look at the table 13.6. - Rationale: This enables new and legacy to connect in band n79 using the same CORESET#0 configuration. - A UE synchronizing on a GSCN with step 1 but not step 16 shall look at the table 13.4. - Rationale: Table 3.4 offers more flexibility on CORESET#0 configuration - Alt-2: Add narrower channel bandwidth to new band nX instead of n79.

In our view, both Alt-1 and Alt-2 may not be preferable, Alt-1 will lead to the situation that only one CORESER#0 configuration can be used for n79, which will much reduce the configuration flexibility of CORESET#0. Alt-2 needs to introduce new band nX instead of n79, which will increase UE complexity to support both n79 and nX.

By comparison, Alt-1a and Alt-1b seems more reasonable. For Alt-1a, both legacy UE and new UE will always look at Table 13.6, thus no confusion will arise for these two types of UE. However, with this method, for an n79 carrier with less than bandwidth 40MHz, CORESET#0 has to been configured based on Table 13.6. Considering Table 13.4 have more configurations than Table 13.6, especially the offset between CORESET#0 and SSB, it will lose some configuration flexibility. Alt-1b can provide configuration flexibility, however, it has some impact on UE’s initial access procedure. For a UE synchronizing on a GSCN with step 1, before receiving SIB1, it will have to re-calculate if the GSCN is with step 16 or not. Note in our understanding it should be step (size) 16 instead of 4 as given in [1].

In summary, in our view, Alt-1a and Alt-1b can both resolve the issue well, and Alt-1b is more preferable if UE/Chip vendors can accept the incremental of UE complexity. Thus, we have the following proposal:

Proposal: From RAN1 perspective, Alt-1b is more preferable assuming the introduced UE complexity is negligible.

Conclusions

Based on the analysis, we have the following proposals:

Proposal: From RAN1 perspective, Alt-1b is more preferable assuming the introduced UE complexity is negligible.

- References

R1-2200907, LS on CORESET#0 impact of CBW narrower than 40MHz of n79, RAN4#101bis-e, 17th – 25th January 2022.