3GPP TSG-RAN WG1 Meeting #108-e	R1-2202474

e-Meeting, February 21 – March 3, 2022

Agenda Item:	5

Source:	Huawei, HiSilicon

Title:	Discussion on NTN Neighbor Cell measurement and Satellite Information

Document for:	Discussion and Decision

# Introduction

NTN RRM measurements and mobility requirements have been discussed in RAN4 for a couple of meetings. In this contribution, we discuss the relevant information that are needed for UE measurements and mobility according to the information in [1] [2].

# Discussion

## Parameters for NTN UE measurement

In the current NR, a UE will setup the SS/PBCH block measurement timing configuration (SMTC) in accordance with the parameters provided by the SMTC configuration. For TN, the timing of SS/PBCH reception at the UE is almost the same as the SS/PBCH transmission at the gNB, the SMTC configuration can be applied during the whole measurement gap. However, in NTN, the timing of SS/PBCH reception at the UE will be changed due to the movement of satellites, which mean that the SMTC configuration will be outdated during the measurement gap. The timing drift of SS/PBCH block measurement will be impacted by the timing drift from both feeder link and service link part.  For the service link part, the timing drift can be calculated by the ephemeris and UE’s location and for the feeder link part, the timing drift can be calculated by the indicated common TA parameters. It should be noted that epoch time is also needed for the propagation of ephemeris and common TA.

Observation 1: The SMTC configuration will be outdated in NTN due to the movement of satellites.

Considering the ephemeris information, both PVT and Keplerian based format are supported according to the current agreement. The validity timer is an assistant information for UE to re-acquire the ephemeris and common TA parameters to ensure the accuracy of satellite position and common TA at the UE side, so it also need to be indicated and should as accurate as that for the serving cell. As the length of validity timer is related to the accuracy of the indicated ephemeris which may be different among satellites, it should be different from the serving cell.

Observation 2:  The validity timer for ephemeris is related to the accuracy of the indicated ephemeris which may be different among satellites

As the DL frequency compensation by gNB for the service link Doppler is not supported in Release 17, the DL frequency compensation is zero and is not need to be indicated.

In RAN1#106bis-e, polarization signalling for non-serving cell in RRM measurement configuration has been agreed. As the neighbor cell may have different polarization mode from the serving cell, indicate the DL polarization of the neighbor cell is beneficial to reduce the measurement complexity, otherwise, UE will have to measure with the both polarization modes. Therefore, DL polarization information is also needed for NTN UE measurement.

Observation 3: Indicate neighbor cell’s polarization information can reduce the complexity of UE measurement.

Proposal 1: For NTN UE measurement, support indication of the following information:

- Neighbor cell ephemeris information

- Common TA parameters

- Epoch time

- Validity timer for common TA and ephemeris

- DL polarization information

## Parameters for NTN UE mobility

For the target cell measurement, the parameters needed is the same as the neighbor cell measurement which has been analysed above. For handover within or inter satellite, all the information that are needed for UL time and frequency synchronization should be indicated including ephemeris, common TA parameters and their validity timer and epoch time. In addition, the parameters for timing relationship is also needed including Koffset for UL scheduling, Kmac for RARwindow offset estimation. DL and UL Polarization information is also needed for the UE to decide which polarization should be applied for DL reception and UL transmission during the PRACH.

Proposal 2: For NTN UE mobility, support indication of the following information:

- Neighbor cell ephemeris information

- Common TA parameters

- Epoch time

- Validity timer for common TA and ephemeris

- DL and UL polarization information

- Koffset

- Kmac

# Conclusions

In this contribution, relevant information that are needed for UE measurements and mobility are discussed. The following observations and proposals are presented:

Observation 1: The SMTC configuration will be outdated in NTN due to the movement of satellites.

Observation 2:  The validity timer for ephemeris is related to the accuracy of the indicated ephemeris which may be different among satellites

Observation 3:  Indicate neighbor cell’s polarization information can reduce the complexity of UE measurement.

Proposal 1: For NTN UE measurement, support indication of the following information:

- Neighbor cell ephemeris information

- Common TA parameters

- Epoch time

- Validity timer for common TA and ephemeris

- DL polarization information

Proposal 2: For NTN UE mobility, support indication of the following information:

- Neighbor cell ephemeris information

- Common TA parameters

- Epoch time

- Validity timer for common TA and ephemeris

- DL and UL polarization information

- Koffset

- Kmac

# References

R4-2120309, LS on NR NTN Neighbor Cell and Satellite Information.

R2-2201884, Reply LS on NR NTN Neighbor Cell and Satellite Information.