3GPP TSG RAN WG4 #108-e	                       		R1-2202489 (a revision of R1-220067)

e-Meeting, Feb 21st – March 3rd, 2022

Title: 	MEO Scenarios and Characteristics to be added in TR 38.821

Source: 	Hughes/EchoStar

## Agenda Item:	          8.4 Maintenance on Solutions for NR to support non-terrestrial networks (NTN)

Document for:	Discussion and Approval

# 1. Introduction

The purpose of this contribution is to ensure study results that apply to GEO and NGSO with circular orbit at altitude greater than 600 km also apply to MEO. MEO scenarios feature characteristics that are less challenging than GEO scenarios in terms of latency, and less challenging than LEO scenarios for the doppler effects. In NTN-NR normative work, many analyses on GEO and LEO scenarios had assumed support for MEO scenarios.

There is a wide range of delay tolerant applications which require global coverage and continuous service. Such services are ideally suited for support by a satellite network offering larger and adequate performance such as MEO. Examples are maritime use cases for non-critical asset tracking where a data logger is currently used, livestock monitoring across dispersed rural areas, and other critical and non-critical asset tracking in landline, environmental monitoring, wildlife monitoring, environmental monitoring, climate change and so on.

# 2. Background of MEO Characteristics in TR 38.821

The following observations and proposal on MEO characteristics in R2-1914721 [1] were approved:

Observation 1: Propagation round trip delay (RTD) for bent pipe GEO satellite at 35786 km is about 545 ms, while a typical LEO system at 600 km has RTD of 28.4 ms. The RTD for MEO or any NGSO satellite with altitude above 600 km falls in the range from 28.4 ms to 545 ms.

Observation 2: The Differential One-Way Delay between nadir and EOC paths for GEO satellite at 35786 km is 16 ms and for LEO satellite at 600 km is 4.44ms. For MEO or any NGSO satellite with altitude between LEO and GEO, the Differential One-Way Delay between nadir and EOC is in the range from 4.44ms to 16ms, therefore it is within the range of LEO and GEO.

Observation 3: The Doppler shift and variation of MEO constellation or any NGSO satellite with altitude between LEO and GEO is smaller than that of for LEO constellation.

Observation 4: The RTT for MEO is less than for GEO but greater than for LEO and hence it is FFS if HARQ can be used

Proposal 1: Add a sentence in the TR 38.821:

“The study results apply to GEO scenarios as well as all NGSO scenarios with circular orbit at altitude greater than or equal to 600 km”.

Studies in TR 38.811 and solutions developed for GEO and LEO scenarios in TR 38.821 demonstrate that the doppler effect and latency for NGSO constellation (with altitude greater than 600 km) are less challenging than GEO scenarios in terms of latency and less challenging than LEO at 600 km scenarios for the doppler effects.

As a result, in TR 38.821 it was noted “The study results apply to GEO scenarios as well as all NGSO scenarios with circular orbit at altitude greater than or equal to 600 km”.

# 3. MEO Characteristics for NTN reference scenarios

This section illustrates the MEO characteristics for NTN reference scenarios. Based on conclusion in section 2, the proposal to RAN1 is to add reference scenario and parameters for MEO based non-terrestrial access network generating fixed beams whose footprints move with the satellite (altitude 10000 km).

Proposal 1: To add MEO scenario E in Table 4.2-1 in TR 38.821

|  | Transparent satellite | Regenerative satellite |
| --- | --- | --- |
| GEO based non-terrestrial access network | Scenario A | Scenario B |
| LEO based non-terrestrial access network: steerable beams | Scenario C1 | Scenario D1 |
| LEO based non-terrestrial access network: the beams move with the satellite | Scenario C2 | Scenario D2 |
| MEO based non-terrestrial access network: the beams move with the satellite | Scenario E1 | Scenario E2 |

Proposal 2: To add MEO NTN reference scenario parameters in Table 4.2-2 in TR 38.821

| Scenarios | GEO based non-terrestrial access network (Scenario A and B) | LEO based non-terrestrial access network (Scenario C & D) | MEO based non-terrestrial access network (Scenario E) |
| --- | --- | --- | --- |
| Orbit type | notional station keeping position fixed in terms of elevation/azimuth with respect to a given earth point | circular orbiting around the earth | Circular orbiting altitude around the earth |
| Altitude | 35,786 km | 600 km 1,200 km | 10,000 km |
| Spectrum (service link) | <6 GHz (e.g. 2 GHz) >6 GHz (e.g. DL 20 GHz, UL 30 GHz) |  |  |
| Max channel bandwidth capability (service link) | 30 MHz for band < 6 GHz 1 GHz for band > 6 GHz |  |  |
| Payload | Scenario A: Transparent (including radio frequency function only) Scenario B: regenerative (including all or part of RAN functions) | Scenario C: Transparent (including radio frequency function only) Scenario D: Regenerative (including all or part of RAN functions) | Scenario E MEO based non-terrestrial access network: the beams move with the satellite Scenario E1: Transparent, Scenario E2: Regenerative |
| Inter-Satellite link | No | Scenario C: No Scenario D: Yes/No (Both cases are possible.) | No |
| Earth-fixed beams | Yes | Scenario C1: Yes (steerable beams), see note 1 Scenario C2: No (the beams move with the satellite) Scenario D 1: Yes (steerable beams), see note 1 Scenario D 2: No (the beams move with the satellite) | The beams move with satellite |
| Max beam foot print size (edge to edge) regardless of the elevation angle | 3500 km (Note 5) | 1000 km | 1500 km |
| Min Elevation angle for both sat-gateway and user equipment | 10° for service link and 10° for feeder link | 10° for service link and 10° for feeder link | 10° for service link and 10° for feeder link |
| Max distance between satellite and user equipment at min elevation angle | 40,581 km | 1,932 km (600 km altitude) 3,131 km (1,200 km altitude) | 14,018 km |
| Max Round Trip Delay (propagation delay only) | Scenario A: 541.46 ms (service and feeder links) Scenario B: 270.73 ms (service link only) | Scenario C: (transparent payload: service and feeder links) 25.77 ms (600km) 41.77 ms (1200km) Scenario D: (regenerative payload: service link only) 12.89 ms (600km) 20.89 ms (1200km) | 95.19 ms (transparent) 47.60 ms (regenerative) |
| Max differential delay within a cell (Note 6) | 10.3 ms | 3.12 ms and 3.18 ms for respectively 600km and 1200km | TBD |
| Max Doppler shift (earth fixed user equipment) | 0.93 ppm | 24 ppm (600km) 21ppm(1200km) | 7.5 ppm |
| Max Doppler shift variation (earth fixed user equipment) | 0.000 045 ppm/s | 0.27ppm/s (600km) 0.13ppm/s(1200km) | 0.003 ppm/s |
| User equipment motion on the earth | 1200 km/h (e.g. aircraft) | 500 km/h (e.g. high speed train) Possibly 1200 km/h (e.g. aircraft) | 500 km/h (e.g. high-speed train) Possibly 1200 km/h (e.g. aircraft) |
| User equipment antenna types | Omnidirectional antenna (linear polarisation), assuming 0 dBi Directive antenna (up to 60 cm equivalent aperture diameter in circular polarisation) |  |  |
| User equipment Tx power | Omnidirectional antenna: UE power class 3 with up to 200 mW Directive antenna: up to 20 W |  |  |
| User equipment Noise figure | Omnidirectional antenna: 7 dB Directive antenna: 1.2 dB |  |  |
| Service link | 3GPP defined New Radio |  |  |
| Feeder link | 3GPP or non-3GPP defined Radio interface | 3GPP or non-3GPP defined Radio interface | 3GPP or non-3GPP defined Radio interface |
| NOTE 1: Each satellite has the capability to steer beams towards fixed points on earth using beamforming techniques. This is applicable for a period of time corresponding to the visibility time of the satellite NOTE 2: Max delay variation within a beam (earth fixed user equipment) is calculated based on Min Elevation angle for both gateway and user equipment NOTE 3: Max differential delay within a beam is calculated based on Max beam footprint diameter at nadir NOTE 4: Speed of light used for delay calculation is 299792458 m/s. NOTE 5: The Maximum beam footprint size for GEO is based on current state of the art GEO High Throughput systems, assuming either spot beams at the edge of coverage (low elevation). NOTE 6: The maximum differential delay at cell level has been computed considering the one at beam level for largest beam size. It does not preclude that cell may include more than one beam when beam size is small or medium size. However the cumulated differential delay of all beams within a cell will not exceed the maximum differential delay at cell level in the table above. |  |  |  |

“The NTN study results apply to GEO scenarios as well as all NGSO scenarios with circular orbit at altitude greater than or equal to 600 km.”

# 4. Scenario Parameters for MEO

The doppler shift/variation and the delay variation for MEO are smaller than for LEO. The maximum delay for MEO is smaller than for GEO. The NTN-NR enhancements for LEO and GEO should be sufficient to support MEO, however it is important to capture MEO specific characteristics and its parameters.

Table 1 below shows the two sets of parameters that were defined for NR NTN studies (i.e., “set 1” and “set 2” specified in TR 38.821)

Proposal 3: To add MEO characteristics, to Set 1 and Set 2 MEO characteristics.

Table 6.1.1.1-1: Set-1 satellite parameters for system level simulator calibration

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 | MEO |
| --- | --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km | 600 km | 10000 km |
| Satellite antenna pattern |  | Section 6.4.1 in [2] | Section 6.4.1 in [2] | Section 6.4.1 in [2] | Section 6.4.1 in [2] |
| Payload characteristics for DL transmissions |  |  |  |  |  |
| Equivalent satellite antenna aperture (Note 1) | S-band (i.e. 2 GHz) | 22 m | 2 m | 2 m | 5 m |
| Satellite EIRP density |  | 59 dBW/MHz | 40 dBW/MHz | 34 dBW/MHz | 51 dBW/MHz |
| Satellite Tx max Gain |  | 51 dBi | 30 dBi | 30 dBi | 38.6 dBi |
| 3dB beamwidth |  | 0.4011 deg | 4.4127 deg | 4.4127 deg | 2.1 degrees |
| Satellite beam diameter (Note 2) |  | 250 km | 90 km | 50 km | 150 km |
| Equivalent satellite antenna aperture (Note 1) | Ka-band (i.e. 20 GHz for DL) | 5 m | 0.5 m | 0.5 m | 2 m |
| Satellite EIRP density |  | 40 dBW/MHz | 10 dBW/MHz | 4 dBW/MHz | 32 dBW/MHz |
| Satellite Tx max Gain |  | 58.5 dBi | 38.5 dBi | 38.5 dBi | 50 dBi |
| 3dB beamwidth |  | 0.1765 deg | 1.7647 deg | 1.7647 deg | 0.53 degrees |
| Satellite beam diameter (Note 2) |  | 110 km | 40 km | 20 km | 77 km |
| Payload characteristics for UL transmissions |  |  |  |  |  |
| Equivalent satellite antenna aperture (Note1) | S-band (i.e. 2 GHz) | 22 m | 2 m | 2 m | 5 m |
| G/T |  | 19 dB K-1 | 1.1 dB K-1 | 1.1 dB K-1 | 13 dB/K |
| Satellite Rx max Gain |  | 51 dBi | 30 dBi | 30 dBi | 38.6 dBi |
| Equivalent satellite antenna aperture (Note1) | Ka-band (i.e. 30 GHz for UL) | 3.33 m | 0.33 m | 0.33 m | 1.5 m |
| G/T |  | 28 dB K-1 | 13 dB K-1 | 13 dB K-1 | 22.5 dB/K |
| Satellite RX max Gain |  | 58.5 dBi | 38.5 dBi | 38.5 dBi | 48.14 dBi |
| NOTE 1: This value is equivalent to the antenna diameter in Sec. 6.4.1 of [2]. NOTE 2: This beam size refers to the Nadir pointing of the satellite NOTE 3: All these satellite parameters are applied per beam. NOTE 4: The EIRP density values are considered identical for all frequency re-use factor options. NOTE 5: The EIRP density values are provided assuming the satellite HPA is operated with a back-off of [5] dB. |  |  |  |  |  |

Table 6.1.1.1-2: Set-2 satellite parameters for system level simulator calibration

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 | MEO |
| --- | --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km | 600 km | 10000 km |
| Satellite antenna pattern |  | Section 6.4.1 in [2] | Section 6.4.1 in [2] | Section 6.4.1 in [2] | Section 6.4.1 in [2] |
| Payload characteristics for DL transmissions |  |  |  |  |  |
| Equivalent satellite antenna aperture (Note 1) | S-band (i.e. 2 GHz) | 12 m | 1 m | 1 m | 1.5 m |
| Satellite EIRP density |  | 53.5 dBW/MHz | 34 dBW/MHz | 28 dBW/MHz | 45.4 dBW/MHz |
| Satellite Tx max Gain |  | 45.5 dBi | 24 dBi | 24 dBi | 28.1 dBi |
| 3dB beamwidth |  | 0.7353 deg | 8.8320 deg | 8.8320 deg | 6.5 degrees |
| Satellite beam diameter (Note 2) |  | 450 km | 190 km | 90 km |  |
| Equivalent satellite antenna aperture (Note 1) | Ka-band (i.e. 20 GHz for DL) | 2 m | 0.2 m | 0.2 m | 0.5 m |
| Satellite EIRP density |  | 32 dBW/MHz | 2 dBW/MHz | -4 dBW/MHz | 20 dB/MHz |
| Satellite Tx max Gain |  | 50.5 dBi | 30.5 dBi | 30.5 dBi | 38.6 dBi |
| 3dB beamwidth |  | 0.4412 deg | 4.4127 deg | 4.4127 deg | 2.01 degrees |
| Satellite beam diameter (Note 2) |  | 280 km | 90 km | 50 km | 150 km |
| Payload characteristics for UL transmissions |  |  |  |  |  |
| Equivalent satellite antenna aperture (Note1) | S-band (i.e. 2 GHz) | 12 m | 1 m | 1 m | 1.5 m |
| G/T |  | 14 dB K-1 | -4.9 dB K-1 | -4.9 dB K-1 | 3.8 dB/K |
| Satellite Rx max Gain |  | 45.5 dBi | 24 dBi | 24 dBi | 28.1 dBi |
| Equivalent satellite antenna aperture (Note1) | Ka-band (i.e. 30 GHz for UL) | 1.33 m | 0.13 m | 0.13 m | 0.33 m |
| G/T |  | 20 dB K-1 | 5 dB K-1 | 5 dB K-1 | 12.9 dB/K |
| Satellite Rx max Gain |  | 50.5 dBi | 30.5 dBi | 30.5 dBi | 38.51 dBi |
| NOTE 1: This value is equivalent to the antenna diameter in Sec. 6.4.1 of [2]. NOTE 2: This beam size refers to the Nadir pointing of the satellite NOTE 3: All these satellite parameters are applied per beam. NOTE 4: The EIRP density values are considered identical for all frequency re-use factor options. |  |  |  |  |  |

Proposal 4: To include MEO parameters for link budget analysis in a new Table 6.1.1.1-1 and 6.1.1.1-2 in TR 38.821, as a representative characterization of NTN-NR scenarios with MEO altitude and characteristics.

# 4.0 MEO Characteristics and Link budgets

For MEO link budget analysis using Set 1 and 2, the following parameters could be utilized:

|  | MEO |
| --- | --- |
| Central beam edge elevation | 30 degrees |
| Central beam edge satellite-UE distance | 12,227 km |

Based on Set 1 parameters and using the computation (6.6-2) and (6.6-3) in TR 38.811 and (6.1.3.1-2) in TR 38.821, the link budget results for CNR are as follows:

- S-band, UE= handheld

|  | Option 1 (bandwidth 30 MHz) |  | Option 2 (bandwidth 10 MHz) |  |
| --- | --- | --- | --- | --- |
| Transmission mode | DL | UL | DL | UL |
| Frequency [GHz] | 2.00 | 2.00 | 2.00 | 2.00 |
| TX: EIRP [dBm] | 95.77 | 23.01 | 91.00 | 23.01 |
| RX: G/T [dB/T] | -31.60 | 13.00 | -31.60 | 13.00 |
| Bandwidth [MHz] | 30.00 | 0.36 | 10.00 | 0.36 |
| Free space path loss [dB] | 180.22 | 180.22 | 180.22 | 180.22 |
| Atmospheric loss [dB] | 0.16 | 0.16 | 0.16 | 0.16 |
| Shadow fading margin [dB] | 3.00 | 3.00 | 3.00 | 3.00 |
| Scintillation Loss [dB] | 2.20 | 2.20 | 2.20 | 2.20 |
| Polarization loss [dB] | 0.00 | 0.00 | 0.00 | 0.00 |
| Additional losses [dB] | 0.00 | 0.00 | 0.00 | 0.00 |
| CNR [dB] | 2.42 | -6.53 | 2.42 | -6.53 |

- Ka-band, UE=VSAT

|  | Option 1 (BW=400 MHz) |  | Option 2 (BW=133 MHz) |  |
| --- | --- | --- | --- | --- |
| Transmission mode | DL | UL | DL | UL |
| Frequency [GHz] | 20.00 | 30.00 | 20.00 | 30.00 |
| TX: EIRP [dBm] | 88.02 | 76.21 | 83.24 | 76.21 |
| RX: G/T [dB/T] | 15.86 | 22.50 | 15.86 | 22.50 |
| Bandwidth [MHz] | 400.00 | 400.00 | 133.00 | 133.00 |
| Free space path loss [dB] | 200.22 | 203.74 | 200.22 | 203.74 |
| Atmospheric loss [dB] | 0.52 | 0.50 | 0.52 | 0.50 |
| Shadow fading margin [dB] | 0.00 | 0.00 | 0.00 | 0.00 |
| Scintillation Loss [dB] | 1.08 | 1.08 | 1.08 | 1.08 |
| Polarization loss [dB] | 0.00 | 0.00 | 0.00 | 0.00 |
| Additional losses [dB] | 0.00 | 0.00 | 0.00 | 0.00 |
| CNR [dB] | 14.64 | 5.97 | 14.64 | 10.75 |

Observation 1: Comparing with the Link Budget Results for NTN in R1-1913351 [3], the values are in line with the expectation that they are lower than LEO-1200 but higher than GEO.

# 5. Description of coexistence scenario to be studied in RAN4

RAN4 may discuss whether it is necessary to do NTN-TN, TN-NTN co-existence analysis with NTN MEO. If calibration results for NTN MEO show that CDF=f(CL) and CDF=f(SINR) demonstrate similar (or lower) values than GEO and/or LEO@1200, coexistence studies may not be required.

Since MEO scenarios demonstrate intermediate parameter (in terms of power, pathloss, antenna gain, and etc.) with respect to LEO and GEO scenarios, evaluation of SINR and Coupling Loss may be sufficient to determine if LEO and GEO are already covering MEO scenarios.

Proposal 5: If calibration results for NTN MEO show equivalent (or lower) SINR and CL as for NTN LEO@1200 and NTN GEO, RAN4 should not consider performing NTN MEO coexistence analysis for deriving NTN requirements.

Proposal 6: RAN4 should consider only (Phase 0) calibration for MEO and not perform any coexistence analysis (Phase 1 and Phase 2) if calibration results show that MEO constellation requirement are within those of LEO and GEO.

# 6. Summary and conclusions

This document presents information of relevance for 3GPP to consider the specific adaptation of NTN-NR for deployment of MEO. The document describes the main characteristics of MEO and its related parameters.

Proposal 1: To add MEO as “scenario E” in Table 4.2-1 in TR 38.821.

Proposal 3: To add MEO NTN reference scenario parameters in Table 4.2-2 in TR 38.821.

Proposal 3: To add MEO characteristics, to Set 1 and Set 2 MEO characteristics.

Proposal 4: To include MEO parameters for link budget analysis in a new Table 6.1.1.1-1 and 6.1.1.1-2 in TR 38.821, as a representative characterization of NTN-NR scenarios with MEO altitude and characteristics.

Proposal 5: If calibration results for NTN MEO show equivalent (or lower) SINR and CL as for NTN LEO@1200 and NTN GEO, RAN4 should not consider performing NTN MEO coexistence analysis for deriving NTN requirements.

Proposal 6: RAN4 should consider only (Phase 0) calibration for MEO and not perform any coexistence analysis (Phase 1 and Phase 2) if calibration results show that MEO constellation requirement are within those of LEO and GEO.

# 7. Proposed Changes for TR 38.821

*** Start of change***

Table 4.2-1: Reference scenarios

|  | Transparent satellite | Regenerative satellite |
| --- | --- | --- |
| GEO based non-terrestrial access network | Scenario A | Scenario B |
| LEO based non-terrestrial access network: steerable beams | Scenario C1 | Scenario D1 |
| LEO based non-terrestrial access network: the beams move with the satellite | Scenario C2 | Scenario D2 |
| MEO based non-terrestrial access network: the beams move with the satellite | Scenario E1 | Scenario E2 |

Table 4.2-2: Reference scenario parameters

| Scenarios | GEO based non-terrestrial access network (Scenario A and B) | LEO based non-terrestrial access network (Scenario C & D) | MEO based non-terrestrial access network (Scenario E) |
| --- | --- | --- | --- |
| Orbit type | notional station keeping position fixed in terms of elevation/azimuth with respect to a given earth point | circular orbiting around the earth | Circular orbiting altitude around the earth |
| Altitude | 35,786 km | 600 km 1,200 km | 10,000 km |
| Spectrum (service link) | <6 GHz (e.g. 2 GHz) >6 GHz (e.g. DL 20 GHz, UL 30 GHz) |  |  |
| Max channel bandwidth capability (service link) | 30 MHz for band < 6 GHz 1 GHz for band > 6 GHz |  |  |
| Payload | Scenario A: Transparent (including radio frequency function only) Scenario B: regenerative (including all or part of RAN functions) | Scenario C: Transparent (including radio frequency function only) Scenario D: Regenerative (including all or part of RAN functions) | Scenario E MEO based non-terrestrial access network: the beams move with the satellite Scenario E1: Transparent, Scenario E2: Regenerative |
| Inter-Satellite link | No | Scenario C: No Scenario D: Yes/No (Both cases are possible.) | No |
| Earth-fixed beams | Yes | Scenario C1: Yes (steerable beams), see note 1 Scenario C2: No (the beams move with the satellite) Scenario D 1: Yes (steerable beams), see note 1 Scenario D 2: No (the beams move with the satellite) | The beams move with satellite |
| Max beam foot print size (edge to edge) regardless of the elevation angle | 3500 km (Note 5) | 1000 km | 1500 km |
| Min Elevation angle for both sat-gateway and user equipment | 10° for service link and 10° for feeder link | 10° for service link and 10° for feeder link | 10° for service link and 10° for feeder link |
| Max distance between satellite and user equipment at min elevation angle | 40,581 km | 1,932 km (600 km altitude) 3,131 km (1,200 km altitude) | 14,018 km |
| Max Round Trip Delay (propagation delay only) | Scenario A: 541.46 ms (service and feeder links) Scenario B: 270.73 ms (service link only) | Scenario C: (transparent payload: service and feeder links) 25.77 ms (600km) 41.77 ms (1200km) Scenario D: (regenerative payload: service link only) 12.89 ms (600km) 20.89 ms (1200km) | 95.19 ms (transparent) 47.60 ms (regenerative) |
| Max differential delay within a cell (Note 6) | 10.3 ms | 3.12 ms and 3.18 ms for respectively 600km and 1200km | TBD. |
| Max Doppler shift (earth fixed user equipment) | 0.93 ppm | 24 ppm (600km) 21ppm(1200km) | 7.5 ppm |
| Max Doppler shift variation (earth fixed user equipment) | 0.000 045 ppm/s | 0.27ppm/s (600km) 0.13ppm/s(1200km) | 0.003 ppm/s |
| User equipment motion on the earth | 1200 km/h (e.g. aircraft) | 500 km/h (e.g. high speed train) Possibly 1200 km/h (e.g. aircraft) | 500 km/h (e.g. high-speed train) Possibly 1200 km/h (e.g. aircraft) |
| User equipment antenna types | Omnidirectional antenna (linear polarisation), assuming 0 dBi Directive antenna (up to 60 cm equivalent aperture diameter in circular polarisation) |  |  |
| User equipment Tx power | Omnidirectional antenna: UE power class 3 with up to 200 mW Directive antenna: up to 20 W |  |  |
| User equipment Noise figure | Omnidirectional antenna: 7 dB Directive antenna: 1.2 dB |  |  |
| Service link | 3GPP defined New Radio |  |  |
| Feeder link | 3GPP or non-3GPP defined Radio interface | 3GPP or non-3GPP defined Radio interface | 3GPP or non-3GPP defined Radio interface |
| NOTE 1: Each satellite has the capability to steer beams towards fixed points on earth using beamforming techniques. This is applicable for a period of time corresponding to the visibility time of the satellite NOTE 2: Max delay variation within a beam (earth fixed user equipment) is calculated based on Min Elevation angle for both gateway and user equipment NOTE 3: Max differential delay within a beam is calculated based on Max beam footprint diameter at nadir NOTE 4: Speed of light used for delay calculation is 299792458 m/s. NOTE 5: The Maximum beam footprint size for GEO is based on current state of the art GEO High Throughput systems, assuming either spot beams at the edge of coverage (low elevation). NOTE 6: The maximum differential delay at cell level has been computed considering the one at beam level for largest beam size. It does not preclude that cell may include more than one beam when beam size is small or medium size. However the cumulated differential delay of all beams within a cell will not exceed the maximum differential delay at cell level in the table above. |  |  |  |

Table 6.1.1.1-1: Set-1 satellite parameters for system level simulator calibration

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 | MEO |
| --- | --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km | 600 km | 10000 km |
| Satellite antenna pattern |  | Section 6.4.1 in [2] | Section 6.4.1 in [2] | Section 6.4.1 in [2] | Section 6.4.1 in [2] |
| Payload characteristics for DL transmissions |  |  |  |  |  |
| Equivalent satellite antenna aperture (Note 1) | S-band (i.e. 2 GHz) | 22 m | 2 m | 2 m | 5 m |
| Satellite EIRP density |  | 59 dBW/MHz | 40 dBW/MHz | 34 dBW/MHz | 51 dBW/MHz |
| Satellite Tx max Gain |  | 51 dBi | 30 dBi | 30 dBi | 38.6 dBi |
| 3dB beamwidth |  | 0.4011 deg | 4.4127 deg | 4.4127 deg | 2.1 degrees |
| Satellite beam diameter (Note 2) |  | 250 km | 90 km | 50 km | 150 km |
| Equivalent satellite antenna aperture (Note 1) | Ka-band (i.e. 20 GHz for DL) | 5 m | 0.5 m | 0.5 m | 2 m |
| Satellite EIRP density |  | 40 dBW/MHz | 10 dBW/MHz | 4 dBW/MHz | 32 dBW/MHz |
| Satellite Tx max Gain |  | 58.5 dBi | 38.5 dBi | 38.5 dBi | 50 dBi |
| 3dB beamwidth |  | 0.1765 deg | 1.7647 deg | 1.7647 deg | 0.53 degrees |
| Satellite beam diameter (Note 2) |  | 110 km | 40 km | 20 km | 77 km |
| Payload characteristics for UL transmissions |  |  |  |  |  |
| Equivalent satellite antenna aperture (Note1) | S-band (i.e. 2 GHz) | 22 m | 2 m | 2 m | 5 m |
| G/T |  | 19 dB K-1 | 1.1 dB K-1 | 1.1 dB K-1 | 13 dB/K |
| Satellite Rx max Gain |  | 51 dBi | 30 dBi | 30 dBi | 38.6 dBi |
| Equivalent satellite antenna aperture (Note1) | Ka-band (i.e. 30 GHz for UL) | 3.33 m | 0.33 m | 0.33 m | 1.5 m |
| G/T |  | 28 dB K-1 | 13 dB K-1 | 13 dB K-1 | 22.5 dB/K |
| Satellite RX max Gain |  | 58.5 dBi | 38.5 dBi | 38.5 dBi | 48.14 dBi |
| NOTE 1: This value is equivalent to the antenna diameter in Sec. 6.4.1 of [2]. NOTE 2: This beam size refers to the Nadir pointing of the satellite NOTE 3: All these satellite parameters are applied per beam. NOTE 4: The EIRP density values are considered identical for all frequency re-use factor options. NOTE 5: The EIRP density values are provided assuming the satellite HPA is operated with a back-off of [5] dB. |  |  |  |  |  |

Table 6.1.1.1-2: Set-2 satellite parameters for system level simulator calibration

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 | MEO |
| --- | --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km | 600 km | 10000 km |
| Satellite antenna pattern |  | Section 6.4.1 in [2] | Section 6.4.1 in [2] | Section 6.4.1 in [2] | Section 6.4.1 in [2] |
| Payload characteristics for DL transmissions |  |  |  |  |  |
| Equivalent satellite antenna aperture (Note 1) | S-band (i.e. 2 GHz) | 12 m | 1 m | 1 m | 1.5 m |
| Satellite EIRP density |  | 53.5 dBW/MHz | 34 dBW/MHz | 28 dBW/MHz | 45.4 dBW/MHz |
| Satellite Tx max Gain |  | 45.5 dBi | 24 dBi | 24 dBi | 28.1 dBi |
| 3dB beamwidth |  | 0.7353 deg | 8.8320 deg | 8.8320 deg | 6.5 degrees |
| Satellite beam diameter (Note 2) |  | 450 km | 190 km | 90 km |  |
| Equivalent satellite antenna aperture (Note 1) | Ka-band (i.e. 20 GHz for DL) | 2 m | 0.2 m | 0.2 m | 0.5 m |
| Satellite EIRP density |  | 32 dBW/MHz | 2 dBW/MHz | -4 dBW/MHz | 20 dB/MHz |
| Satellite Tx max Gain |  | 50.5 dBi | 30.5 dBi | 30.5 dBi | 38.6 dBi |
| 3dB beamwidth |  | 0.4412 deg | 4.4127 deg | 4.4127 deg | 2.01 degrees |
| Satellite beam diameter (Note 2) |  | 280 km | 90 km | 50 km | 150 km |
| Payload characteristics for UL transmissions |  |  |  |  |  |
| Equivalent satellite antenna aperture (Note1) | S-band (i.e. 2 GHz) | 12 m | 1 m | 1 m | 1.5 m |
| G/T |  | 14 dB K-1 | -4.9 dB K-1 | -4.9 dB K-1 | 3.8 dB/K |
| Satellite Rx max Gain |  | 45.5 dBi | 24 dBi | 24 dBi | 28.1 dBi |
| Equivalent satellite antenna aperture (Note1) | Ka-band (i.e. 30 GHz for UL) | 1.33 m | 0.13 m | 0.13 m | 0.33 m |
| G/T |  | 20 dB K-1 | 5 dB K-1 | 5 dB K-1 | 12.9 dB/K |
| Satellite Rx max Gain |  | 50.5 dBi | 30.5 dBi | 30.5 dBi | 38.51 dBi |
| NOTE 1: This value is equivalent to the antenna diameter in Sec. 6.4.1 of [2]. NOTE 2: This beam size refers to the Nadir pointing of the satellite NOTE 3: All these satellite parameters are applied per beam. NOTE 4: The EIRP density values are considered identical for all frequency re-use factor options. |  |  |  |  |  |

*** End of change***

# 8. References

[1] R2-1914721 - Inputs to TR 38.821 on NGSO constellation greater than 600 km

[2] TR 38.811 - Study on New Radio (NR) to support non-terrestrial networks (Release 15)

[3] R1-1913351 - Link Budget Results for NTN