3GPP TSG RAN WG1 #108-e			R1-2202512

e-Meeting, February 21st – March 3rd, 2022

Agenda item:	7.2.11

Source: 	Moderator (NTT DOCOMO, INC.)

Title: 	Summary on [108-e-R16-UE-features-Others-01]

Document for:	Discussion and Decision

- Introduction

This contribution summarizes the following email discussion in AI 7.2.11 regarding Rel-16 UE features for UL MIMO coherence.

[108-e-R16-UE-features-Others-01] Email discussion/approval on UE feature for UL MIMO coherence – Hiroki (DOCOMO)

- Introduce a per-FS UE capability reporting for the UL MIMO coherence as proposed in R1-2202180 for Rel-17

- First check point: February 24

- Final check point: March 1

- Discussion on UE features for others

  - UL MIMO coherence capability (a new FG) [4]

The proposal to introduce a new FG for UL MIMO coherence has been extensively discussed, but following conclusion was made at the RAN1#107-e meeting.

Conclusion RAN1 did not reach a consensus to introduce a new per-FS capability indicating UL MIMO coherence at least in Rel-16.

Nevertheless, following proposal is made in a contribution in AI7.2.11.

| [4] | The UL coherence depends on the UE transmitter architecture. More precisely, it depends on whether a common oscillator signal is applied for up-conversion in the different Tx chains or not. The same UL MIMO coherence capability in a given band likely does not hold across different band combinations even for the same band. For example, the UE may use a different set of antennas for the UL transmission for a given band in one band combination compared to the antennas used for the same band in a different band combination. A common example for this is that the UE may support 4Tx UL MIMO in one band when no UL CA is configured but the same UE may fall back to 2Tx UL MIMO only in the exact same band when UL CA is configured. Note that this was the very reason why the supported number of UL MIMO layers is signaled per band per combination. Obviously, a UE that can support coherent 2Tx UL MIMO may fall into any of the three coherence categories for 4Tx UL MIMO (i.e. it can be either coherent, partial coherent, or non-coherent for 4Tx UL MIMO). The latter is the case, for example, if the 1st and 2nd chains are associated with a common oscillator, while the 3rd and 4th chains are not. Due to this consideration, the UL MIMO coherence capability needs to be signaled per band per band combination. Unfortunately, the current specification doesn’t correctly capture this. The LS from RAN4 is trying to solve the coherence issue when the UE reports and is configured with UL Tx switching. Although we agree that the capability for coherence with UL Tx switching should be per-band and per-band combination, we believe there is a more fundamental issue, as explained above, regardless of the configuration of UL Tx switching. Hence, we think that RAN1 should agree to introduce UL coherence as a per band per band combination capability before making any further decision on UL Tx switching. Proposal: A new UE capability indicating UL MIMO coherence per band per band combination is introduced (regardless of UL Tx switching) in Rel-16. During the email discussion in RAN1 meeting, some infra vendors mentioned about the impact on gNB complexity. We understand the concern that the gNB may have to do a comparison of different band combination options and choose a CA configuration according to some metric. We understand the concern, although the gNB doesn't actually have to do this, i.e. it could just select a band combination first and apply whatever capability is reported for that band combination. However, at the same time, forcing the UE to under-report is making the gNB to always end up with the worst possible choice. We can’t see any benefit in that. Changing the capability structure is not going to change the UE design or the underlying UE capabilities. There seems to be a bit of a naivete in thinking that non-coherent UE design can be forced to be coherent by restricting the reporting granularity. Obviously, in the vast majority of cases, this will work in the opposite direction. The reduced granularity will force the UE to under-report. To promote UL MIMO coherence to take off, we would suggest companies to be positive to the proposal. |
| --- | --- |

During the preparation phase email discussion, following feedbacks were provided.

| Company | Comment |
| --- | --- |
| Moderator (NTT DOCOMO) | FL would like to check whether companies can accept discussing this proposal again for Rel-16. The discussion for Rel-17 is anyway possible, but FL would like to also check whether it should be in AI8.16.17 or in AI7.2.11. |
| Apple | We support this proposal |
| ZTE | We prefer to keep the discussion in Rel-17. Thus, AI8.16.17 would be more reasonable from our side. |
| OPPO | We support to dicuss this issue for R16 and R17. |
| Samsung | As a conclusion in RAN1#107-e, we don’t need to discuss this issue at least in Rel-16. |
| Nokia, NSB | OK to have further discussion. |
| NTT DOCOMO | We are ok to discuss this issue for Rel-16 again, while also ok to limit the target release to Rel-17 or later. Whether AI8.16.17 or AI7.2.11 seems to be dependent on whether we re-open this discussion for Rel-16 or not. |
| Intel | We prefer to discuss this proposal for Rel-17. |
| Qualcomm | We prefer to discuss this in Rel-16 to solve this issue as early as possible. |
| Ericsson | Prefer to discuss in Rel-17: the issue seems to be driven by 4 Tx, and we are now quite late in the Rel-16 UE capability discussion. |
| Moderator (NTT DOCOMO) | Thanks for the feedbacks! Companies’ views can be summarized as below. - Support discussing the proposal for Rel-16 - Apple, OPPO, Nokia/NSB, NTT DOCOMO, Qualcomm - Not support discussing the proposal for Rel-16, but ok for Rel-17 - ZTE, Samsung, (NTT DOCOMO), Intel, Ericsson There are number of companies ok to discuss the proposal again for Rel-16 while multiple other companies don’t prefer to discuss it again for Rel-16 given the conclusion made at RAN1#107-e. Considering the above situation, FL would like to check again if there is a strong objection to discuss the proposal for Rel-16 and/or Rel-17 in AI7.2.11. FL proposal #3 of email discussion/approval: [108-e-AI7.2.11-NR-UEFeature-Others-01] Email discussion/approval on UE feature for UL MIMO coherence - Introduce a per-FS UE capability reporting for the UL MIMO coherence as proposed in R1-2202180 - [for Rel-16 or for Rel-17] |
| Huawei, HiSilicon | We think we should stick to Rel-17 in order to not revert the previous conclusion Conclusion RAN1 did not reach a consensus to introduce a new per-FS capability indicating UL MIMO coherence at least in Rel-16. For discussion for R17, we want to understand whether the proposal is only for 4Tx UE. |
| MTK | We are fine with the “Updated FL proposal #3 of email discussion/approval”. |
| vivo | We are fine with the lastest FL proposal #3 |
| Qualcomm | We support FL proposal #3. In response to the comments of 4Tx, this should be a generic capability without limitation to the Tx numbers. UE may support 2Tx or 4 Tx in the same band in different band combinations which is a well-accepted principle. But there is no reason to believe that the UE's coherence capability would be the same for 4 Tx as it is for 2 Tx. A UE that can support coherent 2Tx UL MIMO may fall into any of the three coherence categories for 4Tx UL MIMO (i.e. it can be either coherent, partial coherent, or non-coherent for 4Tx UL MIMO). If we don’t have this per band per band combination capability, the UE would have to report the lowest coherence capability among all the band combinations and likely miss the coherence benefit if it only supports coherence within limited band combinations. Given this consideration, we think it’s necessary to introduce this new UE capability. On the release consideration, we think the best option is Rel-16 as we can solve this issue for both with and without UL Tx switching together. However, if the majority wants to introduce this in Rel-17, we can live with this. |
| Ericsson2 | We agree with Huawei that the conclusion from RAN1#107 should not be reverted, and so are OK to discuss this only for Rel-17. |
| Moderator (NTT DOCOMO) | Thanks for further feedbacks! Since there are concerns from multiple companies to revert the conclusion made at RAN1#107-e, the discussion on this proposal should target for Rel-17. Regarding 4Tx, as discussed in RAN1#107-e and explained by the proponent, we should discuss possibility of a generic capability without limitation on Tx number first. Any potential limitation can be considered according to the discussion if necessary. As this proposal targets Rel-17, although it may be reasonable to treat this topic in AI8.16.17, it would also be fine for companies to discuss this topic in AI7.2.11 same as RAN1#107-e. Therefore, the final updated FL proposal #3 is below. Updated FL proposal #3 of email discussion/approval: [108-e-AI7.2.11-NR-UEFeature-Others-01] Email discussion/approval on UE feature for UL MIMO coherence - Introduce a per-FS UE capability reporting for the UL MIMO coherence as proposed in R1-2202180 for Rel-17 |

Based on the above, FL suggests discussing whether/how to introduce a per-FS UE capability reporting for the UL MIMO coherence for Rel-17.

FL proposal #1

- Introduce a per-FS UE capability reporting for the UL MIMO coherence as proposed in R1-2202180 for Rel-17

| 22. NR Others | [22-x] | [PUSCH codebook coherency subset for UL MIMO] | [Supported codebook coherency subset type for UL MIMO] - Candidate value set: {non-coherent, partial/non-coherent, full/partial/non-coherent} |  | Yes | N/A |  | Per FS | N/A | N/A | N/A |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Companies are encouraged to check above FL proposal and to provide feedback if any in below. Companies are also encouraged to check [5] to avoid repeating same discussion.

| Company | Comment |
| --- | --- |
| Apple | We are supportive. Regarding the candidate value, we are fine with current proposal, for we can align with the existing 38.331 pusch-TransCoherence ENUMERATED {nonCoherent, partialCoherent, fullCoherent} |
| Qualcomm | We support FL’s proposal. |
| vivo | We support the FL proposal |
| OPPO | We support the FL proposal |
| Samsung | We have concern on introducing this new capability. We cannot see the huge benefit with introduction of new capability when we consider the increase of gNB complexity. |
| Huawei, HiSilicon | The proponent does not seem to provide a clear answer wrt previous question from us (and also mentioned by Ericsson) about 4Tx. In the RAN4 LS [R4-2120039], for UL Tx switching, for cases other than 4Tx there are sufficient tools for UE to report finer granularity of capability, i.e., by per FS as agreed in RAN4. Therefore, first, it is not clear to us why the proposal or the description in the proponent contribution would like to propose this new capability regardless Tx switching, which means Tx switching is included as well. How to handle the overlapping cases by different capabilities is unclear. Second, whether the new FG is applicable to 4Tx only or could be for 2Tx as well. It is obvious that the latter is the main commercial cases today and 2Tx is not expected to be further downgraded to 1Tx or with partial/non-coherence across different bands. If the proposal is only for 4Tx, the use of it would then also be limited since almost no devices are equipped with 4Tx yet today. |
| Ericsson | As Huawei points out, 2 Tx and 4 Tx are different cases, and so I’m not sure this makes sense to discuss as a generic capability. The argument put forward for per FS so far is for partially coherent 4 Tx in our understanding, e.g. where the UE may have different LOs for the Tx chains as discussed above. So we should first agree if the feature makes sense only for 4 Tx UEs or is also beneficial for UEs that support at most 2 Tx in any band. Then regarding the gain in the 4 Tx case, the potential gain seems to be where UEs do support coherent transmission for 2 Tx but only for certain band combinations. The benefit from coherence for 2 Tx tends to not be enormous, and if it is only for very few band combinations, it is hard to expect that there will be much gain in system capacity or coverage. We can certainly understand the desire from a UE product perspective, and would like to be supportive. However, managing/configuring these highly grained UE capabilities is quite complicated for the network as we have discussed before, and so should be thoroughly justified. |
| DOCOMO | We support the proposal. We see no issue on gNB complexity since whether to consider this FG will be up to gNB. |
| Moderator (NTT DOCOMO) | Thank you very much for the feedbacks! Based on the feedbacks, companies’ views can be summarized as below. - Support FL proposal #1 (introduce a new per-FS capability for UL MIMO coherence): Apple, Qualcomm, vivo, OPPO, DOCOMO, (Ericsson: for 4 Tx?) - Not support FL proposal #1: Samsung, Huawei/HiSi, (Ericsson) - Benefit of introducing the new capability is still unclear and there is a concern on the increase of gNB complexity? - The use-case of the new capability is limited to 4Tx capable UE only (as proponent provided example case with 4 Tx capable UE only)? - How to handle the overlapping cases with UL Tx switching (for a band in band combination where UE supports UL Tx switching, there are sufficient tools for UE to report the capability on UL MIMO coherence? How about a band in band combination where UE does not support UL Tx switching)? Therefore, although there are larger number of companies supporting FL proposal #1, further discussion/clarification are necessary based on above feedbacks. FL would like to ask proponents to provide answers/clarifications to solve concerns from other companies. FL would also like to ask companies to check the answers/clarifications provided from proponents and to reply whether you can live with FL proposal #1 (with proper coordination with RAN2 and potential update/clarification). |
| Apple | - MIMO layer itself is per FSPC UE feature, e.g., maxNumberMIMO-LayersCB-PUSCH, that means in the same band in different BC, UE can report UE support different maximum number of PUSCH MIMO layers. Furthermore, UE can even indicate that in the same band per BC, in different UL CC, UE can support different maximum number of PUSCH MIMO layers - pusch-TransCoherence being a per band FG was an overlook from our view or a simple mistake, it should be per FSPC to match the MIMO layer UE reporting. Per FS is one of the very reasonable compromise to correct this issue - We still do not understand the NW complexity issue. Conherency capability report assumes fallback, in other words, when UE reports UE supports an advanded coherency capability, UE has to support all the less advanced coherency capability - Withtout per FS new FG, UE has to under-report its coherency for some UEs in some situation. - However, with per FS new FG, NW can always downgrade UE assuming UE underreport if NW choose to - Rel-15 NW cannot even understand this FG and NW does not even need to reveal its release |
| Qualcomm | Thanks for the FL’s summary. We don’t quite understand why companies still said “unclear” as we already repeated the answers for couples of times since last meeting. 1. The benefit and the gNB complexity The same UL MIMO coherence capability in a given band likely does not hold across different band combinations even for the same band. For example, the UE may use a different set of antennas for the UL transmission for a given band in one band combination compared to the antennas used for the same band in a different band combination. Please note that this was the very reason why the supported number of UL MIMO layers is signaled per band per combination. With this capability, UE could report the coherence capability for one band under certain band combination, even though it may not be capable coherence for all possible band commbinations. In other words, if we don’t have this per band per band combination capability, the UE would have to report the lowest coherence capability among all the band combinations and likely miss the coherence benefit if it only supports coherence within limited band combinations. For gNB complexity, we understand the concern that the gNB may have to do a comparison of different band combination options and choose a CA configuration according to some metric. We understand the concern, although the gNB doesn't actually have to do this, i.e. it could just select a band combination first and apply whatever capability is reported for that band combination. However, at the same time, forcing the UE to under-report is making the gNB to always end up with the worst possible choice. We can’t see any benefit in that. 2. Limited to 4Tx No. this is a common issue, and the proposal is generic for 2Tx, 4Tx and other possible UL antenna ports. In our paper, we provide an examplary UE which has 4Tx chain for Band A. Among the 4Tx chains, the 1st and 2nd chains are associated with a common oscillator, while the 3rd and 4th chains are not. For band combination A+B, UE uses 1st and 2nd chains and for band combination A+C, UE uses 3rd and 4th chains. Following current per band capability, UE would have to report non-coherent capability but with new proposed per FS capability it can achieve coherent gain for Band A+B. 3. Relationship with UL Tx switching The motivation to introduce this generic capability is similar as specific capability for UL Tx switching, where the coherence capability might be changed under different band combination. The proposed generic UL coherence is targeted to solve the fundamental issues and could be used to serve UL Tx switching and any other UL MIMO cases. Given majority wants to introduce this in Rel-17, UE might report the generic capability (if approved) and Rel-16 UL Tx switching related coherence (if UE supported). We expected UE would not report conflict capability at least for same band combination. |
| DOCOMO | We believe the inputs from Apple/Qualcomm have clarified the points well. With that, we support FL proposal #1. |
| Ericsson | @Apple: Regarding the Rel-15 choice to have pusch-TransCoherence per band, the logic as I recall it at the time was that coherence is an RF capability, whereas mimo layer processing is baseband. So it was not a simple oversight in my understanding. To further explain our view, we have strong guidance from RAN2 to not add FSPC features unless needed (in their LS R1-2001513). We would like at a better understanding of the amount of gain expected and how often the scenarios occur for those gains. Considering Qualcomm’s example above, with Rel-15 per band signaling, the first and second Tx chains could be mapped to a band X and indicated as coherent and the third and fourth could be mapped to another band Y and indicated as noncoherent. This already seems pretty flexible, and so how often there can be scenarios with real gains, especially since gains of coherence for 2 Tx are often not enormous, should be made more clear. |
| Huawei, HiSilicon | Thanks for the feedback from Apple and QC. Now finally there are clear calrifiactions about whether it is for 4Tx only and whether it is for non-Tx switching case only :) @Apple We also consider the number of MIMO layers (for data processing in data rate, i.e. baseband) is different from coherence capability (related to RF chain). We’d like also to second the consideration of gNB complexity as it is not simply like UE who can simply choose not to support when optional. Considering different UE capabilities and the possibility of supporting both Tx-switching and non-Tx S, if we specify different capability sets for Tx-S (i.e., per BC in R16 and per FS in R17), and also specify different sets for non-Tx S, from network point of view as we cannot predict which one would be early commercialized considering legacy capabilities, we have to implement all cases. This indeed increase the implementation burden. @Qualcomm For 2Tx with Tx switching case, RAN4 has agreed on per FS capability for R17 as copied below, and explained how it works with R16 capability which provides us clear picture for the functions. Now what we would like to understand is there any issue with it for Tx-S case. We can certainly further discuss non-Tx-S case and but we failed to see the need to repeat the case of Tx-S. Whether it can generated using an unified signalling or else, can be up to RAN2. - For 2CC and 3CC 2Tx-2Tx (within 2 bands) switching - Introduce new per band per BC UL-MIMO coherence capability, and the same capability applies to both 2CC and 3CC (within 2 bands) 2Tx-2Tx switching. - If the per band per BC UL-MIMO coherence capability is absent, the per BC UL-MIMO coherence capability for 1Tx-2Tx switching is applicable to 2Tx-2Tx switching. - If both the per band per BC UL-MIMO coherence capability and per BC UL-MIMO coherence are absent, the existing Rel-15 per band UE capability pusch-TransCoherence is applicable to each of the bands for 2Tx-2Tx switching. Second, for non-Tx S case, although the proponent clarified that it is for also 2Tx, the example given above is actually driven by a 4Tx UE (with 4 chains) where for some BC the UE use 2Tx with further limited capability. For a UE only capable of up to 2Tx and 2 Chains, the two RF chains will be either common or separate and the R15 per band UE capability pusch-TransCoherence is applicable to each of the bands of A, B or C. We don't feel it can be even more different. |
| Qualcomm | We'd like to thank all the companies commenting and giving feedback. Given that we have tried to make the proposed changes happen with the previous compromise, but even with that, companies have problems with any possible option, we don't see a good way forward. As we have explained before, we see the issues as being common for 2Tx and 4 Tx and for Tx switching and non-Tx switching. However, Ericsson is commenting that they see the gain of 2Tx coherence as too small, Huawei is commenting that 4Tx is not used, so no standardization support is needed, so obviously the overall interest in UL coherence is limited. With that, the motivation on the UE side to add the changes enabling implementing this feature should be correspondingly limited. Although we do not quite understand where the increase in network complexity due to the proposal comes from, if there is no UE supporting UL MIMO coherence, both the network and UE implementation can remain as it is today, which is indeed simpler. |
| Moderator (NTT DOCOMO) | Thank you very much for further discussion! Based on the discussion so far, it seems there is no consensus to introduce a new per-FS capability for UE only capable of up to 2Tx as there is already RAN4 agreed per FS capability for 2Tx with Tx switching case and per-band capability would be sufficient for non-UL Tx switching case. So, only the possible proposal for further discussion would be for UE capable of up to 4Tx and non-UL Tx switching case, although Huawei already commented that “If the proposal is only for 4Tx, the use of it would then also be limited since almost no devices are equipped with 4Tx yet today”. I would like to check whether there is any company who want to continue discussion on the proposal for UE capable of up to 4Tx and non-UL Tx switching case. It seems the proponent of the original proposal (Qualcomm) would be ok to stop the discussion. If there is no company asking to continue the discussion, I will ask RAN1 chair to close this email discussion. |
| Moderator (NTT DOCOMO) | It seems there is no company who want to continue discussion on the proposal for UE capable of up to 4Tx and non-UL Tx switching case. FL suggests closing this email discussion without any agreement/conclusion. |

- Conclusion

Based on the email discussion, RAN1 chair declared that this email discussion is closed without any agreement/conclusion.

# Reference

[1]	R1-2201462	Correction on UE feature list for NR positioning and eMIMO	NTT DOCOMO, INC.

[2]	R1-2201757	Rel-16 UE feature remaining issue	Apple

[3]	R1-2202084	On UE capability of maximum number of layers for multi-TRP	MediaTek Inc.

[4]	R1-2202180	Rel-16 UL MIMO coherence capability	Qualcomm Incorporated

[5]	R1-2112539	Summary on [107-e-AI7.2.11-NR-UEFeature-Others-01]	Moderator (NTT DOCOMO, INC.)