3GPP TSG RAN WG1 #108e                                                        R1-2202520

e-Meeting, February 21st – March 3rd, 2022

Source: 	Moderator (Intel Corporation)

Title: 	Feature Lead Summary#1 for E-mail Discussion [108-e-NR-ePos-02]

Agenda item:	8.5.2

Document for: 	Discussion and Decision

# Introduction

This document provides overview of contributions submitted for AI 8.5.2 UL-AOA enhancements [1]-[13] as a part of R17 NR Positioning maintenance work. In addition, document provides feature lead inputs and summary for the following RAN1 e-mail discussion:

[108-e-R17-ePos-02] Email discussion for maintenance on accuracy improvements for UL-AoA positioning solutions – Alexey (Intel)

  - 1st check point: February 25

  - Final check point: March 3

# Proposed Priority Order of Discussion

## Round #1

It is proposed to prioritize discussion on the first five aspects in the first round of discussion:

Aspect #1: Reference Point for UL SRS-RSRPP Measurements

Aspect #2: Definition of UL SRS-RSRPP – Linear Average of REs

Aspect #3: UL SRS-RSRPP Normalization and RX Diversity

Aspect #4: UL-AOA Measurement Definition

Aspect #5: Measurement Report Enhancements for Hybrid Positioning

# Overview of Contributions

## Aspect #1: Reference Point for UL SRS-RSRPP Measurements

RAN4 has provided response LS to RAN1 (R1-2200901) with information on reference point of SRS-RSRP and SRS-RSRPP measurements according to different gNB types.

Working assumption: The reference point for UL SRS-RSRPP shall be: - for type 1-C base station TS 38.104 [1]: the Rx antenna connector, - for type 1-O or 2-O base station TS 38.104 [1]: based on the combined signal from antenna elements corresponding to a given receiver branch, - for type 1-H base station TS 38.104 [1]: the Rx Transceiver Array Boundary connector.

The following views were expressed by companies in RAN1:

[Huawei, [1]]:

  - Revert the working assumption and update the reference as below.

Agreement Apply the following changes to the definition for UL SRS-RSRPP in the previous agreement: Definition UL SRS reference signal received path power (UL SRS-RSRPP) is defined as the power of the received UL SRS signal configured for the measurement at the i-th path delay of the channel response, where UL SRS-RSRPP for 1st path delay is the power corresponding to the first detected path. - The reference point for UL SRS-RSRPP shall be: - for type 1-C base station TS 38.104: the Rx antenna connector, - for type 1-O or 2-O base station TS 38.104: based on the combined signal from antenna elements corresponding to a given receiver branch, - for type 1-H base station TS 38.104: the Rx Transceiver Array Boundary connector. - FFS: For frequency range 1 and 2, if receiver diversity is in use by the gNB, the reported UL SRS-RSRPP value shall not be lower than the corresponding UL SRS-RSRPP of any of the individual receiver branches - FFS: Note: First and additional paths RSRP, when provided in the same report, use the same RX branch(es) selected for the first arrival path and for the UL SRS-RSRP if the UL SRS-RSRP is reported - FFS: whether/how to capture the note in the specifications Note: The following two options are supported by gNB to LMF: - Option 1 (RX diversity for the first path UL SRS-RSRPP) - The same RX branch(es) as applied for the first path UL SRS-RSRPP measurements are used for the additional paths UL SRS-RSRPP measurements if those are provided together - For frequency range 1 and 2, if receiver diversity is in use by the gNB for UL SRS-RSRPP measurements, then reported UL SRS-RSRPP value for the first path shall not be lower than the corresponding UL SRS-RSRPP for the first path of any of the individual receiver branches - Option 2 (RX diversity for UL SRS-RSRP) - The same RX branch(es) as applied for UL SRS-RSRP measurements are used for UL SRS-RSRPP measurements (i.e., the first and additional paths UL SRS-RSRPP if those are provided)

[Huawei, [1]]:

  - Agree to the following TP to section 5.2.5 and 5.2.6 of TS 38.215.

### 5.2.5 UL SRS reference signal received power (UL SRS-RSRP) Definition; UL SRS reference signal received power (UL SRS-RSRP) is defined as linear average of the power contributions (in [W]) of the resource elements carrying sounding reference signals (SRS). UL SRS-RSRP shall be measured over the configured resource elements within the considered measurement frequency bandwidth in the configured measurement time occasions. The reference point for UL SRS-RSRP shall be: - for type 1-C base station TS 38.104 [9]: the Rx antenna connector, - for type 1-O or 2-O base station TS 38.104 [9]: based on the combined signal from antenna elements corresponding to a given receiver branch - for type 1-H base station TS 38.104 [9]: the Rx Transceiver Array Boundary connector. For frequency range 1 and 2, if receiver diversity is in use by the gNB, the reported UL SRS-RSRP value shall not be lower than the corresponding UL SRS-RSRP of any of the individual receiver branches.. ### 5.2.6 UL SRS reference signal received path power (UL SRS-RSRPP) Definition; UL SRS reference signal received path power (UL SRS-RSRPP) is defined as the power of the received UL SRS signal configured for the measurement at the i-th path delay of the channel response, where UL SRS-RSRPP for 1st path delay is the power corresponding to the first detected path. The reference point for UL SRS-RSRPP shall be: - for type 1-C base station TS 38.104 [9]: the Rx antenna connector, - for type 1-O or 2-O base station TS 38.104 [9]: based on the combined signal from antenna elements corresponding to a given receiver branch - for type 1-H base station TS 38.104 [9]: the Rx Transceiver Array Boundary connector. For frequency range 1 and 2, if receiver diversity is in use by the gNB for UL SRS-RSRPP measurements: - The reported UL SRS-RSRPP value for the first and additional paths shall be provided for the same receiver branch(es) as applied for UL SRS-RSRP measurements, or - The reported UL SRS-RSRPP value for the first path shall not be lower than the corresponding UL SRS-RSRPP for the first path of any of the individual receiver branches and the reported UL SRS-RSRPP for the additional paths shall be provided for the same receiver branch(es) as applied UL SRS-RSRPP for the first path..

[OPPO, [4]]:

  - Update the definition of UL SRS-RSRPP as follows:

| Definition | UL SRS reference signal received path power (UL SRS-RSRPP) is defined as the power of the received UL SRS signal configured for the measurement at the i-th path delay of the channel response, where UL SRS-RSRPP for 1st path delay is the power corresponding to the first detected path. For frequency range 1, the reference point for the UL SRS-RSRPP shall be the antenna connector of the gNB. For frequency range 2, UL SRS-RSRPP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch. The reference point for UL SRS-RSRPP shall be: - for type 1-C base station TS 38.104: the Rx antenna connector, - for type 1-O or 2-O base station TS 38.104: the Rx antenna (i.e. the centre location of the radiating region of the Rx antenna), - for type 1-H base station TS 38.104: the Rx Transceiver Array Boundary connector. For frequency range 1 and 2, if receiver diversity is in use by the gNB for UL SRS-RSRPP measurements: - The reported UL SRS-RSRPP value for the first and additional paths shall be provided for the same receiver branch(es) as applied for UL SRS-RSRP measurements, or - The reported UL SRS-RSRPP value for the first path shall not be lower than the corresponding UL SRS-RSRPP for the first path of any of the individual receiver branches and the reported UL SRS-RSRPP for the additional paths shall be provided for the same receiver branch(es) as applied UL SRS-RSRPP for the first path. |
| --- | --- |

[CATT, [5]]:

  - Apply the following changes to the definition of reference point for UL SRS-RSRPP in the previous agreement and also make corresponding changes in TS 38.215.

UL SRS reference signal received path power (UL SRS-RSRPP) is defined as the power of the received UL SRS signal configured for the measurement at the i-th path delay of the channel response, where UL SRS-RSRPP for 1st path delay is the power corresponding to the first detected path. The reference point for ULSRS-RSRPP shall be: - for type 1-C base station TS 38.104: the Rx antenna connector, - for type 1-O or 2-O base station TS 38.104: based on the combined signal from antenna elements corresponding to a given receiver branch, - for type 1-H base station TS 38.104: the Rx Transceiver Array Boundary connector. - Working assumption: For frequency range 1, the reference point for the UL SRS-RSRPP shall be the antenna connector of the gNB. For frequency range 2, UL SRS-RSRPP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch.

[Nokia, [6]]:

  - In TS 38.215, put a square bracket into the current definition on the reference point description.

[Ericsson, [13]]

  - Endorse TP 2.1 for inclusion in 38.215

TP 2.1:

### 5.2.6 UL SRS reference signal received path power (UL SRS-RSRPP) Definition; UL SRS reference signal received path power (UL SRS-RSRPP) is defined as the power of the received UL SRS signal configured for the measurement at the i-th path delay of the channel response, where UL SRS-RSRPP for 1st path delay is the power corresponding to the first detected path. The reference point for UL SRS-RSRPP shall be: - for type 1-C base station TS 38.104 [1]: the Rx antenna connector, - for type 1-O or 2-O base station TS 38.104 [1]: based on the combined signal from antenna elements corresponding to a given receiver branch, - for type 1-H base station TS 38.104 [1]: the Rx Transceiver Array Boundary connector. For frequency range 1 and 2, if receiver diversity is in use by the gNB for UL SRS-RSRPP measurements: - The reported UL SRS-RSRPP value for the first and additional paths shall be provided for the same receiver branch(es) as applied for UL SRS-RSRP measurements, or - The reported UL SRS-RSRPP value for the first path shall not be lower than the corresponding UL SRS-RSRPP for the first path of any of the individual receiver branches and the reported UL SRS-RSRPP for the additional paths shall be provided for the same receiver branch(es) as applied UL SRS-RSRPP for the first path..

### Round #1

FL comment:

It seems measurement definitions of SRS-RSRP and SRS-RSRPP need to be updated to include information on reference point.

Note: necessary changes for R16 SRS-RSRP measurement are in scope of the RAN1 e-mail thread [108-e-R16-Pos-01]

- Proposal 1-1

Measurement definition of UL SRS-RSRPP is revised to include information on reference point as shown below:

| Definition | UL SRS reference signal received path power (UL SRS-RSRPP) is defined as the power of the received UL SRS signal configured for the measurement at the i-th path delay of the channel response, where UL SRS-RSRPP for 1st path delay is the power corresponding to the first detected path. The reference point for UL SRS-RSRPP shall be: - for type 1-C base station TS 38.104 [9]: the Rx antenna connector, - for type 1-O or 2-O base station TS 38.104 [9]: based on the combined signal from antenna elements corresponding to a given receiver branch - for type 1-H base station TS 38.104 [9]: the Rx Transceiver Array Boundary connector. For frequency range 1 and 2, if receiver diversity is in use by the gNB for UL SRS-RSRPP measurements: - The reported UL SRS-RSRPP value for the first and additional paths shall be provided for the same receiver branch(es) as applied for UL SRS-RSRP measurements, or - The reported UL SRS-RSRPP value for the first path shall not be lower than the corresponding UL SRS-RSRPP for the first path of any of the individual receiver branches and the reported UL SRS-RSRPP for the additional paths shall be provided for the same receiver branch(es) as applied UL SRS-RSRPP for the first path. |
| --- | --- |

Comments from companies:

| Company Name | Comments |
| --- | --- |
| Huawei, HiSilicon | Support. |
| vivo | Support. |
| Intel | Support |
| CATT | Support |
| Nokia/NSB | Support |
| ZTE | Support |
| InterDigital | Support |
| OPPO | Support |
| LGE | Support. |
| NTT DOCOMO | Support |
| Ericsson | Support. |
| Qualcomm | Support |

Summary

It is agreeable to endorse Proposal 1-1

## Aspect #2: Definition of UL SRS-RSRPP – Linear Average of REs

Based on the LS from RAN4 (R4-2202780) on DL-PRS RSRPP definition several companies have proposed to make similar changes in UL SRS-RSRPP definition to reflect how the per-RE power is measured. The following views were provided:

[Huawei, [1]]

  - Agree to the following TP to section 5.2.6 of TS 38.215.

### 5.2.6 UL SRS reference signal received path power (UL SRS-RSRPP) Definition; UL SRS reference signal received path power (UL SRS-RSRPP) is defined as the power of the received UL SRS signal configured for the measurement at the i-th path delay of the channel response, where UL SRS-RSRPP for 1st path delay is the power corresponding to the first detected path. For frequency range 1, the reference point for the UL SRS-RSRPP shall be the antenna connector of the gNB. For frequency range 2, UL SRS-RSRPP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch. For frequency range 1 and 2, if receiver diversity is in use by the gNB for UL SRS-RSRPP measurements: - The reported UL SRS-RSRPP value for the first and additional paths shall be provided for the same receiver branch(es) as applied for UL SRS-RSRP measurements, or - The reported UL SRS-RSRPP value for the first path shall not be lower than the corresponding UL SRS-RSRPP for the first path of any of the individual receiver branches and the reported UL SRS-RSRPP for the additional paths shall be provided for the same receiver branch(es) as applied UL SRS-RSRPP for the first path..

[CATT, [5]]:

  - Apply the following changes to the definition for UL SRS-RSRPP in TS 38.215

| Definition | UL SRS reference signal received path power (UL SRS-RSRPP) is defined as linear average of the power contributions (in [W]) of the received UL SRS signal of the resource elements configured for the measurement at the i-th path delay of the channel response, where UL SRS-RSRPP for 1st path delay is the power contribution corresponding to the first detected path. For frequency range 1, the reference point for the UL SRS-RSRPP shall be the antenna connector of the gNB. For frequency range 2, UL SRS-RSRPP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch. For frequency range 1 and 2, if receiver diversity is in use by the gNB for UL SRS-RSRPP measurements: - The reported UL SRS-RSRPP value for the first and additional paths shall be provided for the same receiver branch(es) as applied for UL SRS-RSRP measurements, or - The reported UL SRS-RSRPP value for the first path shall not be lower than the corresponding UL SRS-RSRPP for the first path of any of the individual receiver branches and the reported UL SRS-RSRPP for the additional paths shall be provided for the same receiver branch(es) as applied UL SRS-RSRPP for the first path. |
| --- | --- |

[Nokia, [6]]:

  - RAN1 to consider modification of the current SRS RSRPP definition. E.g.,

    - UL SRS reference signal received path power (UL SRS-RSRPP) is defined as the linear average over the power contributions of the resource elements that carry of the received UL SRS signal configured for the measurement at the i-th path delay of the channel response within the considered measurement frequency bandwidth, where UL SRS-RSRPP for the 1st path delay is the power corresponding to the first detected path.

### Round #1

FL comment:

It is worthwhile to align UL SRS-RSRPP and DL PRS-RSRPP definitions, if RAN1 confirms RAN4 understanding expressed in LS (R4-2202780)

- Proposal 2-1

Definition for UL SRS-RSRPP measurement is modified as follows:

  - “UL SRS reference signal received path power (UL SRS-RSRPP) is defined as the linear average over the power contributions of the resource elements that carry of the received UL SRS signal configured for the measurement at the i-th path delay of the channel response within the considered measurement frequency bandwidth, where UL SRS-RSRPP for the 1st path delay is the power corresponding to the first detected path.”

Comments from companies:

| Company Name | Comments |
| --- | --- |
| Huawei, HiSilicon | The definition based on the understanding is still quite vague how the power contribution of a specific path delay is obtained. |
| Vivo | We cannot accept the wording modification of the last sentence about removing UL SRS RSRPP. For us, the definition is for RSRPP, removing it to explain (1st path delay is the first detected path) is vague. We prefer to modify it as Definition for UL SRS-RSRPP measurement is modified as follows: - “UL SRS reference signal received path power (UL SRS-RSRPP) is defined as the linear average over the power contributions of the resource elements that carry of the received UL SRS signal configured for the measurement at the i-th path delay of the channel response within the considered measurement frequency bandwidth, where UL SRS-RSRPP for 1st path delay is the power corresponding to the first detected path. |
| CATT | We prefer the wording “as linear average of the power contributions (in [W]) of the received UL SRS signal on the resource elements” instead of “linear average over the power contributions of the resource elements…”, since the power is the power power, but not the power of resources elements. We prefer to keep the wording “where UL SRS-RSRPP for the 1st path delay is the power corresponding to the first detected path” w/o change, although the sentence is actually not needed, since the definition of the first part of the paragraph covers all paths, including the first path. |
| Nokia/NSB | We are okay if companies want to keep the last sentence as it was, and open to discussing any modification which does make sense. |
| ZTE | OK to have a clear definition which is similar to SRS-RSRP in 38.215. We prefer not to change the last sentence. |
| OPPO | The wording is not perfect, but it might be the best we can do now. It has been dicussed many times. We can be ok with the proposed wording with no change in the last sentence. |
| LGE | We are generally fine with current proposal 2-1 |
| NTT DOCOMO | Basically, we support the proposal. Regarding the last sentence, we slightly prefer to keep the original wording. |
| Ericsson | OK with Vivo’s update. |

### Round #2

Summary

Based on comments from companies received in the first round it is proposed to simplify the wording of the proposal 2-1 and apply similar changes as for SRS-RSRP reflecting the previous agreements on path specific mesaruements. In summary, UL SRS-RSRPP is defined as the path RSRP per RE.

- Proposal 2-2

Definition for UL SRS-RSRPP measurement is modified as follows:

  - UL SRS reference signal received path power (UL SRS-RSRPP) is defined as the linear average over the power contributions (in [W]) of the resource elements of the received UL SRS signal configured for the measurement at the i-th path delay of the channel response, where UL SRS-RSRPP for 1st path delay is the power corresponding to the first detected path.

Comments from companies:

| Company Name | Comments |
| --- | --- |

## Aspect #3: UL SRS-RSRPP Normalization and RX Diversity

  - [vivo, [2]]:

  - For the two options of RX diversity for the first path UL SRS-RSRPP and UL SRS-RSRP,

    - Option 1 can be adopted when UL SRS-RSRP is not reported;

    - Option 2 should be adopted when UL SRS-RSRP is reported, which makes the Rx branches and normalization can be referred to UL SRS-RSRP.

  - UL SRS-RSRP reporting is the prerequisite of UL SRS-RSRPP:

    - Normalization: UL SRS-RSRP reporting is reported as the difference in dB with respect to UL SRS-RSRP.

  - [Intel, [7]]:

    - For the UL-AOA positioning, the gNB can be requested to measure and report to the LMF the UL SRS-RSRPP of the first path using the following options:

      - Option 1: if the UL SRS-RSRPP of the first path is reported only, then the UL SRS-RSRPP of the first path is reported using the absolute values

      - Option 2: if both UL SRS-RSRP and UL SRS-RSRPP of the first path are reported, then the UL SRS-RSRP is reported using the absolute values and the UL SRS-RSRPP of the first path is reported using the differential values, where the UL SRS-RSRP is selected as a reference measurement

      - The absolute values are reported in the range [-156 dBm, -31 dBm] with 1 dB resolution

      - The differential values are reported in the range [-30 dB, 0 dB] with 1 dB resolution

[InterDigital, [8]]:

  - Confirm the working assumption for the definition of UL SRS-RSRPP with the following changes, “For frequency range 1, if applicable, the reference point for the UL SRS-RSRPP shall be the antenna connector of the gNB. If applicable, for frequency range 1, UL SRS-RSRPP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch. For frequency range 2, UL SRS-RSRPP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch.”

Association between UL SRS-RSRP and UL SRS-RSRPP

[LGE, [12]]:

  - RAN1 needs to support reporting of both UL SRS-RSRPP and UL SRS-RSRP and needs to agree that UL SRS-RSRPP (P value) and UL SRS-RSRP (Q values) are such that P is less than or Q like a as shown below:

    - For reporting of UL SRS-RSRPP and UL SRS-RSRP in UL-AoA

  - Regarding, the maximum number of DL PRS RSRPP, it is up to RAN4.

    - The capabilities for DL PRS RSRPP (M value) and DL PRS RSRP (N values) are such that M is less than or equal to N.

### Round #1

Companies are invited to express their views to continue discussion.

FL comment:

Normalization details of UL SRS-RSRPP reporting need to be finalized. The following is proposed to initiate discussion

- Proposal 3-1

  - For the UL-AOA positioning, the gNB can be requested to measure and report to the LMF the UL SRS-RSRPP of the first path using the following options:

    - Option 1: if the UL SRS-RSRPP of the first path is reported only, then the UL SRS-RSRPP of the first path is reported using the absolute values

    - Option 2: if both UL SRS-RSRP and UL SRS-RSRPP of the first path are reported, then the UL SRS-RSRP is reported using the absolute values and the UL SRS-RSRPP of the first path is reported using the relative values, where the UL SRS-RSRP is selected as a reference measurement

    - The absolute values are reported in the range [-156 dBm, -31 dBm] with 1 dB resolution

    - The relative values are reported in the range [-30 dB, 0 dB] with 1 dB resolution

Comments from companies:

| Company Name | Comments |
| --- | --- |
| Huawei, HiSilicon | We do not support the proposal According to RAN4 agreement in R4-2202776, RAN4 already agreed that no relative reporting of RSRPP with RSRP is adopted. Issue 3-1-3: PRS-RSRPP reporting together with PRS-RSRP Agreements: Should PRS-RSRPP be normalized for reporting if it is reported with PRS-RSRP? - NO Issue 3-1-4: PRS-RSRPP reporting without PRS-RSRP report Agreements: Should PRS-RSRPP be normalized for reporting if it is reported without PRS-RSRP? - NO |
| vivo | We think the Normalization of FL proposal is differential RSRPP and related to the reference of differential RSRPP RAN4 LS PRS-RSRPP may be reported by reusing absolute and differential PRS-RSRP measurement report mapping tables in TS38.133 clause 10.1.24.3.1 and 10.1.24.3.2 respectively. When differential reporting is used, PRS-RSRPP is reported as the difference in dB with respect to a reference measurement. RAN4 understands that it is up to RAN1/2 to decide what reference measurement would be for the PRS-RSRPP differential reporting.. If it is about differential RSRPP and the reference of differential RSRPP, we are okay with the proposal. |
| Intel | We think that this is about differential RSRPP reporting relative to the RSRP value We are supportive of the proposal |
| CATT | The proposal is fine to us in general. Suggest using the same wording as RAN4’s LS with absolute and differential reporting and also refer to existing RAN4 table for RSRPP reporting. - Proposal 3-1 - For the UL-AOA positioning, the gNB can be requested to measure and report to the LMF the UL SRS-RSRPP of the first path using the following options: - Option 1: if the UL SRS-RSRPP of the first path is reported only, then the UL SRS-RSRPP of the first path is reported using the absolute values reporting as defined in the mapping Table 13.3.1-1 of TS 38.133 for SRS-RSRP. - Option 2: if both UL SRS-RSRP and UL SRS-RSRPP of the first path are reported, then the UL SRS-RSRP is reported using the absolute values and the UL SRS-RSRPP of the first path is reported using the relative values, differential reporting in the range [-30 dB, 0 dB] with 1 dB resolution, where the UL SRS-RSRP is selected as a reference measurement - The absolute values are reported in the range [-156 dBm, -31 dBm] with 1 dB resolution - The relative values are reported in the range [-30 dB, 0 dB] with 1 dB resolution |
| Nokia/NSB | We are generally okay with the proposal as it seems to be aligned with RAN4 LS. |
| ZTE | According to RAN4 LS, it’s up to RAN1 to what reference measurement would be for the PRS-RSRPP differential reporting. We’re fine with CATT’s changes. |
| InterDigital | Our proposal is captured here but it was intended for Aspect #1. We respect the majority view in Aspect #1 and ok with the FL’s prposal. |
| OPPO | As in RAN4 LS, the SRS-RSRPP can be reported as differential RSRP. So we support the proposal. |
| LGE | If the intention of the proposal is about differential reporting, we prefer to reuse the way of DL-PRSPP (differential PRS-RSRP measurement report mapping tables in TS38.133 clause 10.1.24.3.1 and 10.1.24.3.2 respectively). |
| NTT DOCOMO | Regarding differential reporting, we support FL’s proposal and CATT’s proposal (slightly prefer CATT’s one). |
| Ericsson | Our understanding from RAN4’s discussion is that differential reporting between measurement of the same types (e.g. RSRPP to RSRPP) will be supported, but not normalization of one type of measurement with another one (e.g. RSRPP normalized with RSRP). Therefore we support option 1 and we may support differential reporting for additional paths/measurements. |

### Round #2

Companies are invited to express their views to continue discussion.

FL comment:

Changes proposed by CATT are implemented in revised proposal 3-2

- Proposal 3-2

  - For the UL-AOA positioning, the gNB can be requested to measure and report to the LMF the UL SRS-RSRPP of the first path using the following options:

    - Option 1: if the UL SRS-RSRPP of the first path is reported only, then the UL SRS-RSRPP of the first path is reported using the absolute reporting as defined in the mapping Table 13.3.1-1 of TS 38.133 for SRS-RSRP.

    - Option 2: if both UL SRS-RSRP and UL SRS-RSRPP of the first path are reported, then UL SRS-RSRPP of the first path is reported using differential reporting in the range [-30 dB, 0 dB] with 1 dB resolution, where the UL SRS-RSRP is selected as a reference measurement

Comments from companies:

| Company Name | Comments |
| --- | --- |
| Qualcomm | We support UL SRS-RSRPP of first path to be absolute reporting only. Differential reporting for additional paths/measurements, but not across different measurement types. |

## Aspect #4: Clarification of UL-AOA Measurement Definition

In [7], it is noticed that the current definition for the UL-AOA measurement in the TS 38.215 is not completely correct with respect to the additional path measurements. In case if the UL-AOA is measured for the additional path, then the UL-AOA does not define the azimuth and vertical angle of a UE. The UL-AOA angle corresponds to the reflected path with respect to a reference direction, which may not correspond to the direction of a UE.

  - [Intel, [7]]:

    - Update the UL-AOA measurement definition in the TS 38.215, adding the following:

      - In case if the UL Angle of Arrival (UL-AOA) is measured for the additional path, it is defined as the estimated azimuth and vertical angle of a reflected path with respect to a reference direction, which may not correspond to the direction of a UE

      - Note: Multiple UL-AOA pairs can be reported per single channel path

### Round #1

FL comment:

Considering that UL AOA measurement definition is not aligned with UL AOA measurement for additional paths, it seems RAN1 needs to introduce per path UL AOA measurement or modify R16 definition

- Proposal 4-1

  - Update the UL-AOA measurement definition in the TS 38.215, adding the following:

    - In case if the UL Angle of Arrival (UL-AOA) is measured for the additional path, it is defined as the estimated azimuth and vertical angle of a reflected path with respect to a reference direction, which may not correspond to the direction of a UE

Comments from companies:

| Company Name | Comments |
| --- | --- |
| Huawei, HiSilicon | We do not think this change is necessary. This UL AoA measurement for an additional path may somehow also be interpreted as the direction of a “virtual UE location”. |
| vivo | We also think this change is unnecessary for Rel-17 since we don’t know which path of AoA is the angle of UE. |
| Intel | Support, we believe that this clarification is required |
| CATT | The change is not needed. |
| Nokia/NSB | We do not see necessity of this update for definition |
| ZTE | It’s unnecessary. |
| OPPO | It is not needed. |
| LGE | We cannot agree with the intention of the proposal and we prefer not to support. |
| Ericsson | While we understand the point made by the proposal, we don’t think it is necessary. |

### Round #2

- FL Observations

It seems companies understand the point of the proposal but do not think it is necessary

Seems more discussion is needed to align views on proposal

Companies are invited to express their views to continue discussion if it is necessary

Comments from companies:

| Company Name | Comments |
| --- | --- |

## Aspect #5: Measurement Report Enhancements for Hybrid Positioning

[vivo, [2]]:

  - For the first arrival path measurements on SRS for positioning resource,

    - Do not support gNB to report to LMF the following set of measurements {multiple SRS-RSRP, multiple UL-AOAs (AoA/ZoA pairs), one UL-RTOA, one-gNB Rx-Tx time difference}.

  - For the first arrival path measurements on SRS for MIMO resource,

    - Do not support gNB to report to LMF the following set of measurements {multiple SRS-RSRP, multiple UL-AOAs (AoA/ZoA pairs), one UL-RTOA}.

[ZTE, [3]]:

  - There is no need for gNB/TRP to report multiple path SRS RSRPs for the first arrival path since the path SRS RSRP should be defined in time domain rather than spatial domain.

  - Don’t support gNB/TRP to report one UL-RTOA and one gNB Rx-Tx time difference simultaneously for the first arrival path

[OPPO, [4]]:

  - Support the gNB to report the following path-specific measurement on one path:

    - the gNB can report {SRS-RSRPP, multiple UL-AoAs, one UL-RTOA or one gNB Rx-Tx time difference} measured from that path.

[Nokia, [6]]:

  - For the first arrival path measurements on SRS for positioning resource or SRS for MIMO resource,

    - gNB can report to LMF the set of measurements {multiple SRS-RSRP, multiple UL-AOAs (AoA/ZoA pairs)}

[CMCC, [9]]:

  - For the first arrival path measurements on SRS for positioning resource, gNB can report to LMF the following measurement tuples {multiple SRS-RSRPPs, multiple UL-AOAs (AoA/ZoA pairs), one UL-RTOA or one gNB Rx-Tx time difference};

  - For the first arrival path measurements on SRS for MIMO resource, gNB can report to LMF the following measurement tuples {multiple SRS-RSRPPs, multiple UL-AOAs (AoA/ZoA pairs), one UL-RTOA or one-gNB Rx-Tx time difference}.

  - For the first arrival path measurements on SRS for positioning resource, do NOT support gNB to report to LMF the following set of measurements {multiple SRS-RSRP, multiple UL-AOAs (AoA/ZoA pairs), one UL-RTOA, one-gNB Rx-Tx time difference}.

[Samsung, [10]]:

  - The following Alt.1 should be supported.

    - Alt.1:

      - For the first arrival path measurements on SRS for positioning resource, gNB can report to LMF multiple sets of the following measurement tuples {one SRS-RSRPP, multiple UL-AOAs (AoA/ZoA pairs), one UL-RTOA or one gNB Rx-Tx time difference}

      - For the first arrival path measurements on SRS for MIMO resource, gNB can report to LMF multiple sets of the following measurement tuples {one SRS-RSRPP, multiple UL-AOAs (AoA/ZoA pairs), one UL-RTOA or one-gNB Rx-Tx time difference}

[LGE, [12]]:

  - Reporting multiple SRS-RSRP for each UL-AoA seems not only appropriate for LMF to estimate which UL-AoA is used for corresponding one UL-ROTA/ gNB Rx-Tx time difference but also helpful for LMF to use reported multiple UL-AoAs more properly.

  - RAN1 needs to support reporting multiple SRS-RSRP together with together with {multiple UL-AOA, UL-RTOA/gNB Rx-Tx measurements}.

[Qualcomm, [11]]:

  - Support the following:

    - For the first arrival path measurements on SRS for positioning resource, gNB can report to LMF multiple sets of the following set of measurements { multiple SRS-RSRPP, multiple UL-AOAs (AoA/ZoA pairs), one UL-RTOA or one gNB Rx-Tx time difference }

    - For the first arrival path measurements on SRS for MIMO resource, gNB can report to LMF multiple sets of the following set of measurements {multiple SRS-RSRPP, multiple UL-AOAs (AoA/ZoA pairs), one UL-RTOA or one-gNB Rx-Tx time difference}

### Round #1

FL comment:

This aspect was discussed at the previous meetings. There is no common view to support multiple SRS-RSRPP values.

Companies are invited to continue discussion aiming to converge on common proposal

- Proposal 5-1

Continue discussion aiming to converge on agreeable proposal

Comments from companies:

| Company Name | Comments |
| --- | --- |
| Huawei, HiSilicon | We do not think this is so essential, given that NRPPa solution can already support this using multi-path reporting. |
| Vivo | Not needed |
| Intel | We still believe if multiple RSRP values are reported, then multiple timing values should be reported as well We do not support multiple RSRPs and one UL-RTOA/gNB Rx-Tx |
| CATT | The issue was discussed many times w/o consensus. It is not essential and the benefit is unclear. |
| Nokia/NSB | If there is no consensus on supping multiple SRS-RSRPP, we are fine with no further enhancement on this aspect. |
| ZTE | It’s not essential. No further discussion on this issue. |
| OPPO | Supporting multiple SRS-RSRPP is benefical for multi-path reporting. We are open to dicuss that. |
| LGE | Even though we have pronounced for reporting multiple SRS-RSRPP in our contribution, we are fine to support reporting single SRS-RSRPP only with other measurements. |
| NTT DOCOMO | We think the benefit should be clarified firstly. |
| Ericsson | We can deprioritize this issue. |

### Round #2

- FL Observations

It seems there is no consensus to support proposed measurement report enhancement in Rel17

Companies are invited to express their views to continue discussion if it is necessary

Comments from companies:

| Company Name | Comments |
| --- | --- |

## Aspect #6: LS to RAN4 on Rel 15 SRS for Multi-RTT Positioning

RAN1 has received LS from RAN4 (R1-2200900) asking to clarify the potential usage of the R15 SRS for NR positioning. The following views were expressed:

  - [Intel, [7]]:

    - Send reply to LS from RAN4 with the following content:

      - Rel-15 SRS is not supported for NR Multi-RTT positioning

      - Rel-15 SRS can be supported for NR UL-TDOA and UL-AOA positioning starting from Rel.16

      - Rel-15 SRS is supported for serving cell UE/gNB Rx-Tx time difference measurements in context of NR RTT-based PDC

        - This functionality is supported starting from the Rel.17

### Round #1

FL comment:

RAN1 needs to provide reply to LS from RAN4 and clarify the use of R15 SRS for NR Multi-RTT positioning

It is up to RAN1 chair to decide whether to treat this under AI 8.5.2. The topic may be treated in another AI.

### Round #2

- FL Observations

LS was discussed under AI 8.5.1.

## Aspect #7: Reporting of Estimated Range of AoA

[OPPO, [4]]:

  - For linear antenna array, in addition to the ZoA relative to the z-axis, the TRP also reports an estimated range of AoA.

### Round #1

FL comment:

Seems to be an optimization aspect rather than essential correction.

Companies are invited to provide their views on proposal to report an estimated range of AoA.

Comments from companies:

| Company Name | Comments |
| --- | --- |
| Huawei, HiSilicon | No need for such an enhancement. |
| vivo | Not needed |
| Nokia/NSB | We think it is not an essential enhancement. |
| ZTE | Not needed. |
| LGE | Not support. |
| Ericsson | Do not support. |

### Round #2

FL comment:

Based on received feedback it seems the discussion can be closed as there is no consensus to support proposed feature

Comments from companies:

| Company Name | Comments |
| --- | --- |

## Aspect #8: Enhancements of ARP Framework

[Nokia, [6]]:

  - LMF may provide a threshold to the gNB to determine the AoA measurement report and its associated ARP information.

    - Note: the threshold value is related to near-field/far-field discrimination

    - If difference of the AoA measurements from different ARPs are less than the threshold, the gNB may report a single AoA measurement and a single ARP, where the ARP includes all reception antennas of the different ARPs.

    - If difference of the AoA measurements from different ARPs are greater than the threshold, the gNB may report those AoA measurements and ARP IDs.

  - LMF may provide the gNB with level/size of an antenna group of an ARP per TRP to help the gNB implementation for AoA estimation.

    - Note: When TRP peforms AoA measurements for the target UE, it may start with the recommended antenna group size assuming a single incident angle for each antenna group.

  - LMF requests the gNB to report angle measurement for a specific ARP ID of a TRP.

### Round #1

FL comment:

Seems to be an optimization aspect rather than essential correction

Companies are invited to provide their views on additional ARP related enhancements

Comments from companies:

| Company Name | Comments |
| --- | --- |
| Huawei, HiSilicon | Up to RAN3 to work. |
| vivo | Not needed |
| Nokia/NSB | We think at least the third bullet is a necessary enhancement. The LMF may be provided with ARP information and it may have prior knowledge on the location of the target UE, so LMF can determine which ARP measurements are critical to accurately find the location. |
| ZTE | Up to RAN3 to decide on how to specify the ARP framework. |
| LGE | It should be up to RAN3. |
| Ericsson | Not needed. |

### Round #2

FL comment:

Based on received feedback it seems the discussion in RAN1 can be closed as companies do not see the need or believe that this aspect can be discussed in RAN3 directly

Comments from companies:

| Company Name | Comments |
| --- | --- |

## Aspect #9: Expected UL-AoD/ZoD

Expected UL-AoD/ZoD

[LGE, [12]]:

  - If expected UL-AoD and uncertainty is proved for UE, UE does not need to transmit all of SRSs within configured SRS resource(s) and it would be helpful for UE to determinate the Tx beamforming.

  - RAN1 should support expected UL-AoD/ZoD which is signaled from LMF to UE and the detail configuration needs to follow the way for configuration of expected UL-AoA and its uncertainty (or details up to RAN2).

### Round #1

FL comment:

Seems to be an optimization aspect rather than essential correction

Companies are invited to provide their views on additional ARP related enhancements

Comments from companies:

| Company Name | Comments |
| --- | --- |
| Huawei, HiSilicon | We think this feature is too late for this release, considering the fact that UE Tx beamforming is rather complicated. |
| vivo | We wonder whether the issue can work in reality since the SRS should be transmitted to multiple TRPs and the expected angle for each TRP is different. So, in our view, whether the optimization can reduce the transmission of SRS is uncertain. |
| Nokia/NSB | We are okay with the enhancement related to expected UL-AoD/ZoD. |
| ZTE | It’s not essential. UE generally has small number of antennas, the expected UL-AoD/ZoD is not useful for a wide beam generated by UE. |
| OPPO | The transmission of SRS-Pos is configured by gNB. And the Tx beam of SRS-Pos is also configured by the gNB. So the provided expected UL-AoD can not be used by the UE. Furthermore, the UE rotates and the physical angle direction can not be used to determine Tx beamforming for uplink transmission. |
| LGE | As we all know, RAN1 already agree to support expected DL-AoD/ZoD, DL-AoA/ZoA for AoD enhancements. We think there is no reason to opposite the mechanism becase the consideration would be same as in DL-AoD enhancement. In addition, we belive that it does not require many times to support it because similar way of configuration for expected UL-AOA coulde be applied. |
| Ericsson | Do not support. The SRS configuration is up to the gNB and should not depend of the LMF. |

### Round #2

FL comment:

Based on received feedback it seems the Rel.17 discussion on this aspect can be closed as companies do not have common understanding and it seems to be challenging to support it at the maintenance stage

Comments from companies:

| Company Name | Comments |
| --- | --- |

Conclusions

In this document, we have provided overview of the contributions submitted to RAN1#108e for R17 NR positioning maintenance on UL AOA enhancements as well as intermediate summary of the RAN1 e-mail discussion [108-e-NR-ePos-02].

References

- R1-2200921	Correction to SRS-RSRP and SRS-RSRPP	Huawei, HiSilicon

- R1-2201094	Maintenance on enhancements for UL-AoA method	vivo

- R1-2201194	Remaining issues on UL-AOA positioning solutions	ZTE

- R1-2201240	Enhancements for UL AoA Positioning	OPPO

- R1-2201362	Remaining issues on enhancements for UL-AoA positioning method	CATT

- R1-2201635	Maintenance of enhancing UL AoA	Nokia, Nokia Shanghai Bell

- R1-2201698	Maintenance of Rel.17 NR positioning solutions for UL-AoA method	Intel Corporation

- R1-2201825	Remaining issues for UL-AoA positioning solutions	InterDigital, Inc.

- R1-2201857	Remaining issues on UL-AoA enhancements	CMCC

- R1-2202015	Discussion on accuracy improvements for UL-AoA positioning solutions	Samsung

- R1-2202141	Maintenance on Potential Enhancements on UL-AOA positioning	Qualcomm Incorporated

- R1-2202292	Discussion on accuracy improvement for UL-AoA positioning	LG Electronics

- R1-2202390	Enhancements of UL-AoA positioning solutions	Ericsson