3GPP TSG RAN WG1 #108-e			  R1-2202547

e-Meeting, February 21st – March 3rd, 2022

Agenda Item:	8.10.1

Source:	Moderator (AT&T)

Title:	Feature Lead Summary #1 of 8.10.1

Document for:	Discussion/Approval

# Introduction

This contribution provides a summary of the discussion in RAN1#108-e for the following email discussion:

[108-e-R17-eIAB-01] Email discussion on enhancements to resource multiplexing between child and parent links of an IAB node – Thomas (AT&T)

- 1st check point: February 25

- Final check point: March 3

# Simultaneous Operation of Access and Backhaul Links

From the eIAB WID:

- Specification of enhancements to the resource multiplexing between child and parent links of an IAB node, including:

  - Support of simultaneous operation (transmission and/or reception) of IAB-node’s child and parent links (i.e., MT Tx/DU Tx, MT Tx/DU Rx, MT Rx/DU Tx, MT Rx/DU Rx)

  - Support for dual-connectivity scenarios defined by RAN2/RAN3 in the context of topology redundancy for improved robustness and load balancing.

Summary of input contributions:

| Company | Proposals |
| --- | --- |
| Huawei, HiSilicon, R1-2200926 | Proposal 1: Revert the following working assumption in RAN1#106bis-e: Working Assumption If both the Rel-16 time domain H/S/NA configuration and Rel-17 frequency domain H/S/NA configuration are provided for a given RB set within a slot, one of the following is selected: - Alt. 1: An IAB node applies the frequency domain H/S/NA only if the IAB node is currently operating in a non-TDM multiplexing mode in the slot, otherwise the Rel-16 time domain H/S/NA configuration is applied.. Proposal 2: Adopt the following TP in 38.213: If an indication of hard, soft, or unavailable type is provided for an RB set in a symbol of a slot, the IAB-DU applies the configuration of hard, soft, or unavailable type provided by HSNA Slot Configuration List.. Proposal 3: If an IAB-MT is additionally provided TDD-UL-DL-ConfigDedicated-IAB-MT, the parameter tdd-UL-DL-ConfigurationDedicated-IAB-MT overrides all symbols per slot over the number of slots as provided by tdd-UL-DL-ConfigurationCommon. Proposal 4: To increase the resources for simultaneous operation, the specification should allow the collision between tdd-UL-DL-ConfigurationDedicated-IAB-MT and cell-specific signals/channels. In the slots with the collision, the IAB node should ignore tdd-UL-DL-ConfigurationDedicated-IAB-MT. The list of cell-specific signals/channels includes: - SS/PBCH block - CORESET for Type0-PDCCH CSS set - PRACH Proposal 5: Adopted the following TP in 38.213: For each serving cell, an IAB-MT can be provided an indication for a slot format over a number of slots by tdd-UL-DL-ConfigurationDedicated-IAB-MT. If the IAB-MT is provided tdd-UL-DL-ConfigurationDedicated-IAB-MT, the statements in clause 11.1 that include "tdd-UL-DL-ConfigurationDedicated" apply to the IAB-MT of an IAB node by replacing "tdd-UL-DL-ConfigurationDedicated" with "tdd-UL-DL-ConfigurationDedicated-IAB-MT" for the IAB-MT, except that the tdd-UL-DL-ConfigurationDedicated-IAB-MT provides - a set of slot configurations by slotSpecificConfigurationsToAddModList-IAB-MT - for each slot configuration from the set of slot configurations - a slot index for a slot provided by slotIndex - a set of symbols for a slot by symbols-IAB-MT where - if symbols-IAB-MT = allDownlink, all symbols in the slot are downlink - if symbols-IAB-MT = allUplink, all symbols in the slot are uplink - if symbols-IAB-MT = explicit, nrofDownlinkSymbols provides a number of downlink first symbols in the slot and nrofUplinkSymbols provides a number of uplink last symbols in the slot. If nrofDownlinkSymbols is not provided, there are no downlink first symbols in the slot and if nrofUplinkSymbols is not provided, there are no uplink last symbols in the slot. The remaining symbols in the slot are flexible. - if symbols-IAB-MT = explicit-IAB-MT, nrofUplinkSymbols provides a number of uplink first symbols in the slot and nrofDownlinkSymbols provides a number of downlink last symbols in the slot. If nrofUplinkSymbols is not provided, there are no uplink first symbols in the slot and if nrofDownlinkSymbols is not provided, there are no downlink last symbols in the slot. The remaining symbols in the slot are flexible. - the parameter tdd-UL-DL-ConfigurationDedicated-IAB-MT overrides all symbols per slot over the number of slots as provided by tdd-UL-DL-ConfigurationCommon. - the parameter tdd-UL-DL-ConfigurationDedicated-IAB-MT does not apply - for a set of symbols of a slot that are indicated to an IAB-MT for reception of SS/PBCH blocks, or - for a set of symbols of a slot that are indicated to an IAB-MT for a CORESET for Type0-PDCCH CSS set, the IAB node should ignore tdd-UL-DL-ConfigurationDedicated-IAB-MT, or - for operation on a single carrier in unpaired spectrum, for a set of symbols of a slot corresponding to a PRACH occasion and Ngap symbols before the valid PRACH occasion,. |
| Vivo, R1-2201109 | Proposal 1: When Case 6 timing is indicated for a given slot, IAB MT TX and IAB DU TX can operate simultaneously in the slot, and TP#1 should be approved. Proposal 2: When multiple sets of restricted DU beams are indicated to child node, where each set corresponds to a given MT beam, RAN1 should further discuss how to activate a given set of restricted DU beams. Proposal 3: Confirm the WA that the MAC CE signaling of Desired/Provided Guard Symbols is enhanced to optionally indicate the number of guard symbols required for switching between Case #6 MT TX to/from Case #1 DU TX. Proposal 4: An IAB-MT informs a parent node via MAC CE when Case 7 timing is to be enabled. Proposal 5: TP#2 should be approved to capture Case 6 and Case 7 timing mode reporting from IAB node to a parent node. Proposal 6: It should be further discussed how to determine the list of slots associated with a given timing cases. |
| ZTE, Sanechips R1-2201456 | Proposal 1: If both the Rel-16 time domain H/S/NA configuration and Rel-17 frequency domain H/S/NA configuration are provided for a given RB set within a slot, Rel-17 frequency domain H/S/NA configuration is applied. Proposal 2: RB set group configuration should be provided by RRC signaling per IAB-DU cell. Proposal 3: If RB set group is not provided, resourceAvailablity indicates availability of soft resources in all RB sets for an IAB-DU cell. |
| NTT DOCOMO R1-2112124 | Proposal 1: Adopt the following CR for section 14 of TS 38.213. ## <Unchanged text is omitted> When an RB set of a downlink, uplink, or flexible symbol is configured as hard, the IAB-DU cell can respectively transmit, receive, or either transmit or receive on the RB set in the symbol. When an RB set of a downlink, uplink, or flexible symbol is configured as soft, the IAB-DU cell can respectively transmit, receive or either transmit or receive on the RB set in the symbol only if - the IAB-MT does not transmit or receive on the RB set during the symbol of the IAB-DU cell, or - the IAB-MT would transmit or receive on the RB set during the symbol of the IAB-DU cell, and the transmission or reception on the RB set during the symbol of the IAB-DU cell is not changed due to a use of the RB set in the symbol by the IAB-DU, or - the IAB-MT detects a DCI format 2_5 with an AI index field value indicating the soft RB set as available When an RB set of a symbol is configured as unavailable, the IAB-DU neither transmits nor receives on the RB set in the symbol. <Unchanged text is omitted>. Proposal 2: Adopt the following CR for section 14 of TS 38.213. ## <Unchanged text is omitted> With reference to slots of an IAB-DU cell, the IAB-DU can be provided an indication of hard, soft or unavailable type per RB set for symbols configured as downlink, uplink or flexible in a slot by Rel-17 frequency-domain IAB-DU-Resource-Configuration-H/S/NA-Config [16, TS 38.473]. The RB set size and the number of RB sets are configured by RB-Set-Configuration. If an indication of hard, soft or unavailable type is not provided for an RB set of a symbol in a slot, the IAB-DU applies the configuration of hard, soft or unavailable type provided by HSNA Slot Configuration List in gNB-DU Cell Resource Configuration [16, TS 38.473] for the RB set of the symbol in the slot. If an indication of hard, soft, or unavailable type is provided for an RB set in a symbol of a slot, the IAB-DU applies the configuration of hard, soft, or unavailable type provided by Rel-17 frequency-domain IAB-DU-Resource-Configuration-H/S/NA-Config HSNA Slot Configuration List when the IAB-node uses simultaneous transmission and reception in the slot. <Unchanged text is omitted>. |
| Samsung, R1-2201526 | Proposal 1: If both the Rel-16 time domain H/S/NA configuration and Rel-17 frequency domain H/S/NA configuration are provided for a given RB set within a slot, which H/S/NA configuration is selected to be used is up to IAB implementation. Proposal 2: Guard band can be generated by each IAB node or parent IAB. Proposal 3: The following H/S/NA resource types are supported for FDM - H type for frequency resource: the IAB DU can assume it can use the frequency resource regardless of the MT’s configuration on the frequency resource - S type for frequency resource: the IAB DU can assume it can use the frequency resource without an impact on the MT’s configuration on the frequency resource - NA type for frequency resource: the IAB DU cannot assume it can use the frequency resource Proposal 4: The Working Assumption for H: Case #6 MT TX to/from Case #1 DU TX is confirmed. |
| ETRI, R1-2201614 | Proposal 4. Adopt the following TP on availability indication for soft resources. When a downlink, uplink, or flexible symbol is configured as soft, the IAB-DU cell can respectively transmit, receive or either transmit or receive in the symbol only if - the IAB-MT does not transmit or receive during the symbol of the IAB-DU cell, or - with respect to all serving cells in a cell group - the IAB-MT would transmit or receive during the symbol of the IAB-DU cell, and the transmission or reception during the symbol of the IAB-DU cell is not changed due to a use of the symbol by the IAB-DU, or - the IAB-MT detects a DCI format 2_5 with an AI index field value indicating the soft symbol as available from at least one serving cell in the cell group. Proposal 5. Down-select one among the following options: - Option 1: Rel-16 time-domain HSNA configuration and Rel-17 frequency-domain HSNA configuration cannot be configured on the same symbol/RB set. - Option 2: Rel-16 time-domain HSNA configuration and Rel-17 frequency-domain HSNA configuration can be configured on the same symbol/RB set. Proposal 6. In case that the option 1 in proposal 4 is the RAN1 understanding, explicitly capture it in the specification. Proposal 7. In case that the option 2 in proposal 4 is the RAN1 understanding, discuss the priorities between time/frequency-domain HSNA configurations on the same symbol/RB set. Frequency-domain HSNA configuration. Hard; Soft (indicated as available); Soft (not indicated as available); NA. Time-domain HSNA configuration; Hard; T = F; T > F; T > F; T > F. Soft (indicated as available); T < F; T = F; T > F; T > F. Soft (not indicated as available); T < F; T < F; T = F; T > F. NA; T < F; T < F; T < F; T = F. Proposal 8. Adopt the following TP on restricted beam indication. In the indicated slot, Tthe IAB-DU does not perform simultaneous transmission on a serving cell using an indicated RS resource index utilize the set of RS resource indices and any RS resources that refer the set of RS resource indices as a source of QCL-TypeD for transmission on a symbol or RB set group configured as soft in a serving cell, when it is not indicated as available by resourceAvailability. - when the symbol or RB set group is not indicated as available by resourceAvailability, and - when the serving cell is associated with a MT CC, if indicated, and - when the simultaneous transmission and/or reception of IAB-DU and IAB-MT are aligned with the indicated multiplexing mode info as described in [xx, TS 38.473], and - when one of the associated TCI state ID, RS ID, or SRI of IAB-MT, if indicated, is simultaneously used for uplink transmission of the IAB-MT.. |
| Nokia, Nokia Shanghai Bell, R1-2201673 | Proposal 2.1: Restricted beam indication from the parent node to the IAB node is indicated to be associated with the following IAB node’s configurations: - {MT CC, DU cell} pair and optionally may be indicated to be associated with only {DU cell} if independent of MT CC(s) - Multiplexing mode info (i.e. multiplexing info in 38.473) and optionally may be indicated to be applicable to non-overlapping frequency resources - Association with IAB-MT’s DL Rx beam via TCI state ID and RS ID (SSB ID and/or CSI-RS ID) or UL TX beam via SRI Proposal 2.2: Recommended beam indication from the IAB node to the parent node is indicated to be associated with the following IAB node’s configurations: - {MT CC, DU cell} pair and optionally may be indicated to be associated with only {DU cell} if independent of MT CC(s) - Multiplexing mode info (i.e. multiplexing info in 38.473) and optionally may be indicated to be applicable to non-overlapping frequency resources Proposal 2.3: Guard symbol indications for new Rel-17 guard symbol field should be in the range of 0-7 symbols. |
| Intel, R1-2201713 | Proposal 1: The maximum number of soft RB set groups per slot should be defined and limited. Proposal 2: The RRC signaling AvailabilityCombination can be enhanced to include multiple soft RB set groups availability indication as follows. AvailabilityCombination ::= SEQUENCE { availabilityCombinationId AvailabilityCombinationId, resourceAvailability-r16 SEQUENCE (SIZE (1..maxNrofResourceAvailabilityPerCombination-r16)) OF INTEGER (0..7) resourceAvailability-freq SEQUENCE (SIZE (1..maxNrofResourceAvailabilityPerCombination)) OF BIT STRING (SIZE (maxNrofSoftRBGroupPerSlot)) } The RRC parameter resourceAvailability-freq is defined to include multiple soft RB set groups indication, where each bit string will bitmap indicate the soft RB set groups availability within one slot. maxNrofSoftRBGroupPerSlot is a new parameter regarding the maximum number of soft RB set groups can be defined within a slot. |
| Lenovo, R1-2202090 | Proposal 1: Revoke the working assumption on application of the frequency-domain resource availability (RAN1#106-bis-e) as it is otherwise captured in the CR. Proposal 2: Support joint application of time-domain and frequency-domain H/S/NA configurations. Proposal 3: Specify rules for determining H/S/NA resource types for time-frequency resources based on separate configurations in time and frequency domains. Proposal 4: Discuss the IAB-DU behaviour in response to receiving DCI 2-5 for Rel-17 (frequency domain) resource configurations. Support conditional availability indication as a balance between existing best-effort non-TDM and TDM-only availability indication. |
| Qualcomm, R1-2202156 | Observation 1: The parameter list included in the LS to RAN3 R1-2112976 does not fully reflect the latest parameter summary made for IAB in R1-2112966 and should be updated. - E.g. new parameters: FDMRequired and AvailabilityCombinationsPerCell-r17 are not captured in R1-2112976. Observation 2: The RAN1-2112840 MAC CE impact excel sheet missed one new MAC CE parameter agreed in RAN1-107e: indication of Case6 Timing Required or not for simultaneous operation by an IAB-node to its parent node, and should be updated. Observation 3: With the constraint that a single value being configured for RB set size N={2,4,8,16,32,64} and the number of configured RB sets being limited by the max value of M=8, the configured RB sets may not be able to cover the entire carrier bandwidth without additional specification. Proposal 1: Down select from the following two options regarding the configuration of contiguous and non-overlapping RB sets. - Alt1: The configured RB sets shall cover the entire carrier bandwidth, where the configured single value of size N is applied for the RB sets except for the last one, and the last RB set shall be defined in the specification with size being equal to the remaining number of RBs of the carrier. - Alt2: The configured RB sets (by the single size N and the number of RB sets) may not cover the entire carrier bandwidth, and for the remaining RBs outside the configured RB sets, the Rel-16 time-domain resource configuration is applied. - The starting RB index shall be specified for the first RB set. Proposal 2: If both the Rel-16 time domain H/S/NA configuration and Rel-17 frequency domain H/S/NA configuration are provided for a given RB set within a slot, Rel-17 frequency domain H/S/NA configuration shall be applied. Proposal 5: There is no need to support indication of Desired/Provided Guard Symbols for the following case: H: Case #6 MT TX to/from Case #1 DU TX. Observation 6: There is no RAN1 agreement to limit the restricted IAB-DU beam indication only for simultaneous MT-TX/DU-TX mode. Proposal 6: Update description for restricted IAB-DU beam indication in Rel-17 TS 38.213 so that it can be applicable for simultaneous multiplexing modes other than MT-TX/DU-TX mode. |
| LG Electronics, R1-2202305 | Proposal 1: Slot resource information on TDM/non-TDM multiplexing mode or Case 7 DU Rx timing at the IAB-node should be shared between IAB node and its parent node. |
| Ericsson, R1-2202402 | Proposal 1 RAN1 considers the following three alternatives for the frequency start of the first RB set of the IAB-DU at Alt1: the lowest index of RB of the IAB-DU cell Alt2: the lowest index of RB of the union of the configured IAB-MT BWPs Alt3: an arbitrary index of RB of the IAB-DU cell Proposal 2 RB sets which are non-overlapping with the IAB-MT’s active BWPs, are treated as Hard IAB-DU resources. Proposal 3 If a resource is configured as Hard, or explicitly indicated as Soft-IA the IAB-DU can transmit, receive, or either transmit or receive according to its configuration only if it does not impact the IAB-MT’s actual ability to operate in any other resourcethat is either configured Not Available, or configured Soft and not explicitly indicated available. Proposal 4 If a resource is configured as Soft (including implicitly indicated available), the IAB-DU can transmit, receive, or either transmit or receive only if it does not impact the IAB-MT’s actual ability to operate in any resource, either configured Not Available, or configured Soft and not explicitly indicated available. Proposal 5 Confirm working assumption that if both Rel-16 time-domain H/S/NA configuration and Rel-17 frequency domain H/S/NA configuration are provided for a given RB set within a slot, an IAB node applies the frequency domain H/S/NA if the IAB node is currently operating in a non-TDM multiplexing mode in the slot, otherwise the Rel-16 time domain H/S/NA configuration is applied. Proposal 6 RAN1 selects the following two signaling alternatives for the fallback request to TDM operation mode: Alt1: UCI-based Alt2: MAC-CE based Proposal 7 Fallback to TDM does not need an ACK response from the parent IAB-node. Proposal 8 RAN1 to clarify which of the following alternatives apply to the enhanced AvailabilityCombinationsPerCell: Alt1: an IAB-DU cell can be provided with two separate availabilityCombinations, for TDM and FDM resource respectively. Alt2: an IAB-DU cell is provided with one availabilityCombinations, which contains both time-domain and frequency domain availability combinations. Proposal 9 RAN1 to clarify whether or not the AI index fields in a single DCI format 2_5 can be mapped to both time-domain and frequency-domain resourceAvailability according to the multiplexing mode of the IAB-DU cells. Proposal 10 Rel-17 enhancement on DCI format 2_5 should at least consider extension of the maximum payload size of DCI format 2_5 to increase the number of IAB-DU cells that can be provided with availability information for Soft resources. Proposal 11 Introducing group indexing bits in DCI format 2_5 to further increase the number of IAB-DU cells that can be indicated AI. Proposal 12 One resourceAvailability element of the availabilityCombinations table is reserved to signal to the IAB-DU that “No new Availability Indication is provided” in DCI format 2_5 for slots which receive multiple Availability Indicators. Proposal 13 RAN1 to clarify that SDM, including beam restrictions/recommendations, is restricted to Soft resources. Proposal 14 Guard symbols are not needed for Case H: Case #6 MT TX to/from Case #1 DU TX. Proposal 15 For directional collision handling in IAB inter-donor and intra-donor DC scenarios, introduce the following capabilities and RRC parameters: TX. a. simultaneousRXTXIABInterDonorDC capability b. simultaneousRXTXIABIntraDonorDC capability c. half-DuplexTDD-IABNRDC-r17 capability Proposal 16 RAN1 adopt the following text changes on DCI format 2_5 indication for an IAB-MT with multiple serving cells: |

ISSUE 2.1: FREQUENCY DOMAIN H/S/NA CONFIGURATION

RAN1 Agreements:

Agreement (RAN1#105-e) For frequency domain multiplexing, H/S/NA configurations for an IAB-node are provided separately in addition to the Rel-16 H/S/NA Agreement (RAN1#106-e) For a given RB set at a symbol, if Rel-17 frequency domain H/S/NA configuration is not provided, the Rel-16 time domain H/S/NA is applied Working Assumption (RAN1#106bis-e) If both the Rel-16 time domain H/S/NA configuration and Rel-17 frequency domain H/S/NA configuration are provided for a given RB set within a slot, one of the following is selected: - Alt. 1: An IAB node applies the frequency domain H/S/NA only if the IAB node is currently operating in a non-TDM multiplexing mode in the slot, otherwise the Rel-16 time domain H/S/NA configuration is applied. For RAN1#107-e, companies to consider the following decision point: An IAB node (or parent node) cannot operate under a given non-TDM multiplexing mode until: - Alt. 1: All required conditions and parameters which have been directly indicated/requested to the parent node (e.g. via MAC-CE) are explicitly acknowledged by the parent node. - Alt. 2: All required conditions and parameters which have been directly indicated/requested to the parent node (e.g. via MAC-CE) are implicitly acknowledged by the parent node or implicitly determined at the child node Agreement (RAN1#107-e) Whether or not an IAB node can operate under a given non-TDM multiplexing mode (i.e. multiplexing info in 38.473) is left to IAB implementation in Rel-17

Current 38.213 Text:

With reference to slots of an IAB-DU cell, the IAB-DU can be provided an indication of hard, soft or unavailable type per RB set for symbols configured as downlink, uplink or flexible in a slot by Rel-17 frequency-domain IAB-DU-Resource-Configuration-H/S/NA-Config [16, TS 38.473]. The RB set size and the number of RB sets are configured by RB-Set-Configuration. If an indication of hard, soft or unavailable type is not provided for an RB set of a symbol in a slot, the IAB-DU applies the configuration of hard, soft or unavailable type provided by HSNA Slot Configuration List in gNB-DU Cell Resource Configuration [16, TS 38.473] for the RB set of the symbol in the slot. If an indication of hard, soft, or unavailable type is provided for an RB set in a symbol of a slot, the IAB-DU applies the configuration of hard, soft, or unavailable type provided by HSNA Slot Configuration List when the IAB-node uses simultaneous transmission and reception in the slot.

FL Observation 2.1.1: The current 38.213 text does not fully capture the relevant RAN1 agreements and working assumption regarding the conditions for applying the frequency domain H/S/NA configuration. Several companies have proposed to clarify that the relevant HSNA Slot Configuration List is provided by Rel-17 frequency-domain IAB-DU-Resource-Configuration-H/S/NA-Config and applies for all non-TDM multiplexing modes, not just DU-Tx/MT-Rx. Several other companies have also indicated concerns with the working assumption and propose to not confirm it, but there are different views on the implication of the resulting behavior in case of simultaneous configuration of Rel-16 and Rel-17 H/S/NA configurations (e.g. joint configuration, orthogonal configurations, up to IAB node implementation, etc.). As a starting point for discussion in the maintenance phase, the existing RAN1 agreements/WA and 38.213 text are taken as the baseline.

Proposal 2.1.1: Clarify in 38.213 that if an indication of hard, soft, or unavailable type is provided for an RB set in a symbol of a slot, the IAB-DU applies the configuration of hard, soft, or unavailable type provided by HSNA Slot Configuration List in Rel-17 frequency-domain IAB-DU-Resource-Configuration-H/S/NA-Config [if the IAB node is currently operating in a non-TDM multiplexing mode (i.e. multiplexing info in 38.473) in the slot].

Discussion: Views on proposal 2.1.1?

| Company | Comments |
| --- | --- |

__________________________________________________________________________________

Current 38.213 Text:

With reference to slots of an IAB-DU cell, a symbol in a slot of an IAB-DU cell can be configured to be of hard, soft, or unavailable type by HSNA Slot Configuration List in gNB-DU Cell Resource Configuration [16, TS 38.473]. When a downlink, uplink, or flexible symbol is configured as hard, the IAB-DU cell can respectively transmit, receive, or either transmit or receive in the symbol. When a downlink, uplink, or flexible symbol is configured as soft, the IAB-DU cell can respectively transmit, receive or either transmit or receive in the symbol only if - the IAB-MT does not transmit or receive during the symbol of the IAB-DU cell, or - with respect to all serving cells - the IAB-MT would transmit or receive during the symbol of the IAB-DU cell, and the transmission or reception during the symbol of the IAB-DU cell is not changed due to a use of the symbol by the IAB-DU, or - the IAB-MT detects a DCI format 2_5 with an AI index field value indicating the soft symbol as available from at least one serving cell When a symbol is configured as unavailable, the IAB-DU neither transmits nor receives in the symbol.

FL Observation 2.1.2: Currently, only the Rel-16 TDM H/S/NA definitions are captured in 38.213 which are applicable to the entire configured symbol, while Rel-17 frequency domain H/S/NA indications can be configured independently per RB set within a slot. Some companies have proposed a direct extension of the TDM H/S/NA definitions, while one company (R1-2202402) proposed to consider adjacent RB sets explicitly in the definition, while another company proposes any necessary guard band is left to IAB node implementation (R1-2201526).

Proposal 2.1.2: Adopt the following definition for H/S/NA configured in an RB set of a symbol:

FDM Hard: When an RB set of a downlink, uplink, or flexible symbol is configured as hard, the IAB-DU cell can respectively transmit, receive, or either transmit or receive on the RB set in the symbol.

FDM Soft: When an RB set of a downlink, uplink, or flexible symbol is configured as soft, the IAB-DU cell can respectively transmit, receive or either transmit or receive on the RB set in the symbol only if

-    the IAB-MT does not transmit or receive on the RB set during the symbol of the IAB-DU cell, or

-    the IAB-MT would transmit or receive on the RB set during the symbol of the IAB-DU cell, and the transmission or reception on the RB set during the symbol of the IAB-DU cell is not changed due to a use of the RB set in the symbol by the IAB-DU, or

-    the IAB-MT detects a DCI format 2_5 with an AI index field value indicating the soft RB set as available

FDM NA: When an RB set of a downlink, uplink, or flexible symbol is configured as unavailable, the IAB-DU neither transmits nor receives in the symbol.

Discussion: Views on proposal 2.1.2?

| Company | Comments |
| --- | --- |

__________________________________________________________________________________

RAN1 Agreements:

Agreement (RAN1 #106e) N is a configured number of PRBs, where the CU configures N - N = {2, 4, 8, 16, 32, 64} - FFS: Value(s) of N in case of multiple configured BWPs at the IAB-MT - This agreement does not revert any existing RAN1 agreement Agreement (RAN1 #106bis-e) A single value for the RB set size, N, is configured for a given IAB-DU cell’s Rel-17 frequency domain H/S/NA configuration Agreement (RAN1#107-e): The maximum number of non-overlapping RB sets configurable per DU cell is M - where, M is to be selected from one of values from 4, 8, 16 DU frequency configuration information should be provided to the parent node Agreement (RAN1#107-e): The value of the maximum number of contiguous and non-overlapping RB sets configurable per DU cell, M is 8.

FL Observation 2.1.3: Based on analysis in R1-2202402 and R1-2202156, for certain carrier bandwidths, the values of RB set size combined with the limitation of M=8 total RB sets, may lead to cases where the entire bandwidth. According to RAN1 agreements and the current 38.213 CR, any frequency resources not configured as a result would be covered by the Rel-16 time domain H/S/NA configuration, however it is ambiguous where the RB sets would be configured relative to the carrier bandwidth in this case.

Proposal 2.1.3:  If the configured RB sets (by the single size N and the number of RB sets) do not cover the entire carrier bandwidth the starting RB index shall be specified for the first RB set.

Alt. 1: The start RB index of the first RB set is the lowest index of RB of the IAB-DU cell

Alt. 2: The start RB index of the first RB set is the lowest index of RB of the union of the configured IAB-MT BWPs

Alt. 3: The start RB index of the first RB set is any arbitrary index of RB of the IAB-DU cell

Discussion: Views on proposal 2.1.3?

| Company | Comments |
| --- | --- |

ISSUE 2.2: FREQUENCY DOMAIN DCI FORMAT 2_5

RAN1 Agreements:

Agreement (RAN1 #107e): For DCI format 2_5 indicating availability for the soft resources of the respective RB sets corresponding to a given time resource of the child IAB-DU cell: - AvailabiltyCombination can be extended to include multiple resourceAvailabilty, where each resourceAvailabilty includes availability indication for one RB set group - One RB set group consists of one or multiple RB sets

Existing 38.213 Text:

If an IAB-node is provided an AvailabilityIndicator, the IAB-node is provided an AI-RNTI by ai-RNTI and a payload size of a DCI format 2_5 by dci-PayloadSizeAI. The IAB-node is also provided a search space set configuration, by SearchSpace, for monitoring PDCCH. For each cell of an IAB-DU in a set of cells of the IAB-DU, the IAB-DU can be provided: - an identity of the IAB-DU cell by iab-DU-CellIdentity - a location of an availability indicator (AI) index field in DCI format 2_5 by positionInDCI-AI - a set of availability combinations by availabilityCombinations, where each availability combination in the set of availability combinations includes - resourceAvailability indicating availability of soft symbols in one or more slots for the IAB-DU cell, or one or multiple resourceAvailability with each resourceAvailablity indicating availability of soft resources in one or more slots for one RB set group where one RB set group includes one or multiple RB sets, and - a mapping for the soft symbol availability combinations provided by resourceAvailability to a corresponding AI index field value in DCI format 2_5 provided by availabilityCombinationId

FL Observation 2.2.1: Currently, the size and details of an RB set group are not defined, and this impacts both the RRC signalling as well as the payload of DCI Format 2_5 as analyzed in R1-2201456 and R1-2201713.

Proposal 2.2.1: Enhance the RRC signaling for the configuration of DCI Format 2_5 to include the configuration of availability indication for multiple soft RB set groups by introducing the following new RRC parameters:

- An extension of resourceAvailability to include multiple RB set groups indication, which indicate the soft RB set groups availability within one slot

- A new parameter which indicates the maximum number of RB set groups which can be defined within a slot.

  - FFS: value range

Discussion: Views on proposal 2.2.1?

| Company | Comments |
| --- | --- |

__________________________________________________________________________________

FL Observation 2.2.2: Another issue was raised about the payload of DCI Format 2_5 in R1-2202402, where it was observed that the number of availability indications is limited to 14 IAB-DU cells out of the maximum possible 512 IAB-DU cells. While the use case for providing an indication for all possible DU cells is not necessarily clear, if 14 cells is not sufficient for practical Rel-17 eIAB use cases, the issue and solutions should be considered.

Proposal 2.2.2: The maximum payload size of DCI format 2_5 is extended to increase the number of IAB-DU cells that can be provided with availability information for soft resources.

Discussion: Views on proposal 2.2.2?

| Company | Comments |
| --- | --- |

ISSUE 2.3: SPATIAL DOMAIN MULTIPLEXING

RAN1 Agreements:

Agreement (RAN1#107-e) The restricted beam indication from the parent node to the IAB node may be indicated to be associated with some combination (one or multiple) of the following IAB-node’s configurations: - {MT CC, DU cell} pair and optionally may be indicated to be associated with only {DU cell} if independent of MT CC(s) - Multiplexing mode info (i.e. multiplexing info in 38.473) and optionally may be indicated to be applicable to non-overlapping frequency resources - Slot index - Association with IAB-MT’s DL Rx beam via TCI state ID and RS ID (SSB ID and/or CSI-RS ID) or UL TX beam via SRI Agreement The IAB-DU is expected to apply the indicated beam restriction within its soft resources that are not explicitly indicated as available.

Existing 38.213 Text:

The IAB-node can be provided a set of RS resource indexes that indicate quasi co-location properties of the IAB-DU serving cell by the parent node where simultaneous transmission from the IAB-MT and IAB-DU serving cells is restricted by Child IAB-DU Restricted Beam Indication MAC CE as described in [11, TS 38.321]. The IAB-DU does not perform simultaneous transmission on a serving cell using an indicated RS resource index on a symbol or RB set group configured as soft when it is not indicated as available by resourceAvailability.

FL Observation 2.3.1: R1-2202156 and R1-2201614 propose that 38.213 should be updated to cover cases besides simultaneous transmission from the IAB-MT and IAB-DU serving cells to align with the optional indication of multiplexing mode info by the restricted beam indication MAC CE. In addition, R1-2201614 highlights that the RS resource indices may be further associated with the MT CC, slot index, or DL Rx or UL Tx beams of the IAB-MT and this is not yet reflected in 38.213. R1-2202402 additionally proposes to further clarify 38.213 that the application of beam restrictions is only for soft resources.

Proposal 2.3.1: Update description for restricted IAB-DU beam indication in 38.213 so that it can be applicable in soft resources for multiplexing modes other than MT-TX/DU-TX, as well as associated with an indicated MT CC, slot index, or IAB-MT DL Rx or UL Tx beam if provided by the restricted beam indication MAC CE.

Discussion: Views on proposal 2.3.1?

| Company | Comments |
| --- | --- |

__________________________________________________________________________________

RAN1 Agreements:

Agreement (RAN1#107-e) The restricted beam indication from the parent node to the IAB node may be indicated to be associated with some combination (one or multiple) of the following IAB-node’s configurations: - {MT CC, DU cell} pair and optionally may be indicated to be associated with only {DU cell} if independent of MT CC(s) - Multiplexing mode info (i.e. multiplexing info in 38.473) and optionally may be indicated to be applicable to non-overlapping frequency resources - Slot index - Association with IAB-MT’s DL Rx beam via TCI state ID and RS ID (SSB ID and/or CSI-RS ID) or UL TX beam via SRI Agreement The IAB-DU is expected to apply the indicated beam restriction within its soft resources that are not explicitly indicated as available. Agreement (RAN1#107-e) - The maximum number of recommended beams per MT CC in a given indication (including all associated parameters/conditions) is 8. - The maximum number of restricted beams per DU cell in a given indication (including all associated parameters/conditions) is 8.

FL Observation 2.3.2: The analysis of R1-2201109 and R1-2201713 raise concerns about ambiguity of the applicability of an indicated beam restriction (or recommendation) to specific resources. Specifically if multiple indications are provided, do subsequent indications override one another, and if not, how does the IAB-DU apply a given restriction from the multiple sets (e.g. union of beams, ambiguity of the slot indices, etc.). RAN1 will likely need to capture this and possibly may need to inform RAN2 to complete their MAC CE design.

Proposal 2.3.2: IAB-DU behavior is defined in case of multiple indicated beam restrictions with different configurations across indications.

Discussion: Views on proposal 2.3.2?

| Company | Comments |
| --- | --- |

ISSUE 2.4: TIMING CASE INDICATION FOR SIMULTANEOUS OPERATION

RAN1 Agreements:

Agreement (RAN1#107-e) An IAB-MT is provided with a Timing Case Indication via MAC-CE that explicitly indicates a list of slots and their associated UL TX timing cases (i.e., one of {Case 1, Case 6, Case 7} for each slot).

FL Observation 2.4.1: In order for RAN2 to complete the Timing Case Indication MAC CE design, the range and periodicity of the slot index list should be clarified. Also, given the intentional alignment of MAC CE parameters across eIAB features (e.g. beam indication, power adjustment, timing case indication, etc.) which relate to simultaneous operation, it would be beneficial to consider extending this agreement to other MAC CE slot index lists.

Proposal 2.4.1: Define the range and periodicity of the list of slots associated with a given timing cases provided to an IAB-MT by the Timing Case Indication MAC CE.

- FFS: Extension of this agreement to other eIAB MAC CEs (e.g. restricted/recommended beam indications)

Discussion: Views on proposal 2.4.1?

| Company | Comments |
| --- | --- |

ISSUE 2.5: GUARD SYMBOLS FOR SIMULTANEOUS OPERATION TIMING CASES

RAN1#107-e Agreement:

Agreement The following RAN1#106bis-e agreement is updated. The MAC-CE signaling of Desired/Provided Guard Symbols is enhanced to optionally indicate the number of guard symbols required for switching between at least the following cases: - Case#6 MT Tx and [Case #7] DU [Tx]/Rx - Case#7 MT Tx (to support Case #7 at parent node) and DU Tx/Rx - A: Case #6 MT TX to/from Case #1 DU RX - D: Case #7 MT TX (to support Case #7 at parent node) to/from Case #1 DU RX - G: Case #7 MT TX (to support Case #7 at parent node) to/from Case #1 DU TX - (Working Assumption) H: Case #6 MT TX to/from Case #1 DU TX

FL Observation 2.5.1: Multiple companies have proposed to confirm or overturn the RAN1#107-e working assumption to support guard symbols for Case H: Case #6 MT TX to/from Case #1 DU TX. Given eIAB is in the Rel-17 maintenance phase and the signaling is optional, there does appear to be a clear need to remove it from the current list of MAC CEs for Rel-17.

Proposal 2.5.1: Confirm the WA to support Case H: Case #6 MT TX to/from Case #1 DU TX in the MAC-CE signaling of Desired/Provided Guard Symbols in Rel-17.

Discussion: Views on proposal 2.5.1?

| Company | Comments |
| --- | --- |

__________________________________________________________________________________

FL Observation 2.5.2:

In addition, the range of the guard symbols introduced in Rel-17 for new timing cases has not been agreed. As pointed out in R1-2201673, for the new DU/MT transition cases introduced in Rel-17, there may be a need to use the reserved values (5-7) to accommodate both propagation delay as well as increased timing offset values.

Proposal 2.5.2: The number of desired/provided guard symbols indicated for Rel-17 cases should be in the range of 0-7 symbols.

Discussion: Views on proposal 2.5.2?

| Company | Comments |
| --- | --- |

# Resource allocation for dual-connectivity scenarios (i.e. IAB-MT with concurrent BH links with two parent nodes)

From the eIAB WID:

- Specification of enhancements to the resource multiplexing between child and parent links of an IAB node, including:

  - Support of simultaneous operation (transmission and/or reception) of IAB-node’s child and parent links (i.e., MT Tx/DU Tx, MT Tx/DU Rx, MT Rx/DU Tx, MT Rx/DU Rx)

  - Support for dual-connectivity scenarios defined by RAN2/RAN3 in the context of topology redundancy for improved robustness and load balancing.

Summary of input contributions:

| Vivo, R1-2201109 | Proposal 7: For an IAB MT in DC, the mode 2 semi-static power control is performed based on both TDD configuration and per-child MT link-NA resource configuration. Proposal 8: For an IAB MT in DC, when performing mode 2 semi-static power control, when UL transmission on a first leg (i.e., MCG or SCG) is overlapped with UL symbol on the second leg (i.e., SCG or MCG), if NA is assumed for overlapped UL symbol, the maximum transmission power of the UL transmission is restricted by PCMAX; Otherwise, restricted by configured value PMCG or PSCG. |
| --- | --- |
| NTT DOCOMO, R1-2201492 | Proposal 3: Adopt the following CR for section 14 of TS 38.213. ## <Unchanged text is omitted> When a downlink, uplink, or flexible symbol is configured as soft, the IAB-DU cell can respectively transmit, receive or either transmit or receive in the symbol only if - the IAB-MT does not transmit or receive during the symbol of the IAB-DU cell, or - with respect to all serving cells - the IAB-MT would transmit or receive during the symbol of the IAB-DU cell, and the transmission or reception during the symbol of the IAB-DU cell is not changed due to a use of the symbol by the IAB-DU, or - the IAB-MT detects a DCI format 2_5 with an AI index field value indicating the soft symbol as available from at least one serving cell ## <Unchanged text is omitted>. |
| ETRI, R1-2201614 | Proposal 1. Down-select one among the following options: - Option 1: Confirm that a parent node with MCG and the other peer parent node with SCG can independently operate DCI format 2_0 in any DC scenario. - Option 2: Adopt the following TP on DC operations. If an IAB-MT is configured with an MCG and an SCG, is not capable of simultaneous transmission and reception, and would simultaneously transmit and receive on the MCG and the SCG, - the IAB-MT does not expect to receive conflicting DCI 2_0 from different parents for intra-donor DC scenarios, or - the IAB-MT operates according to the scheduling from the MCG for inter-donor DC scenarios.. Proposal 2. Down-select one among the following options: - Option 1: Confirm that CA TDD prioritization rules are applied within the same cell group in Rel-17 (i.e., no further enhancements from RAN1 perspectives). - Option 2: Send an LS to RAN2 to extend CA TDD prioritization rules to NR-DC for IAB use cases (i.e., remove/revise the legacy restrictions on directionalCollisionHandling-r16, half-DuplexTDD-CA-SameSCS-r16 and simultaneousRxTxInterBandCA or introduce new IEs). Proposal 3. Down-select one among the following options: - Option 1: Confirm that “Alt. 5: If a conflict occurs, the IAB MT is expected to perform as scheduled by MCG” is applied [for all DC scenarios / for inter-donor DC scenarios]. - Option 2: Adopt the following TP on DC operations. If an IAB-MT is configured with an MCG and an SCG, is not capable of simultaneous transmission and reception, and would simultaneously transmit and receive on the MCG and the SCG, - the IAB-MT does not expect to receive conflicting DCI formats including DCI2_0 and dynamic scheduling grants from different parents for intra-donor DC scenarios, or - the IAB MT does not expect to receive conflicting DCI formats including DCI2_0 and dynamic scheduling grants from different parents for inter-donor DC scenarios when the conflicting symbols configured as semi-static flexible by one parent node, but not the other, or - the IAB-MT operates according to the scheduling from the MCG for inter-donor DC scenarios when the conflicting symbols configured as semi-static flexible by both parent nodes.. |
| Lenovo, R1-2202090 | Proposal 5: Support signaling from an IAB node in the DC mode to its parent node for informing the parent node of the status of availability of soft resources. Further discuss specification of mechanisms to handle availability indication collisions by two parent nodes in the DC mode. Proposal 6: RAN1 to address resource conflicts in the DC mode due to conflicting Case-7 timing. |
| Qualcomm, R1-2202156 | Observation 4: In Rel-16, the application of TDD prioritization rules defined in TS38.213 is limited for CA case because parameter “directionalCollisionHandling-r16” only enables directional collision handling in TDD CA within same cell group. Proposal 3: Extend RRC parameter “directionalCollisionHandling-r16” to enable extension of Rel-16 CA TDD prioritization rules defined in TS38.213 to NR-DC across cell groups. Observation 5: The Alt5 rule agreed by RAN1 for NR-DC is not captured correctly in Rel-17 TS 38.213 for IAB in that the application of the rule is not limited to the TDD confliction case with semi-static flexible symbols configured by both parents for inter-donor NR-DC. Proposal 4: Update Rel-17 TS 38.213 for TDD confliction resolution for IAB with NR-DC: - Constrain the Alt5 rule to be applied for TDD confliction cases with semi-static flexible symbols configured by both parents for inter-donor NR-DC. - Add statement that IAB-MT with NR-DC shall apply the Rel-16 CA TD prioritization rules defined in TS38.213 section 11.1 across cell groups for remaining cases not covered by Alt5 rule. |
| Ericsson, R1-2202402 | Proposal 15 For directional collision handling in IAB inter-donor and intra-donor DC scenarios, introduce the following capabilities and RRC parameters: TX. a. simultaneousRXTXIABInterDonorDC capability b. simultaneousRXTXIABIntraDonorDC capability c. half-DuplexTDD-IABNRDC-r17 capability Proposal 16 RAN1 adopt the following text changes on DCI format 2_5 indication for an IAB-MT with multiple serving cells: |

ISSUE 3.1: CA TDD PRIORITIZATION RULES EXTENSION TO NR_DC

RAN1 Agreements:

Agreements: (RAN1#105-e) In case of intra-band inter-carrier dual connectivity for both inter-donor and intra-donor scenarios the following are supported: - Reusing the Rel-16 CA TDD prioritization rules in case of UL/DL conflict when applicable - FFS: Whether all prioritization rules apply in case of NR-DC - FFS: Need of new prioritization rules in case of NR-DC - Coordinating the IAB-MT’s TDD configurations to avoid conflicts from different parent nodes in case the child IAB-MT does not support simultaneous TX and RX on different carries - FFS: Coordination for scheduling conflicts involving at least DCI Format 2_0 usage (e.g. usage of flexible symbols) Exchanging H/S/NA configurations between parent nodes/donors Agreement: (RAN1#106-e) The following solutions are supported to handle potential indication conflict of overlapping flexible symbols between two parent IAB-nodes: In intra-donor DC scenarios, if the IAB MT does not support simultaneous Tx and Rx on different carriers, it does not expect to receive conflicting DCI 2_0 from different parents. Agreement (RAN1#106bis-e) Select the following alternative to handle potential indication conflict of symbols configured as semi-static flexible by one parent node, but not the other in inter-donor DC scenarios if the IAB MT of the dual-connected IAB-node does not support simultaneous Tx and Rx on different carriers: - Alt. 1. The IAB MT does not expect to receive conflicting DCI formats including DCI2_0 and dynamic scheduling grants from different parents. FFS: Explicitly captured in the specification or left as a network configuration error case without specification impact Select the following alternative to handle potential indication conflict of symbols configured as semi-static flexible by both parent nodes in inter-donor DC scenarios if the IAB MT of the dual-connected IAB-node does not support simultaneous Tx and Rx on different carriers: Alt. 5: If a conflict occurs, the IAB MT is expected to perform as scheduled by MCG

Current 38.213 Text:

If a UE - is configured with multiple serving cells and is provided with directionalCollisionHandling-r16 = 'enabled' for a set of serving cell(s) among the multiple serving cells, and - indicates support of half-DuplexTDD-CA-SameSCS-r16 capability, and - is not configured to monitor PDCCH for detection of DCI format 2_0 on any of the multiple serving cells, - - - - - - - - - - - - - - - - <paragraphs omitted>- - - - - - - - - - - - - - - - the UE - - - - - - - - - - - - - - - - <paragraphs omitted>- - - - - - - - - - - - - - - - - does not expect to detect a first DCI format scheduling a transmission or reception on a symbol on a first cell and a second DCI format scheduling a reception or transmission on the symbol on a second cell, respectively. After the UE applies the procedures described above for directional collision handling within the set of cells that have been configured with directionalCollisionHandling-r16, the UE does not expect any directional collision among the serving cells that the UE is not capable of simultaneous transmission and reception.

Current 38.306 Text:

halfDuplexTDDCASameSCSr16 Indicates whether the UE supports directional collision handling between reference and other cell(s) for half-duplex operation in TDD CA with same SCS. The UE can include this field for band combinations including only intra-band TDD CA or if simultaneousRxTxInterBandCA is not present for band combinations involving mix of intra-band TDD CA and inter-band TDD CA

Current 38.331 Text:

directionalCollisionHandling

Indicates that this serving cell is using directional collision handling between a reference and other cell(s) for half-duplex operation in TDD CA with same SCS as specified in TS 38.213 [13], clause 11.1. The half-duplex operation only applies within the same frequency range and cell group. The network only configures this field for TDD serving cells that are using the same SCS.

FL Observation 3.1.1: Based on RAN1 agreements and analysis in R1-2201614, R1-2202156, R1-2202402, the  Rel-16 CA TDD prioritization rules defined in TS38.213 are to be used for IAB in case of NR-DC across cell groups. Currently in TS 38.213, there are three relevant RRC parameters:

directionalCollisionHandling-r16,

half-DuplexTDD-CA-SameSCS-r16,

simultaneousRxTxInterBandCA

Proposal 3.1.1: In order to support the agreed extension of CA TDD conflict resolution rules to IAB nodes operating under NR-DC (covering both inter-donor and intra-donor NR-DC scenarios), new RRC parameters should be introduced:

- directionalCollisionHandling-r17

- half-DuplexTDD-IABNRDC-r17

- simultaneousRxTxIABInterDonorDC

- simultaneousRxTxIABIntraDonorDC

Discussion: Views on proposal 3.1.1?

| Company | Comments |
| --- | --- |

ISSUE 3.2: DCI 2_5 INDICATION IN DC SCENARIOS

RAN1 Agreements:

Agreements: (RAN1#105-e) In case of intra-band inter-carrier dual connectivity for both inter-donor and intra-donor scenarios the following are supported: - Reusing the Rel-16 CA TDD prioritization rules in case of UL/DL conflict when applicable - FFS: Whether all prioritization rules apply in case of NR-DC - FFS: Need of new prioritization rules in case of NR-DC - Coordinating the IAB-MT’s TDD configurations to avoid conflicts from different parent nodes in case the child IAB-MT does not support simultaneous TX and RX on different carries - FFS: Coordination for scheduling conflicts involving at least DCI Format 2_0 usage (e.g. usage of flexible symbols) Exchanging H/S/NA configurations between parent nodes/donors Agreement (RAN1#106-e) The following solutions are supported to handle potential indication conflict of overlapping flexible symbols between two parent IAB-nodes: In intra-donor DC scenarios, if the IAB MT does not support simultaneous Tx and Rx on different carriers, it does not expect to receive conflicting DCI 2_0 from different parents. Agreement (RAN1#106bis-e) Select the following alternative to handle potential indication conflict of symbols configured as semi-static flexible by one parent node, but not the other in inter-donor DC scenarios if the IAB MT of the dual-connected IAB-node does not support simultaneous Tx and Rx on different carriers: - Alt. 1. The IAB MT does not expect to receive conflicting DCI formats including DCI2_0 and dynamic scheduling grants from different parents. FFS: Explicitly captured in the specification or left as a network configuration error case without specification impact Select the following alternative to handle potential indication conflict of symbols configured as semi-static flexible by both parent nodes in inter-donor DC scenarios if the IAB MT of the dual-connected IAB-node does not support simultaneous Tx and Rx on different carriers: Alt. 5: If a conflict occurs, the IAB MT is expected to perform as scheduled by MCG

Current 38.213 text:

If an IAB MT is configured with an MCG and an SCG, is not capable of simultaneous transmission and reception, and would simultaneously transmit and receive on the MCG and the SCG, the IAB MT operates according to the scheduling from the MCG.

FL Observation 3.2.1: Based on RAN1 agreements and analysis in R1-2201614, R1-2202156, the Alt. 5 conflict resolution rule (MT follows MCG scheduling) should only apply for cases with semi-static flexible symbols configured by both parents under inter-donor NR-DC scenarios. Additionally., the behaviour agreed for the other NR-DC cases is not captured currently in 38.213 (i.e. The IAB MT does not expect to receive conflicting DCI formats including DCI2_0 and dynamic scheduling grants from different parents).

Proposal 3.2.1: Update Rel-17 TS 38.213 in the following manner:

- If an IAB MT is configured with an MCG and an SCG, is not capable of simultaneous transmission and reception, and would simultaneously transmit and receive on the MCG and the SCG, the IAB- MT operates according to the scheduling from the MCG only if semi-static flexible symbols are configured by both parents operating under inter-donor NR-DC.

- For other NR-DC cases, the IAB-MT shall apply the Rel-16 CA TDD prioritization rules defined in TS38.213 section 11.1 across cell groups (i.e. The IAB MT does not expect to receive conflicting DCI formats including DCI2_0 and dynamic scheduling grants from different parents).

Discussion: Views on proposal 3.2.1?

| Company | Comments |
| --- | --- |

ISSUE 3.3: DCI 2_5 INDICATION IN DC SCENARIOS

RAN1#105-e agreement:

Agreement For an IAB-MT with multiple serving cells (including the case with two parent nodes), a per-cell IAB-DU soft resource is considered as available if the resource is either explicitly indicated (via DCI 2_5), or implicitly determined as available with respect to all serving cells. - If the IAB-DU per-cell soft resource neither explicitly indicated as Available, nor implicitly determined as Available by the IAB-DU with respect to at least one serving cell - Alt 1. The IAB-DU per-cell resource is assumed to be not available

Current 38.213 text:

When a downlink, uplink, or flexible symbol is configured as soft, the IAB-DU cell can respectively transmit, receive or either transmit or receive in the symbol only if - the IAB-MT does not transmit or receive during the symbol of the IAB-DU cell, or - with respect to all serving cells - the IAB-MT would transmit or receive during the symbol of the IAB-DU cell, and the transmission or reception during the symbol of the IAB-DU cell is not changed due to a use of the symbol by the IAB-DU, or - the IAB-MT detects a DCI format 2_5 with an AI index field value indicating the soft symbol as available from at least one serving cell

FL Observation 3.3.1: Based on the latest draft of 38.213 and analysis from R1-2201492, R1-2202402, the second subbullet appears to be in conflict with the second main bullet and therefore not aligned with the RAN1#105-e agreement.

Proposal 3.3.1: Adopt the following TP (from R1-2201492, R1-2202402):

## <Unchanged text is omitted> When a downlink, uplink, or flexible symbol is configured as soft, the IAB-DU cell can respectively transmit, receive or either transmit or receive in the symbol only if - the IAB-MT does not transmit or receive during the symbol of the IAB-DU cell, or - with respect to all serving cells - the IAB-MT would transmit or receive during the symbol of the IAB-DU cell, and the transmission or reception during the symbol of the IAB-DU cell is not changed due to a use of the symbol by the IAB-DU, or - the IAB-MT detects a DCI format 2_5 with an AI index field value indicating the soft symbol as available from at least one serving cell <Unchanged text is omitted>

Discussion: Views on proposal 3.3.1?

| Company | Comments |
| --- | --- |

# Other eIAB Resource Multiplexing Maintenance Issues

FL Observation: Priority for RAN1#108-e is to complete RAN2/RAN3 impacting maintenance items (i.e. MAC/RRC/F1-AP parameters). It is encouraged that companies additionally comment on proposals from themselves or others (pro/con) in order to determine prioritization for any issues not yet included in sections 2 or 3.

Discussion: Any additional maintenance features which should be discussed in RAN1#108-e?

| Company | Comments |
| --- | --- |

# Summary

TBD