3GPP TSG RAN WG1 Meeting #108-e	R1-2202647

e-Meeting, February 21 – March 3, 2022

Agenda Item:	8.1.4

Source:	(Moderator) Huawei, HiSilicon

Title:	TP of Rel-17 CSI Enhancement for TS 38.212

Document for:	Discussion and Decision

# Reasons of Changes

In RAN1 107, it was agreed to clarify values of the KNZ indicator field mapped to the allowed values of KNZ with increasing order, where '0' is mapped to KNZ=1 for Rel-16 eType II codebook. Similarly, above clarification shall be applied to Rel-17 FeTypeII PS codebook for corresponding KNZ indicator field in order to elaborate the mapping order and avoid ambiguity between reported values and actual values.

# Text Proposal for Rel-17 CSI enhancement for FDD

##### 6.3.2.1.2	CSI

< Unchanged parts are omitted >

The bitwidth for RI/CQI of codebookType= typeII-r16 or codebookType=typeII-PortSelection-r16 is provided in Table 6.3.2.1.2-8.

Table 6.3.2.1.2-8: RI and CQI of codebookType=typeII-r16 or typeII-PortSelection-r16

| Field | Bitwidth |
| --- | --- |
| Rank Indicator | min2,log2nRI |
| Wide-band CQI | 4 |
| Subband differential CQI | 2 |
| Indicator of the total number of non-zero coefficients summed across all layers KNZ | log2(K0) if max allowed rank is 1; log2(2K0) otherwise |

where nRI is the number of allowed rank indicator values according to Clauses 5.2.2.2.5 and 5.2.2.2.6 [6, TS 38.214], K0=2Lp1×N3Rβ, where p1, N3, R, and β are given by Clause 5.2.2.2.5 and 5.2.2.2.6 in [6, TS 38.214]. The values of the rank indicator field are mapped to allowed rank indicator values with increasing order, where ‘0’ is mapped to the smallest allowed rank indicator value. The values of the KNZ indicator field are mapped to the allowed values of KNZ, according to Clauses 5.2.2.2.5 and 5.2.2.2.6 [6, TS 38.214], with increasing order, where ‘0’ is mapped to KNZ=1.

The bitwidth for RI/CQI of codebookType=typeII-PortSelection-r17 is provided in Table 6.3.2.1.2-9.

Table 6.3.2.1.2-9: RI and CQI of codebookType=typeII-PortSelection-r17

| Field | Bitwidth |
| --- | --- |
| Rank Indicator | min2,log2nRI |
| Wide-band CQI | 4 |
| Subband differential CQI | 2 |
| Indicator of the total number of non-zero coefficients summed across all layers KNZ | log2(K0) if max allowed rank is 1; log2(2K0) otherwise |

where nRI is the number of allowed rank indicator values according to Clauses 5.2.2.2.7 [6, TS 38.214], K0=K1Mβ, where K1, M, and β are given by Clause 5.2.2.2.7 in [6, TS 38.214]. The values of the rank indicator field are mapped to allowed rank indicator values with increasing order, where ‘0’ is mapped to the smallest allowed rank indicator value. The values of the KNZ indicator field are mapped to the allowed values of KNZ, according to Clauses 5.2.2.2.7 [6, TS 38.214], with increasing order, where ‘0’ is mapped to KNZ=1.

< Unchanged parts are omitted >

# Comments

| Company | Comments |
| --- | --- |
| Mod | If there is any polishing preferred or error, please be free to comment. Thank you very much! |