3GPP TSG-RAN WG1 Meeting #108-e											R1-2202654

e-Meeting, February 21st - March 3rd, 2022

Agenda Item:	5.2

Source:			Moderator (ZTE)

Title:			Comments collection on RRC parameters for small data transmission

Document for:	Discussion

Introduction

This summary is to collect comments for RRC parameters for SDT work item. The RRC parameters have been captured in the excel sheet in the same draft folder.

[108-e-R17-RRC-SDT] Email discussion on Rel-17 RRC parameters for SDT – Ziyang (ZTE)

- 1st check point for first LS in [108-e-R17-RRC]: February 24

- Final check point for second LS in [108-e-R17-RRC] if necessary: March 3

Unstable parameters

Discussion

In RAN1#107-e meeting, a list of RRC parameters are provided and some of them are considered as unstable, in this meeting, these unstable parameters will be discussed in another email thread [108-e-R17-SDT-01] and if agreements can be achieved, the corresponding changes will be updated in excel sheet and this document.

To better track the changes, the latest version of unstable parameters are copied in the following table:

Table-1 latest version of unstable parameters

| Row index | Parameter name in the spec | Description | Comment |
| --- | --- | --- | --- |
| 7 | sdt-SSB-PerCG-PUSCH-r17 | The number of SSBs per CG PUSCH. Value 4 corresponds to 4 SSBs per CG PUSCH, and so on. | Agreement Mapping ratio of SSB to CG PUSCH is configured per CG configuration. FFS whether to restrict the same value for all CG configuration?and/or allow different value for different CG configurations. For the candidate value set of SSB to CG PUSCH mapping ratio, support at least {1, 2, 4, 8, 16} FFS {1/8,1/4,1/2} Agreement For CG-SDT, support mapping ratio {1/8, 1/4, 1/2} for SSB to CG PUSCH mapping. |
| 10 | repK | Number of repetitions K | Agreement For CG-SDT, RAN1 cannot reach consensus on whether to support repetition or not, it’s up to RAN2 to decide on it. |
| 11 | repK-RV | The redundancy version (RV) sequence to use. | Agreement For CG-SDT, RAN1 cannot reach consensus on whether to support repetition or not, it’s up to RAN2 to decide on it. |
| 12 | antennaPort | It's not applicable for CG-SDT. | Agreement Introduce a new parameter e.g. sdt-DMRSports to configure the set of DMRS ports for SSB to PUSCH mapping -Parameter antennaPort is not applicable to CG-SDT Support up to 2 DMRS sequences for CG-SDT for CP-OFDM, the generation mechanism and configuration can reuse that of msgA PUSCH. -Introduce a new parameter sdt-NrofDMRS-Sequences to configure 1 or 2 DMRS sequences. -The description of parameter dmrs-SeqInitialization can be revised as “ It’s present when single DMRS sequence is configured for CG-SDT”. |
| 13 | precodingAndNumberOfLayers | Only single antenna port for single layer transmission is supported for CG-SDT. | Agreement Multi-layer PUSCH transmission is not supported for CG-SDT. Agreement Only single antenna port for single layer transmission is supported for CG-SDT -srs-ResourceIndicator in ConfiguredGrantConfig is not applicable to CG-SDT. -precodingAndNumberOfLayers in ConfiguredGrantConfig is always 1 for CG-SDT. |
| 14 | srs-ResourceIndicator | It is not applicable for CG-SDT. | Agreement Only single antenna port for single layer transmission is supported for CG-SDT -srs-ResourceIndicator in ConfiguredGrantConfig is not applicable to CG-SDT. -precodingAndNumberOfLayers in ConfiguredGrantConfig is always 1 for CG-SDT. |
| 16 | pusch-RepTypeIndicator-r16 | Indicates whether UE follows the behavior for PUSCH repetition type A or the behavior for PUSCH repetition type B for each Type 1 configured grant configuration. The value pusch-RepTypeA enables the 'PUSCH repetition type A' and the value pusch-RepTypeB enables the 'PUSCH repetition type B' . | Agreement For CG-SDT, RAN1 cannot reach consensus on whether to support repetition or not, it’s up to RAN2 to decide on it. |
| 17 | frequencyHoppingPUSCH-RepTypeB-r16 | Indicates the frequency hopping scheme for Type 1 CG when pusch-RepTypeIndicator is set to 'pusch-RepTypeB' . The value interRepetition enables 'Inter-repetition frequency hopping', and the value interSlot enables 'Inter-slot frequency hopping'. If the field is absent, the frequency hopping is not enabled for Type 1 CG. | Agreement For CG-SDT, RAN1 cannot reach consensus on whether to support repetition or not, it’s up to RAN2 to decide on it. |
| 24 | uci-OnPUSCH | Selection between and configuration of dynamic and semi-static beta-offset. For Type 1 UL data transmission without grant, uci-OnPUSCH should be set to semiStatic. | Agreement It’s up to RAN2 to decide on whether to support uci-OnPUSCH for CG-SDT. phy-PriorityIndex-r16 in ConfiguredGrantConfig is not applicable to CG-SDT. |
| 33 | dmrs-SeqInitialization | The network configures this field if transformPrecoder is disabled. Otherwise the field is absent. It’s present when single DMRS sequence is configured for CG-SDT. | Agreement Introduce a new parameter e.g. sdt-DMRSports to configure the set of DMRS ports for SSB to PUSCH mapping -Parameter antennaPort is not applicable to CG-SDT Support up to 2 DMRS sequences for CG-SDT for CP-OFDM, the generation mechanism and configuration can reuse that of msgA PUSCH. -Introduce a new parameter sdt-NrofDMRS-Sequences to configure 1 or 2 DMRS sequences. -The description of parameter dmrs-SeqInitialization can be revised as “ It’s present when single DMRS sequence is configured for CG-SDT”. |
| 37 | phy-PriorityIndex-r16 | It is not applicable for CG-SDT. | Agreement It’s up to RAN2 to decide on whether to support uci-OnPUSCH for CG-SDT. phy-PriorityIndex-r16 in ConfiguredGrantConfig is not applicable to CG-SDT. |
| 65 | sdt-DMRSports-r17 | Configuration of DMRS ports for SSB to CG PUSCH mapping for CG-SDT. | Agreement Introduce a new parameter e.g. sdt-DMRSports to configure the set of DMRS ports for SSB to PUSCH mapping -Parameter antennaPort is not applicable to CG-SDT Support up to 2 DMRS sequences for CG-SDT for CP-OFDM, the generation mechanism and configuration can reuse that of msgA PUSCH. -Introduce a new parameter sdt-NrofDMRS-Sequences to configure 1 or 2 DMRS sequences. -The description of parameter dmrs-SeqInitialization can be revised as “ It’s present when single DMRS sequence is configured for CG-SDT”. |
| 66 | sdt-NrofDMRS-Sequences-r17 | Configuration of number of DMRS sequences for SSB to CG PUSCH mapping for CG-SDT. | Agreement Introduce a new parameter e.g. sdt-DMRSports to configure the set of DMRS ports for SSB to PUSCH mapping -Parameter antennaPort is not applicable to CG-SDT Support up to 2 DMRS sequences for CG-SDT for CP-OFDM, the generation mechanism and configuration can reuse that of msgA PUSCH. -Introduce a new parameter sdt-NrofDMRS-Sequences to configure 1 or 2 DMRS sequences. -The description of parameter dmrs-SeqInitialization can be revised as “ It’s present when single DMRS sequence is configured for CG-SDT”. |

Comment

If you have any comments on these unstable parameters, please refer to section 2 in summary of [108-e-R17-SDT-01]. When agreements are made in that email thread, the corresponding changes will be reflected in excel sheet and this document, please companies comment in this section if they have different understanding.

| Company | View |
| --- | --- |
| Intel | - Row 10, 11: RAN1 or RAN2 needs to first conclude whether repetition is supported for CG-PUSCH. - Row 12: we suggest to introduce a new parameter to indicate a set of antenna ports for CG-PUSCH. - Row 13: precodingAndNumberOfLayers is not needed. - Row 14: this is not needed. CG-PUSCH transmission is based on SSB-to-PUSCH mapping rule - Row 16 and 17: we do not think repetition type B should be supported for CG-SDT. These two parameters are not needed. - Row 24: we do not think we need to support UCI multiplexing on CG-PUSCH. The small data transmission during CG-SDT is expected not frequent. - Row 33: Suggest to remove this parameter dmrs-SeqInitialization. Add a new parameter similar to MsgA PUSCH, i.e., nrofDMRS-Sequences. - Row 37: we do not think priority indication should be supported for CG-PUSCH during CG-SDT. This parameter is not needed. |
| Qualcomm | - Rows 10, 11,16, 17 In NR R15/16, repK and repK-RV were introduced to improve the reliability/latency performance of CG transmissions in connected mode. FFS if they are needed for R17 SDT. - Row 12 If a SDT UE supports multiple CG-SDT configurations, multiple DMRS sequences/antenna ports can be provided to UE in RRCRelease message. Whether or not to re-use the configurations in connected mode can be further discussed. - Rows 13-14 can be removed - Row 24 can be supported - Row 33 can be supported - We don’t see a strong motivation to support Row 37, but ok to further discuss it when both SRB and DRB are considered in SDT |
| LG Electronics | - Row 13 and 14: The parameters are not needed. - Row 24 and 37: The parameters seem not essential for Rel-17 SDT. |
| Moderator | - Based on the comments received so far in thread [108-e-R17-SDT-01] and [108-e-R17-RRC-SDT], the RRC parameter excel sheet is updated to v01. The changes based on latest proposals are reflected in the table above as well, please companies check if there is any misalignment between proposals and RRC parameters. The cited proposals and corresponding changes on parameters will be updated based on whether they can be agreed or not. - Please also note that there are 2 new parameters in row 65 and 66, the detailed information e.g. value range can be found in the excel sheet. |
| Moderator | - Based on the agreements we made in thread [108-e-R17-SDT-01] , the RRC parameter excel sheet is updated to v02. The changes include: - In Column T, the status of these parameters are changed to “New stable”. - In Column P, the intermediate proposals are replaced by the final agreements. - In Column G, the square brackets of parameter names are removed. - In Row 7, the value range of mapping ratio {1/8, 1/4, 1/2} is added. - This v02 will be forwarded to Sorour for final check in that email thread, if companies have further questions or modifications, please directly comment in thread [108-e-R17-RRC]. |

Others

Any other comments?

| Company | View |
| --- | --- |

Summary

The final proposals will be added later.

References

- R1-2112976 Consolidated higher layers parameter list for Rel-17 NR	RAN1, Ericsson

- R1-2112977 LS on updated Rel-17 LTE and NR higher-layers parameter list         RAN1, Ericsson