3GPP TSG RAN WG1 #108-e		                                   					       												                 R1-2202856

e-Meeting, February 21st – March 3rd, 2022

Agenda Item:	8.16.14

Source:	Moderator (AT&T)

Title:	Summary of UE features for IoT over NTN

Document for:	Discussion/Decision

# Introduction

This document presents the summary of email discussion/approval [108-e-R17-UE-features-IoT-NTN-01] during RAN1 #108-e. According to the Chairman’s Notes:

[108-e-R17-UE-features-IoT-NTN-01] Email discussion on UE features for IoT over NTN – Ralf (AT&T) - 1st check point: February 25 - Final check point: March 3

The following was discussed and/or agreed during RAN1 #108-e within the scope of [108-e-R17-UE-features-IoT-NTN-01]. All proposals are based on the latest RAN1 UE features list for Rel-17 NR in [1].

# Summary of Contributions Submitted to RAN1 #108-e

The following is the moderator’s summary of contributions submitted to RAN1 #108-e in this agenda item.

| 2. LTE_NBIOT_eMTC_NTN | 2-1 | Basic IoT over NTN support | 1. UE derives its position based on its GNSS measurements [1-1. UE report the validity duration of GNSS] 2. Receive serving satellite ephemeris in either state vector format or orbital element format 4. UE specific TA calculation in RRC_IDLE and RRC_CONNECTED state based on its GNSS-acquired position and the serving satellite ephemeris 6. UE applies common TA in RRC_IDLE and RRC_CONNECTED according to the parameters provided by the network (UE considers common TA as 0 if the parameter is not provided) [7. For TA update in RRC_CONNECTED state, combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops] 8. In RRC_IDLE and RRC_CONNECTED state, UE calculates frequency pre-compensation to counter shift the Doppler experienced on the service link [in DL] [and] [in UL] 10. Support a validity timer of UL synchronization is configured by the network 13. UE applies cell specific K_offset in timing relationship enhancements 14.UE estimates UE-gNB RTT [15. delaying the starts of ra-ResponseWindow] [16. UE receives cell specific K_mac] 17. In RRC_IDLE state and RRC_CONNECTED state, pre-compensate the calculated frequency offset and TA in uplink transmissions |  | Yes | N/A | Release 17 UE cannot access [NTN/satellite] | [per UE/per band] | No | No | FFS: whether this feature group needs to be separate for eMTC and NB-IoT | Optional with capability signalling For UEs supporting NB-IoT/eMTC NTN, it must indicate this FG is supported [Note: This UE feature group is applicable only for IoT-NTN cell, for terrestrial cell this feature is not supported] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | Separate UE feature groups should be defined for eMTC and NB-IoT, which is already the case for terrestrial network. Proposal 1: Define separate UE feature groups for NB-IoT and eMTC over NTN. - On component 1-1: Remove [1-1. UE report the validity duration of GNSS] since there is no further RAN2 support of this. - On component 6: Change “UE applies” to “UE calculates” since it focus on calculation of common TA (corresponding to UE-specific TA calculation in component 4). - On component 7: Remove the bracket to align with NR NTN UE feature description. - On component 15: Remove the bracket and change it to “Support of estimating UE-gNB RTT and delaying the starts of ra-ResponseWindow by UE-gNB RTT” to align with NR NTN UE feature description. - On component 16: Remove the bracket and change to “Delay the UE action and assumption on a downlink configuration carried by MAC CE command by K_mac if it is indicated” to align with NR NTN UE feature description. - This FG is a per UE capability. 2. LTE_NBIOT_eMTC_NTN; 2-1; Basic IoT over NTN support; 1. UE derives its position based on its GNSS measurements [1-1. UE report the validity duration of GNSS] 2. Receive serving satellite ephemeris in either state vector format or orbital element format 4. UE specific TA calculation in RRC_IDLE and RRC_CONNECTED state based on its GNSS-acquired position and the serving satellite ephemeris. 6. UE applies calculates common TA in RRC_IDLE and RRC_CONNECTED according to the parameters provided by the network (UE considers common TA as 0 if the parameter is not provided) [7. For TA update in RRC_CONNECTED state, combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops] 10. Support a validity timer of UL synchronization is configured by the network 12. UE receives cell specific K_offset 13. UE applies K_offset in timing relationship enhancements 14.UE estimates UE-gNB RTT [15. Support of estimating UE-gNB RTT and delaying the starts of ra-ResponseWindow by UE-gNB RTT ] [16. Delay the UE action and assumption on a downlink configuration carried by MAC CE command by K_mac if it is indicated UE receives cell specific K_mac] 17. In RRC_IDLE state and RRC_CONNECTED state, pre-compensate the calculated frequency offset and TA in uplink transmissions; Yes; N/A; Release 17 UE cannot access [NTN/satellite]; [Per UE/per band]; No; No; FFS: whether tThis feature group needs to be separate for eMTC and NB-IoT; Optional with capability signalling For UEs supporting NB-IoT/eMTC NTN, it must indicate this FG is supported [Note: This UE feature group is applicable only for IoT-NTN cell, for terrestrial cell this feature is not supported]. |
| MediaTek Inc. [3] | - Component 1-1 can be kept without yellow highlight considering agreemens in RAN1 107e [2]. Agreement The UE autonomously determines its GNSS validity duration X and reports information associated with this valid duration to the network via RRC signalling. - X = {10s, 20s, 30s, 40s, 50s, 60s, 5 min, 10 min, 15 min, 20 min, 25 min, 30 min, 60 min, 90 min, 120 min, infinity}. - Component 7 can be kept without yellow highlight considering taking NR NTN UE features [3] as baseline. - Component 8 should delete the description of “[in DL] [and] [in UL]”. - Component 14 and Component 15 should be merged to “Start of ra-ResponseWindow is delayed by an offset. The RA-window start offset is defined as sum (current offset, UE-eNB RTT) and current offset is defined in TS36.321 (FFS if applicable to NB-IoT 41ms offset)”, in order to align with RAN2 agreements [4] on start of RAR window. Agreement(RAN2 115e) Start of ra-ResponseWindow is delayed by an offset. Postpone discussion on the offset value until further agreements regarding RACH are made in RAN1. Agreement(RAN2 116e) The ra window start offset is defined as sum (current offset, UE-eNB RTT) and current offset is defined in TS36.321 (FFS if applicable to NB-IoT 41ms offset). - Component 16 can be kept without yellow highlight considering agreemens in RAN1 agreements. Agreement For IoT NTN, the information of K_mac is carried in system information.. 2. LTE_NBIOT_eMTC_NTN; 2-1; Basic IoT over NTN support; 1. UE derives its position based on its GNSS implementation measurements 1-1. UE report the validity duration of GNSS 2. [Indicate/receive] serving Satellite position state vector X,Y,Z in ECEF (m) and serving Satellite velocity state vector VX,VY,VZ in ECEF (m/s) [2-1. Derive epoch time of serving satellite ephemeris from implicit epoch time] 3. Receive ephemeris orbital parameter for the serving satellite 2. Receive serving satellite ephemeris in either state vector format or orbital element format 4. UE specific TA calculation in RRC_IDLE and RRC_CONNECTED state based on its GNSS-acquired position and the serving satellite ephemeris. 5. UE specific TA calculation [in [RRC_IDLE and RRC_CONNECTED state] based on its GNSS-acquired position and the serving satellite ephemeris. 6. UE applies common TA [in RRC_IDLE and RRC_CONNECTED] according to the parameters provided by the network (UE considers common TA as 0 if the parameter is not provided) (if any) 7. For TA update in RRC_CONNECTED state, combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops X. [Common TA estimation is based on indication of common TA drift rate] 8. In RRC_IDLE state calculate frequency pre-compensation to counter shift the Doppler experienced on the service link. 9. in RRC_CONNECTED state, calculate frequency pre-compensation to counter shift the Doppler experienced on the service link. 8. In RRC_IDLE and RRC_CONNECTED state, UE calculates frequency pre-compensation to counter shift the Doppler experienced on the service link [in DL] [and] [in UL] 10. Support a validity timer of UL synchronization is configured by the network 11. Support segment duration configuration is by the network [and updating time and frequency pre-compensation between segments but not within segments during UL repetition] 12. UE receives cell specific K_offset 13. UE applies cell specific K_offset in timing relationship enhancements 14. Start of ra-ResponseWindow is delayed by an offset. The RA-window start offset is defined as sum (current offset, UE-eNB RTT) and current offset is defined in TS36.321 (FFS if applicable to NB-IoT 41ms offset). UE estimates of UE-gNB RTT [15. delaying the starts of ra-ResponseWindow] 16. UE receives cell specific K_mac 17. In RRC_IDLE state and RRC_CONNECTED state, pre-compensate the calculated frequency offset and TA in uplink transmissions; [No] Yes; N/A; Release 17 UE cannot access [NTN/satellite]; [Per UE/per band]; No; No; FFS: whether this feature group needs to be separate for eMTC and NB-IoT FFS: Serving satellite ephemeris Epoch time is implicitly known as a reference time defined by the starting time of a DL slot and/or frame. FFS: Whether this starting time is given by predefined rule or it is indicated by the network FFS: differentiation based on orbits such as LEO/MEO/GEO; Optional with capability signalling For UEs supporting NB-IoT/eMTC NTN, it must indicate this FG is supported [Note: This UE feature group is applicable only for IoT-NTN cell, for terrestrial cell this feature is not supported]. |
| OPPO [4] | In RAN1#107-e meeting, FG 2-1 was discussed as a basic feature for NTN-IOT, where the highlighted in yellow 7 is a bit strange to make it as a basic UE feature for NTN, as the combination of open loop and close loop does not introduce additional UE behavior to open loop TA control. Thus, it would be more reasonable to only mention open loop. But we suggest to replace open loop TA control with the text aligned with NR-NTN, I.e. - UE specific TA calculation based on its GNSS-acquired position and the serving satellite ephemeris - UE applies common TA according to the parameters provided by the network. Proposal 1: for FG2-1 component 7, replace the yellow highlighted text with UE specific TA calculation based on its GNSS-acquired position and the serving satellite ephemeris and UE applies common TA according to the parameters provided by the network. For the component 13. UE applies cell specific K_offset in timing relationship enhancements The basic feature should be applied only for Msg3 transmission and PUCCH transmission. While for the rest of the uplink transmission, the UE capability with signaling should be allowed. Thus, we suggest that in the FG2-1 component 13 only takes UE applies cell specific K offset in timing relationship enhancement for PUCCH and PUSCH transmission. Then introduce a new FG2-1a for other uplink transmission where UE applies cell specific K offset, for which UE may report its capability. Proposal 2: for FG2-1 component 13, only keep that UE applies cell specific K offset in timing relationship enhancement for PUCCH and PUSCH transmission Proposal 3: introduce FG2-1a for other uplink transmission than PUCCH and PUSCH where UE applies cell specific K offset, for which UE may report its capability. |
| Nokia, Nokia Shanghai Bell [5] | - ”Need for the eNB to know if the feature is supported” should be ”yes” - Notes for all FGs can be removed, they are not essential information for specifications - Remove components 12 and 13 as they are duplicated in FG 2-2. Confirm component 16 on K_mac support (see related agreements below). Agreement: For NB-IoT, if the UE has initiated an NPUSCH transmission using pre-configured uplink resources ending in subframe n, the UE shall start or restart to monitor the NPDCCH from DL subframe n+4+K_mac (where K_mac is defined as in NR-NTN). Agreement: For eMTC, if the UE has initiated an PUSCH transmission using pre-configured uplink resources ending in subframe n, the UE shall start or restart to monitor the MPDCCH from DL subframe n+4+K_mac (where K_mac is defined as in NR-NTN). |
| Intel Corporation [6] | - For Component 1-1 in FG 2-1, brackets can be removed since indication of validity duration was agreed in RAN1. - Word Satellite can be removed since it is already clear that operation with satellite is assumed for NTN. - UE capability for NTN can be indicated per band to allow more flexibility for UE implementation. - Separate signaling of UE capabilities for eMTC and NB-IoT can be used to allow more flexibility for UE implementation. |
| Apple [7] | In UE feature 2-1, components 15 and 16 are open. We think these two components belong to the feature “Basic IoT over NTN support”. Actually, these two components are closely correlated and can be merged to a single component “UE applies K_mac in delaying the starts of ra-ResponseWindow”. In UE feature 2-1, the component 7 is open. We think this component belongs to the feature 2-1, since it was agreed that UE in IoT NTN needs to support both open and closed loop TA control. Proposal 1: In feature 2-1, - conform component 7 “For TA update in RRC_CONNECTED state, combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops”. - merge components 15 and 16 to “UE applies K_mac in delaying the starts of ra-ResponseWindow”. It is open the granularity of the features 2-1, 2-1a, 2-2 and 2-3. We think these features only apply on the band for NTN operations. Hence, it is natural these features are defined per band. Proposal 6: The features 2-1, 2-1a, 2-2 and 2-3 are defined per band. |
| Ericsson Hungary Ltd [8] | 2. LTE_NBIOT_eMTC_NTN; 2-1; Basic IoT over NTN support; 1. UE derives its position based on its GNSS measurements 1-1. UE reports the validity duration of GNSS 2. UE receives serving satellite ephemeris in either state vector format or orbital element format 4. UE calculates UE specific TA in RRC_IDLE and RRC_CONNECTED state based on its GNSS-acquired position and the serving satellite ephemeris 6. UE calculates common TA in RRC_IDLE and RRC_CONNECTED according to the parameters provided by the network (UE considers common TA as 0 if the parameter is not provided) 7. For TA update in RRC_CONNECTED state, UE uses a combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops 8. In RRC_IDLE and RRC_CONNECTED state, UE calculates frequency pre-compensation to counter shift the Doppler experienced on the service link in DL and in UL 10. UE supports a validity timer of UL synchronization that is configured by the network 13. UE applies cell specific K_offset in timing relationship enhancements 14.UE estimates UE-gNB RTT 15. UE delays the starts of ra-ResponseWindow 16. UE receives cell specific K_mac 17. In RRC_IDLE state and RRC_CONNECTED state, UE pre-compensates the calculated frequency offset and TA in uplink transmissions; Yes; N/A; Release 17 UE cannot access [NTN/satellite]; [per UE/per band]; No; No; FFS: whether this feature group needs to be separate for eMTC and NB-IoT; Optional with capability signalling For UEs supporting NB-IoT/eMTC NTN, it must indicate this FG is supported [Note: This UE feature group is applicable only for IoT-NTN cell, for terrestrial cell this feature is not supported]. |
| Qualcomm Incorporated [9] | Currently, the FG 2-1 corresponding to “Basic IoT over NTN support” mentions the following as a component: - “…6. UE applies common TA [in RRC_IDLE and RRC_CONNECTED] according to the parameters provided by the network (UE considers common TA as 0 if the parameter is not provided) (if any) …” The common TA parameters in general include the parameters TACommon, TACommonDrift and TACommonDriftVariation, with the latter two indicating adjustments related to the common TA drift. However, in GEO NTNs (which is the intended base case corresponding to FG 2-1), the drift/variation in the common TA is negligible. To this end, for simplified operation over GEO NTNs, we propose to modify the requirements on applying common TA to only include the common TA (i.e., the term TACommon), and not the drift/variation parameters. Proposal 1: For GEO NTNs, a UE applies common TA only in accordance with the common TA term, TACommon, and is not required to process the drift/variation parameters TACommonDrift and TACommonDriftVariation. Proposal 2: A UE’s support of NTN should be a “per band” capability. NB-IoT and eMTC capabilities are captured separately by RAN2. While for the feature list discussion, it may be OK to discuss these together, eventually, these need to be captured separately for eMTC and NB-IoT. Proposal 3: Capture IoT-NTN capabilities separately for eMTC and NB-IoT For the three capabilities (2-1, 2-2 and 2-3), the “Mandatory/Optional” column contains the following text: “For UEs supporting NB-IoT/eMTC NTN, it must indicate this FG is supported” For 2-1, this text is anyway true, and is not adding anything new. We therefore propose that this text be removed from 2-1. For 2-2 and 2-3, however, given that these are features dependent on the mother feature (2-1), a UE supporting 2-1 and NOT supporting 2-2 and/or 2-3 can also communicate with an NTN cell. Additionally, if it possible that initial NTN IOT deployments do not implement features 2-2 and 2-3, and thus there would be a lack of IODT opportunities to test these features. As a result, we propose to remove the quoted text from 2-2 and 2-3. Proposal 4: Remove the text “For UEs supporting NB-IoT/eMTC NTN, it must indicate this FG is supported” from the “Mandatory/Optional” column for FG 2-1, 2-2 and 2-3. |
| ZTE [10] | - For component 7, it’s preferred to remove the bracket [7. For TA update in RRC_CONNECTED state, combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops ]. Because according to the agreement in RAN1#106e[2],combination of both open and closed control loops are supported in NR-NTN and the relevant agreements from NR NTN are re-used for IoT NTN as working assumption. - For component8, the bracket for “[in DL] [and] [in UL]” can be removed. According to the agreement in RAN1#107e[3], the pre-compensation for the DL Doppler experienced in service link is not supported in NR-NTN. In this case, the correction of experienced Doppler should include the impacts for both DL and UL. - For components 10, an additional UE feature, i.e. A UE starts/restarts the validity timer at the epoch time of the assistance information, can be added to capture the agreements in RAN1#106bis [4]. - For component 15, it’s preferred to remove the bracket [15.delaying the starts of ra-ResponseWindow] to capture the IoT-NTN related agreements “start of ra-ResponseWindow is delayed by an offset” in RAN2#115e[5]. Since the legacy ra-ResponseWindow range may not be enough to cover different UEs in the IoT-NTN, the UE may determine the starts of ra-ResponseWindow with an offset after Msg 1 transmission, where the value of offset varies based on whether the UE can have access to the situation information(e.g.,location of the UE, an ephemeris of a satellite or a trajectory of a HAPS used by a BS, and a payload type of the satellite or the HAPS used by the BS). - For FG 2-1, it’s preferred to add an additional component 17, i.e. UE applies cell specific K_mac in timing relationship enhancements, since the cell specific K_mac should also be applied in timing relationship enhancement similar as the cell specific K_offset. Besides, the sequence number of legacy component 17 should be correspondingly updated as component 18. 2. LTE_NBIOT_eMTC_NTN; 2-1; Basic IoT over NTN support; [7. For TA update in RRC_CONNECTED state, combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops] 8.In RRC_IDLE and RRC_CONNECTED state, UE calculates frequency pre-compensation to counter shift the Doppler experienced on the service link [in DL] [and] [in UL] 10. Support a validity timer of UL synchronization is configured by the network. A UE starts/restarts the validity timer at the epoch time of the assistance information. [15.delaying the starts of ra-ResponseWindow] - UE receives cell specific K_mac] - UE applies cell specific K_mac in timing relationship enhancements 17.18. In RRC_IDLE state and RRC_CONNECTED state, pre-compensate the calculated frequency offset and TA in uplink transmissions; Yes; Release 17 UE cannot access [NTN/satellite]; [per UE/per band]; No; No; FFS: whether this feature group needs to be separate for eMTC and NB-IoT; Optional with capability signalling For UEs supporting NB-IoT/eMTC NTN, it must indicate this FG is supported [Note: This UE feature group is applicable only for IoT-NTN cell, for terrestrial cell this feature is not supported]. |

| 2. LTE_NBIOT_eMTC_NTN | 2-1a | Segment duration configuration | Support segment duration configuration by the network |  | Yes | N/A | Release 17 UE cannot access [NTN/satellite] | [Per UE/per band] | No | No | For UEs support NSGO scenarios, it must indicate this FG is supported. | Optional with capability signalling [Note: This UE feature group is applicable only for IoT-NTN cell, for terrestrial cell this feature is not supported] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | - The “Prerequisite feature groups” for FG2-1a should be FG2-1 - This FG is a per UE capability. 2. LTE_NBIOT_eMTC_NTN; 2-1a; Segment duration configuration; Support segment duration configuration by the network; 2-1; Yes; N/A; Release 17 UE cannot access [NTN/satellite]; [Per UE/per band]; No; No; For UEs support NSGO scenarios, it must indicate this FG is supported.; Optional with capability signalling [Note: This UE feature group is applicable only for IoT-NTN cell, for terrestrial cell this feature is not supported]. |
| MediaTek Inc. [3] | - The component can be change to “Support segment duration configuration by the network for NGSO”, considering only agreements for NGSO have been made. Agreement UL Segmented transmission NPRACH/NPUSCH for NB-IoT is not supported in GEO based on UE feature.. 2. LTE_NBIOT_eMTC_NTN; 2-1a; Basic IoT over NTN support for NSGO scenarios Segment duration configuration; 1. In RRC_IDLE and RRC_CONNECTED state, UE calculates frequency pre-compensation to counter shift the Doppler experienced on the service link 2. Support segment duration configuration by the network for NGSO.; Yes; N/A; Release 17 UE cannot access [NTN/satellite]; [Per UE/per band]; No; No; For UEs support NSGO scenarios, it must indicate this FG is supported.; Optional with capability signalling For UEs supporting NB-IoT/eMTC NTN, it must indicate this FG is supported [Note: This UE feature group is applicable only for IoT-NTN cell, for terrestrial cell this feature is not supported]. |
| OPPO [4] |  |
| Nokia, Nokia Shanghai Bell [5] | - Notes for all FGs can be removed, they are not essential information for specifications |
| Intel Corporation [6] | - Word Satellite can be removed since it is already clear that operation with satellite is assumed for NTN. - UE capability for NTN can be indicated per band to allow more flexibility for UE implementation. |
| Apple [7] | A new UE feature 2-1a “Segment duration configuration” was introduced. This feature is about uplink transmission segment for long uplink transmission. Since all the uplink transmissions have to pre-compensate frequency offset and TA, the pre-requisite of feature 2-1a is feature 2-1. Proposal 2: The pre-requisite of feature 2-1a is feature 2-1. It is open the granularity of the features 2-1, 2-1a, 2-2 and 2-3. We think these features only apply on the band for NTN operations. Hence, it is natural these features are defined per band. Proposal 6: The features 2-1, 2-1a, 2-2 and 2-3 are defined per band. |
| Ericsson Hungary Ltd [8] | 2. LTE_NBIOT_eMTC_NTN; 2-1a; Segmented UL transmission; UE applies segmented uplink transmission according to duration configuration by the network; 2-1; Yes; N/A; Release 17 UE cannot access [NTN/satellite]; [Per UE/per band]; No; No; For UEs support NSGO scenarios, it must indicate this FG is supported.; Optional with capability signalling [Note: This UE feature group is applicable only for IoT-NTN cell, for terrestrial cell this feature is not supported]. |
| Qualcomm Incorporated [9] | Proposal 2: A UE’s support of NTN should be a “per band” capability. |
| ZTE [10] | W.r.t the FG 2-1a, an additional UE feature “ The same value is used for segment durations for all PRACH preambles.” can be added according to the agreements in RAN1#106bis [4]. Moreover, according to the agreements in RAN1#107-e[3], UE can support the segment pre-compensation using two methods by UE implementation. In order to capture the relevant agreement[3], an additional component 2 should be added in FG2-1a to illustrate how to achieve the segmented pre-compensation. And the name of FG 2-1a can be revised as “segment pre-compensation”. 2. LTE_NBIOT_eMTC_NTN; 2-1a; Segment duration configuration pre-compensation; 1.Support segment duration configuration by the network The same value is used for segment durations for all PRACH preambles. 2.Support segment pre-compensation by using one of followings methods: 1)UE can drop/insert samples and puncture OFDM symbols 2)UE can blank subframes, slots, and repetition units; Yes; Release 17 UE cannot access [NTN/satellite]; [Per UE/per band]; No; No; For UEs support NSGO scenarios, it must indicate this FG is supported.; Optional with capability signalling [Note: This UE feature group is applicable only for IoT-NTN cell, for terrestrial cell this feature is not supported]. |

| 2. LTE_NBIOT_eMTC_NTN | 2-2 | Enhancing timing relationships using a time offset | UE applies UE specific K_offset in timing relationship enhancements | 2-1 [, 2-3] | Yes | N/A | UE does not know the offset to apply for UL transmission | [per UE/per band] | No | No | The K_offset is a scheduling offset used for the identified timing relationships that need to be modified for IoT NTN. For IoT NTN, support cell-specific Koffset configuration for use during initial access. For IoT NTN, support the use of UE-specific Koffset in CONNECTED mode. FFS: whether this feature group needs to be separate for eMTC and NB-IoT FFS: differentiation based on orbits such as LEO/MEO/GEO | Optional with capability signalling FFS: For UEs supporting NB-IoT/eMTC NTN, it must indicate this FG is supported |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | Separate UE feature groups should be defined for eMTC and NB-IoT, which is already the case for terrestrial network. Proposal 1: Define separate UE feature groups for NB-IoT and eMTC over NTN. - The “Prerequisite feature groups” for FG2-2 should be FG2-1 and FG2-3 - Remove “FFS: differentiation based on orbits such as LEO/MEO/GEO” since there is no need to differentiate scenarios - Remove “FFS: For UEs supporting NB-IoT/eMTC NTN, it must indicate this FG is supported” since it is not a basic UE feature for IoT-NTN - This FG is a per UE capability. 2. LTE_NBIOT_eMTC_NTN; 2-2; Enhancing timing relationships using a time offset; UE applies UE specific K_offset in timing relationship enhancements; 2-1 [, 2-3]; Yes; N/A; UE does not know the offset to apply for UL transmission; [Per UE/per band]; No; No; The K_offset is a scheduling offset used for the identified timing relationships that need to be modified for IoT NTN. For IoT NTN, support cell-specific Koffset configuration for use during initial access. For IoT NTN, support the use of UE-specific Koffset in CONNECTED mode. FFS: whether this feature group needs to be separate for eMTC and NB-IoT FFS: differentiation based on orbits such as LEO/MEO/GEO; Optional with capability signalling FFS: For UEs supporting NB-IoT/eMTC NTN, it must indicate this FG is supported. |
| MediaTek Inc. [3] |  |
| OPPO [4] |  |
| Nokia, Nokia Shanghai Bell [5] | - Per UE, for UEs supporting IOT over NTN - In principle a single FG for NB-IOT and eMTC is enough. - Notes for all FGs can be removed, they are not essential information for specifications |
| Intel Corporation [6] | - For FG 2-2 and FG 2-3, differentiation based on orbits such as LEO/MEO/GEO is not needed since the corresponding UE feature may be supported or not for a band without considering particular deployment characteristics. Also, in our view the corresponding FGs should be supported for all UEs supporting NB-IoT/eMTC NTN since UE-specific slot offset and reporting of UE’s TA are important features to enable half-duplex FDD devices. - Word Satellite can be removed since it is already clear that operation with satellite is assumed for NTN. - UE capability for NTN can be indicated per band to allow more flexibility for UE implementation. - Separate signaling of UE capabilities for eMTC and NB-IoT can be used to allow more flexibility for UE implementation. |
| Apple [7] | The feature 2-2 is that UE applies UE specific K_offset in timing relationship enhancements. The UE specific K_offset is received only after UE reports its TA value. Hence, the pre-requisite of feature 2-2 includes feature 2-3. Proposal 3: The pre-requisite of feature 2-2 includes feature 2-3. The UE’s TA reporting and the application of UE specific K_offset in timing relationship enhancements are not basic feature group of IoT over NTN. If a UE does not report TA and does not apply UE specific K_offset in timing relationship enhancements, the UE can still use cell specific K_offset in timing relationship enhancements. Proposal 5: The features 2-2 and 2-3 are not the basic feature groups for IoT over NTN. - Remove “FFS: For UEs supporting NB-IoT/eMTC NTN, it must indicate this FG is supported” from the notes of these two features. It is open the granularity of the features 2-1, 2-1a, 2-2 and 2-3. We think these features only apply on the band for NTN operations. Hence, it is natural these features are defined per band. Proposal 6: The features 2-1, 2-1a, 2-2 and 2-3 are defined per band. |
| Ericsson Hungary Ltd [8] | 2. LTE_NBIOT_eMTC_NTN; 2-2; Enhancing timing relationships using a time offset; UE applies UE specific K_offset in timing relationship enhancements; 2-1, 2-3; Yes; N/A; UE does not know the offset to apply for UL transmission; [per UE/per band]; No; No; The K_offset is a scheduling offset used for the identified timing relationships that need to be modified for IoT NTN. For IoT NTN, support cell-specific Koffset configuration for use during initial access. For IoT NTN, support the use of UE-specific Koffset in CONNECTED mode. FFS: whether this feature group needs to be separate for eMTC and NB-IoT FFS: differentiation based on orbits such as LEO/MEO/GEO; Optional with capability signalling FFS: For UEs supporting NB-IoT/eMTC NTN, it must indicate this FG is supported. |
| Qualcomm Incorporated [9] | Proposal 2: A UE’s support of NTN should be a “per band” capability. NB-IoT and eMTC capabilities are captured separately by RAN2. While for the feature list discussion, it may be OK to discuss these together, eventually, these need to be captured separately for eMTC and NB-IoT. Proposal 3: Capture IoT-NTN capabilities separately for eMTC and NB-IoT For the three capabilities (2-1, 2-2 and 2-3), the “Mandatory/Optional” column contains the following text: “For UEs supporting NB-IoT/eMTC NTN, it must indicate this FG is supported” For 2-1, this text is anyway true, and is not adding anything new. We therefore propose that this text be removed from 2-1. For 2-2 and 2-3, however, given that these are features dependent on the mother feature (2-1), a UE supporting 2-1 and NOT supporting 2-2 and/or 2-3 can also communicate with an NTN cell. Additionally, if it possible that initial NTN IOT deployments do not implement features 2-2 and 2-3, and thus there would be a lack of IODT opportunities to test these features. As a result, we propose to remove the quoted text from 2-2 and 2-3. Proposal 4: Remove the text “For UEs supporting NB-IoT/eMTC NTN, it must indicate this FG is supported” from the “Mandatory/Optional” column for FG 2-1, 2-2 and 2-3. |
| ZTE [10] | W.r.t the FG 2-2, it’s preferred to keep the component “UE receives UE specific K_offset” since the action of UE receiving is needed in the specification. 2. LTE_NBIOT_eMTC_NTN; 2-2; Enhancing timing relationships using a time offset; 1.UE receives UE specific K_offset 2.UE applies UE specific K_offset in timing relationship enhancements; 2-1 [, 2-3]; Yes; UE does not know the offset to apply for UL transmission; [per UE/per band]; No; No; The K_offset is a scheduling offset used for the identified timing relationships that need to be modified for IoT NTN. For IoT NTN, support cell-specific Koffset configuration for use during initial access. For IoT NTN, support the use of UE-specific Koffset in CONNECTED mode. FFS: whether this feature group needs to be separate for eMTC and NB-IoT FFS: differentiation based on orbits such as LEO/MEO/GEO; Optional with capability signalling FFS: For UEs supporting NB-IoT/eMTC NTN, it must indicate this FG is supported. |

| 2. LTE_NBIOT_eMTC_NTN | 2-3 | UE specific TA pre-compensation reporting | [Support reporting of information about the UE specific TA pre-compensation] | 2-1 | Yes | No | UL scheduling for FDD-HD: Use of UE-specific TA and/or K_offset to avoid UL-DL collisions in FDD-HD | [per UE/per band] | No | No | UE-specific TA reporting is supported in IoT-NTN FFS: Detailed contents of report FFS: whether this feature group needs to be separate for eMTC and NB-IoT FFS: differentiation based on orbits such as LEO/MEO/GEO | Optional with capability signalling FFS: For UEs supporting NB-IoT/eMTC NTN, it must indicate this FG is supported |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | Separate UE feature groups should be defined for eMTC and NB-IoT, which is already the case for terrestrial network. Proposal 1: Define separate UE feature groups for NB-IoT and eMTC over NTN. - Remove bracket in the the component description - Remove “FFS: Detailed contents of report” since there is no need to list the exact content of UE reporting in UE feature - Remove “FFS: differentiation based on orbits such as LEO/MEO/GEO” since there is no need to differentiate scenarios - Remove “FFS: For UEs supporting NB-IoT/eMTC NTN, it must indicate this FG is supported” since it is not a basic UE feature for IoT-NTN - This FG is a per UE capability. All the other change as well as the the corresponding changes according to the above analysis are highlighted in red in the appendix A Proposal 2: Agree on the upadated UE features in the Appendix. 2. LTE_NBIOT_eMTC_NTN; 2-3; UE specific TA pre-compensation reporting; [Support reporting of information about the UE specific TA pre-compensation]; 2-1; Yes; No; UL scheduling for FDD-HD: Use of UE-specific TA and/or K_offset to avoid UL-DL collisions in FDD-HD; [Per UE/per band]; No; No; UE-specific TA reporting is supported in IoT-NTN FFS: Detailed contents of report FFS: whether this feature group needs to be separate for eMTC and NB-IoT FFS: differentiation based on orbits such as LEO/MEO/GEO; Optional with capability signalling FFS: For UEs supporting NB-IoT/eMTC NTN, it must indicate this FG is supported. |
| MediaTek Inc. [3] |  |
| OPPO [4] |  |
| Nokia, Nokia Shanghai Bell [5] | - Per UE, for UEs supporting IOT over NTN - In principle a single FG for NB-IOT and eMTC is enough, but it should be optional for eMTC UE, as it may support full-duplex as well. |
| Intel Corporation [6] | - For FG 2-2 and FG 2-3, differentiation based on orbits such as LEO/MEO/GEO is not needed since the corresponding UE feature may be supported or not for a band without considering particular deployment characteristics. Also, in our view the corresponding FGs should be supported for all UEs supporting NB-IoT/eMTC NTN since UE-specific slot offset and reporting of UE’s TA are important features to enable half-duplex FDD devices. - For FG 2-3, many aspects of UE reporting of information about the UE specific TA pre-compensation were defined in RAN2 including reporting of TA during initial access and in RRC_CONNECTED state and reporting of UE location information. Thus, in our view RAN2 can define the details on the UE capability signaling for the exact content of UE reporting of information about the UE specific TA pre-compensation. - UE capability for NTN can be indicated per band to allow more flexibility for UE implementation. - Separate signaling of UE capabilities for eMTC and NB-IoT can be used to allow more flexibility for UE implementation. |
| Apple [7] | The component of feature 2-3 is open. We think that the component “support reporting of information about the UE specific TA pre-compensation” can be confirmed. Proposal 4: In feature 2-1, confirm the component “support reporting of information about the UE specific TA pre-compensation”. The UE’s TA reporting and the application of UE specific K_offset in timing relationship enhancements are not basic feature group of IoT over NTN. If a UE does not report TA and does not apply UE specific K_offset in timing relationship enhancements, the UE can still use cell specific K_offset in timing relationship enhancements. Proposal 5: The features 2-2 and 2-3 are not the basic feature groups for IoT over NTN. - Remove “FFS: For UEs supporting NB-IoT/eMTC NTN, it must indicate this FG is supported” from the notes of these two features. It is open the granularity of the features 2-1, 2-1a, 2-2 and 2-3. We think these features only apply on the band for NTN operations. Hence, it is natural these features are defined per band. Proposal 6: The features 2-1, 2-1a, 2-2 and 2-3 are defined per band. |
| Ericsson Hungary Ltd [8] | 2. LTE_NBIOT_eMTC_NTN; 2-3; UE specific TA pre-compensation reporting; [Support reporting of information about the UE specific TA pre-compensation]; 2-1; Yes; No; UL scheduling for FDD-HD: Use of UE-specific TA and/or K_offset to avoid UL-DL collisions in FDD-HD; [per UE/per band]; No; No; UE-specific TA reporting is supported in IoT-NTN FFS: Detailed contents of report, including reporting of UE location FFS: whether this feature group needs to be separate for eMTC and NB-IoT FFS: differentiation based on orbits such as LEO/MEO/GEO; Optional with capability signalling FFS: For UEs supporting NB-IoT/eMTC NTN, it must indicate this FG is supported. |
| Qualcomm Incorporated [9] | Proposal 2: A UE’s support of NTN should be a “per band” capability. NB-IoT and eMTC capabilities are captured separately by RAN2. While for the feature list discussion, it may be OK to discuss these together, eventually, these need to be captured separately for eMTC and NB-IoT. Proposal 3: Capture IoT-NTN capabilities separately for eMTC and NB-IoT For the three capabilities (2-1, 2-2 and 2-3), the “Mandatory/Optional” column contains the following text: “For UEs supporting NB-IoT/eMTC NTN, it must indicate this FG is supported” For 2-1, this text is anyway true, and is not adding anything new. We therefore propose that this text be removed from 2-1. For 2-2 and 2-3, however, given that these are features dependent on the mother feature (2-1), a UE supporting 2-1 and NOT supporting 2-2 and/or 2-3 can also communicate with an NTN cell. Additionally, if it possible that initial NTN IOT deployments do not implement features 2-2 and 2-3, and thus there would be a lack of IODT opportunities to test these features. As a result, we propose to remove the quoted text from 2-2 and 2-3. Proposal 4: Remove the text “For UEs supporting NB-IoT/eMTC NTN, it must indicate this FG is supported” from the “Mandatory/Optional” column for FG 2-1, 2-2 and 2-3. |
| ZTE [10] | W.r.t the FG 2-3, according to the latest agreement [3], i.e. NR NTN solutions are a baseline for the following UE-specific TA handling issues, the following component similar as the NR-NTN can be added in the FG 2-3: (1)The granularity of the reported TA is a slot.(2) As for the frequency of reports, the TA is reported when the offset between current information about UE specific TA and the last successfully reported information about UE specific TA is over the threshold. 2. LTE_NBIOT_eMTC_NTN; 2-3; UE specific TA pre-compensation reporting; [Support reporting of information about the UE specific TA pre-compensation] - The granularity of the reported TA is a slot. As for the frequency of reports, the TA is reported when the offset between current information about UE specific TA and the last successfully reported information about UE specific TA is over the threshold.; 2-1; Yes; UL scheduling for FDD-HD: Use of UE-specific TA and/or K_offset to avoid UL-DL collisions in FDD-HD; [per UE/per band]; No; No; UE-specific TA reporting is supported in IoT-NTN FFS: Detailed contents of report FFS: whether this feature group needs to be separate for eMTC and NB-IoT FFS: differentiation based on orbits such as LEO/MEO/GEO; Optional with capability signalling FFS: For UEs supporting NB-IoT/eMTC NTN, it must indicate this FG is supported. |

Others

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| MediaTek Inc. [3] |  |
| OPPO [4] |  |
| Nokia, Nokia Shanghai Bell [5] |  |
| Intel Corporation [6] |  |
| Apple [7] |  |
| Ericsson Hungary Ltd [8] |  |
| Qualcomm Incorporated [9] | RAN1 should discuss how to determine the applicability of all terrestrial network (TN) features up to Rel16 for the case of NTN. This may involve a feature-by-feature determination of applicability to NTNs from the (legacy) feature list for TNs up to Rel16. A default assumption on applicability may be made, which may then be overridden on a case-by-case basis, as appropriate. Proposal 5: RAN1 to discuss the determination of applicability of Terrestrial Network (TN) features up to Release 16 to IoT-NTN in Release 17. - A default assumption may be that a legacy IoT TN feature is applicable for IoT-NTN, unless it is explicitly demonstrated that it cannot be supported/is not applicable to IoT-NTN. - For the features that are applicable to IOT-NTN, a new capability indication (separate from the TN one) is introduced. |
| ZTE [10] |  |

# Discussion/Approval Items during RAN1 #108-e — First Checkpoint

After review of contributions submitted to RAN1 #108-e in this agenda item, the following topics were identified by the moderator for discussion/approval during RAN1 #108-e.

General comments

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Ericsson | Please clarify why all the features are listed as “per band” for IoT NTN. |

# Issue 1: FG 2-1

After review of contributions submitted to RAN1 #108-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 2. LTE_NBIOT_eMTC_NTN | 2-1 | Basic IoT over NTN support | 1. UE derives its position based on its GNSS measurements [1-1. UE reports the validity duration of GNSS] 2. UE receives serving satellite ephemeris in either state vector format or orbital element format 4. UE calculates UE specific TA calculation in RRC_IDLE and RRC_CONNECTED state based on its GNSS-acquired position and the serving satellite ephemeris 6. UE applies calculates common TA in RRC_IDLE and RRC_CONNECTED according to the parameters provided by the network (UE considers common TA as 0 if the parameter is not provided) [7. For TA update in RRC_CONNECTED state, UE uses a combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops] 8. In RRC_IDLE and RRC_CONNECTED state, UE calculates frequency pre-compensation to counter shift the Doppler experienced on the service link [in DL] [and] [in UL] 10. UE supports a validity timer of UL synchronization that is configured by the network 13. UE applies cell specific K_offset in timing relationship enhancements 14.UE estimates UE-gNB RTT [15. UE estimates UE-gNB RTT and delaysing the starts of ra-ResponseWindow by UE-gNB RTT] [16. UE receives cell specific K_mac] Delay the UE action and assumption on a downlink configuration carried by MAC CE command by K_mac if it is indicated 17. In RRC_IDLE state and RRC_CONNECTED state, UE pre-compensates the calculated frequency offset and TA in uplink transmissions |  | Yes | N/A | Release 17 UE cannot access [NTN/satellite] | [per UE/per band] | No | No | FFS: whether this feature group needs to be separate for eMTC and NB-IoT | Optional with capability signalling For UEs supporting NB-IoT/eMTC NTN, it must indicate this FG is supported [Note: This UE feature group is applicable only for IoT-NTN cell, for terrestrial cell this feature is not supported] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Nokia, NSB | Align the notes and consequence if not supported to the same terminology used in NTN feature groups. Component 13 should be removed, as it is repeated in FG2-2. |
| Apple | We are fine with the proposal in general. |
| SoftBank | Agree with Nokia for same terminology used in NTN feature groups. |
| Qualcomm | For component 6 (common TA), as we pointed out, the “parameters required for processing by the UE” may be different for GEO vs non-GEO. Since 2-1 is meant to serve as somewhat of a “GEO only” feature, we would like further discussion on the “specifics” of component 6 on common TA. We do not need to use “drift rate” and “drift rate variation” parameters to calculate common TA for GEO, while we need those for LEO/MEO. Otherwise, seems OK. |
| ZTE | We are fine with the proposal. Regarding the component-6, there is no need to further define the separate set since the indication of “drift” value is already optional as implementation. |
| Intel | Agree with Nokia. Terminology should be aligned. |
| Xiaomi | On the reporting of validity duration of GNSS,we have the agreements in 107e meeting that : Agreement The UE autonomously determines its GNSS validity duration X and reports information associated with this valid duration to the network via RRC signalling. The detailed signaling of reporting is up to RAN2, so it is suggested to update as: 1. UE derives its position based on its GNSS measurements [1-1. UE reports the information associated with the validity duration of GNSS] |
| Huawei, HiSilicon | We would like to keep “1-1. UE report the validity duration of GNSS” with brackets or remove it and wait for further RAN2 agreement |

# Issue 2: FG 2-1a

After review of contributions submitted to RAN1 #108-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 2. LTE_NBIOT_eMTC_NTN | 2-1a | Segmented UL transmission duration configuration | 1. Support UE applies segmented UL transmission according to duration configuration by the network for NGSO The same value is used for segment durations for all PRACH preambles. 2.Support segment pre-compensation by using one of followings methods: 2-1) UE can drop/insert samples and puncture OFDM symbols 2-2) UE can blank subframes, slots, and repetition units | 2-1 | Yes | N/A | Release 17 UE cannot access [NTN/satellite] | [Per UE/per band] | No | No | For UEs support NSGO scenarios, it must indicate this FG is supported. | Optional with capability signalling [Note: This UE feature group is applicable only for IoT-NTN cell, for terrestrial cell this feature is not supported] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Nokia, NSB | Align the notes and consequence if not supported to the same terminology used in NTN feature groups. The extra text in FG description is not needed and it does not belong to FG definitions, but to specifications. |
| Apple | We are fine with the proposal on the FG name and pre-requisite part. |
| SoftBank | Agree with Nokia for same terminology used in NTN feature groups. |
| Qualcomm | Modify as follows: “The same value is used for segment durations for all PRACH preambles of the same preamble format.”, in line with RAN1 agreements (for NB-IoT). Also, consequence of this should be “cannot access NGSO NTNs”. |
| ZTE | We are fine with current updates. |
| Intel | Support |
| Xiaomi | Support |
| Huawei, HiSilicon | We don’t think there a need to mention scenarios in the UE feature definition. Hence component 1 can updated as “UE applies segmented UL transmission according to duration configuration by the network for NGSO” The details and component 2 are not needed for a UE feature description hence should be removed. |
| MediaTek | Support. Component 1 is aligned with RAN1 agreement on UL segmented transmission is not supported by GEO (i.e. only supported by NGSO), hence “for NGSO” should be kept in component 1. Agreement UL Segmented transmission NPRACH/NPUSCH for NB-IoT is not supported in GEO based on UE feature Release 17 UE cannot access [NTN/satellite] should be revised as “Release 17 UE cannot access [NTN GNSO /satellite] |
| Ericsson | It is not clear why GEO is excluded from segmented UL transmission. We note that segmented pre-compensation for GEO is not required for NB-IoT but it is required for eMTC. Therefore, the text “for NGSO” should be deleted. Moreover, the following text should be updated to remove NGSO as well: “For UEs supporting NTN scenarios, it must indicate this FG is supported.” The detailed text on how segmented pre-compensation is implemented is not needed and can be left for the specification. “Release 17 UE cannot access [NTN/satellite]” should be updated to “Release 17 UE cannot access NTN” |

# Issue 3: FG 2-2

After review of contributions submitted to RAN1 #108-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 2. LTE_NBIOT_eMTC_NTN | 2-2 | Enhancing timing relationships using a time offset | UE receives and applies UE specific K_offset in timing relationship enhancements | 2-1 [, 2-3] | Yes | N/A | UE does not know the offset to apply for UL transmission | [per UE/per band] | No | No | The K_offset is a scheduling offset used for the identified timing relationships that need to be modified for IoT NTN. For IoT NTN, support cell-specific Koffset configuration for use during initial access. For IoT NTN, support the use of UE-specific Koffset in CONNECTED mode. FFS: whether this feature group needs to be separate for eMTC and NB-IoT FFS: differentiation based on orbits such as LEO/MEO/GEO | Optional with capability signalling FFS: For UEs supporting NB-IoT/eMTC NTN, it must indicate this FG is supported |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | Support |
| Qualcomm | OK. |
| ZTE | Ok |
| Intel | OK |
| Huawei, HiSilicon | Support |
| MediaTek | It should be per [per UE/per band] per band as the K_offset can be cell-specific or UE-specific Agreement: For IoT NTN, support cell-specific Koffset configuration for use during initial access. Agreement: For IoT NTN, support the use of UE-specific Koffset in CONNECTED mode. Agreement For IoT NTN, the UE specific K_offset is provided and updated by the network using MAC CE. |
| Ericsson | In addition to K_offset, K_mac should also be included. Moreover, 2-2 is an essential feature due to half-duplex constraints. |

# Issue 4: FG 2-3

After review of contributions submitted to RAN1 #108-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 2. LTE_NBIOT_eMTC_NTN | 2-3 | UE specific TA pre-compensation reporting | - [Support reporting of information about the UE specific TA pre-compensation] The granularity of the reported TA is a slot. As for the frequency of reports, the TA is reported when the offset between current information about UE specific TA and the last successfully reported information about UE specific TA is over the threshold. | 2-1 | Yes | No | UL scheduling for FDD-HD: Use of UE-specific TA and/or K_offset to avoid UL-DL collisions in FDD-HD | [per UE/per band] | No | No | UE-specific TA reporting is supported in IoT-NTN FFS: Detailed contents of report FFS: whether this feature group needs to be separate for eMTC and NB-IoT FFS: differentiation based on orbits such as LEO/MEO/GEO | Optional with capability signalling FFS: For UEs supporting NB-IoT/eMTC NTN, it must indicate this FG is supported |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Nokia, NSB | The extra components are not needed, and in fact that kind of text does not belong to FG definitions. Per UE indication is preferred. |
| Apple | Like in NR NTN UE features, we may remove “UE specific” in the feature name and component to avoid confusion. Note that UE specific TA is different from UE’s full TA, where only the latter is reported. We do not need to add the detailed descriptions (i.e., the newly added texts “The granularity …, over the threshold”.) of the component. |
| Qualcomm | Detailed descriptions (additions in red) not required. RAN2 will decide exactly what to report, how often, etc. |
| ZTE | Detailed description is not needed, which will be further discussed in RAN2. |
| Intel | Same view as Qualcomm. Details of this feature were discussed in RAN2, so we can add the following note as for NR NTN. Note: The exact content of UE reporting of information about the UE specific TA pre-compensation is up to RAN2 |
| Xiaomi | We share Qualcomm’s view, the event to trigger the reporting is up to RAN2, related description should be removed. |
| Huawei, HiSilicon | Agree with other companies that the details are not needed in the UE feature description. |
| MediaTek | Remove detailed descriptions (in red text), details not needed and may need further discussion in RAN2 |
| Ericsson | Agree with other companies that detailed descriptions should be removed. |

# Issue 5: Define separate UE feature groups for NB-IoT and eMTC over NTN

After review of contributions submitted to RAN1 #108-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Define separate UE feature groups for NB-IoT and eMTC over NTN

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Qualcomm | This has always been done for other FGs in the past for eMTC and NB-IoT; so, we should follow that convention, and define separate FGs for eMTC and NB-IoT. There are subtle differences between eMTC and NB-IoT: for example, NPRACH preamble “format-specific” segment durations in NB-IoT (there will be others too, once all the details are carefully checked). |
| ZTE | Fine to define separate UE FG. |
| Intel | OK |
| Huawei, HiSilicon | Support |
| MediaTek | Support |
| Ericsson | Support |

# Discussion/Approval Items during RAN1 #108-e — Second Checkpoint

Based on the comments/questions/suggestions received by the first checkpoint, the following are the revised proposals and/or proposed agreements by the moderator. Companies submitted the following views on the moderator’s proposals.

[Please submit all comments/questions/suggestions here, late comments/questions/suggestions submitted in Section 3 will not be considered]

General comments

| Company | Comments/Questions/Suggestions |
| --- | --- |

# Issue 1: FG 2-1

Based on the comments/questions/suggestions received by the first checkpoint, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 2. LTE_NBIOT_eMTC_NTN | 2-1 | Basic IoT over NTN support | 1. UE derives its position based on its GNSS measurements [1-1. UE reports the information associated with the validity duration of GNSS] 2. UE receives serving satellite ephemeris in either state vector format or orbital element format 4. UE calculates UE specific TA calculation in RRC_IDLE and RRC_CONNECTED state based on its GNSS-acquired position and the serving satellite ephemeris 6. UE applies calculates common TA in RRC_IDLE and RRC_CONNECTED according to the parameters provided by the network (UE considers common TA as 0 if the parameter is not provided) [7. For TA update in RRC_CONNECTED state, UE uses a combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops] 8. In RRC_IDLE and RRC_CONNECTED state, UE calculates frequency pre-compensation to counter shift the Doppler experienced on the service link [in DL] [and] [in UL] 10. UE supports a validity timer of UL synchronization that is configured by the network 13. UE applies cell specific K_offset in timing relationship enhancements 14.UE estimates UE-gNB RTT [15. UE estimates UE-gNB RTT and delaysing the starts of ra-ResponseWindow by UE-gNB RTT] [16. UE receives cell specific K_mac] Delay the UE action and assumption on a downlink configuration carried by MAC CE command by K_mac if it is indicated 17. In RRC_IDLE state and RRC_CONNECTED state, UE pre-compensates the calculated frequency offset and TA in uplink transmissions |  | Yes | N/A | Release 17 UE cannot communicate via access [NTN/satellite] | [per UE/per band] | No | No | FFS: whether this feature group needs to be separate for eMTC and NB-IoT | Optional with capability signalling For UEs supporting NB-IoT/eMTC NTN, it must indicate this FG is supported [Note: This UE feature group is applicable only for IoT-NTN cell for communication via satellite, for terrestrial any other cell this feature is not supported] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Ericsson | It is not clear to us why the type is “per band” instead of “per UE” for IoT NTN. We think this should be left open until it has been clarified why "per band" indication is needed. |
| MediaTek | Same comment as Ericsson on “per UE” and “per band” On component 8, UE calculates frequency pre-compensation to counter shift the Doppler experienced on the service link [in DL] [and] [in UL]. To our understanding the [in DL] [and] [in UL] is a level of detail related to the calculation that is not needed and can be removed from UE feature. There is no agreement in RAN1 to justify inclusion in list of UE features. |
| Apple | We think the granularity of this FG is per band to have UE implementation flexibility for all the IoT NTN UE features. For example, a UE may support the basic IoT over NTN operation in a band for GEO operation, but not in another band for LEO operation. |

# Issue 2: FG 2-1a

Based on the comments/questions/suggestions received by the first checkpoint, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 2. LTE_NBIOT_eMTC_NTN | 2-1a | Segmented UL transmission duration configuration | 1. Support UE applies segmented UL transmission according to duration configuration by the network | 2-1 | Yes | N/A | Release 17 UE cannot access [NTN//satellite] NGSO NTNs | [Per UE/per band] | No | No | For UEs support NSGO scenarios, it must indicate this FG is supported. | Optional with capability signalling [Note: This UE feature group is applicable only for IoT-NTN cell, for terrestrial cell this feature is not supported] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Ericsson | We note that segmented pre-compensation for GEO is not required for NB-IoT but it is required for eMTC. Therefore, the text “NGSO” should be deleted throughout: “Release 17 UE cannot access NTNs” “For UEs supporting NTN scenarios, it must indicate this FG is supported.” Moreover, the type should be “per UE”. |
| MediaTek | There was an explicit agreement in RAN1#107-e that UL segmented transmission is not supported in NB-IoT for GEO Agreement - UL Segmented transmission NPRACH/NPUSCH for NB-IoT is not supported in GEO based on UE feature We propose the following wording to align with RAN1 agreements: “ Release 17 NB-IoT UE cannot access [NTN//satellite] NGSO NTNs, Release 17 eMTC cannot access GEO and NGSO NTNs” |

# Issue 3: FG 2-2

Based on the comments/questions/suggestions received by the first checkpoint, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 2. LTE_NBIOT_eMTC_NTN | 2-2 | Enhancing timing relationships using a time offset | UE receives and applies UE specific K_offset, K_mac in timing relationship enhancements | 2-1 [, 2-3] | Yes | N/A | UE does not know the offset to apply for UL transmission | [per UE/per band] | No | No | The K_offset is a scheduling offset used for the identified timing relationships that need to be modified for IoT NTN. For IoT NTN, support cell-specific Koffset configuration for use during initial access. For IoT NTN, support the use of UE-specific Koffset in CONNECTED mode. FFS: whether this feature group needs to be separate for eMTC and NB-IoT FFS: differentiation based on orbits such as LEO/MEO/GEO | Optional with capability signalling FFS: For UEs supporting NB-IoT/eMTC NTN, it must indicate this FG is supported |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Ericsson | Support with type “per UE”. |
| MediaTek | Same view as Ericssson. Generally it is not clear what is meant by “per U” and “per band”. Moderator clarifications will be helpful. |

# Issue 4: FG 2-3

Based on the comments/questions/suggestions received by the first checkpoint, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 2. LTE_NBIOT_eMTC_NTN | 2-3 | UE specific TA pre-compensation reporting | - [Support reporting of information about the UE specific TA pre-compensation] | 2-1 | Yes | No | UL scheduling for FDD-HD: Use of UE-specific TA and/or K_offset to avoid UL-DL collisions in FDD-HD | [per UE/per band] | No | No | UE-specific TA reporting is supported in IoT-NTN FFS: Detailed contents of report FFS: whether this feature group needs to be separate for eMTC and NB-IoT FFS: differentiation based on orbits such as LEO/MEO/GEO | Optional with capability signalling FFS: For UEs supporting NB-IoT/eMTC NTN, it must indicate this FG is supported |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Ericsson | Support with type “per UE”. |
| MediaTek | Same view as Ericssson. Generally it is not clear what is meant by “per U” and “per band”. Moderator clarifications will be helpful. |

# Issue 5: Define separate UE feature groups for NB-IoT and eMTC over NTN

Based on the comments/questions/suggestions received by the first checkpoint, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposed Agreement: Define separate UE feature groups for NB-IoT and eMTC over NTN

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Ericsson | Support |
| MediaTek | Support. This will be very helpful and avoid issue like in FG 2-1a |

# Discussion/Approval Items during RAN1 #108-e — Third Checkpoint

Based on the comments/questions/suggestions received by the second checkpoint, the following are the revised proposals and/or proposed agreements by the moderator. Companies submitted the following views on the moderator’s proposals.

[Please submit all comments/questions/suggestions here, late comments/questions/suggestions submitted in Section 4 will not be considered]

General comments

| Company | Comments/Questions/Suggestions |
| --- | --- |

# Issue 1: FG 2-1

Based on the comments/questions/suggestions received by the second checkpoint, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 2. LTE_NBIOT_eMTC_NTN | 2-1 | Basic IoT over NTN support for eMTC | 1. UE derives its position based on its GNSS measurements [1-1. UE reports the information associated with the validity duration of GNSS] 2. UE receives serving satellite ephemeris in either state vector format or orbital element format 4. UE calculates UE specific TA in RRC_IDLE and RRC_CONNECTED state based on its GNSS-acquired position and the serving satellite ephemeris 6. UE calculates common TA in RRC_IDLE and RRC_CONNECTED according to the parameters provided by the network (UE considers common TA as 0 if the parameter is not provided) 7. For TA update in RRC_CONNECTED state, UE uses a combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops 8. In RRC_IDLE and RRC_CONNECTED state, UE calculates frequency pre-compensation to counter shift the Doppler experienced on the service link [in DL] [and] [in UL] 10. UE supports a validity timer of UL synchronization that is configured by the network 15. UE estimates UE-gNB RTT and delays the starts of ra-ResponseWindow by UE-gNB RTT 16. Delay the UE action and assumption on a downlink configuration carried by MAC CE command by K_mac if it is indicated 17. In RRC_IDLE state and RRC_CONNECTED state, UE pre-compensates the calculated frequency offset and TA in uplink transmissions |  | Yes | N/A | Release 17 eMTC UE cannot communicate via satellite | [per UE/per band] | No | No |  | Optional with capability signalling For UEs supporting NB-IoT/eMTC NTN, it must indicate this FG is supported Note: This UE feature group is applicable only for IoT-NTN cell for communication via satellite, for any other cell this feature is not supported |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2. LTE_NBIOT_eMTC_NTN | 2-1b | Basic IoT over NTN support for NB-IoT | 1. UE derives its position based on its GNSS measurements [1-1. UE reports the information associated with the validity duration of GNSS] 2. UE receives serving satellite ephemeris in either state vector format or orbital element format 4. UE calculates UE specific TA in RRC_IDLE and RRC_CONNECTED state based on its GNSS-acquired position and the serving satellite ephemeris 6. UE calculates common TA in RRC_IDLE and RRC_CONNECTED according to the parameters provided by the network (UE considers common TA as 0 if the parameter is not provided) 7. For TA update in RRC_CONNECTED state, UE uses a combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops 8. In RRC_IDLE and RRC_CONNECTED state, UE calculates frequency pre-compensation to counter shift the Doppler experienced on the service link 10. UE supports a validity timer of UL synchronization that is configured by the network 15. UE estimates UE-gNB RTT and delays the starts of ra-ResponseWindow by UE-gNB RTT 16. Delay the UE action and assumption on a downlink configuration carried by MAC CE command by K_mac if it is indicated 17. In RRC_IDLE state and RRC_CONNECTED state, UE pre-compensates the calculated frequency offset and TA in uplink transmissions |  | Yes | N/A | Release 17 NB-IoT UE cannot communicate via satellite | per band | No | No |  | Optional with capability signalling For UEs supporting NB-IoT NTN, it must indicate this FG is supported Note: This UE feature group is applicable only for IoT-NTN cell for communication via satellite, for any other cell this feature is not supported |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | Fine with the proposal. We agree that the FG is defined per band, similar to that in NR NTN. |
| Ericsson | In the second round, component 13 was removed from the description on the basis that the timing relationship is covered in FG 2-2. However, FG2-2 mentions UE-specific K_offset: “UE receives and applies UE specific K_offset, K_mac in timing relationship enhancements”, whereas component 13 was about cell-specific K_offset. Therefore, we believe that this discrepancy should be addressed. The deleted component 13 is as follows: 13. UE applies cell specific K_offset in timing relationship enhancements Unlike NR NTN, supporting HAPS is out of scope of IoT NTN. The motivation for defining the FG as “per band” is not clear to us. |
| Huawei, HiSilicon | Support the change from Ericsson. In addition, we don’t understand the reason why the FG is defined as “per band” instead of “per UE”. My assumption is that companies may want to differentiate TN and NTN which is also being discussed AI 8.16.9 as well in RAN2. |
| Qualcomm | We are OK with the proposals. The FG should be “per band”, as written. This is because, a UE should not be mandated to pass tests on *all* NTN bands of deployment, if wants to support NTN. For example, if an NTN deployment happens in the Ka band, a UE may not support it—while it may support NTN in, say, in the S-band. The purpose of the FGs is to provide as much granularity as possible. |
| MediaTek | Support the change by Ericsson and Huawei. To help moderator make revisions, we suggest to add component 18 (this is the same as in NR NTN FG 26-1 component 9),and further add back component 13 (which aligns with NR NTN FG 26-1 component 7 which mention cell-specific K_offset) 18 UE receives cell-specific K_offset/K_mac in system information 13. UE applies cell specific K_offset in timing relationship enhancements |
| ZTE | We are fine the proposal including changes from Ericsson. Regarding the discussion on “per UE” or “per band”, it’s more reasonable to define the feature as per UE. Regarding the implementation part, if more than one bands are used for NTN (all these band are also supported by the UE’s RF), It’s strange that the UE only support NTN feature on one of them. |

# Issue 2: FG 2-1a

Based on the comments/questions/suggestions received by the second checkpoint, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 2. LTE_NBIOT_eMTC_NTN | 2-1a | Segmented UL transmission duration configuration for eMTC | 1. Support UE applies segmented UL transmission according to duration configuration by the network | 2-1 | Yes | N/A | Release 17 eMTC UE cannot access communicate via [NTN/satellite] GEO and NGSO NTNs | [Per UE/per band] | No | No | For UEs support communicate via GEO and NGSO NTNs NSGO scenarios, it must indicate this FG is supported. | Optional with capability signalling [Note: This UE feature group is applicable only for IoT-NTN cell, for terrestrial cell this feature is not supported] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2. LTE_NBIOT_eMTC_NTN | 2-1c | Segmented UL transmission for NB-IoT | 1. UE applies segmented UL transmission according to duration configuration by the network | 2-1 | Yes | N/A | Release 17 NB-IoT UE cannot communicate via NGSO NTNs | per band | No | No | For UEs support communicate via NGSO NTNs, it must indicate this FG is supported. | Optional with capability signalling Note: This UE feature group is applicable only for IoT-NTN cell, for terrestrial cell this feature is not supported |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | The pre-requisite of FG 2-1c should be FG 2-1b, which is for NB-IoT. |
| Ericsson | Unlike NR NTN, supporting HAPS is out of scope of IoT NTN. The motivation for defining the FG as “per band” is not clear to us. Typo: For UEs support For UEs supporting |
| Huawei, HiSilicon | Support the editorial comments from Apple and Ericsson. We don’t understand the reason why the FG is defined as “per band” instead of “per UE”. My assumption is that companies may want to differentiate TN and NTN which is also being discussed AI 8.16.9 as well in RAN2. But “per-band” is unnecessary. |
| Qualcomm | Apple and Ericsson’s comments are OK. Again, all FGs should be “per band”, since NTN deployments can happen across a wide range of RF spectrum, and the UE should not be mandated to pass tests on all possible NTN bands (similar comment as described under FG 2-1). I am slightly confused about the eMTC capability for segmented pre-compensation though: if companies think it is needed even for GEO, shouldn’t it be a basic component of 2-1 for eMTC? And if it is not needed for GEO eMTC, then we can edit the description, and make it similar to NB-IoT—i.e., it is needed only for NGSO. I guess Ericsson can comment, since they proposed that segmented pre-compensation is needed for eMTC even for GEO settings. |
| MediaTek | Support Apple proposed correction for FG 2-1c FG 2-1b The ”per band” interpretation by Qualcomm is correct. To our understanding , the motivation for “Per UE” for FG 2-1a and FG 2-1c is that UE support the feature for NTN on one band, and not support the feature for NTN on another band. This seems not needed. TN and NTN are deployed on the same band, and as mentioned by Ericsson HAPS is not supported in IoT NTN. |
| ZTE | Support the correction. And regarding the per band part, please refer to the comment above. |

# Issue 3: FG 2-2

Based on the comments/questions/suggestions received by the second checkpoint, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 2. LTE_NBIOT_eMTC_NTN | 2-2 | Enhancing timing relationships using a time offset for eMTC | UE receives and applies UE specific K_offset, K_mac in timing relationship enhancements | 2-1 , 2-3 | Yes | N/A | eMTC UE does not know the offset to apply for UL transmission | [Per UE/per band] | No | No | The K_offset is a scheduling offset used for the identified timing relationships that need to be modified for IoT NTN. For IoT NTN, support cell-specific Koffset configuration for use during initial access. For IoT NTN, support the use of UE-specific Koffset in CONNECTED mode. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2. LTE_NBIOT_eMTC_NTN | 2-2a | Enhancing timing relationships using a time offset for NB-IoT | UE receives and applies UE specific K_offset, K_mac in timing relationship enhancements | 2-1 , 2-3 | Yes | N/A | NB-IoT UE does not know the offset to apply for UL transmission | per band | No | No | The K_offset is a scheduling offset used for the identified timing relationships that need to be modified for IoT NTN. For IoT NTN, support cell-specific Koffset configuration for use during initial access. For IoT NTN, support the use of UE-specific Koffset in CONNECTED mode. | Optional with capability signalling |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | The pre-requisite of FG 2-2a should be FG 2-1b and FG 2-3a, which are for NB-IoT. |
| Ericsson | Unlike NR NTN, supporting HAPS is out of scope of IoT NTN. The motivation for defining the FG as “per band” is not clear to us. |
| Huawei, HiSilicon | Support the change from Apple. We don’t understand the reason why the FG is defined as “per band” instead of “per UE”. My assumption is that companies may want to differentiate TN and NTN which is also being discussed AI 8.16.9 as well in RAN2. But “per-band” is unnecessary. |
| Qualcomm | Editorial issues should be ironed out, as mentioned by Apple. Reasoning on “per band” is described in the comments for the previous FGs. |
| MediaTek | Support Apple editorial changes. On “per band”, same view as discussed for the previous FGs |
| ZTE | Support the correction. And regarding the per band part, please refer to the comment above. |

# Issue 4: FG 2-3

Based on the comments/questions/suggestions received by the second checkpoint, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 2. LTE_NBIOT_eMTC_NTN | 2-3 | TA pre-compensation reporting for eMTC | Support reporting of information about the UE specific TA pre-compensation | 2-1 | Yes | No | UL scheduling for FDD-HD: Use of UE-specific TA and/or K_offset to avoid UL-DL collisions in FDD-HD | [Per UE/per band] | No | No | UE-specific TA reporting is supported in IoT-NTN | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2. LTE_NBIOT_eMTC_NTN | 2-3a | TA pre-compensation reporting for NB-IoT | Support reporting of information about the UE specific TA pre-compensation | 2-1 | Yes | No | UL scheduling for FDD-HD: Use of UE-specific TA and/or K_offset to avoid UL-DL collisions in FDD-HD | per band | No | No | UE-specific TA reporting is supported in IoT-NTN | Optional with capability signalling |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | The pre-requisite of FG 2-3a should be FG 2-1b, which is for NB-IoT. |
| Ericsson | Unlike NR NTN, supporting HAPS is out of scope of IoT NTN. The motivation for defining the FG as “per band” is not clear to us. |
| Huawei, HiSilicon | Support the change from Apple. We also don’t understand the reason why the FG is defined as “per band” instead of “per UE”. My assumption is that companies may want to differentiate TN and NTN which is also being discussed AI 8.16.9 as well in RAN2. But “per-band” is unnecessary. |
| Qualcomm | Editorial issues should be ironed out, as mentioned by Apple. Reasoning on “per band” is described in the comments for the previous FGs. |
| MediaTek | Support Apple editorial changes. On “per band”, same view as discussed for the previous FGs |
| ZTE | Support the correction. And regarding the per band part, please refer to the comment above. |

# Issue 5: Define separate UE feature groups for NB-IoT and eMTC over NTN

Void

# Summary of Final Agreements

After further discussion on the RAN1 email reflector the following was agreed as part of this email discussion:

## Agreements reached by email at the first check point

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 2. LTE_NBIOT_eMTC_NTN | 2-1 | Basic IoT over NTN support | 1. UE derives its position based on its GNSS measurements [1-1. UE reports the information associated with the validity duration of GNSS] 2. UE receives serving satellite ephemeris in either state vector format or orbital element format 4. UE calculates UE specific TA calculation in RRC_IDLE and RRC_CONNECTED state based on its GNSS-acquired position and the serving satellite ephemeris 6. UE applies calculates common TA in RRC_IDLE and RRC_CONNECTED according to the parameters provided by the network (UE considers common TA as 0 if the parameter is not provided) [7. For TA update in RRC_CONNECTED state, UE uses a combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops] 8. In RRC_IDLE and RRC_CONNECTED state, UE calculates frequency pre-compensation to counter shift the Doppler experienced on the service link [in DL] [and] [in UL] 10. UE supports a validity timer of UL synchronization that is configured by the network 13. UE applies cell specific K_offset in timing relationship enhancements 14.UE estimates UE-gNB RTT [15. UE estimates UE-gNB RTT and delaysing the starts of ra-ResponseWindow by UE-gNB RTT] [16. UE receives cell specific K_mac] Delay the UE action and assumption on a downlink configuration carried by MAC CE command by K_mac if it is indicated 17. In RRC_IDLE state and RRC_CONNECTED state, UE pre-compensates the calculated frequency offset and TA in uplink transmissions |  | Yes | N/A | Release 17 UE cannot communicate via access [NTN/satellite] | [per UE/per band] | No | No | FFS: whether this feature group needs to be separate for eMTC and NB-IoT | Optional with capability signalling For UEs supporting NB-IoT/eMTC NTN, it must indicate this FG is supported [Note: This UE feature group is applicable only for IoT-NTN cell for communication via satellite, for terrestrial any other cell this feature is not supported] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 2. LTE_NBIOT_eMTC_NTN | 2-2 | Enhancing timing relationships using a time offset | UE receives and applies UE specific K_offset, K_mac in timing relationship enhancements | 2-1 [, 2-3] | Yes | N/A | UE does not know the offset to apply for UL transmission | [per UE/per band] | No | No | The K_offset is a scheduling offset used for the identified timing relationships that need to be modified for IoT NTN. For IoT NTN, support cell-specific Koffset configuration for use during initial access. For IoT NTN, support the use of UE-specific Koffset in CONNECTED mode. FFS: whether this feature group needs to be separate for eMTC and NB-IoT FFS: differentiation based on orbits such as LEO/MEO/GEO | Optional with capability signalling FFS: For UEs supporting NB-IoT/eMTC NTN, it must indicate this FG is supported |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 2. LTE_NBIOT_eMTC_NTN | 2-3 | UE specific TA pre-compensation reporting | - [Support reporting of information about the UE specific TA pre-compensation] | 2-1 | Yes | No | UL scheduling for FDD-HD: Use of UE-specific TA and/or K_offset to avoid UL-DL collisions in FDD-HD | [per UE/per band] | No | No | UE-specific TA reporting is supported in IoT-NTN FFS: Detailed contents of report FFS: whether this feature group needs to be separate for eMTC and NB-IoT FFS: differentiation based on orbits such as LEO/MEO/GEO | Optional with capability signalling FFS: For UEs supporting NB-IoT/eMTC NTN, it must indicate this FG is supported |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Define separate UE feature groups for NB-IoT and eMTC over NTN

## Agreements reached by email at the final check point

Agreement:  Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 2. LTE_NBIOT_eMTC_NTN | 2-1 | Basic IoT over NTN support for eMTC | 1. UE derives its position based on its GNSS measurements [1-1. UE reports the information associated with the validity duration of GNSS] 2. UE receives serving satellite ephemeris in either state vector format or orbital element format 4. UE calculates UE specific TA in RRC_IDLE and RRC_CONNECTED state based on its GNSS-acquired position and the serving satellite ephemeris 6. UE calculates common TA in RRC_IDLE and RRC_CONNECTED according to the parameters provided by the network (UE considers common TA as 0 if the parameter is not provided) 7. For TA update in RRC_CONNECTED state, UE uses a combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops 8. In RRC_IDLE and RRC_CONNECTED state, UE calculates frequency pre-compensation to counter shift the Doppler experienced on the service link [in DL] [and] [in UL] 10. UE supports a validity timer of UL synchronization that is configured by the network 13. UE applies cell specific K_offset in timing relationship enhancements 15. UE estimates UE-gNB RTT and delays the starts of ra-ResponseWindow by UE-gNB RTT 16. Delay the UE action and assumption on a downlink configuration carried by MAC CE command by K_mac if it is indicated 17. In RRC_IDLE state and RRC_CONNECTED state, UE pre-compensates the calculated frequency offset and TA in uplink transmissions 18. UE receives cell-specific K_offset/K_mac in system information |  | Yes | N/A | Release 17 eMTC UE cannot communicate via satellite | [Per UE/per band] | No | No |  | Optional with capability signalling For UEs supporting NB-IoT/eMTC NTN, it must indicate this FG is supported Note: This UE feature group is applicable only for IoT-NTN cell for communication via satellite, for any other cell this feature is not supported |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2. LTE_NBIOT_eMTC_NTN | 2-1b | Basic IoT over NTN support for NB-IoT | 1. UE derives its position based on its GNSS measurements [1-1. UE reports the information associated with the validity duration of GNSS] 2. UE receives serving satellite ephemeris in either state vector format or orbital element format 4. UE calculates UE specific TA in RRC_IDLE and RRC_CONNECTED state based on its GNSS-acquired position and the serving satellite ephemeris 6. UE calculates common TA in RRC_IDLE and RRC_CONNECTED according to the parameters provided by the network (UE considers common TA as 0 if the parameter is not provided) 7. For TA update in RRC_CONNECTED state, UE uses a combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops 8. In RRC_IDLE and RRC_CONNECTED state, UE calculates frequency pre-compensation to counter shift the Doppler experienced on the service link 10. UE supports a validity timer of UL synchronization that is configured by the network 13. UE applies cell specific K_offset in timing relationship enhancements 15. UE estimates UE-gNB RTT and delays the starts of ra-ResponseWindow by UE-gNB RTT 16. Delay the UE action and assumption on a downlink configuration carried by MAC CE command by K_mac if it is indicated 17. In RRC_IDLE state and RRC_CONNECTED state, UE pre-compensates the calculated frequency offset and TA in uplink transmissions 18. UE receives cell-specific K_offset/K_mac in system information |  | Yes | N/A | Release 17 NB-IoT UE cannot communicate via satellite | [Per UE/per band] | No | No |  | Optional with capability signalling For UEs supporting NB-IoT NTN, it must indicate this FG is supported Note: This UE feature group is applicable only for IoT-NTN cell for communication via satellite, for any other cell this feature is not supported |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 2. LTE_NBIOT_eMTC_NTN | 2-1a | Segmented UL transmission duration configuration for eMTC | 1. Support UE applies segmented UL transmission according to duration configuration by the network | 2-1 | Yes | N/A | [Release 17 eMTC UE cannot access communicate via [NTN/satellite] GEO and NGSO NTNs] | [Per UE/per band] | No | No | [For UEs support communicate via GEO and NGSO NTNs NSGO scenarios, it must indicate this FG is supported.] | Optional with capability signalling [Note: This UE feature group is applicable only for IoT-NTN cell, for terrestrial cell this feature is not supported] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2. LTE_NBIOT_eMTC_NTN | 2-1c | Segmented UL transmission for NB-IoT | 1. UE applies segmented UL transmission according to duration configuration by the network | 2-1b | Yes | N/A | Release 17 NB-IoT UE cannot communicate via NGSO NTNs | [Per UE/per band] | No | No | For UEs supporting communicate via NGSO NTNs, it must indicate this FG is supported. | Optional with capability signalling Note: This UE feature group is applicable only for IoT-NTN cell, for terrestrial cell this feature is not supported |

Agreement:  Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 2. LTE_NBIOT_eMTC_NTN | 2-2 | Enhancing timing relationships using a time offset for eMTC | UE receives and applies UE specific K_offset, K_mac in timing relationship enhancements | 2-1 , 2-3 | Yes | N/A | eMTC UE does not know the offset to apply for UL transmission | [Per UE/per band] | No | No | The K_offset is a scheduling offset used for the identified timing relationships that need to be modified for IoT NTN. For IoT NTN, support cell-specific Koffset configuration for use during initial access. For IoT NTN, support the use of UE-specific Koffset in CONNECTED mode. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2. LTE_NBIOT_eMTC_NTN | 2-2a | Enhancing timing relationships using a time offset for NB-IoT | UE receives and applies UE specific K_offset, K_mac in timing relationship enhancements | 2-1b, 2-3a | Yes | N/A | NB-IoT UE does not know the offset to apply for UL transmission | [Per UE/per band] | No | No | The K_offset is a scheduling offset used for the identified timing relationships that need to be modified for IoT NTN. For IoT NTN, support cell-specific Koffset configuration for use during initial access. For IoT NTN, support the use of UE-specific Koffset in CONNECTED mode. | Optional with capability signalling |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 2. LTE_NBIOT_eMTC_NTN | 2-3 | TA pre-compensation reporting for eMTC | Support reporting of information about the UE specific TA pre-compensation | 2-1 | Yes | No | UL scheduling for FDD-HD: Use of UE-specific TA and/or K_offset to avoid UL-DL collisions in FDD-HD | [Per UE/per band] | No | No | UE-specific TA reporting is supported in IoT-NTN | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2. LTE_NBIOT_eMTC_NTN | 2-3a | TA pre-compensation reporting for NB-IoT | Support reporting of information about the UE specific TA pre-compensation | 2-1b | Yes | No | UL scheduling for FDD-HD: Use of UE-specific TA and/or K_offset to avoid UL-DL collisions in FDD-HD | [Per UE/per band] | No | No | UE-specific TA reporting is supported in IoT-NTN | Optional with capability signalling |

# References

- R1-2200780, Updated RAN1 UE features list for Rel-17 NR after RAN1 #108-e, Moderators (AT&T, NTT DOCOMO, INC.)

- R1-2200943, Rel-17 UE features for IoT over NTN, Huawei/HiSilicon

- R1-2201221, UE features for IoT over NTN, MediaTek Inc.

- R1-2201304, Discussion on UE features for NTN-IoT, OPPO

- R1-2201421, On UE features for IoT over NTN, Nokia/Nokia Shanghai Bell

- R1-2201726, On UE features for IOT over NTN, Intel Corporation

- R1-2201802, Views on Rel-17 IoT over NTN UE Features, Apple

- R1-2201811, On UE features for IoT over NTN, Ericsson Hungary Ltd

- R1-2202178, UE features for IoT over NTN, Qualcomm Incorporated

- R1-2202213, Discussion on UE feature for IoT-NTN, ZTE