3GPP TSG RAN WG1 Meeting #108-e	R1-2202937

e-Meeting, February 21th  – March 3rd, 2022

Agenda Item:	8.15 - NB-IoT/eMTC support for Non-Terrestrial Network

Source:	WI rapporteur (MediaTek)

Title:	3GPP TSG-RAN WG1 Agreements for Release-17  IoT NTN (post RAN1#108-e)

Document for:	Information

# Introduction

This document lists the RAN1 agreements in NB-IoT/eMTC support for Non-Terrestrial Network Work Item up to and including RAN1#108-e.

The WI objective from the Rel-17 IoT NTN WID is inserted below for convenience.

The moderator summary and spreadsheet for Rel-17 RRC parameters for IoT NTN were provided in [13]. Feature lead summaries for 8.15.1 and 8.15.2 are referenced in Section 5. The Technical Report TR 36.763 for Rel-17 IoT NTN is also referenced in Section 5.

# WID Objective

The objective of this Work Item is to specify support of NB-IoT and eMTC over NTN. Work on both NB-IoT and eMTC will start in August 2021 meetings. Enhancements shall be specified as described hereafter with the following assumptions: - Standalone deployment for NB-IoT / eMTC (i.e. operating in carrier(s) used only for NB-IoT NTN (resp. eMTC NTN)) for support in Rel-17 timeframe will be prioritized. - GNSS capability in the UE is taken as a working assumption for both NB-IoT and eMTC devices. With this assumption, UE can estimate and pre-compensate timing and frequency offset with sufficient accuracy for UL transmission. Simultaneous GNSS and NTN NB-IoT/eMTC operation is not assumed. - NB-IoT/eMTC design for terrestrial networks shall be reused as much as possible. - Transparent payload ### RAN1 Objectives Specify the following time and frequency synchronization enhancements, using NR_NTN_solutions WI agreements as baseline, according to Section 8 in TR 36.763: - UE pre-compensation including ephemeris format (orbital / Position -Velocity) - UE pre-compensation for UL synchronization in RRC_IDLE and RRC_CONNECTED states based at least on its GNSS-acquired position and the serving satellite ephemeris - Timing advance formula (granularity of the timing advance may be different) - Combination of Open (i.e. UE autonomous TA estimation, and common TA estimation) and Closed TA (i.e., received TA commands) control loops in RRC_CONNECTED state Agreements on the above are up to the decision in NR_NTN_Solutions WI and will be used for IoT NTN with minimum changes, if any. Specify the following time and frequency synchronization enhancements that are not covered by NR_NTN_Solutions WI agreements, according to Section 8 in TR 36.763: - Long PUSCH and PRACH Transmission enhancements: segmented UE pre-compensations, new UL gaps and/or implementation solutions, time units and duration of segments. - Validity timer for UL synchronization: satellite ephemeris, and potentially other aspects - DL synchronization enhancements: A single solution will be selected between: new channel raster, (part of) ARFCN-indication-in-MIB. - GNSS Measurements: Validity of a GNSS position fix and details of acquiring a GNSS position fix, duration of validity, in RRC CONNECTED mode for sporadic short transmission Specify the following IoT NTN specific timing relationships enhancements according to Section 8 in TR 36.763: - Timing relationships for NB-IoT / eMTC: as listed in Section 6.6.3 in TR 36.763 - UL scheduling for FDD-HD: Use of UE-specific TA and/or K_offset to avoid UL-DL collisions in FDD-HD - Signalling aspects in UE-specific TA maintenance and reporting, techniques to reduce the signalling load and determination of the UE-specific TA.

# 3GPP TSG RAN WG1 Meeting #106-e

## Enhancements to time and frequency synchronization

Agreement:

The following agreements from NR NTN are re-used for IoT NTN as working assumption.

- The Doppler shift over the feeder link and any transponder frequency error for both Downlink and Uplink is compensated by the GW and satellite-payload without any specification impacts in Release 17.

- The orbital propagator model to be used at UE side can be left to implementation

- Timing Advance formula can be transposed to IoT-NTN with Ts used instead of Tc

The Timing Advance applied by an NR NTN UE in RRC_IDLE/INACTIVE and RRC_CONNECTED is given by:

TTA=NTA+NTA,UE-specific+NTA,common+NTA,offset×Ts

Where:

- NTA  is defined as 0 for PRACH and updated based on TA Command field in msg2/msgB and MAC CE TA command.

  - FFS: details of NTA update/accumulation.

- NTA,UE-specific  is UE self-estimated TA to pre-compensate for the service link delay.

- NTA,common is network-controlled common TA, and may include any timing offset considered necessary by the network.

- NTA,common with value of 0 is supported.

  - FFS:  details of signaling including granularity.

- NTA,offset is a fixed offset used to calculate the timing advance.

Note-1: Definition of NTA is different from that in RAN1#103-e agreement in NR NTN WI.

Note-2: UE might not assume that the RTT between UE and gNB is equal to the calculated TA for Msg1/Msg A.

Note-3: NTA,common is the common timing offset X as agreed in RAN1 #103-e in NR NTN WI.

- Support the delivery of ephemeris information using both ephemeris formats, i.e., state vectors and orbital elements

- Set 1: Satellite position and velocity state vectors (position/velocity)

  - Position X,Y,Z in ECEF (m)

  - Velocity VX,VY,VZ in ECEF (m/s)

- Set 2: Parameters in orbital parameter ephemeris format

  - Semi-major axis α [m]

  - Eccentricity e

  - Argument of periapsis ω [rad]

  - Longitude of ascending node Ω [rad]

  - Inclination i [rad]

  - Mean anomaly M [rad] at epoch time to

  - FFS: Whether pre-provisioned ephemeris based on orbital elements can be used as reference. Thereby, only delta corrections can be broadcast in order to reduce the overhead

- For TA update in RRC_CONNECTED state, combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops shall be supported for IoT-NTN

Agreement:

The following agreement from NR NTN are re-used for IoT NTN as working assumption

- In Rel-17 IoT-NTN, at least support UE which can compute timing advance and frequency adjustment for serving link based on its GNSS position and serving satellite ephemeris signalled by the network and apply corresponding timing advance and frequency adjustment in RRC_IDLE and RRC_CONNECTED modes

- Serving satellite ephemeris Epoch time is implicitly known as a reference time defined by the starting time of a DL slot and/or frame.

FFS: Whether this starting time is given by predefined rule or it is indicated by the Network

Agreement:

For sporadic short transmission, UE in RRC_CONNECTED should go back to idle mode and re-acquire a GNSS position fix if GNSS becomes outdated.

Agreement:

- Satellite ephemeris read on SIB are valid for the duration of sporadic short transmission in RRC_CONNECTED.

- Common TA parameters if indicated and read on SIB are valid for the duration of sporadic short transmission in RRC_CONNECTED.

- Note: The duration of the short transmission is not longer than the “validity timer for UL synchronization” referred to in the WID objective (but which still needs further discussion for specifying further details)

Agreement:

The validity timer of UL synchronization is configured by the network

- FFS: Whether a single validity timer or separate validity timers are used for satellite ephemeris and common TA parameters

Agreement:

UE in RRC_IDLE reads the satellite ephemeris on SIB and the common TA parameters if indicated on SIB and (re-)start the validity timer(s) for UL synchronization before moving to RRC_CONNECTED.

- FFS: Details of the precise (re-)start time for the validity timer for UL synchronization to ensure a common understanding between gNB and UE.

- Other signaling details for validity timer are up to RAN2

Agreement:

Duration of UL transmission segment for UE pre-compensation for PRACH transmission is a number of RACH repetition units configured by the network

- For NB-IoT, repetition unit is P symbol groups.

- For eMTC, repetition unit is one preamble including guard period.

- FFS: Configuration details

Agreement:

Duration of UL transmission segment for UE pre-compensation for PUSCH transmission is a number of PUSCH repetition units configured by the network

- For NB-IoT, repetition unit is MidenticalNPUSCH×NslotUL×Tslot

- For eMTC, repetition unit is  NslotUL×Tslot for sub-PRB allocation, where Tslot = 0.5 ms. For full-PRB allocation, repetition unit is one subframe.

- NOTE1: MidenticalNPUSCH, NslotUL, Tslot are defined in TS 36.211 10.1.2.3 and 10.1.3.6 for NB-IoT

- NOTE2: MsymbUL, MslotULM_^UL_slot is defined in TS 36.211, 5.2.3A for eMTC

- FFS: RAN1 to further discuss valid and invalid subframes

- FFS: Configuration details

Agreement:

For NB-IoT, if a mapping to Nslots slots or a repetition of the mapping in an UL transmission segment for UE pre-compensation for NPUSCH transmission contains a resource element which overlaps with any configured NPRACH resource, the NPUSCH transmission in overlapped Nslots slots is postponed until the next Nslots slots not overlapping with any configured NPRACH resource.

- NOTE: Nslots is defined in TS 36.211, 10.1.3.6

Agreement:

The UL transmission segment duration is configured by the network

- FFS: Details of the configuration signalling.

Agreement:

- For NB-IoT NTN, the network configures one of K values for the UL transmission segment duration of each PRACH preamble format in a k-bit field, where the size of the k-bit field and the number of K candidate values depend on the preamble format.

- Format 0 and format 1: 3-bit field, K=6 candidate values 2.4.(TCP+TSEQ), 4.4.(TCP+TSEQ), 8.4.(TCP+TSEQ), 16.4.(TCP+TSEQ), 32.4.(TCP+TSEQ), 64.4.(TCP+TSEQ)

- Format 2:  2-bit field, K=4 candidate values 2.6.(TCP+TSEQ), 4.6.(TCP+TSEQ), 8.6.(TCP+TSEQ), 16.6.(TCP+TSEQ)

- FFS: Down scoping of K candidate values, size of k-bit field

- FFS: Whether the same segment duration can be used for all preambles within a preamble format

Agreement:

For eMTC, the network configures one of K values for the UL transmission segment duration of PRACH in a k-bit field.

- FFS: K candidate values, size of k-bit field

Agreement:

- For NB-IoT/eMTC NTN, the network configures one of K candidate values for the UL transmission segment duration of NPUSCH/PUSCH in a k-bit field.

  - For NB-IoT, maximum 3-bit field with a maximum number of K=8 candidate values 2 ms, 4 ms, 8 ms, 16 ms, 32 ms, 64 ms, 128 ms, 256 ms

- FFS: Down scoping of K candidate values, size of k-bit field

Agreement:

- The UL transmission segment duration is provided by UE-specific RRC signalling or by signalling in SIB.

- NOTE: the values of UL transmission segment duration for NB-IoT can be different to those for eMTC

## Timing relationship enhancements

Agreement:

For NB-IoT, on receiving UL grant on DCI format N0 in subframe n, NPUSCH Format 1 is transmitted with a delay of Koffset as compared to transmission as per current specification.

Agreement:

For NB-IoT, on receiving a NPDSCH with a RAR message that ends in subframe n, the corresponding Msg3 is transmitted on NPUSCH format 1, with a delay of Koffset as compared to transmission as per current specification.

Agreement:

For NB-IoT, a UE upon detection of a NPDSCH transmission for which it should provide an ACK/NACK feedback, shall transmit the HARQ ACK/NACK with a delay of Koffset as compared to transmission as per current specification.

Agreement:

For NB-IoT, on receiving a timing advance command ending in DL subframe n, the corresponding adjustment of the uplink transmission timing by the received time advance shall be delayed by Koffset as compared to current specification.

Agreement:

For eMTC, on receiving an UL grant via MPDCCH that ends in DL subframe n, PUSCH is transmitted with a delay of Koffset as compared to transmission as per current specification.

Agreement:

For eMTC, on receiving a RAR in a PDSCH that ends in subframe n, PUSCH for Msg3 is transmitted with a delay of Koffset as compared to transmission as per current specification.

Agreement:

For eMTC, when an MPDCCH ending in subframe n activates UL SPS, the time of the first subframe in which the UE is allowed to transmit SPS-PUSCH is delayed by Koffset as compared to transmission per current specification.

Agreement:

For eMTC, on reception of a PDSCH ending in subframe n, the corresponding HARQ-ACK feedback on PUCCH is transmitted with a delay of Koffset as compared to transmission as per current specification.

Agreement:

For eMTC, the ending time for DL physical resources forming a CSI reference resource set is advanced by Koffset as compared to current specification.

Agreement:

For eMTC, for an MPDCCH received in subframe n that triggers aperiodic SRS transmission, SRS is transmitted with a delay of Koffset as compared to transmission as per current specification.

Agreement:

For eMTC, on receiving a timing advance command ending in subframe n, the corresponding adjustment of the uplink transmission timing by the received time advance shall be delayed by Koffset as compared to current specification.

Agreement:

For IoT NTN, support cell-specific Koffset configuration for use during initial access.

Agreement:

For IoT NTN, support the use of UE-specific Koffset in CONNECTED mode.

Agreement:

UE-specific TA reporting is supported in IoT-NTN

- FFS: Detailed contents of report

Conclusion:

In IoT NTN the initialisation of generators for scrambling codes for UL channels and DM-RS shall use the subframe number of the UL channel or UL signal that is indicated by the Koffset-modified timing relationship.

NOTE: In the view of RAN1, this does not necessarily involve a specification change.

Conclusion:

For IoT NTN, no modifications are needed for the calculation in NR NTN for estimate of UE-eNB RTT.

## Others

No TDoc treated

# 3GPP TSG RAN WG1 Meeting #106-bis-e

## Enhancements to time and frequency synchronization

Agreement:

The validity timer for UL synchronization is started/restarted with configured timer validity duration at the epoch time of the assistance information (i.e. serving satellite ephemeris data).

- FFS: Precise definition of epoch time taking into account SIB repetitions

Agreement:

A single validity duration for both serving satellite ephemeris and common TA related parameters is defined at least if serving satellite ephemeris and common TA parameters are signalled in the same SIB message.

Agreement:

Configuration of UL transmission segment is indicated on SIB at least for initial access

- FFS via UE-specific RRC signalling in RRC_CONNECTED.

Agreement:

For eMTC PUSCH, a 3-bit field to indicate K=8 values for the uplink transmission segment duration:

- Full-PRB allocation (unit: subframes): 2 4 8 16 32 64 128 256

- Sub-PRB allocation (unit: resource units): 1 2 4 8 16 32 64 128

Agreement:

For eMTC, a 3-bit field is defined in the SIB to indicate the following K=8 values for the uplink transmission segment duration of PRACH:

(TCP+TSEQ+TGP), 2*(TCP+TSEQ+TGP), 4*(TCP+TSEQ+TGP), 8*(TCP+TSEQ+TGP), 16*(TCP+TSEQ+TGP), 32*(TCP+TSEQ+TGP), 64*(TCP+TSEQ+TGP), 128*(TCP+TSEQ+TGP)

Agreement:

For eMTC, the same value is used for segment durations for all PRACH preambles

Agreement:

For NB-IOT, the same value is used for segment durations for all NPRACH preambles for a particular NPRACH format

Agreement:

In eMTC/NB-IoT, NTA update based on TA Command field in msg2 and MAC CE TA command is used for UL timing alignment correction as follows:

- No extension on TAC 11-bit field in Random Access Response

- When TAC (TA) in Msg2 is received, UE first adjustment and NTA is adjusted as follows: NTA,new = TA 16, where TA is the timing advance command in msg2.

- When TACs (TA) provided within the MAC CE is received, NTA is updated as follows:

  - NTA_new=NTA_old+TA-31.16 ,

- Where TA is the TAC field received in MAC CE command.

Agreement:

RAN1 has discussed the following aspects and leaves it up to RAN2 to specify UE behaviour related to expiry of UL synchronization validity timer and determine which of the following aspects are to be specified:

- Mechanisms for UE to declare loss of UL synchronization including mechanisms for UL synchronization recovery procedure when UL synchronization is lost if UL synchronization validity timer expires in RRC_CONNECTED

  - It is up to RAN2 to specify this new behaviour for connected UE within RLF set of procedures or a new procedure for re-acquiring satellite ephemeris

  - Mechanism for UL synchronization includes re-acquiring the satellite ephemeris and common TA parameters if indicated on SIB

  - A new clause of RLF for loss of UL synchronization if validity timer for UL synchronization expires assuming a new re-interpretation of RLF set of procedures is specified for recovery of UL synchronization with re-acquisition of satellite ephemeris and common TA parameters if indicated

  - Potential additional RACH after re-acquisition of satellite ephemeris and common TA parameters if indicated for the UL synchronization recovery procedure in case of potential residual TA error.

- If validity timer for UL synchronization expires and no UL synchronization recovery mechanisms specified as above, UE behaviour shall declare RLF and go into idle mode  autonomously to re-acquire ephemeris SIB. UE will then need to re-access the cell via Random Access procedure.

- UE signalling to indicate the validity timer for UL synchronization is about to expire

R1-2110652	Draft LS on Validity Timer for UL Synchronization	Moderator (MediaTek)

Final LS approved in R1-2110673

## Timing relationship enhancements

Agreement:

In IoT NTN, for a random access procedure initiated by a N/MPDCCH order, the UE shall delay the transmission of the random access preamble by Koffset as compared to the current specification.

Agreement:

For IoT NTN, with respect to the granularity, configuration, indication and update of K_Offset, the mechanisms concluded in NR-NTN shall be taken as baseline.

Agreement:

For eMTC in IoT NTN, if the UE determines that a preamble retransmission is necessary, the choice of a suitable preamble retransmission subframe shall be delayed by Koffset as compared to current specifications.

Agreement:

For NB-IoT, if the UE has initiated an NPUSCH transmission using pre-configured uplink resources ending in subframe n, the UE shall start or restart to monitor the NPDCCH from DL subframe n+4+K_mac (where K_mac is defined as in NR-NTN).

Agreement:

For eMTC, if the UE has initiated an PUSCH transmission using pre-configured uplink resources ending in subframe n, the UE shall start or restart to monitor the MPDCCH from DL subframe n+4+K_mac (where K_mac is defined as in NR-NTN).

Agreement:

Support PUR at least for GEO-based IoT NTN in Rel-17

FFS: for NGSO-based IoT NTN.

Agreement:

NPDCCH monitoring restrictions have been identified for further checking to see if changes for NB-IoT need to be made for the following cases:

- case 1: MTBG NPUSCH

- case 2: 2 NPUSCH HARQ processes scheduled

- case 3: long single NPUSCH when MTBG or 2HARQ configured

- case 4: single NPUSCH scheduled by DCI format N0 or RAR

- case 5: NPUSCH format 2 in response to DCI format N1

- case 6: NPRACH in response to PDCCH order

- case 7: NPUSCH with same HARQ process when 2 HARQ configured

- case 8: subframes after NPUSCH processing

- case 9: subframes after NPUSCH carrying Msg3

- case 10: NPRACH for SR for long NPRACH transmissions

- case 11: NPRACH for SR for short NPRACH transmissions

- FFS: the changes in each case

- FFS: additional cases

## Others

No TDoc treated

# 3GPP TSG RAN WG1 Meeting #107-e

## Enhancements to time and frequency synchronization

GNSS validity:

Agreement

The UE autonomously determines its GNSS validity duration X and reports information associated with this valid duration to the network via RRC signalling.

- X = {10s, 20s, 30s, 40s, 50s, 60s, 5 min, 10 min, 15 min, 20 min, 25 min, 30 min, 60 min, 90 min, 120 min, infinity}

Agreement

Send LS to RAN2 to take the following RAN1 agreements into consideration to specify the aspects related to GNSS position validity:

- For sporadic short transmission, UE in RRC_CONNECTED should go back to idle mode and re-acquire a GNSS position fix if GNSS becomes outdated

- The UE autonomously determines its GNSS validity duration X and reports information associated with this valid duration to the network via RRC signalling.

  - X = {10s, 20s, 30s, 40s, 50s, 60s, 5 min, 10 min, 15 min, 20 min, 25 min, 30 min, 60 min, 90 min, 120 min, infinity}

- Note: The duration of the short transmission is not longer than the “validity timer for UL synchronization” referred to in the WID objective (but which still needs further discussion for specifying further details)

R1-2112847 –DRAFT LS to RAN2 on GNSS validity duration for IoT NTN

- Final LS in R1-2112848

Validity timer for UL Synchronization:

Agreement

The serving satellite ephemeris and common TA related parameters are signalled in the same SIB message and have the same epoch time.

Agreement

A single validity duration for both serving satellite ephemeris and common TA related parameters is broadcast on the SIB.

Agreement

Validity timer for UL synchronization should be started/restarted with configured timer validity duration at the epoch time of the assistance information.

Agreement

Validity timer duration is configured per cell and indicated to the UE in X bits with:

·       Value range {5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 55, 60, 120, 180, 240}

·       Unit is second

·       FFS Additional values for GEO

For NPUSCH for NB-IoT and PUSCH/PUCCH for eMTC:

Agreement

For eMTC PUCCH/PUSCH with frequency hopping enabled, the UE can adjust the uplink transmit timing when hopping to a new narrowband if the frequency hopping interval is less than or equal to the configured transmission segment duration.

Agreement

For eMTC PUCCH, a 3-bit field to indicate K=7 values for the uplink transmission segment duration:

·         2 4 8 16 32 64 128 subframes

Agreement

For eMTC PUCCH/PUSCH with frequency hopping enabled, the UE can adjust the uplink transmit timing and transmit frequency when hopping to a new narrowband if the frequency hopping interval is less than or equal to the configured transmission segment duration.

Agreement

UE pre-compensation per segment of NPUSCH for NB-IoT and PUSCH/PUCCH for eMTC is applied from one segment to the next segment by using one or more of the following methods if supported by UE implementation

1. UE may drop / Insert samples / Puncture OFDM symbols

2. UE may blank subframes / slots where UE skip a slot or a subframe

The total transmission time is not changed

UE autonomously Drop / insert samples / Puncture OFDM symbols or Blank subframes / slots where UE drops a subframe / slot

The method used for the UE pre-compensation is known to the eNB by a single UE capability

- UE Blank subframes / slots where UE skip a slot or a subframe (slot is based on Sub Carrier Spacing)

FFS Details of method(s) to drop / insert samples, blanking subframes / slots (slot is based on Sub Carrier Spacing)

For NPRACH for NB-IoT and PRACH for eMTC:

Agreement

For NB-IoT, UE pre-compensation per segment of NPRACH is applied from one segment to the next segment by using one or more of the following methods if supported by UE implementation

- UE may drop / Insert samples

- UE may blank subframe / repetition unit where UE drops a subframe / repetition unit

The total transmission time is not changed

FFS Details of method(s) to drop / insert samples / blank subframe / repetition unit

FFS Specification impact

Agreement

For eMTC, UE pre-compensation per segment of PRACH is applied from one segment to the next segment by drop / insert samples in Guard Period of PRACH preamble.

- The total transmission time is not changed

- FFS Details of method(s) to drop / insert samples

UL segmented transmission configuration:

Agreement

UL transmission segment duration with one value X per NPUSCH for NB-IoT and PUSCH/PUCCH for eMTC may be indicated on SIB.

- For NB-IoT/eMTC, X is one of K candidate values for the UL transmission segment duration of NPUSCH/PUSCH/PUCCH

- The value X for eMTC PUSCH applies for full-PRB allocation and should be divided by 2, 4 and 8 for sub-PRB allocation of 6, 3 and 2-out-of-3 tones allocation, respectively.

Agreement

At least UL transmission segment duration with one value X for NPRACH for NB-IoT and PRACH for eMTC may be indicated on SIB

- For NB-IoT/eMTC, X is one of K candidate values for the UL transmission segment duration of NPRACH/PRACH

- FFS One value X, one or more values Xi

Agreement

UL Segmented transmission NPRACH/NPUSCH for NB-IoT is not supported in GEO based on UE feature

.

Agreement

For NB-IoT NTN, the network configures one of K values for the UL transmission segment duration of each PRACH preamble format in a k-bit field, where the size of the k-bit field and the number of K candidate values depend on the preamble format.

- Format 0 and format 1: 3-bit field, K=6 candidate values 2.4.(TCP+TSEQ), 4.4.(TCP+TSEQ), 8.4.(TCP+TSEQ), 16.4.(TCP+TSEQ), 32.4.(TCP+TSEQ), 64.4.(TCP+TSEQ)

- Format 2:  3-bit field, K=5 candidate values 1.6.(TCP+TSEQ), 2.6.(TCP+TSEQ), 4.6.(TCP+TSEQ), 8.6.(TCP+TSEQ), 16.6.(TCP+TSEQ)

Agreement

Support network re-configuration of UL transmission segment by dedicated RRC Signalling

DL Synchronization enhancements:

Agreement

For DL synchronization enhancements:

- Signal Part-of ARFCN indication on MIB for bands where RAN4 cannot introduce a 200 kHz channel raster and the legacy 100 kHz raster is used, otherwise for bands where RAN4 can introduce a 200 kHz channel raster there is no signalling of the part-of ARFCN indication on MIB.

Agreement

For IoT NTN, indicate two LSBs of the ARFCN in the MIB.

R1-2112689 –DRAFT LS to RAN4 on DL synchronization enhancements for IoT NTN

- Final LS in R1-2112768

Synchronization aspects common to IoT NTN and NR NTN:

Agreement

The following agreements from NR NTN are re-used for IoT NTN

The granularity of Common TA is set to be 1.Ts

Conclusion

The following conclusion from NR NTN is re-used for IoT NTN

Conclusion: Do not define a TA margin.

Working assumption:

Higher-layer parameters TACommon, TACommonDrift, TACommonDriftVariation are indicated with the following range, granularity and bits allocation:

| Parameter name | Value range | Granularity | Bits allocation |
| --- | --- | --- | --- |
| TACommon | 0 ...8316827 (i.e: 0… 270.73 ms) | 32.55208 ×10-3 μs | 23 bits |
| TACommonDrift | - 261935… + 261935 (i.e: -53.33 μs/s… +53.33 μs/s) | 0.2×10-3μs/s | 19 bits |
| TACommonDriftVariation | 0…29470 (0…0.60 μs/s2) | 0.2×10-4μs/s2 | 15 bits |
| - Value ranges are given in unit of corresponding granularity |  |  |  |

Agreement

Confirm the working assumption made at RAN1#106-bis-e on serving satellite ephemeris bit allocations for LEO/MEO/GEO based non-terrestrial access network:

- Support serving satellite ephemeris format bit allocations for LEO/MEO/GEO based non-terrestrial access network:

  - Position and velocity state vector ephemeris format is 17 bytes payload.

    - The field size for position (m) is 78 bits

      - Position range is driven by GEO : +/- 42 200 km

      - The quantization step is 1.3m for position

    - The field size for velocity (m/s) is 54 bits

      - Velocity range is driven by LEO@600 km: +/- 8000 m/s

      - The quantization step is 0.06 m/s for Velocity

  - Orbital parameter ephemeris format 18 byte payload

    - Semi-major axis α (m) is 33 bits

      - Range: [6500, 43000]km

    - Eccentricity e is 19 bits

      - Range: ≤ 0.015

    - Argument of periapsis ω (rad) is 24 bits

      - Range: [0, 2π]

    - Longitude of ascending node (Ω rad) is 21 bits

      - Range: [0, 2π]

    - Inclination i (rad) is 20 bits

      - Range: [- π/2 , + π/2]

    - Mean anomaly M (rad) at epoch time to is 24 bits

      - Range: [0, 2π]

Agreement

Using indicated Higher-layer Common TA parameters, if configured, the UE can determine the one-way propagation time ( Delaycommon) used for NTA, common  calculation as follows:

Delaycommont= DCommon tepoch+ DCommonDrift× t-tepoch+DCommonDriftVariation× t-tepoch2

Where:

- DCommon = TACommon 2, DCommonDrift=TACommonDrift2 and DCommonDriftVariation=TACommonDriftVariation2

- TACommon, TACommonDrift and TACommonDriftVariation are Common TA parameter defined in RAN1 Meeting #106-bis-e

- Delaycommont is the distance between the satellite and the uplink time synchronization reference point divided by the speed of light. DL and UL are frame aligned at the reference point with an offset given by NTA,offset.

- NTA, common is derived by the UE based on Delaycommont to pre-compensate the two-way transmission delay between the uplink time reference point and the satellite.

## Timing relationship enhancements

Agreement

For IoT NTN, signalling one value for cell-specific K_offset in system information is supported.

Agreement

For IoT NTN, the unit of K_offset is subframe based on a 15kHz subcarrier spacing (i.e. 1 ms).

- Further discuss the case where UL is using 3.75 kHz SCS

Agreement

For IoT NTN, the UE specific K_offset is provided and updated by the network using MAC CE.

Agreement

For IoT NTN, the information of K_mac is carried in system information.

Agreement

For IoT NTN, the unit of K_mac is subframe based on a 15kHz subcarrier spacing (i.e. 1 ms).

- Further discuss the case where UL is using 3.75 kHz SCS

Agreement

Modification of the designation of subframes with NPDCCH monitoring restrictions is needed for at least Cases 1 to 6.

Agreement

Whether/how the “indicated value” of K_offset is translated into number of slots for different numerologies (i.e., 15 kHz and 3.75 kHz) is left to the spec-editor.

- This resolves the bullet from previous agreement: Further discuss the case where UL is using 3.75 kHz SCS

Agreement

For IoT NTN, adopt the NR NTN agreement without modification for FR1: (a) the value range (i.e. 1 ms), (b) the quantity signalled (e.g. a differential UE specific K_offset) for the UE specific K_offset.

Agreement

For IoT NTN, adopt the NR NTN agreement without modification for FR1 for the value range of Kmac.

Leave it to spec editor to formulate in the specs the NPDCCH monitoring restrictions for Cases 1 to 6.

Explanatory Note for editor

When the UE changes from receiving on the DL to transmitting on the UL (or vice versa), immediately before/after the DL/UL switch the UE is not required to monitor an NPDCCH candidate in some DL subframes. The designation of these subframes in the spec needs to take the “effect” of the TA into consideration. There may be multiple ways to capture this in the specifications for (at least) Cases 1 to 6. Two options (in principle) are described below, to guide the spec editor to capture this as best he/she sees it. Examples of where the changes may apply for cases 1 to 6 can be found as examples in appendix A in R1-2112554.

Option 1: The DL subframes during which the UE is not required to monitor an NPDCCH candidate are described in terms of downlink subframe timing. This would typically involve inserting a “-TA” term in their indexing.

Option 2: The DL subframes during which the UE is not required to monitor an NPDCCH candidate are described in terms of uplink subframe timing using the indexing of the UL subframes that coincide in time with the DL subframes in question.

Agreement

Network can configure UE-specific TA reporting either a TA or UE location for connected mode UE

- In case a TA is configured, NR NTN solutions are a baseline for the following UE-specific TA handling issues,

  - Signaling – quantity (full or delta), range, number of bits

  - Granularity of report

  - Frequency of reporting

  - Means of reporting

  - NOTE: Any changes needed for IoT NTN can be made.

- In case the UE location is configured, RAN2 will design solutions for the UE location information, and it is left to RAN2 to decide whether to support UE location reporting

## Others

No TDoc treated

# 3GPP TSG RAN WG1 Meeting #108-e

## Enhancements to time and frequency synchronization

The RAN1#108-e agreements are listed below:

TP to TS 36.300:

Agreement

- For IoT NTN, capture into 36.300 a stage-2 description of concept of K_offset, K-mac, UE pre-compensation of timing and frequency pre-compensation/adjustment for uplink transmission with modification as needed

NOTE: NR NTN agreements on TS 38.300 relating to a stage-2 description of concept of K_offset, K-mac, UE pre-compensation of timing and frequency pre-compensation/adjustment for uplink transmission can be adopted for TS 36.300 with modification as needed and can include aspects specific to IoT NTN as needed.

TPs to TS 36.211:

The TP below for TS 36.211 Section 8.1 is endorsed

--------------------------------------- Start of TP for 3GPP TS 36.211 ----------------------------------------

8.1	Uplink-downlink frame timing

<Unchanged Text Omitted>

Transmission of the uplink radio frame number  from the UE shall start TTA=NTA+NTA,offset+NTA,adjcommon+NTA,adjUETs seconds before the start of the corresponding downlink radio frame at the UE.

Figure 8.1-1: Uplink-downlink timing relation

<Unchanged Text Omitted>

---------------------------------------- End of TP for 3GPP TS 36.211 ----------------------------------------

The TP below for TS 36.211 Section 8.1 is endorsed

---------------------------------------- Start of TP for 3GPP TS 36.211 ----------------------------------------

8.1	Uplink-downlink frame timing

<Unchanged Text Omitted>

The quantity NTA,adjUE is computed by the UE based on UE position and serving satellite-ephemeris-related higher-layers parameters if configured, otherwise NTA,adjUE=0.

<Unchanged Text Omitted>

---------------------------------------- End of TP for 3GPP TS 36.211 ----------------------------------------

The TP below for TS 36.211 Section 8.1 is endorsed

---------------------------------------- Start of TP for 3GPP TS 36.211 ----------------------------------------

8.1	Uplink-downlink frame timing

<Unchanged Text Omitted>

The quantity NTA,adjcommon is derived from the higher-layer parameters TACommon, TACommonDrift, and TACommonDriftVariation if configured (see Clause 4.2.3 in [TS 36.213]), otherwise NTA,adjcommon=0.

<Unchanged Text Omitted>

---------------------------------------- End of TP for 3GPP TS 36.211 ----------------------------------------

Aspects common to IoT NTN / NR NTN:

Agreement:

- Modify bit allocations for orbital parameters ephemeris format as follows:

  - Orbital parameters are indicated in 21 bytes payload:

    - Semi-major axis α (m) is 33 bits

      - Range: from 6500 km to 43000 km

      - The quantization step is 4.249 ×10-3m

    - Eccentricity e is 20 bits

      - Range: ≤ 0.015

      - The quantization step is 1.431 ×10-8

    - Argument of periapsis ω (rad) is 28 bits

      - Range: from 0 to 2π

      - The quantization step is 2.341 ×10-8 rad

    - Longitude of ascending node (Ω rad) is 28 bits

      - Range: from 0 to 2π

      - The quantization step is 2.341  ×10-8 rad

    - Inclination i (rad) is 27 bits

      - Range: from - π/2  to + π/2

      - The quantization step is 2.341  ×10-8 rad

    - Mean anomaly M (rad) at epoch time to is 28 bits

      - Range: from 0 to 2π

      - The quantization step is 2.341  ×10-8 rad

Epoch time:

Agreement

For epoch time signaling for IoT NTN:

- When explicitly provided through SIB, Epoch time of assistance information (i.e. Serving satellite ephemeris and Common TA parameters) is the starting time of a DL sub-frame, indicated by a SFN and a sub-frame number signaled together with the assistance information.

- When provided through dedicated signaling, epoch time of assistance information (i.e. Serving satellite ephemeris and Common TA parameters) is the starting time of a DL sub-frame, indicated by a SFN and a sub-frame number.

- The reference point for epoch time of the serving satellite ephemeris and Common TA parameters is the uplink time synchronization reference point.

Working assumption

Adopt NR NTN solution for interpretation SFN indicating Epoch time.

Common TA:

Working assumption

Adopt NR NTN solution for negative TACommonDriftVariation values.

Validity timer values for GEO:

Agreement

First discuss for additional values of validity timer for GEO in NR NTN AI 8.4.2. For IoT NTN, adopt the NR NTN agreement without modification for additional values of validity timer for GEO.

RAN2 related aspects of IoT NTN:

Conclusion

RAN1 can wait for RAN2 to conclude discussions on GNSS Measurements.

Conclusion

RAN1 can wait for RAN2 to conclude discussions on validity timer / re-acquisition on NTN-specific SIB.

Conclusion

RAN2 can further discuss when the UE-specific TA report is reported.

## Timing relationship enhancements

Agreement

RAN1 kindly suggests to spec editor of TS 36.213 to move the following text to the head of section 6.1.1:

========= Unchanged Text Omitted ==========

Throughout this clause, for a BL/CE UE, if the UE is configured with the higher layer parameter CellSpecificKoffset,

-	Koffset= Kcell_offset-KUE_offset where

Kcell_offset is the parameter CellSpecificKoffset provided by higher layers, and

KUE_offset is the parameter UESpecificKoffset provided by higher layers, otherwise KUE_offset=0

otherwise,

-	Koffset=0, Kcell_offset=0.

======== Unchanged Text Omitted ===============

Agreement

RAN1 kindly suggests to spec editor of TS 36.213 to adopt the following TP for section 9.1.5:

<<<<< Start of TP to TS 36.213 section 9.1.5 >>>>>>>

If the UE has initiated a PUSCH transmission using preconfigured uplink resource ending in subframe n, the UE shall monitor the MPDCCH UE-specific search space in a search space window starting in subframe n+4+Kmac Kmac with duration given by higher layer parameter pur-MPDCCH-SS-window-duration where Kmac is provided by higher layer parameter K-mac, otherwise Kmac=0. Upon detection of a MPDCCH with DCI format 6-0A/6-0B with CRC scrambled by PUR-RNTI intended for the UE within the search space window and the corresponding DCI is for PUR ACK/fallback indication (as defined in [4]), the UE is not required to monitor the MPDCCH UE-specific search space for the remaining search space window duration.

<<<<< End of TP to TS 36.213 section 9.1.5 >>>>>>>

Agreement

RAN1 kindly suggests to spec editor of TS 36.213 to adopt the following TP for section 16.5.1:

<<<< START of TEXT PROPOSAL for TS36.213 section 16.5.1 >>>>>>>>>>>>>

16.5.1	UE procedure for transmitting format 1 narrowband physical uplink shared channel

NPUSCH format 1 transmission can be scheduled by a NPDCCH with DCI format N0, or the transmission can correspond to using preconfigured uplink resource configured by higher layers. Transmission using preconfigured uplink resource is initiated by higher layers as specified in [14] , while retransmission of transport blocks transmitted using preconfigured uplink resource are scheduled by a NPDCCH with DCI format N0.

A UE shall upon detection on a given serving cell of a NPDCCH with DCI format N0 ending in NB-IoT DL subframe n scheduling NPUSCH intended for the UE, perform, at the end of

-	n+k0+Koffset DL subframe for FDD,

-	k0 NB-IoT UL subframes following the end of n+8+Koffset subframe for TDD,

a corresponding NPUSCH transmission using NPUSCH format 1 in N consecutive NB-IoT UL slots ni with i = 0, 1, …, N-1 according to the NPDCCH information where

-	subframe n is the last subframe in which the NPDCCH is transmitted and is determined from the starting subframe of NPDCCH transmission and the DCI subframe repetition number field in the corresponding DCI; and

-	, where the value of is determined by the repetition number field in the corresponding DCI (see Clause 16.5.1.1), the value of is determined by the resource assignment field in the corresponding DCI (see Clause 16.5.1.1), the value of  is the number of NB-IoT UL slots of the resource unit (defined in clause 10.1.2.3 of [3]) corresponding to the  allocated number of subcarriers (as determined in Clause 16.5.1.1) in the corresponding DCI, and the value of is determined by the Number of scheduled TB for Unicast field, if present, in the corresponding DCI,  otherwise

-	n0 is the first NB-IoT UL slot starting after the end of subframe n+k0+Koffset for FDD

-	n0 is the first NB-IoT UL slot starting after k0 NB-IoT UL subframes following the end of n+8+Koffset subframe for TDD

-	value of k0 is determined by the scheduling delay field () in the corresponding DCI according to Table 16.5.1-1 for FDD and Table 16.5.1-1A for TDD

<<<< END of TEXT PROPOSAL for TS36.213 section 16.5.1 >>>>>>>>>>>>>

Agreement

RAN1 kindly suggests to spec editor of TS 36.213 to adopt the following TP for section 8.0:

<<<< START of TEXT PROPOSAL for TS36.213 section 8.0 >>>>>>>>>>>>>

A BL/CE UE shall upon detection on a given serving cell of an MPDCCH with DCI format 6-0A/6-0B scheduling PUSCH intended for the UE, perform a corresponding PUSCH transmission in subframe(s) ni = n+ki+Koffset if a transport block(s) corresponding to the HARQ process(es) of the PUSCH transmission is generated as described in [8] with i = 0, 1, …, NTBN-1 according to the MPDCCH, where

-	subframe n is the last subframe in which the MPDCCH is transmitted;

-	the value of is the number of scheduled TB determined by the corresponding DCI if present,  otherwise;

-	 and the value of  is determined by the repetition number field in the corresponding DCI, where

-	if the UE is configured with higher layer parameter ce-pdsch-puschEnhancement-config with value 'On' are given by {1,2,4,8,12,16,24,32}

-	otherwise, are given in Table 8-2b and Table 8-2c; and

-	if the UE is configured with higher layer parameter ce-PUSCH-SubPRB-Config-r15, and the PUSCH resource assignment in the corresponding DCI is using uplink resource allocation type 5,  where N ≤ 32 for CE Mode A and N ≤ 2048 for CE Mode B,  is defined in [3] and  is determined according to procedure in clause 8.1.6,  otherwise

-	in case N>1, subframe(s) n+ki+Koffset with i=0,1,…, NTBN-1 are NTBN consecutive BL/CE UL subframe(s) starting with subframe n+x+Koffset, and in case N=1, k0=x;

<<<< Portion of specification removed >>>>>

-	for FDD, x = 4;

<<<< END of TEXT PROPOSAL for TS36.213 section 8.0 >>>>>>>>>>>>>

Agreement

In IoT NTN, the Koffset value signalled in system information (cell specific Koffset) is always used for NPDCCH and MPDCCH ordered NPRACH and PRACH timing relationships, respectively.

Agreement

RAN1 kindly suggests to spec editor of TS 36.213 to adopt the following TP for section 16.3.2:

<<<< START of TEXT PROPOSAL for TS36.213 section 16.3.2 >>>>>>>>>>>>>

In case a random access procedure is initiated by a "PDCCH order" ending in subframe n, the UE shall, if requested by higher layers, start transmission of random access preamble at the end of the first subframe n+k2+Kcell_offset, , where a NPRACH resource is available.

<<<< END of TEXT PROPOSAL for TS36.213 section 16.3.2 >>>>>>>>>>>>>

Working assumption

For IoT NTN, the unit of K_mac and Koffset when subcarrier spacing is 3.75kHz is 1 ms.

Agreement

Confirm the WA on units of Kmac and Koffset with 3.75kHz SCS:

Working assumption

For IoT NTN, the unit of K_mac and Koffset when subcarrier spacing is 3.75kHz is 1 ms.

Agreement

For IoT NTN, calculate UE-eNB RTT using the following equation: RTTUEeNB=floor(TTATf)+Kmac

where Tf = subframe duration (1ms).

Agreement

The TP below for TS 36.213 Clause 16.1.2 is endorsed

<<< Start of TP to Clause 16.1.2, TS 36.213 >>>

For a timing advance command reception ending in DL subframe n, the corresponding adjustment of the uplink transmission timing shall apply from the first available NB-IoT uplink slot following the end of n+12+Koffset DL subframe and the first available NB-IoT uplink slot is the first slot of a NPUSCH transmission.

<<< End of TP to Clause 16.1.2, TS 36.213 >>>

## Others

# Reference

| - RP-211601, Rapporteur (MediaTek), NB-IoT/eMTC support for Non-Terrestrial Networks, RAN#92-e, June 2021. - RP-211456, 3GPP TR 36.763 v1.0.0: Study on Narrow-Band Internet of Things (NB-IoT) / enhanced Machine Type Communication (eMTC) support for Non-Terrestrial Networks (NTN), RAN#92-e, June 2021 - R1-2110434, Report of RAN1#106-e meeting, RAN1#106bis-e, October 2021 - 3GPP TSG RAN WG1 Meeting #106bis-e RAN1 Chairman’s Notes - R1-2202915, Moderator (MediaTek), Summary #4 of AI 8.15.2 Enhancements to time and frequency synchronization, RAN1#108-e, February 2022 - R1-2112803, Moderator (MediaTek), Summary #4 of AI 8.15.2 Enhancements to time and frequency synchronization, RAN1#107-e, November 2021 - R1-2110645, Moderator (MediaTek), Summary #5 of AI 8.15.2 Enhancements to time and frequency synchronization, RAN1#106bis-e, October 2021 - R1-2108640, Moderator (MediaTek), Summary #7 of AI 8.15.2 Enhancements to time and frequency synchronization, RAN1#106e, August 2021 - R1-2202587, Moderator (Sony), FL summary#4 of AI 8.15.3, RAN1#108-e, February 2022 - R1-2112554, Moderator (Sony), FL summary#4 of AI 8.15.3, RAN1#107-e, November 2021 - R1-2110534, Moderator (Sony), FL summary#4 of AI 8.15.3, RAN1#106bis-e, October 2021 - R1-2108519, Moderator (Sony), FL summary#5 of AI 8.15.3, RAN1#106-e, August 2021 - R1-2201220, Summary of [108-e-R17-RRC-IoT-NTN] IoT over NTN, RAN1#108-e, February2022 - R1-2111376, Summary of [107-e-R17-RRC-IoT-NTN] IoT over NTN, RAN1#107-e, November 2021 - R1-2111377, List of IoT over NTN Rel-17 RRC parameters, RAN1#107-e, November 2021 |  |
| --- | --- |