3GPP TSG-RAN WG1 #108-e		R1-2202940

eMeeting, Feb 21 – Mar 03, 2022

Source: 	Rapporteur (Ericsson)

Title:      	Summary of RAN1 agreements for Rel17 NR DSS WI

Agenda item:	8.13

Document for:	Information

# 1 Introduction

This document provides a list of RAN1 agreements made until RAN1#108-e for the Rel17 NR DSS WI [1].

# 2. RAN1 Agreements

## 2.1 Cross-carrier scheduling from SCell to PCell/PSCell

### RAN1#102-e (Aug 2020)

Agreements:

- Following scheduling combinations are allowed/not allowed when cross-carrier scheduling from an SCell to PCell/PSCell is configured

  - self-scheduling on PCell/PSCell is allowed

  - cross-carrier scheduling from PCell/PSCell to another SCell is not allowed

  - self-scheduling on the ‘SCell used for scheduling PCell/PSCell’ is allowed

  - cross-carrier scheduling from the ‘SCell used for scheduling PCell/PSCell’ to another serving cell is allowed

  - cross-carrier scheduling from another serving cell to the ‘SCell used for scheduling PCell/PSCell’ is not allowed

- FFS: Search space and DCI format handling for the allowed cases above

Agreements:

- Configuring 2 or more Scells to schedule the PCell/PSCell is not allowed

### RAN1#103-e (Oct/Nov 2020)

Agreements:

- When CCS from an SCell (sSCell) to PCell/PSCell is configured, UE monitors Type 0/0A/1/2 CSS sets (for the DCI formats associated with those SS sets) only on the PCell/PSCell and not on the sSCell

  - Note: UE monitors Type 0/0A/2 CSS only on PCell while Type 1 CSS can be monitored on PCell/PSCell

Conclusion

- When CCS from sSCell to PCell/PSCell is configured, the configuration of Type 3 CSS set for DCI formats 2_0, 2_1, 2_2, 2_3, 2_4 and applicability of the information in the DCI formats are the same as in Rel-15/Rel-16

  - FFS: DCI format 2_5 and DCI Format 2_6 handling

- Note: The SCell configured with CCS to Pcell/PSCell is referred to as ‘sSCell’

Conclusion

- When the PCell/PSCell and sSCell use different numerologies, the PDSCH reception preparation time between the PDCCH on the sSCell and the PDSCH on the PCell/PSCell is applied (i.e., as specified in TS38.214 Section 5.5).

Agreements:

- Discuss in RAN1#104-e how to handle ‘DCI formats 0_1,1_1,0_2,1_2 scheduling PDSCH/PUSCH on PCell/PSCell’ from USS set(s), when CCS from sSCell to PCell/PSCell is configured.. Below alternatives can be considered in the discussion (other alternatives are not precluded)

- Below alternatives can be considered in the discussion (other alternatives are not precluded)

  - Alt 1: When CCS from sSCell to PCell/PSCell is configured, UE cannot be configured to monitor DCI formats 0_1,1_1,0_2,1_2 on PCell/PSCell USS set(s), and can be configured to monitor them only on the sSCell USS set(s)

  - Alt 2: When CCS from sSCell to PCell/PSCell is configured, UE can be configured to monitor DCI formats 0_1/1_1/0_2/1_2 on PCell/PSCell USS set(s), and/or on sSCell USS set(s). The PDCCH monitoring is based on following alternatives (other alternatives are not precluded)

    - Alt 2-1:

      - UE can monitor DCI formats 0_1,1_1,0_2,1_2 on both PCell USS set(s) and sSCell USS sets simultaneously

        - FFS activation/deactivation of scheduling from sSCell to PCell/PSCell

    - Alt 2-2:

      - Dynamic switching of PDCCH monitoring of DCI formats 0_1,1_1,0_2,1_2 between monitoring on PCell/PSCell USS sets and monitoring on sSCell USS sets is supported

        - FFS: Details of switching mechanism (e.g. based on SS group switching, based on BWP switching,…)

      - UE does not monitor DCI formats 0_1,1_1,0_2,1_2 on both PCell USS set(s) and sSCell USS sets simultaneously

    - Alt 2-3:

      - UE does not monitor the same DCI format on both PCell USS set(s) and sSCell USS sets simultaneously. UE can monitor some DCI formats on sSCell USS sets and other DCI formats on PCell/PSCell USS sets simultaneously

    - Alt 2-4:

      - The USS set(s) on PSCell/PCell and the USS set(s) on sSCell are configured such that UE does not monitor DCI formats 0_1,1_1,0_2,1_2 on both PCell USS set(s) and sSCell USS set(s) simultaneously

- FFS following aspects

  - Impact of sSCell activation/deactivation and sSCell dormancy

  - Impact on BD/CCE limit handling including considering PDCCH monitoring on CSS sets and PDCCH monitoring of ‘DCI formats 0_0, 1_0 scheduling PUSCH/PDSCH on PCell/PSCell’

  - Whether PDCCH overbooking on sSCell is supported or not supported and impact (if any) on overbooking handling on PCell/PSCell

  - Impact from different numerologies between PDCCH on the PCell/PSCell and that on the sSCell

  - Whether or not to have mechanism for activation/deactivation of scheduling from sSCell to PCell/PSCell

  - USS configuration details (e.g. handling of USS type (self-scheduling, cross carrier scheduling) for a configured USS set configured for scheduling of in PCell/PSCell)

### RAN1#104-e (Jan/Feb 2021)

Agreement

When CCS from sSCell to PCell/PSCell is configured,

- Out of order scheduling is not allowed between a) PDSCH on PCell/PSCell scheduled by PDCCH on PCell/PSCell and b) PDSCH on PCell/PSCell scheduled by PDCCH on sSCell

- Out of order scheduling is not allowed between a) PUSCH on PCell/PSCell scheduled by PDCCH on PCell/PSCell and b) PUSCH on PCell/PSCell scheduled by PDCCH on sSCell

FFS: Whether this agreement requires RAN1 specification impact.

Agreement

When CCS from sSCell to PCell/PSCell is configured,

- Simultaneous reception of a) unicast PDSCH on PCell/PSCell scheduled from PCell/PSCell and b) unicast PDSCH on PCell/PSCell scheduled from sSCell is not allowed

- Simultaneous transmission of a) PUSCH on PCell/PSCell scheduled from PCell/PSCell and b) PUSCH on PCell/PSCell scheduled from sSCell is not allowed

- Note: Simultaneous implies full/partial time overlapping

FFS: Whether this agreement requires RAN1 specification impact.

Agreement

- When CCS from sSCell to PCell/PSCell is configured, CA activation/deactivation operation for the sSCell is supported

Working Assumption

- When CCS from sSCell to PCell/PSCell is configured, UE can be configured to monitor DCI formats 0_1/1_1/0_2/1_2 that schedule PDSCH/PUSCH on PCell/PSCell on PCell/PSCell USS set(s), and/or on sSCell USS set(s)

- The WA to be confirmed after agreements are made on PDCCH BD/CCE handling and PDCCH overbooking handling for CCS from sSCell to PCell/PSCell

- Specs also allow UEs supporting functionality of only Alt-1. Capability signaling details, if any, can be handled during the UE capability discussion for Rel17

- FFS: Whether the UE can monitor PDCCH from both cells in the same slot.

Agreement

- When CCS from sSCell to PCell/PSCell is configured, UE monitors ‘DCI formats 0_0 and 1_0 in CSS that schedule PDSCH/PUSCH on PCell/PSCell’ only on the PCell/PSCell and not on the sSCell

### RAN1#104b-e (Apr 2021)

Agreement

When CCS from sSCell to PCell/PSCell is configured

- CIF=0 used for sSCell self-scheduling, and CIF for sSCell to PCell cross-carrier scheduling is explicitly configured using RRC signalling

Agreement

PDCCH overbooking on sSCell USS set(s) is not allowed

### RAN1#105-e (May 2021)

Agreement

Two types of UEs (Type A and Type B) can support CCS from sSCell to P(S)Cell

- For Type A UE

  - At least following search space sets on P(S)Cell and search space sets on sSCell are configured so that the UE does not monitor them in overlapping [slot/symbol] of P(S)Cell and sSCell

    - search space sets on P(S)Cell

      - USS sets for DCI formats 0_1,1_1,0_2,1_2 (if supported for Type A UE)

      - USS sets for DCI formats 0_0,1_0

      - Type3-CSS set(s) for DCI formats 1_0/0_0 with C-RNTI/CS-RNTI/MCS-C-RNTI

    - search space sets on sSCell

      - USS set(s) for scheduling P(S)Cell

  - FFS: BD/CCE handling

- For Type B UE

  - Following search space sets on P(S)Cell and search space sets on sSCell can be configured so that the UE monitors them in overlapping [slot/symbol] of P(S)Cell and sSCell

    - search space sets on P(S)Cell

      - USS sets for DCI formats 0_0,1_0

      - Type3-CSS set(s) for DCI formats 1_0/0_0 with C-RNTI/CS-RNTI/MCS-C-RNTI

    - search space sets on sSCell

      - USS set(s) for scheduling P(S)Cell

  - For handling ‘USS sets for scheduling P(S)Cell’ on P(S)Cell and/or on sSCell for DCI formats 0_1,1_1,0_2,1_2

    - Alt 2-1 is adopted

  - There is no restriction on Type-0/0A/1/2-CSS sets configurations

  - FFS: BD/CCE handling

- For Type A and/or Type B UE

  - FFS: switching to ‘normal’ PDCCH monitoring on P(S)Cell when sSCell is deactivated

- FFS: Whether Type A is specified or is Type-B with restrictions (as part of UE features discussion)

- FFS: Whether the UE can be configured with unaligned CA

- FFS: Whether the above applies for multicast PDSCH

### RAN1#106-e (Aug 2021)

Agreement

Specification supports dormant BWP operation on sSCell for a UE is configured CCS from sSCell to P(S)Cell.

Agreement

- When CCS from sSCell to P(S)Cell is configured for a UE

  - at least the number of PDCCH monitoring candidates monitored on sSCell (for scheduling P(S)Cell) is indicated to the UE using the SS set linking approach as in Rel16

  - FFS: If any modifications to Rel16 approach are introduced for monitoringSlotPeriodicityAndOffset, monitoringSymbolsWithinSlot, duration for the PDCCH monitoring candidates monitored on sSCell (for scheduling P(S)Cell)

Agreement

- At least for Type B UE, when the UE is configured for CCS from sSCell to P(S)Cell and when P(S)Cell SCS (μ) is less than or equal to sSCell SCS (μ1), and at least when UE is not provided monitoringCapabilityConfig for any cell, down select one from [based on Option A/C] or [based Option C] below

  - [based on Option A/C]

    - On P(S)Cell (for self-scheduling)

      - UE is not required to monitor more than α*minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ  PDCCH BD candidates per P(S)Cell slot

    - On sSCell (for cross-carrier scheduling to P(S)Cell)

      - UE is not required to monitor more than [minMPDCCHmax,slot,μ1,MPDCCHtotal,slot,μ1 or MPDCCHmax,slot,μ1] PDCCH BD candidates per sSCell slot (Note: this is assumed per Rel16)

      - UE is additionally not required to monitor more than β*minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ PDCCH BD candidates per P(S)Cell slot

    - 0≤α≤1  and0≤β≤1   are based on RRC configuration and at least cases of α+β≤1 are supported

    - FFS the following for [based on Option A/C]

      - Distribution of PDCCH BD candidates between multiple sSCell slots overlapping a P(S)Cell slot including whether the above additional BD limitation is defined per sSCell slot or per P(S)Cell slot.

        - Discuss further using following alternatives as starting point (other alternatives/further refinement of alternatives not precluded)

          - Alt1

            - The additional BD limitation is per sSCell slot with further limitation that UE is not required to monitor more than 2µ-µ1*β*minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ PDCCH BD candidates per sSCell slot

          - Alt 2

            - The additional BD limitation is per P(S)Cell slot and no further restrictions

          - Alt 3

            - The additional BD limitation is per P(S)SCell slot with below further limitation

              - All search space configurations monitored on sSCell for cross-carrier scheduling to P(S)Cell are within a single span of 3 consecutive OFDM symbols within a duration spanning P(S)Cell slot

      - Whether/how the definition of MPDCCHtotal,slot,μ orMPDCCHtotal,slot,μ1 is modified compared to Rel16 when UE is configured with CCS from sSCell to P(S)Cell

      - Whether separate α and  β  are configured by RRC or if β=1-α and only  α is configured

  - [based on Option C]

    - On P(S)Cell (for self-scheduling)

      - UE is not required to monitor more than mins1*MPDCCHmax,slot,μ,MPDCCHtotal,slot,μ  PDCCH BD candidates per P(S)Cell slot

    - On sSCell (for cross-carrier scheduling to P(S)Cell)

      - UE is not required to monitor more than mins2*MPDCCHmax,slot,μ1,MPDCCHtotal,slot,μ1 PDCCH BD candidates per sSCell slot

    - When determining MPDCCHtotal,slot,μ and MPDCCHtotal,slot,μ1

      - P(S)Cell self-scheduling is counted by applying scaling factor s1,

      - sSCell to PCell scheduling is counted additionally (assuming SCS of sSCell) by applying scaling factor s2

    - 0≤s1≤1  and 0≤s2≤1

    - FFS the following

        - Allowed combinations of s1 and s2 , and whether they are fixed or configured via RRC

        - Whether/how the definition of MPDCCHtotal,slot,μ orMPDCCHtotal,slot,μ1 is modified compared to Rel16 when UE is configured with CCS from sSCell to P(S)Cell

- FFS the following

  - Multi-TRP handling

  - PDCCH BD handling when monitoringCapabilityConfig = r16monitoringcapability is configured for any cell

Agreement

- Endorse below TP to 38.300 from RAN1 perspective

- Send LS to RAN2 with the TP and list of RAN1 agreements, to update Stage 2 spec are needed to reflect the RAN1 agreements

----------------------------------------- start TP1 for 38.300 v.xyz -------------------------------------------

10.8	Cross Carrier Scheduling

Cross-carrier scheduling with the Carrier Indicator Field (CIF) allows the PDCCH of a serving cell to schedule resources on another serving cell but with the following restrictions:

-	Cross-carrier scheduling does not apply to Pcell i.e. When cross-carrier scheduling from an SCell to Pcell is not configured, Pcell can only be is always scheduled via its PDCCH;

-	When cross-carrier scheduling from an SCell to Pcell is configured, PDCCH on that SCell can schedule Pcell’s PDSCH and PUSCH, and PDCCH on the Pcell can also schedule Pcell’s PDSCH and PUSCH, and PDCCH on Pcell cannot schedule PDSCH and PUSCH on any other cell. Only one SCell can be configured to be used for cross-carrier scheduling to Pcell;

-	When an SCell is configured with a PDCCH, that cell’s PDSCH and PUSCH are always scheduled by the PDCCH on this SCell;

-	When an SCell is not configured with a PDCCH, that SCell’s PDSCH and PUSCH are always scheduled by a PDCCH on another serving cell;

-	The scheduling PDCCH and the scheduled PDSCH/PUSCH can use the same or different numerologies.

--------------------------------------------------- end TP1 -----------------------------------------------

Draft LS R1-2108576 is endorsed in principle

Final LS R1-2108662 is endorsed

### RAN1#106b-e (Oct 2021)

Agreement

Option A is supported in Rel-17

- At least for Type B UE, when the UE is configured for CCS from sSCell to P(S)Cell and when P(S)Cell SCS (μ) is less than or equal to sSCell SCS (μ1),[and at least when UE is not provided monitoringCapabilityConfig for any cell, ]

  - Option A

    - On P(S)Cell (for self-scheduling)

      - UE is not required to monitor more than α*minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ  PDCCH BD candidates per P(S)Cell slot

    - On sSCell (for cross-carrier scheduling to P(S)Cell)

      - UE is not required to monitor more than [minMPDCCHmax,slot,μ1,MPDCCHtotal,slot,μ1 or MPDCCHmax,slot,μ1] PDCCH BD candidates per sSCell slot

      - UE is additionally not required to monitor more than (1-α)*minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ PDCCH BD candidates per P(S)Cell slot

    - 0≤α≤1  is based on RRC configuration

    - α*minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ   is used for P(S)Cell overbooking procedure

    - When determining MPDCCHtotal,slot,μ and MPDCCHtotal,slot,μ1

      - P(S)Cell self-scheduling is counted by applying scaling factor s1

      - sSCell to P(S)Cell scheduling is counted additionally (assuming SCS of sSCell) by applying scaling factor s2

      - s1=1 and s2=0, FFS other s1 and s2

      - 0≤s1≤1  and 0≤s2≤1 are based on RRC configuration

        - FFS: additional constraints on s1 and s2 e.g., 1 ≤ s1+s2 ≤ 2 or s1 + s2 ≤ 1

      - Note: Ncellscap is as in Rel16

    - UE capability/incapability indication for below to be discussed as part of UE features discussion

      - All search space configurations monitored on sSCell for cross-carrier scheduling to P(S)Cell are within a single span of [3] consecutive OFDM symbols within a duration spanning P(S)Cell slot

    - Same approach as above is used for CCE limits

      - FFS: Separate vs. same RRC configured scaling factors (corresponding to α) for BD and CCE limits.

- When P(S)Cell SCS (μ) is larger than sSCell SCS (μ1), for CCS from sSCell to P(S)Cell and, it is not supported Rel-17 DSS.

Conclusion

- When sSCell to PCell cross-carrier scheduling is configured, DCI format 2_6 (if configured) is monitored only on P(S)Cell

Working Assumption

- When CIF for sSCell to PCell cross-carrier scheduling is configured, non-fallback DCI formats on P(S)Cell include same number of CIF bits as the corresponding non-fallback DCI formats on sSCell that are used for sSCell to P(S)Cell scheduling

Conclusion

- A UE configured for cross-carrier scheduling from SCell to P(S)Cell can also be configured with unaligned CA (i.e., using  ca-SlotOffset ), and a non-zero value for ca-SlotOffset can be configured at least for SCells other than the sSCell

  - FFS: Whether case when sSCell is configured with non-zero ca-SlotOffset is supported and any associated capability signalling

- Note: No additional L1 spec impact related to ca-SlotOffset had been identified

Conclusion

- When CCS from sSCell to P(S)Cell is configured for a UE

  - monitoringSlotPeriodicityAndOffset, monitoringSymbolsWithinSlot, duration for the PDCCH monitoring candidates monitored on sSCell as determined per Rel16 SS linking approach

Agreements from “UE features for DSS” Agenda Item

Agreement

The agreements listed in Section 6 of R1-2109917 are endorsed.

### RAN1#107-e (Nov 2021)

Agreement

If no additional set of (s1, s2) is introduced,

- For Option A BD/CCE limit handling and (s1=1, s2=0) agreed in RAN1#106bis-e, down-select from

  - Option 2

    - On sSCell (for cross-carrier scheduling to P(S)Cell)

      - UE is not required to monitor more than MPDCCHmax,slot,μ1 PDCCH BD candidates per sSCell slot

Agreement

- BD/CCE limits for Type B UEs are applicable for Type A UEs supporting cross-carrier scheduling from sSCell to P(S)Cell

Agreement

- Following approaches for PDCCH monitoring and BD limit handling is supported for Type A UE

    - Additional simplifications to PDCCH monitoring

      - Type A UE as per RAN1#105-e agreement and

        - no simultaneous monitoring between ‘USS sets (for P(S)Cell scheduling) on sSCell’ and ‘Type 0/0A/1/2/CSS sets on P(S)Cell for DCI formats with CRC scrambled by C-RNTI/MCS-C-RNTI/CS-RNTI’

        - simultaneous monitoring of ‘USS sets (for P(S)Cell scheduling) on sSCell’ and ‘Type 0/0A/1/2/CSS sets on P(S)Cell for DCI formats with CRC not scrambled by C-RNTI/MCS-C-RNTI/CS-RNTI’

Agreement

Confirm the WA from RAN1#106bis-e with addition of below Note (shown in blue)

Working Assumption

- When CIF for sSCell to Pcell cross-carrier scheduling is configured, non-fallback DCI formats on P(S)Cell include same number of CIF bits as the corresponding non-fallback DCI formats on sSCell that are used for sSCell to P(S)Cell scheduling

- Note: per RAN1#102-e agreement, when sSCell to P(S)Cell scheduling is configured for the UE, cross-carrier scheduling from P(S)Cell to another cell is not allowed. The CIF bits included in non-fallback DCI formats on P(S)Cell are considered reserved.

Agreement

- When CCS from sSCell to P(S)Cell is configured for the UE,

  - Multiple CORESET pools are not configured for PDCCH monitoring on P(S)Cell and not configured for PDCCH monitoring on sSCell;

  - Other Scells can be configured with multiple CORESET pools

Agreement

- For Option A BD/CCE limit handling agreed in RAN1#106bis-e

  - Value of parameter α for BD limit handling is configured via RRC from a set of

    - 8 possible values

Agreement

- For Option A BD/CCE limit handling agreed in RAN1#106bis-e

  - For CCE limit handling (i.e., scaling of maximum number of non-overlapping CCEs)

    - same parameter α agreed for BD limit handling is used

Agreement

- Alt1: When CCS from sSCell to P(S)Cell is configured for the UE,

  - r16monitoringcapability is not configured for PDCCH monitoring on P(S)Cell and not configured for PDCCH monitoring on sSCell;

  - r16monitoringcapability can be configured for PDCCH monitoring on Scells other than sSCell

Agreements from “UE features for DSS” Agenda Item

Agreement

The agreements listed in Section 6 of R1-2111812 are endorsed.

### RAN1#107b-e (Jan 2022)

Topic not included in agenda for this meeting

### RAN1#108-e (Feb/Mar 2022)

Agreement

- α (RRC parameter PCell-CCSscaling in RAN1 specs) is configured from below value set

  - {1/7, 3/14, 2/7, 3/7, 1/2, 5/7, [reserved1], [reserved2]}

Conclusion

- For a UE configured with cross-carrier scheduling from a sSCell to Pcell/PSCell, enableDefaultBeamForCCS can be configured in CrossCarrierSchedulingConfig in the Pcell/PSCell, which configures default beam determination for a PDSCH on the Pcell/PSCell scheduled by a PDCCH on the sSCell

Agreement

- When UE is configured for CCS from sSCell to P(S)SCell, and if SS set (x_p) of P(S)Cell and SS set (x_s) of sSCell are configured with same searchSpaceId value

  - x_s is used for CCS from sSCell to P(S)Cell (Note: already agreed)

  - x_s can be used for sSCell self-scheduling

  - x_p is not used for P(S)Cell self-scheduling and parameters other than searchSpaceId and nrofCandidates are not configured for that SS set

- Note: RAN2 spec may need some update, but it depends on RAN2 decision.

Agreement

- If a DCI format on the P(S)Cell for self-scheduling the P(S)Cell includes X bits and the corresponding DCI format on the sSCell for cross-carrier scheduling the P(S)Cell includes Y bits, |X-Y| bits are padded to the DCI format with the smaller size

Conclusion

When CCS from sSCell to P(S)Cell is configured, the configuration of Type 3 CSS set for DCI format 2_5 and applicability of the information in the DCI format is same as in Rel-16

Agreement

The TP to Section 10.1.1 of TS38.213(Identical to the TP in Annex A of R1-2202221) is endorsed.

Agreement

The following TP for 38.214 sub clause 5.1 and 6.1 is endorsed.

*** Unchanged text is omitted *** ## 5.1 UE procedure for receiving the physical downlink shared channel *** Unchanged text is omitted *** For any two HARQ process IDs in a given scheduled cell, if the UE is scheduled to start receiving a first PDSCH starting in symbolj by a PDCCH ending in symbol i on a scheduling cell, the UE is not expected to be scheduled to receive a PDSCH starting earlier than the end of the first PDSCH with a PDCCH that ends later than symbol i of the scheduling cell *** Unchanged text is omitted *** ## 6.1 UE procedure for transmitting the physical uplink shared channel *** Unchanged text is omitted *** For any two HARQ process IDs in a given scheduled cell, if the UE is scheduled to start a first PUSCH transmission starting in symbol j by a PDCCH ending in symbol i on a scheduling cell, the UE is not expected to be scheduled to transmit a PUSCH starting earlier than the end of the first PUSCH by a PDCCH that ends later than symbol i of the scheduling cell. *** Unchanged text is omitted ***

Agreement

- Update α (RRC parameter PCell-CCSscaling in RAN1 specs) value set as below

  - {1/7, 3/14, 2/7, 3/7, 1/2, 5/7, 4/7, reserved}

Agreement

- For a UE configured for CCS from sSCell to P(S)Cell, scaling factor α is not applied for PDCCH overbooking/BD/CCE limit computation when sSCell is deactivated, or when an activated sSCell is switched to dormant BWP; otherwise scaling factor α is applied

  - Timing for disabling scaling factor α when sSCell is deactivated follows sSCell deactivation timing in current specifications, i.e., no later than the minimum requirement defined in TS 38.133 as captured in 38.213 subclause 4.3

  - Timing for disabling scaling factor α follows the non-dormant to dormant BWP switching delay in current specifications ( TS 38.133).

  - Introduce separate FG to indicate UE support for disabling scaling factor α when sSCell is deactivated

  - Introduce separate FG to indicate UE support for disabling scaling factor α when activated sSCell is switched to dormant BWP

  - Note: It is up to UE implementation whether/when to apply the scaling factor α during sSCell activation and during sSCell BWP switch

Agreements from “UE features for DSS” Agenda Item

Agreement

All agreements for this agenda item are captured in Section 6 and 7 of R1-2202855.

# 3 References

- RP-211345 “Revised WID on NR Dynamic spectrum sharing (DSS)”, Ericsson, RAN1#92e, e-Meeting, June 2021.

# 4 Annex A – Agreements for Single DCI for scheduling PDSCH on multiple cells

In this section, agreements made for single DCI for scheduling PDSCH on multiple cells are included for reference.

RAN1#102-e (August 2020)

Agreements:

- For the study on single DCI scheduling PDSCH on two cells

  - Consider the following scenarios as baseline for evaluation

    - UE configured with Inter-band CA with PCell and an SCell

      - PCell for the UE is operated on a DSS carrier (i.e.,  same carrier is also used for serving LTE users)

      - Case 1: Different SCS for PCell and SCell

      - Case 2: Same SCS for PCell and Scell

  - Additional scenarios can also be evaluated, e.g. as below

    - Intra-band CA case with multiple serving cells having same SCS (all cells operated on non DSS carriers)

    - Inter-band CA case with PCell and more than one SCell (at least the SCells are operated on non DSS carriers)

    - Note: other combinations not precluded

- Note: Further details of evaluation framework (including carrier BW, slot format etc.) to be discussed in next stage

RAN1#103-e (Oct/Nov 2020)

Rapporteur Note: Yellow highlighted text below is not part of the agreements.

Agreements:

Further study multi-cell PDSCH scheduling via a single DCI with below simulation assumptions:

Table 1: Link level simulation assumptions

| Parameters | Values |
| --- | --- |
| Carrier frequency | Option 1: Inter-band CA (700MHz + 4GHz) Intra-band CA (2GHz) Option 2: Only 4GHz is considered |
| SCS | 15 kHz for 700MHz/2GHz 30 kHz for 4GHz |
| Bandwidth | Option 1: Baseline: PCell 10MHz + SCell 10/40MHz Optional: PCell 20MHz + SCell 20/40/100MHz Option 2: Baseline: Scheduling cell 100 MHz Optional: Scheduling cell 20 MHz |
| Channel model | TDL-C |
| Delay spread | 300 ns |
| Number of symbols for CORESET | [1], 2 or 3 |
| CORESET BW (contiguous PRB allocation) | 24/48/96 RBs depending on the bandwidth |
| CCE-to-REG mapping | Interleaved, [non-interleaved] |
| REG bundle size | 6 |
| Interleaver size | 2 |
| DCI payload size (excluding CRC) | Single PDSCH scheduling: 60 bits as baseline payload size Multi-cell PDSCH scheduling: 72/84/96/104 bits |
| BLER target for multi-cell scheduling DCI | Option 1: 1% Option 2: 0.5% |
| Number of BS antennas | 2 Tx for 700MHz/2GHz carrier frequency 4 Tx for 4GHz |
| Number of UE antennas | 2 Rx for 700MHz/2GHz carrier frequency 4 Rx for 4GHz carrier frequency |
| Modulation | QPSK |
| Channel coding | Polar code |
| UE speed | 3km/h |
| Aggregation level | 1/2/4/8/16 |
| Tx Diversity | One port precoder cycling |

Note 1: For two-cell scheduling via a single DCI, PDCCH transmitted on SCell schedules one PDSCH on the SCell and another PDSCH on PCell.

Note 2: For comparison, for single-cell scheduling, one PDCCH transmitted on SCell schedules one PDSCH on the SCell via self-scheduling and another PDCCH transmitted on the SCell schedules another PDSCH on PCell via cross-carrier scheduling.

Further discussion which rows are applicable to the scheduling cell/the scheduled cell for PDCCH

Agreements:

Further study with below simulation assumptions:

Simulation scenarios:

- For two-cell scheduling via a single DCI, PDCCH transmitted on a first cell schedules one PDSCH on the first cell and another PDSCH on a second cell.

- For single-cell scheduling (baseline), one PDCCH transmitted on a first cell schedules one PDSCH on the first cell via self-scheduling and another PDCCH transmitted on the first cell schedules another PDSCH on a second cell via cross-carrier scheduling.

  - Companies can optionally compare to the case of PDCCH transmitted on each of the two cells via self-scheduling. In this case, company should provide details on how to calculate the PDCCH blocking rate.

Simulation assumptions on carrier frequency, SCS, antenna configuration, carrier bandwidth as well as CORESET configuration

- Combination 1: 2 GHz, 15 kHz SCS, 2 Tx, 2 Rx, 20 MHz carrier BW, 2-symbol CORESET with 96RBs

- Combination 2: 4 GHz, 30 kHz SCS, 4 Tx, 4 Rx, 100 MHz carrier BW, 1-symbol CORESET with 270RBs

- [Combination 3: 700MHz, 15 kHz SCS, 2 Tx, 2 Rx, 10 MHz carrier BW, 3-symbol CORESET with 48RBs]

- [Combination 4: 4GHz, 30 kHz SCS, 4 Tx, 4 Rx, 40 MHz carrier BW, 2-symbol CORESET with 96RBs]

Payload size of two-cell scheduling DCI (excluding CRC):

- 60 for single-cell scheduling DCI (baseline).

- 72/84/96/108 for two-cell scheduling DCI.

- Companies are encouraged to report how the values are obtained, e.g., via separate or shared fields in DCI format.

Target BLER for two-cell scheduling DCI: 1% (baseline), 0.5%(optional)

- Option 1: 1%.

- Supported by OPPO, vivo, Nokia, Qualcomm, CATT, Ericsson, Huawei, Lenovo, Intel, MediaTek

- Option 2: 0.5%.

- Supported by Samsung, LG

Regarding the CCE-to-REG mapping, based on the agreed interleaved CCE-to-REG mapping, whether to adopt non-interleaved CCE-to-REG mapping is up to the proponent.

Agreements:

- Further study with below simulation assumptions:

Table 2: System level simulation assumptions

| Parameters | Values |
| --- | --- |
| Carrier frequency | For scheduling cell, follow agreed link level simulation assumptions For scheduled cell, consider 700MHz/2GHz with 10/20MHz BW (LTE overhead on DSS carrier can be optionally provided, up to proponent) |
| SCS |  |
| Simulation bandwidth |  |
| BS antenna height | 25 m |
| UE height | 1.5m |
| TRP transmit power | 46 dBm for 10MHz |
| Scenario | Urban Macro |
| ISD | 500m |
| TRP antenna configuration | (M,N,P,Mg,Ng;Mp,Np)= (1,2,2,1,1;1,1) for 700MHz (M,N,P,Mg,Ng;Mp,Np)= (2,8,2,1,1;1,1) for 2GHz (M,N,P,Mg,Ng;Mp,Np)= (8,4,2,1,1;1,1) for 4GHz |
| UE antenna configuration | (M,N,P,Mg,Ng;Mp,Np)= (1,1,2,1,1;1,1) for 700MHz/2GHz (M,N,P,Mg,Ng;Mp,Np)= (1,2,2,1,1;1,1) for 4GHz |
| Device deployment | 80% indoor, 20% outdoor |
| UE speeds of interest | Indoor users: 3km/h |
|  | Outdoor users (in-car): 30 km/h |
| BS noise figure | 5 dB |
| BS antenna element gain | 8 dBi |
| UE noise figure | 9 dB |
| Thermal noise level | -174 dBm/Hz |
| Traffic | Full Buffer(baseline), FTP model 1 or 3 up to company |
| Macro sites | 19 |
| Number of UEs per cell | 10/15/20 UEs |
| Downtilt | 102° |
| Minimum BS to UE distance | 35m |

RAN1#104-e (Jan/Feb 2021)

Agreement

- The proposal in section 2.6 of R1-2102138 for PDCCH blocking probability is taken as RAN1 observation

- The proposal in section 2.6 of R1-2102138 for PDSCH throughput is taken as RAN1 observation with the following revision for the note:

  - Note: Combinations 1 and 2 were agreed for evaluation. Some companies provided evaluation results for Combinations 3 and 4.

Agreement

The observations for multi-cell PDSCH scheduling via a single DCI to be summarized in the status report along with explantion on different combinations that were considered for submission to RAN.

RAN1#104b-e (Apr 2021)

Rapporteur Note: Topic was not discussed in this meeting

RAN1#105-e (May 2021)

Conclusion

Stop the RAN1 work on two-cell PDSCH scheduling via a single DCI for specification support in Rel-17 DSS

- Due to Rel-17 WID scope only focusing on two DL carriers within FR1, RAN1 did not study the potential benefits for more than 2 carriers for both UL and DL within FR1 and FR2.

- It is up to RAN to decide whether or not the feature is studied/specified in Rel-18 with extension to more than 2 carriers for both UL and DL within FR1 and FR2.