3GPP TSG-RAN WG1 Meeting #108-e	R1-2202959

# Electronic Meeting, February 21st – March 3rd, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | 0253 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on NR coverage enhancements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_cov_enh-Core |  |  |  |  |  | Date: |  |  | 2022-03-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Corrections on NR coverage enhancements |  |  |  |  |  |  |  |  |
| Summary of change: |  | In clause 6.1.2.1, the use of TBoMS for HD-FDD UE with counting on available slot is supported. Note: existing mechanism as in AI8.8.1.1 should be applied for this case, In clause 6.1.2.1, corrected few typos in Table 6.1.21.-2 which happened during the implementation of the CR. In clause 6.1.2.3.3, for CG-PUSCH transmissions of TBoMS, the UE is not expected to be configured with the time duration for the N*M transmissions larger than the time duration derived by the periodicity P. In clause 6.1.4.1, added the procedures for MSC index indication for retransmission of Msg3 with repetition. In clause 6.1.7, if DMRS bundling and UL beam switching for multi-TRP operation are configured simultaneously, UL beam switching for multi-TRP operation is regarded as a semi-static event. In section 6.3.1, the inter-slot frequency hopping pattern for PUSCH repetitions with DM-RS bundling is determined based on physical slot index |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete support of NR coverage enhancements |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.2.1, 6.1.2.3.1, 6.1.2.3.3, 6.1.4.1, 6.1.7, 6.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.212 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS 38.213 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<omitted text>

### 6.1.2	Resource allocation

#### 6.1.2.1	Resource allocation in time domain

<omitted text>

For TB processing over multiple slots, when transmitting PUSCH scheduled by DCI format 0_1 or 0_2 in PDCCH with CRC scrambled with C-RNTI, MCS-C-RNTI, or CS-RNTI with NDI=1,

-	the number of slots used for TBS determination N is indicated by numberOfSlotsTBoMS.

-	the number of repetitions K of the number of slots N used for TBS determination is determined as

-	if numberOfRepetitions is present in the resource allocation table, the number of repetitions K is equal to numberOfRepetitions;

-	otherwise, K=1.

-	when the UE supports repetition of TB processing over multiple slots, the UE does not expect that N∙K is larger than 32KmaxKmax≤32.

When configured with m= 5 or 6 the UE does not expect to be scheduled with more than one PUSCH in a slot, by a single DCI or multiple DCIs.

For PUSCH repetition Type A, when transmitting PUSCH scheduled by DCI format 0_1 or 0_2 in PDCCH with CRC scrambled with C-RNTI, MCS-C-RNTI, or CS-RNTI with NDI=1, the number of repetitions K is determined as

-	if numberOfRepetitions is present in the resource allocation table, the number of repetitions K is equal to numberOfRepetitions;

-	elseif the UE is configured with pusch-AggregationFactor, the number of repetitions K is equal to pusch-AggregationFactor;

-	otherwise K=1.

-	the number of slots used for TBS determination N is equal to 1.

For PUSCH repetition type A, when transmitting PUSCH scheduled by RAR UL grant, the 2 MSBs of the MCS information field of the RAR UL grant provide a codepoint to determine the number of repetitions K according to Table 6.1.2.1-1A, based on whether or not the higher layer parameter numberOfMsg3Repetitions is configured. The number of slots used for TBS determination N is equal to 1.

For PUSCH repetition type A, when transmitting PUSCH scheduled by DCI format 0_0 with CRC scrambled by TC-RNTI, the 2 MSBs of the MCS information field of the DCI format 0_0 with CRC scrambled by TC-RNTI provide a codepoint to determine the number of repetitions K according to Table 6.1.2.1-1A, based on whether or not the higher layer parameter numberOfMsg3Repetitions is configured. The number of slots used for TBS determination N is equal to 1.

Table 6.1.2.1-1A: Number of repetition K as a function of 2 MSBs of MCS information field

| numberOfMsg3Repetitions is configured |  |  | numberOfMsg3Repetitions is not configured |  |
| --- | --- | --- | --- | --- |
| Codepoint | K |  | Codepoint | K |
| 00 | First value of numberOfMsg3Repetitions |  | 00 | 1 |
| 01 | Second value of numberOfMsg3Repetitions |  | 01 | 2 |
| 10 | Third value of numberOfMsg3Repetitions |  | 10 | 3 |
| 11 | Fourth value of numberOfMsg3Repetitions |  | 11 | 4 |

If a UE is configured with higher layer parameter pusch-TimeDomainAllocationListForMultiPUSCH, the UE does not expect to be configured with pusch-AggregationFactor.

If a UE is configured with pusch-TimeDomainAllocationListForMultiPDSCH-r17 in which one or more rows contain multiple SLIVs for PUSCH on a UL BWP of a serving cell, the UE does not apply pusch-AggregationFactor, if configured, to DCI format 0_1 on the UL BWP of the serving cell and the UE does not expect to be configured with numberOfRepetitions in pusch-TimeDomainAllocationListForMultiPDSCH-r17.

For unpaired spectrum:

-	When AvailableSlotCounting is enabled, and in case K>1, the UE determines N∙K slots for a PUSCH transmission of a PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, based on tdd-UL-DL-ConfigurationCommon, tdd-UL-DL-ConfigurationDedicated and ssb-PositionsInBurst, and the TDRA information field value in the DCI format 0_1 or 0_2.

-	A slot is not counted in the number of N∙K slots for PUSCH transmission of a PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2 if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst.

-	Otherwise, the UE determines N∙K consecutive slots for a PUSCH transmission of a PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, based on the TDRA information field value in the DCI format 0_1 or 0_2.

-	The UE determines N∙K slots for a PUSCH transmission of TB processing over multiple slots scheduled by DCI format 0_1 or 0_2, based on tdd-UL-DL-ConfigurationCommon, tdd-UL-DL-ConfigurationDedicated and ssb-PositionsInBurst, and the TDRA information field value in the DCI format 0_1 or 0_2.

-	A slot is not counted in the number of N∙K slots for a PUSCH transmission of TB processing over multiple slots if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst.

-	The UE determines N∙K slots for a PUSCH transmission of a PUSCH repetition Type A scheduled by RAR UL grant, based on tdd-UL-DL-ConfigurationCommon and ssb-PositionsInBurst, and the TDRA information field value in the RAR UL grant.

-	A slot is not counted in the number of N∙K slots for a PUSCH transmission of a PUSCH repetition Type A scheduled by RAR UL grant, if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a DL symbol indicated by tdd-UL-DL-ConfigurationCommon if provided, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst.

-	The UE determines N∙K slots for a PUSCH transmission of a PUSCH repetition Type A scheduled by DCI format 0_0 with CRC scrambled by TC-RNTI, based on tdd-UL-DL-ConfigurationCommon and ssb-PositionsInBurst and the TDRA information field value in the DCI scheduling the PUSCH.

-	A slot is not counted in the number of N∙K slots for a PUSCH transmission of a PUSCH repetition Type A scheduled by DCI format 0_0 scrambled by TC-RNTI, if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a DL symbol indicated by tdd-UL-DL-ConfigurationCommon if provided, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst.

For paired spectrum and SUL band:

-	The UE determines N∙K consecutive slots for a PUSCH transmission of a PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, or for a PUSCH transmission of TB processing over multiple slots scheduled by DCI format 0_1 or 0_2, based on the TDRA information field value in the DCI format 0_1 or 0_2.

-	For the case of a reduced capability half-duplex UE, and when AvailableSlotCounting is enabled, the UE determines N∙K slots for a PUSCH transmission of a PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, or for a PUSCH transmission of TB processing over multiple slots scheduled by DCI format 0_1 or 0_2, based on the TDRA information field value in the DCI format 0_1 or 0_2. A slot is not counted in the number of N∙K slots if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst.

N∙K

-	The UE determines N∙K consecutive slots for a PUSCH transmission of a PUSCH repetition Type A scheduled by RAR UL grant, based on the TDRA information field value in the RAR UL grant.

-	The UE determines N∙K consecutive slots for a PUSCH transmission of a PUSCH repetition Type A scheduled by DCI format 0_0 with CRC scrambled by TC-RNTI, based on the TDRA information field value in the DCI scheduling the PUSCH.

If AvailableSlotCounting is enabled and a UE would transmit a PUSCH of a TB processing over multiple slots or PUSCH repetition Type A over N∙K slots, and the UE does not transmit the PUSCH of a TB processing over multiple slots or the PUSCH repetition Type A in a slot from the N∙K slots, according to clause 9, clause 11.1 and clause 11.2A of [6, TS 38.213], the UE counts the slots in the number of N∙K slots.

For PUSCH repetition Type A, in case K>1,

-	If the PUSCH is scheduled by DCI format 0_1 or 0_2

-	if AvailableSlotCounting is enabled, the same symbol allocation is applied across the N∙K slots determined for the PUSCH transmission and the PUSCH is limited to a single transmission layer. The UE shall repeat the TB across the N∙K slots determined for the PUSCH transmission, applying the same symbol allocation in each slot.

-	Otherwise, the same symbol allocation is applied across the N∙K consecutive slots and the PUSCH is limited to a single transmission layer. The UE shall repeat the TB across the N∙K consecutive slots applying the same symbol allocation in each slot.

-	Else if the PUSCH is scheduled by RAR UL grant or by DCI format 0_0 with CRC scrambled by TC-RNTI, the same symbol allocation is applied across the N∙K slots determined for the PUSCH transmission and the PUSCH is limited to a single transmission layer. The UE shall repeat the TB across the N∙K slots determined for the PUSCH transmission, applying the same symbol allocation in each slot.

For TB processing over multiple slots:

-	For unpaired spectrum, the same symbol allocation is applied across the N∙K slots determined for the PUSCH transmission and the PUSCH is limited to a single transmission layer. The UE shall transmit the TB across the N∙K slots determined for the PUSCH transmission, applying the same symbol allocation in each slot.

-	For paired spectrum or supplementary uplink band, the same symbol allocation is applied across the  N∙K consecutive slots and the PUSCH is limited to a single transmission layer. The UE shall transmit the TB across the N∙K consecutive slots applying the same symbol allocation in each slot.

- For the case of reduced capability half-duplex UE, [and when AvailableSlotCounting is enabled,] the same symbol allocation is applied across the N∙K slots determined for the PUSCH transmission and the PUSCH is limited to a single transmission layer. The UE shall transmit the TB across the N∙K slots determined for the PUSCH transmission, applying the same symbol allocation in each slot.

For a PUSCH transmission scheduled by DCI format 0_1, or 0_2, or 0_0 with CRC scrambled by TC-RNTI, the redundancy version to be applied on the nth transmission occasion of the TB, where n = 0, 1, … N∙K -1, is determined according to table 6.1.2.1-2.

For a PUSCH transmission of a PUSCH repetition Type A scheduled by RAR UL grant, the redundancy version to be applied on the nth transmission occasion of the TB, where n = 0, 1, … N∙K -1, is determined according to the first row of Table 6.1.2.1-2.

Table 6.1.2.1-2: Redundancy version for PUSCH transmission

| rvid indicated by the DCI scheduling the PUSCH | rvid to be applied to nth transmission occasion (repetition Type A) or TB processing over multiple slots) or nth actual repetition (repetition Type B) |  |  |  |
| --- | --- | --- | --- | --- |
|  | ((n-(n mod N))/N) mod 4 = 0 | ((n-(n mod N))/N) mod 4 = 1 | ((n-(n mod N))/N) mod 4 = 2 | ((n-(n mod N))/N) mod 4 = 3 |
| 0 | 0 | 2 | 3 | 1 |
| 2 | 2 | 3 | 1 | 0 |
| 3 | 3 | 1 | 0 | 2 |
| 1 | 1 | 0 | 2 | 3 |

<omitted text>

##### 6.1.2.3.1	Transport Block repetition for uplink transmissions of PUSCH repetition Type A with a configured grant

The procedures described in this clause apply to PUSCH transmissions of PUSCH repetition Type A with a Type 1 or Type 2 configured grant.

The higher layer parameter repK-RV defines the redundancy version pattern to be applied to the repetitions. If cg-RetransmissionTimer is provided, the redundancy version for uplink transmission with a configured grant is determined by the UE. If the parameter repK-RV is not provided in the configuredGrantConfig and cg-RetransmissionTimer is not provided, the redundancy version for uplink transmissions with a configured grant shall be set to 0. If the parameter repK-RV is provided in the configuredGrantConfig and cg-RetransmissionTimer is not provided, for the nth transmission occasion among K repetitions, n=1, 2, …, K, it is associated with (mod((n-mod(n, N))/N,4)+1)th value in the configured RV sequence, where N=1. If a configured grant configuration is configured with startingFromRV0 set to 'off', the initial transmission of a transport block may only start at the first transmission occasion of the K repetitions. Otherwise, the initial transmission of a transport block may start at

-	the first transmission occasion of the K repetitions if the configured RV sequence is {0,2,3,1},

-	any of the transmission occasions of the K repetitions that are associated with RV=0 if the configured RV sequence is {0,3,0,3},

-	any of the transmission occasions of the K repetitions if the configured RV sequence is {0,0,0,0}, except the last transmission occasion when K≥8.

When the transmission occasions are associated with the first and second SRS resource sets, if the parameter repK-RV is provided in the configuredGrantConfig, for the nth transmission occasion among all transmission occasions that are associated with the SRS resource set of the first transmission occasion, it is associated with (mod(n-1,4)+1)th value in the configured RV sequence, and for the nth transmission occasion among all transmission occasions that are not associated with the SRS resource set of the first transmission occasion, it is associated with (mod(n-1,4)+1)th value in the adjusted RV sequence and the adjustment is based on additional shifting operation on the configured RV sequence, where the shifting operation is defined as rvi+ rvsmod 4 where rvi is the ith RV value (i=1, 2, 3, 4) in the configured RV sequence and rvs is configured by the higher layer parameter sequenceOffsetforRV in configuredGrantConfig. When the transmission occasions are associated with the first and second SRS resource sets, if a configured grant configuration is configured with startingFromRV0 set to 'off', the initial transmission of a transport block may only start at the first transmission occasion of the K repetitions. Otherwise, the initial transmission of a transport block may start at

-	the first transmission occasion associated with RV = 0 corresponding to the first or second SRS resource set of the K repetitions if the configured RV sequence is {0,2,3,1},

-	any of the transmission occasions of the K repetitions that are associated with RV=0 if the configured RV sequence is {0,3,0,3},

-	any of the transmission occasions of the K repetitions if the configured RV sequence is {0,0,0,0}, except the last transmission occasion when K≥8.

After the initial transmission of a transport block, later transmission occasions among the K repetitions associated with any RV value and associated to any of the first or second SRS resource set can be used for transmitting the transport block.

For any RV sequence, the repetitions shall be terminated after transmitting K repetitions, or at the last transmission occasion among the K repetitions within the period P, or from the starting symbol of the repetition that overlaps with a PUSCH with the same HARQ process scheduled by DCI format 0_0, 0_1 or 0_2, whichever is reached first. In addition, the UE shall terminate the repetition of a transport block in a PUSCH transmission if the UE receives a DCI format 0_1 with DFI flag provided and set to '1', and if in this DCI the UE detects ACK for the HARQ process corresponding to that transport block.

The UE is not expected to be configured with the time duration for the transmission of K repetitions larger than the time duration derived by the periodicity P. If the UE determines that, for a transmission occasion, the number of symbols available for the PUSCH transmission in a slot is smaller than transmission duration L, the UE does not transmit the PUSCH in the transmission occasion.

For both Type 1 and Type 2 PUSCH transmissions with a configured grant, when K > 1,

-	For unpaired spectrum:

-	If AvailableSlotCounting is enabled, the UE shall repeat the TB across the N∙K slots determined for the PUSCH transmission applying the same symbol allocation in each slot.

-	A slot is not counted in the number of N∙K slots if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst.

-	Otherwise, the UE shall repeat the TB across the N∙K consecutive slots applying the same symbol allocation in each slot, except if the UE is provided with higher layer parameters cg-nrofSlots and cg-nrofPUSCH-InSlot, in which case the UE repeats the TB in the repK earliest consecutive transmission occasion candidates within the same configuration.

-	For paired spectrum and SUL band:

-	The UE shall repeat the TB across the N∙K consecutive slots applying the same symbol allocation in each slot, except if the UE is provided with higher layer parameters cg-nrofSlots and cg-nrofPUSCH-InSlot, in which case the UE repeats the TB in the repK earliest consecutive transmission occasion candidates within the same configuration.

-	If AvailableSlotCounting is enabled, and in case of reduced capability half-duplex UE, the UE shall repeat the TB across the N∙K slots applying the same symbol allocation in each slot. A slot is not counted in the number of N∙K slots if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst.

N∙K

A Type 1 or Type 2 PUSCH transmission with a configured grant in a slot is omitted according to the conditions in Clause 9, Clause 11.1 and Clause 11.2A of [6, TS 38.213].

<omitted text>

##### 6.1.2.3.3	Transport Block repetition for uplink transmissions of TB processing over multiple slots with a configured grant

The procedures described in this clause apply to PUSCH transmissions of TB processing over multiple slots with a Type 2 configured grant.

The higher layer parameter repK-RV defines the redundancy version pattern to be applied to the repetitions. If the parameter repK-RV is not provided in the configuredGrantConfig, the redundancy version for uplink transmissions with a configured grant shall be set to 0. If the parameter repK-RV is provided in the configuredGrantConfig, the nth transmission occasion among N∙K transmissions occasions, n=0,1, …, N∙K -1, is associated with (mod((n-mod(n, N))/N,4)+1)th value in the configured RV sequence. When K=1, or when K>1 and the configured grant configuration is configured with startingFromRV0 set to 'off', the initial transmission of the transport block may only start at the first transmission ocasion of the N∙K transmission occasions. Otherwise, the initial transmission of the transport block may start at

-	The first transmission occasion of the N∙K transmission occasions if the configured RV sequence is {0,2,3,1}.

-	Any transmission occasion n associated with RV=0, and for which n mod N =0, if the configured RV sequence is {0,3,0,3} or {0,0,0,0}.

The UE is not expected to be configured with the time duration for N∙K transmissions larger than the time duration derived by the periodicity P. If the UE determines that, for a transmission occasion, the number of symbols available in a slot for the PUSCH transmission of TB processing over multiple slots is smaller than transmission duration L, the UE does not transmit the PUSCH in the transmission occasion.

For Type 2 PUSCH transmission with a configured grant of TB processing over multiple slots, the UE shall transmit the TB across the N∙K slots determined for the PUSCH transmission applying the same symbol allocation in each slot. A Type 2 PUSCH transmission with a configured grant of TB processing over multiple slots is omitted in a slot according to the conditions in clause 9, clause 11.1 and clause 11.2A of [6, TS 38.213].

<omitted text>

#### 6.1.4.1	Modulation order and target code rate determination

For a PUSCH scheduled by RAR UL grant or

for a PUSCH scheduled by a fallbackRAR UL grant or

for a MsgA PUSCH transmission, or

for a PUSCH scheduled by a DCI format 0_0 with CRC scrambled by C-RNTI, MCS-C-RNTI, TC-RNTI, CS-RNTI, or

for a PUSCH scheduled by a DCI format 0_1 or DCI format 0_2 with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, SP-CSI-RNTI, or

for a PUSCH with configured grant using CS-RNTI, and

if transform precoding is disabled for this PUSCH transmission according to Clause 6.1.3

-	if mcs-TableDCI-0-2 in pusch-Config is set to 'qam256', and PUSCH is scheduled by a PDCCH with DCI format 0_2 with CRC scrambled by C-RNTI or SP-CSI-RNTI,

-	the UE shall use IMCS and Table 5.1.3.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel;

-	elseif the UE is not configured with MCS-C-RNTI, mcs-TableDCI-0-2  in pusch-Config is set to 'qam64LowSE', and the PUSCH is scheduled by a PDCCH by a PDCCH with DCI format 0_2 with CRC scrambled by C-RNTI or SP-CSI-RNTI,

-	the UE shall use IMCS and Table 5.1.3.1-3 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel.

-	elseif mcs-Table in pusch-Config is set to 'qam256', and PUSCH is scheduled by a PDCCH with DCI format 0_1 with CRC scrambled by C-RNTI or SP-CSI-RNTI,

-	the UE shall use IMCS and Table 5.1.3.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel.

-	elseif the UE is not configured with MCS-C-RNTI, mcs-Table in pusch-Config is set to 'qam64LowSE', and the PUSCH is scheduled by a PDCCH with a DCI format other than DCI format 0_2 in a UE-specific search space with CRC scrambled by C-RNTI or SP-CSI-RNTI,

-	the UE shall use IMCS and Table 5.1.3.1-3 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel.

-	elseif the UE is configured with MCS-C-RNTI, and the PUSCH is scheduled by a PDCCH with CRC scrambled by MCS-C-RNTI,

-	the UE shall use IMCS and Table 5.1.3.1-3 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel.

-	elseif mcs-Table in configuredGrantConfig is set to 'qam256',

-	if PUSCH is scheduled by a PDCCH with CRC scrambled by CS-RNTI or

-	if PUSCH is transmitted with configured grant

-	the UE shall use IMCS and Table 5.1.3.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel.

-	elseif mcs-Table in configuredGrantConfig is set to 'qam64LowSE',

-	if PUSCH is scheduled by a PDCCH with CRC scrambled by CS-RNTI or

-	if PUSCH is transmitted with configured grant,

-	the UE shall use IMCS and Table 5.1.3.1-3 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel.

-	elseif for a MsgA PUSCH transmission,

-	the UE shall use higher layer parameter msgA-MCS for IMCS and Table 5.1.3.1-1 to determine the Target code rate (R) used in the physical uplink shared channel.

-	elseif the UE requests repetition of PUSCH scheduled by RAR UL grant [10, TS 38.321], when transmitting PUSCH scheduled by RAR UL grant,

-	the 2 LSBs of the MCS information field of the RAR UL grant provide a codepoint to determine the MCS index IMCS according to Table 6.1.4.1-3, based on whether or not the higher layer parameter mcs-Msg3Repetition is configured. The UE shall use the determined IMCS and Table 5.1.3.1-1 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel.

- elseif the UE requests repetition of PUSCH scheduled by RAR UL grant [10, TS 38.321], when transmitting PUSCH scheduled by DCI format 0_0 with CRC scrambled by the TC-RNTI,

- the 3 LSBs of the MCS information field of the DCI format 0_0 with CRC scrambled by the TC-RNTI provide a codepoint to determine the MCS index IMCS according to Table 6.1.4.1-4, based on whether or not the higher layer parameter mcs-Msg3Repetition is configured. The UE shall use the determined IMCS and Table 5.1.3.1-1 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel.

-	else

-	the UE shall use IMCS and Table 5.1.3.1-1 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel.

else

-	if mcs-TableTransformPrecoderDCI-0-2 in pusch-Config is set to 'qam256', and PUSCH is scheduled by a PDCCH with DCI format 0_2 with CRC scrambled by C-RNTI or SP-CSI-RNTI,

-	the UE shall use IMCS and Table 5.1.3.1.-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel.

-	elseif the UE is not configured with MCS-C-RNTI, mcs-TableTransformPrecoderDCI-0-2 in pusch-Config is set to 'qam64LowSE', and the PUSCH is scheduled by a PDCCH with DCI format 0_2 with CRC scrambled by C-RNTI or SP-CSI-RNTI,

-	the UE shall use IMCS and Table 6.1.4.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel.

-	elseif mcs-TableTransformPrecoder in pusch-Config is set to 'qam256', and PUSCH is scheduled by a PDCCH with DCI format 0_1 with CRC scrambled by C-RNTI or SP-CSI-RNTI,

-	the UE shall use IMCS and Table 5.1.3.1.-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel.

-	elseif the UE is not configured with MCS-C-RNTI, mcs-TableTransformPrecoder in pusch-Config is set to 'qam64LowSE', and the PUSCH is scheduled by a PDCCH with a DCI format other than DCI format 0_2 in a UE-specific search space with CRC scrambled by C-RNTI or SP-CSI-RNTI,

-	the UE shall use IMCS and Table 6.1.4.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel.

-	elseif the UE is configured with MCS-C-RNTI, and the PUSCH is scheduled by a PDCCH with CRC scrambled by MCS-C-RNTI,

-	the UE shall use IMCS and Table 6.1.4.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel.

-	elseif mcs-TableTransformPrecoder in configuredGrantConfig is set to 'qam256',

-	if PUSCH is scheduled by a PDCCH with CRC scrambled by CS-RNTI or

-	if PUSCH is transmitted with configured grant,

-	the UE shall use IMCS and Table 5.1.3.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel.

-	elseif mcs-TableTransformPrecoder in configuredGrantConfig is set to 'qam64LowSE',

-	if PUSCH is scheduled by a PDCCH with CRC scrambled by CS-RNTI or

-	if PUSCH is transmitted with configured grant,

-	the UE shall use IMCS and Table 6.1.4.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel.

-	elseif for a MsgA PUSCH transmission,

-	the UE shall use higher layer parameter MsgA-MCS for IMCS and Table 6.1.4.1-1 to determine the Target code rate (R) used in the physical uplink shared channel.

-	the UE shall use q=2 for determining modulation order Qm in Table 6.1.4.1-1.

-	elseif the UE requests repetition of PUSCH scheduled by RAR UL grant [10, TS 38.321] when transmitting PUSCH scheduled by RAR UL grant,

-	the 2 LSBs of the MCS information field of the RAR UL grant provide a codepoint to determine the MCS index IMCS according to Table 6.1.4.1-3, based on whether or not the higher layer parameter mcs-Msg3Repetition is configured. The UE shall use the determined IMCS and Table 6.1.4.1-1 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel.

- elseif the UE requests repetition of PUSCH scheduled by RAR UL grant [10, TS 38.321], when transmitting PUSCH scheduled by DCI format 0_0 with CRC scrambled by the TC-RNTI,

- the 3 LSBs of the MCS information field of the DCI format 0_0 with CRC scrambled by the TC-RNTI provide a codepoint to determine the MCS index IMCS according to Table 6.1.4.1-4, based on whether or not the higher layer parameter mcs-Msg3Repetition is configured. The UE shall use the determined IMCS and Table 6.1.4.1-1 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel.

-	else

-	the UE shall use IMCS and Table 6.1.4.1-1to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel.

end

For Table 6.1.4.1-1 and Table 6.1.4.1-2, if higher layer parameter tp-pi2BPSK is configured, q = 1 otherwise q=2.

Table 6.1.4.1-1: MCS index table for PUSCH with transform precoding and 64QAM

| MCS Index IMCS | Modulation Order Qm | Target code Rate R x 1024 | Spectral efficiency |
| --- | --- | --- | --- |
| 0 | q | 240/ q | 0.2344 |
| 1 | q | 314/ q | 0.3066 |
| 2 | 2 | 193 | 0.3770 |
| 3 | 2 | 251 | 0.4902 |
| 4 | 2 | 308 | 0.6016 |
| 5 | 2 | 379 | 0.7402 |
| 6 | 2 | 449 | 0.8770 |
| 7 | 2 | 526 | 1.0273 |
| 8 | 2 | 602 | 1.1758 |
| 9 | 2 | 679 | 1.3262 |
| 10 | 4 | 340 | 1.3281 |
| 11 | 4 | 378 | 1.4766 |
| 12 | 4 | 434 | 1.6953 |
| 13 | 4 | 490 | 1.9141 |
| 14 | 4 | 553 | 2.1602 |
| 15 | 4 | 616 | 2.4063 |
| 16 | 4 | 658 | 2.5703 |
| 17 | 6 | 466 | 2.7305 |
| 18 | 6 | 517 | 3.0293 |
| 19 | 6 | 567 | 3.3223 |
| 20 | 6 | 616 | 3.6094 |
| 21 | 6 | 666 | 3.9023 |
| 22 | 6 | 719 | 4.2129 |
| 23 | 6 | 772 | 4.5234 |
| 24 | 6 | 822 | 4.8164 |
| 25 | 6 | 873 | 5.1152 |
| 26 | 6 | 910 | 5.3320 |
| 27 | 6 | 948 | 5.5547 |
| 28 | q | reserved |  |
| 29 | 2 | reserved |  |
| 30 | 4 | reserved |  |
| 31 | 6 | reserved |  |

Table 6.1.4.1-2: MCS index table 2 for PUSCH with transform precoding and 64QAM

| MCS Index IMCS | Modulation Order Qm | Target code Rate R x 1024 | Spectral efficiency |
| --- | --- | --- | --- |
| 0 | q | 60/q | 0.0586 |
| 1 | q | 80/q | 0.0781 |
| 2 | q | 100/q | 0.0977 |
| 3 | q | 128/q | 0.1250 |
| 4 | q | 156/q | 0.1523 |
| 5 | q | 198/q | 0.1934 |
| 6 | 2 | 120 | 0.2344 |
| 7 | 2 | 157 | 0.3066 |
| 8 | 2 | 193 | 0.3770 |
| 9 | 2 | 251 | 0.4902 |
| 10 | 2 | 308 | 0.6016 |
| 11 | 2 | 379 | 0.7402 |
| 12 | 2 | 449 | 0.8770 |
| 13 | 2 | 526 | 1.0273 |
| 14 | 2 | 602 | 1.1758 |
| 15 | 2 | 679 | 1.3262 |
| 16 | 4 | 378 | 1.4766 |
| 17 | 4 | 434 | 1.6953 |
| 18 | 4 | 490 | 1.9141 |
| 19 | 4 | 553 | 2.1602 |
| 20 | 4 | 616 | 2.4063 |
| 21 | 4 | 658 | 2.5703 |
| 22 | 4 | 699 | 2.7305 |
| 23 | 4 | 772 | 3.0156 |
| 24 | 6 | 567 | 3.3223 |
| 25 | 6 | 616 | 3.6094 |
| 26 | 6 | 666 | 3.9023 |
| 27 | 6 | 772 | 4.5234 |
| 28 | q | reserved |  |
| 29 | 2 | reserved |  |
| 30 | 4 | reserved |  |
| 31 | 6 | reserved |  |

Table 6.1.4.1-3: MCS index IMCS as a function of 2 LSBs of MCS information field in RAR UL grant

| mcs-Msg3Repetition is configured |  |  | mcs-Msg3Repetitions is not configured |  |
| --- | --- | --- | --- | --- |
| Codepoint | IMCS |  | Codepoint | IMCS |
| 00 | First value of mcs-Msg3Repetition |  | 00 | 0 |
| 01 | Second value of mcs-Msg3Repetition |  | 01 | 1 |
| 10 | Third value of mcs-Msg3Repetition |  | 10 | 2 |
| 11 | Fourth value of mcs-Msg3Repetition |  | 11 | 3 |

Table 6.1.4.1-4: MCS index IMCS as a function of 3 LSBs of MCS information field in DCI format 0_0 with CRC scrambled by the TC-RNTI

| mcs-Msg3Repetition is configured |  |  | mcs-Msg3Repetitions is not configured |  |
| --- | --- | --- | --- | --- |
| Codepoint | IMCS |  | Codepoint | IMCS |
| 000 | First value of mcs-Msg3Repetition |  | 000 | 0 |
| 001 | Second value of mcs-Msg3Repetition |  | 001 | 1 |
| 010 | Third value of mcs-Msg3Repetition |  | 010 | 2 |
| 011 | Fourth value of mcs-Msg3Repetition |  | 011 | 3 |
| 100 | Fifth value of mcs-Msg3Repetition |  | 100 | 4 |
| 101 | Sixth value of mcs-Msg3Repetition |  | 101 | 5 |
| 110 | Seventh value of mcs-Msg3Repetition |  | 110 | 6 |
| 111 | Eighth value of mcs-Msg3Repetition |  | 111 | 7 |

#### 6.1.4.2	Transport block size determination

<omitted text>

### 6.1.7	UE procedure for determining time domain windows for bundling DM-RS

<omitted text>

Events which cause power consistency and phase continuity not to be maintained across PUSCH transmissions of PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, or PUSCH repetition Type A with a configured grant, or PUSCH repetition type B or TB processing over multiple slots, or PUCCH transmissions of PUCCH repetition, within the nominal TDW, are:

-	A downlink slot or downlink reception or downlink monitoring based on tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated for unpaired spectrum.

-	The gap between any two consecutive PUSCH transmissions, or the gap between any two consecutive PUCCH transmissions, exceeds 13 symbols for normal cyclic prefix or exceeds 11 symbols for extended cyclic prefix.

-	The gap between any two consecutive PUSCH transmissions, or the gap between any two consecutive PUCCH transmissions, does not exceed 13 symbols but other uplink transmissions are scheduled between the two consecutive PUSCH transmissions or the two consecutive PUCCH transmissions.

-	For PUSCH transmissions of PUSCH repetition type A, or PUSCH repetition type B or TB processing over multiple slots, a dropping or cancellation of a PUSCH transmission according to clause 9, clause 11.1 and clause 11.2A of [6, TS 38.213].

-	For PUCCH transmissions of PUCCH repetition, a dropping or cancellation of a PUCCH transmission according to clause 9, clause 9.2.6 and clause 11.1 of [6, TS 38.213].

-	For any two consecutive PUSCH transmissions of PUSCH repetition type A, or PUSCH repetition type B, and when two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'codebook' or 'noncodebook', a different SRS resource set association is used for the two PUSCH transmissions of PUSCH repetition type A, or PUSCH repetition type B, according to Clause 6.1.2.1.

-	For any two consecutive PUCCH transmissions of PUCCH repetition, and when a PUCCH resource used for repetitions of a PUCCH transmission by a UE includes first and second spatial relations or first and second sets of power control parameters, as described in [10, TS 38.321] and in clause 7.2.1 of [6, TS 38.213], different spatial relations or different power control parameters are used for the two PUCCH transmissions of PUCCH repetition, according to Clause 9.2.6 of [6, TS 38.213].

-	Uplink timing adjustment in response to a timing advance command according to clause 4.2 of [6, TS 38.213].

-	Frequency hopping.

-	For reduced capability half-duplex UEs,

-	a dropping or cancellation of a PUSCH transmission according to clause 17.2 of [6, TS 38.213] or

-	an overlapping of the gap between two consecutive PUSCH transmissions and any symbol of downlink reception or downlink monitoring.

The UE shall maintain power consistency and phase continuity within an actual TDW, across PUSCH transmissions of PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2, or PUSCH repetition Type A with a configured grant, or PUSCH repetition type B or TB processing over multiple slots, or across PUCCH transmissions of PUCCH repetition, in case the actual TDW is created in response to frequency hopping, or in response to the use of a different SRS resource set association for the two PUSCH transmissions of PUSCH repetition type A, or PUSCH repetition type B, or in response to the use of different spatial relations or different power control parameters for the two PUCCH transmissions of PUCCH repetition, or in response to any event not triggered by DCI or MAC-CE. The UE maintains power consistency and phase continuity within an actual TDW, across PUSCH transmissions of PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2, or PUSCH repetition Type A with a configured grant, or PUSCH repetition type B or TB processing over multiple slots, or across PUCCH transmissions of PUCCH repetition, in case the actual TDW is created in response to an event triggered by DCI other than frequency hopping or by MAC-CE, subject to UE capability.

<omitted text>

### 6.3.1	Frequency hopping for PUSCH repetition Type A and for TB processing over multiple slots

For PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2 and for TB processing over multiple slots (as determined according to procedures defined in Clause 6.1.2.1 for scheduled PUSCH, or Clause 6.1.2.3 for configured PUSCH), a UE is configured for frequency hopping by the higher layer parameter frequencyHoppingDCI-0-2 in pusch-Config for PUSCH transmission scheduled by DCI format 0_2, and by frequencyHopping provided in pusch-Config for PUSCH transmission scheduled by a DCI format other than 0_2, and by frequencyHopping provided in configuredGrantConfig for configured PUSCH transmission. For PUSCH repetition Type A scheduled by RAR UL grant or by DCI format 0_0 with CRC scrambled by TC-RNTI, a UE is configured for frequency hopping by the frequency hopping flag information field of the RAR UL grant, and by the frequency hopping flag information field of DCI format 0_0 with CRC scrambled by TC-RNTI, respectively. One of two frequency hopping modes can be configured:

-	Intra-slot frequency hopping, applicable to single slot and multi-slot configured PUSCH transmission and multi-slot PUSCH transmission scheduled by DCI format 0_1 or 0_2 and each of multiple PUSCH transmissions scheduled by a DCI if the higher layer parameter pusch-TimeDomainAllocationListForMultiPUSCH is configured.

-	Inter-slot frequency hopping, applicable to multi-slot PUSCH transmission.

For operation with shared spectrum channel access, the UE does not expect that two hops of a PUSCH transmission are in different RB sets.

In case of resource allocation type 2, the UE transmits PUSCH without frequency hopping.

In case of resource allocation type 1, whether or not transform precoding is enabled for PUSCH transmission, the UE may perform PUSCH frequency hopping, if the frequency hopping field in a corresponding detected DCI format or in a random access response UL grant is set to 1, or if for a Type 1 PUSCH transmission with a configured grant the higher layer parameter frequencyHoppingOffset is provided, otherwise no PUSCH frequency hopping is performed. When frequency hopping is enabled for PUSCH, the RE mapping is defined in clause 6.3.1.6 of [4, TS 38.211].

For a PUSCH scheduled by RAR UL grant, fallbackRAR UL grant, or by DCI format 0_0 with CRC scrambled by TC-RNTI, frequency offsets are obtained as described in clause 8.3 of [6, TS 38.213]. Otherwise, for a PUSCH scheduled by DCI format 0_0/0_1 or a PUSCH based on a Type2 configured UL grant activated by DCI format 0_0/0_1 and for resource allocation type 1, frequency offsets are configured by higher layer parameter frequencyHoppingOffsetLists in pusch-Config. For a PUSCH scheduled by DCI format 0_2 or a PUSCH based on a Type2 configured UL grant activated by DCI format 0_2 and for resource allocation type 1, frequency offsets are configured by higher layer parameter frequencyHoppingOffsetListsDCI-0-2 in pusch-Config.

-	When the size of the active BWP is less than 50 PRBs, one of two higher layer configured offsets is indicated in the UL grant.

-	When the size of the active BWP is equal to or greater than 50 PRBs, one of four higher layer configured offsets is indicated in the UL grant.

For PUSCH based on a Type1 configured UL grant the frequency offset is provided by the higher layer parameter frequencyHoppingOffset in rrc-ConfiguredUplinkGrant.

For a MsgA PUSCH the frequency offset is provided by the higher layer parameter as described in [6, TS 38.213].

In case of intra-slot frequency hopping, the starting RB in each hop is given by:

,

where i=0 and i=1 are the first hop and the second hop respectively, and  is the starting RB within the UL BWP, as calculated from the resource block assignment information of resource allocation type 1 (described in Clause 6.1.2.2.2) or as calculated from the resource assignment for MsgA PUSCH (described in [6, TS 38.213]) and is the frequency offset in RBs between the two frequency hops. The number of symbols in the first hop is given by , the number of symbols in the second hop is given by , where NsymbPUSCH,s is the length of the PUSCH transmission in OFDM symbols in one slot.

In case of inter-slot frequency hopping and when PUSCH-DMRS-Bundling is not enabled, the starting RB during slot  is given by:

,

where  is the current slot number within a system radio frame, where a multi-slot PUSCH transmission can take place,  is the starting RB within the UL BWP, as calculated from the resource block assignment information of resource allocation type 1 (described in Clause 6.1.2.2.2) and is the frequency offset in RBs between the two frequency hops.

In case of inter-slot frequency hopping and when PUSCH-DMRS-Bundling is enabled, the starting RB during slot  is given by: [

RBstartns,fμ=RBstart RBstart+RBoffsetmodNBWPsize   ns,fμ+nf∙Nslotframe,μNFH mod 2=0ns,fμ+nf∙Nslotframe,μNFH mod 2=1

]

where ns,fμ is the current slot number within a system radio frame, nf is the number of the system radio frame containing the current slot, Nslotframe,μ is the number of slots per frame for subcarrier spacing configuration μof the UL BWP that the PUSCH is transmitted on, NFH is the value of the higher layer parameter PUSCH-Frequencyhopping-Interval,  is the starting RB within the UL BWP, as calculated from the resource block assignment information of resource allocation type 1 (described in Clause 6.1.2.2.2) andis the frequency offset in RBs between the two frequency hops.

<omitted text>