3GPP TSG RAN WG1 #109-e	R1-2203276

e-Meeting, May 9th – 20th, 2022

Agenda item:		9.10.1

Source:	Nokia, Nokia Shanghai Bell

Title:	On multi-cell PUSCH/PDSCH scheduling with a single DCI

Document for:		Discussion and Decision

# Introduction

In this contribution, we discuss multi-cell PUSCH/PDSCH scheduling with a single DCI as part of the multi-carrier enhancements for NR WI based on the following justification ad related objective defined in the WID in RP-220834:

Justification: One motivation is to increase flexibility and spectral/power efficiency on scheduling data over multiple cells including intra-band cells and inter-band cells. The current scheduling mechanism only allows scheduling of single cell PUSCH/PDSCH per a scheduling DCI. With more available scattered spectrum bands or wider bandwidth spectrum, the need of simultaneous scheduling of multiple cells is expected to be increasing. To reduce the control overhead, it is beneficial to extend from single-cell scheduling to multi-cell PUSCH/PDSCH scheduling with a single scheduling DCI. Meanwhile, trade-off between overhead saving and scheduling restriction has to be taken into account. Objective: 1. Specify a solution for multi-cell PUSCH/PDSCH scheduling (one PDSCH/PUSCH per cell) with a single DCI [RAN1] - Identify the maximum number of cells that can be scheduled simultaneously - Consider both intra-band and inter-band CA operation - Consider both FR1 and FR2 - The single DCI shall be optimized for 3 or more cells for the multi-cell PUSCH/PDSCH scheduling

In Section 2, we discuss general considerations regarding the related objective of the multi-cell scheduling DCI. Aspects on the overall scheduling framework for multi-cell DCI operation are discussed in Section 3.

# General considerations

Looking at the issue at hand, we see that mainly the following points (on highest level) will need to be addressed when introducing multi-cell scheduling with a single DCI in Rel-18:

- Overall PDSCH/PUSCH scheduling framework for a cell when introducing the multi-cell DCI, which may at least include

  - New DCI format(s) for PDSCH/PUSCH vs. extending legacy DCI formats

  - Interaction of single-cell scheduling DCI and multi-cell scheduling DCI operation for a cell – and related PDCCH monitoring and decoding (restrictions)

  - …

- Overall intended application scenarios for multi-cell DCI scheduling operation and overall design framework of the multi-cell scheduling DCI, including at least

  - Maximum number of schedulable cells, definition of schedulable cells and how to indicate the scheduled cell(s) in the multi-cell DCI

  - Support for scheduling cells with different SCS, inter-band CA, different UL/DL BWP bandwidths,…

    - Note: the assumptions here will have a large effect on possible optimizations in terms of DCI size (i.e. common versus cell-specific DCI fields)

  - Optimization for specific application scenarios (e.g. inter-band CA with same SCS) vs generic DCI design for all potential use cases?

- Detailed multi-cell scheduling DCI definition

  - This is mainly focusing on discussions with DCI fields are required for multi-cell scheduling with a single DCI – as well as with DCI fields are common to all scheduled cells or are to be scheduled cell specific

These different areas are clearly not independent. Especially the detailed multi-cell DCI definition (e.g. common vs. cell-specific fields) will be very much dependent on the overall assumptions on the overall scheduling framework for a cell as well as the design framework of the multi-cell scheduling DCI. Therefore, we think it would be better to focus the discussions on these aspects in the early WI phase before discussing detailed multi-cell DCI definition DCI field per DCI field.

Proposal 2.1: Focus the discussions in the early WI phase on overarching principles for multi-cell PDSCH/PUSCH scheduling incl. overall scheduling framework for a cell, intended application scenarios and multi-cell DCI design framework assumptions before discussing the details of the multi-cell DCI content DCI field per DCI field.

Consequently, in the rest of this contribution, we focus on the first two bullet points while expect to address the 3rd point after the overarching principles have been stabilized.

The work done for cross-carrier scheduling in Rel-15…Rel-17 should be used for the multi-cell DCI as well. The multi-cell DCI should not differ from any operational standpoint from multiple single-cell scheduling DCIs of cross-carrier scheduling. The one difference naturally being that the multiple scheduling DCIs are encoded as a single multi-cell DCI delivered over one PDCCH. That is, apart from PDCCH processing and DCI encoding, the cross-carrier scheduling framework as specified in the earlier releases applies as-is.

Proposal 2.2: The Rel-15…Rel-17 cross-carrier scheduling framework applies as-is; The multi-cell DCI is considered the scheduling DCI, and the PDCCH transmitting it is considered the scheduling PDCCH and the scheduled PxSCH processing and timelines as specified for cross-carrier scheduling are used the same way as with single-cell DCI.

# Scheduling framework for multi-cell DCI operation

## New DCI formats vs. extending existing DCI formats

One of the first questions to answer is that if the single DCI scheduling PDSCH or PUSCH on multiple cells is enabled via new RAN1 DCI formats 0_X an 1_X or if existing legacy DCI formats (e.g. DCI formats 0_1 & 1_1) are extended in Rel-17 to introduce the multi-cell scheduling functionality. The decision will affect further considerations on the overall scheduling framework. For example, if a cell can be scheduled with larger multi-cell DCI and single-cell DCI, less serving-cell-specific flexibility in terms of cell-specific DCI fields for the multi-cell DCI would be needed as the serving cell can still be scheduled with full flexibility (if needed case by case) using the legacy single-cell DCIs. This will led to a smaller overall multi-cell DCI size making the feature more useful in terms of control resource savings and/or detection reliability.

From our perspective, we think that it would be better to introduce some new DCI formats 0_X and 1_X, as this will simplify the specification text and give more flexibility in the DCI design, as the potential legacy DCI content not applicable for multi-cell scheduling (needing further discussion, e.g. support for CBG, multi-slot or multi-TRP scheduling, …) would not need to be described in the same DCI format. Moreover, introducing a new DCI format would be more aligned with related decision taken when the ‘compact DCI’ was introduced for Rel-16 URLLC. Therefore, we suggest introducing new DCI formats.

Proposal 3.1: Introduce new DCI formats 0_X (e.g. 0_3) for multi-cell PUSCH scheduling with a single DCI and 1_X (e.g. 1_3) for multi-cell PDSCH scheduling with a single DCI.

## Scheduling possibilities for a cell that can be scheduled by a multi-cell DCI

The most basic question to answer is if the UE can monitor several multi-cell-scheduling DCIs in several scheduling cells that are configured to schedule a particular (set of) cell(s). This would loosely correspond to a cross-carrier scheduling configuration where several scheduling cells could be configured to schedule a PxSCH in one cell. This would add to the network DCI loading flexibility but is likely adding more UE side implementation burden and add to the blind decoding budget management complexity.

Figure 1: Only one scheduling cell can be configured to schedule one set of scheduled cells.

Proposal 3.2.1: Each scheduled cell can be configured to be scheduled by a multi-cell DCI in one and only one scheduling cell.

Another question to be answered in an early phase is: if a scheduled cell is configured for multi-cell DCI scheduling, can it also still be scheduled by single-cell DCI scheduling (using the legacy DCIs)? We think there is still a need to support  single-cell DCI scheduling as well, because using the larger multi-cell DCI when scheduling only a single cell seems to be rather inefficient.

Proposal 3.2.2: Support the combination of multi-cell DCI scheduling and single-cell DCI scheduling (using legacy DCI formats) for PDSCH (or PUSCH) of a serving cell.

Moreover, it needs to be discussed whether the single-cell DCI and multi-cell DCI for PDSCH (or PUSCH) is to be monitored at the same scheduling cell or if a combination of self-scheduling using single-cell (legacy) DCI and (cross-carrier scheduling) using the multi-cell DCI monitored on another serving cell is to be supported. Three possibilities are depicted in Figure 2, Figure 3 and Figure 4.

In Figure 2, the single-cell-scheduling DCIs are monitored in the same cell as the multi-cell scheduling DCI. In Figure 3, the single-cell-scheduling DCIs are “self-scheduling”, i.e. on the same cell as the scheduled PxSCH, Finally, Figure 4 represents the case where single-cell-scheduling DCIs are monitored on a different cell through cross-carrier scheduling as the multi-cell scheduling DCI.

Figure 2: Single-cell-scheduling DCIs are monitored in the same cell as the multi-cell-scheduling DCIs only

Figure 3: Single-cell-scheduling DCIs are all “self-scheduling”, i.e. in the same cell as the scheduled PxSCH

Figure 4: Single-cell-scheduling DCIs are monitored in a different serving cell through 
cross-carrier scheduling as the multi-cell-scheduling DCI is not supported

In the current specifications there is no support of self- and cross-carrier scheduling for an SCell, and only a single scheduling cell for a scheduled SCell is supported. While for single-DCI scheduling (up to Rel-18) this may be a reasonable restriction, we think that with the introduction of the multi-cell scheduling DCI this restriction should be lifted. Note that the motivation for the multi-cell DCI is DL control overhead saving whereas the intention for cross-carrier scheduling has been more reliability aspect of DL control information and PDCCH capacity relief. If only multi-cell DCI scheduling and single-cell DCI scheduling from the same scheduling cell is to be supported based on Figure 2, this would very quickly lead to very (too) high DL control load on the scheduling cell. Moreover, having a scheduling combination of single-cell DCI self-scheduling and multi-cell DCI scheduling as illustrated in Figure 3 enabled could be used to operate initial PDSCH/PUSCH transmissions using multi-cell DCI more independently and potentially needed re-transmissions using single-cell DCI without the need for e.g. independent/cell-specific HARQ-ID, RV & NDI reducing the multi-cell DCI size. Therefore, we think a combination of self-scheduling for single-cell (legacy DCI) and ‘cross-carrier’ scheduling using a multi-cell DCI should be supported. At the same time, we do not really see the need to support the combination of ‘cross-carrier’ multi-cell DCI scheduling from one scheduling cell and single-cell DCI cross-carrier scheduling from another serving cell based on Figure 4.

Proposal 3.2.3: For a scheduled cell,

- support multi-cell DCI and single-cell DCI scheduling from one scheduling cell

- support multi-cell DCI scheduling from one scheduling cell and single-cell DCI self-scheduling

- do not support multi-cell DCI and single-cell DCI cross-carrier scheduling from more than one (other) scheduling cell.

## Maximum number of scheduled cells and scheduled cell indication in the DCI

The WID already defines that at least 3 cells should be schedulable, but the maximum schedulable number of cells would need to be further discussed.

First we think, that RAN1 should be clear if the MC scheduling is to be limited to be within a single cell group or across cell groups. As the usual operation (and also the related e.g. HARQ feedback) is per cell group, we think that the multi-cell scheduling should be restricted to be within a single cell group.

Proposal 3.3.1: The multi-cell PUSCH scheduling with a single DCI it to be restricted within a single (PUCCH) cell group.

On the maximum number of schedulable cells within a cell group, clearly the number will need to be at least 3 (based on the WID) but can be definitely larger. Overall, we think that having too many cells schedulable by a single DCI will just increase the DCI size unnecessarily, as the probability of scheduling at the same time a very large number of cells  will likely get smaller when the number of cells increases, while the achievable relative reduction in PDCCH loading compared to single-cell scheduling DCIs gets also smaller. So from this perspective, we think that a reasonable absolute maximum of schedulable cells should be either 4, 6 or 8. But considering the rather small probability of (a) needing to schedule more than 4 cells at the same time and (b) having the ability to having more than one scheduling DCI transmitted to scheduled up to 4 cells, we think that limiting this to a maximum of 4 cells seems to be a reasonable assumption. Note that for FR2 deployments where a UE can be configured with up to 8 schedulable cells, since the DCI payload size for 8 cells could be excessive, later in the document we discuss and propose to allow the configuration of at least 2 multi-cell DCIs within a single PUCCH cell group.

Proposal 3.3.2: Support a maximum of 4 cells that can be scheduled simultaneously by a single DCI.

But having the maximum number of cells (from specification perspective) that can be scheduled limited to e.g. 4 cells, does not mean that for each operation up to 4 cells should be schedulable or the UE is having CA with 4 cells configured in the first place. As discussed above, the probability for such large number of cells will be rather small and at the same time the DCI size would still be large. Therefore, we think the actual maximum number of schedulable cells should be left to the gNB and therefore, some type of RRC configuration on the maximum number of schedulable cells should be supported.

Moreover, considering the multi-cell scheduling DCI operation for PDSCH and PUSCH, the probability for having several PUSCHs scheduled is much lower considering the lower UL CA capabilities for a typical NR UE as well as the generic issue of limited transmission power in UL (compared to DL). As a consequence, we think that the related configuration should be independent for scheduling PDSCH and PUSCH.

Proposal 3.3.3: To limit the DCI size, the maximum number of cells that can be scheduled should be based on RRC configuration (i.e. from the set of {2,3,4}).

Proposal 3.3.4: Support separate configurations for the multi-cell scheduling DCI for PDSCH and PUSCH.

Another point that is related is how to define which cells can be scheduled by a single DCI from a certain cell and how to indicate (in the scheduling DCI) which (subset) of these cells are actually scheduled. For this, two well-known approaches can be considered:

- Alt. 1: The gNB configures a list of K cells that can be scheduled by the multi-cell DCI and the multi-cell DCI contains a bitmap of length K, indicating which K cells are actually scheduled by the DCI.

  - This allows for full freedom to the gNB to select any combination of cells from a single cell up to the maximum number of cells that can be scheduled, but will lead to a DCI overhead of K bits.

- Alt. 2: The gNB configures a table of scheduled cells, where each table entry / row defines the scheduled cell(s) and the DCI indicates which table entry / row to define the scheduled cells.

  - This could lead to a smaller DCI overhead, as the indication will be limited to the L configured multi-cell scheduling combinations and may therefore require less DCI overhead for the scheduled cell indications.

  - An example list or table could look like this:

| Multi-cell scheduling combination | Scheduled cells |
| --- | --- |
| 0 | Cell A, Cell B |
| 1 | Cell A, Cell C, Cell D |
| 2 | Cell B, Cell C |
| … |  |
| L-1 | Cell C, Cell F, Cell G |

As Alt. 2 can provide a lower DCI overhead for the indication of the scheduled cells, we prefer Alt. 2 as the configuration method of schedulable cells and dynamic indication method.

Proposal 3.3.5: The scheduled cells are indicated in a DCI field pointing to a table of scheduled cell(s).

- The table of scheduled cell(s) to be scheduled is RRC configured for the UE.

- Support separate table configurations for the multi-cell scheduling DCI for PDSCH and PUSCH.

The discussions so far only focused on the multi-cell DCI to be monitored on a single scheduling cell for a scheduled cell but not yet if the UE within a PUCCH cell group could be configured with more than one multi-cell DCI for PDSCH (or PUSCH) that can schedule a different sub-group of serving cells within the (PUCCH) cell group. Allowing the multi-cell DCI for PDSCH (or PUSCH) to be only monitored for a single cell would result in extremely high DL control load on a single DL serving cell, as potentially more than one multi-cell scheduling DCI would need to be transmitted from the same serving cell for a single UE. By allowing to configure e.g. at least up to 2 multi-cell DCIs to be monitored on different serving cells, the DL control load thereby could be managed better by the network. An example of such operation is shown in Figure 5 below, where the UE monitors for a multi-cell DCI for a first subgroup of cells on PCell and for a second multi-cell DCI for a second subgroup of cells on SCell K within a single PUCCH cell group.

Figure 5: Operation of more than one multi-cell DCI within one PUCCH cell group

Proposal 3.3.6: Support the monitoring for at least two multi-cell DCIs for PDSCH (or PUSCH) on different scheduling cells within a PUCCH cell group, where each of the multi-cell DCIs can schedule a different (non-overlapping) subgroup of cells within a PUCCH cell group.

## DCI size budget considerations with MC-DCI scheduling

When introducing new DCI formats (of different sizes) as proposed in Sec. 3.1, the question that arises immediately is of course how to manage the current DCI size budget of a maximum of 3 DCI sizes with C-RNTI to be monitored per serving  cell (and the related DCI size alignment). Up to now it was possible to consider the DCI size budget per serving cell and apply the DCI size alignment procedures of Sec. 7.3.1.0 of 38.212 per scheduled cell, as there had not been any DCIs scrambled by C-RNTI applicable for more than one serving cell. But with the introduction of the multi-cell scheduling DCI, some further considerations on how to define the DCI size budget will be needed.

Observation 3.4.1: RAN1 needs to discuss the definition & handling of the DCI size budget for multi-cell DCI scheduling.

As we suggest supporting single-cell DCI and multi-cell DCI scheduling for a scheduled cell in Sec. 3.2, one needs to consider how to define the DCI size budget limits and how to take this into account in the DCI size alignment procedure.

One could argue that the MC-DCI(s) could be counted towards the DCI size budget for each serving cell the DCI is monitored, which is clearly not true as there is only a single DCI size applicable for the MC-DCI for PDSCH (or PUSCH). Using such assumption would require changing the DCI size alignment procedures taking the multi-cell DCI into account for all the serving cells and the need to discuss increasing the DCI size budget per serving cell, as otherwise the benefits of single-cell & multi-cell scheduling (especially for PCell, where fallback DCI monitoring is needed) could not be provided.

In contrast, we think it would be better to consider the MC-DCI outside the current DCI size budget framework. One option that comes to mind here would be to not count the MC-DCI(s) in the DCI size budget per serving cell at all but define an additional limitation across the serving cells handled by the MC-DCI. As an example, let’s consider the MCI-DCI scheduling combined with SC-DCI self-scheduling illustrated in Figure 3, where the UE may be required to monitor fallback DCIs 0_0/1_0 as well as the UE configurable DCI formats 0_1/0_2/1_1/1_2 for a scheduled cell in addition needs to monitor for the MC-DCIs 0_X/1_X, with the following assumptions:

- For each of the K serving cells (PCell, SCells 1…K-1), the DCI size alignment procedure and restrictions in Sec. 7.3.1.0 of TS 38.212 guarantee the maximum of 3 DCI sizes for single-cell DCI formats 0_0/0_1/0_2/1_0/1_1/1_2 scrambled by C-RNTI.

- In addition, with the introduction of the multi-cell DCI, an additional restriction could be imposed that the gNB would guarantee that across the K serving cells the total DCI size budget of 3*K DCI sizes is not exceeded, namely MC_DCI_sizes +k=1KSC_DCI_sizes(k)≤3*K where,

  - SC_DCI_sizes(k) is the number of single cell DCI sizes for the k-th serving cell

  - MC_DCI_sizes is the number of DCI sizes (1 or 2) of the multi-cell scheduling DCI(s) of the group of K serving cells.

With such kind of operation, (i) the current UE processing requirements in terms of number of DCI sizes would not be exceeded, (ii) there would be no need to discuss or specify a nominal DCI size increase per serving cell from 3 to e.g. 4 and (iii) maybe most importantly, there would be no need to change the DCI size alignment procedures and the related handling with the introduction of the multi-cell scheduling DCI.

Proposal 3.4.2: The multi-cell DCI size(s) are not counted towards the DCI size budget (for DCI formats scrambled by C-RNTI) per serving cell and not considered in the related serving cell specific DCI size alignment procedure. Instead, the gNB will guarantee that across the K cells applicable for multi-cell DCI scheduling that the total budget of 3*K DCI sizes is not exceeded.

Another issue to be considered is how to define the DCI size for the multi-cell scheduling DCI as the size of some cell-specific fields depend on the scheduled serving cells, such as e.g. FDRA depending on the UL/DL BWP size and TDRA on the configured set of serving cell combinations. As the DCI size itself should not vary depending on which cells are actually scheduled without exploding the number of different DCI sizes that the UE would need to be able to decode, some way to define the DCI size independently of the actual scheduled cell(s) will be needed. Here, defining the DCI size based on the scheduled cell combination resulting in the maximum DCI size would be one option, but also other and simpler options such as having the DCI size of the multi-cell scheduling defined by RRC configuration (also enabling UL & DL multi-DCI size alignment by RRC configuration) could be considered.

Observation 3.4.3: RAN1 needs to discuss the definition the DCI size of the multi-cell scheduling DCI.

## Intra-/Inter-band and same/mixed SCS multi-cell DCI scheduling

When considering aspects that will impact the size of the MC-DCI one of the key aspects to consider is the scheduling of cells which are intra-band vs inter-band. For intra-band cells there are number of DCI fields which could be common to all cells within the MC-DCI configuration reducing the overall size of the MC-DCI. On the other hand, inter-band could be a prevalent scenario specially in FR1. For inter-band MC-DCI there will be a need to consider different numerologies and even different duplexing (TDD and FDD) Furthermore, some of the intra-band MC-DCI size optimizations could also be applicable for inter-band when the configured cells have common characteristics.

Intra-band vs. inter-band optimization aspects to consider:

|  | intra-band | inter-band |
| --- | --- | --- |
| Duplexing | Same duplexing can be assumed | May mix TDD and FDD |
| SCS | Same SCS can be assumed | May mix different SCS |
| MCS | Similar MCS could be assumed | More complicated correlation between the MCSs |
| Antenna config | Same antenna configuration (MIMO mode, max # of layers) could be assumed | Different antenna configurations possible |
| Time-Domain resource assignment | Same TDRA configs maybe assumed | If duplexing and/or SCS differ, then also the TDRA configs will likely differ |
| PRB bundling size | Same PRB bundling size could be assumed | PRB bundling size may differ |
| Rate matching Indication | Same rate matching patterns could be assumed | Rate matching patterns may differ |
| … |  |  |

Proposal 3.5.1: The design of the MC-DCI should allow for optimization of the MC-DCI size when all cells within the MC-DCI have some commonalities, e.g. same numerology and duplexing mode. Note these optimizations need not be limited to intra-band case.

As discussed above, clearly for the same SCS handling there could be some DCI size optimizations possible (e.g. in terms of common TDRA indication for all scheduled cells) which are not possible for mixed SCS handling.

For the mixed SCS scheduling further considerations are needed:

- PDCCH-to-PDSCH minimum timing: Self-scheduling and cross-carrier scheduling with the same SCS have the same timeline from PDCCH to PDSCH. In contrast, cross-carrier scheduling with different SCS a longer minimum timing Npdsch as a function of the subcarrier spacing of the scheduling PDCCH. 
Clearly, RAN1 would need to discuss how to treat the different scheduled cells here, if the minimum PDCCH-to-PDSCH minimum timing is then given per scheduled cell individually (i.e. different earliest PDSCH start possible) or if the same minimum timing is to be applicable for all the scheduled cells (i.e. resulting in additional scheduling delays for same SCS cells). However, there doesn’t seem to be any reason to depart from single-cell PDCCH behaviour, where the PDCCH scheduling the multi-cell DCI is understood the same way as if it was multiple single-cell DCIs, and the timelines are treated as specified in Rel-16.

- Scheduling & number of DCIs UE can expect per DL slot / monitoring span: Mixed SCS will also lead to the case that different cells may need to be scheduled with a different ‘minimum scheduling frequency’.The PxSCH scheduling e.g. a 30 kHz cell would need to occur potentially twice as often as for a 15 kHz cell. Using the MC-DCI only per 15 kHz SCS slot will only allow scheduling of one of the two overlapping 30 kHz SCS slots, the question is how to operate the 2nd overlapping slot for the 30 kHz SCS cell. The options that clearly come to mind here would be to use SC-DCI scheduling the 2nd overlapping slots, or another (2nd) MC-DCI that would schedule these slots for the cells of the higher SCS (and not including the lower SCS cells). Clearly, RAN1 for this scheduling combination needs to discuss the intended possible scheduling options and also check if all the related UE capability signaling is in place in terms of number of DL/UL DCIs per slot / span that UE may expect to receive. Notably, the very same question was debated in Rel-16 where the cross-carrier scheduling with different SCS was introduced, and the multi-cell DCI doesn’t change the picture.

Proposal 3.5.2: For mixed SCS multi-cell DCI scheduling operation, apply the Rel-16 processing timelines as if the multi-DCI represented individual single-cell DCI, each scheduling a different carrier.

Observation 3.5.3: The question of contiguous scheduling when the scheduling carrier is of lower SCS than the scheduled carrier (low-to-high scheduling) was discussed in rel-16, and additional UE capabilities for increased multiple DCI processing capabilities per slot were introduced. Application of these capabilities with multi-cell DCI should be considered in the later stage of the WI.

# Conclusion

In this contribution with discussed multi-cell DCI scheduling aspects for Rel-18 CA enhancements.

Based on the general considerations in Sec. 2 and the following related proposals:

- Proposal 2.1: Focus the discussions in the early WI phase on overarching principles for multi-cell PDSCH/PUSCH scheduling incl. overall scheduling framework for a cell, intended application scenarios and multi-cell DCI design framework assumptions before discussing the details of the multi-cell DCI content DCI field per DCI field.

- Proposal 2.2: The Rel-15…Rel-17 cross-carrier scheduling framework applies as-is; The multi-cell DCI is considered the scheduling DCI, and the PDCCH transmitting it is considered the scheduling PDCCH and the scheduled PxSCH processing and timelines as specified for cross-carrier scheduling are used the same way as with single-cell DCI.

our considerations on the overarching principles for the Rel-18 multi-cell DCI scheduling operation in Sec. 3 can be summarized in the following observations and proposals:

- Proposal 3.1: Introduce new DCI formats 0_X (e.g. 0_3) for multi-cell PUSCH scheduling with a single DCI and 1_X (e.g. 1_3) for multi-cell PDSCH scheduling with a single DCI.

- Proposal 3.2.1: Each scheduled cell can be configured to be scheduled by a multi-cell DCI in one and only one scheduling cell.

- Proposal 3.2.2: Support the combination of multi-cell DCI scheduling and single-cell DCI scheduling (using legacy DCI formats) for PDSCH (or PUSCH) of a serving cell.

- Proposal 3.2.3: For a scheduled cell,

  - support multi-cell DCI and single-cell DCI scheduling from one scheduling cell

  - support multi-cell DCI scheduling from one scheduling cell and single-cell DCI self-scheduling

  - do not support multi-cell DCI and single-cell DCI cross-carrier scheduling from more than one (other) scheduling cell.

- Proposal 3.3.1: The multi-cell PUSCH scheduling with a single DCI it to be restricted within a single (PUCCH) cell group.

- Proposal 3.3.2: Support a maximum of 4 cells that can be scheduled simultaneously by a single DCI.

- Proposal 3.3.3: To limit the DCI size, the maximum number of cells that can be scheduled should be based on RRC configuration (i.e. from the set of {2,3,4}).

- Proposal 3.3.4: Support separate configurations for the multi-cell scheduling DCI for PDSCH and PUSCH.

- Proposal 3.3.5: The scheduled cells are indicated in a DCI field pointing to a table of scheduled cell(s).

  - The table of scheduled cell(s) to be scheduled is RRC configured for the UE.

  - Support separate table configurations for the multi-cell scheduling DCI for PDSCH and PUSCH.

- Proposal 3.3.6: Support the monitoring for at least two multi-cell DCIs for PDSCH (or PUSCH) on different scheduling cells within a PUCCH cell group, where each of the multi-cell DCIs can schedule a different (non-overlapping) subgroup of cells within a PUCCH cell group.

- Observation 3.4.1: RAN1 needs to discuss the definition & handling of the DCI size budget for multi-cell DCI scheduling.

- Proposal 3.4.2: The multi-cell DCI size(s) are not counted towards the DCI size budget (for DCI formats scrambled by C-RNTI) per serving cell and not considered in the related serving cell specific DCI size alignment procedure. Instead, the gNB will guarantee that across the K cells applicable for multi-cell DCI scheduling that the total budget of 3*K DCI sizes is not exceeded.

- Observation 3.4.3: RAN1 needs to discuss the definition the DCI size of the multi-cell scheduling DCI.

- Proposal 3.5.1: The design of the MC-DCI should allow for optimization of the MC-DCI size when all cells within the MC-DCI have some commonalities, e.g. same numerology and duplexing mode. Note these optimizations need not be limited to intra-band case.

- Proposal 3.5.2: For mixed SCS multi-cell DCI scheduling operation, apply the Rel-16 processing timelines as if the multi-DCI represented individual single-cell DCI, each scheduling a different carrier.

- Observation 3.5.3: The question of contiguous scheduling when the scheduling carrier is of lower SCS than the scheduled carrier (low-to-high scheduling) was discussed in rel-16, and additional UE capabilities for increased multiple DCI processing capabilities per slot were introduced. Application of these capabilities with multi-cell DCI should be considered in the later stage of the WI.